SCHEDULE B.

NAME OF WORK 	S/R TO D.S. FLAT NO.37 AT GOR BATH ISLAND KARACHI.

(RESIDENCE OF MR. GHAZANFAR HUSSAIN QADRI)

| S.NO. | DESCRAPTION OF ITEMS | QUTY | RATE | UNIT | AMOUNT |
| --- | --- | --- | --- | --- | --- |
| 1 | Scraping ordinary distemper oil bound distemper on walls(S.I.NO.54-A/13) | 5760.44Sft | 226/88 | %Sft | 13069 |
| 2 | First class deodar wood wrought joinery in doors and windows etc fixed in position including chowkats hold fasts hinges, iron tower bolts chocks cleats handles and cords with hooks etc deodar paneled or paneled and glazed or fully glazed (b)1-3/4” thick(S.I.NO.7-b/58).(1273/76(-)370/83=902/93(Only Shutter). | 73.50 | 902/93 | Sft | 66365 |
| 3 | Distempering two coats (S.I.NO.24-b/P-54) | 1570.15 | 1043/90 | %Sft | 16391 |
| 4 | First class deodar wood wrought joinery work in wire gauze door & windows with 22 SWG galvanized wire gauze 144 mesh per sq inch iron fittings complete. 1 ½” thick deodar wood framing in wire gauze with ordinary hinges (S.I.14-b/p-59) | 19.50 Sft | 562/98 | P.Sft | 10978 |
| 5 | Painting doors and windows any type any type(S.I.NO.4-C-I+II/76) | 965.50Sft. | 1160/06 | %Sft | 11200 |
| 6 | Painting guard bars, gates of iron bars gratings railings including standard braces etc and similar open work first oat each subsequent cost(S.I.4-D-I+II/69) | 239.0Sft | 674/60 | %Sft | 1612 |
| 7 | Preparing the surface & painting with matt finish paint of approved make to old matt finish surface 2nd each subsequent coat (S.I.37-A+B/55) | 4190.00 Sft | 1772/38 | %Sft | 74263 |
| 8 | Providing and fixing approved quality mort ice lock (S.I.NO.21/60)- | 4-Nos | 1786/13 | Each | 7144 |
| 9 | P/f deodar wooden ward rob i/c boxing with back sheves shutters drawers,and brass fitting such as handles locking arrangement hanger rod shoe rod and mirror measuring 2’x3’x1’ complete as per approved design (S.I.NO 24-p/61) | 51.00 Sft | 2364/63 | P. Sft | 120596 |
| 10 | Nylon wire gauze 144 mesh per square inch fixed to chow Kats with3/4’ deodar strips and screws(S.I.63-a/68) | 48.75 | 193/53 | P.Sft | 9435 |
| 11 | Fixing expanded metal with 1” deodar wooden strips and screws(22-a/61) | 48.75 | 296/05 | P.Sft | 14432 |
| 12 | Providing and fixing with sunk iron screw wooden Architrave approved design / shape having with not less than 2-1/2 inches as directed by Engineer incharge..(S.I.No.60 /66) | 114.0 Sft | 49/97 | P. Sft | 5697 |
| 13 | P/f aluminieum sheet on doors pasted with glue as per requirement. (S.I.61/p-66). | 40.50 Sft | 63/77 | P.Sft | 2583 |
|  | Total |  |  |  | 353765 |
|  | Part B S.Item |  |  |  |  |
| 1 | Supplying and fixing in position aluminum channels framing for sliding windows and ventilators of alcop make with 5mm thik tinted glass glazing Belgium and aluminum glass glazing Belgium and aluminum fly screen including handles stoppers and locking arrangement etc complete duluxe model white (S.I.NO.84-a/108) | 72.00 | 1592/69 | P.Sft | 114674 |
|  | TOTAL |  |  |  | 114674 |
|  | Part C W/S & S/F |  |  |  |  |
| 1 | P/F 22”x16” lavatory basin in white glazed earthen ware complete with & i/c the cost of WI or CI cantilever brackets 6 inches built into walls painted white in 2 coats after a primary coat of red lead paint a pair of ½”rubber plug & chrome brass waste of approved patten 1-1/4” dia mallable iron of cp brass unons and making requisite number of holes in walls plinth and floor for pipe connection and making good in cement concrete 1.2.4 (foreign or equilent) (S.I.13/4). | 1-Nos | 6237/- | Each | 6237 |
| 3 | S/F wash basin mixture of superior quality with CP head ½”dia (S.I.14-a/19) | 1-No | 2882/- | Each | 2882 |
| 4 | S/F sink mixture cock of superior quality with cp head etc (S>i.17/19) | 1-Nos | 2745/60 | Each | 2746 |
| 5 | Providing and fixing 15”x12” beveled edge mirror of Belgium glass complete with 1/8” thick hard board and C.P screws fixed to wooden pleat (b)superior patern(S.I.NO.3-b | 1-Nos | 2047/76 | Each | 2048 |
| 6 | Supplying & fixing bath room accessories set(7 piece)I/c towel rod, brush holder, soap tray , shelf of approved design I/c cost of screws, nuts etc. complete ()master brand)(25/16) | 1-No | 8122/- | Each | 8122 |
| 7 | S/F muslim Shower with double bib cock etc complete. | 1-Nos | 3432/- | Each | 3432 |
|  | Total |  |  |  | 25467 |
|  | Part C Electric Items |  |  |  |  |
| 1 | P/F brass ceiling fan 56” good quality | 3-Nos | 3185/- | Each | 9555 |
|  | Total C |  |  |  | 9555 |
|  | Part D NSI |  |  |  |  |
| 1 | S/F of Steel Sink Bowl as approved shape &size fixing with existing marble top etc complete | 1-No |  | Each |  |
| 2 | Making and fixing wooden cabinet American /Italian style with shutters of lasani wood sheet ¾” thick frame work of first class deodar wood 2”x1” pasted with classic Formica and 18” deep including necessary hanges catcher handle sliding wheels in draws nails /screws etc with approved design and shape. The cost also includes necessary tools and plants to be used in making etc complete as directed by the engineer incharge etc complete | 129.44 Sft |  | P.Sft |  |
| 3 | Providing and fixing marble top ¾” thick as approved colour make and design including making edges with round shape and chemical polishing including the cost of material labor Westgate and cartage etc complete | 44.00 Sft |  | P.Sft |  |
| 4 | S/F of fancy switch /socket imported make with double sheets fixing with existing concealed board on approved shape and design by the E.E. incharge etc complete. | 10-Nos |  | Each |  |
| 5 | S/F fancy type fan dimmer imported make with existing fancy sheet as approved shape design and make by the E.E. incharge including necessary electric connection etc. | 3-nos |  | Each |  |
| 6 | Providing and fixing universal POWER plug 3 pin 20 amps fancy type imported quality with fancy PVC concealed board and necessary screws and electric wiring and connection (N.S.I) | 1-No |  | Each |  |
| 7 | Providing and fixing universal light plug 3 pin 13 amps fancy type imported quality with fancy PVC concealed board and necessary screws and electric wiring and connection (N.S.I) | 1-Nos |  | Each |  |
|  | Total D |  |  |  |  |
|  | G.Total |  |  |  |  |
|  | Say Rs |  |  |  |  |

TERMS AND CONDTION.

Any typographical errors in the schedule B are subject to the correction with

Reference to the Schedule of rate General 2012 enforced from 12.07.2012 as

Approved by the S R C, Sindh Karachi

Arbitration clause stands deleted from the agreement.

No premium shall be paid on a Non schedule items.

100% WELL Graded crushed bajri shall be used in items of work without any

extra payment

No Any items of exvceed then the Quantity Provided in the Schedule B without

Prior of the Executive Engineer.

6.  All sample of items should be got approved by the Executive Engineer.

CONTRACTOR       SUB ENGINEER                          ASSISTANT ENGINEER

PROVINCIAL BUILDINGS SUB DIVISION-I

KARACHI.