Draft  Bidding Document for works upto 1.00 M.

Work No. 2

N.I.T – 11 Works.

NIT-075/15

NAME OF WORK: Improvement of footpath main road Memon Goth and Mullah Essa Goth Jam

Goth, Bhiroo Goth old Thana UC Mruad Memon

BILL OF QUANTITIES

- Description and rate of Items based on Composite Schedule of Rates.

| ### Sr. No | Quantities | Description of item to be executed at site | Rate | Unit | Amount in Rupees |
| --- | --- | --- | --- | --- | --- |
| ### 1 | 2 | 3 | 4 | 5 | 6 |
| 1 | 842.00 Rft | Kerb Stone Providing & Fixing precast Edge Block 3750 PSI Industrial Made size 6 inches thick x 12 inches long x12 inches high including the cost of cartage, excavation, from work for haunching 1450 PSI lean concrete 2250 PSI concrete for haunching 1:4 cement sand mortor (Highway Sch P-16/14) | 297/01 | P Rft | 2,50,042/= |
| 2 | 3850.00 Cft | Providing / Laying cement concrete brick or stone ballast 1-1/2” to 2” gauge etc complete (Gen Sch P-16/4 b) (in ratio 1:4:8). | 9416/28 | % Cft | 3,62,527/= |
| 3 | 231.00 Cft | P/L cement concrete plain including placing compacting finishing & curing etc complete i/c screening and washing stone aggregate without shuttering (Gen Sch P-15/ 5 f) (in ratio 1:2:4 ) | 14429/25 | % Cft | 33,332/= |
| 4 | 6600.00 Sft | Cement concrete tiles laid flat in 1:2 cement mortar over ¾” thick bed of cement mortar 1:2 (Gen Sch P-41/item 14-a) | 5247/82 | % Sft | 3,46,356/= |
| 5 | 2200.00 Sft | SCRAPING (b) Ordinary distemper oil bound distemper or paint on walls (Gen Sch P-13/54-b) | 226/88 | % Sft | 4,991/= |
| 6 | 2200.00 Sft | Painting old surfaces (a) Painting corrugated surfaces, patent roofing etc with oil paint (03 coats ) (Gen Sch P-68/4) | 1486/21 | % Sft | 32,697/= |

Amount Total (a) Rs. 10,29,985/=

______________ % Above / Below on the rates of CSR.		Amount to be added / deducted on the basis

Of premium quoted                     Total (b)

Total (A) = a+b in words & figures:

Contractor							Executive Engineer / Procuring Agency

(Cont…….Page No. 2)

(NIT-11 Works)

Page No. 02  (Work No. 02)

Summary of Bill of Quantities

Cost of Bid 								Amount__________________

________________________

- ( A) Cost based on composite Schedule of Rate

- (B) Cost of based on Non / Offered Schedule of Rates.			__________________________

TOTAL COST OF BID ( C ) = Total (A) + Total (B) 			__________________________

Contractor						Executive Engineer / Procuring Agency