Draft  Bidding Document for works upto 1.00 M.                                                                                                                      Work No. 7

N.I.T – 11 Works. NIT-075/15

NAME OF WORK: P / L / J & Testing  Water Line at Fateh Muhammad Goth to Machar Colony

UC Gujro

BILL OF QUANTITIES

- Description and rate of Items based on Composite Schedule of Rates.

| ### Sr. No | Quantities | Description of item to be executed at site | Rate | Unit | Amount in Rupees |
| --- | --- | --- | --- | --- | --- |
| ### 1 | 2 | 3 | 4 | 5 | 6 |
| 1 | 20885.00 Cft | Excavation of pipe line in trenches and pits in all kind of soils i/c trimming and dressing sides to true alignment and shape leveling of beds of trenches to correct level and grade, cutting joints holes and disposal of surplus earth within a one chain as directed by Engineer Incharge providing fence guards lights flags and temporary crossings for non-vehicular traffic where ever required lift upto 5 ft & lead upto one chain (PHED Sch P.No.62/5) | 4650.00 | ‰Cft | 97,115/= |
| 2 | 5238.00 Cft | Excavation of pipe line in trenches and pits SOFT ROCK by hammering & chelsing where blasting is not practicable or prohibited i/c trimming and dressing sides to true alignment and shape leveling of beds of trenches to correct level and grade, cutting joints holes and disposal of surplus earth within a one chain as directed by Engineer Incharge providing fence guards lights flags and temporary crossings for non-vehicular traffic where ever required lift upto 5 ft & lead upto one chain (PHED Sch P.No.62/7) | 10770.00 | ‰Cft | 56,413/= |
| 3 | 2700.00 Rft 1477.00 Rft | Providing / Laying & Fixing in trench i/c fitting jointing & testing etc complete in all respect the high Density Polythylene PE pipes (HDPE-100) PN-10 for W/S confirming ISO 4427/DIN8074/8075 B.S 3580 & PSI 3051 PHEDSchW.SPage25/1 ) 3” dia 2” dia | 148/= 81/= | P Rft P Rft | 3,99,600/= 1,19,637/= |
| 4 | 03 Nos | Supplying / Fixing 3” dia Cast iron sluice valve with flange ends rubber washer nut bolts heavy pattern etc complete (PHSMI P-2 / 9) | 5973/= | Each | 17,919/= |
| 5 | 06 Nos | Providing / Fixing C.I short piece (PHSI P-9/15) 3” dia | 375/= | Each | 2,250/= |
| 6 | 03 Nos | Providing Chamber size 21/2’ x 2’ x ½’ (750 x 750mm ) inside dimension 41/2” (1372mm) deep as per approved design for sluice valve 3” to 12” dia with 18” (457mm ) dia inside cost iron cover and frame (wt=1 cwt 3 qr) fixed in RCC 1:2:4 (102mm) thick (with 5 lbs steel per cft) 9” (299mm) thick cement concrete 1:3:6 in foundation 1: (25mm) thick cement concrete 1:2:4 flooring ½” (12/.5 mm) thick cement plaster 1:3 to all inside wall surface and to top i/c providing and fixing M/S foot rest at every one foot beyond 21/2” ft depth curing excavation back filling and disposal of surplus earth etc complete (PHSI No. 5 Page No. 51) | 23921/= | Each | 71,763/= |
| 7 | 23511.00 Cft | Refilling the excavated stuff in trenches 6” thick layer i/c watering ramming to full compaction etc complete (PHED Sch P-No. 77/24) | 2760.00 | ‰ Cft | 64,890/= |

Amount Total (a) Rs. 8,29,587/=

______________ % Above / Below on the rates of CSR.		Amount to be added / deducted on the basis

Of premium quoted                     Total (b)

Total (A) = a+b in words & figures:

Contractor							Executive Engineer / Procuring Agency

(Cont…….Page No. 2)

Work No. 7

N.I.T – 11 Works.

NIT-075/15

Page No. 02

Summary of Bill of Quantities

Cost of Bid 								Amount__________________

________________________

- ( A) Cost based on composite Schedule of Rate

- (B) Cost of based on Non / Offered Schedule of Rates.			__________________________

TOTAL COST OF BID ( C ) = Total (A) + Total (B) 			__________________________

Contractor						Executive Engineer / Procuring Agency