Draft  Bidding Document for works upto 1.00 M.

Work No. 7

N.I.T – 11 Works.

NIT-076/15

NAME OF WORK: P / L / J & Testing Sewerage line at different streets of Paradise society near

Bhiroo Goth UC Murad Memon

BILL OF QUANTITIES

- Description and rate of Items based on Composite Schedule of Rates.

| ### Sr. No | Quantities | Description of item to be executed at site | Rate | Unit | Amount in Rupees |
| --- | --- | --- | --- | --- | --- |
| ### 1 | 2 | 3 | 4 | 5 | 6 |
| 1 | 625.00 Cft | Dismantling & Removing road metaling (Gen Sch No. 13 / 51 ) | 605/= | % Cft | 3,781/= |
| 2 | 313.00 Cft | Dismantling Cement concrete plain 1:2:4 (Gen Sch No. 19 (c) P-10) | 3327.50 | % Cft | 10,415/= |
| 3 | 413.00 Cft | Dismantling Cement concrete 1:4:8 (Gen Sch No. 19 (b) P-10) | 1663/75 | % Cft | 6,871/= |
| 4 | 8125.00 Cft | Excavation of pipe line in trenches and pits in all kind of soils i/c trimming and dressing sides to true alignment and shape leveling of beds of trenches to correct level and grade, cutting joints holes and disposal of surplus earth within a one chain as directed by Engineer Incharge providing fence guards lights flags and temporary crossings for non-vehicular traffic where ever required lift upto 5 ft & lead upto one chain (PHED Sch P.No.47/5) | 4650.00 | ‰Cft | 37,781/= |
| 5 | 1000.00 Rft | Providing R.C.C pipes of rubbering joints and fixing in trenches i/c cutting fitting and jointing with rubber ring i/c testing with water to specified pressure (PHED Sch P-18/b(2) (i,ii,iii)) For 12” dia | 367.00 | P Rft | 3,67,000/= |
| 6 | 7313.00 Cft | Refilling the excavated stuff in trenches 6” thick layer i/c watering ramming to full compaction etc complete (PHED Sch P-No. 53/24) | 2760.00 | ‰ Cft | 20,184/= |
| 7 | 33 Nos | Construction of Manhole 4 ft dia 5 ft depth (inner dimension) as per approved design of K.W & S.B including the cost of excavation, 6 inch thick C.C 1:4:8 using stone ballast 2 inch and done gauge 9 inch thick C.C. 1:2:4 plain walls with finishing refilling the excavated stuff around structure curing etc complete as per directed engineer incharge (R.A) | 13707/= | Each | 4,52,331/= |
| 8 | 32.34 Rft | Add or less depth beyond 5 ft depth in manhole etc compete ( R.A ) | 3046/= | P Rft | (-) 98,508/= |
| 9 | 33 Nos | Manufacturing and supplying of precast RCC ring slab of 21” dia inner side and 36” dia outer side 7.5” width and 6” thick i/c 3/8” dia 8 Nos. cross link bars welded and two sunk type hooks casted in 1:1½:3 concrete with embedded 15kg CI frame in perfect position i/c transportation charges of average lead of 20km per trip from casting yard to town offices (a minimum of 25 slabs per trip will be transported)(PHED Sch P-No. 30/2) | 1906.60 | Each | 62,918/= |
| 10 | 33 Nos | Manufacturing and supplying manhole cover cast in 1:2:4 concrete ration 3” deep at center, reinforced with 3/8” dia tor steel bars at 4” C.C welded to 1/8” thick 2.5” deep MS plate i/c curing, stacking and transportation within 10 miles. (W.S. Sch P-27 / 1(a) 21” dia | 913/63 | Each | 30,150/= |

Amount Total (a) Rs. 9,92,923/=

______________ % Above / Below on the rates of CSR.		Amount to be added / deducted on the basis

Of premium quoted                     Total (b)

Total (A) = a+b in words & figures:

Contractor							Executive Engineer / Procuring Agency

(Cont…….Page No. 2)

Work No. 7

N.I.T – 11 Works.

NIT-076/15

Page No. 02

Summary of Bill of Quantities

Cost of Bid 								Amount__________________

________________________

- ( A) Cost based on composite Schedule of Rate

- (B) Cost of based on Non / Offered Schedule of Rates.			__________________________

TOTAL COST OF BID ( C ) = Total (A) + Total (B) 			__________________________

Contractor						Executive Engineer / Procuring Agency