Draft  Bidding Document for works upto 1.00 M.

Work No. 9

N.I.T – 11 Works.

NIT-076/15

NAME OF WORK: Construction of Metalled road at Khan Farm House UC Darsano Channo

BILL OF QUANTITIES

- Description and rate of Items based on Composite Schedule of Rates.

| ### Sr. No | Quantities | Description of item to be executed at site | Rate | Unit | Amount in Rupees |
| --- | --- | --- | --- | --- | --- |
| ### 1 | 2 | 3 | 4 | 5 | 6 |
| 1 | 2626.00 Cft | P/L aggregate base course in proper grade and camber having C.B.R 80 % as per AASHTO standard specification i/c spreading and compacting by approved mechnanical means (motor grader, vibratory roller and smooth wheel roller and etc watering to maintain the moisture content the compaction of each layer shall 100 percent to the max dry density ( rate i/c all cost of materials T & P and carriage upto.(H.W Sch P-5/13/B ) | 5542/24 | %CFT | 1,45,539/= |
| 2 | 10300.00 Sft | P/S prime coat with medium curing asphalt of penetration grade 60/70 prime coats shall be used @ of 11 hires % Sft (1.2 with adequate quantity of kerosene oil using uniformly pressure spray distributor / boiler with automatic temperature controller. Prior to application of the prime coat all used material shall be removed from the base course and the same shall be cleaned y means of approved mechanical sweepers blowers hard brooms and disposal the same within three chains after the application traffic shall be kept of from surface till the laying of bituminous course materials and methodology shall fully comply the tests and specifications AASHTO and as directed by the Engineer Incharge ( R.A ) | 831/76 | % Sft | 85,671/= |
| 3 | 10300.00 Sft | Providing / Laying to proper line and grade 1-1/2” thick pre-mix carpet with paver machine, prepared to specified formula according to job mix formula approved by Engineer incharge including rolling and finishing to proper line, grade level and camber etc (Rate all cost f materials T&P and carriage upto 3 chains (H.Way Sch P-8/20) | 6682/84 | % Sft | 6,88,332/= |

Amount Total (a) Rs. 9,19,542/=

______________ % Above / Below on the rates of CSR.		Amount to be added / deducted on the basis

Of premium quoted                     Total (b)

Total (A) = a+b in words & figures:

Contractor							Executive Engineer / Procuring Agency

(Cont…….Page No. 2)

Work No. 9

N.I.T – 11 Works.

NIT-076/15

Page No. 02

Summary of Bill of Quantities

Cost of Bid 								Amount__________________

________________________

- ( A) Cost based on composite Schedule of Rate

- (B) Cost of based on Non / Offered Schedule of Rates.			__________________________

TOTAL COST OF BID ( C ) = Total (A) + Total (B) 			__________________________

Contractor						Executive Engineer / Procuring Agency