SCHEDULE B.

NAME OF WORK: - S/R to Secretary Flats at C-112, Clifton Karachi (Residence of Mr, Pervaiz Sehar ).

| S.NO. | DESCRAPTION OF ITEMS | QUTY | RATE | UNIT | AMOUNT |
| --- | --- | --- | --- | --- | --- |
| 1 | Providing and laying 1.3.6. cement concrete solid block masonary wall6” and below thickness set in `1.6. cement mortar in ground floor super structure including raking out joints and curing etc complete(s.i.no.24/19) | 4.86 Cft | 15771/01 | %Cft | 766 |
| 2 | Cement plaster 1:4.upto 20’ height ½” thick.(s.i.No.13-b/52) | 29.52 sft | 2206/60 | % sft | 651 |
| 3 | Providing and laying tiles glazed 6” x6”x1/4” on floor or wall facing in required colour and pattern of stile specification jointed in white cement and pigment over a base of 1.2. grey cement mortar ¾” thick inclut and pigement in desired shpae with finishing cleaning and cost of wax polish etc complete including cutting tiles to proper profile.(S.I.NO.60/47) | 35.86 Sft | 30509/77 | %Sft | 10941 |
| 4 | S/F inposition aluminum channels framing for hinged door of alcop made with 5mm thick tinted glass glazing (Belgium ) and alpha (japan) locks i/c handle stoppers etc, complete (b) Deluxe Modle (s.i.No.83-b/108) | 170.0 sft | 1507/66 | P. Sft | 256302 |
| 5 | Scraping ordinary distemper oil bound distemper on walls(S.I.NO.54-b/13) | 281.22 | 226/88 | % Sft | 638 |
| 6 | S/F in position aluminum channel Windows of alcop made with 5mm thick tinted glass glazing Belgium and Alpha (Japan locks i/c handles stoppers etc.(Dulexmodelwhite)(S.I.83b/108). | 25.50 sft | 1647/69 | P. sft | 42016 |
| 7 | Preparing the surface & painting with matt finish paint of approved make to old matt finish surface 2nd each subsequent coat (S.I.36-A+B/55) | 281.22 Sft | 2717/- | %Sft | 7641 |
|  | Total A. |  |  |  | 318955 |
|  | PART B,W/S & S/F |  |  |  |  |
| 1 | P/F 24”x18” lavatory basin in white glazed earthen ware complete with & i/c the cost of WI or CI cantilever brackets 6 inches built into walls painted white in 2 coats after a primary coat of red lead paint a pair of ½”rubber plug & chrome brass waste of approved patten 1-1/4” dia mallable iron of cp brass unons and making requisite number of holes in walls plinth and floor for pipe connection and making good in cement concrete 1.2.4 (standard pattern ) (S.I.8/3). | 1-Nos | 4253/70 | Each | 4254 |
| 2 | S/F wash basin mixture of superior quality with CP head ½”dia (S.I.14-a/19) | 1-No | 2882/- | Each | 2882 |
|  | Total |  |  |  | 7136 |
|  | Add, % above |  |  |  |  |
|  | B. |  |  |  |  |
|  | PART C, N.S.I. |  |  |  |  |
| 1 | P/F wall paper of imported make of approved shade/ colour/paint as approved by the competent authority fixed with gluepreparing smooth the surface with plaster of paris/chemical etc, complete. | 368.42 sft |  | P. sft |  |
|  | Total |  |  |  |  |
|  | ‘G’ Total Rs, |  |  |  |  |
|  | Say, |  |  |  |  |

TERMS AND CONDTION.

Any typographical errors in the schedule B are subject to the correction with

Reference to the Schedule of rate General 2012 enforced from 12.07.2012 as

Approved by the S R C, Sindh Karachi

Arbitration clause stands deleted from the agreement.

No premium shall be paid on a Non schedule items.

100% WELL Graded crushed bajri shall be used in items of work without any

extra payment

No Any items of exvceed then the Quantity Provided in the Schedule B without

Prior of the Executive Engineer.

6.  All sample of items should be got approved by the Executive Engineer.

CONTRACTOR       SUB ENGINEER                                          ASSISTANT ENGINEER

PROVINCIAL BUILDINGS SUB DIVISION-I

KARACHI.