SCHEDULE B.

NAME OF WORK: - S/R to VIP Suit No. 10  at SJC Karachi . ( Residence of  Mr. Riaz Ahmed Saitho Irrigation   Department  ).

| S.NO. | DESCRAPTION OF ITEMS | QUTY | RATE | UNIT | AMOUNT |
| --- | --- | --- | --- | --- | --- |
| 1 | Scraping ordinary distemper oil bound distemper on wals(S.I.NO.54-A/13) | 2557.0 Sft | 226/88 | %Sft | 5801 |
| 2 | Distempering two coats (S.I.NO.54/24-b) | 605.0 Sft | 1043/90 | %Sft | 6316 |
| 3 | Preparing the surface & painting with matt finish paint of approved make to old matt finish surface 2nd each subsequent coat (S.I.37-A+B/61) | 2557.0 Sft | 1772/38 | %Sft | 45310 |
| 4 | P/f in position doors windows and ventilators of First class deodar wood frames and 1-3/4 “ thick commercial ply veneer shutter of 1st class deodar skeleton (hollow) and commercial ply wood (3 ply on both sides) i/c hold fasts,hinges,iron tower bolts, handles and cleats with cord and one mortice lock and hooks(S.I.9/58). | 108.50 | 875/13 | Sft | 133168 |
| 5 | First class deodar wood wrought joinery work in wire gauze door & windows with 22 SWG galvanized wire gauze 144 mesh per sq inch iron fittings complete. 1 ½” thick deodar wood framing in wire gauze with ordinary hinges (S.I.14-b/p-67) | 32.50 Sft | 562/98 | P.sft | 18297 |
| 6 | Painting doors and windows any type any type(S.I.NO.4-C-II/68) | 217.0 Sft | 1160/06 | %Sft | 2517 |
| 7 | Providing and fixing with sunk iron screw wooden Architrave approved design / shape having with not less than 2-1/2 inches as directed by Engineer incharge..(S.I.No.60 P- | 130-Rft | 49/97 | P.Rft | 6496 |
| 8 | Providing and fixing approved quality mortise lock(S.I.,21/60) | 3-Nos | 1786/13 | Each | 5358 |
|  | TOTAL RS. |  |  |  | 223263 |
|  | PART B, |  |  |  |  |
| 1 | Providing and laying tiles glazed 6” x6”x1/4” on floor or wall facing in required colour and pattern of stile specification jointed in white cement and pigment over a base of 1.2. grey cement mortar ¾” thick including washing and filling of joints with slaury of white cement and pigement in desired shpae with finishing cleaning and cost of wax polish etc complete including cutting tiles to proper profile.(S.I.NO.60/47) | 295.0 Sft | 30509/77 | %Sft | 90004 |
|  | Total |  |  |  | 90004 |
|  | Less % Below |  |  |  |  |
|  | B |  |  |  |  |
|  | C- NON SCHEDULE ITEMS |  |  |  |  |
| 1 | Supplying & Fixing Water heater (Geyser 30-gallons) capacity best quality of singer make i/c cartage etc complete. | 1-No |  | Each |  |
| 2 | Making and fixing wooden cabinet with shutters of lasani wood sheet ¾” thick frame work of first class deodar wood 2”x1” pasted with classic Formica and 18” deep including necessary hinges catcher handle sliding wheels in draws nails /screws etc with approved design and shape. The cost also includes necessary tools and plants to be used in making etc complete as directed by the engineer Incharge etc complete | 77.75Sft |  | P.Sft |  |
| 3 | Providing and fixing marble top ¾” thick as approved colour make and design including making edges with round shape and chemical polishing including the cost of material labor Westgate and cartage etc complete | 36.0Sft |  | P.Sft |  |
|  | Total D |  |  |  |  |
|  | G Total |  |  |  |  |
|  | Say Rs |  |  |  |  |

TERMS AND CONDTION.

Any typographical errors in the schedule B are subject to the correction with

Reference to the Schedule of rate General 2012 enforced from 12.07.2012 as

Approved by the S R C, Sindh Karachi

Arbitration clause stands deleted from the agreement.

No premium shall be paid on a Non schedule items.

100% WELL Graded crushed bajri shall be used in items of work without any

extra payment

No Any items of exvceed then the Quantity Provided in the Schedule B without

Prior of the Executive Engineer.

6.  All sample of items should be got approved by the Executive Engineer.

CONTRACTOR       SUB ENGINEER                                          ASSISTANT ENGINEER

PROVINCIAL BUILDINGS SUB DIVISION-I

KARACHI.