| NAME OR WORK :- | M/R TO CHIEF SECRETARY HOUSE KARACHI (ESTABLISHMENT OF WAITING ROOM WITH ATTACH BATH ROOM FOR SECRETARIES AND MISC: REPAIR WORK IN MAIN BUIILDING ) |
| --- | --- |

“ SCHEDULE ‘B’ “

| S.# | DESCRIPTION OF ITEMS | QTY: | RATE | UNIT | AMOUNT |
| --- | --- | --- | --- | --- | --- |
|  | Part ‘A’ Schedule Items (Civil Works ) |  |  |  |  |
| 1 | Excavation in foundation buildings bridges and other structure in/c dabbling dressing refilling around structure with excavated earth watering and ramming lead upto 5ft. (b) In ordinary soil. (SI.18-b/P-4) | 156.00 Cft | 3176/25 | %ocft | 495/- |
|  | Rupees. Three thousand one hundred seventy six. And. Twenty five. Paisa |  |  |  |  |
| 2 | Cement concrete brick or stone ballast 1 ½” to 2” gauge. ( b ).Ratio 1: 4: 8.( SI.4-b / P-15 ). | 123.00 Cft. | 9416/28 | %Cft | 11582/- |
|  | Rupees. Nine thousand four hundred sixteen. And. Twenty eight. Paisa |  |  |  |  |
| 3 | P/L 1:3:6 cement concrete solid block masonry wall above 6” in thickness set in 1:6 cement mortar in ground floor super structure in/c raking out joints & curing etc complete.(SI.22/P-19) | 69.68 Cft | 14268/53 | %Cft. | 9942/- |
|  | Rupees. Fourteen thousand two hundred sixty eight. And. Fifty three. Paisa |  |  |  |  |
| 4 | Providing and laying 1: 3: 6 cement concrete solid block masonry wall 6” and below in thickness set in 1:6 cement mortar in ground floor Super Structure including raking out joints & curring etc, complete.( SI.23 / P-19 ). | 365.82 Cft | 14621/44 | % Cft | 53488/- |
|  | Rupees. Fourteen thousand six hundred twenty one. And. Forty four. Paisa |  |  |  |  |
| 5 | Cement plaster 1:6 upto 12” height I ¾”thick. (SI.13-c/P-52) | 1431.50 Sft | 2590/50 | %Sft. | 37083/- |
|  | Rupees. Two thousand five hundred ninety. And. Fifty. Paisa. |  |  |  |  |
| 6 | P/F in position doors windows and ventilators of 1st class deodar wood frames and 1-1/2” thick teak wood ply shutter of 2nd class deodar wood skeleton (solid) styles and rails core of partal wood and teak ply wood (3 ply) on both sides in/c hold fasts hinges iron tower bolts handles and cleats with cord etc complete. (SI.57/P-65) | 35.00 Sft | 1182/56 | P/Sft | 41390/- |
|  | Rupees. One thousand one hundred eighty two. And. Fifty six. Paisa |  |  |  |  |
| 7 | Painting new surfaces( c ). Preparing surface and painting of doors and Windows any type, ( including edges ). ( i ) Priming coat. ( ii ) Each subsequent coat of paint. ( SI.5c-i / P-70) + ( SI.5c-ii / P-70 ) Rs.862/95 (+) Rs.626/73 = Rs.2116/41 | 109.00 Sft. | 2116/41 | %Sft | 2307/- |
|  | Rupees. Two thousand one hundred sixteen., and. Forty one. Paisa |  |  |  |  |
| 8 | Preparing the surface and painting with matt finish in/c rubbing the surface with Bathy (Silicon carbide rubbing brick) filling the voids with zink / chalk / plaster of Paris mixture, applying first coat premix, making the surface smooth and then painting 3 coats with matt finish of approved mae et complete (New surface ) (B) 2nd & subsequent coat. (SI.36-A+B)/P-55) (three coats) | 1206.50 Sft | 2717/- | %Sft. | 32781/- |
|  | Rupees. Two thousand seven hundred seventeen. Only. |  |  |  |  |
| 9 | Preparing the surface and painting with matt finish paint of approved make to old matt finish ( b ). 2nd & subsequent coat. ( SI.37(A+B) / P-55 ). | 4125.00 Sft | 1772/38 | %Sft | 73111/- |
|  | Rupees. One thousand seven hundred seventy two. And. Thirty eight. Paisa |  |  |  |  |
| 10 | Providing and fixing G.I frames /Chowkats of size 7” x 2” or 4 1/2” x 3” for door using 20 gauge G.I sheet in/c welded hinges and fixing at site with necessary hold fasts, filling with cement sand slurry of ratio 1:6 and repairing the jambs. The cost also in/c all carriage , tools and plants used in making and fixing. (29-93). | 16.00-Rft | 228/90 | PRft | 3662/- |
|  | Rupees. Two hundred twenty eight. And. Ninety. Paisa |  |  |  |  |
| 11 | Providing and fixing with sunk iron screws wooden Architrave approved design/ shape having width not less than 2- 1/2 inches as directed by Engineer Incharge. ( SI.60/P-65 ) | 33.00 Rft | 49/97 | P.Rft | 1649/- |
|  | Rupees. Forty nine. And. Ninety seven. Paisa |  |  |  |  |
| 12 | Providing and fixing Aluminum sheet on doors pasted with glue as per requirement.( SI.61 / P-66 ). | 22.00 Sft. | 63/77 | P.Sft | 1403/- |
|  | Rupees. Sixty three. And. Seventy seven. Paisa |  |  |  |  |
| 13 | nyloon wire gauge 144 mesh per square inch fixed to chowkats with 3/4” deodar strips and screws(SI63-A/P-66) | 373.00-Sft | 193/53 | P/Sft. | 72202/- |
|  | Rupees. One hundred ninety three. And. Fifty three. Paisa |  |  |  |  |
| 14 | P/F GI expended metal 1/8 – 20 (20 gauge) foxed to chowkats (SI.66-B/P-66) | 373.00-Sft | 68/13 | P/Sft. | 25418/- |
|  | Rupees. Sixty eight. And. Thirteen. Paisa |  |  |  |  |
| 15 | Providing and laying 1” thick topping cement concrete ( 1:2:4 ) including Surface finishing and dividing into panels: ( c ) 2” thick.( SI.16 ( c ) / P-42 ). | 168.00 Sft | 3275/50 | %Sft | 5503/- |
|  | Rupees. Three thousand two hundred seventy five. And. Fifty. Paisa |  |  |  |  |
| 16 | Fabrication of mild steel reinforcement for cement concrete including cutting, bending, laying in position, making joints and fastenings including cost of binding wire ( also includes removal of rust.( b ) Using Tor bars. ( SI.8(b) /P-17 ) | 04.63 Cwt | 5001/70 | P/Cwt | 23158/- |
|  | Rupees. Five thousand & one. And. Seventy. Paisa |  |  |  |  |
| 17 | Reinforced cement concrete work including all labour and material except the cost of steel reinforcement and its labour for bending and binding which will be paid separately. This rate also includes all kinds for forms moulds: lifting shuttering curing rendering and finishing the exposed surface ( including screening and washing of shingle. ) ( a ) R.C work in roof slab, beams columns rafts, lintels and other structural members laid in situ or precast laid in position complete in all respects. ( I ) Ratio ( 1: 2: 4 ) 90 Lbs. cement 2 Cft sand 4 Cft shingle 1/8” to ¼” gauge. ( SI.6(a)(I) / P-16 ). | 118.84 Cft | 337/- | P/Cft | 40049/- |
|  | Rupees. Three hundred thirty seven. Only. |  |  |  |  |
| 18 | Preparing the surface and painting with weather coat of approved make to old weather coat surface (b) 2nd & subsequent coat (39-A+B P/56). | 6818.25 | 1498/48 | %Sft | 102170/- |
|  | Rupees. One thousand four hundred ninety eight. And. Forty. Paisa |  |  |  |  |
| 19 | P/F cement paving blocks flooring having size 197x97x60 (mm) of city / quddra / cobble shape with pigmented having strength b/w 5000 psi to 8500 psi in/c filling the joints with hill sand and laying in specified manner / pattern and design etc complete. (SI-72/P-49) | 190.00-Sft | 223/97 | P/Sft. | 42554/- |
|  | Rupees. Two hundred twenty three. And. Ninety seven. Paisa |  |  |  |  |
| 20 | P/F cement paving blocks flooring having size 197x97x60 (mm) of city / quddra / cobble shape with natural colour having strength b/w 5000 psi to 8500 psi in/c filling the joints with hill sand and laying in specified manner / pattern and design etc complete. (SI-71/P-49) | 1567.50-Sft | 199/77 | P/Sft | 313139/- |
|  | Rupees. One hundred ninety nine. And. Seventy seven. Paisa |  |  |  |  |
|  |  | Total Rs. |  |  | 893086 |
|  |  | ( A ) Total Rs. |  |  |  |
|  | Part ‘B’ W/S & S/F Schedule Items. |  |  |  |  |
| 1 | Providing & fixing 6”x2” or 6”x3” C.I floor trap of the approved salt cleaning design with a C.I screwed down granting with or without a vent arm complete with & I/c making requisite number of holes in walls, plinth & floor for Pipe connections & making good cement concrete 1: 2: 4. ( SI.20 / P-06 ). | 03 Nos | 2042/43 | Each | 6127/- |
|  | Rupees. Two thousand forty two. And. Forty three. Paisa |  |  |  |  |
| 2 | S/F CP Muslim shower with double bib cock & ring pipe etc complete. (SI.19-b/P-19) | 02 Nos | 3432/- | Each | 6864/- |
|  | Rupees. Three thousand four hundred thirty two. Only. |  |  |  |  |
| 3 | Supplying & fixing Bath room accessories set ( 7-piece ) in/c towel rod, brush holder soaptray shelf of approved design in/c cost of screws, nuts etc complete. ( Master Brand ). ( SI.23 / P-19 ). | 01 Set | 10322/40 | Each | 10322/- |
|  | Rupees. Ten thousand three hundred twenty two. And. Forty paisa |  |  |  |  |
|  |  | Total Rs. |  |  | 23313/- |
|  |  | (B)Total Rs. |  |  |  |
|  | Part ‘C’ Electric Schedule Items. |  |  |  |  |
| 1 | P/L ( Main or sub main ) PVC insulated with size 2-7 / .029 copper conductor in ¾” dia PVC conduit recessed in the wall or column as required ( SI.10 / P-2 ). | 23.47 Meter | 222/- | P. Meter | 5210/- |
|  | Rupees. Two hundred twenty two. Only. |  |  |  |  |
| 2 | Wiring for light or fan point with 3/.029 PVC insulated wire in 20mm ( ¾” ) PVC conduit recessed in the wall or column as required.( SI.124 / P-15 ). | 12 Point | 1130/- | P.Point | 13560/- |
|  | Rupees. One thousand one hundred thirty. Only. |  |  |  |  |
| 3 | Wiring for plug point with 3/.029 PVC insulated wire in 20mm ( ¾” ) PVC conduit recessed in the wall or column as required.( SI.126 / P-15 ). | 03 Nos | 985/- | P.Point | 2955/- |
|  | Rupees. Nine hundred eighty five. Only. |  |  |  |  |
| 4 | Providing & fixing circuit breaker 6,10,15,20, 30,40,50 & 63amp SP (TB-5S) on prepared board as required. (203 P / 31). | 03-Nos | 916/- | P/ No | 2748/- |
|  | Rupees. Nine hundred sixteen. Only. |  |  |  |  |
| 5 | Providing & fixing Bakelite ceiling rose with two terminals. ( SI.228 / P-33 ). | 03 Nos | 72/- | P/No. | 216/- |
|  | Rupees. Seventy two. Only. |  |  |  |  |
|  |  | ( C ) Total Rs. |  |  | 24689/- |
|  | PART ‘D’ NON SCHEDULE ITEMS. |  |  |  |  |
| 1 | P/L precast slab 3’x2’ as per required size in/c filling the joints with cement mortar ratio 1:2 in/c finishing, curing etc complete as directed by the Engineer Incharge, the rate in/c all cost of material, labour & cartage etc complete | 80.00 Sft |  | P/Sft |  |
| 2 | P/L precast beam size 4”x9” as per required size as directed by Engineer Incharge, the rates includes all cost of material, labour, cartage etc complete | 36.00 Rft |  | P/Rft |  |
| 3 | Removing of dismantled material from roof and other places in/c stacking at one place by wheel barrow & then loading on truck and disposing out side KMC premises. The cost inc hire charges of truck / dumper as directed by the Engineer Incharge. | 01 Truck |  | P/Truck |  |
| 4 | P/F false ceiling of Gypsum board in panels of required design & size in/c frame work of aluminum T-Section hanged with nail wire to ceiling etc complete. | 168.00 Sft |  | P/Sft |  |
| 5 | S/F full porcelain laser cut tiles of size (18’x13’)or as approved size by the competent Authority or Consultant fully imported made in Italy laid with S-tile bond of approved make & quality in/c filling the joints with white cement or tile grout in desired shape in/c finishing after cutting tiles to proper profile in floor or dado in/c all cost of material, labour, cartage & wastage as directed by the Consultant Incharge. | 311.50 Sft. |  | P/Sft |  |
| 6 | P/F Artificial marble as per approved color & shade with chemical polish in/c making edges with half round shape in/c cutting edge shape in front and down 6” & 3” centre to centre jointing with solution / jelly with marble pieces & angle iron frame in side for support and embedded 1” in wall after cutting & chiseling carefully as directed by the Engineer Incharge in/c all cost of labor material & cartage etc complete. | 17.75 Sft. |  | PSft |  |
| 7 | P/F floor trap jail (PVC type) size 6”x6” of approved color and design (Master A Class) as desired by the Engineer Incharge. | 03-Nos |  | Each |  |
| 8 | Providing and fixing looking mirror (Imported ) in/c. cutting in proper size making edges carriage from shop to site fixing with silicon and clips in wall as directed by the Engineer Incharge. | 10.50-Sft |  | PSft |  |
| 9 | S/F fancy type fine quality imported make wall mirror light with shaded glass cover as approved design & shape in/c assembling and necessary electric wiring & connection etc complete. | 01-No. |  | Each |  |
| 10 | P/F fancy type spot light 8” dia with glass and 02-Nos holder (fully imported) of approved quality & make in/c 02-Nos energy saver of Philips or equivalent make in/c fixing in false ceiling by cutting hole as directed by the Engineer Incharge. | 10-Nos |  | Each |  |
| 11 | S/F fancy switch / socket imported make with double sheet fixing with existing concealed board on approved shape and design by the Engineer Incharge etc complete. | 12-Nos |  | Each |  |
| 12 | P/F fine quality Exhaust fan royal make or equal quality 6” sweep with glass cover plastic body etc in/c making frame after making hole in/c plaster work with edges finishing and necessary electric work wiring & connection etc complete. | 01-No |  | Each |  |
| 13 | P/F bath room sanitary fittings / Fixture imported “GROHE” make lever type high quality sink mixture as directed by the consultant with the approval of competent authority of Chief Secretary House Karachi in/c all cost of labour, material & cartage etc complete. | 01-No |  | Each |  |
| 14 | P/F bath room sanitary fittings / Fixture imported “GROHE” make lever type high quality basin mixture as directed by the consultant with the approval of competent authority of Chief Secretary House Karachi in/c all cost of labour, material & cartage etc complete. | 01-No |  | Each |  |
| 15 | P/F bath room sanitary fittings / fixture of imported “GROHE” make lever type high quality shower mixture as directed by the consultant with the approval of competent authority of Chief Secretary House Karachi in/c all cost of labour, material & cartage etc complete | 01-No |  | Each |  |
| 16 | P/F PVC bottle trap fine quality with necessary PVC fittings for wash basin and kitchen sink as directed by the consultant with the approval of competent authority of Chief Secretary House Karachi in/c all cost of labour material & cartage etc complete. | 01-No |  | Each |  |
| 17 | S/F fancy type CP or powder coated concealed stop cock of approved quality & make as directed by the Engineer Incharge. | 03-Nos |  | Each |  |
| 18 | French polishing of doors, windows, ventilators, stair Jeffery, floor work etc includes preparing surface and polishing by using Indian Lakh Dana, spirit, thinner, pigment ICI Lacquer, ICI hardener, Regmal (Germany) material ICI sealer, tarpine oil, polishing as per satisfaction of competent authority etc complete. | 2293.00-Sft |  | PSft |  |
| 19 | P/F bath room European commode set imported “MIRACHI” make style # 3597 in one piece with same make fancy heavy pattern seat cover & having colour, design shape as directed by consultant with the approval of competent authority of Chief Secretary House in/c making hole in walls, plinth or floor & making good in c.c. 1:2:4 in/c carriage charges from show room to site of work etc complete job. | 01 Set |  | P/Set |  |
| 20 | P/F bath room fittings / fixture, wash vanity cabinet type imported make 23”/36” long with colour, design & shape & make as directed by the consultant with the approval of competent authority of Chief Secretary House in/c making hole in wall, plinth or floor & making good in c.c. 1;2:4 in/c carriage charges from show room to site of work etc complete. | 01 No |  | Each. |  |
| 21 | Refilling of fire extinguisher cylinder after removing of expired gas and cleaning in side cylinder in company work shop then filling and checking etc complete the job in/c all cost of labour material & up down cartage etc complete | 10 Nos |  | Each. |  |
| 22 | Supply 30 gallons Geyser Singer company to approved quality gauge selector switch, thermostat in all respect fitted with complete UPVC fitting & pipe installed on cement concrete platform to required size & height with 1;3;6 finishing curing etc complete. | 01 No. |  | Each. |  |
| 23 | P/M graffito paint to wall surface to provide durable & aesthetic surface having thickness between 1:6-2.0mm in specified color / shade & working upto height of 30.0 using scaffolding as directed by the Engineer Incharge. | 932.50-Sft |  | P/Sft. |  |
| 24 | P/L Gadap / Gizri stone tiles upto 1-1/2” thick as required size in/c tucking existing plastered surface, applying floating coat of cement & then laying stone tiles on wall facing jointed in grey cement with pigment laid over 1:2 cement sand mortar ¾” thick etc complete as directed by the Engineer Incharge. | 144.00 Sft. |  | P/Sft. |  |
|  |  | G. Total Part (D) Total Rs. |  |  |  |
|  |  | G. Total Part (C) Total Rs. |  |  |  |
|  |  | G. Total Part (B) total Rs. |  |  |  |
|  |  | G. Total Part (A) Total Rs. |  |  |  |
|  | G. Total Part (A+B+C+D) Total Rs. |  |  |  |  |
|  |  | SAY Rs. |  |  |  |

CONDITION.

- Typographical errors in the schedule “B” are the subject to the correction with ref: to the schedule of rate Gen: vol: III Part II in forced: 2012 as approved by the standing rate committee.

- No cartage will be paid on any items of works.

- No premium shall be paid on Non-Schedule items.

- Arbitrative clause stand deleted from agreement.

- Water shall be arranged by the Contractor.

| CONTRACTOR | ASSISTANT ENGINEER PROVINCIAL BUILDINGS SUB- DIVISION-V, KARACHI. | EXECUTIVE ENGINEER PROVINCIAL BUILDINGS DIVISION-I, KARACHI. |
| --- | --- | --- |