Name of work:  M/R TO CHIEF MINISTER HOUSE KARACHI (A.S OFFICES,DS OFFICES AND PROGRAMME

SECTION ETC)

SCHEDULE B

| S.No. | Description | Qty | Rate | Unit | Amount |
| --- | --- | --- | --- | --- | --- |
| 1 | Dismentalling from any height asbestors sheet and ridge copping (S.I.23/11) | 1422.00 Cft | 272/25 | %Cft | 3871 |
| 2 | Dismentalling of roof of wooden planks and battens from any height.(S.I.24-a/11) | 1422.00 Sft | 157/- | %Sft | 2233 |
| 3 | Dismentalling Cement Block masonary (S.I.24/10) | 202.50 Cft | 1134/38 | %Cft | 2297 |
| 4 | Dismentalling glazed or encaustic tiles (S.I.55/13) | 1422.00 Sft | 786/50 | %Sft | 11184 |
| 5 | Excavation in foundation of buildings bridges & other structure i/c dag belling dressing refilling around the structure with excavated earth watering & ramming lead upto 5 ft (S.I.18c/4) | 4107.24 Cft | 3176/25 | %0cft | 13046 |
| 6 | Cement Concrete brick or stone ballast 1-1/2”to 2” guage ratio 1.4.8(S.I.14b/15) | 1651.24 Cft | 9416/28 | %CFt | 155485 |
| 7 | RCC work i/c all labour & material except the cost of steel reinforcement and its labour for bending & binding which will be paid separately this rate also i/c all kinds of forms moulds lifting shuttering curing & rendering and finishing the exposed surface i/c screening and washing of shingle ratio 1.3.6 RCC work in roof slab beams col rafts lintels & other structural members laid situ or precast laid in position complete ratio 1.2.4 90lbs cement 2 cft sand 4cft shingle 1/8”to ¼”guage ratio (S.I.6-A(i)/17) | 2576.80 Cft | 337/- | Cft | 868412 |
| 8 | Cement plaster 1.4 upto 20’ft height ½”thick (S.I.11-b/52) | 8135.00 Sft | 2283/93 | %Sft | 185798 |
| 9 | Cement Plaster 1.4 upto 20’ft height 3/8”thick (S.I.11-a/52) | 8135.00 | 2197/52 | %sft | 178768 |
| 10 | P/L c.c. block masonary 1:3:6 cement concrete solid block masonary . Wall 6” and below in thickness set in 1:6. Cement mortor in foundation super structure I/c raking out joints & curring etc, complete.(s.i.No.22/19). | 698.24 Cft. | 14268/53 | % Cft. | 99629 |
| 11 | P/L c.c. block masonary 1:3:6 cement concrete solid block masonary set in 1.6 cement & mortor plinth & foundation i/c racking out joints & curing etc complete. .(s.i.No.24/19). | 1963.50 Cft. | 15771/01 | % Cft. | 309664 |
| 12 | Distempering three coats (S.I.24-c/54) | 1819.25 Sft | 1079/65 | %Sft | 19642 |
| 13 | Preparing the surface & Painting with matt finish i/c rubbing the surface with bathy silicon carbide rubbing brick filling the joints with zink chalk plaster of paris mixture applying first coat premix making the surface smooth and then painting 3 coats with matt finish of approved make etc complete.(new surface) (S.I.35/55) | 6424.50 | 1989/62 | %sft | 127823 |
| 14 | Preparing the surface & Painting with weather coat i/c rubbing the surface with rubbing brick / sand paper filling the voids with chalk plaster of paris and then painting with weather coat of approved make 2nd & each subsequent coat. (S.I.38=A+B+B/56). | 4284.00 Sft | 2567/95 | %Sft | 110011 |
| 15 | P/F in position doors & windows & ventilators of first class deodar wood frame & 1-3/4”thick commercial ply venner shutter of first class deodar skeleton hollow and commercial ply wood (3ply)on both sides.(S.I.9/58) | 304.50 | 1227/36 | Sft | 373731 |
| 16 | First Class deodar wood wrought joinery in doors and windows etc fixed in position i/c chowkats hold fasts hinges iron tower bolts chocks cleats handles and cords with hooks etc(Deodar paneled or paneled and glazed or fully glazed 1-3/4”thick (S.I.7b/58) | 217.00 | 1273/76 | Sft | 276406 |
| 17 | P/F with sunk Iron screwes wooden archtrative approved design shape having with not less then 2-1/2 inches as directed by Engineer Incharge.(S.I.60/66) | 402.00 | 49/97 | Sft | 20088 |
| 18 | S/F Windows printed blinds horizontal / vertical with plain design and of approved colour i/c fixing in windows with necessary accessories.(S.I.73/67) | 309.50 | 91/11 | Sft | 28199 |
| 19 | S/F False Ceiling of plaster of paris in panels of i/c making frame work of deodar wood i/c painting with solignum paint.(S.I.52/64). | 1215.00 | 25293/42 | %Sft | 307315 |
| 20 | P/F approved quality mortice lock | 2-Nos | 1786/13 | Each | 3572 |
|  | Total |  |  |  | 3097174 |
|  | Less % below Item No. 15 & 16 |  |  |  |  |
|  | Total A |  |  |  |  |
|  | Part A-I S.Item Steel |  |  |  |  |
| 1 | Fabrication of mild steel reinforcement for cc i/c cutting bending laying in position making joints and fastening i/c cost of binding wire also i/c removal of rust from bars (S.I.8/17) | 132.94 Cwt | 5001/70 | Cwt | 664926 |
|  | Total |  |  |  | 664926 |
|  | Add % above |  |  |  |  |
|  | Total |  |  |  |  |
|  | Part B W/S & S/F |  |  |  |  |
| 1 | P/F 22x16 lavatory basin in white glazed earthen ware complete with and i/c the cost of WI or CI ventilator brackets 6”built into walls pointed white in two coats after a primary coat of red head of paint a pair of 1-1/2 dia rubber plug and chrome plated brass chain 1-1/4 dia maloable iron or brass unions and making requisite number of holes in walls plinth and floor for pipe connection and making good in cement mortor 1.2.4 Foreig or equilent)(S.I.13/4) | 6-Nos | 6237/- | Each | 37422 |
| 2 | Add extra for labour for providing and laying of earthen ware pedestal white or coloured glazed foreign or equilent.(S.I 11/3) | 6-Nos | 2533/47 | Each | 15201 |
| 3 | S/F CP Muslim Shower with double bib cock & Ring Pipe etc complete.(S.I.19-b/19) | 6-Nos | 3432/- | Each | 20592 |
| 4 | S/F Bath Room Accessories (7-pices) i/c towel rod brush holder soaping shelf of approved design i/c cost of screwes nuts etc complete.(Master brand)(S.I.23/19) | 6-Nos | 10322/40 | Each | 61934 |
| 5 | P/F 24”x18” beveled edge mirror of Belgium glass complete with 1/8”thick hard board and cp screwes fixed to wooden cleats (b) Superior quality.(S.I.3-b/7) | 6-Nos | 2451/- | Each | 14706 |
| 6 | S/F Fiber Glass Tank of approved make and design and wall thickness as specified i/c cost of nuts bolts and fixing in plate form of cement concrete 1.3.6 and make in connection for in let & out let & over flow pipes etc complete.(S.I.3-d/21)(1000 Gallons) | 1-No | 58962/94 | Each | 58963 |
| 7 | Providing chamber 15x9 (inside dimension)x24”deep for house matters with 4-1/2 thick burnt brick masonary walls in 1.6 CM 6”thick CC 1.4.8 in foundation ½”thick cement plaster in 1.3 CM to all inside walls surface and to top 1”thick CC 1.2.4 flooring complete with cost iron hinged cover and frame 15x9 (inside) clear opening (wr Qr) etc fixed in cement concrete 1.2.4 i/c curring excavation back filling and disposal of earth etc complete.(S.I.4/20) | 3-Nos | 4905/67 | Each | 14717 |
|  | Total |  |  |  | 223935 |
|  | Add 5% above |  |  |  |  |
|  | B |  |  |  |  |
|  | Part C Public Health S.Items |  |  |  |  |
| 1 | Construction of man hole or inspection chamber for the diameter of circular sewer and 3-6 (1067mm)depth with walls of BB in cement sand mortor 1.3 cement plastered 1.3 ½”thick inside of walls and 1”(25mm ) thick over benching and channel i/c fixing CI man hole cover with frame of clear opening 1-1/2”x1-1/2”(457x457mm) of 1.75 cwt (88.9kg) embedded in plain cc 1.2.4 and fixing 1” (25mm dia) MS steps 6” 150mm wide projecting 4”102 mm for the face of wall at 12”(305mm)CC duly painted etc complete as per standard specification and drawings(S.I. P/46) | 4-Nos | 14748/- | Each | 58992 |
|  | Total |  |  |  | 58992 |
|  | Add % above |  |  |  |  |
|  | Total C |  |  |  |  |
|  | Part D Electric S.Items |  |  |  |  |
| 1 | Wiring for light or fan point with 3/029 PVC insulated wire in 20mm ¾”PVC conduit recessed in the wall or col as required.(S.I.124/15) | 124-Nos | 1130/- | Each | 140120 |
| 2 | Wiring for Plug point with 3/029 PVC insulated wire in 20mm ¾”PVC conduit recessed in the wall or col as required.(S.I.126/15) | 26-Nos | 985/- | Each | 25610 |
| 3 | Wiring for call bell point with 3/029 PVC insulated wire in 20mm ¾”PVC conduit recessed in the wall or col as required.(S.I.127/15) | 7-Nos | 1380/- | “ | 9660 |
| 4 | P/L main or Sub main PVC insulated with size 2-7/036 (4mm2) copper conductor in ¾ ” dia PVC conduit on surface.(S.I.4/1) | 138.43 Meter | 200/- | p.Meter | 27686 |
| 5 | P/L main or Sub main PVC insulated with size 2-7/044 (6mm2) copper conductor in 1” dia PVC conduit on surface.(S.I.5/1) | 243.24 Meter | 305/- | P.Meter | 74188 |
| 6 | P/F Brass Batten Holder (S.I 232/33) | 18-Nos | 70/- | “ | 1260 |
|  | Total (D) |  |  |  | 278524 |
|  | Part E N.S.I |  |  |  |  |
| 1 | P/F Bath Room tiles 18”x12” ‘laid on ¾” over cement sand mortor ratio 1.2 i/c filling the joints with white cement or tiles grout in desired shape i/c finishing and cutting tiles to proper profile i/c all cost of material labour & cartage etc complete. | 976.15 |  | Sft |  |
| 2 | P/F approved quality Porceilien floor tiles (Imported quality) size upto 20’’x20 ‘laid over cement chemical bond i/c filling the joints with white cement or tiles grout in desired shape i/c finishing and cutting tiles to proper profile for floor or dado i/c all cost of material labour & cartage etc complete. | 1795.85 |  | Sft |  |
| 3 | P/F UPVC Pipe of AGM Pak Arab or equilent quality make sch 80 © water supply for roof and water and joints with AGM solution and making hole in walls plinth and floor for pipe connection etc complete as directed by the Engineer Incharge.4” dia. | 87.00 |  | Rft |  |
| B | ---do---3” dia | 90.00 |  | Rft |  |
| C | ---do---6”dia | 115.00 Rft |  | Rft |  |
| 4 | P/F UPVC Sanitary fitting make Pak or equilent quality jointing with AGM solution as directed by the Engineer Incharge. |  |  |  |  |
| A | 3”dia floor trape | 12-Nos |  | Each |  |
| B | 3”dia tee | 12-Nos |  | “ |  |
| C | 3”dia Y-tee | 6-Nos |  | “ |  |
| D | 3”dia elbow | 12-Nos |  | “ |  |
| E | 3”dia Plug tee | 3-Nos |  | “ |  |
| F | 4”dia elbow bend | 12-Nos |  | “ |  |
| G | 4”dia Cowel | 3-Nos |  | “ |  |
| H | 4”dia Plug tee | 3-Nos |  | “ |  |
| I | Floor trape SS | 12-Nos |  | “ |  |
| 5 | P/F CPVC Pipe of AGM Pak Arab or equilent quality make sch 80 © water supply for roof and water and joints with AGM solution and making hole in walls plinth and floor for pipe connection etc complete as directed by the Engineer Incharge.1/2 dia. | 180-Rft |  | Rft |  |
| B | ---do---CPVC Pipe 1”dia | 100-Rft |  | Rft |  |
| 6 | P/F CP Tee Cock ½”dia pattern as directed by Engineer Incharge. | 18-Nos |  | Each |  |
| 7 | P/F CP Coated double bib cock ½”dia as approved directed by Engineer Incharge. | 6-No |  | Each |  |
| 8 | P/F European Commode Couple set master make use of equilent quality earthen ware with flush tank seal cover approved design shape i/c making holes good in cc 1.2.4 i/c carriage from shower room to site as directed by the Engineer Incharge. | 6-No |  | each |  |
| 9 | P/F cp or coated basin mixture (master make) or equilent quality shape & design as directed by the Engineer Incharge. | 6-No |  | Each |  |
| 10 | P/F handle valve ½” dia valve jointing with water supply line make (Baven) or equilent quality as directed by the Engineer Incharge. | 12-Nos |  | Each |  |
| 11 | P/F Water heater geyser 30 gallons make singer or equilent quality i/c necessary GI fitting etc complete. | 1-No |  | “ |  |
| 12 | Removal of Debries material Site of work to Out of KMC Limit as directed by the Engineer Incharge etc complete. | 2-Truck |  | P.Truck |  |
| 13 | P/F Tube Light double 1-40 fancy shade i/c chowk starter i/c necessary electric connection etc complete. | 8-Nos |  | Each |  |
| 14 | P/F Wall Brcaket Fan 18” dia make Pak or equelent quality necessary election etc complete. | 14-Nos |  | “ |  |
| 15 | P/F Exhaust Fan Plastic Body 10”dia equelent quality i/c necessary electric connection as per approved quality etc complete. | 3-Nos |  | “ |  |
| 16 | P/F Ceiling Fan 56” Sweep make Pak or equelent quality i/c necessary electric connection etc complete. | 3-Nos |  | “ |  |
| 17 | P/F fancy ceiling ligfht i/c Saver Bulb & shade approved quality & design i/c holder necessary electric connection etc complete. | 70-Nos |  | “ |  |
| 18 | P/F Fancy Switch Socket with board as per approved quality by the Engineer Incharge etc complete.(6-Guage) | 16-Nos |  | “ |  |
| 19 | P/F Fancy Switch Socket with board as per approved quality by the Engineer Incharge etc complete.(3-Guage) | 6-Nos |  | “ |  |
| 20 | P/F Fancy Switch Socket with board as per approved quality by the Engineer Incharge etc complete.(4-Guage) | 1--Nos |  | “ |  |
| 21 | P/F Universal 5 amps with PVC board i/c necessary electric connection etc complete. | 14-Nos |  | Each |  |
| 22 | P/F Power Plug 15 amps with board etc complete. | 7-Nos |  | “ |  |
|  | Total E |  |  |  |  |
|  | Grand Total A+B+C+D+E |  |  |  |  |
|  | Say Rs |  |  |  |  |

TERMS & CONDITION

- Any typographical errors in the schedule B are subject to the correct condition with Reference to the schedule of rate General 2012 enforced from 12-07-2012 as approved by the SRC Sindh.

- Arbitraction clause stands deleted from the agreement.

- No Premium shall be paid on Non Schedule Items

- 100% well Graded crushed bajri shall be used in items of work without any extra payment.

- No any items of exveceed then the quantity provided in the schedule B without prior of the Executive Engineer.

- All sample of items should be got approved by the Executive Engineer.

Contractor

Assistant Engineer,

Provincial Buildings sub Division-IX,

Karachi