NAME OF WORK  CONSTRUCTION OF OFFICE HOSTEL RESIDENTIAL BUILDINGS FOR POPULATION WELFARE TRAINING INSTITUTE (P.W.T.I)SCHEME 33 NEAR

BANDU KHAN HOTEL KARACHI SINDH REMAINING WORK (ADP No.2042)

(2014-15)

SCHEDULE “ B “

MAIN BUILDING

BASEMENT

| S.NO. | DESCRAPTION OF ITEMS | QUTY | RATE | UNIT | AMOUNT |
| --- | --- | --- | --- | --- | --- |
| 1 | P/l 2” thick topping c.c. 1:2:4 i/c surface finishing & dividing into panels(S.I.16-d/42) | 5135.77 | 3275/50 | %sft | 168222 |
|  |  | Rupees Three Thousand Two Hundred Seventy Five and fifty Paisa Only |  |  |  |
| 2 | Cement plaster 1.6 upto 12’ft height ½ ”thick (S.I.13-b/52). | 11079.03 | 2206/60 | %Sft | 244470 |
|  |  | Rupees Two Thousand Two Hundred Six and Sixty Paisa Only |  |  |  |
| 3 | Cement plaster 1.4 upto 12’ft height 3/8 ”thick (S.I.11-a/52). | 11079.03 | 2197/52 | %Sft | 243464 |
|  |  | Rupees Two Thousand One Hundred Ninty Seven and Fifty Two Only |  |  |  |
| 4 | S/f and fixing in position aluminium channel framing for sliding windows & ventilators alcop made with 5 mm thick tinted glass glazing (Belgium)& aluminium glass glazing (Belgium) & Aluminium fly Screen i/c handles stoppers & locking Arrangement etc complete(DeluxeModelWhite)(S.I.NO.84-A/108) | 200.00 Sft | 1592/69 | P.Sft | 318538 |
|  |  | Rupees Fifteen Hundred Ninty Two and sixty Nine Paisa Only |  |  |  |
| 5 | Distempering three coats (S.I.NO.54/24-c) | 9303.00 | 1079/65 | % Sft | 100440 |
|  |  | Rupees One Thousand Seventy Nine and sixty Five Paisa |  |  |  |
| 6 | Preparing the surface & Painting with weather coat of approved make to new weather coat surface (2nd & each subsequent coat).(S.I.38A+38B+38B/56) | 1776.00 | 2567/95 | %Sft | 45607 |
|  |  | Rupees Two Thousand Five Hundred sixty Seven and ninty Five Only |  |  |  |
|  | TOTAL |  |  |  | 1120741 |
|  | Above / Below Add or Less |  |  |  |  |
|  | Total |  |  |  |  |
|  | G.FLOOR |  |  |  |  |
| 1 | RCC work i/c all labour & material except the cost of steel reinf.for its labour of bending & binding which will be paid separately this rate also i/c all kinds of forms moulds lifting shuttering curring rendering & finishing the exposed surface (i/c screening & washing of shingle)(a)RCC work in roof slab beams col rafts lintels and other structure members laid in situ or precast laid in position complete in all respect ratio 1.2.4 90 lbs cement 2cft sand 4cft shingle 1/8”to ¼”guage.(S.I.6A(i)/17) | 4715.17 | 337/- | P.Cft | 1589012 |
|  |  | Rupees Three Hundred Thirty Seven Only |  |  |  |
| 1b | (ii) ratio 1-1/2:3 | 3280.30 | 349/- | P.Cft | 1144825 |
|  |  | Rupees Three Hundred Forty Nine Only |  |  |  |
|  | Total At Par |  |  |  | 2733837 |
| 2 | Fabrication of mild steel reinforcement for cc i/c cutting bending laying in position making joints & fastening i/c cost of binding wire also i/c removal of rust from bars )Using of Tor Bars (S.I.8(b)/17) | 545.96 Cwt | 5001/70 | P.Cwt | 2730728 |
|  |  | Rupees Five Thousand One and Seventy Paisa Only |  |  |  |
|  | TOTAL |  |  |  | 2730728 |
|  | ADD OR LESS |  |  |  |  |
|  | TOTAL |  |  |  |  |
| 3 | Providing and laying 1.3.6. cement concrete solid block masonary wall6” and below thickness set in `1.6. cement mortar in ground floor super structure including raking out joints and curing etc complete(s.i.no.22/19) | 2060.02 | 14621/44 | %Cft | 301205 |
|  |  | Rupees Fourteen Thousand Six Hundred Twenty One and Forty Four Only |  |  |  |
| 4 | Providing and fixing G.I. frames/chowkats fo size 7”x2” or 4 ½ x 3” for doors using 20 gauge G.I. sheet i/c welded hinges and fixing at site with necessary hold fasts filling with cement sand slurry of ratio 1.6. and repairing the jambs. The cost also in/c all carriage tools and plants used in making and fixing(S.I.NO.28/93) | 416.50 | 228/90 | Rft | 95337 |
|  |  | Rupees Two Hundred Twenty Eight and Ninty Paisa Only |  |  |  |
| 5 | First class deodar wood wrought joinery in doors and windows etc fixed in position including chow Kats hold fasts hinges, iron tower bolts chocks cleats handles and cords with hooks etc deodar paneled or paneled and glazed or fully glazed (b)1-3/4” thick(S.I.NO.7-b/58).(1273/76(-)370/83=902/93 (Only Shutter). | 577.50 | 902/93 | P.Sft | 521442 |
|  |  | Rupees Nine Hundred Two and Ninty Three Paisa Only |  |  |  |
| 6 | S/f and fixing in position aluminium channel framing for sliding windows & ventilators alcop made with 5 mm thick tinted glass glazing (Belgium)& aluminium glass glazing (Belgium) & Aluminium fly Screen i/c handles stoppers & locking Arrangement etc complete(DeluxeModelWhite)(S.I.NO.84-A/108) | 300.00 Sft | 1592/69 | P.Sft | 477807 |
|  |  | Rupees Fifteen Hundred Ninty Two and sixty Nine Paisa Only |  |  |  |
| 7 | Cement plaster 1.6 upto 12’ft height ½ ”thick (S.I.13-b/52). | 14268.00 | 2206/60 | %Sft | 314838 |
|  |  | Rupees Two Thousand Two Hundred Six and Sixty Paisa Only |  |  |  |
| 8 | Cement plaster 1.4 upto 12’ft height 3/8 ”thick (S.I.11-a/52). | 18464.00 | 2197/52 | %Sft | 405750 |
|  |  | Rupees Two Hundred One Hundred Ninty Seven and Fifty Two Only |  |  |  |
| 9 | P/l 2” thick topping c.c. 1:2:4 i/c surface finishing & dividing into panels(S.I.16-d/42) | 4446.50 | 3275/50 | %sft | 145645 |
|  |  | Rupees Three Thousand Two Hundred Seventy Five and fifty Paisa Only |  |  |  |
| 10 | Glazed tile dado ¼”thick laid in pigment over 1.2 cement sand mortor ¾”thick i/c finishing (S.I.28/45). | 850.00 | 28299/30 | %Sft | 240544 |
|  |  | Rupees Twenty Eight Thousand Two Hundred Ninty Nine and Thirty Paisa Only |  |  |  |
| 11 | Laying floors of approved coloured glazed tiles ¼”thick laid in white cement and pigment on a bed of ¾”thick cement mortor 1.2 (S.I.25/43). | 397.50 | 27747/06 | %Sft | 110295 |
|  |  | Rupees Twenty Seven Thousand Seven Hundred forty Seven and six Paisa Only |  |  |  |
| 12 | Distempering three coats (S.I.NO.54/24-c) | 4653.00 | 1079/65 | % Sft | 50236 |
|  |  | Rupees One Thousand Seventy Nine and sixty Five Paisa |  |  |  |
| 13 | Preparing the surface & painting with matt finish paint of including rubbing the surface with bathyuent coat.(S.I.36 A 36B/56) | 8434.00 | 3444/38 | %Sft | 290499 |
|  |  | Rupees Three Thousand Four Hundred forty Four and Thirty Eight Paisa Only |  |  |  |
| 14 | Preparing the surface & Painting with weather coat of approved make to old weather coat surface (2nd & each subsequent coat).(S.I.38A+38B+38B/56) | 6980.00 | 2567/95 | %Sft | 179243 |
|  |  | Rupees Two Thousand Five Hundred sixty Seven and ninty Five Only |  |  |  |
| 15 | Preparing the surface & Painting of doors and windows any type i/c edges (Each subsequent coat) (S.I.5-c/ii/70) | 1022.00 | 2116/41 | %Sft | 21630 |
|  |  | Rupees Two Thousand One Hundred Sixteen and Forty One Paisa Only |  |  |  |
|  | Total |  |  |  | 3154471 |
|  | Add or Less Below or Above |  |  |  |  |
|  | Total |  |  |  |  |
|  | Non Schedule Item |  |  |  |  |
| 1. | Providing & laying porceline tiles of shabbir or national make size 16”x16”x1/4” or 24”x24”x1/4” floor in required pattern and design and jointed with high bound of required base including filling of joints with slaurry of white cement and pigment as desired wax polish etc complete in all respect as directed by the Engineer Incharge. | 360.00 |  | P.Sft |  |
|  | Total |  |  |  |  |
|  | G Total G.Floor (Sch+Non Sch Items |  |  |  |  |
|  | FIRST FLOOR |  |  |  |  |
| 1 | RCC work i/c all labour & material except the cost of steel reinf.for its labour of bending & binding which will be paid separately this rate also i/c all kinds of forms moulds lifting shuttering curring rendering & finishing the exposed surface (i/c screening & washing of shingle)(a)RCC work in roof slab beams col rafts lintels and other structure members laid in situ or precast laid in position complete in all respect ratio 1.2.4 90 lbs cement 2cft sand 4cft shingle 1/8”to ¼”guage.(S.I.6A+B/17) | 3201.00 | 337/- | P.Cft | 1078737 |
|  |  | Rupees Three Hundred Thirty Seven Only |  |  |  |
| 2 | Add extra labour for RCC in 2nd and each subsequent storeys.(S.I.6D/17) | 3201.00 | 12/10 | P.Cft | 38732 |
|  |  | Rupees Twele and Ten Paisa Only |  |  |  |
| 3 | (ii) ratio 1-1/2:3 | 3280.30 | 349/- | P.Cft | 1144825 |
|  |  | Rupees Three Hundred Forty Nine only |  |  |  |
| 4 | Add extra labour for RCC in 2nd and each subsequent storeys.(S.I.6D/17) | 3280.30 | 12/10 | P.Cft | 39691 |
|  |  | Rupees Twele and Ten Paisa Only |  |  |  |
|  | Total At Par |  |  |  | 2301985 |
| 5 | Fabrication of mild steel reinforcement for cc i/c cutting bending laying in position making joints & fastening i/c cost of binding wire also i/c removal of rust from bars )Using of Tor Bars (S.I.6/19) | 561.89 Cwt | 5001/70 | P.Cwt | 2810405 |
|  |  | Rupees Five Thousand One and Seventy Paisa Only |  |  |  |
|  | Total |  |  |  | 2810405 |
|  | Add Or Less |  |  |  |  |
|  | Total |  |  |  |  |
| 6 | Providing and laying 1.3.6. cement concrete solid block masonary wall6” and below thickness set in `1.6. cement mortar in ground floor super structure including raking out joints and curing etc complete(s.i.no.22/19) | 2282.60 | 14621/44 | %Cft | 333749 |
|  |  | Rupees Fourteen Thousand Six Hundred Twenty One and Forty Four Only |  |  |  |
| 7 | Add extra Labour for Block Masonary in First Floor (S.I.30(i)/19) | 2282.60 | 328/97 | %Sft | 7509 |
|  |  | Rupees Three Hundred Twenty Eight and Ninty Seven Only |  |  |  |
| 8 | Providing and fixing G.I. frames/chowkats fo size 7”x2” or 4 ½ x 3” for doors using 20 gauge G.I. sheet i/c welded hinges and fixing at site with necessary hold fasts filling with cement sand slurry of ratio 1.6. and repairing the jambs. The cost also in/c all carriage tools and plants used in making and fixing(S.I.NO.28/93) | 314.00 | 228/90 | P.Rft | 71875 |
|  |  | Rupees Two Hundred Twenty Eight and Ninty Paisa Only |  |  |  |
| 9 | First class deodar wood wrought joinery in doors and windows etc fixed in position including chow Kats hold fasts hinges, iron tower bolts chocks cleats handles and cords with hooks etc deodar paneled or paneled and glazed or fully glazed (b)1-3/4” thick(S.I.NO.7-b/58).(1273/76(-)370/83=902/93 (Only Shutter). | 416.50 | 902/93 | P.Sft | 376070 |
|  |  | Rupees Nine Hundred Two and Ninty Three Paisa Only |  |  |  |
| 10 | S/f and fixing in position aluminium channel framing for sliding windows & ventilators alcop made with 5 mm thick tinted glass glazing (Belgium)& aluminium glass glazing (Belgium) & Aluminium fly Screen i/c handles stoppers & locking Arrangement etc complete(DeluxeModelWhite)(S.I.NO.p84-A/108) | 323.00 Sft | 1592/69 | P.Sft | 514439 |
|  |  | Rupees Fifteen Hundred Ninty Two and sixty Nine Paisa Only |  |  |  |
| 11 | Cement plaster 1.6 upto 12’ft height ½ ”thick (S.I.13-a/52). | 10386.50 | 2206/60 | %Sft | 229188 |
|  |  | Rupees Two Thousand Two Hundred Six and Sixty Paisa Only |  |  |  |
| 12 | Extra for lime murded or cement plaster & pointing from 12’ & above (For each additional 10’ft height)(Rs 140/97)(S.I.29/54) | 10386.50 | 140/97 | %Sft | 14642 |
|  |  | Rupees One Hundred Forty and ninty Seven paisa only |  |  |  |
| 13 | Cement plaster 1.4 upto 12’ft height 3/8 ”thick (S.I.13-a/52). | 13440.00 | 2197/52 | %Sft | 295347 |
|  |  | Rupees Two Thousand One Hundred Ninty Seven and Fifty Two Only |  |  |  |
| 14 | Extra for lime murded or cement plaster & pointing from 12’ & above (For each additional 10’ft height)(Rs 140/97)(S.I.29/54) | 13440.00 | 140/97 | %Sft | 18946 |
|  |  | Rupees One Hundred Forty and ninty Seven paisa only |  |  |  |
| 15 | P/l 2” thick topping c.c. 1:2:4 i/c surface finishing & dividing into panels(S.I.16-d/42) | 4997.00 | 3275/50 | %sft | 163676 |
|  |  | Rupees Three Thousand Two Hundred Seventy Five and fifty Paisa Only |  |  |  |
| 16 | Glazed tile dado ¼”thick laid in pigment over 1.2 cement sand mortor ¾”thick i/c finishing (S.I.28/45). | 112.50 | 28299/30 | %Sft | 31837 |
|  |  | Rupees Twenty Eight Thousand Two Hundred Ninty Nine and Thirty Paisa Only |  |  |  |
| 17 | Laying floors of approved coloured glazed tiles ¼”thick laid in white cement and pigment on a bed of ¾”thick cement mortor 1.2 (S.I.25/43). | 36.00 | 27747/06 | %Sft | 9989 |
|  |  | Rupees Twenty Seven Thousand Seven Hundred forty Seven and six Paisa Only |  |  |  |
| 18 | Distempering three coats (S.I.NO.54/24-c) | 5033.00 | 1079/65 | % Sft | 54339 |
|  |  | Rupees One Thousand Seventy Nine and sixty Five Paisa |  |  |  |
| 19 | Preparing the surface & painting with matt finish paint of approved make to old matt finish surface 2nd each subsequent coat.(S.I.36 A 36B/56) | 9426.00 | 3444/38 | %Sft | 324667 |
|  |  | Rupees Three Thousand Four Hundred forty Four and Thirty Eight Paisa Only |  |  |  |
| 20 | Preparing the surface & Painting with weather coat of approved make to old weather coat surface (2nd & each subsequent coat).(S.I.38A+38B+38B/56) | 4015.00 | 2567/95 | %Sft | 103103 |
|  |  | Rupees Two Thousand One Hundred Sixteen and Forty One Paisa Only |  |  |  |
| 21 | Preparing the surface & Painting of doors and windows any type i/c edges (Each subsequent coat) (S.I.5-c/ii/70) | 833.00 | 2116/41 | %Sft | 17630 |
|  |  | Rupees Two Thousand One Hundred Sixteen and Forty One Paisa Only |  |  |  |
|  | Total |  |  |  | 2567006 |
|  | Add or Less Below or Above |  |  |  |  |
|  | Total |  |  |  |  |
|  | Non Schedule Item |  |  |  |  |
| 1. | Providing & laying porceline tiles of shabbir or national make size 16”x16”x1/4” or 24”x24”x1/4” floor in required pattern and design and jointed with high bound of required base including filling of joints with slaurry of white cement and pigment as desired wax polish etc complete in all respect as directed by the Engineer Incharge. | 360.00 |  | P.Sft |  |
|  | Total |  |  |  |  |
|  | G TOTAL F.FLOOR |  |  |  |  |
|  | SECOND FLOOR |  |  |  |  |
| 1 | RCC work i/c all labour & material except the cost of steel reinf.for its labour of bending & binding which will be paid separately this rate also i/c all kinds of forms moulds lifting shuttering curring rendering & finishing the exposed surface (i/c screening & washing of shingle)(a)RCC work in roof slab beams col rafts lintels and other structure members laid in situ or precast laid in position complete in all respect ratio 1.2.4 90 lbs cement 2cft sand 4cft shingle 1/8”to ¼”guage.(S.I.6A+B/17) | 6456.00 | 337/- | P.Cft | 2175672 |
|  |  | Rupees Three Hundred Thirty Seven Only |  |  |  |
| 2 | Add extra labour for RCC in 2nd and each subsequent storeys.(S.I.6D/17) | 6456.00 | 24/20 | P.Cft | 156235 |
|  |  | Rupees Twenty Four and twenty Paisa Only |  |  |  |
| 3 | (ii) ratio 1-1/2:3 | 2670.00 | 349/- | P.Cft | 931830 |
|  |  | Rupees Three Hundred Forty Nine |  |  |  |
| 4 | Add extra labour for RCC in 2nd and each subsequent storeys.(S.I.6D/17) | 2670.00 | 24/20 | P.Cft | 64614 |
|  |  | Rupees Twenty Four and twenty Paisa Only |  |  |  |
|  | Total |  |  |  | 3328351 |
| 5 | Fabrication of mild steel reinforcement for cc i/c cutting bending laying in position making joints & fastening i/c cost of binding wire also i/c removal of rust from bars )Using of Tor Bars (S.I.6/19) | 807.09 Cwt | 5001/70 | P.Cwt | 4036822 |
|  |  | Rupees Five Thousand One and Seventy Paisa Only |  |  |  |
| 6 | Extra Labour Lifting of steel above first floor for every additional floor (S.I.29/19) | 807.09 | 151/25 | P.Cwt | 122072 |
|  |  | Rupees One Hundred Fifty One and twenty five paisa |  |  |  |
|  | TOTAL |  |  |  | 4158894 |
|  | ADD OR LESS |  |  |  |  |
|  | TOTAL |  |  |  |  |
| 7 | Providing and laying 1.3.6. cement concrete solid block masonary wall6” and below thickness set in `1.6. cement mortar in ground floor super structure including raking out joints and curing etc complete(s.i.no.22/19) | 1809.08 | 14621/44 | %Cft | 264514 |
|  |  | Rupees Fourteen Thousand Six Hundred Twenty One and Forty Four Only |  |  |  |
| 8 | Add extra Labour for Block Masonary in First Floor (S.I.30(i)/19) | 1809.08 | 760/03 | %Sft | 13750 |
|  |  | Rupees Seven Hundred Sixty and Three Paisa Only |  |  |  |
| 9 | Providing and fixing G.I. frames/chowkats fo size 7”x2” or 4 ½ x 3” for doors using 20 gauge G.I. sheet i/c welded hinges and fixing at site with necessary hold fasts filling with cement sand slurry of ratio 1.6. and repairing the jambs. The cost also in/c all carriage tools and plants used in making and fixing(S.I.NO.28/93) | 192.00 | 228/90 | P.Rft | 43949 |
|  |  | Rupees Two Hundred Twenty Eight and Ninty Paisa Only |  |  |  |
| 10 | First class deodar wood wrought joinery in doors and windows etc fixed in position including chow Kats hold fasts hinges, iron tower bolts chocks cleats handles and cords with hooks etc deodar paneled or paneled and glazed or fully glazed (b)1-3/4” thick(S.I.NO.7-b/58).(1273/76(-)370/83=902/93 (Only Shutter). | 280.00 | 902/93 | P.Sft | 252820 |
|  |  | Rupees Nine Hundred Two and Ninty Three Paisa Only |  |  |  |
| 11 | S/f and fixing in position aluminium channel framing for sliding windows & ventilators alcop made with 5 mm thick tinted glass glazing (Belgium)& aluminium glass glazing (Belgium) & Aluminium fly Screen i/c handles stoppers & locking Arrangement etc complete(DeluxeModelWhite)(S.I.NO.p84-A/108) | 435.00 Sft | 1592/69 | P.Sft | 692820 |
|  |  | Rupees Fifteen Hundred Ninty Two and sixty Nine Paisa Only |  |  |  |
| 12 | Cement plaster 1.6 upto 12’ft height ½ ”thick (S.I.13-a/52). | 9230.00 | 2206/60 | %Sft | 203669 |
|  |  | Rupees Two Thousand Two Hundred Six and Sixty Paisa Only |  |  |  |
| 13 | Extra for lime murded or cement plaster & pointing from 12’ & above (For each additional 10’ft height)(Rs 140/97)(S.I.29/54) | 9230.00 | 281/94 | %Sft | 26023 |
|  |  | Rupees Two Hundred Eighty One and Ninty Four Paisa Only |  |  |  |
| 14 | Cement plaster 1.4 upto 12’ft height 3/8 ”thick (S.I.13-a/52). | 11394.00 | 2197/52 | %Sft | 250385 |
|  |  | Rupees Two THousand One Hundred Ninty Seven and Fifty Two Only |  |  |  |
| 15 | Extra for lime murded or cement plaster & pointing from 12’ & above (For each additional 10’ft height)(Rs 140/97)(S.I.29/54) | 11394.00 | 281/94 | %Sft | 32124 |
|  |  | Rupees Two Hundred Eighty One and Ninty Four Paisa Only |  |  |  |
| 16 | P/l 2” thick topping c.c. 1:2:4 i/c surface finishing & dividing into panels(S.I.16-d/42) | 4861.50 | 3729/26 | %sft | 181298 |
|  |  | Rupees Three Thousand seven Hundred twenty Nine and twenty six paisa only |  |  |  |
| 17 | Glazed tile dado ¼”thick laid in pigment over 1.2 cement sand mortor ¾”thick i/c finishing (S.I.28/45). | 375.00 | 28299/30 | %Sft | 106122 |
|  |  | Rupees Twenty Eight Thousand Two Hundred Ninty Nine and Thirty Paisa Only |  |  |  |
| 18 | Laying floors of approved coloured glazed tiles ¼”thick laid in white cement and pigment on a bed of ¾”thick cement mortor 1.2 (S.I.25/43). | 178.25 | 27747/06 | %Sft | 49459 |
|  |  | Rupees Twenty Seven Thousand Seven Hundred forty Seven and six Paisa Only |  |  |  |
| 19 | Distempering three coats (S.I.NO.54/24-c) | 4985.00 | 1079/65 | % Sft | 53821 |
|  |  | Rupees One Thousand Seventy Nine and sixty Five Paisa |  |  |  |
| 20 | Preparing the surface & painting with matt finish paint of approved make to old matt finish surface 2nd each subsequent coat.(S.I.36 A 36B/56) | 7335.00 | 3444/38 | %Sft | 252645 |
|  |  | Rupees Three Thousand Four Hundred forty Four and Thirty Eight Paisa Only |  |  |  |
| 21 | Preparing the surface & Painting with weather coat of approved make to old weather coat surface (2nd & each subsequent coat).(S.I.38A+38B+38B/56) | 4059.00 | 2567/95 | %Sft | 104233 |
|  |  | Rupees Two Thousand One Hundred Sixteen and Forty One Paisa Only |  |  |  |
| 22 | Preparing the surface & Painting of doors and windows any type i/c edges (Each subsequent coat) (S.I.5-c/ii/70) | 560.00 | 2116/41 | %Sft | 11852 |
|  |  | Rupees Two Thousand One Hundred Sixteen and Forty One Paisa Only |  |  |  |
| 23 | P/l 2” thick topping c.c. 1:2:4 i/c surface finishing & dividing into panels(S.I.16-d/42) | 5730.50 | 3956/14 | %sft | 226707 |
|  |  | Rupees Three Thousand Nine Hundred Fifty Six and fourteen Paisa |  |  |  |
| 24 | Two coats of bitumen laid hot using 34 lbs for % sft ober roof and blinded with sand at one cft per % sft. | 5730.50 | 1887.40 | % Sft | 108157 |
|  |  | Rupees One Thousand Eight Hundred and Eighty Seven and forty paisa only |  |  |  |
|  | Total |  |  |  | 2874348 |
|  | Add or Less Below or Below |  |  |  |  |
|  | Total |  |  |  |  |
|  | Non Schedule Item |  |  |  |  |
| 1. | Providing & laying porceline tiles of shabbir or national make size 16”x16”x1/4” or 24”x24”x1/4” floor in required pattern and design and jointed with high bound of required base including filling of joints with slaurry of white cement and pigment as desired wax polish etc complete in all respect as directed by the Engineer Incharge. | 384.00 |  | P.Sft |  |
|  | Total |  |  |  |  |
|  | G TOTAL SECOND FLOOR |  |  |  |  |