Tender Reference No. 01(25)/ 2014-2015

SCHEDULE RATE BASED ON (2012)

| S. NO | DESCRIPTION OF ITEMS | QTY | RATE IN FIGURE | RATE IN WORD | UNIT | AMOUNT |
| --- | --- | --- | --- | --- | --- | --- |
| 1. | Dismantling of Cement Concrete Plain Flooring (10/19 a-b-c) 1:2:4 | 1200/Cft | 3327.50 | Three Thousand Three Hundred Twenty Seven & Fifty Paisa | %Cft | 39930/- |
| 2. | Dismantling if Cement Concrete flooring 1:3:6 plain | 1800/Cft | 1306.80 | One Thousand Six & Eight Paisa Onlu | %Cft | 23522/- |
| 3. | Cartage of 100 Cft 5 tons all materials like stone aggregate spwals lime surkhi BG rail fasting points crossing bridge guders pipe sheet M/S Brass sheets 1000 trenches 10×5×3" 1000 tiles 12×6×2" 150 cft timber 100 mould fuel wood by trenches owned by the contractor withing 10 miles etc comp: | 10900/Cft | 1065.04/- | One Thousand Sixty Five & Four Paisa Only | %Cft | 116089/- |
| 4. | Preparing of Sub grade i/c earth excavation leveling to an average depth of 9” dressing to camber and consolidation with power roller | 14000/Sft | 526.28 | Five Hundred Twenty Six & Twenty Eight Paisa Only | %Sft | 73679/- |
| 5. | P/L Granular Sub Base Course Material Over prepared Sub Grade in conformity with lines, grade thickness and typical croos with sand and silt having a smooth gradation I as per AASHTO-M-147-65 (1980) and compacting dry density approved mechanical means of i/c watering with all lead and lift as per direction by E/I | 18000/Cft | Open Rate |  | P.Cft | - |

| S. NO | DESCRIPTION OF ITEMS | QTY | RATE IN FIGURE | RATE IN WORD | UNIT | AMOUNT |
| --- | --- | --- | --- | --- | --- | --- |
| 6. | P/L aggtt base course material over prepared sub-grade in conformity with lines, grades, thickness and typical cross section as shown in drawing for new pavements at as per AASHTO M-147-65 (1980) and compacting to 100% of the minium dry density according to AASHTO-180 method D using approved mechanical means of i/c watering with all lead and lift complete as per specification as directed by the Engineer Inch. | 13000/Cft | Open Rate |  | P.Cft | - |
| 7. | P/L 1 ½” thick topping C.C (1:2:9) i/c surface finishing & diving in to panels | 14000/Sft | 3275.50 | Three Thousand Two Hundred Seventy Five & Fifty Paisa Only | %Sft | 458570/- |
| 8. | Extra for using (S.R Cement) Sulphate resistant cement in place of ordinary Portland cement | 400/Cwt | Open Rate |  | P.Cwt | Open Rate |
| 9. | Excavation of pipe line trenches pipe all kind of soil murum i/c trimming dressing leveling beds trenches correct level and graded cutting joints holes disposal of surplus earth with in 1 chains as directed by the Engg: Inch: P. fence guard lights flags temporary crossing non vehicler traffic where ever req: a lift upto 5 ft & lead up to 1 chain. | 4625/Cft | 4650/- | Rupees Four Thousand Six Hundred & Fifty Only | %0Cft | 21828/- |
| 10. | Excavation of pipe line trenches pipe all kind of soil murum i/c trimming dressing leveling beds trenches correct level and graded cutting joints holes disposal of surplus earth with in 1 chains as directed by the Engg: Inch: P. fence guard lights flags temporary crossing non vehicler traffic where ever req: a lift upto 5 ft & lead up to 1 chain. | 2950/Cft | 10770/- | Ten Thousand Seven Hundred & Seventy Only | %0Cft | 38954/- |

| S. NO | DESCRIPTION OF ITEMS | QTY | RATE IN FIGURE | RATE IN WORD | UNIT | AMOUNT |
| --- | --- | --- | --- | --- | --- | --- |
| 11. | P. Rcc pipe ASTM class class ‘C’ 76-62 T/C 76-70 class II wall ‘B’ and fixing in trenches i/c cutting fitting jointing with rubber ring i/c testing with water to specified pressure 8″ & 12″dia pipe. | 1200/P.Rft | 367/- | Three Hundred & Sixty Seven Only | P.Rft | 440400/- |
| 12. | Const of Main Holes 4 inside with Rcc covers 21″dia 3×¾ with flat iron frame 5ft depth 1:2:4 in laying CC cost in situ 9″ thick wall and disposal of surplus stuff etc comp: | 50 No | 13064/- | Rupees Thirteen Thousand & Sixty Four Only | Each | 653200/- |
| 13. | Add or deduct for depth of man holes above or beyond 5 ft (RA). | 100/Rft | 1540/- | Rupees One Thousand & Forty Only | P.Rft | -154000/- |
| 14. | Refilling the excavated stuff in trenches 6″ thick layer i/c watering ramming to full compaction etc comp: | 4534/Cft | 2760/- | Rupees Two Thousand Seven Hundred & Sixty Only | %0Cft | 12721/- |
| 15. | M/S 21”dia Rcc Manhole covers cost is 1:2:4 concrete ratio 3" depth at i/c reinforced with 3/8"dia tor steel bars 4" C.C welded to 1/8" thick 2.5" depth M.S plat i/c curing stacking and transportation with in 10 Miles | 40Nos | 913.63 | Rupees Nine Hundred Thirteen & Sixty Three Only | Each | 36545/- |
| 16. | M/S of Rcc Ring Slab of 21"dia inside 36"dia out side 75" width and 6" thick i/c dia tor steel bars two concrete ring with 3/8"dia 8 No cross liaked bars welded and two sunk type hooks cost in a 1:1 1/2 3 concrete with embaded 15kg C.I from in prefund position i/c transportation cage etc | 10 Nos | Open Rate |  |  | Open Rate |
| Total Amount of Schedule Items |  |  |  |  |  | 1761438/-+O/R |