| NAME OR WORK :- | M/R TO SINDH SECRETARIAT NO.V, KARACHI ( RENOVATION OF COMMITTEE ROOM OF LOCAL GOVT: DEPARTMENT ). |
| --- | --- |

“ SCHEDULE-B “

| S.# | DESCRIPTION OF ITEMS | QTY: | RATE | UNIT | AMOUNT |
| --- | --- | --- | --- | --- | --- |
|  | PART ‘A’ ELECTRIC SCHEDULE ITEMS. |  |  |  |  |
| 1. | Wiring for plug point with 3/.029 PVC insulated wire in 20mm ( ¾” ) PVC conduit recessed in the wall or column as required. ( SI.126 / P-15 ). | 09 Point | 985/- | P.Point | 8865/- |
|  | ( Rupees Nine Hundred Eighty Five ) Only. |  |  |  |  |
| 2. | Wiring for call bell point with 3/.029 PVC insulated wire in 20mm ( ¾”) PVC conduit recessed in the wall or column as required. ( SI.128 / P-15 ). | 02 Point | 1764/- | P.Point | 3528/- |
|  | ( Rupees One Thousand Seven Hundred Sixty Four ) Only. |  |  |  |  |
| 3. | Wiring for main 2-3/.029 PVC insulated copper wire in existing concealed pipe in/c removing burnt wire from concealed wiring and clean the pipe as directed by the Engineer Incharge.(SI.01 / P-01 ). | 268.00 Meter | 121/- | P.Meter | 32428/- |
|  | ( Rupees One Hundred Twenty One ) Only. |  |  |  |  |
|  | (A). Total Rs: |  |  |  | 44821/- |
|  | PART ‘B’ NON SCHEDULE ITEMS. |  |  |  |  |
| 1. | P/F laminated wood floor fully imported made in Germany having size 51”x4” in/c preparing base, laying laminated wood floor strips, pasting with glue or bonding solution, filling joints with pigments grout as directed by Engineer Incharge. | 642.77 Sft |  | P.Sft |  |
| 2. | P/F Gypsum board pelmet or drop box to be hanged from ceiling in specified shape and design as approved by the consultant. This in/c cutting the existing Gypsum board false ceiling in proper shape, hanging the structure by aluminum angle 1’x1” @ 3’-0 apart drilled and screwed in RCC slab, making hanging frame by using Aluminum U-channel section 1-1/2” wide in 18”x18” rafters and fixing Gypsum board sheet with clips and screw. Then filling the joints & screws holes with plastic mesh and plaster of Paris and making surface smooth as directed by the Engineer Incharge. | 591.61 Sft |  | P.Sft |  |
| 3. | P/F fine quality roller type blind with approved design false & in/c steel angle fixing with steel screws as per window as required @ site as directed by the Engineer Incharge. | 70.0 Sft |  | P.Sft |  |
| 4. | Removing debries from Sindh Secretariat Buildings in/c collecting from different floors, throwing to stair to ground floor at proper place and then loading on truck and throwing out of city government premises, fuel & hire charges of truck, loading & unloading etc complete. | 02 Jobs |  | P.Job |  |
| 5. | French polish of doors windows ventilators stair floor work etc includes preparing surface and polishing by using lakh Dana, Sprit, thinner, pigment, ICI lacquer, Regmal (Germany) material ICI sealer tar pine oil, polishing as per satisfaction of competent authority etc complete. . | 931.0 Sft |  | P.Sft |  |
| 6. | Providing & Fixing Telephone board Sheet 4”x4” make Clipsal or equivalent as directed by the Engineer Incharge. | 04 Nos |  | Each |  |
| 7. | Providing laying cable wire for telephone concealed with PVC pipe in/c all cost of labour material & cartage etc complete. | 51.0 Rft |  | P.Rft |  |
| 8. | P/L cable wire for internet concealed with PVC pipe in/c all cost of labour material & cartage etc complete., | 61.0 Rft |  | P.Rft |  |
| 9. | S/F fancy switch / socket imported make with double sheets fixing with existing Concealed board on approved shape and design by the Engineer Incharge etc complete. | 49 Nos |  | Each |  |
| 10. | P/F 15-20 Amps Light Plug with board (China make) of approved quality & make as directed by the Engineer Incharge. | 09 Nos |  | Each |  |
| 11. | P/F 3-pin 13-amps universal / computer sockets Clipsal make with fancy sheet as directed by Engineer Incharge. | 09 Nos |  | Each |  |
| 12. | P/F false ceiling spot light fancy type round shape with glass and 2 holder and energy saver of approved make & quality including fixing in false ceiling by cutting hole as directed by the Engineer Incharge. | 44 Nos |  | Each |  |
| 13. | S/F LED spot light China make with 7 Nos lamps each 6 watts fitted on walls, ceiling of approved quality as directed by the Engineer Incharge. | 40 Nos |  | Each |  |
| 14 | P/F wall bracket fan 18” dia of approved make & quality (Royal/Pak, Millat or equivalent) in/c fixing on wall as directed by the Engineer Incharge | 03-Nos |  | P/Each |  |
| 15 | P/F wall paneling with lassani sheet ¾’ thick and teak ply in/c aluminum sheet on back site for leakage control & fixing lassani sheet & then teak ply in approved design and texture with 3” beading including French polish 3 coats including all cost of labour making & cartage etc complete. | 180.00 Sft. |  | P/Sft. |  |
|  |  | (B) Total Rs. |  |  |  |
|  | (A). Total Rs: |  |  |  |  |
|  |  | G. Total (A + B ) Rs: |  |  |  |
|  | Say Rs. |  |  |  |  |

CONDITION.

1).	Typographical errors in the schedule-“B” are the subject to the correction with reference to the schedule of rate ( Gen: Vol: III, Part-II, inforced: 2012, as approved by the standing rates committee.

2).	No Cartage will be paid on any items of works.

3).	No premium shall be paid on Non-Schedule items.

4).	Water shall be arranged by the Contractor.

CONTRACTOR.

| ASSISTANT ENGINEER PROVINCIAL BUILDINGS SUB-DIVISION-V, KARACHI. | EXECUTIVE ENGINEER PROVINCIAL BUILDINGS DIVISION-I, KARACHI. |
| --- | --- |