| NAME OR WORK :- | M/R TO SINDH SECRETARIT NO-I, REHABILITATION OF GENERAL BATH ROOMS GROUND FLOOR TO 7TH FLOOR INCLUDING REPLACEMENT OF GI PIPE LINE OVER ROOF. |
| --- | --- |

“ SCHEDULE ‘B’

| S.# | DESCRIPTION OF ITEMS | QTY: | RATE | UNIT | AMOUNT |
| --- | --- | --- | --- | --- | --- |
|  | SCHEDULE ITEMS PART-“A” ( CIVIL WORKS ) |  |  |  |  |
| 1 | Scraping ( b) Ordinary distemper oil bound distemper or Pain on Walls. ( SI.54-(b) / P-13 ). | 5268.00 Sft | 226/88 | %Sft | 11952/- |
|  | Rupees (Two hundred twenty six & Ps. Eighty eight) only. |  |  |  |  |
| 2 | Preparing the surface & painting with matt finish paint of approved make to old matt finish surface (2nd & subsequent coat (37-a+b / P-55) | 3779.00 Sft | 1772/38 | %Sft | 66978/- |
|  | Rupees (One thousand seven hundred seventy two & Ps. Thirty eight) only. |  |  |  |  |
| 3 | Distempering (a) 1st cost over priming coat) (b) Two coats(SI 24-a+b P/54) | 1489.00-Sft | 1043/90 | %Sft | 15544/- |
|  | Rupees (One thousand forty three & Ps. Ninety) only. |  |  |  |  |
| 4 | Painting old surface (c ) Painting doors and windows any type. (i) 1st coat (ii) second & subsequent coat ( SI.4( c ) ( i + ii )/P-68 ). | 1424.00-Sft | 1160/06 | %Sft | 16519/- |
|  | Rupees (One thousand one hundred sixty & Ps. Six ) only. |  |  |  |  |
| 5 | Painting old surfaces ( d ) painting guard bars, gates iron bars gratings, railings including standard braces ( etc ) and similar open work.( i ). First Coat.Rs.371/80 ( ii ). Each subsequent coat. Rs.302/80 ( i ) + ( ii ) = Rs.674/60 ( SI.4d( i+ii ) / P-69 ). | 210.00 Sft | 674/60 | % Sft | 1417/- |
|  | Rupees. Six hundred seventy four. And. Sixty. Paisa |  |  |  |  |
|  |  |  | ( A ) Total Rs. |  | 112410/- |
|  | Part (B) NON-SCHEDULE ITEMS. |  |  |  |  |
| 1 | Supplying & fixing earthen ware European commode set (Porta /USA Make) coupled with flush tank & seat cover complete with internal fittings, fixtures, clamps, necessary lead connection and making requisite No of holes in wall plinth or floor for pipe connection & making good in c.c. 1:2:4 as directed by the Engineer Incharge. | 05 Nos |  | Each |  |
| 2 | P/F white/colored glazed earthen ware squatting/Indian type W.C pan of not less than 23” (ACL make or equivalent) with low level flush tank (Push button type) having capacity of 3.0 gallon in/c necessary lead connection with pipe & bend etc making requisite number of holes in wall, plinth & floor for pipe connection and making good in c.c. 1:2:4 as per instruction of the Engineer Incharge. . | 02 Nos |  | Each |  |
| 3 | Providing and fixing White or colored glazed earthen ware Vanity Basin (Asif local or equivalent ) Standard size including cutting of marble top fixed with paste or Jelly with fine finishing etc. complete as directed by Engineer Incharge. | 05 Nos |  | Each |  |
| 4 | P/F marble top 1” thick (botesina or teravera or equivalent) having size upto 8’-0x2’-0 in/c cutting into proper size, making round gola & cutting for fixing vanity basin or sink bowl and filling joints with white cement or jelly in/c grinding and chemical polishing etc complete as directed by the Engineer Incharge. | 52.08 Sft |  | P/Sft |  |
| 5 | S/F looking glass or mirror of Belgium or Italian make of approved quality & make in/c fixing with clips as directed by the Engineer Incharge | 28.12 Sft |  | P/Sft |  |
| 6 | P/F white or colored glazed earthen ware Urinal basin 17” height (Asia local or equivalent) with 01-gallon automatic flush tank in/c necessary waste pipe, nylon connection, screws etc complete as directed by Engineer Incharge | 02 Nos |  | Each |  |
| 7 | P/F floor trap jali (PVC type) size 6”x 6”of approved color & design (Master A Class) as directed by the Engineer Incharge. | 24 Nos |  | Each. |  |
| 8 | P/F Flush Tank Golden make with waste pipe of approved quality as directed by the Engineer Incharge. | 18 Nos |  | Each. |  |
| 9 | P/F sanitary fitting chrome plated heavy gauge of approved quality (Master / Sonex make) fitted with completed accessories as directed by the Engineer Incharge. Swan type pillar cock . ½” dia | 12 Nos |  | Each. |  |
| 10 | P/F sanitary fitting chrome plated heavy gauge of approved quality (Irfan or equivalent make) fitted with completed accessories as directed by the Engineer Incharge. Stop Cock /Bib Cock / Tee Cock . ½” dia | 52 Nos |  | Each |  |
| 11 | P/F Double Bib Cock / Long Bib Cock 1/2” dia plastic coated A Class as directed by the Engineer Incharge. (Irfan Make) | 11 Nos |  | Each |  |
| 12 | P/F Long Bib Cock 1/2” dia plastic coated A Class as directed by the Engineer Incharge. (Irfan Make) | 14 Nos |  | Each |  |
| 13 | P/F UPVC water supply pipe of AGM or Pak Arab make schedule 40(E) ASTM standard D-1785 with all necessary fittings. This includes fixing on walls with clamps or Jubilee clips upto height of in vertical or horizontal position using Jhoola or long ladder or scaffolding and jointing pipe & fittings with approved cement solvent / JTG solution. The rate includes all cost of labour, material, and cartage, as directed by Engineer Incharge |  |  |  |  |
| a. | 2” dia pipe UPVC | 160.00 Rft |  | P/Rft. |  |
| b. | 1” dia Pipe UPVC | 60.00 Rft. |  | P/Rft. |  |
| 14 | P/F UPVC fittings of schedule 40(E) ASTM –D-1785 Standard (AGM or Pak Arab make) with the cost of breaking through walls and roof in/c jointing with PVC solvent/JTG solution and fixing at any height/floor using Jhoola or long ladder or scaffolding in horizontal or vertical position as directed by the Engineer Incharge |  |  |  |  |
| a. | 2” dia 45 degree / Plug Tee | 04 Nos |  | Each. |  |
| b. | 2” dia socket | 06 Nos |  | Each. |  |
| c. | 2” dia Tee | 06 Nos |  | Each. |  |
| d. | 2” dia elbow 90/45 degree/. | 08 nos |  | Each. |  |
| 15 | Providing & Fixing G.I Gate Valve Heavy of approved quality & make as directed by the Engineer Incharge. |  |  |  |  |
| a. | Gate valve 2” dia | 03 Nos |  | Each. |  |
| b. | Gate valve 1-1/2” dia | 04 Nos |  | Each. |  |
| 16 | S/F 1-40 watts tube light with 40 watts rod, chock, starter & base fitted on wall ceiling with 23/.076 flexible wire in/c connection to ceiling rose as directed by the Engineer Incharge.(Phillips make ) or equivalent | 20 nos |  | Each. |  |
| 17 | P/F sanitary fitting chrome plated heavy gauge of approved quality (Sonex / Master make) fitted with completed accessories as directed by the Engineer Incharge. Muslim shower. ½” dia. | 01 No |  | Each. |  |
|  |  | (B). Total Rs. |  |  |  |
|  |  | (A). Total Rs. |  |  |  |
|  |  | (A + B). Total Rs. |  |  |  |
|  |  | Say Rs. |  |  |  |

CONDITION.

1).	Typographical errors in the schedule-“B” are the subject to the correction with                       reference to the schedule of rate ( Gen: Vol: III, Part-II, inforced: 2012, as approved by the standing rates committee.

2).	No Cartage will be paid on any items of works.

3).	No premium shall be paid on Non-Schedule items.

4).	Water shall be arranged by the Contractor.

CONTRACTOR.

| ASSISTANT ENGINEER PROVINCIAL BUILDINGS SUB-DIVISION-V, KARACHI. | EXECUTIVE ENGINEER PROVINCIAL BUILDINGS DIVISION-I, KARACHI. |
| --- | --- |