SCHEDULE “B”

| NAME OF WORK | M/R TO DISTRCT JAIL MALIR (NON-RESIDENTIAL) |
| --- | --- |

| S.No. | Description of items | Quantity | Rate | Unit | Amount |
| --- | --- | --- | --- | --- | --- |
|  | Part “A” Civil work |  |  |  |  |
| 1 | Dismentling cement concrete plain 1:2:4 (S.I.No.19-c-P-10) | 367.00 | 3327.50 | %Cft | 12212/- |
|  | Rupees Three thousand three hundred twenty seven & Ps: Fifty)only |  |  |  |  |
| 2 | Dismantling glazed or encaustic tiles etc(S.I.No.55-p-13) | 416.00 | 786.50 | %Sft | 3272/- |
|  | Rupees Seven hundred Eighty Six & Ps: Fifty)only |  |  |  |  |
| 3 | Removig cement or lime plaster (S.I.No.53-p-13) | 300.00 | 121.00 | %Sft | 363/- |
|  | Rupees One hundred twenty one)only |  |  |  |  |
| 4 | Applying floating coat of Hi Bond (Universal) as a bond coat or as an adhesive link between old & fresh concrete or plaster as dircted by Engineer incharge. (S.I.No.13(i)-P-52) | 300.00 | 1029.05 | %Sft | 3087/- |
|  | Rupees One thousand twenty nine & Ps:five)only |  |  |  |  |
| 5 | Cement plaster 1:4 upto 12’ height © ¾” thick (S.I.No.11-c-P-52) | 300.00 | 3015.76 | %Sft | 9047/- |
|  | Rupees Three thousand fifteen & Ps :Seventy six)only |  |  |  |  |
| 6 | Providing & Laying 1”thick topping cement concrete (1:2:4) including Surface finishing and dividing into panels: ©2”thick (S.I.No.16-c P-42) | 2160.00 | 3275.50 | %Sft | 70751/- |
|  | Rupees Three thousand two hundred seventy five & Ps:fifty)only |  |  |  |  |
| 7 | Providing and fixing G.I frames /Chowkhats of size 7" x 2" or 4 1/2" x 3" for door using 20 gauge G.I sheet I/c welded hinges and fixing at site with necessary hold fasts, filling with cement sand slurry of ratio 1:6 and repairing the jambs. The cost also i/c all carriage, tools and plants used in making and fixing (S.I. 29/93). | 48.00 | 228.90 | P Rft | 10987/- |
|  | Rupees Two hundred twenty eight & Ps: Ninety)only |  |  |  |  |
| 8 | Providing and fixing in position, Doors, Windows and Ventilators of first class deodar wood frames, and 1 3/4” thick commercial , ply veneer shutters of first class deodar skeleton (Hollow) and commercial ply wood (3 ply ) on both sides i/c .hold fasts, hinges, iron tower bolts ,handles and cleats with cord and one Mortice Lock and hooks( 1227/36 (-) 370/83) (S.I. 9/58). | 45.00 | 856.53 | P.Sft | 38544/- |
|  | Rupees Eight hundred fifty six &Ps: fifty three)only |  |  |  |  |
| 9 | Galvanized wire gauge 144 mesh per square inch of 22 S.W.G fixed to chowkats without deodar patti.(S.I. 62/66). | 396.00 | 104.15 | P.Sft | 41243/- |
|  | Rupees One hundred four & Ps:Fifteen)only |  |  |  |  |
| 10 | P/F GI expanded metal 1/8 -20 gauge fixed to chowkats with 3/4" deodar strips and screws. (S.I. 66-a/66). | 396.00 | 178.68 | P Sft | 70757/- |
|  | Rupees One hundred seventy eight & Ps: Sixty eight)only |  |  |  |  |
| 11 | Providing & laying tiles glazed 6”x6”x1/4” on floor or wall facing in required color & pattern of STILE specification jointed in white cement and pigment over a base of 1:2 grey cement mortar ¾” thick i/c washing and fillilng of joints with slaurry of white cement and pigment in desired shape with finishing, clearing and cost of wax polish etc complete i/c. cutting tiles to proper profile.(S.I.No.60-P-47) | 416.00 | 30509.77 | %sft | 126921/- |
|  | Rupees Thirty thousand five hundred Nine & Ps: Seventy seven)only |  |  |  |  |
| 12 | Providing and laying 1:3:6 cement concrete solid bock maonary wall 6" and below in thickness set in 1:6 cement mortar inground floor superstructure including raking out joints and curring etc.(S.I.No.24-P-19) | 50.00 | 15771.01 | %Cft | 7886/- |
|  | Rupees Fifteen thousand seven hundred seventy one & Ps:one)only |  |  |  |  |
| 13 | (b) Ordinary distemper, oil bound distemper or paint on walls (S.I.No.54-b-p-13) | 6855.00 | 226.88 | %Sft | 15553/- |
|  | Rupees Two hundred twenty six & Ps: eighty eight)only |  |  |  |  |
| 14 | Distempering (b) 2-coats S.I.No.24-b-P-54) | 6855.00 | 1043.90 | %Sft | 71559/- |
|  | Rupees One thousand Forty three & Ps:Ninety)only |  |  |  |  |
| 15 | Painting old surfaces (c) Painting doors and windows any type (i) First coat (ii) Each subsequent coat. (S.I. 4-c (i+ii)/68). | 2707.00 | 1160.06 | %Sft | 31403/- |
|  | Rupees One thousand one hundred Sixty & Ps: six)only |  |  |  |  |
|  | Total Rs; |  |  |  | 513585/- |
|  | Non-Schedule items |  |  |  |  |
| 1 | P/L precast beam size (4"x9") as per required size as directed by Engineer In-charge the rate i/c all cost of material lbor cartage etc complete (R.A) | 150.00 |  | P Rft |  |
| 2 | P/L precast slab size 3’-0x2-0’ as per required size i/c filing the joints with cement mortar ratio 1:2 i/c finishing curing etc complete as directed by the Engineer in charge the rate i/c all cost of material labor & cartage etc complete (R.A) | 480.00 |  | P.Sft |  |
|  | Total Rs: Non-Schedule items |  |  |  |  |
|  | Part "B" Water Supply & Sanitary work |  |  |  |  |
| 1 | Providing & fixing squating type white glazed eatrhen ware w.c. pan with i/c the cost of flushing cistern with internal fitting and flush pipe with bend and making requisite number of holes in walls plinth and floor for pipe connetion and making good in cement concrete 1:2:4 A.W.C pan of not less then 23" clear opening between flushing rims and 3 gallons flushing tank with 4"dia C.I.trap (S.I.No.1-A-1) | 4.00 | 5044.60 | Each | 20176/- |
|  | Rupees Five thousand forty four & Ps: Sixty)only |  |  |  |  |
| 2 | P/F 24" x 18"lavatory basin in white glazed earthen ware complete with & i/c the cost of W.I or C.I cantilever brackets 6 inches built into wall, painted white in two coast after a primary coat of red lead paint, a pair of 1/2" dia rubber plug & chrome plate brass chain 1-1/4" dia malloable iron or c.p brass traps malloable iron on brass unions and making requisite number of holes in wals, plinth & floor for pipe connection and making good in cement concrete 1:2:4 Standard Pattern) (S.I.No.3-p-8) | 3.00 | 4253.70 | Each | 12761/- |
|  | Rupees Four thousand two hundred fifty three & Ps: Seventy)only |  |  |  |  |
| 3 | Add extra for labour for providing & fixing of earthen ware pedestal white or coloured glazed (Foreign or Equivaent)(S.I.No.9-p-3) | 3.00 | 938.47 | Each | 2815/- |
|  | Rupees Nine hundred thirty eight & Ps :Forty seven)only |  |  |  |  |
| 4 | Providing & fixing 6"x2" or 6"x3" C.I.floor trap of the aproved selt cleaning design with a C.I screwed down gratting with or without a vent arm complete with & i/c making requisite number of holes in walls, plinth & floor for pipe connections & making good cement concrete 1:2:4 (S.I.No.20-p-06) | 4.00 | 2042.43 | Each | 8170/- |
|  | Rupees Two thousand forty two & Ps: forty three)only |  |  |  |  |
| 5 | Cancealed C.P fittings of Superior quality for tiles Bath Rooms (a) S/Fixing cancealed stop cock of superior quality with c.p head 1/2"dia.(S.I.No.11(a)-P-18) | 12.00 | 478.28 | Each | 5739/- |
|  | Rupees Four hundred seventy eight & Ps :Twenty eight)only |  |  |  |  |
| 6 | Supplying & fixing in position C.P Bib cock (b)3/4" dia C.P bib cock,standard pattern (S.I.No.2(ii)-P-16) | 8.00 | 348.92 | Each | 2791/- |
|  | Rupees Three hundred forty eight & Ps :Ninety two)only |  |  |  |  |
| 7 | Supplying & fixing in position C.P Bib cock (b) 1/2" dia C.P bib cock, Standard pattern. (S.I.No.2-b-P-16) | 15.00 | 150.92 | Each | 2264/- |
|  | Rupees One hundred fifty & Ps: Ninety two)only |  |  |  |  |
| 8 | Providing & Fiing handle valves (china) (i) 1/2"dia (S.I.No.5-(i)-P-17) | 4.00 | 200.42 | Each | 802/- |
|  | Rupees Two hundred & Ps: forty two)only |  |  |  |  |
|  | Total Rs: |  |  |  | 55518/- |
|  | NON SCHEDULE ITEMS |  |  |  |  |
| 1 | P/F UPVC pipe ½” dia (Pak Arab) Sch 40 to surface by using clips / coddled / sockets / bush / tee etc. or recessed in masonary CC or RCC upto 20 ft height & making good with c.c. i/c. cutting finishing etc. complete as per instruction of Engineer Incharge specification of the matrial should meet the reinforcement f class 12456-E in ordinate with the ASTM-D 1784 type grade-1 rate i/c. all cost of labour material cartage scaffolding / ladders etc. complete (R.A.). | 60.00 |  | P.Rft |  |
| 2 | Providing & Fixing CPVC pipe 3/4”dia (Pak Arab) i/c. cutting making & jointing with sulation paste with special, all required fittings approve quality i/c all cost of labour upto height Ground to 7th floor 80’Height as per directed by Enginer incharge etc complete (R.A) | 43.00 |  | P.Rft |  |
| 3 | P/F UPVC pipe 3”dia (Pak Arab) i/c cutting fitting with colours clamps & rubber solution this rate also i/c making holes in walls plinth floor and roof for fixing pipe and making good in C.C.1:2:4 testing pressure i/c leader,seafolliding as per directed as per Engineer incharge.(R.A) | 24.00 |  | P.Rft |  |
| 4 | P/F UPVC pipe 4”dia (Pak Arab) i/c cutting fitting with colours clamps & rubber solution this rate also i/c making holes in walls plinth floor and roof for fixing pipe and making good in C.C.1:2:4 testing pressure i/c leader,seafolliding as per directed as per Engineer incharge.(R.A) | 10.00 |  | P.Rft |  |
| 5 | P/F UPVC Plain Y-Tee 4”dia (R.A) | 6.00 |  | Each |  |
| 6 | P/F UPVC plug bend 4”dia (R.A) | 6.00 |  | Each |  |
| 7 | P/F UPVC Plain Bend 4”dia (R.A) | 6.00 |  | Each |  |
| 8 | P/F UPVC plug Tee 4”dia (R.A) | 4.00 |  | Each |  |
|  | Total Rs: Non-Schedule items |  |  |  |  |
|  | Part “C” Electric work |  |  |  |  |
| 1 | Wiring for light or fan point with 3/.029 PVC insulated wire in 20mm (3/4") channel patti on surface as required (S.I.No.129-P-15) | 1.00 | 910.00 | P.Point | 910/- |
|  | Rupees Nine hundred ten)only |  |  |  |  |
| 2 | Proviing & Fixing one way SP 5amp switch flush type.(S.I.No.219-p-33) | 2.00 | 54.00 | P.No | 108/- |
|  | Rupees Fifty four)only |  |  |  |  |
| 3 | Providing & Fixing two pin 5 amp plug & socket flush type.(S.I.No.225-p-33) | 9.00 | 83.00 | P.No | 747/- |
|  | Rupees Eighty three)only |  |  |  |  |
|  | Total Rs: |  |  |  | 1765/- |
|  | Non-Schedule items |  |  |  |  |
| 1 | Providing & Fixing Bracket light holder.(R.A) | 1.00 |  | Each |  |
| 2 | P/F energy saver superior quality i/c fixing on existing holder etc complete (R.A)) | 4.00 |  | Each |  |
|  | Total Rs: Non-Schedule items |  |  |  |  |

GENERAL : ABSTRACT

1  Part “A” Civil Work

Schedule items

Non-Schedule items				Rs:

2  Part “B”

Water Supply & sanitary Work

Schedule items				Rs:

Non-Schedule items

3 Part “C”

Electric Work

Schedule items

Non-Schedule items				Rs:

Total Rs: A+B+C  	=			Rs:

TERMS & CONDITIONS

- Any typographical error in the schedule “B” are subject to correction with the reference to the schedule of rates General item & Water supply and Sanitary item 2012 in force from 12th July 2012 Publication No.52 & 54 as approved by the standing rates committee Sindh Karachi.

- 100% well graded trushed bajri shall be used in item of RCC 1:2:4

- Water shall be arranged by the contractor at site of work without any extra payment.

- No  Premium  shall be paid on Non-Schedule items.

- N0  Cartage of any items of material either supplied by the Department or arranged by the contractor shall be paid.

- C.C Block shall be machine made.

- All RCC/CC cast in situ shall be mechanically vibrated.

- Contractor has to bring samples of the material and handed to the Engineer in charge free of cost.

- All buildings debris & surplus stuff not req. for use & construction shall  be removed from the site the suitable disposal off by

- The contractors for which no extra cartage shall be paid.

- The work will be carried out as per PWD Speciation’s.

| CONTRACTOR | Assistant Engineer |  |  |  |
| --- | --- | --- | --- | --- |
|  | Provincial Building Sub Division No VI |  |  |  |
|  | Karachi |  |  |  |
|  | Executive Engineer |  |  |  |
|  | Provincial Buildigs Division No:II |  |  |  |
|  |  | Karachi |  |  |