Name of Work:	M/R TO BARRACK NO. 19 OFFICE OF THE CHIEF PILOT TO GOVERNOR / CHIEF MINISTER SINDH, SINDH SECRETARIAT BLOCK-4/A KARACHI (P/F STEEL GRIDER & CHANNEL PATTI FOR ROOF FRAMING & ROOF TRUSSES) STAFF ROOM)

SCHEDULE – B

PART – A	 CIVIL WORK (Schedule Item):

| S/No. | Description | Qty | Rate | Unit | Amount |
| --- | --- | --- | --- | --- | --- |
|  | Preparing the surface and painting with matt finish I/c rubbing the surface with Bathy (silicon carbide rubbing brick) filling the voids with zink /chalk / plaster of paris mixture, applying first coat premix, making the surface smooth and then painting 3 coats with matt finish of approved make etc: complete .(new surface). 2nd & subsequent coat. (S.I. 36-a+b/55). | 3125.25 Sft. | 2717/- | % Sft. | 84913/- |
|  |  | (Rupees Two Thousand Seven Hundred Seventeen Only) |  |  |  |
|  | Providing and Fixing Jute felt paper of 60 lbs over roof in/c. cleaning of roof with wire brush on removing dust applying bitumen coat at the rate of 34 lbs per % Sft. As premix inter coats and then laying felt paper with 10% over laps, then applying and spreading hill sand at the rate of 1 Cft. For 100 Sft. The cost also in/c. necessary fire material kerosene oil wood etc. (S.I. 42/39). | 777.0 Sft. | 106/73 | P. Sft. | 82929/- |
|  |  | (Rupees One Hundred Six and Seventy Three Paisa Only) |  |  |  |
|  | Providing and fixing in position, Doors, Windows and Ventilators of first class deodar wood frames, and 1 3/4" thick commercial , ply veneer shutters of first class deodar skeleton (H ollow ) and commercial ply wood (3 ply ) on both sides. (S.I. 9/58). (1227.36 – 370.83). | 19.50 Sft. | 856/53 | P. Sft. | 16702/- |
|  |  | (Rupees Eight Hundred Fifty Six and Fifty Three Paisa Only) |  |  |  |
| TOTAL RS. |  |  |  |  | 184544/- |
| Add / Deduct Premium / Below Rs. |  |  |  |  |  |
| TOTAL PART – A Rs. |  |  |  |  |  |

PART – B	Non Schedule Items:

| S/No. | Description | Qty | Rate | Unit | Amount |
| --- | --- | --- | --- | --- | --- |
|  | Providing & Fixing Iron Steel Grinder 8’x3’x1/4” II-Type including cutting, welding upto 20’ height and red oxide paint etc. complete | 40.0 Rft. |  | P. Rft. |  |
|  | Providing & Fixing irons teel channel in U-Shape 4’x2’x1/4’ including cutting welding upto 20’ height & including red oxide paint etc. complete. | 144.0 Sft. |  | P. Rft. |  |
|  | Providing and Fixing False Ceiling of Gypsum Fibre board in/c. frame work of aluminum double channel section hanged with Nails, Hooks, wire to ceiling etc. as directed by the Consultant. | 470.0 Sft. |  | P. Sft. |  |
| TOTAL PART – B Rs. |  |  |  |  |  |

PART – C	 ELECTRIC WORK (Schedule Item):

| S/No. | Description | Qty | Rate | Unit | Amount |
| --- | --- | --- | --- | --- | --- |
|  | Wiring for light or fan point with 3/.029 PVC insulated wire in 20mm (3/4” ) PVC conduit recessed in the wall or surface as required (S.I. 124/15). | 15 Nos. | 1130/- | P. Point | 16950/- |
|  |  | (Rupees One Thousand One Hundred Thirty Only) |  |  |  |
| TOTAL RS. |  |  |  |  | 16950/- |
| Add / Deduct Premium / Below Rs. |  |  |  |  |  |
| TOTAL PART – C Rs. |  |  |  |  |  |

PART – D	Electric Work (Non Schedule Items):

| S/No. | Description | Qty | Rate | Unit | Amount |
| --- | --- | --- | --- | --- | --- |
|  | P/F False Ceiling Spot light fancy type round shape or square shape with glass and 2 holder i/c. steel box, Jalli and energy saver or tube light 2’-0 long rod as approved by Engineer Incharge. | 20 Nos. |  | Each |  |
| TOTAL PART – D Rs. |  |  |  |  |  |

| PART – A | CIVIL WORK (Schedule Item): | Rs. |
| --- | --- | --- |
| PART – B | Non Schedule Items: | Rs. |
| PART – C | Electric Work (Schedule Item): | Rs. |
| PART – D | Electric Work (Non Schedule Items): | Rs. |
| TOTAL |  | Rs. |

TERMS AND CONDITIONS:

- Any typographical errors in the schedule “B” are subject to correction with the reference to the schedule of rates General Item & Water Supply and Sanitary Item 2012 in force from 12th July 2012 publication No. 52 & 54 as approved by the standing rates committee Sindh, Karachi.

- 100% well graded trushed bajri shall be used in item of RCC 1:2:4.

- Water shall be arranged by the contractor at site of work without any extra payment.

- No premium shall be paid on non – Schedule Items.

- No Cartage or any items of material either supplied by the Department or arranged by the contractor shall be paid.

- C.C. Block shall be machine made.

- All RCC/CC cast in situ shall be mechanically viberated.

- Contractor has to bring samples of the material and handed to the Engineer Incharge free of cost.

- All buildings debries & surplus stuff not req. for use and construction shall be removed from the site the suitable disposal off by

- the contractors for which no extra cartages shall be paid.

- The work will be carried out as per PWD Specifications.

| CONTRACTOR | ASSISTANT ENGINEER Provincial Building Sub Division-VII Karachi | EXECUTIVE ENGINEER Provincial Building Division-II Karachi |
| --- | --- | --- |