Name of Work:	M/R TO BARRACK NO. 88 (CONSTRUCTION OF KITCHEN ROOM) SINDH SECRETARIAT BLOCK-4/B KARACHI.

SCHEDULE – B

PART – A	 CIVIL WORK (Schedule Item):

| S/No. | Description | Qty | Rate | Unit | Amount |
| --- | --- | --- | --- | --- | --- |
|  | Excavation in foundation of Building Bridges and other structures including dabbling dressing, refilling around structure with excavated earth Watering and ramming lead upto 5 ft. (b) In ordinary soil. (S.I. 18-b/4). | 11.48 | 3176/25 | % Cft. | 364/63 |
|  |  | (Rupees Three Thousand One Hundred Seventy Six & Twenty Five Paisa Only) |  |  |  |
|  | Cement concrete brick or stone ballast 1 1/2" to 2" gauge (b) Ratio 1 : 4 : 8 (S.I. 4-b/15). | 10.87 Sft. | 9416/28 | % Cft. | 1024/- |
|  |  | (Rupees Nine Thousand Four Hundred Sixteen and Twenty Eight Paisa Only) |  |  |  |
|  | Providing and laying 1:3:6 Cement concrete solid Block masorany set in 1 :6 cement sand mortar in plinth and foundation in including raking out joints and curing etc. complete. (S.I. 22/19). | 21.45 Sft. | 14268/53 | % Cft. | 3061/- |
|  |  | (Rupees Fourteen Thousand Two Hundred Sixty Eight and Fifty Three Paisa Only) |  |  |  |
|  | Providing and laying 1:3:6 Cement concrete solid Block masorany wall above 6" in thickness set in 1 : 6 cement mortar in G.F ground floor superstructure including raking out joints & curing etc, complete. (S.I. 23/19). | 113.50 Cft. | 14621/44 | % Cft. | 16595/- |
|  |  | (Rupees Fourteen Thousand Six Hundred Twenty One & Forty Four Paisa Only) |  |  |  |
|  | Fixing corrugated galvanized iron sheets with G.I bolts, nuts, limpet and bitumen washers, wind ties complete in all respect without valleys and ridges. (b) 22 B.W.G (S.I. 16-b/36). | 4.0 Sft. | 12643/44 | % Sft. | 505/- |
|  |  | (Rupees Twelve Thousand Six Hundred Forty Three and Forty Four Paisa Only) |  |  |  |
|  | Providing and fixing G.I frames /Chowkhats of size 7" x 2" or 4 1/2" x 3" for door using 20 gauge G.I sheet I/c welded hinges and fixing at site with necessary hold fasts, filling with cement sand slurry of ratio 1:6 and repairing the jambs. The cost also i/c all carriage, tools and plants used in making and fixing. (S.I. 29/93). | 16.50 Rft. | 228/90 | P. Rft. | 3776/85 |
|  |  | (Rupees Two Hundred Twenty Eight and Ninety Paisa Only) |  |  |  |
|  | Providing and fixing in position, Doors, Windows and Ventilators of first class deodar wood frames, and 1 3/4" thick commercial , ply veneer shutters of first class deodar skeleton (H ollow ) and commercial ply wood (3 ply ) on both sides. (S.I. 9/58). (1227.36 – 370.83). | 17.50 Sft. | 856/53 | P. Sft. | 14989/30 |
|  |  | (Rupees Eight Hundred Fifty Six and Fifty Three Paisa Only) |  |  |  |

|  | Cement plaster 1:4 upto 12' height. (c) 3/4" thick. (S.I. 11-c/52). | 589.04 Sft. | 3191/76 | % Sft. | 18801/- |
| --- | --- | --- | --- | --- | --- |
|  |  | (Rupees Three Thousand One Hundred Ninety One & Seventy Six Paisa Only) |  |  |  |
|  | Distempering (b) Two coats (S.I. 24-b/54). | 589.04 Sft. | 1043/90 | % Sft. | 6149/- |
|  |  | (Rupees One Thousand Forty Three and Ninety Paisa Only) |  |  |  |
|  | Preparing the surface and painting with matt finish I/c rubbing the surface with Bathy (silicon carbide rubbing brick) filling the voids with zink /chalk / plaster of paris mixture, applying first coat premix, making the surface smooth and then painting 3 coats with matt finish of approved make etc: complete .(new surface). 2nd & subsequent coat. (S.I. 36-a+b/55). | 1404.51 Sft. | 2717/- | % Sft. | 38161/- |
|  |  | (Rupees Two Thousand Seven Hundred Seventeen Only) |  |  |  |
| TOTAL RS. |  |  |  |  | 103427/- |
| Add / Deduct Premium / Below Rs. |  |  |  |  |  |
| TOTAL PART – A Rs. |  |  |  |  |  |

PART – B	Water Supply & Sanitary Items:

| S/No. | Description | Qty | Rate | Unit | Amount |
| --- | --- | --- | --- | --- | --- |
|  | P/F Steel sink stainless steel local make complete with cost iron or wrought iron brackets 6” built in wall 1-1/2” CP bubber plug chrome platted brass chain 1-1/2” CP brass wise with 1-1/2” PVC waste pipe & Making requisite number of holes in wall & plinth & floor for pipe connection & making good in cement concrete 1:2:4 (S.I. 19/6). | 1 No. | 5052/30 | Each | 5052/30 |
|  |  | (Rupees Five Thousand Fifty Two & Thirty Paisa Only) |  |  |  |
|  | S/Fixing long bib- cock of superior quality with c.p head 1/2" dia. (S.I. 13-a/19). | 1 No. | 1109/46 | Each | 1109/46 |
|  |  | (Rupees One Thousand One Hundred Nine and Forty Six Paisa Only) |  |  |  |
|  | Providing and Fixing 6”x2” or 6”x3” C.I. floor trap of the approved self cleaning design with a C.I. screwed down grating with or without a vent arm complete with & i/c. making requisite number of holes in walls, plinth & floor for pipe connections & making good cement concrete 1:2:4 (S.I. 20/6). | 1 No. | 2042/43 | Each | 2042/43 |
|  |  | (Rupees Two Thousand Forty Two and Forty Three Paisa Only) |  |  |  |
|  | P/F Handle valves (Chine ½” dia) (S.I. 5(i)/17). | 1 No. | 200/42 | Each | 200/42 |
|  |  | (Rupees Two Hundred and Forty Two Paisa Only) |  |  |  |
| TOTAL RS. |  |  |  |  | 8404/61 |
| Add / Deduct Premium / Below Rs. |  |  |  |  |  |
| TOTAL PART – B Rs. |  |  |  |  |  |

PART – C	NON – Schedule Items

| S/No. | Description | Qty | Rate | Unit | Amount |
| --- | --- | --- | --- | --- | --- |
|  | P/F UPVC w/s pipe AGM pipe of AGM or Pak Arab make schedule 40e ASTM standard D-1785 without fitting which will be paid separately this i/c. fixing on walls with clamps or jubilee clips upto height of in vertical or horizontal position using jhulloa or long ladder or scaffolding and jointing pipe & fitting withja pproved cement solvent / JTG this i/c. cost of labour material as directed by engineer ¾” dia. | 63.0 Rft. |  | P. Rft. |  |
|  | P/F UPVC pipe 4” dia i/c. fixing jointing with solution pest cutting fixing as required etc. | 28.0 Rft. |  | P. Rft. |  |
| TOTAL PART – C Rs. |  |  |  |  |  |

PART – D	Electric Work (Schedule Items)

| S/No. | Description | Qty | Rate | Unit | Amount |
| --- | --- | --- | --- | --- | --- |
|  | Wiring for light or fan point with 3/.029 PVC insulated wire in 20mm (3/4”) PVC conduit recessed in the wall or columns as required (S.I. 1/123-A). | 2 Nos. | 797/- | P. Point | 1594/- |
|  |  | (Rupees Seven Hundred Ninety Seven Only) |  |  |  |
|  | P/L (Main Sub main) PVC insulated wire size 2-7/.029 copper conductor in ¾” dia PC conduit recessed in the wall or columns as required (S.I. 10/2).) | 10.67 | 222/- | P. Mtr. | 2369/- |
|  |  | (Rupees Two Hundred Twenty Two Only) |  |  |  |
| TOTAL RS. |  |  |  |  | 3963/- |
| Add / Deduct Premium / Below Rs. |  |  |  |  |  |
| TOTAL PART – D Rs. |  |  |  |  |  |

PART – E	NON – Schedule Items

| S/No. | Description | Qty | Rate | Unit | Amount |
| --- | --- | --- | --- | --- | --- |
|  | P/F Energy Saver Bulb Holder | 1 No. |  | Each |  |
|  | P/F Energy Saver Superior Quality i/c. fixing on existing holder etc. complete. | 1 No. |  | Each |  |
|  | S/F Bracket fan best quality Pak Royal make with necessary electric connection & wiring etc. complete. | 4 Nos. |  | Each |  |
|  | P/F Tube light complete with 40 watt 4-0 long rod chock starter with Philips components i/c. necessary electric connection and fixing on wall ceiling etc. complete. | 12 Nos. |  | Each |  |
|  | S/F Chock of 220 volts fitted on ceiling light upto required height as directed by Engineer Incharge (a) 40 watt | 4 Nos. |  | Each |  |
|  | P/F Power Plug i/c. fixing and jointing with connection etc. | 3 Nos. |  | Each |  |
|  | P/F Fancy type 3 Pin 5 Amps SP Plug Socket switch & Shoe unit imported qualified with board i/c. necessary connection and recessed in the wall etc. | 2 Nos. |  | Each |  |
| TOTAL PART – E Rs. |  |  |  |  |  |

| PART – A | CIVIL WORK (Schedule Item): | Rs. |
| --- | --- | --- |
| PART – B | W/S & S/F (Schedule Item): | Rs. |
| PART – C | Non Schedule Items: | Rs. |
| PART – D | Electric Work (Schedule Item): | Rs. |
| PART – E | Non Schedule Items: | Rs. |
| TOTAL |  | Rs. |

TERMS AND CONDITIONS:

- Any typographical errors in the schedule “B” are subject to correction with the reference to the schedule of rates General Item & Water Supply and Sanitary Item 2012 in force from 12th July 2012 publication No. 52 & 54 as approved by the standing rates committee Sindh, Karachi.

- 100% well graded trushed bajri shall be used in item of RCC 1:2:4.

- Water shall be arranged by the contractor at site of work without any extra payment.

- No premium shall be paid on non – Schedule Items.

- No Cartage or any items of material either supplied by the Department or arranged by the contractor shall be paid.

- C.C. Block shall be machine made.

- All RCC/CC cast in situ shall be mechanically viberated.

- Contractor has to bring samples of the material and handed to the Engineer Incharge free of cost.

- All buildings debries & surplus stuff not req. for use and construction shall be removed from the site the suitable disposal off by

- the contractors for which no extra cartages shall be paid.

- The work will be carried out as per PWD Specifications.

| CONTRACTOR | ASSISTANT ENGINEER Provincial Building Sub Division-VII Karachi | EXECUTIVE ENGINEER Provincial Building Division-II Karachi |
| --- | --- | --- |