| NAME OR WORK :- | M/R TO SINDH SECRETARIAT NO.II ( TUGHLAQ HOUSE ) KARACHI (RELAYING OF SEWERAGE LINE AT BACK STREET IN/C ENVICRETE FLOORING. ) |
| --- | --- |

“ SCHEDULE ‘B’ “

| S.# | DESCRIPTION OF ITEMS | QTY: | RATE | UNIT | AMOUNT |
| --- | --- | --- | --- | --- | --- |
|  | SCHEDULE ITEMS PART-“A” ( CIVIL WORK ) |  |  |  |  |
| 1. | P/F cement paving blocks flooring having size 197x97x60 (mm) of city / quddra / cobble shape with having strength b/w 5000 psi to 8500 psi in/c filling the joints with hill sand and laying in specified manner / pattern and design etc complete. (SI-71/P-49) | 3040.00 Sft | 199/77 | P/Sft | 607300/80 |
|  | Rupees (One hundred Ninety Nine & Ps. Seventy Seven) only. |  |  |  |  |
|  |  | Total Rs. |  |  | 607300/80 |
|  | Less % below .Rs. (-) |  |  |  |  |
|  | (A). Total Rs. |  |  |  |  |
|  | SCHEDULE ITEMS PART-“B” (Sanitary works) |  |  |  |  |
| 1 | Providing RCC pipe with collar of class *B* and digging the trenches to required depth &fixing inposition including cutting, fitting & jointing with maxphalt composition & cement mortar 1:1 and testing with water pressure to a head of 4 feet above the top of the highest pipe & refilling with excavated staff (e) 12” dia R.C.C. pie class “B” (SI.2-e/P-24) . | 330.00 Rft | 401/97 | P/Sft | 132650/- |
|  | Rupees. Four hundred & one. And. Ninety seven. Paisa |  |  |  |  |
|  |  | Total Rs. |  |  | 132650/- |
|  | Add: % above .Rs. (+) |  |  |  |  |
|  | (B). Total Rs. |  |  |  |  |
|  | NON-SCHEDULE ITEMS PART-“C” |  |  |  |  |
| 2 | P/F RCC main hole cover of 3” thickness embedded with angle iron frame 1-1/2”x1-1/2”x1-1/4” and bars of 3/8” dia mild steel placed at 6” i/c both way i/c handle in centre for lifting poured with C.C. 1:2:4 in/c curing, finishing etc complete. | 72.00 Sft |  | P/Sft |  |
| 3 | Construction of manhole or inspection chamber 3’-0x3’-0 & upto 4’-0” inside dimension with RCC cover for required dia of square sewer with walls of solid block masonry 1:3:6 set in 1:6 cement sand mortar 6” thick with 6” thick C.C. 1:4:8 in foundation, ½” thick plaster in 1:4 ratio to all inside wall surface & to top 1” thick C.C. flooring etc including, curing, excavation, back filling & disposal of surplus earth as directed by the Engineer Incharge. | 12 Nos |  | Each. |  |
|  |  | Total Part-“C” Rs. |  |  |  |
|  |  | Total Part-“B” Rs. |  |  |  |
|  |  | Total Part-“A” Rs. |  |  |  |
|  |  | G: Total Part ( A+B+C ) Rs. |  |  |  |
|  |  | Say Rs. |  |  |  |

CONDITION.

1).	Typographical errors in the schedule-“B” are the subject to the correction with reference to the schedule of rate ( Gen: Vol: III, Part-II, inforced: 2012, as approved by the standing rates committee.

2).	No Cartage will be paid on any items of works.

3).	No premium shall be paid on Non-Schedule items.

4).	Water shall be arranged by the Contractor.

CONTRACTOR.

| ASSISTANT ENGINEER PROVINCIAL BUILDINGS SUB-DIVISION-V, KARACHI. | EXECUTIVE ENGINEER PROVINCIAL BUILDINGS DIVISION-II, KARACHI. |
| --- | --- |