| NAME OR WORK :- | M/R TO SINDH SECRRETARIAT BUILDING NO-III (OLD KDA BUIDING) KARACHI (RENOVATION / RE-CONSTRUCTION OF POLICE GUARD ROOM, PWD ENQUIRY WITH PRE-CAST SLAB @ GATE # 2 P/F PAVING BLOCKS @ BACK SIDE, MARKING LAYING OF SEWERAGE LINE, & OTHER WORKS). |
| --- | --- |

“ SCHEDULE ‘B’  “

| S.# | DESCRIPTION OF ITEMS | QTY: | RATE | UNIT | AMOUNT |
| --- | --- | --- | --- | --- | --- |
|  | SCHEDULE ITEMS PART-“A” |  |  |  |  |
| 1. | Providing and laying 1: 3: 6 cement concrete solid block masonry wall 6” and below in thickness set in 1:6 cement mortar in ground floor Super Structure including raking out joints & curring etc, complete.( SI.24 / P-19 ). | 195.50-Cft | 15771/01 | % Cft | 30832/- |
|  | ( Rupees Fifteen Thousand Seven Hundred Seventy One & One Paisa ) Only. |  |  |  |  |
| 2 | Cement plaster 1:4 upto 12' height. (d) 3/8" thick.(only for Block masonry) ( SI.11(d)/P-52 ). | 872.00-Sft | 3191/76 | %Sft | 27832/- |
|  | Rupees. Three thousand one hundred ninety one. And. Seventy six. Paisa |  |  |  |  |
| 3 | Providing and fixing G.I frames /Chowkats of size 7" x 2" or 4 1/2" x 3" for door using 20 gauge G.I sheet in/c welded hinges and fixing at site with necessary hold fasts, filling with cement sand slurry of ratio 1:6 and repairing the jambs. The cost also i/c all carriage , tools and plants used in making and fixing.( SI.29/P-93 ). | 17.00 Rft | 228/90 | P.Rft | 3891/- |
|  | Rupees. Two hundred twenty eight. And. Ninety. Paisa |  |  |  |  |
| 4 | Providing and fixing in position doors, windows and ventilators of 1st class deodar wood frames and 1-1/2” thick Teak wood ply shutters of 2nd class deodar wood skeleton ( solid ) styles and rails core of partal wood and commercial ply wood ( 3-Ply ) on both side in/c hold fasts, hinges, Iron tower bolts, handles and cleats with cord etc complete. ( SI.58 / P-65 ) – ( SI.25-b / P-61 ) Rs.1077/06 ( - ) Rs.370/83 = 706/23 | 58.57-Sft | 706/23 | P.Sft | 41364/- |
|  | Rupees. Seven hundred & six. And. Twenty three. Paisa |  |  |  |  |
| 5 | Priming coat of chalk distemper. ( SI.23 / P-54 ). | 275.00 Sft | 442/75 | %Sft | 1218/- |
|  | Rupees. Four hundred forty two. And. Seventy five. Paisa |  |  |  |  |
| 6 | Preparing the surface and painting with matt finish paint of approved make to old matt finish ( b ). 2nd & subsequent coat. ( SI.37(A+B) / P-55 ). | 830.00 Sft | 1772/38 | %Sft | 14711/- |
|  | Rupees. One thousand seven hundred seventy two. And. Thirty eight. Paisa |  |  |  |  |
| 7 | Distempering.( b ) Two coats.( SI.24(b) / P-54 ). | 304.00 Sft | 1043/90 | %Sft | 3173/- |
|  | Rupees. One thousand & forty eighty three. And. Ninety. Paisa |  |  |  |  |
| 8 | Painting old new surface (c ) Painting doors and windows any type. (i) 1st coat (ii) second & subsequent coat ( SI.4( c ) ( i + ii )/P-68 ). | 184.15-Sft | 1160/06 | %Sft | 2136/- |
|  | Rupees. One thousand one hundred sixty. And. Six. Paisa |  |  |  |  |
| 9 | Providing and laying tiles glazed 6" x 6" x 1/4" on floor or wall facing in required colour and apttern of STILE specification jointed in white cement and pigment over a base of 1:2 grey cement mortar 3/4" thick including washing and filling of joints with slurry of white cement and pigment in desired shape with finishing, clearing and cost of wax polish etc. complete including cutting tiles to proper profile. ( SI.60/P-47 ). | 204.00 Sft | 30509/77 | %Sft | 62240/- |
|  | Rupees. Thirty thousand five hundred & nine. And. Seventy seven. Paisa |  |  |  |  |
| 10 | P/F cement paving blocks flooring having size 197x97x60 (mm) of city / quddra / cobble shape with having strength b/w 5000 psi to 8500 psi in/c filling the joints with hill sand and laying in specified manner / pattern and design etc complete. (-71/P-49) | 3233.00 Sft | 199/77 | P/Sft. | 645856/- |
|  | Rupees. One hundred ninety nine. And. Seventy seven. Paisa |  |  |  |  |
|  |  | Total Rs: |  |  | 833252/- |
|  | Less: % below Item No. 4 ,9 & 10 (41364 + 62240 + 645856 =749460/-) ( - ) Rs: |  |  |  |  |
|  |  | (A). Total Rs. |  |  |  |
|  | PART ‘B’ ELECTRIC ITEMS. |  |  |  |  |
| 1 | Wiring for light or fan point with 3/.029 PVC insulated wire in 20mm ( ¾” ) PVC conduit on surface as required. ( SI.123 / P-15 ) | 07-Nos | 797/- | P.Point | 5579/- |
|  | Rupees. Seven hundred ninety seven. Only. |  |  |  |  |
| 2 | Providing & fixing one way SP 10 / 15amp switch surface type.( SI.218 / P-33 ). | 18-Nos | 58/- | Each | 1044/- |
|  | Rupees. Fifty eight. Only. |  |  |  |  |
|  |  |  | (B). Total Rs. |  | 6623/- |
|  | SCHEDULE ITEMS PART-“C” (Sanitary works/PHE) |  |  |  |  |
| 1 | Providing RCC pipe with collar of class *B* and digging the trenches to required depth &fixing inposition including cutting, fitting & jointing with maxphalt composition & cement mortar 1:1 and testing with water pressure to a head of 4 feet above the top of the highest pipe & refilling with excavated staff (e) 6” dia R.C.C. pie class “B” (SI.2-d/P-24) . | 25.00 Rft | 199/25 | P/Rft | 4981/- |
|  | Rupees. One hundred ninety nine. And. Twenty five. Paisa |  |  |  |  |
| 2 | Constructing manhole or inspection chamber for the required diameter of circular sewer and 3’6” (1067mm) depth with walls of B-B in cement sand mortar 1:3 cement plastered 1:3 ½” thick inside of walls and 1” (25mm) thick over benching and channel in/c fixing CI manhole cover with frame of clear opening 1-1/2”x1-1/2” (457x457mm) of 1.75 cwt (88.9 kg) embedded in plain C.C. 1:2:4 and fixing 1” (25mm) dia M.S. steps 6” (150mm) wide projecting 4” (102mm) from the face of wall at 12” (305mm) C/C duly painted etc complete as per standard specification and drawing. (a) 4” to 12” dia 2’x2’x3’x6” (PHE SI.1(a)/P-46) | 06 Nos | 7191/50 | Each. | 43149/- |
|  | Rupees. Seven thousand one hundred ninety one. And. Fifty. Paisa |  |  |  |  |
| 3 | P/F Precast Edge Block 3750 PSI industrial Made Size 6 inches thick x 12 inches long x 12 inches high including the cost of cartage excavation from work for haunching 1450 PSI lean concrete 2250 PSI concrete for haunching 1:4 cement sand mortar (Highway SI.14/P-16) | 132.00 Rft. | 297/01 | P/Sft. | 39205/- |
|  | Rupees. Two hundred ninety seven. and. One. Paisa |  |  |  |  |
|  | Total Rs: |  |  |  | 87335/- |
|  | Add: % above Item No. 1 ( + ) Rs: |  |  |  |  |
|  | Add: % above Item No. 2 ( + ) Rs: |  |  |  |  |
|  | (C). Total Rs. |  |  |  |  |
|  | NON-SCHEDULE ITEMS PART-“D” |  |  |  |  |
| 1 | P/L Pre cast RCC beam / slab roof by using pre coat beams of required size & no and precast slab of size 4.0x2.0x0.25 after leveling of wall by CC erection of beams & slab, filling joints in cement sand mortar as directed by the Engineer Incharge. | 348.00 Sft- |  | P.Sft |  |
| 2 | P/F ceiling fan 56” sweep Pak/Royal/GFC or equivalent Deluxe model in/c carriage from shop to site of work, fixing & necessary lead connection as directed by the Engineer Incharge. | 02-No |  | Each |  |
| 3 | S/F 1-40 watts tube light with 40 watts rod, chock, starter & base fitted on wall ceiling with 23/.076 flexible wire in/c connection to ceiling rose as directed by the Engineer Incharge.(Phillips make ) or equivalent | 04 Nos |  | Each |  |
| 4 | Supplying and fixing in position Aluminum channels framing for hinged doors of Lucky or equivalent made with 5mm thick tinted glass glazing (Belgium) and ALPHA (Japan) locks in/c handles, stoppers etc complete. (b) Deluxe Model (Bronze of Champion color) | 24.00 Sft. |  | P/Sft. |  |
| 7 | P/F RCC main hole cover of 3” thickness embedded with angle iron frame 1-1/2”x1-1/2”x1-1/4” and bars of 3/8” dia mild steel placed at 6” i/c both way i/c handle in centre for lifting poured with C.C. 1:2:4 in/c curing, finishing etc complete. | 48.00 Sft |  | P/Sft |  |
| 9 | P/F Iron Steel grill using solid square bars of size 1/2" x 1/2” placed @ 4” centre to centre in approved design and pattern in/c frame of flat iron Patti of 3/4"x1/8” fixed with screws or pins and painting with two or required coats after first coat of red oxide. The cost in/c all cartage, tools, plants, welding machine etc used in making as directed by the Engineer Incharge. | 130.00-Sft |  | PSft |  |
|  |  | Total Part-“D” Rs. |  |  |  |
|  |  | Total Part-“C” Rs. |  |  |  |
|  |  | Total Part-“B” Rs. |  |  |  |
|  |  | Total Part-“A” Rs. |  |  |  |
|  |  | G: Total Part ( A+B+C+D ) Rs. |  |  |  |
|  |  | Say Rs. |  |  |  |

CONDITION.

1).	Typographical errors in the schedule-“B” are the subject to the correction with reference to the schedule of rate ( Gen: Vol: III, Part-II, inforced: 2012, as approved by the standing rates committee.

2).	No Cartage will be paid on any items of works.

3).	No premium shall be paid on Non-Schedule items

4).	Water shall be arranged by the Contractor.

CONTRACTOR.

| ASSISTANT ENGINEER PROVINCIAL BUILDINGS SUB-DIVISION-V, KARACHI. | EXECUTIVE ENGINEER PROVINCIAL BUILDINGS DIVISION-II, KARACHI. |
| --- | --- |