NAME OF WORK:       M/R TO 182 FLATS AT SHADMAN TOWN KARACHI.

(P/L C.C. ROAD AND PATH)

SCHEDULE  “ B “

S.NO. ITEM OF WORK. QUANTITY. RATE. UNIT. AMOUNT.

1).Dismantling of  existing asphalt  cement  road

Including  base  sub base and disposal of

Dismantled material to designated  placed      4840.00-Sft	773/86	     %Sft	37455/-

(S.I.NO.11/P-15)(Public Health).

2). Earth work compaction (Soft, ordinary or

hard soil Laying earth in 6” layers leveling

dressing and Watering for compaction etc

complete (13-b P-3).			        2511.00-Cft  	354/-	      %oCft	 889/-

3)  cement concrete brick or stone ballast

1-1/2” to 2” gauge (b) Ratio 1:4:8

(4-b P/15).					        1848.00-Cft	9416/28      %Cft	 174013/-

4)  P/L aggregate base course in proper grade

and camber having CBR 80% as per AASHTO

standard specifications in/c spreading and

compacting by approval mechanical

means (motor grader, vibratory roller and

smooth wheel roller etc) watering to maintain

the moisture content the compaction of each

layer shall 100 percent to the max dry density

(Rate in/c all cost of materials T&P and carriage

upto 3-chains) (13-B P/5)(public Health).

2420.00-Cft      5542/24      %Cft	134122/-

5)  Applying priming coat of back coat with

approved binder at the required rate in/c

cleaning the road surface thoroughly, heating

to the required temperature and spraying the

binder with pressure as directed etc complete

(31 P-12)(P.H).

4840.00-Sft	64/96	      %Sft	 3144/-

6)  1-1/2” Carpet laying with pave3r Machine.

Providing and laying to proper line and grade

1-1/2” thick premix carpet with paver machine,

prepared to specified formula according to job

mix formula approved by Engineer incharge

in/c rolling and finishing to proper line, grade

level and camber etc  (Rate all cost of materials,

T&P and carriage upto   3-chains).(S.I.NO.20/P-8(P.H

4840.00-Sft         6488/32      %Sft               314035/-

7).P/L 1” thick topping cement concrete (1:2:4

in/c surface finishing and dividing into panels

© 2” thick.(16-c P/42).			 5021.08-Sft       3275/50      %Sft	164465/-

Total Rs:     828,123/-

Add:              %     Total Rs:

G. Total Rs:

Terms and Conditions:

- Any typographical errors in the schedule ‘B’ are subjected to any correction with the reference to the schedule of Rates General item, 2012 in force from 12-07-2012 as approved by the standing rate committee Sindh Karachi.

- No Cartage will be paid on any items of works.

- No premium shall be paid on Non-Schedule items.

.

--s/d--					--s/d--

CONTRACTOR		  ASSISTANT ENGINEER			EXECUTIVE ENGINEER

Electrical Sub Division No.IV		           Provincial Building Division-III

Karachi.				             Karachi.