SCHEDULE B.

NAME OF WORK: -     M/R to VIP Rest House At Shireen Jinnah Colony Karachi (Sevrage line).

| S.NO | DESCRAPTION OF ITEMS | QUTY | RATE | UNIT | AMOUNT |
| --- | --- | --- | --- | --- | --- |
| 1 | Providing chamber 15”x19” (inside dimension )x24’ deep for house meters with 6” thick CC 1;3;6. Block set on 1;6. Cement 6” thick CC 1:4:8 in foundation ½” thick cement plaster 1:3. To all morotr inside wall surface and to top 1’ thick c.c. 1:2:4 flooring complete ( s.i.no. 2/20). | 12-nos. | 5913/22 | Each | 70959 |
| 2 | Providing and laying 1.3.6. cement concrete solid block masonary wall6” and below thickness set in `1.6. cement mortar in ground floor super structure including raking out joints and curing etc complete(s.i.no.22/19) | 191.25 cft | 15771/01 | % cft | 30162 |
| 3 | Cement plaster 1.4. upto 20’ height 3/4 ” thick ( s.i.No.11-c/52) | 510.0 | 3015/76 | %Sft | 15380 |
| 4 | P/l 1” thick topping c.c. 1:2:4 i/c surface finishing & dividing into panels 3’ thick(S.I.16-c/p-42) | 130.0 Sft | 4411/82 | %Sft | 5735 |
| 5 | Distempering three coats ( s.i.No.24-c/520. | 640.0 | 1079/65 | % sft | 6910 |
| 6 | Asbestoers cement corrugated sheet roofing i/c necessary over laps and GI hook bolts nuts screwes washers and bitumen washers etc (excluding valleys and ridges etc) (S.I.28/37). | 130.0 sft | 12141/25 | % sft | 15784 |
|  | Total |  |  |  | 144930 |
|  | Add, % above on item No. 1. |  |  |  |  |
|  | A |  |  |  |  |
|  | PART B, N.S.I. |  |  |  |  |
| 1 | P/f UPVC soil and vent pipe of pak Arab make of approved quality on wall upto height of 50’ ft, with plant i/c clamps paid separately this rate i/c making joints upvc fitting by using approved pest solution (AGM) make etc, complete. |  |  |  |  |
| (i) | -do- ½” dia | 50.0 |  | P. Rft. |  |
| (ii) | -do- ¾” dia | 80.0 |  | -do- |  |
| (iii) | -do- 4” dia (RAS ) | 90.0 |  | -do- |  |
| (iv) | -do- 3” dia (RAS ) | 90.0 |  | -do- |  |
| 2 | P/F UPVC fitting of pak arab of approved quality on wall upto 50 ft, with plastic clamp paid separately this also i/c cutting making jointing with switch pest with special approved quality i/c all cost labour etc, complete. |  |  |  |  |
|  | -do- 4” dia upvc plug tee (RAS) | 3-nos |  | Each |  |
| (ii) | -do- 4” dia upvc plain bend (RAS ) | 6-nos |  |  |  |
| (iii) | 4” dia upvc Y-Tee | 3-Nos |  |  |  |
| (iv) | 4” dia cowel (RAS ) | 2-Nos. |  |  |  |
| (v) | 3” dia upvc plug Tee (RAS ) | 3-nos |  |  |  |
| (vi) | 3” dia upvc plug bend (RAS ) | 3-nos |  |  |  |
| (vii) | 3” dia upvc Y-Tee | 3-Nos |  |  |  |
| (viii) | 3” dia cowel (RAS ) | 2-nos |  |  |  |
| 3. | P/f UPVC pipe pak arab make etc, complete 14” dia with one side collar and jointing with solution and packing rate i/c cost of material labour and cartage etc, complete. | 300.0 Rft |  |  |  |
| 4 | P/L UPVC pipe schedule 40 pak arab make 6” dia with one side collar and jointing with solution and packing rate i/c cost material labour and cartage etc, complete. | 72.0 Rft |  |  |  |
| 5 | Construction manhole for inspection chamber for the required diamension of circular 3’-6’ (1067)mm depth of BB in sand mortor etc, complete. | 20-nos. |  |  |  |
| 6 | P/f RCC man hole cover of 3’ thickness embedded with angle iron frame 1-1/2”x 1-1/2”x1-1/4” and bars of 3/8’ dia mild steel placed at 6’ i/c both way i/c handle in center for lifting poured with c.c. 1:2:4. Etc, complete. | 180.0 Sft |  |  |  |
| 7 | P/A weather shiled approved shade upto 40’-0ft, height using scaffolding i/c craping best coat primer and coats etc, complete, the rate i/c cost of material labour and cartage etc, complete. | 11700.0 |  |  |  |
| 8 | P/F National tiles 5’ x 10” approved colour and design overs ¾” cement martor bed cutting fitting curing etc, complete. | 180.0 |  | P. sft. |  |

TERMS AND CONDTION.

- Any typographical errors in the schedule B are subject to the correction with Reference to the Schedule of rate General 2012 enforced from 12.07.2012 as Approved by the S R C, Sindh Karachi

- No premium shall be paid on a Non schedule items.

- 100% WELL Graded crushed bajri shall be used in items of work without any extra payment

4.   No Any items of exvceed then the Quantity Provided in the Schedule B without Prior of the Executive Engineer.

5.   All sample of items should be got approved by the Executive Engineer.

.

CONTRACTOR       SUB ENGINEER                         ASSISTANT ENGINEER

PROVINCIAL BUILDINGS SUB DIVISION-I

KARACHI.