# BILL OF QUANTITIES NO-1

# BORING & INSTALLATION OF 12 NOS. NEW TUBE WELL OF 2.0 CUSECS (HIGH SALINE) IN TUBEWELL DIVISION KHAIRPUR

| # S# | # Item | # Quantity | # Rate |  | # Unit | # Amount |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | # Figures | # In Words |  |  |
| # 1. | # Drilling of 22” diameter for Tubewell up to depth of 210 Rft | # 2400 |  |  | # Rft |  |
| # 2. | # Furnishing and Installation FG pipe housing casing ¼” thick 14” dia | # 900 |  |  | # Rft |  |
| # 3. | # Furnishing and Installation Fiber Glass casing filter 8” dia 4.5 mm thick | # 1320 |  |  | # Rft |  |
| # 4. | # Furnishing and Installing Fiber Glass casing blank 8” dia | # 180 |  |  | # Rft |  |
| # 5. | # Furnishing and Installing FG Reducer 16”x8” dia | # 12 |  |  | # No. |  |
| # 6. | # Furnishing and placing Gravel Shrouding granular space as per specification | # 4848 |  |  | # Cft |  |
| # 7. | # Development and testing as per specification | # 12 |  |  | # Job |  |
| # 8. | # Furnishing and Installing KSB/equaling vertical Turbine Pump Type B-12 B/2 Stages of 2.00 cusecs (High Saline) or Equivalent and 60 ft head with column pipe 7” dia (Internal) and column shaft 25 mm dia suction and delivery pipe 7” dia all KSB make (High Saline) installed with vertical hallow shaft motor itio20 HP Motor Siemens make 1450 RPM squirrel gage induction motor i/c all fitting and fixtures discharge head Bowl assembly etc. complete i/c testing for discharge (High Saline) with Motor Control Unit of 20 HP Motor. | # 12 |  |  | # No. |  |
| # 9. | # Making Earth Connection | # 12 Job |  |  | # Job |  |
| # 10. | # Final Acceptance Test | # 12 Job |  |  | # Job |  |
| # 11. | # P/F Bail Plug | # 12 No. |  |  | # No |  |
| # 12. | # P/F Centralizer | # 60 Nos. |  |  | # No |  |
| # TOTAL |  |  |  |  |  |  |

# CONTRACTOR

# EXECUTIVE ENGINEER

# TUBEWELL DIVISION

# KHAIRPUR

# BILL OF QUANTITIES NO.VI

# WORK FOR REPLACEMENT OF OLD PUMPS,MOTORS AND PANELS FOR 26 NOS. SUMPS/PUMPS IN SURFACE DRAINAGE DIVISION KHAIRPUR

| # S# | # Item | # Quantity | # Rate |  | # Unit | # Amount |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | # Figures | # In Words |  |  |
| # 1. | # Furnishing and installation of # KSB/Equivalent vertical Turbine Pump type HBP 110-1, 1.00 cusec (High Saline with column pipe 6” dia 10 HP motor SIEMENS 1450 RPM required gauge induction motor i/c all fitting and fixture discharge head bowel assembly etc complete with motor control unit of 10 HP motor. | # 46 Nos. |  |  | # Each |  |
| # 2. | # Furnishing and installation of # KSB/Equivalent vertical Turbine Pump type HBP125-1 of 1.50 cusec (High Saline with column pipe 8” dia and column shaft 25mm dia delivery pipe 8” dia 20 HP motor SIEMENS 1450 RPM required gauge induction motor i/c all fitting and fixture discharge head bowel assembly etc complete with motor control unit of 15 HP motor.. | # 2 Nos. |  |  | # Each |  |

CONTRACTOR

# EXECUTIVE ENGINEER

# TUBEWELL DIVISION

KHAIRPUR

# BILL OF QUANTITIES NO-V

# WORK FOR REPLACEMENT OF OLD PUMPS, MOTORS AND PANELS PUMPING STATIONS A

| # S# | # ITEM | # Quantity | # Rate |  | # Unit | # Amount |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | # Figures | # In Words |  |  |
| # 1. | # Supplying and installation of vertical tabular casing pump PNW 500-470 capacity 25 cusecs 20’ ft head coupled with 100 HP Motor i/c Motor Control unit and 4 core 120 mm cable and 50’ ft length. The pump assembly i/c ever clean blade G-CU AL-Ni lead free material of proper design blade, anti vortex vanes reside bearing medium lubricated, silicon carbide bearing, over the pump shaft column shaft and drive shaft are coupled with cylindrical sleave coupling. | # 3 No. |  |  | # Eath |  |
| # 2. | # Supplying and fixing 4 core wire 37/083 | # 90 |  |  | # Per Meter |  |

CONTRACTOR

# EXECUTIVE ENGINEER

# TUBEWELL DIVISION

KHAIRPUR