A B S T R A C T

BILL OF QUANTITIES

Pak Rupees

| Sr. No. | Bill Quantity | Item | Bid Cost |
| --- | --- | --- | --- |
| 1. | BOQ-I | Boring and Installation of 12 Nos. New Tubewells of 2.00 cusecs (High Saline) |  |
| 2. | BOQ-II | Pump House, Duty Room and Discharge Box for 12 Nos. Tubewells |  |
| 3. | BOQ-III | Making Direct Disposal Arrangement of 12 Nos. Tubewells |  |
| 4. | BOQ-IV | Replacement of Old Pump, Motor and Panels for 26 Nos. Sump/Pump |  |
| 5. | BOQ-V | Replacement of Old Pump, Motor and Panels of Pumping Station-A |  |
|  |  | TOTAL BID Cost Rs. |  |

(In words__________________________________________________________)

# CONTRACTOR							       EXECUTIVE ENGINEER

# TUBEWELL DIVISION

KHAIRPUR

BILL OF QUANTITIES NO-II

# WORK FOR CONSTRUCTION OF PUMP HOUSE, DUTY ROOM AND DISCHARGE BOX OF 12 NOS. TUBE WELLS IN TUBEWELLS DIVISION KHAIRPUR.

| # S# | # Item | # Quantity | # Rate | # Unit | # Amount |
| --- | --- | --- | --- | --- | --- |
| # 1. | # Excavation in foundation of building, bridges and other structure i/c dressing, refilling around structure with excavated earth watering and ramming lead up to one chain and lift up to 5 ft. # b) in ordinary soil | # 270 | # 3176.25 | # %0 Cft | # 858 |
| # 2. | # Cement Concrete i/c placing compacting finishing and curing complete (i/c screening and washing of stone aggregate without shuttering) # Ratio 1:4:8 | # 126 | # 11288.75 | # % Cft | # 14224 |
| # 3. | # Pacca brick work in foundation and plinth # Cement sand mortar ratio 1:5 | # 195 | # 12176.05 | # % Cft | # 23743 |
| # 4. | # Damp proof course with cement sand shingle concrete (1:2:4) include two coats of asphaltic mixture # c) 3” thick | # 51 | # 4982.18 | # % Sft | # 2541 |
| # 5. | # Pacca brick work in ground floor in cement sand mortar 1:5 including stricking of joints | # 290 | # 12902.08 | # % Cft | # 37416 |
| # 6. | # Reinforced cement concrete work i/c all labour and material except the cost of steel reinforcement and its labour for bending and binding which will be paid separately. This rate also includes all kinds of forms, molds, lifting shuttering, curring, rendering and finishing and exposed surface (i/c screening and washing of shingle) # a) R.C.C work in roof, slab, beams coluns, rafts, lintels and other structural members laid in site or precase laid in position. # i) Ratio 1:2:4 90Lbs: cement sand 4 Cft: shingle 1/8" to 1/4" # (GSS-2012, P-16/I-5(a) | # 63 | # 337 | # Cft | # 21231 |
| # 7. | # Fabrication of mild steel reinforcement for cement concrete i/c cutting, bending laying in position making joints and fastenings i/c cost of binding wire (also includes removal of rust from bars. Using Tor Bars # (GSS-2012, P-16/I-8(a) | # 3.87 Cwt | # 5001.70 | # Cwt | # 19354 |

| # S# | # Item | # Quantity | # Rate | # Unit | # Amount |
| --- | --- | --- | --- | --- | --- |
| # 8. | # Filling watering and ramming earth under floor with surplus earth from foundation lead up to 50’ ft # Floor | # 126 Cft | # 1512.50 | # % Cft | # 1906 |
| # 9. | # Providing and lying 1” thick topping cement concrete 1:2:4 ratio including surface finishing and dividing into panels. | # 108 | # 4411.82 | # % Sft | # 4765 |
| # 10. | # Making and fixing Steel grated door with 1/16” thick sheeting including angle iron frame 2”x2”x3/8” and ¾” square bars 4” c/c with locking arrangement. # (GSS-2012, P-91/I-24 | # 24.50 | # 726.72 | # Sft | # 17805 |
| # 11. | # Cement Plaster up to 20’ height 1:3 # b) ½” thick | # 564.50 | # 2344.59 | # % Sft | # 13235 |
| # 12. | # Cement Pointing struck joints on walls # Ratio 1:2 | # 528.50 | # 1287.44 | # % Sft | # 6804 |
| # 13. | # Painting new surfaces # Preparing surface and painting of doors and windows any type (i/c edges ) 3 coats | # 49 | # 2116.41 | # % Sft | # 1037 |
| # 14.0 | # White washing 3 coats # Inner side walls # Slab | # 528 | # 829.95 | # % Sft | # 4380 |
| # 15. | # Khassi parnala in cement and sand mortar (1:2) 12” out side width finished smooth with a floating coat of neat cement. | # 10.42 | # 70.34 | # Rft | # 733 |
| # 16. | # Bottom Khurass of brick masonry in cement mortar (1:6) 4’x2’x41/2” over 3” cement concrete (1:4:8) | # 1 | # 248.66 | # Each | # 249 |
| # 17. | # Color washing # Two coats | # 528.5 | # 859.9 | # % Sft | # 4545 |
| # 18. | # Second class tile roofing consisting of 4” earth and 1” mud plaster with gobri leaping over ½” thick cement plaster 1:6 with 24 lbs of hot bitumen coating sand blinded provided over 1 layer of 12”x6”x2” tiles laid in 1:6 cement pointing underneath of tiles complete including curing etc. | # 108 | # 7607.25 | # % Sft | # 8216 |
| # Total |  |  |  |  | # 183042 |
| # 183042 X 12 Nos. Tubewells = |  |  |  |  | # 2196504 |
| # Add ____ % above or below on schedule-A |  |  |  |  |  |
| # Total Bid cost Rs. |  |  |  |  |  |

CONTRACTOR

# EXECUTIVE ENGINEER

# TUBEWELL DIVISION

# KHAIRPUR

BILL OF QUANTITIES NO-III

# WORK FOR MAKING DIRECT DISPOSAL ARRANGEMENT OF 12 NOS. TUBEWELLS IN TUBEWELL DIVISION KHAIRPUR

| # S# | # Item | # Quantity | # Rate | # Unit | # Amount |
| --- | --- | --- | --- | --- | --- |
| # 1. | # Excavation for pipe line in trenches and pits in soft soil including trimming and dressing sides to true alignment and shape leveling of beds of trenches to correct level and grade, cutting, joints, holes and disposal of surplus earth within one chain as directed by Engineer incharge. # Providing fence guards lights flags and temporary crossing for non vehicular traffic wherever required lift up to 5’ ft (1.52 m) and lead up to one chain 30.50 m # (GSS- 2012) PHED V-III P-III PAGE 61 I/NO.4(1) | # 17280 | # 3600 | # %0 Cft | # 62208 |
| # 2. | # Providing, laying and fixing in trenches i/c cutting jointing and testing etc complete in all respect the high density Poly Ethylene PE pipes (HDPE-100) for W/S confirming ISO 4427/DIN8075 B.S 3580 and PSI 3051.(GSS-PHED-2012) 10” dia schedule of rates V-III P-III Item No.F(0) Page 25 PN-08 | # 1920 | # 1043 | # Rft | # 2002560 |
| # 3. | # Refilling the excavated stuff in trenches 6” thick layer i/c watering ramming to full compaction etc. complete. # (GSS) PHED V-IV P-IV page 77 I/No.24 | # 12240 | # 2760 | # %0 Cft | # 44822 |
| # 4. | # Supplying and fixing M.S Pipe 8” dia PHES 2011/P-30/Item-I(11)Vol-iv Part-vi | # 110 | # 2363.71 | # Rft | # 260008 |
|  |  |  |  | # TOTAL | # 2369598 |
| # 2369598 x 12 = |  |  |  |  | # 28435176 |
| # Add ____ % above or below on schedule-A |  |  |  |  |  |
| # Total Bid cost Rs. |  |  |  |  |  |

CONTRACTOR

# EXECUTIVE ENGINEER

# TUBEWELL DIVISION

# KHAIRPUR