| OFFICE OF THE EXECUTIVE ENGINEER (B&R) DMC (WEST) BALDIA ZONE |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ESTIMATE COST |  | 9,51,063/- | TIME LIMIT |  |  | 30 Days |  |  |
| TENDER FEES |  | 1500/- | PANELTY PER DAY |  |  | 1500/- |  |  |
| SUBJECT: - P/L 2” Dia R.C.C Pipe drains at Hussani Nager in UC-08 DMC (W) Baldia Zone. |  |  |  |  |  |  |  |  |
| S.NO | DISCREPTION |  |  | QTY | RATE |  | PER | AMOUNT |
| 1 | Excavation for pipe line in trenches, and pits in all kind of soil of Murum shape leveling of beds of trenches to correct level and grade, surplus earth within a one chain as directed by Engineers Incharge |  |  | 14850 | 4650 |  | %0Cft | 69053/- |
| 2 | Excavation for pipe line in trenches, and pits in hard rock by hammering and chiseling i/c trimming and dressing sides to true alignment and shape leveling of beds of trenches to correct level and grade cutting joints holes and disposal of surplus earth within a one chain as directed by Engineers Incharge. Providing fence guards. Lights flags and temporary crossings for non-vehicular traffic where ever required lift up to 5ft. (1.52m) and lead upto one chain (30.5m). |  |  | 900 | 24150 |  | %0Cft | 21735/- |
| 3 | P/L reinforced cement concrete pipes with rubber rings i/c cartage, loading unloading at site of work fixing in trenches cutting fitting, jointing with rubber rings etc complete KWSB Approved Rate, M/S Balochistan R.C.C Pipe Co. M/S Pakistan construction (Pvt.) Ltd. M/S Kamal R.C.C Pipe Co. 12” dia pipe. |  |  | 1100 | 367 |  | %/Rft | 403700/- |
| 4 | Construction of 4” dia manhole inside complete with 21”dia c c 1:2:4 pre cast C.I frame 12kg i/c the cost of M.S bars reinforcement and two handles 5” clear depth c c 1:2:4 cast in situ walls with 50% screened graded bajri of 9” thick i/c bending and channel i/c inside walls surface complete i/c cutting asphalt roads excavation in all kind of soil back filling and disposal of surplus stuff (KW&SB) approved rate |  |  | 46-Nos | 9307 |  | Each | 428162/- |
| 5 | Extra depth beyond 5” ft or less than 5 deducted at the same rate (KWSB) Approved rate |  |  | (-)23-ft | 1609 |  | Pft | (-)37010/- |
| 6 | Refilling excavation stuff in trenches 6” thick layer i/c watering ramming to full compaction etc complete. |  |  | 14175 | 2760 |  | %Cft | 39123/- |
| 7 | Carriage of 100 Cft Tons of all kind material like Cole, brick i/c loading unloading, 10 miles. |  |  | 1575 | 595.80 |  | %Cft | 9384/- |
| 8 | Manufacturing and supplying R.C.C main hole covers cast in situ 1:2:4 C.C. 3” inch deep in center reinforced with 3/8” dia M.S Bars welded on M.S. sheet i/c curing transportation 21” dia 24” dia. |  |  | 05-Nos | 913.63 |  | Each | 4568/- |
| 9 | Manufacturing and supplying of R.C.C Ring Slab of 21” inside 36” dia outside 75 wide and to thick i/c 3/8” dia tor steel bar two concentric rings with 3/8” dia Nos. cross linkes bars welded and two sunk type hook, lasted ina 1:1-1/2:3 concrete with embedded 15kg C.I frame in per feet position i/c terms partition charges for an average lead of 20 km per trip from casting yard to town office (A minimum of 25 slabs per trip will be transported.) |  |  | 05-Nos | 1906.60 |  | Each | 9533/- |
| 10 | Rate analysis for shifting to the site and fixing 26” dia Ring Slab in perfect position on damage manhole i/c cutting of damage portion of mentioned in past shape and lying of cement concrete mortar of 2” thickness and disport. |  |  | 05-Nos | 563.09 |  | Each | 2815/- |

A.E (B&R)							             A.E.E (B&R)

I/We hereby quoted rate@_________% above/below the schedule of rates.

Contractor’s Signature__________________________

Address_____________________________________

___________________________________________