| OFFICE OF THE EXECUTIVE ENGINEER (B&R) DMC (WEST) BALDIA ZONE |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ESTIMATE COST |  | 5,96,424/- | TIME LIMIT |  |  | 30 Days |  |  |
| TENDER COST |  | 1500/- | PANELTY PER DAY |  |  | 1500/- |  |  |
| SUBJECT:- P/L 12” dia RCC Pipe Drain at Waqar Petrol Pump to House No.688 Gali No. 1, Block-D, Gulshan-e-Ghazi in UC-01, DMC (West) Baldia Zone. |  |  |  |  |  |  |  |  |
| S.NO | DISCREPTION |  |  | QTY | RATE |  | PER | AMOUNT |
| 01 | Excavation for pipe line in trenches, and pits in all kind of soil of Murum Shape Leveling of beds of trenches to correct lever & grade, surplus earth within a one chain as directed by Eng incharge. |  |  | 6300-Cft | 4650 |  | %0Cft | 29295/- |
| 02 | Excavation for pipe line in trenches, and pits in hard rock by hammering and chiseling i/c trimming and dressing sides to true alignment and shape leveling of beds of trenches to correct level and grade cutting joints holes and disposal of surplus earth within a one chain as directed by Engineers Incharge. Providing fence guards. Lights flags and temporary crossings for non-vehicular traffic where ever required lift up to 5ft. (1.52m) and lead upto one chain (30.5m) |  |  | 3600-Cft | 24150 |  | %0Cft | 86940/- |
| 03 | P/L reinforced cement concrete pipes with rubber rings i/c cartage, loading unloading at site of work fixing in trenches cutting fitting, jointing with rubber rings etc complete KWSB Approved Rate, M/S Balochistan R.C.C Pipe Co. M/S Pakistan construction (Pvt.) Ltd. M/S Kamal R.C.C Pipe Co. 12”dia pipe |  |  | 600-Rft | 367 |  | P/Rft | 220200/- |
| 04 | Construction of 4” dia manhole inside complete with 21”dia c c 1:2:4 pre cast C.I frame 15kg i/c the cost of M.S bars reinforcement and two handles 5” clear depth c c 1:2:4 cast in situ walls with 50% screened graded bajri of 9” thick i/c bending and channel i/c inside walls surface complete i/c cutting asphalt roads excavation in all kind of soil back filling and disposal of surplus stuff (KW&SB) approved rate. |  |  | 25-Nos | 9307.86 |  | Each | 232697/- |
| 05 | Extra depth beyond 5” ft or less then 5 deducted at the same rate (KWSB) approved rate. |  |  | (-) 12.5-ft | 1609.15 |  | P/ft | (-) 20114/- |
| 06 | Refilling the excavated stuff & surplus Earth etc. complete. |  |  | 8910-Cft | 2760 |  | %Cft | 24592/- |
| 07 | Carriage of 100 cft tons of all kinds’ material like Cole bricks i/c loading unloading 10 miles. |  |  | 990-Cft | 595.80 |  | % Cft | 5898/- |
| 08 | Manufacturing and supplying R.C.C main hole covers cast in situ 1:2:4 C.C. 3” inch deep in center reinforced with 3/8” dia M.S Bars welded on M.S. sheet i/c curing transportation 21” dia 24” dia. |  |  | 05Nos | 913.63 |  | Each | 4568/- |
| 09 | Manufacturing and supplying of R.C.C Ring Slab of 21” inside 36” dia out side 75 wide and to thick i/c 3/8” dia tor steel bar two concentric rings with 3/8” dia Nos. cross linkes bars welded and two sunk type hook, lasted ina 1:1-1/2:3 concrete with embedded 15kg C.I frame in per feet position i/c terms partition charges for an average lead of 20 km per trip from casting yard to town office (A minimum of 25 slabs per trip will be transported.) |  |  | 05Nos | 1906.60 |  | Each | 9533/- |
| 10 | Shifting to the site and fixing of 36” dia ring slab in perfect position on damaged manholes including cutting of damage portion of manhole in proper shape and laying of cement moter of 2” tuckness and disposal of debris |  |  | 05 | 563.09 |  | Each | 2815/- |

Nomenclature, quantities and rates

Etc checked and found correct.

A.E (B&R) 			       				                              A.E.E (B&R)

I/We hereby quoted rate@_________% above/below the schedule of rates.

Contractor’s Signature_____________________                            Address_______________ _________________