SCHEDULE - B

DETAILED WORKING ESTIMATE FOR 10 NOS: TUBE WELLS, PUMP HOUSE, R.C.C RESERVOIR & PVC RISING MAIN IN 07 UNION COUNCILS OF PS-62 (PART-I) VILLAGE GOONGRI, BIRTALA & VERNAI DISTRICT THARPARKAR.

ABSTRACT OF COST

Part-A	Boring of Tube Well	(04 Nos:)			Rs. 55,10,208/=

Part-B	Pump House (8' x 8')	(04 Nos:)			Rs. 7,19,828/=

Part-C	R.C.C Reservoir		(03 Nos:)			Rs. 40,06,999/=

Part-D	Rising Main 3" dia:	3000 Rft 			Rs. 4,59,156/=

Total 		Rs. 1,06,96,191/=

Say		Rs. 1,06,96,200/=

| CONTRACTOR |  | EXECUTIVE ENGINEER PUBLIC HEALTH ENGG: DIVISION THARPARKAR @ MITHI |
| --- | --- | --- |

SCHEDULE – B

PART-A	BORING OF TUBE WELL (GOONGRI 380')

| 1. | Mobilization Tube Well construction. (R.A.A) |
| --- | --- |

1-Trip		@ Rs. 31,570/=				P.Trip		Rs. 31,570/=

(Rupees Thirty One Thousand Five Hundred

Seventy Only) (Per Trip)

| 2. | Drill for tube well at 16" inch minimum dia meter from ground level up to 100' depth. (R.A) |
| --- | --- |

100 ft		@ Rs. 1997/12				P.Ft		Rs. 1,99,712/=

(Rupees One Thousand Nine Hundred

Ninety Seven and Paisa Twelve Only) (P.ft)

| 3. | Drill for tube well at 16" inch minimum dia meter from 100' depth maximum 200' ft depth. (R.A ) |
| --- | --- |

100 ft		@ Rs. 2302/53				P.Ft		Rs. 2,30,253/=

(Rupees Two Thousand Three Hundred

Two and Paisa Fifty Three Only) (P.ft)

| 4. | Drill hole to 16" dia 200' ft to 300' depth. (R.A) |
| --- | --- |

100 ft		@ Rs. 2607/95				P.Ft		Rs. 2,60,795/=

(Rupees Two Thousand Six Hundred

Seven and Paisa Ninety Five Only) (P.ft)

| 5. | Drill hole 16" dia 300' ft to 400 ft (R.A ) |
| --- | --- |

80 ft		@ Rs. 2913/36				P.Ft		Rs. 2,33,069/=

(Rupees Two Thousand Nine Hundred

Thirteen and Thirty Six Paisa Only) (P.ft)

| 7. | Supplying and install 8" nominal dia under PVC class “D” pump housing casing (Blind Pipe). (R.A) |
| --- | --- |

260 ft		@ Rs. 934/70				P.Ft		Rs. 2,43,022/=

(Rupees Nine Hundred Thirty Four and

Paisa Seventy Only) (P.ft)

| 8. | Supplying and install 8" nominal diameter PVC class “D” slatted.(R.A) |
| --- | --- |

120 ft		@ Rs. 1005/79				P.Ft		Rs. 1,20,695/=

(Rupees One Thousand Five and

Paisa Seventy Nine Only) (P.ft)

| 9. | Supply and install 8" nominal dia meter plug 5'-0 ft long complete i/c casing and cap 7 concrete plug. (R.A Approved) |
| --- | --- |

1 No:		@ Rs. 6033/50				Each		Rs. 6,034/=

(Rupees Six Thousand Thirty Three and Paisa Fifty Only) (Each)

| 10. | Supply and install well head casing and security cap.(R.A Approved) |
| --- | --- |

1 No:		@ Rs. 4854/	=				Each		Rs. 4,854/=

(Rupees Four Thousand Eight Hundred Fifty Four Only) (Each)

| 11. | Supply and place graded Bajri 3/8" to 1/8" pack in well. |
| --- | --- |

380 ft		@ Rs. 232/=					P.Ft		Rs. 88,160/=

(Rupees Two Hundred Thirty Two Only) (P.ft)

| 12. | Develop and test well. |
| --- | --- |

1 No:		@ Rs. 1,07,239/=				Each		Rs. 1,07,239/=

(Rupees One Lac Seven Thousand Two Hundred Thirty Nine Only) (Each)

| 13. | Dis-infection of well. |
| --- | --- |

1 No:		@ Rs. 500/=					Each		Rs. 500/=

(Rupees Five Hundred Only) (Each)

| 14. | Construction concrete Class “B” around well head casing complete. |
| --- | --- |

1 No:		@ Rs. 3035/	=				Each		Rs. 3035/=

(Rupees Three Thousand Thirty Five Only) (Each)

Total	Rs. 15,28,938/=

SCHEDULE – B

PART-A	BORING OF TUBE WELL (BIRTALA 280'  2 NOS:)

| 1. | Mobilization Tube Well construction. (R.A.A) |
| --- | --- |

1-Trip		@ Rs. 31,570/=				P.Trip		Rs. 31,570/=

(Rupees Thirty One Thousand Five Hundred

Seventy Only) (Per Trip)

| 2. | Drill for tube well at 16" inch minimum dia meter from ground level up to 100' depth. (R.A) |
| --- | --- |

100 ft		@ Rs. 1997/12				P.Ft		Rs. 1,99,712/=

(Rupees One Thousand Nine Hundred

Ninety Seven and Paisa Twelve Only) (P.ft)

| 3. | Drill for tube well at 16" inch minimum dia meter from 100' depth maximum 200' ft depth. (R.A ) |
| --- | --- |

100 ft		@ Rs. 2302/53				P.Ft		Rs. 2,30,253/=

(Rupees Two Thousand Three Hundred

Two and Paisa Fifty Three Only) (P.ft)

| 4. | Drill hole to 16" dia 200' ft to 300' depth. (R.A) |
| --- | --- |

80 ft		@ Rs. 2607/95				P.Ft		Rs. 2,08,636/=

(Rupees Two Thousand Six Hundred

Seven and Paisa Ninety Five Only) (P.ft)

| 7. | Supplying and install 8" nominal dia under PVC class “D” pump housing casing (Blind Pipe). (R.A) |
| --- | --- |

200 ft		@ Rs. 934/70				P.Ft		Rs. 1,86,940/=

(Rupees Nine Hundred Thirty Four and

Paisa Seventy Only) (P.ft)

| 8. | Supplying and install 8" nominal diameter PVC class “D” slatted.(R.A) |
| --- | --- |

80 ft		@ Rs. 1005/79				P.Ft		Rs. 80,463/=

(Rupees One Thousand Five and

Paisa Seventy Nine Only) (P.ft)

| 9. | Supply and install 8" nominal dia meter plug 5'-0 ft long complete i/c casing and cap 7 concrete plug. (R.A Approved) |
| --- | --- |

1 No:		@ Rs. 6033/50				Each		Rs. 6,034/=

(Rupees Six Thousand Thirty Three and Paisa Fifty Only) (Each)

| 10. | Supply and install well head casing and security cap.(R.A Approved) |
| --- | --- |

1 No:		@ Rs. 4854/	=				Each		Rs. 4,854/=

(Rupees Four Thousand Eight Hundred Fifty Four Only) (Each)

| 11. | Supply and place graded Bajri 3/8" to 1/8" pack in well. |
| --- | --- |

280 ft		@ Rs. 232/=					P.Ft		Rs. 64,960/=

(Rupees Two Hundred Thirty Two Only) (P.ft)

| 12. | Develop and test well. |
| --- | --- |

1 No:		@ Rs. 1,07,239/=				Each		Rs. 1,07,239/=

(Rupees One Lac Seven Thousand Two Hundred Thirty Nine Only) (Each)

| 13. | Dis-infection of well. |
| --- | --- |

1 No:		@ Rs. 500/=					Each		Rs. 500/=

(Rupees Five Hundred Only) (Each)

| 14. | Construction concrete Class “B” around well head casing complete. |
| --- | --- |

1 No:		@ Rs. 3035/	=				Each		Rs. 3035/=

(Rupees Three Thousand Thirty Five Only) (Each)

Total	Rs. 11,24,196/=

Cost of Two Tube wells 1 x 2 = 2 No:  @ Rs. 11,24,196/-	Each	Total	Rs. 22,48,392/-

SCHEDULE – B

PART-A	BORING OF TUBE WELL (VERNAI 425')

| 1. | Mobilization Tube Well construction. (R.A.A) |
| --- | --- |

1-Trip		@ Rs. 31,570/=				P.Trip		Rs. 31,570/=

(Rupees Thirty One Thousand Five Hundred

Seventy Only) (Per Trip)

| 2. | Drill for tube well at 16" inch minimum dia meter from ground level up to 100' depth. (R.A) |
| --- | --- |

100 ft		@ Rs. 1997/12				P.Ft		Rs. 1,99,712/=

(Rupees One Thousand Nine Hundred

Ninety Seven and Paisa Twelve Only) (P.ft)

| 3. | Drill for tube well at 16" inch minimum dia meter from 100' depth maximum 200' ft depth. (R.A ) |
| --- | --- |

100 ft		@ Rs. 2302/53				P.Ft		Rs. 2,30,253/=

(Rupees Two Thousand Three Hundred

Two and Paisa Fifty Three Only) (P.ft)

| 4. | Drill hole to 16" dia 200' ft to 300' depth. (R.A) |
| --- | --- |

100 ft		@ Rs. 2607/95				P.Ft		Rs. 2,60,795/=

(Rupees Two Thousand Six Hundred

Seven and Paisa Ninety Five Only) (P.ft)

| 5. | Drill hole 16" dia 300' ft to 400 ft (R.A ) |
| --- | --- |

100 ft		@ Rs. 2995/67				P.Ft		Rs. 2,99,567/=

(Rupees Two Thousand Nine Hundred

Ninety Five and Paisa Sixty Seven Only) (P.ft)

| 6. | Drill hole 16" dia 400' ft to 500 ft (R.A ) |
| --- | --- |

25 ft		@ Rs. 3383/40				P.Ft		Rs. 84,585/=

(Rupees Three Thousand Three Hundred

Eighty Three and Paisa Forty Only) (P.ft)

| 7. | Supplying and install 8" nominal dia under PVC class “D” pump housing casing (Blind Pipe). (R.A) |
| --- | --- |

300 ft		@ 934/70					P.Ft		Rs. 2,80,410/=

(Rupees Nine Hundred Thirty Four and

Paisa Seventy Only) (P.ft)

| 8. | Supplying and install 8" nominal diameter PVC class “D” slatted.(R.A) |
| --- | --- |

125 ft		@ 1005/79					P.Ft		Rs. 1,25,724/=

(Rupees One Thousand Five and

Paisa Seventy Nine Only)  (P.ft)

| 9. | Supply and install 8" nominal dia meter plug 5'-0 ft long complete i/c casing and cap 7 concrete plug. (R.A Approved) |
| --- | --- |

1 No:		@ Rs. 6033/50				Each		Rs. 6,034/=

(Rupees Six Thousand Thirty Three and

Paisa Fifty Only) (Each)

| 10. | Supply and install well head casing and security cap.(R.A Approved) |
| --- | --- |

1 No:		@ Rs. 4854/	=				Each		Rs. 4,854/=

(Rupees Four Thousand Eight Hundred

Fifty Four Only) (Each)

| 11. | Supply and place graded Bajri 3/8" to 1/8" pack in well. |
| --- | --- |

425 ft		@ Rs. 232/=					P.Ft		Rs. 98,600/= 			(Rupees Two Hundred Thirty Two Only) (P.ft)

| 12. | Develop and test well. |
| --- | --- |

1 No:		@ Rs. 1,07,239/=				Each		Rs. 1,07,239/=

(Rupees One Lac Seven Thousand Two Hundred Thirty Nine Only) (Each)

| 13. | Dis-infection of well. |
| --- | --- |

1 No:		@ Rs. 500/=					Each		Rs. 500/=

(Rupees Five Hundred Only) (Each)

| 14. | Construction concrete Class “B” around well head casing complete. |
| --- | --- |

1 No:		@ Rs. 3035/	=				Each		Rs. 3035/=

(Rupees Three Thousand Thirty Five Only) (Each)

Total	Rs. 17,32,878/=

SCHEDULE – B

PART-B	PUMP HOUSE SIZE (8' X 8')

| 1. | Excavation in foundation of building bridges and other structure i/c dag belling dressing, refilling around structure with excavated earth watering and ramming lead up to one chain and lift up to 5 ft.(G.S.I.No: 18 P-4) (in sand, ashes or loos sail). |
| --- | --- |

184.00 Cft	@ Rs. 2722/50				P‰ Cft	Rs. 501/=

(Rupees Two Thousand Seven Hundred Twenty Two 			and Paisa Fifty Only)(Per Thousand Cft)

| 2. | Cement concrete plain including placing, compacting, finishing and curing complete (including screening and washing of stone aggregate without shuttering (G.S.I.No: 5 (i) P-16). |
| --- | --- |

65.00 Cft	@ Rs. 11288/75				P% Cft	Rs. 7,338/=

(Rupees Eleven Thousand Two Hundred Eighty Eight and Paisa Seventy Five Only)  (Per Hundred Cft)

| 3. | Pucca brick work in foundation and plinth in (1:6). (G.S.I.No.4 P-23). |
| --- | --- |

118.0 Cft	@ Rs. 11948/36 				P% Cft	Rs. 14,099/=

(Rupees Eleven Thousand Nine Hundred Forty Eight and Paisa Thirty Six Only)  (Per Hundred Cft)

| 4. | Pucca brick work in ground floor in i/c stricking of joints with cement sand mortar (1:6) (G.S.I.No. 5 P-25). |
| --- | --- |

234.00 Cft	@ Rs. 12674/36				P% Cft	Rs. 29,658/=

(Rupees Twelve Thousand Six Hundred Seventy Four and Paisa Thirty Six Only)  (Per Hundred Cft)

| 5. | Reinforced cement concrete work i/c all labour and material except the cost of steel reinforcement and its labour for bending and binding which will be paid separately. This rate also i/c of forms moulds lifting shuttering curing rendering and finishing the exposed surfaces i/c screening and washing of shingle 1/8" to 1/4" gauge.(P.H.S. I. No: 6 P-16). |
| --- | --- |

80.00 Cft	@ Rs. 337/=					P.Cft		Rs. 26,960/=

(Rupees Three Hundred Thirty Seven Only)  (P.Cft)

| 6. | Fabrication of mild steel reinforcement for cement concrete i/c cutting bending lying in position making joints and fastening i/c cost of binding wire also i/c removal of rust from bars. (G.S.I.No. 7 P-16). |
| --- | --- |

3.21 Cwt	@ RS. 5001/70 				P.Cwt		Rs. 16,055/=

(Rupees Five Thousand One and

Paisa Seventy Only) (P.Cwt)

| 7. | Filling, watering and ramming earth under floor with surplus earth from foundation lead upto one chain and lift upto 5 feet. (G.S.I.No. 21 P-4). |
| --- | --- |

87.00 Cft	@ Rs. 1512/50 				P‰ Cft	Rs. 132/=

(Rupees One Thousand Five Hundred Twelve and Paisa Fifty Only)  (P. Thousand Cft)

| 8. | Making and fixing steel grated door with 1/16" thick sheeting including and iron frame 2"x 2" x 3/8" and ¾" square bars 4" center to center with locking arrangement. (G.S.I.No.24 P-91). |
| --- | --- |

21.00 Sft	@ RS. 726/72 				P.Sft		Rs. 15,261/=

(Rupees Seven Hundred Twenty Six and

Paisa Seventy Two Only)  (P.Sft)

| 9. | Supplying and fixing steel windows z section window of 3/4" x ¾" x 1/8" fixed and partially open able i/c glazing with 24.0z Class fitting and painting 3 coats complete. (R.A. Approved). |
| --- | --- |

2.03 Sft	@ Rs. 4928/49 				P.Sft		Rs. 10,005/=				(Rupees Four Thousand Nine Hundred Twenty Eight

and Paisa Forty Nine Only) (P.Sft)

| 10. | Supplying and fixing iron steel grill of 3/4" x 3/4" x 1/8" slice flat iron of approved design i/c paining 3 coats etc complete weight not to be less than 3.70 lbs per sq: ft of finished grill. (G.S.I.No. 26 P-92). |
| --- | --- |

24.00 Sft	@ Rs. 180/50 				P.Sft		Rs. 4,332/=

(Rupees One Hundred Eighty and Paisa Fifty Only) (P.Sft)

| 11. | Cement Plaster (1:6) upto 20’ height 1/2" thick (G.S.I.NO 13 (a) P-51). |
| --- | --- |

389.0 Sft	@ Rs. 2206/60   				P% Sft		Rs. 8,584/=

(Rupees Two Thousand Two Hundred Six and

Paisa Sixty Only) (P. Hundred Sft)

| 12. | Cement Plaster (1:4) upto 20’ height 3/8" thick (G.S.I.No: 11 (a) P-51). |
| --- | --- |

389.00 Sft	@ Rs. 2197/52  				P% Sft		 Rs. 8,548/=

(Rupees Two Thousand One Hundred Ninety Seven and Paisa Fifty Tow Only)  (P. Hundred Sft)

| 13. | Cement pointing (1:3) struck joint on wall (G.S.I.No. 19 (b) P-52). |
| --- | --- |

358.00 Sft	@ Rs. 1213/58  				P% Sft		 Rs. 4,345/=

(Rupees One Thousand Two Hundred Thirteen and Paisa Fifty Eight Only)  (P. Hundred Sft)

| 14. | Providing and laying 2" thick topping concrete ratio (1:2:4) surface finishing and dividing into panels 2" thick (G.S.I.No. 16 (c) P-41). |
| --- | --- |

190.00 Sft	@ Rs. 3275/50 				P% Sft		Rs. 6,223/=

(Rupees Three Thousand Two Hundred Seventy Five and Paisa Fifty Only) (P. Hundred Sft)

| 15. | P/F C.I. main hole covering frame i/c cost of material (P.H.S.I.No: 1 P-35). |
| --- | --- |

1.75 Cwt	 @ Rs. 6985/=				P.Cwt		Rs. 12,224/=

(Rupees Six Thousand Nine Hundred Eighty Five Only) (P. Cwt)

| 16. | Painting new surface preparing surfaces and painting of doors and windows of any type i/c edges two coats.(G.S.I No. 4 P-69). |
| --- | --- |

42.00 Sft	@ Rs. 2116/41 				P% Sft		Rs. 889/=

(Rupees Two Thousand One Hundred Sixteen and Paisa Forty One Only)(P. Hundred Sft)

| 17. | White washing (a) one coat (G.S.I.No. 26 P-54). |
| --- | --- |

389.00 Sft	@ Rs. 416/63				P% Sft		Rs. 1,621/=

(Rupees Four Hundred Sixteen and Paisa Sixty Three Only)(P. Hundred Sft)

(b) Three Coats.

106.00 Sft	@ Rs. 829/95				P% Sft		Rs. 880/=

(Rupees Eight Hundred Twenty Nine and Paisa Ninety Five Only)(P. Hundred Sft)

| 18. | Colour washing (2 coats) (G.S.I.No. 25 (b) P-54). |
| --- | --- |

747.00 Sft	@ Rs. 859/90				P% Sft		Rs. 6,423/=

(Rupees Eight Hundred Fifty Nine and Paisa Ninety Only) (P. Hundred Sft).

| 19. | S/F M/S. Girder size 8" x 4". (Sch: Mat: Item 140 P-6) |
| --- | --- |

1.527 Cwt	@ Rs. 3850/=				P.Cwt		Rs. 5,879/=

(Rupees Three Thousand Eight Hundred Fifty Only) (P. Cwt)

Total		Rs. 1,79,957/=

1 x 4	=	4 Nos:

4 Nos:		@ Rs. 1,79,957/-		Each		Total		Rs. 7,19,828/-

SCHEDULE – B

PART-C	R.C.C U.G RESERVOIR (GOONGRI) (28 ft dia:)

| 1. | Excavation for tanks and reservoirs in sandy Soil i/c trimming and dressing sides to true alignment / design correct / profiles and shape, leveling of bed of tanks to correct level & grade i/c laying of earth in 6” layers for construction of tanks and dressing and disposal of surplus excavated earth within one chain as directed by Engineer Incharge i/c providing fence guards lights flags wherever required lift upto 5 feet and lead upto one chain (P.H.S.I.No. 2 P-68). |
| --- | --- |

0'-5'

11025.0 Cft 	@ Rs. 3150/= 				P‰ Cft	Rs. 34,729/=

(Rupees Three Thousand One Hundred

Fifty Only) (P. Thousand Cft)

5'-8'

4769.0 Cft 	@ Rs. 3700/= 				P‰ Cft	Rs. 17,645/=

(Rupees Three Thousand Seven Hundred)

(P. Thousand Cft)

8'-11'

3582.00 Cft 	@ Rs. 4250/= 				P‰ Cft	Rs. 15,224/=

(Rupees Four Thousand Two Hundred

Fifty Only) (P. Thousand Cft)

| 2. | Cement Concrete plain including placing compacting finishing and curing complete (including screening and washing of stone aggregate with out shutting (G.S.I. No. 5 (h) P-15). |
| --- | --- |

Ratio (1:4:8)

509.0 Cft	@ Rs. 11288/75				P% Cft	Rs. 57,460/=

(Rupees Eleven Thousand Two Hundred Eighty Eight and Paisa Seventy Five Only)  (Per Hundred Cft)

| 3. | Reinforced cement concrete work including all labour and material except the cost of steel reinforcement and its labour for bending and binding, which will be paid separately. This rate also includes all kinds of forms moulds lifting shuttering curing rendering and finishing the exposed surface (including screening and washing of shingle) (a) R.C.C work in roof slab beams columns, rafts, lintels and other structural members laid in situ or precast laid in position complete in all respects Ratio (1:2:4) 90 lbs cement 2 Cft sand and 4 Cft shingle 1/8” to ¼” gauge. (G.S.I. No. 6 P-16). |
| --- | --- |

2284.0 Cft	@ Rs. 337/=					P.Cft		Rs. 7,69,708/=

(Rupees Three Hundred Thirty Seven Only)  (P.Cft)

| 4. | Fabrication of Mild steel reinforcement for cement concrete including cutting bending laying in position, making joints & fastenings including cost of binding wire (also includes removal of rust from bars) (G.S.I. No. 7 (b) P-16) (USING TAR BARS). |
| --- | --- |

101.964 Cwt	@ Rs. 5001/70				P.Cwt		Rs. 5,09,993/=

(Rupees Five Thousand One and

Paisa Seventy Only) (P.Cwt)

| 5. | Cement Plaster (1:3) upto 20’ height ¾” thick (G.S.I.No: 10 (c) P-51) (For Water Proofing). |
| --- | --- |

1007.00 Sft	@ Rs. 2795/30 				P% Sft		Rs. 28,149/=

(Rupees Two Thousand Seven Hundred Ninety Five and Paisa Thirty Only)  (P. Hundred Sft)

| 6. | Supplying Dampo (For mixing with cement plaster for water proofing purposes) Gen: Sch: of Mat: Item No. 46 P-4) 3.50 Lbs per bag of cement. |
| --- | --- |

21.0 Kg	@ Rs. 125/=					P.Kg		Rs. 2,625/=

(Rupees One Hundred Twenty five Only)  (P.Kg)

| 7. | Providing and fixing C.I .Main hole cover with frame i/c cost of material (P.H.S.I. No: 1 P-35). |
| --- | --- |

2.0 Cwt	@ Rs. 6985/=				P.Cwt		Rs. 13,970/=

(Rupees Six Thousand Nine Hundred

Eighty Five Only) (P. Cwt)

| 8. | Providing/ Fixing C.I ventilator (Weight not less than 80 – Lbs/ Each) in tank roof of approved design with water light joint i/c painting etc complete. (P.H.S.I.No 4 P-55). |
| --- | --- |

2 Nos:		@ Rs. 4870/=				Each		Rs. 9,740/=

(Rupees Four Thousand Eight Hundred

Seventy Only) (Each)

| 9. | Providing construction joint in concrete work of 9” wide corrugated P/C water stopper (with bulb) i/c soldering cost of material and labour etc complete (P.H.S.I No. 2 P-56). |
| --- | --- |

185.00 Rft	@ Rs. 86/=					P.Rft		Rs. 15,910/=

(Rupees Eighty Six Only) (P.Rft)

| 10. | Supplying C.I Tail Piece with one end flanged and other end spigot (for all size) (P.H.S. of Mat: I. No. 5 P-11). |
| --- | --- |

0.131 Cwt	@ Rs. 6096/=				P.Cwt		Rs. 799/=

(Rupees Six Thousand Ninety Six Only) (P.Cwt)

| 11. N.S.I | Supplying and embedding C.I Foot steps of approved design weight not less than 15 Lbs (R.A. Approve). |
| --- | --- |

10 Nos:	@ Rs. 181/65				Each		Rs. 1,817/=

(Rupees One Hundred Eighty One and Paisa Sixty Five Only) (Each)

| 12. | Providing and laying 1” thick topping cement concrete (1:2:4) including surface finishing and dividing into panels. (G.S.I. No: 16 P-41). |
| --- | --- |

2" thick

1335.00 Sft	@ Rs. 3275/50				P% Sft		Rs. 43,728/=

(Rupees Three Thousand Two Hundred Seventy Five and Paisa fifty Only) (P. Hundred Sft)

| 13. | Providing, Laying, jointing and testing G.I Pipe of standard approved quality with thickness 6SWG test pressure 725 PSI (R.A) |
| --- | --- |

3" dia

12.0 Rft	@ Rs. 338/17				P.Rft		Rs. 4,058/=

(Rupees Three Hundred Thirty Eight and

Paisa Seventeen Only) (P.Rft)

| 14. | Providing/fixing M.S flanges made of M.S plate hanging in thickness and total weight as mentioned against each item i/c the cost of making hocks, framing, welding, nuts, bolts, rubber packing, white lead filler cartage etc complete. (2.020 Kg) (P.H.S.I No. E P-32) |
| --- | --- |

3" dia

04 Nos:	@ Rs. 945/=					Each		Rs. 3,780/=

(Rupees Nine Hundred Forty Five Only) (Each)

| 15. | C.I bend with flange and with hole including turning and facing of flanges for all size (P.H.S of Mat: I No: 8 P-11). |
| --- | --- |

3" dia

0.525 Cwt	@ Rs. 6096/=				P.Cwt		Rs. 3,200/=

(Rupees Six Thousand Ninety Six Only) (P. Cwt)

| 16. | Jointing C.I /M.S Flanged Pipes and special Flange inside trenches including I/C applying rubber packing of required thickness nuts and bolts with washer etc. and other rules required for jointing and testing be same to be specified presser etc.. complete (P.H.S.I No: 1 P-40). |
| --- | --- |

3" dia

6 Joint	@ Rs. 499/=					P.Joint	Rs. 2,994/=

(Rupees Four Hundred Ninety Nine Only) (P. Joint)

Total		Rs. 15,35,529/=

SCHEDULE – B

PART-C	R.C.C U.G RESERVOIR (BIRTALA) (26 ft dia:)

| 1. | Excavation for tanks and reservoirs in sandy Soil i/c trimming and dressing sides to true alignment / design correct / profiles and shape, leveling of bed of tanks to correct level & grade i/c laying of earth in 6” layers for construction of tanks and dressing and disposal of surplus excavated earth within one chain as directed by Engineer Incharge i/c providing fence guards lights flags wherever required lift upto 5 feet and lead upto one chain (P.H.S.I.No. 2 P-68). |
| --- | --- |

0'-5'

10209.0 Cft 	@ Rs. 3150/= 				P‰ Cft	Rs. 32,158/=

(Rupees Three Thousand One Hundred

Fifty Only) (P. Thousand Cft)

5'-8'

4354.0 Cft 	@ Rs. 3700/= 				P‰ Cft	Rs. 16,110/=

(Rupees Three Thousand Seven Hundred)

(P. Thousand Cft)

8'-11'

3224.00 Cft 	@ Rs. 4250/= 				P‰ Cft	Rs. 13,702/=

(Rupees Four Thousand Two Hundred

Fifty Only) (P. Thousand Cft)

| 2. | Cement Concrete plain including placing compacting finishing and curing complete (including screening and washing of stone aggregate with out shutting (G.S.I.No. 5 (h) P-15). |
| --- | --- |

Ratio (1:4:8)

454.0 Cft	@ Rs. 11288/75				P% Cft	Rs. 51,251/=

(Rupees Eleven Thousand Two Hundred Eighty Eight and Paisa Seventy Five Only)  (Per Hundred Cft)

| 3. | Reinforced cement concrete work including all labour and material except the cost of steel reinforcement and its labour for bending and binding, which will be paid separately. This rate also includes all kinds of forms moulds lifting shuttering curing rendering and finishing the exposed surface (including screening and washing of shingle) (a) R.C.C work in roof slab beams columns, rafts, lintels and other structural members laid in situ or precast laid in position complete in all respects Ratio (1:2:4) 90 lbs cement 2 Cft sand and 4 Cft shingle 1/8” to ¼” gauge. (G.S.I.No. 6 P-16). |
| --- | --- |

2077.0 Cft	@ Rs. 337/=					P.Cft		Rs. 6,99,949/=

(Rupees Three Hundred Thirty Seven Only)  (P.Cft)

| 4. | Fabrication of Mild steel reinforcement for cement concrete including cutting bending laying in position, making joints & fastenings including cost of binding wire (also includes removal of rust from bars) (G.S.I. No. 7 (b) P-16) (USING TAR BARS). |
| --- | --- |

92.72 Cwt	@ Rs. 5001/70				P.Cwt		Rs. 4,63,758/=

(Rupees Five Thousand One and

Paisa Seventy Only) (P.Cwt)

| 5. | Cement Plaster (1:3) upto 20’ height ¾” thick (G.S.I.No: 10 (c) P-51) (For Water Proofing). |
| --- | --- |

934.00 Sft	@ Rs. 2795/30 				P% Sft		Rs. 26,108/=

(Rupees Two Thousand Seven Hundred Ninety Five and Paisa Thirty Only)  (P. Hundred Sft)

| 6. | Supplying Dampo (For mixing with cement plaster for water proofing purposes) Gen: Sch: of Mat: Item No. 46 P-4) 3.50 Lbs per bag of cement. |
| --- | --- |

19.56 Kg	@ Rs. 125/=					P.Kg		Rs. 2,445/=

(Rupees One Hundred Twenty five Only)  (P.Kg)

| 7. | Providing and fixing C.I .Main hole cover with frame i/c cost of material (P.H.S.I. No:1 P-35). |
| --- | --- |

2.0 Cwt	@ Rs. 6985/=				P.Cwt		Rs. 13,970/=

(Rupees Six Thousand Nine Hundred

Eighty Five Only) (P. Cwt)

| 8. | Providing/ Fixing C.I ventilator (Weight not less than 80 – Lbs/ Each) in tank roof of approved design with water light joint i/c painting etc complete. (P.H.S.I.No 4 P-55). |
| --- | --- |

2 Nos:		@ Rs. 4870/=				Each		Rs. 9,740/=

(Rupees Four Thousand Eight Hundred

Seventy Only) (Each)

| 9. | Providing construction joint in concrete work of 9” wide corrugated P/C water stopper (with bulb) i/c soldering cost of material and labour etc complete (P.H.S.I No. 2 P-56). |
| --- | --- |

173.00 Rft	@ Rs. 86/=					P.Rft		Rs. 14,878/=

(Rupees Eighty Six Only) (P.Rft)

| 10. | Supplying C.I Tail Piece with one end flanged and other end spigot (for all size) (P.H.S. of Mat: I. No. 5 P-11). |
| --- | --- |

0.131 Cwt	@ Rs. 6096/=				P.Cwt		Rs. 799/=

(Rupees Six Thousand Ninety Six Only) (P.Cwt)

| 11. N.S.I | Supplying and embedding C.I Foot steps of approved design weight not less than 15 Lbs (R.A. Approve). |
| --- | --- |

10 Nos:	@ Rs. 181/65				Each		Rs. 1,817/=

(Rupees One Hundred Eighty One and Paisa Sixty Five Only) (Each)

| 12. | Providing and laying 1” thick topping cement concrete (1:2:4) including surface finishing and dividing into panels. (G.S.I. No: 16 P-41). |
| --- | --- |

2" thick

1158.00 Sft	@ Rs. 3275/50				P% Sft		Rs. 37,930/=

(Rupees Three Thousand Two Hundred Seventy Five and Paisa fifty Only) (P. Hundred Sft)

| 13. | Providing, Laying, jointing and testing G.I Pipe of standard approved quality with thickness 6SWG test pressure 725 PSI (R.A) |
| --- | --- |

12.0 Rft	@ Rs. 304/02				P.Rft		Rs. 4,058/=

(Rupees Three Hundred four and

Paisa Two Only) (P.Rft)

| 14. | Providing/fixing M.S flanges made of M.S plate hanging in thickness and total weight as mentioned against each item i/c the cost of making hocks, framing, welding, nuts, bolts, rubber packing, white lead filler cartage etc complete. (2.020 Kg) (P.H.S.I No. E P-32) |
| --- | --- |

3" dia

04 Nos:	@ Rs. 945/=					Each		Rs. 3,780/=

(Rupees Nine Hundred Forty Five Only) (Each)

| 15. | C.I bend with flange and with hole including turning and facing of flanges for all size (P.H.S of Mat: I No:8 P-11). |
| --- | --- |

3" dia

0.525 Cwt	@ Rs. 6096/=				P.Cwt		Rs. 3,200/=

(Rupees Six Thousand Ninety Six Only) (P. Cwt)

| 16. | Jointing C.I /M.S Flanged Pipes and special Flange inside trenches including I/C applying rubber packing of required thickness nuts and bolts with washer etc. and other rules required for jointing and testing be same to be specified presser etc complete. (P.H.S.I No: 1 P-40). |
| --- | --- |

3" dia

6 Joint	@ Rs. 499/=					P.Joint	Rs. 2,994/=

(Rupees Four Hundred Ninety Nine Only) (P. Joint)

Total		Rs. 13,98,647/=

SCHEDULE – B

PART-C	R.C.C U.G RESERVOIR (VARNAI) (21 ft dia)

| 1. | Excavation for tanks and reservoirs in sandy Soil i/c trimming and dressing sides to true alignment / design correct / profiles and shape, leveling of bed of tanks to correct level & grade i/c laying of earth in 6” layers for construction of tanks and dressing and disposal of surplus excavated earth within one chain as directed by Engineer Incharge i/c providing fence guards lights flags wherever required lift upto 5 feet and lead upto one chain (P.H.S.I.No. 2 P-68). |
| --- | --- |

0'-5'

8305.0 Cft 	@ Rs. 3150/= 				P‰ Cft	Rs. 26,161/=

(Rupees Three Thousand One Hundred

Fifty Only) (P. Thousand Cft)

5'-8'

3401.0 Cft 	@ Rs. 3700/= 				P‰ Cft	Rs. 12,584/=

(Rupees Three Thousand Seven Hundred)

(P. Thousand Cft)

8'-11'

2412.00 Cft 	@ Rs. 4250/= 				P‰ Cft	Rs. 10,251/=

(Rupees Four Thousand Two Hundred

Fifty Only) (P. Thousand Cft)

| 2. | Cement Concrete plain including placing compacting finishing and curing complete (including screening and washing of stone aggregate with out shutting (G.S.I. No. 5 (h) P-15). |
| --- | --- |

Ratio (1:4:8)

330.0 Cft	@ Rs. 11288/75				P% Cft	Rs. 37,253/=

(Rupees Eleven Thousand Two Hundred Eighty Eight and Paisa Seventy Five Only)  (Per Hundred Cft)

| 3. | Reinforced cement concrete work including all labour and material except the cost of steel reinforcement and its labour for bending and binding, which will be paid separately. This rate also includes all kinds of forms moulds lifting shuttering curing rendering and finishing the exposed surface (including screening and washing of shingle) (a) R.C.C work in roof slab beams columns, rafts, lintels and other structural members laid in situ or precast laid in position complete in all respects Ratio (1:2:4) 90 lbs cement 2 Cft sand and 4 Cft shingle 1/8” to ¼” gauge. (G.S.I. No. 6 P-16). |
| --- | --- |

1581.0 Cft	@ Rs. 337/=					P.Cft		Rs. 5,32,797/=

(Rupees Three Hundred Thirty Seven Only)  (P.Cft)

| 4. | Fabrication of Mild steel reinforcement for cement concrete including cutting bending laying in position, making joints & fastenings including cost of binding wire (also includes removal of rust from bars) (G.S.I. No. 7 (b) P-16) (USING TAR BARS). |
| --- | --- |

70.58 Cwt	@ Rs. 5001/70				P.Cwt		Rs. 3,53,020/=

(Rupees Five Thousand One and

Paisa Seventy Only) (P.Cwt)

| 5. | Cement Plaster (1:3) upto 20’ height ¾” thick (G.S.I.No: 10 (c) P-51) (For Water Proofing). |
| --- | --- |

754.00 Sft	@ Rs. 2795/30 				P% Sft		Rs. 21,077/=

(Rupees Two Thousand Seven Hundred Ninety Five and Paisa Thirty Only)  (P. Hundred Sft)

| 6. | Supplying Dampo (For mixing with cement plaster for water proofing purposes) Gen: Sch: of Mat: Item No. 46 P-4) 3.50 Lbs per bag of cement. |
| --- | --- |

15.80 Kg	@ Rs. 125/=					P.Kg		Rs. 1,975/=

(Rupees One Hundred Twenty five Only)  (P.Kg)

| 7. | Providing and fixing C.I .Main hole cover with frame i/c cost of material (P.H.S.I. No: 1 P-35). |
| --- | --- |

2.0 Cwt	@ Rs. 6985/=				P.Cwt		Rs. 13,970/=

(Rupees Six Thousand Nine Hundred

Eighty Five Only) (P. Cwt)

| 8. | Providing/ Fixing C.I ventilator (Weight not less than 80 – Lbs/ Each) in tank roof of approved design with water light joint i/c painting etc complete. (P.H.S.I.No 4 P-55). |
| --- | --- |

2 Nos:		@ Rs. 4870/=				Each		Rs. 9,740/=

(Rupees Four Thousand Eight Hundred

Seventy Only) (Each)

| 9. | Providing construction joint in concrete work of 9” wide corrugated P/C water stopper (with bulb) i/c soldering cost of material and labour etc complete (P.H.S.I No. 2 P-56). |
| --- | --- |

141.00 Rft	@ Rs. 86/=					P.Rft		Rs. 12,126/=

(Rupees Eighty Six Only) (P.Rft)

| 10. | Supplying C.I Tail Piece with one end flanged and other end spigot (for all size) (P.H.S. of Mat: I. No. 5 P-11). |
| --- | --- |

0.131 Cwt	@ Rs. 6096/=				P.Cwt		Rs. 799/=

(Rupees Six Thousand Ninety Six Only) (P.Cwt)

| 11. N.S.I | Supplying and embedding C.I Foot steps of approved design weight not less than 15 Lbs (R.A. Approve). |
| --- | --- |

10 Nos:	@ Rs. 181/65				Each		Rs. 1,817/=

(Rupees One Hundred Eighty One and Paisa Sixty Five Only) (Each)

| 12. | Providing and laying 1” thick topping cement concrete (1:2:4) including surface finishing and dividing into panels. (G.S.I. No: 16 P-41). |
| --- | --- |

2" thick

770.00 Sft	@ Rs. 3275/50				P% Sft		Rs. 25,221/=

(Rupees Three Thousand Two Hundred Seventy Five and Paisa fifty Only) (P. Hundred Sft)

| 13. | Providing, Laying, jointing and testing G.I Pipe of standard approved quality with thickness 6SWG test pressure 725 PSI (R.A) |
| --- | --- |

3" dia

12.0 Rft	@ Rs. 338/17				P.Rft		Rs. 4,058/=

(Rupees Three Hundred Thirty Eight and

Paisa Seventeen Only) (P.Rft)

| 14. | Providing/fixing M.S flanges made of M.S plate hanging in thickness and total weight as mentioned against each item i/c the cost of making hocks, framing, welding, nuts, bolts, rubber packing, white lead filler cartage etc complete. (2.020 Kg) (P.H.S.I No. E P-32) |
| --- | --- |

3" dia

04 Nos:	@ Rs. 945/=					Each		Rs. 3,780/=

(Rupees Nine Hundred Forty Five Only) (Each)

| 15. | C.I bend with flange and with hole including turning and facing of flanges for all size (P.H.S of Mat: I No: 8 P-11). |
| --- | --- |

3" dia

0.252 Cwt	@ Rs. 6096/=				P.Cwt		Rs. 3,200/=

(Rupees Six Thousand Ninety Six Only) (P. Cwt)

| 16. | Jointing C.I /M.S Flanged Pipes and special Flange inside trenches including I/C applying rubber packing of required thickness nuts and bolts with washer etc. and other rules required for jointing and testing be same to be specified presser etc.. complete (P.H.S.I No: 1 P-40). |
| --- | --- |

3" dia

6 Joint	@ Rs. 499/=					P.Joint	Rs. 2,994/=

(Rupees Four Hundred Ninety Nine Only) (P. Joint)

Total		Rs. 10,72,823/=

SCHEDULE – B

PART-D	RISING MAIN 3" DIA.

| 1. | Providing PVC pipe of class “B” equivalent make fixing in trench i/c cutting, fitting and jointing with “Z” Joint with one rubber ring i/c testing with water to head of 61 meter or 200 ft. (P.H.S.I. No. 1 P-21). |
| --- | --- |

3" Dia

3000.0 Rft	@ Rs. 90/=					P.Rft		Rs. 2,70,000/=			(Rupees Ninety) only (Per Rft)

| 2. | Supplying C.I sluice valve heavy pattern test pressure 300 lbs/sq: inch (S.M.I No. 2 P-9). Imported |
| --- | --- |

3" Dia

03 Nos	@ Rs. 4290/=				Each		Rs. 12,870/=

(Rupees Four Thousand Two Hundred Ninety)only (Each)

| 3. | PVC “Z” Joint fitting (P.H.S.O.M.Item No. 16 P-28). |
| --- | --- |
|  | Cast iron special for A.C pressure pipe (Standard Weight A.C.I.L “B” Class) (P.H.S.O.M.I No:10 P-12). |

Short Piece/Tail Piece

3" dia:

06 Nos:	@ Rs. 375/=    				Each		Rs. 2,250/=

(Rupees Three Hundred Seventy Five)only (Each)

4.        PVC Bend 3” dia:

03 Nos:	@ Rs 731 /=        				Each   	Rs. 2,193/=

(Rupees Seven Hundred Thirty One)only (Each)

| 5. | Jointing C.I /M.S Flanged Pipes and special Flange inside trenches including I/C applying rubber packing of required thickness nuts and bolts with washer etc. and other rules required for jointing and testing be same to be specified presser etc.. complete (P.H.S.I No: 1 P-40). |
| --- | --- |

3" dia

6 Joints	@ Rs. 499/=  				P.Joint	Rs. 2,994/=

(Rupees Four Hundred Ninety Nine) only (P. Joint)

| 6. | Making joints to PVC specials fitting including laying of specials and cost of solvent cement of required diameter and testing the joints with along with PVC pipe “B, “C” & “D” class to the specied pressure and making good o all leaky joints etc complete.. (P.H.S.I.No. 1 P-41). |
| --- | --- |

3" dia

3 Joints	@ Rs. 59/=  					P.Joint	Rs. 177/=

(Rupees Fifty Nine) only (Per Joint)

| 7. | Excavation for pipe line trenches and pits in sandy soil including trimming and dressing sides to true alignment and shape leveling of beds of trenches to correct level and grade, cutting joint holes and disposal of surplus earth within a one chain as directed by Engineer Incharge. Providing fence guards, lights, flags, temporary crossings for non vehicular traffic where ever required lift upto 5 ft (1.52 m) and lead upto one chain (30.5 m). (P.H.S.I.No. 2 P-60). |
| --- | --- |

3" dia

18000.0 Cft	@ Rs. 3750/=				P‰ Cft	Rs. 67,500/=

(Rupees Three Thousand Seven Hundred Fifty)

Only (Per Thousand Cft)

| 8. | Refilling the excavated stuff in trenches 6" thick layer i/c watering ramming to full compaction etc complete. (P.H.S.I.No. 24 P-77). |
| --- | --- |

16200.0 Cft	@ Rs. 2760/=				P‰ Cft	Rs. 44,712/=

(Rupees Two Thousand Seven Hundred Sixty)

Only (Per Thousand Cft)

| 9. | Providing chamber 3' x 2' (915 x 615 mm) inside dimension 4 ½' (1372 mm) deep as per approved design for sluice valve 3'' to 12'' dia with 18'' (457 mm) dia inside cost cover and frame (wt = 1 cwt 3 qr) fixed in RCC 1:2:4 (102 mm) thick (with 5 Lbs steel per cft) 9'' (299 mm) thick brick masonry wall set in 1:6 cement mortar 6'' (1152 mm) thick cement concrete 1:3:6 in foundation 1'' (25 mm) thick cement concrete 1:2:4 flooring ½'' (12.5 mm) thick cement plaster 1:3 to all inside wall surface and top i/c providing and fixing M.S foot rest at every one foot beyond 2 ½ ft depth curing, excavation, back filling and disposal of surplus earth etc complete. (P.H.S.I No. 1 P-49). |
| --- | --- |

3 Nos:		@ Rs. 18820/=				Each		Rs. 56,460/=

(Rupees Eighteen Thousand Eight Hundred Twenty)

only (Each)

Total		Rs. 4,59,156/=

CONDITIONS

- The rates should be inclusive of all the Taxes i/c. Sales Tax, Octroi. Royalties etc, if any.

- No separate carriage will be allowed to the contractor.

- The Testing of the works/Pumping Machinery/Pipe will be arranged by the contractor at his own cost, No separate payment will be made.

- No premium will be allowed on Non-Schedule items.

- Any error or omission in the specifications/items will be governed by the relevant specifications and schedule of rates.

- The decision of the Executive Engineer will be final and binding on all the parties in any dispute.

- The contractor will have to submit the certificate of manufacturer at the time of supply of Machinery.

| CONTRACTOR |  | EXECUTIVE ENGINEER PUBLIC HEALTH ENGG: DIVISION THARPARKAR @ MITHI |
| --- | --- | --- |

| 1. | Providing PVC pipe of class “B” equivalent make fixing in trench i/c cutting, fitting and jointing with “Z” Joint with one rubber ring i/c testing with water to head of 61 meter or 200 ft. (P.H.S.I. No. 1 P-21). |
| --- | --- |

3" Dia

3000.0 Rft	@ Rs. 90/=					P.Rft		Rs. 2,70,000/=

| 2. | Supplying C.I sluice valve heavy pattern test pressure 300 lbs/sq: inch (S.M.I No: 2 P-9) Imported. |
| --- | --- |

03 Nos:	@ Rs. 8650/=    				Each		Rs. 25,950/=

| 3. | PVC “Z” Joint fitting (P.H.S.O.M.Item No. 16 P-28). |
| --- | --- |
|  | Cast iron special for A.C pressure pipe (Standard Weight A.C.I.L “B” Class) (P.H.S.O.M.I No:10 P-12). |

Short Piece/Tail Piece

3" dia:

06 Nos:	@ Rs. 375/=    				Each		Rs. 2,250/=

4.        PVC Bend 3” dia:

03 Nos:	@ Rs 731 /=        				Each   	Rs. 2,193/=

| 5. | Jointing C.I /M.S Flanged Pipes and special Flange inside trenches including I/C applying rubber packing of required thickness nuts and bolts with washer etc. and other rules required for jointing and testing be same to be specified presser etc.. complete (P.H.S.I No: 1 P-40). |
| --- | --- |

3" dia

6 Joints	@ Rs. 499/=  				P.Joint	Rs. 2,994/=

| 6. | Making joints to PVC specials fitting including laying of specials and cost of solvent cement of required diameter and testing the joints with along with PVC pipe “B, “C” & “D” class to the specied pressure and making good o all leaky joints etc complete.. (P.H.S.I.No. 1 P-41). |
| --- | --- |

3" dia

3 Joints	@ Rs. 59/=  					P.Joint	Rs. 177/=

| 7. | Excavation for pipe line trenches and pits in sandy soil including trimming and dressing sides to true alignment and shape leveling of beds of trenches to correct level and grade, cutting joint holes and disposal of surplus earth within a one chain as directed by Engineer Incharge. Providing fence guards, lights, flags, temporary crossings for non vehicular traffic where ever required lift upto 5 ft (1.52 m) and lead upto one chain (30.5 m). (P.H.S.I.No. 2 P-60). |
| --- | --- |

3" dia:

18000.0 Cft	@ Rs. 3750/=				P‰ Cft	Rs. 67,500/=

| 8. | Refilling the excavated stuff in trenches 6" thick layer i/c watering ramming to full compaction etc complete. (P.H.S.I.No.24 P-53). |
| --- | --- |

16200.0 Cft	@ Rs. 2760/=				P‰ Cft	Rs. 44,712/=

| 9. | Providing chamber 3' x 2' (915 x 615 mm) inside dimension 4 ½' (1372 mm) deep as per approved design for sluice valve 3" to 12" dia with 18" (457 mm) dia inside cost iron cover and frame (wt = 1 cwt 3 qr) fixed in RCC 1:2:4 (102 mm) thick (with 5 Lbs steel per cft) 9" (299 mm) thick brick masonry wall set in 1:6 cement mortar 6" (1152 mm) thick cement concrete 1:3:6 in foundation 1" (25 mm) thick cement concrete 1:2:4 flooring ½" (12.5 mm) thick cement plaster 1:3 to all inside wall surface and to top i/c providing and fixing M.S foot rest at every one foot beyond 2 ½ ft depth curing, excavation, back filling and disposal of surplus earth etc complete. (P.H.S.I.No. 1 P-49). |
| --- | --- |

3 Nos:		@ Rs. 18820/=				Each		Rs. 56,460/=

Total		Rs. 4,72,236/=

PART-C	R.C.C U.G RESERVOIR (VARNAI) (22 ft dia)

| 1. | Excavation for tanks and reservoirs in sandy Soil i/c trimming and dressing sides to true alignment / design correct / profiles and shape, leveling of bed of tanks to correct level & grade i/c laying of earth in 6” layers for construction of tanks and dressing and disposal of surplus excavated earth within one chain as directed by Engineer Incharge i/c providing fence guards lights flags wherever required lift upto 5 feet and lead upto one chain (P.H.S.I.No. 2 P-68). |
| --- | --- |

0'-5'

8670.0 Cft 	@ Rs. 3150/= 				P‰ Cft	Rs. 27,311/=

5'-8'

3582.0 Cft 	@ Rs. 3700/= 				P‰ Cft	Rs. 13,253/=

8'-11'

2565.00 Cft 	@ Rs. 4250/= 				P‰ Cft	Rs. 10,901/=

| 2. | Cement Concrete plain including placing compacting finishing and curing complete (including screening and washing of stone aggregate with out shutting (G.S.I. No. 5 (h) P-15). |
| --- | --- |

Ratio (1:4:8)

353.0 Cft	@ Rs. 11288/75				P% Cft	Rs. 39,849/=

| 3. | Reinforced cement concrete work including all labour and material except the cost of steel reinforcement and its labour for bending and binding, which will be paid separately. This rate also includes all kinds of forms moulds lifting shuttering curing rendering and finishing the exposed surface (including screening and washing of shingle) (a) R.C.C work in roof slab beams columns, rafts, lintels and other structural members laid in situ or precast laid in position complete in all respects Ratio (1:2:4) 90 lbs cement 2 Cft sand and 4 Cft shingle 1/8” to ¼” gauge. (G.S.I.No. 6 P-16). |
| --- | --- |

1680.0 Cft	@ Rs. 337/=					P.Cft		Rs. 5,66,160/=

| 4. | Fabrication of Mild steel reinforcement for cement concrete including cutting bending laying in position, making joints & fastenings including cost of binding wire (also includes removal of rust from bars) (G.S.I. No. 7 (b) P-16) (USING TAR BARS). |
| --- | --- |

75.0 Cwt	@ Rs. 5001/70				P.Cwt		Rs. 3,75,128/=

| 5. | Cement Plaster (1:3) upto 20’ height ¾” thick (G.S.I.No: 10 (c) P-51) (For Water Proofing). |
| --- | --- |

790.00 Sft	@ Rs. 2795/30 				P% Sft		Rs. 22,083/=

| 6. | Supplying Dampo (For mixing with cement plaster for water proofing purposes) Gen: Sch: of Mat: Item No. 46 P-4) 3.50 Lbs per bag of cement. |
| --- | --- |

24.0 Kg	@ Rs. 125/=					P.Kg		Rs. 3,000/=

| 7. | Providing and fixing C.I .Main hole cover with frame i/c cost of material (P.H.S.I. No:1 P-35). |
| --- | --- |

2.0 Cwt	@ Rs. 6985/=				P.Cwt		Rs. 13,970/=

| 8. | Providing/ Fixing C.I ventilator (Weight not less than 80 – Lbs/ Each) in tank roof of approved design with water light joint i/c painting etc complete. (P.H.S.I.No 4 P-55). |
| --- | --- |

2 Nos:		@ Rs. 4870/=				Each		Rs. 9,740/=

| 9. | Providing construction joint in concrete work of 9” wide corrugated P/C water stopper (with bulb) i/c soldering cost of material and labour etc complete (P.H.S.I No.2 P-56). |
| --- | --- |

103.00 Rft	@ Rs. 86/=					P.Rft		Rs. 8,858/=

| 10. | Supplying C.I Tail Piece with one end flanged and other end spigot (for all size) (P.H.S.of Mat: I.No. 5 P-11). |
| --- | --- |

0.491 Cwt	@ Rs. 6096/=				P.Cwt		Rs. 2,993/=

| 11. N.S.I | Supplying and embedding C.I Foot steps of approved design weight not less than 15 Lbs (R.A. Approve). |
| --- | --- |

10 Nos:	@ Rs. 181/65				Each		Rs. 1,817/=

| 12. | Providing and laying 1” thick topping cement concrete (1:2:4) including surface finishing and dividing into panels. (G.S.I. No: 16 P-41). |
| --- | --- |

2" thick

842.00 Sft	@ Rs. 3275/50				P% Sft		Rs. 27,580/=

| 13. | Providing, Laying, jointing and testing G.I Pipe of standard approved quality with thickness 6SWG test pressure 725 PSI (R.A) |
| --- | --- |

3" dia:

12.0 Rft	@ Rs. 304/02				P.Rft		Rs. 3,648/=

| 14. | Providing/fixing M.S flanges made of M.S plate hanging in thickness and total weight as mentioned against each item i/c the cost of making hocks, framing, welding, nuts, bolts, rubber packing, white lead filler cartage etc complete. (2.020 Kg) (P.H.S.I No. E P-32) |
| --- | --- |

3" dia:

04 Nos:	@ Rs. 945/=					Each		Rs. 3,780/=

| 15. | C.I bend with flange and with hole including turning and facing of flanges for all size (P.H.S of Mat: I No: 8 P-11). |
| --- | --- |

3" dia:

1.0 Cwt	@ Rs. 6096/=				P.Cwt		Rs. 6,096/=

| 16. | Jointing C.I /M.S Flanged Pipes and special Flange inside trenches including I/C applying rubber packing of required thickness nuts and bolts with washer etc. and other rules required for jointing and testing be same to be specified presser etc.. complete (P.H.S.I No: 1 P-40). |
| --- | --- |

3" dia

6 Joint	@ Rs. 499/=					P.Joint	Rs. 2,994/=

Total		Rs. 11,39,161/=