SCHEDULE “B”

REHABILITATION OF RURAL WATER SUPPLY SCHEMES

DISTRICT THARPARKAR

ABSTRACT OF COST

Part-1	Sobharo Shah New					Rs. 32,40,074/=

Part-2	Dambhario 							Rs. 52,84,358/=

Part-3	Kantio  							Rs. 96,17,616/=

Part-4	Mithrio Halepota 						Rs. 38,89,644/=

Total 		Rs. 2,20,31,692/=

| CONTRACTOR |  | EXECUTIVE ENGINEER PUBLIC HEALTH ENGG: DIVISION THARPARKAR @ MITHI |
| --- | --- | --- |

PART – 1 	SOBHARO SHAH NEW

PART-A	PUMPING MACHINERY (7.5 BHP)

| 1. NSI | Providing and Installing in position i/c transportation site of work at Sobharo Shah New Golden-Saer (Original Saer Italy) Submersible pump type NS-95 EA/24 type 4" (2") with 7.5 BHP 3-phase Motor complete capable to discharge 35 GPM against a head of 350 ft 7.5 HP Golden Motor (3x380Vx2 Pole) i/c riser pipe of 2" dia x 28 Nos. of 10 ft each with flanged ends. The flanges are notched to accommodate the cable i/c top set of 2" dia containing bore hole cover plate with 900 bend, sluice valve, jacket, nuts & bolts & 2 sets of installation clamps (G.I) i/c nipple of 2" dia (G.I) i/c 7.5 HP Motor Control panel consist of metallic box, circuit breaker, phase failure, over load relay, magnetic contractor, dry running protector, ampere/volt meter, led, indication lamps & On/Off switch i/c electrodes and cable 1 x 1.5 mm2 310 ft, electric drop cable 1 x 16 mm2 3 core x 300 ft. wire rope with clamps 600 ft , Cable Clams 28 Nos: Cable Termination Kit (CTK) x 01 No: and making necessary arrangement for testing of the complete pumping sets against the required head for 72 hours etc complete. |
| --- | --- |

2 Set			@ Rs. 7,79,625/=		P.Set			Rs.15,59,250/=

(Rupees Seven Lac Seventy Nine Thousand Six

Hundred Twenty Five) only (Per Set)

Total		Rs. 15,59,250/=

PART-B	REPAIR OF DISTRIBUTION SYSTEM

| 1. | Providing PVC pipe of class “B” equivalent make fixing in trench i/c cutting, fitting and jointing with “Z” Joint with one rubber ring i/c testing with water to head of 61 meter or 200 ft. |
| --- | --- |

4" Dia

2500.0 Rft		@ Rs. 137/=			P.Rft			Rs. 3,42,500/=

(Rupees One Hundred Thirty Seven) only (Per Rft)

3" Dia

5600.0 Rft		@ Rs. 90/=			P.Rft			Rs. 5,04,000/=

(Rupees Ninety) only (Per Rft)

| 2. | PVC “Z” Joint fitting. |
| --- | --- |

(i)	PVC Tee  4" dia

6 Nos:			@ Rs. 1181/=    		Each			Rs. 7,086/=

(Rupees One Thousand One Hundred Eighty One)

only (Each)

(ii)	PVC Tee  3" dia

14 Nos:		@ Rs. 900/=    		Each			Rs. 12,600/=

(Rupees Nine Hundred ) only (Each)

(iii)	End Plug 3" dia

16 Nos:		@ Rs. 382/=    		Each			Rs. 6,112/=

(Rupees Three Hundred Eighty Two) only (Each)

(iv)	PVC Reducer 4" dia

4 Nos:			@ Rs. 1258/=    		Each			Rs. 5,032/=

(Rupees One Thousand Two Hundred Fifty Eight)

only (Each)

(v)	PVC Bend 4" dia

5 Nos:			@ Rs. 956/=    		Each			Rs. 4,780/=

(Rupees Nine Hundred Fifty Six) only (Each)

(vi)	PVC Bend 3" dia

1 x 10	 = 10 Nos:	@ Rs. 731/=    		Each			Rs. 7,310/=

(Rupees Seven Hundred Thirty One) only (Each)

| 3. | C.I sluice valve heavy pattern test pressure 21 kg/sq:cm or 300 lbs/sq: inch. (Imported) |
| --- | --- |

4" Dia

3 Nos:			@ Rs. 5460/=		Each			Rs. 16,380/=

(Rupees Five Thousand Four Hundred Sixty) only

(Each)

3" Dia

5 Nos:			@ Rs. 4290/=		Each			Rs. 21,450/=

(Rupees Four Thousand Two Hundred Ninety) only

(Each)

| 4. | Cast iron special for A.C pressure pipe (Standard Weight A.C.I.L “B” Class). |
| --- | --- |

(i)	Short Piece/Tail Piece

4" dia

6 No:			@ Rs. 487/50    		Each			Rs. 2,925/=

(Rupees Four Hundred Eighty Seven and Paisas

Fifty)  only (Each)

(ii)	Short Piece/Tail Piece

3" dia

10 Nos:		@ Rs. 375/=    		Each			Rs. 3,750/=

(Rupees Three Hundred Seventy Five) only

(Each)

| 5. | Jointing C.I /M.S Flanged Pipes and special Flange inside trenches including I/C applying rubber packing of required thickness nuts and bolts with washer etc. and other rules required for jointing and testing be same to be specified presser etc complete. |
| --- | --- |

4" dia

6 Joints		@ Rs. 513/= 	 		P.Joint			Rs. 3,078/=

(Rupees Five Hundred Thirteen) only

(Per Joint)

3" dia

10 Joints		@ Rs. 499/=  			P.Joint			Rs. 4,990/=

(Rupees Four Hundred Ninety Nine) only

(Per Joint)

| 6. | Making joints to PVC specials fitting including laying of specials and cost of solvent cement of required diameter and testing the joints with along with PVC pipe “B, “C” & “D” class to the specied pressure and making good o all leaky joints etc complete. |
| --- | --- |

3" dia

44 Joints		@ Rs. 59/=  			P.Joint			Rs. 2,596/=

(Rupees Fifty Nine) only (Per Joint)

4" dia

21 Joints		@ Rs. 70/=  			P.Joint			Rs. 1,470/=

(Rupees Seventy) only (Per Joint)

| 7. | Excavation for pipe line trenches and pits in sandy soil including trimming and dressing sides to true alignment and shape leveling of beds of trenches to correct level and grade, cutting joint holes and disposal of surplus earth within a one chain as directed by Engineer Incharge. Providing fence guards, lights, flags, temporary crossings for non vehicular traffic where ever required lift upto 5 ft (1.52 m) and lead upto one chain (30.5 m). |
| --- | --- |

48600.0 Cft		@ Rs. 3750/=		P‰ Cft		Rs. 1,82,250/=

(Rupees Three Thousand Seven Hundred Fifty)

Only (Per Thousand Cft)

| 8. | Refilling the excavated stuff in trenches 6" thick layer i/c watering ramming to full compaction etc complete. |
| --- | --- |

43740.0 Cft		@ Rs. 2760/=		P‰ Cft		Rs. 1,20,722/=

(Rupees Two Thousand Seven Hundred Sixty)

Only (Per Thousand Cft)

| 9. | Dismantling of pucca brick work in lime or cement mortar. |
| --- | --- |

294.0 Cft		@ Rs. 1285/63		P% Cft		Rs. 3,780/=

(Rupees One Thousand Two Hundred Eighty Five)

Only (Per Hundred Cft)

| 10. | Cement Concrete plain including placing, compacting, finishing and curing complete (including screening and washing of stone aggregate with out shutting. |
| --- | --- |

Ratio (1:3:6)

65.34 Cft		@ Rs. 12595/=		P% Cft		Rs. 8,230/=

(Rupees Twelve Thousand Five Hundred Ninety Five

only (Per Hundred Sft)

| 11. | Pucca brick work in foundation & plinth in cement sand mortar (1:6) |
| --- | --- |

357.00 Cft		@ Rs. 11948/36		P% Cft		Rs. 42,656/=

(Rupees Eleven Thousand Nine Hundred Forty Eight

and Paisas Thirty Six) only (Per Hundred Cft)

| 12. | Reinforced cement concrete work including all labour and material except the cost of steel reinforcement and its labour for bending and binding, which will be paid separately. This rate also includes all kinds of forms moulds lifting shuttering curing rendering and finishing the exposed surface (including screening and washing of shingle) (a) RCC work in roof slab beams columns, rafts, lintels and other structural members laid in situ or precast laid in position complete in all respects Ratio (1:2:4) 90 lbs cement 2 Cft sand and 4 Cft shingle 1/8” to ¼” gauge. |
| --- | --- |

Ratio (1:2:4)

61.0 Cft		@ Rs. 337/=			P.Cft			Rs. 20,557/=

(Rupees Three Hundred Thirty Seven) only (Per Cft)

| 13. | Fabrication of Mild steel reinforcement for cement concrete including cutting bending laying in position, making joints & fastenings including cost of binding wire (also includes removal of rust from bars) (USING TAR BARS) |
| --- | --- |

2.45 Cwt		@ Rs. 5001/70		P.Cwt			Rs. 12,254/=

(Rupees Five Thousand One and Paisas Seventy)

only (Per Cwt)

| 14. | Cement Plaster (1:4) upto 12’ height 1/2" thick. |
| --- | --- |

392.00 Sft		@ Rs. 2283/93 		P% Sft			Rs. 8,953/=

(Rupees Two Thousand Two Hundred Eighty Three

and Paisas Ninety Three) only (Per Hundred Sft)

Total		Rs. 13,45,461/=

PART-C	REPAIR OF PUMP HOUSE

| 1. | Removing Cement or Lime Plaster. |
| --- | --- |

1057.00 Sft		@ Rs. 121/=			P% Sft			Rs. 1,279/=

(Rupees One Hundred Twenty One

only (Per Hundred Sft)

| 2. | Making and fixing steel grated door with 1/16" thick sheeting i/c angle iron frame 2" x 2" x 3/8" & 3/4" squire bar 4" center to center with locking arrangement. |
| --- | --- |

24.50 Sft		@ Rs. 726/72		P.Sft			Rs. 17,805/=

(Rupees Seven Hundred Twenty Six and Paisas

Seventy Two) only (Per Sft)

| 3. | Cement Plaster (1:6) upto 12’ height 1/2" thick. |
| --- | --- |

960.0 Sft		@ Rs. 2206/60 		P% Sft			Rs. 21,183/=

(Rupees Two Thousand Two Hundred Six

and Paisas Sixty) only (Per Hundred Sft)

| 4. | Cement Plaster (1:4) upto12’ height 3/8" thick. |
| --- | --- |

960.00 Sft		@ Rs. 2197/52		P% Sft			Rs. 21,096/=

(Rupees Two Thousand One Hundred Ninety Seven

and Paisas Fifty Two) only (Per Hundred Sft)

| 5. | Painting Old Surface preparing surface and painting of door and windows any type (including edges) (2 Coats). |
| --- | --- |

194.00 Sft		@ Rs. 1160/06		P% Sft			Rs. 2,251/=

(Rupees One Thousand One Hundred Sixty

and Paisas Zero Six) only (Per Hundred Sft)

| 6. | White washing (a) two coats. |
| --- | --- |

408.00 Sft		@ Rs. 425/84		P% Sft			Rs. 1,737/=

(Rupees Four Hundred Twenty Five and Paisas

Eighty Four) only (Per Hundred Sft)

| 7. | Distempering (3 coats). |
| --- | --- |

2024.00 Sft		@ Rs. 1079/65		P% Sft			Rs. 21,852/=

(Rupees One Thousand Seventy Nine

and Paisas Sixty Five ) only (Per Hundred Sft)

Total	Rs. 87,203/=

PART- D	REPAIR OF L.S.R

| 1. | Removing Cement or Lime Plaster. |
| --- | --- |

2612.00 Sft		@ Rs. 121/=			P% Sft			Rs. 3,161/=

(Rupees One Hundred Twenty One

only (Per Hundred Sft)

| 2. | Cement Concrete plain including placing, compacting, finishing and curing complete (including screening and washing of stone aggregate with out shutting. |
| --- | --- |

Ratio (1:2:4)

361.0 Cft		@ Rs. 14429/29		P% Cft		Rs. 52,090/=

(Rupees Fourteen Thousand Four Hundred

Twenty Nine and Paisas Twenty Nine) only

(Per Hundred Cft)

| 3. | Cement Plaster (1:4) upto 12’ height 1/2" thick. |
| --- | --- |

2612.0 Sft		@ Rs. 2283/93 		P% Sft			Rs. 59,656/=

(Rupees Two Thousand Two Hundred Eighty Three

and Paisas Ninety Three) only (Per Hundred Sft)

| 4. | P/F C.I. manhole cover and frame i/c cost of material. |
| --- | --- |

5.0 Cwt	 	@ Rs. 6985/=		P.Cwt			Rs. 34,925/=

(Rupees Six Thousand Nine Hundred Eighty Five)

only (Per Cwt)

| 5. | Providing G.I Pipe, approved quality. |
| --- | --- |

50.0 Rft		@ Rs. 338/17		P.Rft			Rs. 16,909/=

(Rupees Three Hundred Thirty Eigh and Paisas

Seventeen) only (Per Rft)

| 6. | Manufacturing, Supplying M.S flanges of 3” dia made out M.S Sheet of 3/8” thickness i/c cutting, drilling holes and welding of flanges with G.I/M. S Pipe etc complete. |
| --- | --- |

3" dia

10 Nos:		@ Rs. 500/89		Each			Rs. 5,009/=

(Rupees Five Hundred and Paisas Eighty Nine)

only (Each)

| 7. | C.I bend with flange and with hole including turning and facing of flanges for all size. |
| --- | --- |

3" dia

6.56 Cwt		@ Rs. 6096/=		P.Cwt			Rs. 39,990/=

(Rupees Six Thousand Ninety Six) only (Per Cwt)

| 8. | Supplying and fixing C.I sluice valve heavy pattern. |
| --- | --- |

3" dia

5 Nos:			@ Rs. 4290/=		Each 			Rs. 21,450/=

(Rupees Four Thousand Two Hundred Ninety)

only (Each)

| 9. | Jointing C.I /M.S Flanged Pipes and special Flange inside trenches including I/C applying rubber packing of required thickness nuts and bolts with washer etc. and other rules required for jointing and testing be same to be specified presser etc complete. |
| --- | --- |

3" dia

30 Joint		@ Rs. 499/=			P.Joint		Rs.14,970/=

(Rupees Four Hundred Ninety Nine) only (Per Joint)

Total	Rs. 2,48,160/=

PART – 2 	DAMBHARIO TALUKA NAGARPARKAR

PART-A	PUMPING MACHINERY (7.5 BHP)

| 1. NSI | Providing and Installing in position i/c transportation at site Grund Fos HMA Submersible pump type HVT 8A-30+7.5 HP Motor complete capable to discharge 32 GPM against a head of 380 ft 7.5 HP HMA Motor (3 x 380V x 2 Pole) i/c riser pipe of 2" dia x 30 Nos. of 7.5 ft each with flanged ends. The flanges are notched to accommodate the cable i/c top set of 2" dia containing bore hole cover plate with 900 bend, sluice valve, jacket, nuts & bolts and 2 sets of installation clamps i/c nipple of 2" dia i/c 7.5 HP Motor Control Unit consist of metallic box, circuit breaker, am-meter, am-meter selector switch, volt meter, volt meter selector switch, electronic over current relay, magnetic contactor, under/Over voltage relay (EVR), dry running protection relay, indication lamps & On/Off switch i/c Electrodes & cable 1 x 1.5 mm2 x 175 m Drop cable 3 x 10 mm2 x 175 meter. Cable Clams 30 Nos: Cable Termination Kit (CTK) x 01 No: i/c motor jacket of 7" dia with electric generating set 24 KVA (China made) and making necessary arrangement for testing of the complete pumping sets against the required head for 72 hours etc complete. |
| --- | --- |

1 Set			@ Rs. 13,03,000/=		P.Set			Rs. 13,03,000/=

(Rupees Thirteen Lac Three Thousand) only

(Per Set)

Cost of 01 Set						Total		Rs. 13,03,000/=

03 Sets		@ Rs. 13,03,000/=		P.Set			Rs. 39,09,000/=

PART-B 	PUMP HOUSE SIZE (10 x 12)

| 1. | Removing cement lime plaster |
| --- | --- |

354.0 Sft 		@ Rs. 121/=			P% Sft			Rs. 428/=

(Rupees One Hundred Twenty One) only

(Per Hundred Sft)

| 2. | Cement Plaster (1:6) upto 12' height 1/2" thick (G.S.I.No: 13 (a) P-52). |
| --- | --- |

4138.0 Sft		@ Rs. 2206/60		P% Sft			Rs. 91,309/=

(Rupees Two Thousand Two Hundred Six and

Paisas Sixty) only (Per Hundred Sft)

| 3. | Distempering (b) two coat (G.S.I.No. 24 (b) P-54). |
| --- | --- |

3414.0 Sft		@ Rs. 1043/90		P% Sft		Rs. 35,639/=

(Rupees One Thousand Forty Three

and Paisas Ninety)  only (Per Hundred Sft)

| 4. | Painting old surface preparing surfaces and painting of doors and windows of any type i/c edges two coats.(G.S.I No. 4 (c) P-67). |
| --- | --- |

219.00 Sft		@ Rs. 1160/06 		P% Sft			Rs. 2,540/=

(Rupees One Thousand One Hundred Sixty

and Paisas Zero Six)  only (Per Hundred Sft)

Total		Rs. 1,29,916/=

PART-C 	INTER CONNECTION

| 1. | Providing, Laying, jointing and testing G.I Pipe of standard approved quality with thickness 6SWG test pressure 725 PSI (R.A.A) (based on schedule) |
| --- | --- |

3" dia

112.0 Rft		@ Rs. 338/17		P.Rft			Rs. 37,875/=

(Rupees Three Hundred Thirty Eight and Paisas

Seventeen) only (Per Rft)

| 2. | Providing fixing M.S flanges made of M.S sheet 3/8'' thick etc complete R.A.A (Based as Sch: Item) |
| --- | --- |

3" dia

26 Nos		@ Rs. 500/89		Each			Rs. 13,023/=

(Rupees Five Hundred and Paisas Eighty Nine)

only  (Each)

| 3. | C.I bend with flange and with hole including turning and facing of flanges for all size (P.H.S of Mat: I No: 8 P-11) |
| --- | --- |

3" dia

3.412 Cwt		@ Rs. 6096/=		P.Cwt			Rs. 20,800/=

(Rupees Six Thousand Ninety Six)  only (P.Cwt)

| 4. | Jointing C.I /M.S Flanged Pipes & special Flange inside trenches including applying rubber packing of required thickness nuts & bolts with washer etc. & other rules required for jointing and testing be same to be specified presser etc. complete (P.H.S.I No: 1 P-40). |
| --- | --- |

3" dia

52 Joint		@ Rs. 499/=			P.Joint		Rs. 25,948/=

(Rupees Four Hundred Ninety Nine) only (P.Joint)

Total		Rs. 97,646/=

PART-D 	U.PVC PIPE 4" & 3" DIA.

| 1. | Providing PVC pipe of class “B” equivalent make fixing in trench i/c cutting, fitting and jointing with “Z” Joint with one rubber ring i/c testing with water to head of 61 meter or 200 ft. (P.H.S.I. No. 1 P-22). |
| --- | --- |

4" Dia

2000.0 Rft		@ Rs. 137/=			P.Rft			Rs.2,74,000/=

(Rupees One Hundred Thirty Seven) only (Per Rft)

3" Dia

5000.0 Rft		@ Rs. 90/=			P.Rft			Rs. 4,50,000/=

(Rupees Ninety) only (Per Rft)

| 2. | C.I sluice valve heavy pattern test pressure 21 kg/sq:cm or 300 lbs/sq: inch. (P.H.S. of Mat: Item No. 2 P-12). |
| --- | --- |

4" Dia

02 Nos:		@ Rs. 5460/=		Each			Rs. 10,920/=

(Rupees Five Thousand Four Hundred Sixty) only

(Each)

3" Dia

10 Nos:		@ Rs. 4290/=		Each			Rs. 42,900/=

(Rupees Four Thousand Two Hundred Ninety) only

(Each)

| 3. | PVC “Z” Joint fitting (P.H.S.O.M.Item No. 18 P-28). |
| --- | --- |

(i)	PVC Tee  3" dia

2 Nos:			@ Rs. 900/=    		Each			Rs. 1,800/=

(Rupees Nine Hundred) only (Each)

(ii)	End Plug 3" dia

2 Nos:			@ Rs. 175/=    		Each			Rs. 350/=

(Rupees One Hundred Seventy Five) only (Each)

(iii)	Valve socket 3" dia

20 No:			@ Rs. 350/=    		Each			Rs. 7,000/=

(Rupees Three Hundred Fifty) only (Each)

(iv)	Valve socket 4" dia

4 Nos:			@ Rs. 500/=    		Each			Rs. 2,000/=

(Rupees Five Hundred) only (Each)

| 4. | Cast iron special for A.C pressure pipe (Standard Weight A.C.I.L “B” Class) (P.H.S.O.M.I No:9 P-10). |
| --- | --- |

(i)	Short Piece/Tail Piece

4" dia

04 Nos:		@ Rs. 487/50    		Each			Rs. 1,950/=

(Rupees Four Hundred Eighty Seven and Paisas

Fifty)  only (Each)

3" dia

20 Nos:		@ Rs. 375/=    		Each			Rs. 7,500/=

(Rupees Three Hundred Seventy Five) only

(Each)

| 5. | Jointing C.I /M.S Flanged Pipes and special Flange inside trenches including I/C applying rubber packing of required thickness nuts and bolts with washer etc. and other rules required for jointing and testing be same to be specified presser etc.. complete (P.H.S.I No: 1 P-40). |
| --- | --- |

4" dia

4 Joints		@ Rs. 513/=			P.Joint			Rs. 2,052/=

(Rupees Five Hundred Thirteen) only

(Per Joint)

3" dia

20 Joints		@ Rs. 499/=  			P.Joint			Rs. 9,980/=

(Rupees Four Hundred Ninety Nine) only (Per Joint)

| 6. | Making joints to PVC specials fitting including laying of specials and cost of solvent cement of required diameter and testing the joints with along with PVC pipe “B, “C” & “D” class to the specied pressure and making good o all leaky joints etc complete.. (P.H.S.I.No. 1 P-41). |
| --- | --- |

3" dia

4 Joint			@ Rs. 59/=  			P.Joint			Rs. 236/=

Rupees Fifty Nine) only (Per Joint)

| 7. | Excavation for pipe line trenches and pits in sandy soil including trimming and dressing sides to true alignment and shape leveling of beds of trenches to correct level and grade, cutting joint holes and disposal of surplus earth within a one chain as directed by Engineer Incharge. Providing fence guards, lights, flags, temporary crossings for non vehicular traffic where ever required lift upto 5 ft (1.52 m) and lead upto one chain (30.5 m). (P.H.S.I.No. 2 P-60). |
| --- | --- |

42000.0 Cft		@ Rs. 3750/=		P‰ Cft		Rs. 1,57,500/=

(Rupees Three Thousand Seven Hundred Fifty)

Only (Per Thousand Cft)

| 8. | Refilling the excavated stuff in trenches 6" thick layer i/c watering ramming to full compaction etc complete. (P.H.S.I.No.24 P-77). |
| --- | --- |

37800.0 Cft		@ Rs. 2760/=	P‰ Cft			Rs. 1,04,328/=

(Rupees Two Thousand Seven Hundred Sixty)

Only (Per Thousand Cft)

| 9. | Providing chamber 3' x 2' (915 x 615 mm) inside dimension 4 ½' (1372 mm) deep as per approved design for sluice valve 3" to 12" dia with 18" (457 mm) dia inside cost iron cover and frame (wt = 1 cwt 3 qr) fixed in RCC 1:2:4 (102 mm) thick (with 5 Lbs steel per cft) 9" (299 mm) thick brick masonry wall set in 1:6 cement mortar 6" (1152 mm) thick cement concrete 1:3:6 in foundation 1" (25 mm) thick cement concrete 1:2:4 flooring ½" (12.5 mm) thick cement plaster 1:3 to all inside wall surface and to top i/c providing and fixing M.S foot rest at every one foot beyond 2 ½ ft depth curing, excavation, back filling and disposal of surplus earth etc complete. (P.H.S.I.No. 1 P-49). |
| --- | --- |

4 Nos:			@ Rs. 18820/=	Each				Rs. 75,280/=

(Rupees Eighteen Thousand Eight Hundred Twenty)

Only (Each)

Total		Rs. 11,47,796/=

PART – 3 	KANTIO TALUKA CHACHRO

PART-A	CONSTRUCTION OF TUBE WELL

| 1. NSI | Mobilization tube well construction. |
| --- | --- |

1-Tripe		@ Rs. 31570/=		P.Trip			Rs. 31,570/=

(Rupees Thirty One Thousand Five Hundred

Seventy)  only (Per Trip)

| 2. NSI | Drill for tube well at 16" inch minimum dia meter from ground level up to 100' depth. |
| --- | --- |

100 ft			@ Rs. 1997/12		P.ft			Rs. 1,99,712/=

(Rupees One Thousand Nine Hundred Seventy Nine

and Paisas Twelve) only (Per ft)

| 3. NSI | Drill for tube well at 16" inch minimum dia meter from 100' depth maximum 200' ft depth. |
| --- | --- |

100 ft			@ Rs. 2302/53		P.ft			Rs. 2,30,253/=

(Rupees Two Thousand Three Hundred Two and

Paisas Fifty Three) only (Per ft)

| 4. NSI | Drill hole to 16" dia 200' ft to 300' depth. |
| --- | --- |

100 ft			@ Rs. 2607/95		P.ft			Rs. 2,60,795/=

(Rupees Two Thousand Six Hundred Seven

and Paisas Ninety Five) only (Per ft)

| 5. NSI | Drill hole 16" dia 300' ft to ft (300' ft 750' ft). |
| --- | --- |

50 ft			@ Rs. 2913/36		P.ft			Rs. 1,45,668/=

(Rupees Two Thousand Nine Hundred Thirteen

and Paisas Thirty Six) only (Per ft)

| 6. | Supplying and install 8" nominal dia under PVC class “D” pump housing casing (Blind Pipe). |
| --- | --- |

230 ft			@ Rs. 934/70		P.ft			Rs. 2,14,981/=

(Rupees Nine Hundred Thirty Four and Paisas

Seventy) only (Per ft)

| 7. | Supplying and install 8" nominal diameter PVC class “D” slatted. |
| --- | --- |

120 ft			@ Rs. 1005/79		P.ft			Rs. 1,20,695/=

(Rupees One Thousand Five and Paisas Seventy Nine)

only (Per ft)

| 8. | Supply and install 8" nominal dia meter plug 5'-0 ft long complete i/c casing and cap 7 concrete plug. |
| --- | --- |

01 No:			@ Rs. 6033/50		Each			Rs. 6,034/=

(Rupees Six Thousand Thirty Three and Paisas Fifty)

only (Each)

| 9. NSI | Supply and install well head casing and security cap. |
| --- | --- |

01 No:			@ Rs. 4854/=		Each			Rs. 4,854/=

(Rupees Four Thousand Eight Hundred Fifty Four)

only (Per ft)

| 10. | Supply and place graded bajri (3/8" x 1/8") pack in well. (P.H.S.I.No. 13 (h) P-45. |
| --- | --- |

350 ft			@ Rs. 232/=			P.ft			Rs. 81,200/=

(Rupees Two Hundred Thirty Two) only (Per ft)

| 11. NSI | Develop and test well. |
| --- | --- |

01 No:			@ Rs. 107239/=		Each			Rs. 1,07,239/=

(Rupees One Lac Seven Thousand Two Hundred

Thirty Nine) only (Each)

| 12. NSI | Dis-infection of well. |
| --- | --- |

01 No:			@ Rs. 500/=			Each			Rs. 500/=

(Rupees Five Hundred) only (Each)

| 13. NSI | Construction concrete Class “B” around well head casing complete. |
| --- | --- |

01 No:			@ Rs. 3035/=		Each			Rs. 3,035/=

(Rupees Three Thousand Thirty Five) only (Each)

Cost of 01 No: Tube Well							Rs. 14,06,536/=

2 No:			@ Rs. 14,06,536/=					Rs. 28,13,072/=

For Mobilization 						(-)		Rs. 31,570/=

Net		Rs. 27,81,502/=

PART-B	PUMPING MACHINERY (7.5 BHP)

| 1. NSI | Providing and Installing in position i/c transportation site of work at Kantio Grund Fos HMA Submersible pump type HSP 8A-25+7.5 HP Motor complete capable to discharge 40 GPM against a head of 350 ft 7.5 HP HMA Motor (3 x 400V x 2 Pole) i/c G.I. riser pipe of 2" dia x 30 Nos. of 10 ft each with flanged ends. The flanges are notched to accommodate the cable i/c top set of 2" dia containing bore hole cover plate with 900 bend, sluice valve, jacket, nuts & bolts and 2 sets of installation clamps (G.I) i/c nipple of 2" dia (G.I) i/c 7.5 HP Motor Control Unit consist of metallic box, circuit breaker, am-meter, am-meter selector switch, volt meter, volt meter selector switch, electronic over current relay, magnetic contactor, under/Over voltage relay (EVR), dry running protection relay, indication lamps & On/Off switch i/c Electrodes & cable 1 x 1.5 mm2 x 100 m Drop cable 3 x 4 mm2 x 100 meter. Cable Clams 30 Nos: Cable Termination Kit (CTK) x 0.1 No: and making necessary arrangement for testing of the complete pumping sets against the required head for 72 hours etc complete. |
| --- | --- |

1 Set			@ Rs. 8,97,360/=		P.Set			Rs. 8,97,360/=

(Rupees Eight Lac Ninety Seven Thousand

Three Hundred Sixty) only (Per Set)

Cost of 01 Set						Total		Rs. 8,97,360/=

3 Set			@ Rs. 897360/=		P.Set			Rs. 26,92,080/=

PART-C  	ELECTRIC PUMPING SET 15 BHP COUPLED WITH KSB CENTRIFUGAL PUMP

| 1. NSI | Providing, installing and testing in position including transportation to site of work @ Kantio Electric Pumping set consisting of Electric Motor 15 BHP 3-Phase, 50 cycle, 400/420 volts of 2900 RPM (Siemens Made) coupled with KSB centrifugal pump type ETA Norm 80-200 size 4" x 3" mounted on common steel frame or base plate capable to give discharge 190 IGPM against a head of 80 ft i/c installing of pumping set on cement concrete foundation ratio 1:2:4 with M.S nuts and bolts of 5/8" dia to be embedded in C.C foundation including P/F star delta starter 100 ampere (Green Power) with Motor control Unit consist of circuit breaker main switch 150 amperes (Siemens) automatic star delta starter under over voltage relay, phase failure relay, ampere meter 60 amperes, volt meter 500 volts indicating lights all fixed in metallic vernyle proof box. Providing internal wiring for mains with 2-7/0.044 PVC insulated wire 660 volts grade in 16 mm (1") PVC conduit fitted on surface as required including Providing/fixing earthen set with 0.6 M x 0.6 M x 6.3 mm (2 ft x 2 ft x ¼") copper plate buried in the ground at a depth of 3.7 M (12 ft) or less as per PWD specification and testing the pumping set against the required head and discharge for 72 hours etc complete. |
| --- | --- |

02 Sets		@ Rs. 3,85,535/=		P.Set		         Rs. 7,71,070/=

(Rupees Three Lac Eighty Five Thousand

Five Hundred Thirty Five) only (Per Set)

| 2. | Providing G.I Pipe, specials and clamps etc i/c fixing, cutting and fitting complete with end i/c the cost of breaking through walls and roof making good etc. painting tow coats after cleaning the pipe etc. with white zinc paint with pigment to match the colour of the building and testing with water to a pressure head of 200 ft and handling (S.S.I No. 1 P-12). |
| --- | --- |

4" dia

36.0 Rft		@ Rs. 896/17		P.Rft			Rs. 32,262/=

(Rupees Eight Hundred Ninety Six and

Paisas Seventeen) only (Per Rft)

6" dia

23.0 Rft		@ Rs. 818/86		P.Rft			Rs. 18,834/=

(Rupees Eight Hundred Eighteen and

Paisas Eighty Six) only (Per Rft)

| 3. | Providing M.S flanges made out 3/8" (10 mm) thick sheet i/c cutting drilling holes etc complete (R.A attached) based on schedule. |
| --- | --- |

4" dia

8 Nos:			@ Rs. 596/75		Each			Rs. 4,774/=

(Rupees Five Hundred Ninety Six and

Paisas Seventy Five) only (Each)

6" dia

6 Nos:			@ Rs. 804/77		Each			Rs. 4,829/=

(Rupees Eight Hundred Four and

Paisas Seventy Seven) only (Each)

| 4. | Supplying and fixing main circuit breakers (15 to 100 amperes) fixing on prepared board as required. (S.E.I No.207 P-31). |
| --- | --- |

1 Nos:			@ Rs. 9261/=		Each			Rs. 9,261/=

(Rupees Nine Thousand Two Hundred Sixty One)

only (Each)

| 5. | Supplying C.I sluice valve heavy pattern test pressure 300 lbs/sq: inch (imported) (S.M.I No.2 P-12). |
| --- | --- |

3" dia

2 Nos:			@ Rs. 4290/=		Each			Rs. 8,580/=

(Rupees Four Thousand Two Hundred Ninety)

only (Each)

| 6. | Reflex valve heavy pattern (Test pressure 21.0 kg/sq cm or 300 Lbs/sq inch (imported) (PHS M.I No.6 P-12). |
| --- | --- |

3" Dia

2 Nos:			@ Rs. 1543/75		Each			Rs. 3,088/=

(Rupees One Thousand Five Hundred Forty Three

and Paisas Seventy Five) only (Each)

| 7. | Supplying/fixing foot valve heavy pattern with cone type gate (G.S.I. No. 4 P-12). |
| --- | --- |

4" Dia

2 Nos:			@ Rs. 893/75		Each			Rs. 1,788/=

(Rupees Eight Hundred Ninety Three and

Paisas Seventy Five) only (Each)

| 8. | C.I bend with flanged end with holes including turning and facing flanges for all size (S.M.I. No. 8 P-13). |
| --- | --- |

1.10 Cwt		@ Rs. 6096/=		P.Cwt			Rs. 6,706/=

(Rupees Six Thousand Ninety Six) only (Per Cwt)

| 9. | C.I Tee flange ends with holes i/c curing and facing of flanged for all size (SOM Item No. 12 P-13). |
| --- | --- |

Tee 6" x 6" x 3"

0.806 Cwt		@ Rs. 6096/=		P.Cwt			Rs. 4,913/=

(Rupees Six Thousand Ninety Six) only (Per Cwt)

| 10. | Jointing C.I/M.S flanged pipe and special and in jointing trenches i/c supply rubber packing of the required thickness nuts bolts with washier etc and other tools required for jointing and testing joint to the specified etc complete.(PHI NO. 1 P-40). |
| --- | --- |

3" Dia

8 Joints		@ Rs. 499/=			P.Joint		Rs. 3,992/=

(Rupees Four Hundred Ninety Nine) only

(P.Joint)

4" Dia

8 Joints		@ Rs. 513/=			P.Joint		Rs. 4,104/=

(Rupees Five Hundred Thirteen) only

(P.Joint)

6" Dia

8 Joints		@ Rs. 938/=			P.Joint		Rs. 7,504/=

(Rupees Nine Hundred Thirty Eight) only

(P.Joint)

Total 		Rs. 8,81,705/=

PART-D	PUMP HOUSE SIZE (8' X 8')

| 1. | Excavation in foundation of building bridges and other structure i/c dag belling dressing, refilling around structure with excavated earth watering and ramming lead up to one chain and lift up to 5 ft.(G.S.I.No: 18 P-4). |
| --- | --- |

184.00 Cft		@ Rs. 2722/50		P‰ Cft		Rs. 501/=

(Rupees Two Thousand Seven Hundred Twenty Two

and Paisas Fifty)  only (Per Thousand Cft)

| 2. | Cement concrete plain including placing, compacting, finishing and curing complete (including screening and washing of stone aggregate without shuttering (G.S.I.No: 5 (i) P-16). |
| --- | --- |

65.00 Cft		@ Rs. 11288/75		P% Cft		Rs. 7,338/=

(Rupees Eleven Thousand Two Hundred Eighty Eight

and Paisas Seventy Five)  only (Per Thousand Cft)

| 3. | Pucca brick work in foundation and plinth in (1:6). (G.S.I.No.4 P-21). |
| --- | --- |

118.0 Cft		@ Rs. 11948/36 		P% Cft		Rs. 14,099/=

(Rupees Eleven Thousand Nine Hundred Forty Eight

and Paisas Thirty Six)  only (Per Hundred Cft)

| 4. | Pucca brick work in ground floor in cement sand mortar (1:6) (G.S.I.No. 5 P-21). |
| --- | --- |

234.00 Cft		@ Rs. 12674/36		P% Cft		Rs. 29,658/=

(Rupees Twelve Thousand Six Hundred Seventy Four

and Paisas Thirty Six)  only (Per Hundred Cft)

| 5. | Reinforced cement concrete work i/c all labour and material except the cost of steel reinforcement and its labour for bending and binding which will be paid separately. This rate also i/c of forms moulds lifting shuttering curing rendering and finishing the exposed surfaces i/c screening and washing of shingle 1/8" to 1/4" gauge.(P.H.S. I. No: 6 P-17). |
| --- | --- |

90.00 Cft		@ Rs. 337/=			P.Cft			Rs. 30,330/=

(Rupees Three Hundred Thirty Seven) only

(Per Cft)

| 6. | Fabrication of mild steel reinforcement for cement concrete i/c cutting bending lying in position making joints and fastening i/c cost of binding wire also i/c removal of rust from bars. (G.S.I.No. 6 P-17). |
| --- | --- |

3.61 Cwt		@ RS. 5001/70 		P.Cwt			Rs. 18,056/=

(Rupees Five Thousand One and Paisas Seventy)

only (Per Cwt)

| 7. | Filling, watering and ramming earth under floor with surplus earth from foundation lead upto one chain and lift upto 5 feet.(G.S.I.No. 21 P-4). |
| --- | --- |

131.00 Cft		@ Rs. 1512/50 		P‰ Cft		Rs. 198/=

(Rupees One Thousand Five Hundred Twelve and

Paisas Fifty)  only (Per Thousand Cft)

| 8. | Making and fixing steel grated door with 1/16" thick sheeting including and iron frame 2"x 2" x 3/8" and ¾" square bars 4" center to center with locking arrangement. (G.S.I.No.24 P-91). |
| --- | --- |

21.00 Sft		@ Rs. 726/72 	P.Sft				Rs. 15,261/=

(Rupees Seven Hundred Twenty Six and

Paisas Seventy Two)  only (Per Sft)

| 9. | Fabrication of heavy steel work with angles, tees, flat iron, iron and sheet iron for making trusses, girders, tanks etc, including cutting riveting, handing assembling and fixing but eluding erection in position (G.S.I.No. 2 P-91). |
| --- | --- |

2.03 Cwt		@ Rs. 4928/49 	P.Cwt				Rs. 10,005/=					(Rupees Four Thousand Nine Hundred Twenty Eight

and Paisas Forty Nine) only (Per Cwt)

| 10. | Supplying and fixing iron steel grill of 3/4" x 3/4" x 1/8" slice flat iron of approved design i/c paining 3 coats etc complete weight not to be less than 3.70 lbs per sq: ft of finished grill. (G.S.I.No. 26 P-93). |
| --- | --- |

24.00 Sft		@ Rs. 180/50 	P.Sft				Rs. 4,332/=

(Rupees One Hundred Eighty and Paisas Fifty)

only (Per Sft)

| 11. | Cement Plaster (1:6) upto 12’ height 1/2" thick (G.S.I.No: 13 (b) P-52). |
| --- | --- |

379.0 Sft		@ Rs. 2206/60   	P% Sft				Rs. 8,363/=

(Rupees Two Thousand Two Hundred Six and

Paisas Sixty) only (Per Hundred Sft)

| 12. | Cement Plaster (1:4) upto 12’ height 3/8" thick (G.S.I.No: 11 (a) P-52). |
| --- | --- |

379.00 Sft		@ Rs. 2197/52  	P% Sft		 		Rs. 8,329/=

(Rupees Two Thousand One Hundred Ninety Seven

and Paisas Fifty Two) only (Per Hundred Sft)

| 13. | Cement pointing (1:3) struck joint on wall (G.S.I.No. 19 (b) P-52). |
| --- | --- |

358.00 Sft		@ Rs. 1213/58  	P% Sft		 		Rs. 4,345/=

(Rupees One Thousand Two Hundred Thirteen and

Paisas Fifty Eight) only (Per Hundred Sft)

| 14. | Providing and laying 2" thick topping concrete ratio (1:2:4) surface finishing and dividing into panels 2" thick (G.S.I.No. 16 (c) P-41). |
| --- | --- |

190.00 Sft		@ Rs. 3275/50 	P% Sft				Rs. 6,223/=

(Rupees Three Thousand Two Hundred Seventy Five

and Paisas Fifty) only (Per Hundred Sft)

| 15. | P/F C.I. main hole covering frame i/c cost of material (P.H.S.I.No: 1 P-35). |
| --- | --- |

1.75 Cwt		 @ Rs. 6985/=	P.Cwt				Rs. 12,224/=

(Rupees Six Thousand Nine Hundred Eighty Five)

only (Per Cwt)

| 16. | Painting new surface preparing surfaces and painting of doors and windows of any type i/c edges two coats.(G.S.I No. 4 P-69). |
| --- | --- |

42.00 Sft		@ Rs. 2116/41 	P% Sft				Rs. 889/=

(Rupees Two Thousand One Hundred Sixteen

and Paisas Forty One) only (Per Hundred Sft)

| 17. | White washing (a) one coat (G.S.I.No. 26 P-54). |
| --- | --- |

379.00 Sft		@ Rs. 416/63	P% Sft				Rs. 1,579/=

(Rupees Four Hundred Sixteen and Paisas

Sixty Three)  only (Per Hundred Sft)

(b) Three Coats.

106.00 Sft		@ Rs. 829/95	P% Sft				Rs. 880/=

(Rupees Eight Hundred Twenty Nine and Paisas

Ninety Five)  only (Per Hundred Sft)

| 18. | Colour washing (2 coats) (G.S.I.No. 25 (b) P-54). |
| --- | --- |

737.00 Sft		@ Rs. 859/90	P% Sft				Rs. 6,337/=

(Rupees Eight Hundred Fifty Nine and Paisas

Ninety)  only (Per Hundred Sft)

| 19. | S/F M/S. Girder size 8" x 4". (Sch: Mat: Item 140 P-6) |
| --- | --- |

1.527 Cwt		@ Rs. 3850/=	P.Cwt				Rs. 5,879/=

(Rupees Three Thousand Eight Hundred Fifty)

only (Per Cwt)

Cost of 01 No: Pump House				Total		Rs. 1,84,826/=

###### 2 No:			@ Rs. 	1,84,826/=		Each			Rs. 3,69,652/=

PART-E	BRICK MASONRY PUMP HOUSE (14' X 12') (1 NO:)

| 1. | Excavation in foundation of building bridges on other structure i/c dag-belling, dressing, refilling around the excavated earth watering and ramming lead upto one chain lift upto 5 ft (in sandy soil) (G.S.I.No. 18 P-4). |
| --- | --- |

359.00 Cft		@ Rs. 2722/50 	P‰ Cft			Rs. 977/=

(Rupees Two Thousand Seven Hundred Twenty Two

and Paisas Fifty)  only (Per Thousand Cft)

| 2. | Cement Concrete plain including placing, compacting, finishing and curing complete (including screening and washing of stone aggregate with out shutting (G.S.I.No.5 (i) P-16). |
| --- | --- |

135.00 Cft		@ Rs. 11288/75	P% Cft			Rs. 14,240/=

(Rupees Eleven Thousand Two Hundred Eighty Eight

and Paisas Seventy Five)  only (Per Thousand Cft)

| 3. | Pucca brick work in foundation and plinth in cement sand mortar (1:6) (G.S.I. No. 4 P-21). |
| --- | --- |

329.00 Cft		@ Rs. 11948/36	P% Cft			Rs. 39,310/=

(Rupees Eleven Thousand Nine Hundred Forty Eight

and Paisas Thirty Six)  only (Per Hundred Cft)

| 4. | Pucca brick work in ground floor in Cement sand mortar (1:4) (G.S.I. No. 5 P-21). |
| --- | --- |

543.0 Cft		@ Rs. 13227/41	P% Cft			Rs. 71,825/=

(Rupees Thirteen Thousand Two Hundred Twenty

Seven and Paisas Forty One)  only (Per Hundred Cft)

| 5. | Reinforced cement concrete work including all labour and material except the cost of steel reinforcement and its labour for bending and binding, which will be paid separately. This rate also includes all kinds of forms moulds lifting shuttering curing rendering and finishing the exposed surface (including screening and washing of shingle) (a) RCC work in roof slab beams columns, rafts, lintels and other structural members laid in situ or precast laid in position complete in all respects Ratio (1:2:4) 90 lbs cement 2 Cft sand and 4 Cft shingle 1/8” to ¼” gauge. (G.S.I.No. 6 (a) P-16). |
| --- | --- |

208.0 Cft		@ Rs. 337/=		P.Cft				Rs.70,096/=

(Rupees Three Hundred Thirty Seven) only

(Per Cft)

| 6. | Fabrication of Mild steel reinforcement for cement concrete including cutting bending laying in position, making joints & fastenings including cost of binding wire (also includes removal of rust from bars) (G.S.I.No.8 (b) P-17) (USING TAR BARS) |
| --- | --- |

8.35 Cwt		@ Rs. 5001/70	P.Cwt		   		Rs. 41,764/=

(Rupees Five Thousand One and Paisas Seventy)

only (Per Cwt)

| 7. | Filling Watering and ramming earth in floors with surplus earth form foundation lead upto one chain and lift upto 5’ ft: (G.S.I. No. 21 P-5). |
| --- | --- |

237.00 Cft		@ Rs. 1512/50	P‰ Cft			Rs. 358/=

(Rupees One Thousand Five Hundred Twelve and

Paisas Fifty)  only (Per Thousand Cft)

| 8. NSI | Fabrication of heavy steel work with angles, tees, flat iron, iron and sheet iron for making trusses, girders, tanks etc, including cutting riveting, handing assembling and fixing but eluding erection in position (G.S.I.No. 2 P-91). |
| --- | --- |

4.07 Cwt		@ Rs. 4928/49	P.Cwt				Rs. 20,059/=

(Rupees Four Thousand Nine Hundred Twenty Eight

and Paisas Forty Nine) only (Per Cwt)

| 9. | Supplying & fixing in position Iron/ Steel grill of 3/8” x ¼” size flat Iron or approved design i/c painting 3 Coats etc complete (Weight net less than 3.7 lb/sp: ft: of in finished grill. (G.S.I.No.26 P-93). |
| --- | --- |

48.00 Sft		@ Rs. 180/50	P.Sft				Rs. 8,664/=

(Rupees One Hundred Eighty and Paisas Fifty)

only (Per Sft)

| 10. | Making and fixing steel grated door with 1/16" thick sheeting i/c angle iron frame 2" x 2" x 3/8" & 3/4" squire bar 4" center to center with locking arrangement. (G.S.I.No. 24 P-91). |
| --- | --- |

28.00 Sft		@ Rs. 726/72	P.Sft				Rs. 20,348/=

(Rupees Seven Hundred Twenty Six and

Paisas Seventy Two)  only (Per Sft)

| 11. | Cement Plaster (1:6) upto 12’ height 1/2" thick (G.S.I.NO 13 (a) P-52). |
| --- | --- |

1156.00 Sft		@ Rs. 2206/60 	P% Sft				Rs. 25,508/=

(Rupees Two Thousand Two Hundred Six and

Paisas Sixty) only (Per Hundred Sft)

| 12. | Cement Plaster (1:4) upto 12’ height 3/8" thick (G.S.I.NO 11 (a) P-52). |
| --- | --- |

1156.00 Sft		@ Rs. 2197/52	P% Sft				Rs. 25,403/=

(Rupees Two Thousand One Hundred Ninety Seven

and Paisas Fifty Two) only (Per Hundred Sft)

| 13. | Preparing the surface and applying rock wall/shield (Natural wall texture) coating to provider durable crust to wall, thickness b/w 2mm to 32mm (1/8") with acrylic co-polymer emulsion, selected marble chips, adhesive and bacteicdes, water resistance and fire and termite resistance (Upto 20'-0 height) (G.S.I.No. 43 P-56). |
| --- | --- |

695.00 Sft		@ Rs. 4504/50		P% Sft			Rs. 31,306/=

(Rupees Four Thousand Five Hundred Four and

Paisas Fifty) only (Per Hundred Sft)

| 14. | Providing and laying 1” thick topping cement concrete (1:2:4) including surface finishing and dividing into panels. (G.S.I. No: 16 (c) P-42). |
| --- | --- |

2" thick

168.0 Sft		@ Rs. 3275/50	P% Sft				Rs. 5,503/=

(Rupees Three Thousand Two Hundred Seventy Five

and Paisas Fifty) only (Per Hundred Sft)

1 ½" thick

296.56 Sft		@ Rs. 2548/29	P% Sft				Rs. 7,557/=

(Rupees Two Thousand Five Hundred Forty Eight

and Paisas Twenty Nine) only (Per Hundred Sft)

| 15. | Distempering 3 Coats (G.S.I .NO. 24 P-60). |
| --- | --- |

729.00 Sft		@ Rs. 1079/65	P% Sft				Rs. 7,871/=

(Rupees One Thousand Seventy Nine and Paisas

Sixty Five) only (Per Hundred Sft)

| 16. | Painting New Surface preparing surface and painting of door and windows any type (including edges) (3 Coats) (G.S.I. No. 5 (c) P-70). |
| --- | --- |

56.00 Sft		@ Rs. 2116/41	P% Sft				Rs. 1,185/=

(Rupees Two Thousand One Hundred Sixteen

and Paisas Forty One) only (Per Hundred Sft)

Total		Rs. 3,91,974/=

PART-F	R.C.C U.G RESERVOIR (23 ft dia) (0.020 MG CAPACITY)

| 1. | Excavation for tanks and reservoirs in sandy Soil i/c trimming and dressing sides to true alignment / design correct / profiles and shape, leveling of bed of tanks to correct level & grade i/c laying of earth in 6” layers for construction of tanks and dressing and disposal of surplus excavated earth within one chain as directed by Engineer Incharge i/c providing fence guards lights flags wherever required lift upto 5 feet and lead upto one chain (P.H.S.I.No.2. P-68). |
| --- | --- |

10557.00 Cft 	@ Rs. 3150/= 	P‰ Cft			Rs. 33,255/=

(Rupees Three Thousand One Hundred Fifty) only

(Per Thousand Cft)

| 2. | Cement Concrete plain including placing compacting finishing and curing complete (including screening and washing of stone aggregate with out shutting (G.S.I.No.5 (h) P-16). |
| --- | --- |

Ratio (1:4:8)

377.00. Cft		@ Rs. 11288/75	P% Cft			Rs. 42,580/=

(Rupees Eleven Thousand Two Hundred Eighty Eight

and Paisas Seventy Five) only (Per Hundred Cft)

| 3. | Reinforced cement concrete work including all labour and material except the cost of steel reinforcement and its labour for bending and binding, which will be paid separately. This rate also includes all kinds of forms moulds lifting shuttering curing rendering and finishing the exposed surface (including screening and washing of shingle) (a) RCC work in roof slab beams columns, rafts, lintels and other structural members laid in situ or precast laid in position complete in all respects Ratio (1:2:4) 90 lbs cement 2 Cft sand and 4 Cft shingle 1/8” to ¼” gauge. (G.S.I.No.6 P-16). |
| --- | --- |

1805.0 Cft		@ Rs. 337/=		P.Cft				Rs. 6,08,285/=

(Rupees Three Hundred Thirty Seven) only

(Per Cft)

| 4. | Fabrication of Mild steel reinforcement for cement concrete including cutting bending laying in position, making joints & fastenings including cost of binding wire (also includes removal of rust from bars) (G.S.I.No.7 (b) P-16) (USING TAR BARS). |
| --- | --- |

80.58 Cwt		@ Rs. 5001/70	P.Cwt				Rs. 4,03,037/=

(Rupees Five Thousand One and Paisas Seventy)

only (Per Cwt)

| 5. | Cement Plaster (1:3) upto 12’ height ¾” thick (G.S.I.No: 10 (c) P-51) (For Water Proofing). |
| --- | --- |

682.00  Sft		@ Rs. 2795/30 	P% Sft				Rs. 19,064/=

(Rupees Two Thousand Seven Hundred Ninety Five

and Paisas Thirty)  only (Per Hundred Sft)

| 6. | Supplying Dampo (For mixing with cement plaster for water proofing purposes) Gen: Sch: of Mat: Item No. 46 P-4) 3.50 Lbs per bag of cement. |
| --- | --- |

14.32 Kg		@ Rs. 125/=		P.Kg				Rs. 1,790/=

(Rupees One Hundred Twenty Five) only

(Per Kg)

| 7. | Providing and fixing C.I .Man Hole cover with frame i/c cost of material (P.H.S.I. No:1 P-35). |
| --- | --- |

2.00 Cwt		@ Rs. 6985/=	P.Cwt				Rs. 13,970/=

(Rupees Six Thousand Nine Hundred Eighty Five) only

(Per Cwt)

| 8. | Providing / Fixing C.I ventilator (Weight not less than 80 – Lbs/ Each) in tank roof of approved design with water light joint i/c painting etc complete. (P.H.S.I.No 4 P-55). |
| --- | --- |

2 Nos:			@ Rs. 4870/=	Each				Rs. 9,740/=

(Rupees Four Thousand Eight Hundred Seventy)  only

(Each)

| 9. | Providing construction joint in concrete work of 9” wide corrugated P/C water stopper (with bulb) i/c soldering cost of material and labour etc complete (P.H.S.I No.2 P-56). |
| --- | --- |

153.72 Rft		@ Rs. 86/=		P.Rft				Rs 13,220/=

(Rupees Eighty Six)  only  (Per Rft)

| 10. | Supplying C.I Tail Piece with one end flanged and other end spigot (for all size) (P.H.S.of Mat: I.No. 5 P-11). |
| --- | --- |

0.262 Cwt		@ Rs. 6096/=	P.Cwt				Rs. 1,597/=

(Rupees Six Thousand Ninety Six)  only (P.Cwt)

| 11. N.S.I | Supplying and embedding C.I Foot steps of approved design weight not less than 15 Lbs . |
| --- | --- |

16 Nos:		@ Rs. 181/65	Each				Rs. 2,906/=

(Rupees One Hundred Eighty One and

Paisas Sixty Five)  only (Each)

| 12. | Providing and laying 1” thick topping cement concrete (1:2:4) including surface finishing and dividing into panels. (G.S.I. No: 16 P-41). |
| --- | --- |

2" thick

916.14 Sft		@ Rs. 3275/50	P% Sft				Rs. 30,008/=

(Rupees Three Thousand Two Hundred Seventy Five

and Paisas Fifty)  only (Per Hundred Sft)

| 13. | Providing, Laying, jointing and testing G.I Pipe of standard approved quality with thickness 6SWG test pressure 725 PSI (R.A). based on schedule. |
| --- | --- |

3" dia

8.0 Rft		@ Rs. 338/17	P.Rft				Rs. 2,705/=

(Rupees Three Hundred Thirty Eight and Paisas

Seventeen) only (Per Rft)

| 14. | Providing/fixing M.S flanges made of M.S sheet 3/8" thick etc complete (R.A.A based on schedule). |
| --- | --- |

3" dia

02 Nos:		@ Rs. 500/89	Each				Rs. 1,002/=

(Rupees Five Hundred and Paisas Eighty Nine)

only  (Each)

| 15. | Supplying C.I Sluice valve heavy pattern test pressure 21.0 kg/sq:cm or 300 lbs/sq: inch (P.H.S of Mat: I No: 2 P-12). |
| --- | --- |

3" dia

01 No:			@ Rs. 4290/=	Each				Rs. 4,290/=

(Rupees Four Thousand Two Hundred Ninety) only

(Each)

| 16. | C.I bend with flange and with hole including turning and facing of flanges for all size (P.H.S of Mat: I No:8 P-11). |
| --- | --- |

3" dia

0.262 Cwt		@ Rs. 6096/=	P.Cwt				Rs. 1,597/=

(Rupees Six Thousand Ninety Six)  only (P.Cwt)

| 17. | Jointing C.I /M.S Flanged Pipes and special Flange inside trenches including I/C applying rubber packing of required thickness nuts and bolts with washer etc. and other rules required for jointing and testing be same to be specified presser etc.. complete (P.H.S.I No: 1 P-40). |
| --- | --- |

3" dia

5 Joint		@ Rs. 499/=		P.Joint			Rs. 2,495/=

(Rupees Four Hundred Ninety Nine) only (P.Joint)

Total		Rs. 11,91,541/=

PART-G 	REPAIR OF COMPOUND WALL

| 1. | Removing Cement lime plaster (G.S.I No. 53 P-13 |
| --- | --- |

2952.0 Sft		@ Rs. 121/=		P% Sft				Rs. 3,572/=

(Rupees One Hundred Twenty One) only

(Per Hundred Sft)

| 2. | Cement plaster (1:6) upto 12' height ½" thick |
| --- | --- |

3084.0 Rft		@ Rs. 2206/60	P.Sft				Rs.68,052/=

(Rupees Two Thousand Two Hundred Six and

Paisas Sixty) only (Per Hundred Sft)

| 3. | Cement plaster (1:4) upto 12' height 3/8" thick |
| --- | --- |

3084.0 Rft		@ Rs. 2197/52	P% Sft				Rs. 67,772/=

(Rupees Two Thousand One Hundred Ninety Seven

and Paisas Fifty Two) only (Per Hundred Sft)

| 4. | Dismantling bricks work in lime or cement mortar (G.S.I No. 13 P-10) |
| --- | --- |

83.00 Cft		@ Rs. 1285/63    	P% Cft				Rs. 1,067/=

(Rupees One Thousand Two Hundred Eighty Five

and Paisas Sixty Three)  only (Per Hundred Cft)

| 5. | Pucca brick work other than building i/c striking of joints upto 20 ft height (G.S.I No: 7 P-21). |
| --- | --- |

93.00 Cft		@ Rs. 12899/70    	P% Cft				Rs. 11,997/=

(Rupees Twelve Thousand Eight Hundred Ninety Nine

and Paisas Seventy) only (Per Hundred Cft)

| 6. | Dismantling cement concrete palin (1:2:4) (G.S.I No. 19 (c) P-10) |
| --- | --- |

33.00 Cft		@ Rs. 3327/50    	P% Cft				Rs. 1,098/=

(Rupees Three Thousand Three Hundred Twenty Seven

and Paisas Fifty)  only (Per Hundred Cft)

| 7. | Providing & laying 1" thick topping cement concrete (1:2:4) i/c surface finishing & dividing into panels (G.S.I No. 16 (c) P-41 |
| --- | --- |

543.0 Sft		@ Rs. 3275/50	P% Sft				Rs. 17,786/=

(Rupees Three Thousand Two Hundred Seventy Five

and Paisas Fifty)  only (Per Hundred Sft)

| 8. | Distempering two coats (G.S.I.No. 24 (b) P-54). |
| --- | --- |

4483.0 Sft		@ Rs. 1043/90	P% Sft				Rs. 46,798/=

(Rupees One Thousand Forty Three

and Paisas Ninety)  only (Per Hundred Sft)

| 9. | Painting Old Surface preparing surface and painting of door and windows any type (including edges) (2 Coats) (G.S.I. No. 4 (c) P-67). |
| --- | --- |

265.0 Sft		@ Rs. 1160/06	P% Sft		         		Rs. 3,074/=

(Rupees One Thousand One Hundred Sixty

and Paisas Zero Six)  only (Per Hundred Sft)

Total	Rs. 2,21,216/=

PART-H 	U.PVC PIPE 6",4" & 3" DIA.

| 1. | Dismantling and removing road matling. (G.S.I. No. 51 P-13). |
| --- | --- |

101.0 Cft		@ Rs. 605/=			P% Cft		Rs. 611/=

(Rupees Six Hundred Five)  only (Per Hundred Sft)

| 2. | Providing PVC pipe of class “B” equivalent make fixing in trench i/c cutting, fitting and jointing with “Z” Joint with one rubber ring i/c testing with water to head of 61 meter or 200 ft. (P.H.S.I. No. 1 P-22). |
| --- | --- |

6" Dia

1200.0 Rft		@ Rs. 262/=			P.Rft			Rs. 3,14,400/=

(Rupees Two Hundred Sixty Two) only (Per Rft)

4" Dia

1500.0 Rft		@ Rs. 137/=			P.Rft			Rs. 2,05,500/=

(Rupees One Hundred Thirty Seven) only (Per Rft)

3" Dia

2000.0 Rft		@ Rs. 90/=			P.Rft			Rs.1,80,000/=

(Rupees Ninety) only (Per Rft)

| 3. | C.I sluice valve heavy pattern test pressure 21 kg/sq:cm or 300 lbs/sq: inch. (P.H.S. of Mat: Item No. 2 P-9). |
| --- | --- |

6" Dia

02 Nos:		@ Rs. 9360/=		Each			Rs.18,720/=

(Rupees Nine Thousand Three Hundred Sixty) only

(Each)

4" Dia

4 No:			@ Rs. 5460/=		Each			Rs. 21,840/=

(Rupees Five Thousand Four Hundred Sixty) only

(Each)

3" Dia

6 Nos:			@ Rs. 4290/=		Each			Rs. 25,740/=

(Rupees Four Thousand Two Hundred Ninety) only

(Each)

| 4. | PVC “Z” Joint fitting (P.H.S.O.M.Item No. 18 P-28). |
| --- | --- |

(i)	PVC Tee  6" dia

02 No:			@ Rs. 1800/=    		Each			Rs. 3,600/=

(Rupees One Thousand Eight Hundred) only (Each)

(ii)	PVC Tee  4" dia

6 No:			@ Rs. 1181/=    		Each			Rs. 7,086/=

(Rupees One Thousand One Hundred Eighty One)

only (Each)

(iii)	PVC Tee  3" dia (P.H.S (M) I.No. 1 P-30).

4 No:			@ Rs. 900/=    		Each			Rs. 3,600/=

(Rupees Nine Hundred) only (Each)

(iv)	PVC Reducer 4" dia

3 No:			@ Rs. 225/=    		Each			Rs. 675/=

(Rupees Two Hundred Twenty Five) only (Each)

(v)	PVC Reducer 6" dia

3 No:			@ Rs. 550/=    		Each			Rs. 1,650/=

(Rupees Five Hundred Fifty) only (Each)

(vi)	End Plug 3" dia

10 Nos:		@ Rs. 175/=    		Each			Rs. 1,750/=

(Rupees One Hundred Seventy Five) only (Each)

(vii)	Valve socket 3" dia

6 Nos:			@ Rs. 350/=    		Each			Rs. 2,100/=

(Rupees Three Hundred Fifty) only (Each)

(viii)	Valve socket 4" dia

8 Nos:			@ Rs. 500/=    		Each			Rs. 4,000/=

(Rupees Five Hundred) only (Each)

(ix)	Valve socket 6" dia

4 Nos:			@ Rs. 1650/=    		Each			Rs. 6,600/=

(Rupees One Thousand Six Hundred Fifty) only

(Each)

(x)	PVC Bend 6" dia

1 No:			@ Rs. 1913/=    		Each			Rs. 1,913/=

(Rupees One Thousand Nine Hundred Thirteen)

only (Each)

(xi)	PVC Bend 4" dia

1 No:			@ Rs. 956/=    		Each			Rs. 956/=

(Rupees Nine Hundred Fifty Six) only (Each)

(xii)	PVC Bend 3" dia

1 No:			@ Rs. 731/=    		Each			Rs. 731/=

(Rupees Seven Hundred Thirty One) only (Each)

| 5. | Cast iron special for A.C pressure pipe (Standard Weight A.C.I.L “B” Class) (P.H.S.O.M.I No: 10 P-12). |
| --- | --- |

(i)	Short Piece/Tail Piece

6" dia

04 No:			@ Rs. 618/75    		Each			Rs. 2,475/=

(Rupees Six Hundred Eighteen and Paisas

Seventy Five)  only (Each)

4" dia

8 No:			@ Rs. 487/50    		Each			Rs. 3,900/=

(Rupees Four Hundred Eighty Seven and Paisas

Fifty)  only (Each)

3" dia

12 No:			@ Rs. 375/=    		Each			Rs. 4,500/=

(Rupees Three Hundred Seventy Five) only

(Each)

| 6. | Jointing C.I /M.S Flanged Pipes and special Flange inside trenches including I/C applying rubber packing of required thickness nuts and bolts with washer etc. and other rules required for jointing and testing be same to be specified presser etc.. complete (P.H.S.I No: 1 P-40). |
| --- | --- |

6" dia

4 Joints		@ Rs. 938/=			P.Joint		Rs. 3,752/=

(Rupees Nine Hundred Thirty Eight) only

(Per Joint)

4" dia

2 Joints		@ Rs. 513/=			P.Joint		Rs. 1,026/=

(Rupees Five Hundred Thirteen) only

(Per Joint)

3" dia

6 Joints		@ Rs. 499/=  		P.Joint		Rs. 2,994/=

(Rupees Four Hundred Ninety Nine) only

(Per Joint)

| 7. | Making joints to PVC specials fitting including laying of specials and cost of solvent cement of required diameter and testing the joints with along with PVC pipe “B, “C” & “D” class to the specied pressure and making good o all leaky joints etc complete.. (P.H.S.I.No. 1 P-41). |
| --- | --- |

6" dia

5 Joints		@ Rs. 96/=  			P.Joint			Rs. 480/=

(Rupees Ninety Six) only (Per Joint)

4" dia

16 Joints		@ Rs. 70/=  			P.Joint			Rs. 1,120/=

(Rupees Seventy) only (Per Joint)

3" dia

19 Joint		@ Rs. 59/=  			P.Joint			Rs. 1,121/=

(Rupees Fifty Nine) only (Per Joint)

| 8. | Excavation for pipe line trenches and pits in sandy soil including trimming and dressing sides to true alignment and shape leveling of beds of trenches to correct level and grade, cutting joint holes and disposal of surplus earth within a one chain as directed by Engineer Incharge. Providing fence guards, lights, flags, temporary crossings for non vehicular traffic where ever required lift upto 5 ft (1.52 m) and lead upto one chain (30.5 m). (P.H.S.I.No. 2 P-60). |
| --- | --- |

30450.0 Cft		@ Rs. 3750/=	P‰ Cft			Rs. 1,14,188/=

(Rupees Three Thousand Seven Hundred Fifty)

Only (Per Thousand Cft)

| 9. | Refilling the excavated stuff in trenches 6" thick layer i/c watering ramming to full compaction etc complete. (P.H.S.I.No.24 P-77). |
| --- | --- |

27405.0 Cft		@ Rs. 2760/=	P‰ Cft			Rs. 75,638/=

(Rupees Two Thousand Seven Hundred Sixty)

Only (Per Thousand Cft)

| 10. | Providing chamber 3' x 2' (915 x 615 mm) inside dimension 4 ½' (1372 mm) deep as per approved design for sluice valve 3" to 12" dia with 18" (457 mm) dia inside cost iron cover and frame (wt = 1 cwt 3 qr) fixed in RCC 1:2:4 (102 mm) thick (with 5 Lbs steel per cft) 9" (299 mm) thick brick masonry wall set in 1:6 cement mortar 6" (1152 mm) thick cement concrete 1:3:6 in foundation 1" (25 mm) thick cement concrete 1:2:4 flooring ½" (12.5 mm) thick cement plaster 1:3 to all inside wall surface and to top i/c providing and fixing M.S foot rest at every one foot beyond 2 ½ ft depth curing, excavation, back filling and disposal of surplus earth etc complete. (P.H.S.I.No. 1 P-49). |
| --- | --- |

4 Nos:			@ Rs. 18820/=	Each				Rs. 75,280/=

(Rupees Eighteen Thousand Eight Hundred Twenty)

Only (Each)

Total		Rs. 10,87,946/=

PART – 4	MITHRIO HALEPOTA

PART – A 	PUMPING MACHINERY

| 1. N.S.I | Providing and installing in position i/c transportation site of work at Mithrio Halepoto Grund Fos HMA submersible pump type HSP 8A-25+7.5 HP Motor complete capable to discharge 40 GPM against a head of 350 ft 7.5 HP HMA (3 x 400V x 2 Pole) i/c G.I riser pipe of 2'' dia x 30 Nos. of 10 ft each with flanged ends. The flanges are notched to accommodate the cable i/c top set of 2'' dia containing bore hole cover plate with 90º bend, sluice valve, jacket, nuts & bolts and 2 sets of installation clamps (G.I) i/c nipple of 2'' dia (G.I) i/c 7.5 HP Motor Control Unit consist of metallic box, circuit breaker, am-meter, am-meter selector switch, volt meter, volt meter selector switch, electronic over current relay, magnetic contractor, under / over voltage relay (EVR), dry running protection relay, indication lamps & On/Off switch i/c Electrides & cable 1 x 1.5 mm² x 100 m Drop cable 3 x 4 mm² x 100 meter. Cable Clams 30 Nos. Cable Termination Kit (CTK) x 0.1 No: and making necessary arrangement for testing of complete pumping sets against the required head for 72 hours etc complete. |
| --- | --- |

02  Sets		@ Rs. 897360/-		P.Set			Rs. 17,94,720/=

(Rupees Eight Lac Ninety Seven Thousand

Three Hundred Sixty) only (Per Set)

| 2. N.S.I | Supplying and fixing 12 KVA Generator with 24.0 BHP self starter diesel engine (China) |
| --- | --- |

02 Sets		@ Rs. 350000/-		P.Set			Rs. 7,00,000/=

(Rupees Three Lac Fifty Thousand) only

(Per Set)

| 3. N.S.I | Repair of submersible pump 7.5 BHP i/c replacement of burnt motor and cash etc complete ( R.A approved) |
| --- | --- |

01 Set			@ Rs. 150000/- 		P.Set			Rs. 1,50,000/=

(Rupees One Lac Fifty Thousand) only

(Per Set)

| 4. N.S.I | Replacement of G.I pipe 2'' dia (10 ft each length) (R.A approved) |
| --- | --- |

30 Nos:		@ Rs. 3154/89		Each			Rs. 94,647/=

(Rupees Three Thousand One Hundred Fifty Four

and Paisas Eighty Nine)  only (Each)

| 5. N.S.I | Lowering of submersible pump motor and riser pipe etc complete (R.A approved) |
| --- | --- |

01 Job		@ Rs. 20000/-		P.Job			Rs. 20,000/=

(Rupees Twenty Thousand) only (Per Job)

| 6. N.S.I | Providing / Fixing Motor control unit, consist of circuit breaker main switch 40 amperes (Siemens) automatic star delta starter 45 amperes under / over load relay, over voltage relay, phase failure relay, ampere meter 30 ampere, volt meter 500 volts selector switch indicating light all fixed in metallic vernyle proof box (M.R) |
| --- | --- |

01 No			@ Rs. 47500/=		Each			Rs. 47,500/=

(Rupees Forty Seven Thousand Five Hundred)

only (Each)

| 7. N.S.I | Supplying and fixing 24 KVA Generator with 24 BHP Diesel Engine (China) for 2nd stage P/station. |
| --- | --- |

01 No			@ Rs. 350000/= 		Each			Rs. 3,50,000/=

(Rupees Three Lac Fifty Thousand) only

(Per Set)

Total	Rs. 31,56,867/=

PART-B	REPAIR AND RENOVATION OF CIVIL STRUCTURE

- R.C.C RESERVOIR 30' DIA

| 1. | Dismantling, lying cement concrete reinforced separating reinforce cement from concrete cleaning. (G.S.I No. 20 P-10) |
| --- | --- |

651.53 Cft		@ RS. 5445/- 		P% Cft		Rs. 35,476/=

(Rupees Five Thousand Four Hundred Forty Five)

only (Per Hundred Cft)

| 2. | Reinforced cement concrete work i/c all labour & material except the cost of steel reinforcement and its labour for bending and binding which will be paid separately. This rate also i/c of forms moulds lifting shuttering curing rendering and finishing the exposed surfaces i/c screening and washing of shingle 1/8" to 1/4" gauge.(P.H.S. I. No: 6 P-16). |
| --- | --- |

511.0 Cft		@ RS. 337/- 			P. Cft			Rs. 1,72,207/=				(Rupees Three Hundred Thirty Seven) only (Per Cft)

| 3. | Fabrication of Mild steel reinforcement for cement concrete including cutting bending laying in position, making joints & fastenings including cost of binding wire (also includes removal of rust from bars) (G.S.I.No.7 (b) P-16) (USING TAR BARS). |
| --- | --- |

20.531 Sft		@ Rs. 5001/70 		P.Cwt			Rs. 1,02,691/=

(Rupees Five Thousand One and Paisas Seventy)

only (Per Cwt)

| 4. | P/F C.I. main hole covering frame i/c cost of material (P.H.S.I.No: 1 P-35). |
| --- | --- |

2.0 Cwt		@ Rs. 6985/-   		P.Cwt			Rs. 13,970/=

(Rupees Six Thousand Nine Hundred Eighty Five)

only (Per Cwt)

| 5. | Providing Fixing C.I Ventilator (weight not less than 80-LbsEach) in tank roof of approved design with water light joint i/c painting ect complete. (P.H.S.I.No: 4 P-55). |
| --- | --- |

2 Nos			@ Rs. 4870/-   		Each			Rs. 9,740/=

(Rupees Four Thousand Eight Hundred Seventy) only

(Each)

| 6. | Providing and lying 1'' thick topping cement concrete (1:2:4) including surface finishing and dividing into panels (G.S.I No. 16 (c) P-42) |
| --- | --- |

2'' thick

1535.65 Sft		@ Rs. 3275/50  		P% Sft		 	Rs. 50,300/=

(Rupees Three Thousand Two Hundred Seventy Five

and Paisas Fifty)  only (Per Hundred Sft)

Total		Rs. 3,84,384/=

- PUMP HOUSE SIZE

| 1. | Removing cement lime plaster ( G.S.I No. 53 P-13) |
| --- | --- |

478.0 Sft 		@ Rs. 121/-			P% Sft			Rs. 578/=

(Rupees One Hundred Twenty One) only

(Per Hundred Sft)

| 2. | Cement Plaster (1:6) upto 12' height 1/2" thick (G.S.I.No: 13 (a) P-52). |
| --- | --- |

544.0 Sft		@ Rs. 2206/60		P% Sft			Rs. 12,004/=

(Rupees Two Thousand Two Hundred Six and

Paisas Sixty) only (Per Hundred Sft)

| 3. | Pucca brick work in ground floor in i/c stricking of joints with cement sand mortar (1:4) (G.S.I.No. 5 P-21). |
| --- | --- |

25.00 Cft		@ Rs. 13227/41		P% Cft		Rs. 3,307/=

(Rupees Thirteen Thousand Two Hundred Twenty Seven

and Paisas Forty One)  only (Per Hundred Cft)

| 4. | Supply and Fixing M.S Girder size 8'' x 4''. (Sch: Mat: Item 140 P-6) |
| --- | --- |

2.169 Cwt		@ Rs. 3850/=		P.Cwt			Rs. 8,351/=

(Rupees Three Thousand Eight Hundred Fifty)

only (Per Cwt)

| 5. | Distempering (b) two coat (G.S.I.No. 24 (b) P-54). |
| --- | --- |

2610.00 Sft		@ Rs. 1043/90		P% Sft			Rs. 27,246/=

(Rupees One Thousand Forty Three and

Paisas Ninety) only (Per Hundred Sft)

| 6. | Distempering 2 coat. |
| --- | --- |

2106.00 Sft		@ Rs. 1043/90 		P% Sft			Rs. 21,985/=

(Rupees One Thousand Forty Three and

Paisas Ninety) only (Per Hundred Sft)

| 7. | Painting old surface preparing surfaces and painting of doors and windows of any type i/c edges two coats.(G.S.I No. 4 (c) P-67). |
| --- | --- |

412.00 Sft		@ Rs. 1160/06 		P% Sft			Rs. 4,779/=

(Rupees One Thousand One Hundred Sixty

and Paisas Zero Six) only (Per Hundred Sft)

| 8. | P/F C.I. main hole covering frame i/c cost of material (P.H.S.I.No: 1 P-35). |
| --- | --- |

3.0 Cwt		@ Rs. 6985/- 		P.Cwt			Rs. 20,955/=

(Rupees Six Thousand Nine Hundred Eighty Five)

only (Per Cwt)

Total		Rs. 99,205/=

(iii) 	COMPOUND WALL

| 1. | Distempering two coat (G.S.I No 24 (b) P-53 |
| --- | --- |

2849.00 Sft		@ Rs. 1043/90 	P% Sft				Rs. 29,741/=

(Rupees One Thousand Forty Three and

Paisas Ninety) only (Per Hundred Sft)

| 2. | Painting old surface preparing surfaces and painting of doors and windows of any type i/c edges two coats.(G.S.I No. 4 P-69) |
| --- | --- |

144.0 Sft		@ Rs. 1160/06   	P% Sft				Rs. 1,670/=

(Rupees One Thousand One Hundred Sixty

and Paisas Zero Six) only (Per Hundred Sft

Total		Rs. 31,411/=

(iv) INTER CONNECTION

| 1. | Providing, Laying, jointing and testing G.I Pipe of standard approved quality with thickness 6SWG test pressure 725 PSI (R.A) (based on schedule). |
| --- | --- |

3" dia

72.0 Rft		@ Rs. 338/17		P.Rft			Rs. 24,348/=

(Rupees Three Hundred Thirty Eight and Paisas

Seventeen) only (Per Rft)

| 2. | Providing fixing M.S flanges made of M.S sheet 3/8'' thick etc complete R.A.A (Based as Sch: Item) |
| --- | --- |

26 Nos		@ Rs. 500/89		Each			Rs. 13,023/=

(Rupees Five Hundred and Paisas Eighty Nine)

only  (Each)

| 3. | C.I bend with flange and with hole including turning and facing of flanges for all size (P.H.S of Mat: I No: 8 P-11) |
| --- | --- |

3" dia

1.05 Cwt		@ Rs. 6096/-			P.Cwt			Rs. 6,401/=

(Rupees Six Thousand Ninety Six)  only (P.Cwt)

| 4. | Jointing C.I /M.S Flanged Pipes & special Flange inside trenches i/c applying rubber packing of required thickness nuts and bolts with washer etc. and other rules required for jointing and testing be same to be specified presser etc. complete (P.H.S.I No: 1 P-40). |
| --- | --- |

3" dia

26 Joint		@ Rs. 499/=			P.Joint		Rs. 12,974/=

Rupees Four Hundred Ninety Nine) only (P.Joint)

| 5. | S/F foot valve heavy pattern with cone type gate (imported) (S.M.I.No. 3 P-11). |
| --- | --- |

3" dia

02 Nos		@ Rs. 731/25		Each			Rs. 1,463/=

(Rupees Seven Hundred Thirty One and

Paisas Twenty Five) only  (Each)

Total		Rs. 58,209/=

(v) 	PVC PIPE

| 1. | Providing PVC pipe of class “B” equivalent make fixing in trench i/c cutting, fitting and jointing with “Z” Joint with one rubber ring i/c testing with water to head of 61 meter or 200 ft. (P.H.S.I. No. 1 P-21). |
| --- | --- |

3" Dia

1000.0 Rft		@ Rs. 90/=			P.Rft			Rs. 90,000/=

(Rupees Ninety) only (Per Rft)

| 2. | C.I sluice valve heavy pattern test pressure 21 kg/sq:cm or 300 lbs/sq: inch (P.H.S of Mat: Item No. 2 P-9. |
| --- | --- |

3" Dia

02 Nos		@ Rs. 4290/=		Each			Rs. 8,580/=

(Rupees Four Thousand Two Hundred Ninety)

only (Each)

| 3. | PVC “Z” Joint fitting (P.H.S.O.M.Item No. 16 P-28). |
| --- | --- |

PVC Tee 3" dia

1 No:			@ Rs. 900/=    		Each			Rs. 900/=

(Rupees Nine Hundred) only (Each)

End Plug 3" dia

2 Nos:			@ Rs. 175/=    		Each			Rs. 350/=

(Rupees One Hundred Seventy Five) only (Each)

Valve Socket 3" dia

4 Nos:			@ Rs. 350/=    		Each			Rs. 1400/=

(Rupees Three Hundred Fifty) only (Each)

| 4. | Jointing C.I /M.S Flanged Pipes and special Flange inside trenches including I/C applying rubber packing of required thickness nuts and bolts with washer etc. and other rules required for jointing and testing be same to be specified presser etc.. complete (P.H.S.I No: 1 P-40). |
| --- | --- |

3" dia

4 Joints		@ Rs. 499/=  		P.Joint		Rs. 1,996/=

(Rupees Four Hundred Ninety Nine) only

(Per Joint)

| 5. | Making joints to PVC specials fitting including laying of specials and cost of solvent cement of required diameter and testing the joints with along with PVC pipe “B, “C” & “D” class to the specied pressure and making good o all leaky joints etc complete.. (P.H.S.I.No. 1 P-41). |
| --- | --- |

3" dia

2 Joints		@ Rs. 59/=  			P.Joint		Rs. 118/=

(Rupees Fifty Nine) only (Per Joint)

| 6. | Excavation for pipe line trenches and pits in sandy soil including trimming and dressing sides to true alignment and shape leveling of beds of trenches to correct level and grade, cutting joint holes and disposal of surplus earth within a one chain as directed by Engineer Incharge. Providing fence guards, lights, flags, temporary crossings for non vehicular traffic where ever required lift upto 5 ft (1.52 m) and lead upto one chain (30.5 m). (P.H.S.I.No. 2 P-60). |
| --- | --- |

3" dia

6000.0 Cft		@ Rs. 3750/=		P‰ Cft		Rs. 22,500/=

(Rupees Three Thousand Seven Hundred Fifty)

Only (Per Thousand Cft)

| 7. | Refilling the excavated stuff in trenches 6" thick layer i/c watering ramming to full compaction etc complete. (P.H.S.I.No.24 P-77). |
| --- | --- |

5400.0 Cft		@ Rs. 2760/=		P‰ Cft		Rs. 14,904/=

(Rupees Two Thousand Seven Hundred Sixty)

Only (Per Thousand Cft)

| 8. | Providing chamber 3' x 2' (915 x 615 mm) inside dimension 4 ½' (1372 mm) deep as per approved design for sluice valve 3'' to 12'' dia with 18'' (457 mm) dia inside cost cover and frame (wt = 1 cwt 3 qr) fixed in RCC 1:2:4 (102 mm) thick (with 5 Lbs steel per cft) 9'' (299 mm) thick brick masonry wall set in 1:6 cement mortar 6'' (1152 mm) thick cement concrete 1:3:6 in foundation 1'' (25 mm) thick cement concrete 1:2:4 flooring ½'' (12.5 mm) thick cement plaster 1:3 to all inside wall surface and top i/c providing and fixing M.S foot rest at every one foot beyond 2 ½ ft depth curing, excavation, back filling and disposal of surplus earth etc complete. (P.H.S.I No. 1 P-49). |
| --- | --- |

01 No			@ Rs. 18820/=		Each			Rs. 18,820/=

(Rupees Eighteen Thousand Eight Hundred Twenty)

only (Each)

Total		Rs. 1,59,568/=

CONDITIONS

- The rates should be inclusive of all the Taxes i/c. Sales Tax, Octroi. Royalties etc, if any.

- No separate carriage will be allowed to the contractor.

- The Testing of the works/Pumping Machinery will be arranged by the contractor at his own cost, No separate payment will be made.

- No premium will be allowed on Non-Schedule items.

- Any error or omission in the specifications/items will be governed by the relevant specifications and schedule of rates.

- The decision of the Executive Engineer will be final and binding on all the parties in any dispute.

- The contractor will have to submit the certificate of manufacturer at the time of supply of Machinery.

| CONTRACTOR |  | EXECUTIVE ENGINEER PUBLIC HEALTH ENGG: DIVISION THARPARKAR @ MITHI |
| --- | --- | --- |