SCHEDULE “B”

CONSTRUCTION OF SEMI PUCCA/ LINED STORAGE TANK, FEEDING CHANEL, PUMP HOUSE & COMPOUND WALL @ NAUKOT FOR PIPE LINE WATER SUPPLY SCHEME LUNIO-DANO-DANDHAL-NAGARPARKAR DISTRICT THARPARKAR.

ABSTRACT OF COST

Part-A	Storage Tank					Rs. 1,58,36,458/=

Part-B	Interconnection					Rs. 11,96,122/=

Part-C	Feeding Channel 					Rs. 10,26,509/=

Part-D	Pump House (14' x 16')				Rs. 4,21,715/=

Part-D	Compound Wall					Rs. 32,56,513/=

Total 		Rs. 2,17,37,317/=

| CONTRACTOR |  | EXECUTIVE ENGINEER PUBLIC HEALTH ENGG: DIVISION THARPARKAR @ MITHI |
| --- | --- | --- |

PART-A	CONSTRUCTION OF STORAGE/SETTLING TANK (02 NO:) OF SIZE (300' X 175')

| 1. | Excavation for tanks and reservoirs in soft soil i/c trimming and dressing sides to true alignment / Design specification / profiles and shape, leveling of bed of tanks to correct level and grade i/c laying of earth in 6” layers for construction of tanks and dressing and disposal of surplus excavated earth with in one chain as directed by Engineer Incharge, i/c providing fence lift upto 5’ ft: and lead upto one chain. |
| --- | --- |

Slope (1:2)

0'-5'

532148 Cft		@ Rs. 3000/-		P ‰ Cft			Rs. 15,96,444/=

(Rupees Three Thousand) only

(Per Thousand Cft)

5'-8'

280043.0 Cft		@ Rs. 3550/-		P ‰ Cft			Rs. 9,94,153/-

(Rupees Three Thousand Five Hundred Fifty) only

(Per Thousand Cft)

8'-11'

51460.0 Cft		@ Rs. 4100/-		P ‰ Cft			Rs. 2,10,986/=

(Rupees Four Thousand One Hundred) only

(Per Thousand Cft)

| 2. | Excavation in foundation of buildings bridges and other structures i/c dag belling, dressing refilling around structures with excavated earth watering and ramming lead upto 5’ ft: |
| --- | --- |

2618.40 Cft		@ Rs. 3176/25	P‰ Cft			Rs. 8,317/=

(Rupees Three Thousand One Hundred Seventy Six

and Paisas Twenty Five) only (Per Thousand Cft)

| 3. | Earth work compaction (soft ordinary) or hard soil) (c) compacting by rolling (this does not i/c higher charges of roller |
| --- | --- |

33944.0 Cft		@ Rs. 354/= 		P‰ Cft			Rs. 12,016/=

(Rupees Three Hundred Fifty Four) only

(Per Thousand Cft)

| 4. | Cement concrete plain i/c placing compacting, finishing & curing, complete (including screening and washing of stone aggregate without shuttering |
| --- | --- |

Ratio (1:3:6)

32427.00 Cft		@ Rs. 12595/-		P% Cft				Rs. 40,84,181/=

(Rupees Twelve Thousand Five Hundred Ninety Five)

only (Per Hundred Cft)

| 5. | Cement concrete lining Ratio (1:3:6) |
| --- | --- |

11265 Cft		@ Rs. 11920/43	P% Cft				Rs. 13,42,836/=

(Rupees Eleven Thousand Nine Hundred Twenty)

and Paisas Forty Three only (Per Hundred Cft)

| 6. | Pucca brick work in foundation and plinth in cement sand mortar |
| --- | --- |

4726.0 Cft		@ Rs. 11948/36	P% Cft				Rs. 5,64,679/=

(Rupees Eleven Thousand Nine Hundred Forty

Eight and Paisas Thirty Six) only (Per Hundred Cft)

| 7. | Brick lining (9” x 4 ½” x 3”) in (1:3) cement sand mortar. |
| --- | --- |

(a) 	In Bed

24508.0 Cft		@ Rs. 13765/81	P % Cft				Rs. 33,73,725/=

(Rupees Thirteen Thousand Seven Hundred Sixty

Five and Paisas Eighty One) only  (Per Hundred Cft)

- On Slopes

12512.0 Cft		@ Rs. 14204/44	P % Cft				Rs. 17,77,260/=

(Rupees Fourteen Thousand Two Hundred Four

and Paisas Forty Four) only  (Per Hundred Cft)

| 8. | Cement Plaster upto 12’ height |
| --- | --- |

(a) ½”  Thick (1:6)

7192.0 Sft		@ Rs. 2206/60	P % Sft				Rs. 1,58,699/=

(Rupees Two Thousand Two Hundred Six

and Paisas Sixty)  only (Per Hundred Sft)

(b) ¾” Thick (1:4)

4548.0 Sft		@ Rs. 3015/76 	P % Sft			Rs. 1,37,157/=

(Rupees Three Thousand Fifteen and

Paisas Seventy Six) only (Per Hundred Sft)

(c) 3/8” Thick (1:4)

7192.0 Sft		@ Rs. 2197/52 	P % Sft			Rs. 1,58,046/=

(Rupees Two Thousand One Hundred Ninety Seven

and Paisas Fifty Two) only (Per Hundred Sft)

| 9. | Cement Pointing struck joints on wall. |
| --- | --- |

- Ratio (1:3)

29269.0 Sft		@ Rs. 1213/58	P% Sft				Rs.3,55,203/=

(Rupees One Thousand Two Hundred Thirteen

and Paisas Fifty Eight) only (Per Hundred Sft)

| 10. | Extra for every 50 ft additional lead or part thereof (a) for earth work (soft ordinary, hard and very hard) this rate shall apply to lead upto ½ mile then when the lead is more than ½ mile the lead shall be paid as carriage of material by mechanical. |
| --- | --- |

753236 Cft		@ Rs. 1410/92	P‰ Cft			Rs.10,62,756/=

(Rupees One Hundred Seventy Eight and

Paisas Sixty Five) only (Per Hundred Cft)

Total         	Rs. 1,58,36,458/=

PART-B		INTER CONNECTION  & SUMP WELL

| 1. | P/L R.C.C pipe of ASTM C-76-62 T/C-76-70 of Class II wall Band fixing in trench i/c cutting fitting and jointing with rubber ring i/c testing with water to splnd person. |
| --- | --- |

i. ASTM Pipe 18" Dia

80.0 Rft		@ Rs. 618/=		P.Rft				Rs. 49,440/=

(Rupees Six Hundred Eighteen) only (Per Rft)

| 2. | Supplying C.I Sluice Valve Heavy Pattern (Test Pressure 300 Lbs/ Sq: inch |
| --- | --- |

C.I S/Valve 18" Dia

2 Nos:			@ Rs. 1,10,700/=	P.Each				Rs. 2,21,400/=

(Rupees One Lac Ten Thousand Seven Hundred) only

(Per Each)

| 3. | Cost Iron specials for A.C Pressure Pipe (Standard Weight for A.C.I.L B-Class) Short Pieces |
| --- | --- |

Tail Piece/Short Piece 18" Dia

4 Nos:			@ Rs. 8287/50	P.Each				Rs. 33,150/=

(Rupees Eight Thousand Two Hundred

Eighty Seven and Paisas Fifty) only (Per Each)

| 4. | Excavation for Pipe lines on trenches and joints in sandy soil i/c trimming and dressing sides to true alignment and shape leveling of beds of trenches to correct level and grade cutting joint holes and disposal of surplus earth within one chain as directed by Engineer Incharge Providing fence guards lights flags and temporary crossing for Non Vehicular traffic wherever required lift upto 5’ ft: and lead upto to one chain |
| --- | --- |

3000.00 Cft		@ Rs. 3750/=		P‰ Cft			Rs. 11,250/=

(Rupees Three Thousand Seven Hundred

Fifty) only (Per Thousand Cft)

1200.0 Cft 		@ Rs. 4300/= 	P‰ Cft			Rs. 5,160/=

(Rupees Four Thousand Three Hundred) only

(Per Thousand Cft)

| 5. | Jointing C.I/M.S Flanged Pipes and specials flanged ends in side trench i/c supplying rubber packing of the required thickness, nuts bolts with washers etc and other tools required for jointing and testing the joints to the specified pressure etc complete |
| --- | --- |

18" Dia Joint

4 Joints		@ Rs. 3456/= 		P. Joint			Rs. 13,824/=

(Rupees Three Thousand Four Hundred

Fifty Six) only (Per Joint)

| 6. | Refilling the excavated stuff in trenches 6" thick layers i/c watering ramming to full compaction etc complete |
| --- | --- |

3780.0 Cft		@ Rs. 2760/=		P% Cft				Rs. 10,433/=

(Rupees Two Thousand Seven Hundred

Sixty) only (Per Hundred Cft)

| 7. | Providing chamber 3½ x 2½ 7' deep. |
| --- | --- |

2 Nos:			@ Rs. 28691/=	Each				Rs. 57,382/=

(Rupees Twenty Eight Thousand Six Hundred

Ninety One) only (Each)

SUMP WELL (SIZE 14' X 10')

| 8. | Excavation for tanks and reservoirs in sandy Soil i/c trimming and dressing sides to true alignment / design correct / profiles and shape, leveling of bed of tanks to correct level & grade i/c laying of earth in 6” layers for construction of tanks and dressing and disposal of surplus excavated earth within one chain as directed by Engineer Incharge i/c providing fence guards lights flags wherever required lift upto 5 feet and lead upto one chain. |
| --- | --- |

0 – 5.0'

1600.0 Cft 		@ Rs. 3000/= 	P‰ Cft			Rs. 4,800/=

(Rupees Three Thousand) only (Per Thousand Cft)

5.1' – 8.0'

960.0 Cft 		@ Rs. 3550/= 	P‰ Cft			Rs. 3,408/=

(Rupees Three Thousand Five Hundred Fifty) only

(Per Thousand Cft)

8.1' – 11.0'

960.0 Cft 		@ Rs. 4100/= 	P‰ Cft			Rs. 3,936/=

(Rupees Four Thousand Two Hundred Fifty) only

(Per Thousand Cft)

11.1' – 14.0'

320.0 Cft 		@ Rs. 4650/= 	P‰ Cft			Rs. 1,488/=

(Rupees Four Thousand Six Hundred Fifty) only

(Per Thousand Cft)

| 9. | Cement Concrete plain including placing compacting finishing and curing complete (including screening and washing of stone aggregate without shutting. |
| --- | --- |

Ratio (1:4:8)

160.0 Cft		@ Rs. 12595/=	P% Cft			Rs.20,152/=

(Rupees Twelve Thousand Five Hundred Ninety

Five) only  (Per Hundred Cft)

| 10. | Reinforced cement concrete work including all labour and material except the cost of steel reinforcement and its labour for bending and binding, which will be paid separately. This rate also includes all kinds of forms moulds lifting shuttering curing rendering and finishing the exposed surface (including screening and washing of shingle) (a) RCC work in roof slab beams columns, rafts, lintels and other structural members laid in situ or precast laid in position complete in all respects Ratio (1:2:4) 90 lbs cement 2 Cft sand and 4 Cft shingle 1/8” to ¼” gauge. |
| --- | --- |

1238.0 Cft		@ Rs. 337/=		P.Cft				Rs. 4,17,206/=

(Rupees Three Hundred Thirty Seven) only

(Per Cft)

| 11. | Fabrication of Mild steel reinforcement for cement concrete including cutting bending laying in position, making joints & fastenings including cost of binding wire (also includes removal of rust from bars) (USING TAR BARS). |
| --- | --- |

55.26 Cwt		@ Rs. 5001/70	P.Cwt				Rs. 2,76,394/=

(Rupees Five Thousand One and Paisas Seventy)

only (Per Cwt)

| 12. | Cement Plaster (1:3) upto 12’ height ¾” thick (For Water Proofing). |
| --- | --- |

693.0  Sft		@ Rs. 2795/30	 P% Sft			Rs. 19,371/=

(Rupees Two Thousand Seven Hundred Ninety Five

and Paisas Thirty)  only (Per Hundred Sft)

| 13. | Supplying Dampo (For mixing with cement plaster for water proofing purposes) 3.50 Lbs per bag of cement. |
| --- | --- |

15.0 Kg		@ Rs. 125/=		P.Kg				Rs. 1,875/=

(Rupees One Hundred Twenty Five) only

(Per Kg)

| 14. | Providing and fixing C.I .Man Hole cover with frame i/c cost of material. |
| --- | --- |

2.0 Cwt		@ Rs. 6985/=	P.Cwt				Rs. 13,970/=

(Rupees Six Thousand Nine Hundred Eighty Five) only

(Per Cwt)

| 15. | Providing construction joint in concrete work of 9” wide corrugated P/C water stopper (with bulb) i/c soldering cost of material and labour etc complete. |
| --- | --- |

212.0 Rft		@ Rs. 86/=		P.Rft				Rs. 18,232/=

(Rupees Eighty Six)  only  (Per Rft)

| 16. N.S.I | Supplying and embedding C.I Foot steps of approved design weight not less than 15 Lbs. |
| --- | --- |

12 Nos:		@ Rs. 181/65	Each				Rs. 2,180/=

(Rupees One Hundred Eighty One and

Paisas Sixty Five)  only (Each)

| 17. | Providing and laying 1” thick topping cement concrete (1:2:4) including surface finishing and dividing into panels. |
| --- | --- |

2" thick

338.0 Sft		@ Rs. 3275/50	P% Sft				Rs. 11,071/=

(Rupees Three Thousand Two Hundred Seventy Five

and Paisas Fifty)  only (Per Hundred Sft)

Total	Rs.11,96,122/=

PART-C		FEEDING CHANNEL

| 1. | Excavation in foundation of buildings bridges and other structures including dag belling, dressing refilling around structures with excavated earth watering and ramming lead upto 5’ ft: |
| --- | --- |

7500.0 Cft		@ Rs. 3176/25	P‰ Cft			Rs. 23,822/=

(Rupees Three Thousand One Hundred Seventy Six

and Paisas Twenty Five)  only (Per Thousand Cft)

| 2. | Cement concrete plain including placing, compacting, finishing and curing complete (Including screening and washing of stone aggregate without shuttering. |
| --- | --- |

(i) 	Ratio 1:4:8

1250.0 		@ Rs. 11288/75	P%Cft				Rs. 1,41,109/=

(Rupees Eleven Thousand Two Hundred Eighty Eight

and Paisas Seventy Five)  only (Per Thousand Cft)

(ii) 	Ratio 1:2:4

362.0 		@ Rs. 14429/25	P%Cft				Rs.52,234/=

(Rupees Fourteen Thousand Four Hundred Twenty

Nine and Paisas Twenty Five) only (Per Hundred Cft)

| 3. | Pucca brick work n foundation and plinth in cement sand mortar (1:4). |
| --- | --- |

4510.0 Cft		 @ Rs. 12501/41	P% Cft			Rs. 5,63,814/=

(Rupees Twelve Thousand Five Hundred One

and Paisas Forty One)  only (Per Hundred Cft)

| 4. | Cement Plaster (1:6) upto 12’ height 1/2” thick. |
| --- | --- |

4330.0 Sft 		@ Rs. 2206/60	P% Sft				Rs. 95,546/=

(Rupees Two Thousand Two Hundred Six and

Paisas Sixty) only (Per Hundred Sft)

| 5. | Cement Plaster (1:4) upto 12’ height 2/8” thick. |
| --- | --- |

4330.0 Sft 		@ Rs. 2197/52	P% Sft				Rs. 95,153/=

(Rupees Two Thousand One Hundred Ninety Seven

and Paisas Fifty Two) only (Per Hundred Sft)

| 6. | Reinforced cement concrete work including all labour and material except the cost of binding which will be paid separately. This rate also includes all kinds of forms moulds lifting shuttering curing rendering and fining washing of shingle (a) RCC work in roof slab beams columns rafts lintels and others structural members laid in situ or precast laid in position complete in all respects Ratio (1:2:4) 90 Lbs cement 2 Cft sand and 4 Cft shingle 1/8” to ¼" gauge (G.S.I. No.6) (a) Page No.16). |
| --- | --- |

102.0 Cft		@ Rs. 	337/=		P.Cft				Rs. 34,374/=

(Rupees Three Hundred Thirty Seven) only (Per Cft)

| 7. | Fabrication of mild steel reinforcement for cement concrete including cutting, bending laying in position making joints and fastening laying in position binding wire (also includes removal of rust from bars) (G.S.I. No.8 (b) P-17). |
| --- | --- |

4.09 Cwt		@ Rs. 5001/70	P.Cwt				Rs. 20,457/=

(Rupees Five Thousand One and Paisas Seventy)

only (Per Cwt)

Total	Rs. 10,26,509/=

PART-D	PUMP HOUSE OF SIZE (16' x 14')

| 1. | Excavation in foundation of buildings bridges and other structures including dag belling, dressing refilling around structures with excavated earth watering and ramming lead upto 5’ ft: (In sand, ash or loose Soil). |
| --- | --- |

409.0 Cft		@ Rs. 3176/25	P‰ Cft			Rs. 1,299/=

(Rupees Three Thousand One Hundred Seventy Six

and Paisas Twenty Five)  only (Per Thousand Cft)

| 2. | Cement concrete plain including placing compacting finishing and curing complete (Including screening and washing of stone aggregate without shuttering. |
| --- | --- |

(Ratio (1:4:8)

197.0 Cft		@ Rs. 11288/75	P% Cft			Rs. 22,239/=

(Rupees Eleven Thousand Two Hundred Eighty Eight

and Paisas Seventy Five)  only (Per Thousand Cft)

| 3. | Pucca brick work n foundation & plinth in cement sand mortar(1:6). |
| --- | --- |

422.0 Cft		@ Rs. 11948/36	P% Cft			Rs. 50,422/=

(Rupees Eleven Thousand Nine Hundred Forty Eight

and Paisas Thirty Six)  only (Per Hundred Cft)

| 4. | Filling Watering and ramming earth in floors with surplus earth form foundation lead upto one chain and lift upto 5’ ft:. |
| --- | --- |

320.0 Cft		@ Rs. 1512/50	P‰ Cft			Rs. 484/=

(Rupees One Thousand Five Hundred Twelve and

Paisas Fifty)  only (Per Thousand Cft)

| 5. | Pucca brick work in ground floor in Cement sand mortar (1:6. |
| --- | --- |

596.0 Cft		@ Rs. 12674/36	P% Cft			Rs. 75,539/=

(Rupees Twelve Thousand Six Hundred Seventy Four

and Paisas Thirty SIx)  only (Per Hundred Cft)

| 6. | Reinforced cement concrete work including all labour and material except the cost of binding which will be paid separately. This rate also includes all kinds of forms moulds lifting shuttering curing rendering and fining washing of shingle (a) RCC work in roof slab beams columns rafts lintels and others structural members laid in situ or precast laid in position complete in all respects Ratio (1:2:4) 90 Lbs cement 2 Cft sand and 4 Cft shingle 1/8” to ¼" gauge. |
| --- | --- |

240.0 Cft		@ Rs. 337/=		P.Cft				Rs. 80,880/=

(Rupees Three Hundred Thirty Seven) only

(Per Cft)

| 7. | Fabrication of mild steel reinforcement for cement concrete including cutting, bending laying in position making joints and fastening laying in position binding wire (also includes removal of rust form bars). |
| --- | --- |

9.642 Cwt		@ Rs. 5001/70	P.Cwt				Rs. 48,226/=

(Rupees Five Thousand One and Paisas Seventy)

only (Per Cwt)

| 8. | Making & Fixing steel grated doors complete with 1/16” thick sheeting including angle Iron frame 2” x 2” x 3/8” and 3 /4” square bars 4” center to center with locking arrangement. |
| --- | --- |

28.0 Sft		@ Rs. 726/72	P Sft				Rs. 20,348/=

(Rupees Five Hundred Ninety Four and

Paisas Fifty Seven)  only (Per Sft)

| 9. | Fabrication of heavy steel work with angles, tees, flat iron, iron and sheet iron for making trusses, girders, tanks etc, including cutting riveting, handing assembling and fixing but eluding erection in position. |
| --- | --- |

5.428 Cwt		@ Rs. 4928/49	P.Cwt				Rs. 26,752/=

(Rupees Four Thousand Nine Hundred Twenty Eight

and Paisas Forty Nine) only (Per Cwt)

| 10. | Supplying and fixing in position Iron/ Steel grill of 3/8” x ¼” size flat Iron or approved design including painting 3 Coats etc complete (Weight net less than 3.7 lb/sp: ft: of in finished grill. |
| --- | --- |

64.0 Sft		@ Rs. 180/50	P.Sft				Rs. 11,552/=

(Rupees One Hundred Eighty and Paisas Fifty)

only (Per Sft)

| 11. | Cement Plaster (1:6) upto 12’ height ½” thick. |
| --- | --- |

622.0 Sft		@ Rs. 2206/60	P% Sft				Rs. 13,725/=

(Rupees Two Thousand Two Hundred Six and

Paisas Sixty) only (Per Hundred Sft)

| 12. | Cement plaster (1:4) upto 12’ height 3/8” thick. |
| --- | --- |

622.0 Sft		@ Rs. 2197/52	P% Sft				Rs. 13,669/=

(Rupees Two Thousand One Hundred Ninety Seven

and Paisas Fifty Two) only (Per Hundred Sft)

| 13. | Cement pointing Struck joints on Walls Ratio (1:3). |
| --- | --- |

681.00 Sft		@ Rs. 1213/58	P% Sft				Rs. 8,264/=

(Rupees One Thousand Two Hundred Thirteen and

Paisas Fifty Eight) only (Per Hundred Sft)

| 14. | White washing (a) one coat. |
| --- | --- |

1303.0 Sft		@ Rs. 416/63	P % Sft			Rs. 5,429/=

(Rupees Four Hundred Sixteen and Paisas

Sixty Three)  only (Per Hundred Sft)

| 15. | Distempering two coats. |
| --- | --- |

1303.0 Sft		@ Rs. 1043/90	P % Sft			Rs. 13,602/=

(Rupees One Thousand Forty Three and Paisas

Ninety)  only (Per Hundred Sft)

| 16. | White washing 3 coats. |
| --- | --- |

347.0 Sft		@ Rs. 829/95	P % Sft			Rs. 2,880/=

(Rupees Eight Hundred Twenty Five and

Paisas  Ninety Five) only (Per Hundred Sft)

| 17. | Providing and laying 1” thick topping cement concrete (1:2:4) including surface finishing and dividing into panels. |
| --- | --- |

(i) 2" thick Flooring

224.0 Sft		@ Rs. 3275/50	P% Sft				Rs. 7,337/=

(Rupees Three Thousand Two Hundred Seventy Five

and Paisas Fifty) only (Per Hundred Sft)

(ii) 1 ½"  R/Topping

370 Sft		@ Rs. 2548/29	P% Sft				Rs. 9,429/=

(Rupees Two Thousand Five Hundred Forty Eight

and Paisas Twenty Nine) only (Per Hundred Sft)

| 18. | Painting New Surface preparing surface and painting of door and windows any type (including edges) (3 Coats). |
| --- | --- |

56.0 Sft		@ Rs. 2116/41	P% Sft				Rs. 1,185/=

(Rupees Two Thousand One Hundred Sixteen

and Paisas Forty One ) only (Per Hundred Sft)

| 19. | Erection & fitting in position of iron trusses of water tanks etc |
| --- | --- |

5.428 Cwt		@ Rs. 271/04		P.Cwt			Rs. 1,471/=

(Rupees Two Hundred Seventy One and

Paisas  Zero Four) only (Per Cwt)

| 20. | Two Coats of Bitumen laid bat using 34 Lbs for % Sft over roof and blinded with sand stone at 1 Cft for 100 Sft. |
| --- | --- |

370.0 Sft		@ Rs. 1887/40 	P% Sft				Rs.6,983/=

(Rupees One Thousand Eight Hundred Eighty Seven

and Paisas Forty ) only (Per Hundred Sft)

Total		Rs. 4,21,715/=

PART-E		CONSTRUCTION OF BRICK MASONRY COMPOUND WALL

| 1. | Excavation in foundation of buildings bridges and other structures including dag belling, dressing refilling around structures with excavated earth watering and ramming lead upto 5’ ft: (In sand, ash or loose Soil) |
| --- | --- |

7645.0 Cft		@ Rs. 3176/25	P‰ Cft			Rs. 24,282/=

(Rupees Three Thousand One Hundred Seventy Six

and Paisas Twenty Five)  only (Per Thousand Cft)

| 2. | Cement concrete plain including placing compacting finishing and curing complete (Including screening and washing of stone aggregate without shuttering. |
| --- | --- |

(Ratio (1:4:8)

1911.0 Cft		@ Rs. 11288/75	P% Cft				Rs. 2,15,728/=

(Rupees Eleven Thousand Two Hundred Eighty

Eight and Paisas Seventy Five) only  (Per Hundred Cft)

| 3. | Pucca brick work in foundation and plinth in cement sand mortar (1:6). |
| --- | --- |

7701.00 Cft		@ Rs. 11948/36	P% Cft				Rs. 9,20,143/=

(Rupees Eleven Thousand Nine Hundred Forty

Eight and Paisas Thirty Six) only  (Per Hundred Cft)

| 4. | Pucca brick work other than building including strucking of joint upto 20’ cement sand mortar (1:4). |
| --- | --- |

4123.0 Cft		@ Rs. 12899/70	P% Cft				Rs. 5,31,855/=

(Rupees Twelve Thousand Eight Hundred

Ninety Nine and Paisas Seventy) only (Per Hundred Cft)

| 5. | Damp proof course with cement sand and shingle concrete (1:2:4) i/c two coat of asphalt mixture. |
| --- | --- |

2" thick

2153.0 Cft		@ Rs. 3912/85	P% Cft				Rs. 84,244/=

(Rupees Three Thousand Nine Hundred Twelve

and Paisas Eighty Five) only  (Per Hundred Cft)

| 6. | Cement pointing Struck joints on Walls Ratio (1:3). |
| --- | --- |

6858.0 Sft		@ Rs. 1213/58	P % Sft				Rs. 83,227/=

(Rupees One Thousand Two Hundred Thirteen

and Paisas Fifty Eight) only  (Per Hundred Sft)

| 7. | Cement Plaster (1:6) upto 12’ height. |
| --- | --- |

½” thick

20342.0 Sft		@ Rs. 2206/60	P% Sft				Rs. 4,48,867/=

(Rupees Two Thousand Two Hundred Six

and Paisas Sixty) only  (Per Hundred Sft)

| 8. | Cement Plaster (1:4) upto 12’ height. |
| --- | --- |

3/8” thick

20342.0 Sft		@ Rs. 2197/52	P% Sft				Rs. 4,47,020/=

(Rupees Two Thousand One Hundred Ninety

Seven and Paisas Fifty Two) only  (Per Hundred Sft)

| 9. | White Washing one Coat. |
| --- | --- |

20342.0 Sft		@ Rs. 416/63		P% Sft				Rs. 84,751/=

(Rupees Four Hundred Sixteen and Paisas

Sixty Three) only  (Per Hundred Sft)

| 10. | Distempering two coats. |
| --- | --- |

27200.0 Sft		@ Rs. 1043/90	P% Sft				Rs. 2,83,941/=

(Rupees One Thousand Forty Three

and Paisas Ninety) only  (Per Hundred Sft)

| 11. | Reinforced cement concrete work including all labour and material except the cost of binding which will be paid separately. This rate also includes all kinds of forms moulds lifting shuttering curing rendering and fining washing of shingle (a) RCC work in roof slab beams columns rafts lintels and others structural members laid in situ or precast laid in position complete in all respects Ratio (1:2:4) 90 Lbs cement 2 Cft sand and 4 Cft shingle 1/8” to ¼" gauge. |
| --- | --- |

58.0 Cft		@ Rs. 337/=		P.Cft				Rs. 19,546/=

(Rupees Three Hundred Thirty Seven) only

(Per Cft)

| 12. | Fabrication of mild steel reinforcement for cement concrete including cutting, bending laying in position making joints and fastening laying in position binding wire (also includes removal of rust form bars). |
| --- | --- |

2.33 Cwt		@ Rs. 5001/70	P.Cwt				Rs. 11,654/=

(Rupees Five Thousand One and Paisas Seventy)

only (Per Cwt)

| 13. | Making & Fixing steel grated doors complete with 1/16” thick sheeting including angle Iron frame 2” x 2” x 3/8” and 3 /4” square bars 4” center to center with locking arrangement. |
| --- | --- |

72.0 Sft		@ Rs. 726/72	P Sft				Rs. 52,324/=

(Rupees Five Hundred Ninety Four and

Paisas Fifty Seven)  only (Per Sft)

| 14. | Painting New Surface preparing surface and painting of door and windows any type (including edges) (3 Coats). |
| --- | --- |

144.0 Sft		@ Rs. 2116/41	P% Sft				Rs. 3,048/=

(Rupees Two Thousand One Hundred Sixteen

and Paisas Forty One) only (Per Hundred Sft)

| 15. | Dry brick of edge paving sand grouted i/c preparation of bed by watering, ramming and bringing the same to proper camber by ½” thick mud plaster. |
| --- | --- |

1200.00 Sft		@ Rs. 3823/57	P% Sft				Rs. 45,883/=

(Rupees Three Thousand Eight Hundred Twenty

Three and Paisas Fifty Seven) only (Per Hundred Sft)

Total	Rs. 32,56,513/=

CONDITIONS

- The rates should be inclusive of all the Taxes i/c. Sales Tax, Octroi. Royalties etc, if any.

- No separate carriage will be allowed to the contractor.

- The Testing of the works/Pumping Machinery/Pipe will be arranged by the contractor at his own cost, No separate payment will be made.

- No premium will be allowed on Non-Schedule items.

- Any error or omission in the specifications/items will be governed by the relevant specifications and schedule of rates.

- The decision of the Executive Engineer will be final and binding on all the parties in any dispute.

| CONTRACTOR |  | EXECUTIVE ENGINEER PUBLIC HEALTH ENGG: DIVISION THARPARKAR @ MITHI |
| --- | --- | --- |