SCHEDULE “B”

CONSTRUCTION OF WATER SUPPLY SCHEME NEWABAD, BAJEER PARO, HAJAM PARO, ROLLA COLONY & KUNBHAR COLONY CHELHAR DISTRICT THARPARKAR

ABSTRACT OF COST

Part-A	Trunk Main/Distribution 6", 4" & 3" dia 		Rs. 18,58,098/=

Part-B	Pump House (10' x 12') (01 No:)			Rs. 2,33,681/=

Part-C	Pumping Machinery 20 BHP (2 Set)			Rs. 9,62,108/=

Total 		Rs. 30,53,887/=

Say		Rs. 30,53,900/=

| CONTRACTOR |  | EXECUTIVE ENGINEER PUBLIC HEALTH ENGG: DIVISION THARPARKAR @ MITHI |
| --- | --- | --- |

PART-A 	U.PVC PIPE 6",4" & 3" DIA.

| 1. | Dismantling and removing road matling. |
| --- | --- |

113.0 Cft			@ Rs. 605/=			P% Cft		Rs. 684/=

(Rupees Six Hundred Five) only (Per Hundred Cft)

| 2. | Providing PVC pipe of class “B” equivalent make fixing in trench i/c cutting, fitting and jointing with “Z” Joint with one rubber ring i/c testing with water to head of 61 meter or 200 ft. |
| --- | --- |

6" Dia		2500.0 Rft	@ Rs. 262/=			P.Rft			Rs.6,55,000/=

(Rupees Two Hundred Sixty Two) only (Per Rft)

4" Dia		2800.0 Rft	@ Rs. 137/=			P.Rft			Rs.3,83,600/=

(Rupees One Hundred Thirty Seven) only (Per Rft)

3" Dia		3600.0 Rft	@ Rs. 90/=			P.Rft			Rs.3,24,000/=

(Rupees Ninety) only (Per Rft)

| 3. | C.I sluice valve heavy pattern test pressure 21 kg/sq:cm or 300 lbs/sq: inch. |
| --- | --- |

6" Dia

04 Nos:		@ Rs. 9360/=		Each			Rs.37,440/=

(Rupees Nine Thousand Three Hundred Sixty) only (Each)

| 4. | PVC “Z” Joint fitting |
| --- | --- |

(i)	PVC Tee  6" dia

03 No:			@ Rs. 1800/=    		Each			Rs. 5,400/=

(Rupees One Thousand Eight Hundred) only (Each)

(ii)	PVC Tee  4" dia

6 No:			@ Rs. 1181/=    		Each			Rs. 7,086/=

(Rupees One Thousand One Hundred Eighty One)

only (Each)

(iii)	PVC Tee  3" dia

4 No:			@ Rs. 900/=    		Each			Rs. 3,600/=

(Rupees Nine Hundred) only (Each)

(iv)	PVC Reducer 4" dia

3 No:			@ Rs. 225/=    		Each			Rs. 675/=

(Rupees Two Hundred Twenty Five) only (Each)

(v)	PVC Reducer 6" dia

3 No:			@ Rs. 550/=    		Each			Rs. 1,650/=

(Rupees Five Hundred Fifty) only (Each)

(vi)	End Plug 3" dia

10 Nos:		@ Rs. 175/=    		Each			Rs. 1,750/=

(Rupees One Hundred Seventy Five) only (Each)

(vii)	Valve socket 3" dia

10 Nos:		@ Rs. 350/=    		Each			Rs. 3,500/=

(Rupees Three Hundred Fifty) only (Each)

(viii)	Valve socket 4" dia

6 Nos:			@ Rs. 500/=    		Each			Rs. 3,000/=

(Rupees Five Hundred) only (Each)

(ix)	Valve socket 6" dia

4 Nos:			@ Rs. 1650/=    		Each			Rs. 6,600/=

(Rupees One Thousand Six Hundred Fifty) only

(Each)

(x)	PVC Bend 6" dia

2 Nos:			@ Rs. 1913/=    		Each			Rs. 3,826/=

(Rupees One Thousand Nine Hundred Thirteen)

only (Each)

(xi)	PVC Bend 4" dia

1 No:			@ Rs. 956/=    		Each			Rs. 956/=

Rupees Nine Hundred Fifty Six) only (Each)

(xii)	PVC Bend 3" dia

1 No:			@ Rs. 731/=    		Each			Rs. 731/=

(Rupees Seven Hundred Thirty One) only (Each)

| 5. | Cast iron special for A.C pressure pipe (Standard Weight A.C.I.L “B” Class). |
| --- | --- |

(i)	Short Piece/Tail Piece

6" dia 	   8 No:	@ Rs. 618/75    		Each			Rs. 4,950/=

(Rupees Six Hundred Eighteen and Paisas

Seventy Five)  only (Each)

| 6. | Jointing C.I /M.S Flanged Pipes and special Flange inside trenches including i/c applying rubber packing of required thickness nuts and bolts with washer etc. and other rules required for jointing and testing be same to be specified presser etc complete. |
| --- | --- |

6" dia

8 Joints		@ Rs. 938/=			P.Joint			Rs. 7,504/=

(Rupees Nine Hundred Thirty Eight) only

(Per Joint)

| 7. | Making joints to PVC specials fitting including laying of specials and cost of solvent cement of required diameter and testing the joints with along with PVC pipe “B, “C” & “D” class to the specied pressure and making good o all leaky joints etc complete. |
| --- | --- |

6" dia 	   5 Joints	@ Rs. 96/=  			P.Joint			Rs. 480/=

(Rupees Ninety Six) only (Per Joint)

4" dia 	  13 Joints	@ Rs. 70/=  			P.Joint			Rs. 910/=

(Rupees Seventy) only (Per Joint)

3" dia 	  20 Joint	@ Rs. 59/=  			P.Joint			Rs. 1,180/=

(Rupees Fifty Nine) only (Per Joint)

| 8. | Excavation for pipe line trenches and pits in sandy soil including trimming and dressing sides to true alignment and shape leveling of beds of trenches to correct level and grade, cutting joint holes and disposal of surplus earth within a one chain as directed by Engineer Incharge. Providing fence guards, lights, flags, temporary crossings for non vehicular traffic where ever required lift upto 5 ft (1.52 m) and lead upto one chain (30.5 m). |
| --- | --- |

58700.0 Cft		@ Rs. 3750/=	P‰ Cft			Rs. 2,20,125/=

(Rupees Three Thousand Seven Hundred Fifty)

Only (Per Thousand Cft)

| 9. | Refilling the excavated stuff in trenches 6" thick layer i/c watering ramming to full compaction etc complete. |
| --- | --- |

52830.0 Cft		@ Rs. 2760/=	P‰ Cft			Rs. 1,45,811/=

(Rupees Two Thousand Seven Hundred Sixty)

Only (Per Thousand Cft)

| 10. | Providing chamber 3' x 2' (915 x 615 mm) inside dimension 4 ½' (1372 mm) deep as per approved design for sluice valve 3" to 12" dia with 18" (457 mm) dia inside cost iron cover and frame (wt = 1 cwt 3 qr) fixed in RCC 1:2:4 (102 mm) thick (with 5 Lbs steel per cft) 9" (299 mm) thick brick masonry wall set in 1:6 cement mortar 6" (1152 mm) thick cement concrete 1:3:6 in foundation 1" (25 mm) thick cement concrete 1:2:4 flooring ½" (12.5 mm) thick cement plaster 1:3 to all inside wall surface and to top i/c providing and fixing M.S foot rest at every one foot beyond 2 ½ ft depth curing, excavation, back filling and disposal of surplus earth etc complete. |
| --- | --- |

02 Nos:		@ Rs. 18820/=	Each				Rs. 37,640/=

(Rupees Eighteen Thousand Eight Hundred Twenty)

Only (Each)

Total		Rs. 18,58,098/=

PART-B	BRICK MASONRY PUMP HOUSE (10' X 12')

| 1. | Excavation in foundation of building bridges on other structure i/c dag-belling, dressing, refilling around the excavated earth watering and ramming lead upto one chain lift upto 5 ft (in sandy soil). |
| --- | --- |

235.00 Cft		@ Rs. 2722/50 	P‰ Cft			Rs. 640/=

(Rupees Two Thousand Seven Hundred Twenty Two

and Paisas Fifty)  only (Per Thousand Cft)

| 2. | Cement Concrete plain including placing, compacting, finishing and curing complete (including screening and washing of stone aggregate without shutting. |
| --- | --- |

Ratio (1:4:8)

84.00 Cft		@ Rs. 11288/75	P%Cft				Rs. 9,883/=

(Rupees Eleven Thousand Two Hundred Eighty Eight

and Paisas Seventy Five)  only (Per Thousand Cft)

| 3. | Pucca brick work in foundation and plinth in cement sand mortar (1:6). |
| --- | --- |

224.00 Cft		@ Rs. 11948/36	P% Cft				Rs. 26,764/=

(Rupees Eleven Thousand Nine Hundred Forty Eight

and Paisas Thirty Six)  only (Per Hundred Cft)

| 4. | Pucca brick work in ground floor in Cement sand mortar (1:4). |
| --- | --- |

281.0 Cft		@ Rs. 13227/41	P% Cft				Rs. 37,169/=

(Rupees Thirteen Thousand Two Hundred Twenty Seven

and Paisas Forty One) only (Per Hundred Cft)

| 5. | Reinforced cement concrete work including all labour and material except the cost of steel reinforcement and its labour for bending and binding, which will be paid separately. This rate also includes all kinds of forms moulds lifting shuttering curing rendering and finishing the exposed surface (including screening and washing of shingle) (a) RCC work in roof slab beams columns, rafts, lintels and other structural members laid in situ or precast laid in position complete in all respects Ratio (1:2:4) 90 lbs cement 2 Cft sand and 4 Cft shingle 1/8” to ¼” gauge. |
| --- | --- |

133.0 Cft		@ Rs. 337/=		P.Cft				Rs.44,821/=

(Rupees Three Hundred Thirty Seven) only (Per Cft)

| 6. | Fabrication of Mild steel reinforcement for cement concrete including cutting bending laying in position, making joints & fastenings including cost of binding wire (also includes removal of rust from bars) (USING TAR BARS). |
| --- | --- |

5.343 Cwt		@ Rs. 5001/70	P.Cwt		   		Rs. 26,724/=

(Rupees Five Thousand One and Paisas Seventy)

only (Per Cwt)

| 7. | Filling Watering and ramming earth in floors with surplus earth form foundation lead upto one chain and lift upto 5’ ft:. |
| --- | --- |

168.00 Cft		@ Rs. 1512/50	P‰ Cft			Rs. 254/=

(Rupees One Thousand Five Hundred Twelve and

Paisas Fifty)  only (Per Thousand Cft)

| 8. | Fabrication of heavy steel work with angles, tees, flat iron, iron and sheet iron for making trusses, girders, tanks etc, including cutting riveting, handing assembling and fixing but eluding erection in position. |
| --- | --- |

3.053 Cwt		@ Rs. 4928/49	P.Cwt				Rs. 15,047/=

(Rupees Four Thousand Nine Hundred Twenty Eight

and Paisas Forty Nine) only (Per Cwt)

| 9. | Supplying and fixing in position Iron/ Steel grill of 3/8” x ¼” size flat Iron or approved design including painting 3 Coats etc complete (Weight net less than 3.7 lb/sp: ft: of in finished grill.. |
| --- | --- |

36.00 Sft		@ Rs. 180/50		P.Sft				Rs. 6,498/=

(Rupees One Hundred Eighty and Paisas Fifty)

only (Per Sft)

| 10. | Making and fixing steel grated door with 1/16" thick sheeting i/c angle iron frame 2" x 2" x 3/8" & 3/4" squire bar 4" center to center with locking arrangement.. |
| --- | --- |

24.50 Sft		@ Rs. 726/72		P.Sft				Rs. 17,805/=

(Rupees Seven Hundred Twenty Six and

Paisas Seventy Two) only (Per Sft)

| 11. | Cement Plaster (1:6) upto 12’ height 1/2" thick. |
| --- | --- |

431.00 Sft		@ Rs. 2206/60 	P% Sft				Rs. 9,510/=

(Rupees Two Thousand Two Hundred Six and

Paisas Sixty) only (Per Hundred Sft)

| 12. | Cement Plaster (1:4) upto 12’ height 3/8" thick. |
| --- | --- |

431.00 Sft		@ Rs. 2197/52	P% Sft				Rs. 9,471/=

(Rupees Two Thousand One Hundred Ninety Seven

and Paisas Fifty Two) only (Per Hundred Sft)

| 13. | Cement pointing struck joint on wall Ration 1:3. |
| --- | --- |

471.00 Sft		@ Rs. 1213/58		P% Sft			Rs. 5,716/=

(Rupees One Thousand Two Hundred Thirteen

and Paisas Fifty Eight) only (Per Hundred Sft)

| 14. | Providing and laying 1” thick topping cement concrete (1:2:4) including surface finishing and dividing into panels. |
| --- | --- |

2" thick

351.0 Sft		@ Rs. 3275/50	P% Sft				Rs. 11,497/=

(Rupees Three Thousand Two Hundred Seventy Five

and Paisas Fifty) only (Per Hundred Sft)

| 15. | Color Wash (2 Coats). |
| --- | --- |

902.00 Sft		@ Rs. 859/90		P% Sft				Rs. 7,756/=

(Rupees Eight Hundred Fifty Nine and Paisas

Ninety) only (Per Hundred Sft)

| 16. | White Wash (1 Coat). |
| --- | --- |

(a)	431.00 Sft		@ Rs. 416/63		P% Sft				Rs. 1,796/=

(Rupees Four Hundred Sixteen and Paisas

Sixty Three) only (Per Hundred Sft)

(b) Three Coats

204.00 Sft		@ Rs. 829/95		P% Sft				Rs. 1,693/=

(Rupees Eight Hundred Twenty Nine and Paisas

Ninety Five) only (Per Hundred Sft)

| 17. | Painting New Surface preparing surface and painting of door and windows any type (including edges) (3 Coats). |
| --- | --- |

49.00 Sft		@ Rs. 2116/41	P% Sft				Rs. 1,037/=

(Rupees Two Thousand One Hundred Sixteen

and Paisas Forty One) only (Per Hundred Sft)

Total		Rs. 2,33,681/=

PART-C  	ELECTRIC PUMPING SET 20 BHP COUPLED WITH KSB CENTRIFUGAL PUMP

| 1. | Providing, installing and testing in position including transportation to site of work @ Islamkot Electric Pumping set consisting of Electric Motor 20 BHP 3-Phase, 50 cycle, 400/420 volts of 2900 RPM (Siemens Made) coupled with KSB centrifugal pump type ETA Norm 80+200 size 3" x 4" mounted on common steel frame or base plate capable to give discharge 286 IGPM against a head of 130 ft i/c installing of pumping set on cement concrete foundation ratio 1:2:4 with M.S nuts and bolts of 5/8" dia to be embedded in C.C foundation including P/F star delta starter 100 ampere (Green Power) with Motor control Unit consist of circuit breaker main switch 150 amperes (Siemens) automatic star delta starter under over voltage relay, phase failure relay, ampere meter 60 amperes, volt meter 500 volts indicating lights all fixed in metallic vernyle proof box. Providing internal wiring for mains with 2-7/0.064 PVC insulated wire 660 volts grade in 16 mm (1") PVC conduit fitted on surface as required including Providing/fixing earthen set with 0.6 M x 0.6 M x 6.3 mm (2 ft x 2 ft x ¼") copper plate buried in the ground at a depth of 3.7 M (12 ft) or less as per PWD specification and testing the pumping set against the required head and discharge for 72 hours etc complete (R.A approved). |
| --- | --- |

02 Sets		@ Rs. 4,30,349/=		P.Set		         Rs. 8,60,698/=

(Rupees Four Lac Thirty Thousand Three

Hundred Forty Nine) only (Per Set)

| 2. | Providing G.I Pipe, specials and clamps etc i/c fixing, cutting and fitting complete with end i/c the cost of breaking through walls and roof making good etc. painting tow coats after cleaning the pipe etc. with white zinc paint with pigment to match the colour of the building and testing with water to a pressure head of 200 ft and handling. |
| --- | --- |

4" dia

36.0 Rft		@ Rs. 896/17			P.Rft			Rs. 32,262/=

(Rupees Eight Hundred Ninety Six and

Paisas Seventeen) only (Per Rft)

6" dia

23.0 Rft		@ Rs. 818/86			P.Rft			Rs. 18,834/=

(Rupees Eight Hundred Eighteen and

Paisas Eighty Six) only (Per Rft)

| 3. | Providing M.S flanges made out 3/8" (10 mm) thick sheet including cutting, drilling holes etc complete (R.A.A) based on schedule. |
| --- | --- |

4" dia

8 Nos:			@ Rs. 596/75		Each			Rs. 4,774/=

(Rupees Five Hundred Ninety Six and

Paisas Seventy Five) only (Each)

6" dia

6 Nos:			@ Rs. 804/77		Each			Rs. 4,829/=

(Rupees Eight Hundred Four and

Paisas Seventy Seven) only (Each)

| 4. | Supplying C.I sluice valve heavy pattern test pressure 300 lbs/sq: inch (S.M.I No.2 P-12). Imported |
| --- | --- |

3" dia

2 Nos:			@ Rs. 4290/=		Each			Rs. 8,580/=

(Rupees Four Thousand Two Hundred Ninety) only

(Each)

| 5. | Reflex valve heavy pattern (Test pressure 21.0 kg/sq cm or 300 Lbs/sq inch (PHS M.I No.6 P-10). Imported |
| --- | --- |

3" Dia

2 Nos:			@ Rs. 1543/75		Each			Rs. 3,088/=

(Rupees One Thousand Five Hundred Forty Three

and Paisas Seventy Five) only (Each)

| 6. | Supplying/fixing foot valve heavy pattern with cone type gate (G.S.I. No. 4 P-12). |
| --- | --- |

4" Dia

2 Nos:			@ Rs. 893/75		Each			Rs. 1,788/=

(Rupees Eight Hundred Ninety Three and Paisas

Seventy Five) only (Each)

| 7. | C.I bend with flanged end with holes including turning and facing flanges for all size (S.M.I. No. 8 P-11). |
| --- | --- |

1.106 Cwt		@ Rs. 6096/=		P.Cwt			Rs. 6,742/=

(Rupees Six Thousand Ninety Six) only (Per Cwt)

| 8. | C.I Tee flange ends with holes i/c curing and facing of flanged for all size. |
| --- | --- |

Tee 6" x 6" x 3"

0.806 Cwt		@ Rs. 6096/=		P/Cwt			Rs. 4,913/=

(Rupees Six Thousand Ninety Six) only (Per Cwt)

| 9. | Jointing C.I/M.S flanged pipe and special and in jointing trenches i/c supply rubber packing of the required thickness nuts bolts with washier etc and other tools required for jointing and testing joint to the specified etc complete.(PHI NO. 1 P-35). |
| --- | --- |

4" Dia

08 Joints		@ Rs. 513/=			P.Joint		Rs. 4,104/=

(Rupees Five Hundred Thirteen) only

(Per Joint)

6" Dia

8 Joints		@ Rs. 938/=			P.Joint		Rs. 7,504/=

(Rupees Nine Hundred Thirty Eight) only

(Per Joint)

3" Dia

8 Joints		@ Rs. 499/=			P.Joint		Rs. 3,992/=

(Rupees Four Hundred Ninety Nine) only

(Per Joint)

Total 		Rs. 9,62,108/=

CONDITIONS

- The rates should be inclusive of all the Taxes i/c. Sales Tax, Octroi. Royalties etc, if any.

- No separate carriage will be allowed to the contractor.

- The Testing of the works/Pumping Machinery/Pipe will be arranged by the contractor at his own cost, No separate payment will be made.

- No premium will be allowed on Non-Schedule items.

- Any error or omission in the specifications/items will be governed by the relevant specifications and schedule of rates.

- The decision of the Executive Engineer will be final and binding on all the parties in any dispute.

- The contractor will have to submit the certificate of manufacturer at the time of supply of Machinery.

| CONTRACTOR |  | EXECUTIVE ENGINEER PUBLIC HEALTH ENGG: DIVISION THARPARKAR @ MITHI |
| --- | --- | --- |