| NAME OF WORK: | M&R TO MALIR COURT BUILDING KARACHI ( REPAIR OF DISTRICT & SESSION JUDGES CHAMBERS, BATHROOM, KITCHNET FLOORING WORK , WALL PANNELLING , FALSE CEILING COLORING WOODEN DOORS SANITARY FIXTURES & ELECTRIC WORK). |
| --- | --- |

SCHEDUELE”B”

| S/No | Description | Qty | Rate | Unit | Amount |
| --- | --- | --- | --- | --- | --- |
|  | PART-A-I(SCHEDULE ITEM). |  |  |  |  |
| 1. | Dismantling glazed or encaustic tiles etc (S.I-(55/13). | 811.0 Sft. | 786/50 | %Sft. | 6379/- |
| 2. | Distempering two coats( 24-b/54). | 95.0 Sft. | 1043/90 | %Sft. | 992/- |
| 3. | Preparing the surface and painting with matt finish in/c rubbing the surface with bathy (silicon carbide rubbing brick) filling the voids with zink / chalk / plaster of paris mixture , applying first coat premix, making the surface smooth and then painting 3 coats with matt finish of approved make etc complete (b) 2nd & subsequent coat. (36-a+b+b/55). | 429.0 Sft. | 3444/38 | %Sft. | 14776/- |
| 4. | First class deodar wood wrought , joinery in doors and windows etc fixed in position including chowkats hold fasts hinges, iron tower bolts , chocks , cleats handles and cords with hooks etc. ( 7-b/58(-) 25-b/61). | 82.25 Sft. | 902/93 | P.Sft. | 74266/- |
|  |  | Total A-I Rs. |  |  | 96413/- |
|  | PART-A-II |  |  |  |  |
| 1. | Laying floor of approved coloured glazed tiles ¼” thick in white cement 1:2 over ¾” thick cement mortar 1:2 complete. (24/43). | 30.0 sft. | 27678/86 | %Sft. | 8304/- |
| 2. | White glazed tiles ¼” thick dado jointed in white cement and laid over 1:2 cement sand mortar ¾” thick including finishing (37/45). | 104.50 Sft. | 28253/61 | %Sft. | 29525/- |
| 3. | S/F in position aluminum channels framing for hinges doors or alcop made with 5 mm thick tinted glass glazing (Belguim) and alpha (Japan) locks in/c handles stoppers etc (b) deluxe model (Bronze). (83-b/108). | 22.50 Sft. | 1507/66 | P.Sft. | 33922/- |
|  |  |  | Rs. |  | 71751/- |
|  |  |  | Total A-ii Rs. |  |  |
|  | PART-A-III NON SCHEDULE ITEM. |  |  |  |  |
| 1. | Providing and laying porcelain tiles of (Master make) to required color/ pattern & size (20”x20”x1/4 to 24”x24”x1/4” )in/c laying ¾” thick cement sand mortar bed ratio 1:6 in/c filling of joints with pigment . | 483.50 Sft. |  | P.Sft. |  |
| 2. | Providing and fixing false ceiling gypsum fibre board in/c panel of size (24”x24) in/c frame work of aluminum double channel section hanged with nails, hooks, wire to ceiling etc as directed by the consultants. | 441.0 Sft. |  | P.sft. |  |
| 3. | Providing and fixing paneling with lassani sheet ¾” thick and teak ply in/c aluminum sheet on back site for leakage control and fixing lassani sheet and then teak ply in approved design and texture with 3” beading including french polish 3 coats including frnch polish 3 coats including all cost of labour making and cartage etc complete. | 762.83 Sft. |  | P.sft. |  |
| 4. | Providing and laying matt/ glazed tiles make size (12”x24”x1/4”) (Shabbir/ Master). on floor & wall to required pattern and design i/c with cement sand mortar ¾” thick 1:6 ratio finishing cleaning etc complete. | 307.50 Sft. |  | P.Sft. |  |
| 5. | French polishing complete, First scraping old paint with chemical & bleach of (ICI make)1st coating of sealer (ICI Make) 3rd coating of varnish then rubbing and filling the joints and rubbing full and making base good & polishing coat of ICI lacquer etc complete. | 288.34 Sft. |  | P.Sft. |  |
| 6. | P/F Window & door Glass pane (5mm) frosted i/c removing the old Gola & P/F New 2 Burma teak Gola of required & existing size etc complete. | 47.27 Sft. |  | P.Sft. |  |
| 7. | P/F Marble Top 1” thick upto 24” wide chaina Verona super finish i/c makes half round Gola and P/F Stainless steel sink hole (16”x13”) i/cs the cost of cutting and fixing etc complete in all respect. | 20.0 Sft. |  | P.Sft. |  |
| 8. | Making and fixing kitchen cabinet (Stylish) Wall unit 12” depth with lamented both side lassani sheet ¾” thick in/c P/F Superior quality of hard ware S.S Hangers, fancy handle, catcher, grocery rack in/c the all cost of material labour etc complete. | 12.0 Sft. |  | P.sft. |  |
| 9. . | Making and fixing floor cabinet shutter with lassani sheet ¾” thick lamented i/c framing with (2”x1 ¼”) portal wood i/cs the cost of handle, catcher, wood half round Gola etc complete in all respect. | 26.12 sft. |  | P.Sft.. |  |
|  |  | Total A-iii Rs. |  |  |  |
|  | PART-B-SANITARY WORK (N.S.I). |  |  |  |  |
| 1. | Providing and fixing English commode set complete (Porta or equvelent make ) in/c all necessary fitting, seat cover, 4” dia bend and brass thumble etc complete. | 1-No. |  | Each. |  |
| 2. | Providing and fixing wash basin with padistol 24”x16” size (Plorta make ) fixing with s.s bracket 12” size (Porta make) fixing with S.S waste, 1 ¼” dia waster pipe and Nyloon connection etc complete. | 01-No. |  | Each. |  |
| 3. | Providing and fixing Bathroom accessories set (Sonex make) i/c mirror shelves, Towel rod, soap dish, ring brush holder Tissue paper holder and mirror. | 01-No. |  | Each. |  |
| 4. | Providing and fixing Basin mixtures (Master or sonex make) plastic coated etc complete. | 1-No. |  | Each. |  |
| 5. | Providing and fixing Tee stop cock (Master or sonex make ) plastic coated etc complete. | 04-No. |  | Each. |  |
| 6. | Providing and fixing Muslim shower with chain (Master or Sonex make). | 02-No. |  | Each. |  |
| 7. | Providing and fixing Double bib cock (Master or sonex make) plastic coated etc complete. | 2-No. |  | Each. |  |
|  |  | Total b- Rs. |  |  |  |
|  | PART-C-I (ELECTRIC WORK). (S.I). |  |  |  |  |
| 1. | Wiring for light or fan point with 3/.029 PVC insulated wire in 20mm(3/4 ”)channel patti on surface as required (S.I.No 129/15). | 16-Nos | 910/- | P.Point. | 14560/- |
| 2. | Wiring for plug point with 3/.029 PVC insulated wire in 20mm(3/4 ”)channel patti on surface as required (S.I.No 130/15). | 12-Nos . | 742/- | Each. | 8904/- |
| 3. | Providing and laying (main or sub main ) pvc insulated with single core copper conductor 250/440 volts size ( 2-7/.044)/6mm2.(47/6). | 30.10 meter. | 213/- | P/Meter. | 6411/- |
| 4. | Providing and fixing circuit breaker 6,10,15,20,30,40,50 and 63 amp SP (TB-5C) on prepared board as required. ( 203/31). | 2-Nos. | 916/- | Each. | 1832/- |
|  |  |  |  | Total C-I Rs. | 31707/- |
|  | PART-C-II NON SCHEDULE ITEM. |  |  |  |  |
| 1. | Providing and fixing fancy type switch etc complete. | 16-Nos |  | Each. |  |
| 2. | Providing and fixing fancy type 2 pin switch socket etc complete. | 12-Nos |  | Each. |  |
| 3. | Providing and fixing 3 pin 15 amp plug socket fancy type. | 4-Nos |  | Each. |  |
| 4. . | Providing and fixing wall bracket lower type fan (18”x24” dia ). Make (Pak ) in/c electric making connection etc complete. | 2-Nos |  | Each |  |
| 5. | Providing and fixing Exhaust fan plastic body best quality complete in/c electric connection as pre required. | 01-No. |  | Each. |  |
| 6. | Providing and fixing bell push fancy type etc complete. | 1-No. |  | Each. |  |
| 7. | Providing and fixing fancy type S.M.D ceiling light round/ square shape size 6”x6”/8”x8” in/c adopter (imported quality etc complete. | 12-Nos |  | Each. |  |
|  |  |  | C-II. Rs. |  |  |
|  |  |  | Total (C-i+ii) Rs. |  |  |
|  |  |  | G.Total Rs. |  |  |

TERMS AND CONDITION

1. 	Any typographical  errors in the schedule ‘B’ are subjected to any correction with the 	reference to the schedule of Rates General item 2012 in force from `12-07-2012 as 	approved by the standing rates committee Sindh Karachi.

2.	100% well graded crushed bajri shall be used in item of RCC 1:2:4.

3.	No Premium shall be paid on Non schedule items.

4.	The work will be carried out as per PWD Specifications.

| Contractor. | --s/d-- ASSISTANT ENGINEER Provincial Building Sub Division-XI, Karachi | --s/d-- EXECUTIVE ENGINEER Provincial Building Division-No.III Karachi |
| --- | --- | --- |