BILL OF QUANTITES

(A) DESCRIPTION AND RATE OF ITEMS BASED ON COMPOSITE SCHEDULE OF RATES

Name of Scheme:-  CONSTRUCTION OF SCHOOL BUILDING AT GBPS:VILLAGE NAZAR MOHAMMAD  PANHYAR TALUKA KOTDIJI DISTRICT KHAIRPUR

| S.NO | NAME OF ITEMS |  |  | QTY | RATE | UNIT | AMOUNT |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Excavation in foundation of building bridges and other str:with excavated earth lead unto one chain and lift upto 5 feet" ft in ordinary soil. (S.I.N-18-b/P-4) |  |  |  |  |  |  |
|  |  |  | 2595.0 |  | 3176/25 | %cft | 8242/0 |
| 2 | Cement concrete brick or stone ballast 1 ½” to 2" guage ratio 1:5:10 (S.I.N:4/P-14) |  |  |  |  |  |  |
|  |  |  | 1310.0 |  | 8694/95 | %cft | 113904/0 |
| 3 | Pacca brick work in foundation and plinth in cement sand mortar ratio 1:6 (S.I.No: 4e P-20) |  |  |  |  |  |  |
|  |  |  | 2523.0 |  | 11948/36 | %cft | 301457/0 |
| 4 | RCC work in roof slab columns rafts lintel and other structure member laid in situ or recast laid in position complete in all respect ratio 1:2:4 .( S.I.No:6-A/P-16) |  |  |  |  |  |  |
|  |  |  | 699.0 |  | 337/0 | P.sft | 233541/0 |
| 5 | Fabrication of tor steel reinforcement for cement concrete i/c cutting, bending, and laying in position i/c the cost of binding wire also removal of rust from bars.(S.I.N:7-ii/P-19) Tar Bar |  |  |  |  |  |  |
|  |  |  | 31.205 |  | 5001/70 | P.cwt | 156078/0 |
| 6 | Filling watering & ramming earth from foundation lead upto one chain lift upto 5 feet (S.I.N 21/P-4) |  |  |  |  |  |  |
|  |  |  | 1730.0 |  | 1512/50 | %ocft | 2617/0 |
| 7 | Filling watering & ramming earth from N/earth from o/side lead up to one chain lift up to 5 feet (S.I.No: 2/ P- 4) |  |  |  |  |  |  |
|  |  |  | 1976.0 |  | 3630/0 | %ocft | 7183/0 |
| 8 | Extra lead from 05 miles. |  |  |  |  |  |  |
|  |  |  | 1976.0 |  | 1013/24 | %cft | 20022/0 |
| 9 | Pacca brick work in G/Floor i/c stricking of joints cement sand mortar ratio1:6.(S.I.N: 51/P-20) |  |  |  |  |  |  |
|  |  |  | 3051.0 |  | 12674/36 | %cft | 386695/0 |
| 10 | S/Fixing G.I frame chowkata steel door frame size 7”x2” of 3”x4” type i/c fixing in position using 20 guage sheet welding fixing (S.I.N.29-P/32) |  |  |  |  |  |  |
|  |  |  | 100.0 |  | 228/90 | P.rft | 22890/0 |
| 11 | S/Fixing G.I frame chowkata steel window frame size 7”x2” of 3”x4” type i/c fixing in position using 20 guage sheet welding fixing (S.I.N.29-P/32) |  |  |  |  |  |  |
|  | C/R window | 10x28 | 280.0 |  |  |  |  |
|  |  |  | 280.0 |  | 240/50 | P.rft | 67340/0 |
| 12 | S/F steel grill 3/4" x1/4" flate of iron approved design i/c 3 coats painting (wt:not less than 3-7) (S.I.N: 26 P-92) |  |  |  |  |  |  |
|  |  |  | 382.0 |  | 180/50 | P.sft | 68951/0 |
| 13 | S/Fixing Iron Girders at of site work (Material Schedule 140) |  |  |  |  |  |  |
|  |  |  | 31.071 |  | 3850/0 | P.cwt | 119623/0 |
| S.NO | NAME OF ITEMS |  | QTY; RATE; UNIT; AMOUNT. | RATE | UNIT | AMOUNT |
| 14 | S/Fixing Iron Tears at Site of Work. ((Material Schedule 144) |  |  |  |  |  |  |
|  |  |  | 24.928 |  | 3575/0 | P.cwt | 89118/0 |
| 15 | Errection of Rolled steel beams (S.I.No.6 /P.90) |  |  |  |  |  |  |
|  |  |  | 55.999 |  | 186/34 | P.cwt | 10435/0 |
| 16 | 2nd class tiles roofing consisting 4” earth 1” mud plaster with Gobri leaping (S.I.No.2 /P-32) |  |  |  |  |  |  |
|  |  |  | 1457.0 |  | 7607/25 | %cft | 110838/0 |
| 17 | R C C spout i/c fixing in position. |  |  |  |  |  |  |
|  |  |  | 4.0 |  | 261/25 | P.No | 1045/0 |
| 18 | First class deodar wod wrought joinery in door and windows any type i/c fixing in position (Only shelters) (S.I.N.7b/ P-57) |  |  |  |  |  |  |
|  |  |  | 267.0 |  | 902/93 | P.sft | 241082/0 |
| 19 | Making notice board made in cement. (S.I.No.1 P/-94) |  |  |  |  |  |  |
|  |  |  | 64 |  | 58/11 | P.sft | 3719/0 |
| 20 | Cement plaster 1:6 uotp 20ht 1/2"thick.(S.I.N. 10-A/P-51) |  |  |  |  |  |  |
|  |  |  | 5312.0 |  | 2206/60 | %sft | 117215/0 |
| 21 | Cement plaster 1:4 uotp 20ht 3/8"thick (S.I.No: 13 a/ P-51) |  |  |  |  |  |  |
|  |  |  | 5312.0 |  | 2197/52 | %sft | 116732/0 |
| 22 | Cement pointing struck joints on walls (S.I.N: 19 /P-52) |  |  |  |  |  |  |
|  |  |  | 1382.0 |  | 1287/44 | %sft | 17792/0 |
| 23 | P/Laying 3" C C Topping 1.2.4. i/c surface finishing & dividing into panels(S.I.No: 16 d / P-41) |  |  |  |  |  |  |
|  |  |  | 1304.0 |  | 4411/82 | %sft | 57530/0 |
| 24 | P/Laying 2" C C Topping 1.2.4. i/c surface finishing & dividing into panels(S.I.No: 16 C / P-41) |  |  |  |  |  |  |
|  |  |  | 2425.0 |  | 3275/50 | %sft | 79431/0 |
| 25 | C.C Plain i/c placing compacting finishing and curring ratio 1:2:4 (S.I.No: 5/ P-15) |  |  |  |  |  |  |
|  |  |  | 66.0 |  | 14429/25 | %cft | 9523/0 |
| 26 | Laying Floor of approved white glazed tiles ¼” thick in white cement 1:2 over ¾” thick cement 1:2 complete (S.I.N: 24/P-42) |  |  |  |  |  |  |
|  |  |  | 74.0 |  | 27678/86 | %cft | 20482/0 |
| 27 | White glazed tiles ¼” thick dado jointed in white cement and laid over 1:2 cement sand mortor ¾” thick including finishing (S.I.N.37/P-44) |  |  |  |  |  |  |
|  |  |  | 134.0 |  | 28253/06 | %cft | 37860/0 |
| 28 | Providing and laying HALLA or pattren tiles glazed 6”x6”x1/4” on floor or wall facing in required colour and pattren of STILE specification jointed in white cement and pigment over a base of 1:2 grey cement mortor ¾” thick i/c washing and filling of joints with slurry of white cement and pigment in desired shape with finishing, cleaning and cost of wax polish etc. complete i/c cutting tiles to proper profile (S.I.No. 61/ P-47) |  |  |  |  |  |  |
|  |  |  | 155.0 |  | 47651/56 | %sft | 73860/0 |
| 29 | White wash three coats.(S.I.No.26b/P.No.53) |  |  |  |  |  |  |
|  |  |  | 2225.0 |  | 829/95 | %sft | 18466/0 |
| S.NO | NAME OF ITEMS |  | QTY; RATE; UNIT; AMOUNT. | RATE | UNIT | AMOUNT |
| 30 | Primary coats of chalk under distempering. (S.I.No.23/P.No.53) |  |  |  |  |  |  |
|  |  |  | 4193.0 |  | 442/75 | %sft | 18565/0 |
| 31 | Distempering in Two coats (S.I.No:24b/P-53) |  |  |  |  |  |  |
|  |  |  | 4193.0 |  | 1043/90 | %sft | 43771/0 |
| 32 | Preparing surface painting to doors & windows any type three coats. (S.I.No.4c(ii) P-67) |  |  |  |  |  |  |
|  |  |  | 509.0 |  | 2116/41 | %sft | 7668/0 |
| 33 | Painting guarders barse gates iron barse 3 coats. (S.I.No.14d/P-68) |  |  |  |  |  |  |
|  |  |  | 1346.0 |  | 1270/80 | %sft | 17105/0 |
| 34 | Colour wash two coat. (S.I.No.25 B/P-53) |  |  |  |  |  |  |
|  |  |  | 1382.0 |  | 859/90 | %sft | 11884/0 |
|  |  |  |  |  | Total: |  | 2617544/0 |

Amount Total (a)

________% above/below on the rates of CSR Rs.______________/- Amount to be added/deducted   on the basis of premium quoted

Total (A) = in words & figures __________________________________________________

___________________________________________________________________________

CONTRACTOR

EXECUTIVE ENGINEER                        EDUCATION WORKS DIVISION                             KHAIRPUR