| Name of Work :- | Construction of road from Drib Mehar Shah to Village Mithu Thebo mile 0/0-0/5 (1.0 K.m). |
| --- | --- |

SCHEDULE “B”

| Quantity. | Item of Work. | Rate. | Unit. | Amount. |
| --- | --- | --- | --- | --- |

01. Earth work for embankment by bulldozer including ploughing mixing, clod breaking dressing and compacting with optimum moisture content, lead Upto 100ft: and lift upto 5ft: in all types of soil except rock.

(A) COMPACTING UPTO 85%MODIFIED AASHO DENSITY.

| 154000 | Cft | @ | Rs: | 8679.15 | P.%0 Cft | Rs: | 1336581.00 |
| --- | --- | --- | --- | --- | --- | --- | --- |

(B) COMPACTING UPTO 95-100%MODIFIED AASHO DENSITY.

| 47586 | Cft | @ | Rs: | 11208.9 | P.%0 Cft | Rs: | 533387.00 |
| --- | --- | --- | --- | --- | --- | --- | --- |

02. Preparing Sub-base by supplying and spreading stone metal 1-1/2”-2”guage of approved quality from approved quarry in required thickness to proper camber and grade including hand packing filling voids with 10 Cft screening & non plastic quarry fines of approved source, watering & compacting to achieve 98-100%density as per modified AASHO Specification (Rate includes all cost of materials T&P and carriage upto site of work)

| 20394 | Cft | @ | Rs: | 6214.46 | P.% Cft | Rs: | 1267376.00 |
| --- | --- | --- | --- | --- | --- | --- | --- |

03. Laying brick on end edging including supplying 9”x4-1/2”x3” first class bricks, excavation for having edging with small size parallel to the road (Rate includes lead upto site of work).

| 6798 | Rft | @ | Rs: | 2690.88 | P. % Rft | Rs: | 182926.00 |
| --- | --- | --- | --- | --- | --- | --- | --- |

04. Preparing base course i/c supplying and spreading stone metal of approved quality properly graded to maximum size of 1-1/2”in required thickness to proper camber and grade including supplying and spreading 15 Cft screenings and non plastic quarry fines, filling depressions with stone metal after rolling including watering and compacting the same so as to achieve 100%density as per modified AASHO Specifications. (Rate includes providing and using templates, camber plates, screens, forms as directed (Rate includes all cost of materials T&P and carriage upto site of work).

| 10197 | Cft | @ | Rs: | 7171.96 | P. % Cft | Rs: | 731324.00 |
| --- | --- | --- | --- | --- | --- | --- | --- |

05. Providing surface dressing Ist coat on new or existing surface with 30 lbs of bitumen and 4.0 Cft of Crush bajri of required size including cleaning the road surface rolling etc, complete (Rate includes all cost of materials T&P and carriage upto site of work).

| 40788 | Sft | @ | Rs: | 1602.65 | P. % Sft | Rs: | 653688.00 |
| --- | --- | --- | --- | --- | --- | --- | --- |

| Quantity. | Item of Work. | Rate. | Unit. | Amount. |
| --- | --- | --- | --- | --- |

06. Providing 1”Thick (Consolidated) premixed carpet in proper camber and grade including supplying 10 Cft Crush bajri 4 Cft hill sand (of approved quality and grade) 67 lbs bitumen of 80/100 penetration including mixing in mechanical mixer in required proportion, including heating materials and cleaning the road surface (Hill sand 2 Cft for mixing and 2 Cft for dusting (Rate includes all cost of materials T&P and carriage upto site of work).

| 40788 | Sft | @ | Rs: | 4487.09 | P. % Sft | Rs: | 1830194.00 |
| --- | --- | --- | --- | --- | --- | --- | --- |

07. Earth work embankment from barrow Pits including laying in 6”layers, clod breaking ramming dressing complete, lead upto 100’ft:lift upto 5ft: ( in ordinary soil.

| 23165 | Cft | @ | Rs: | 7233.57 | P. %0 Cft | Rs: | 167565.00 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Total | Rs: | 6870614.00 |

| Note:- Subject to any changes variation in rates/ | Executive Engineer |
| --- | --- |
| Quantities in Technical Section. | Highway Division |

Khairpur

Contractor ______________

Part-B-(   )-Construction of 3.0’Span Culverts.

| Name of Work: - | Construction of road from Drib Mehar Shah to Village Mithu Thebo mile 0/0-0/5 (1.0 K.m). |
| --- | --- |

SCHEDULE “B”

| Quantity | Item of work | No.L.B.D. | Rate | Unit | Amount |
| --- | --- | --- | --- | --- | --- |

01. Excavation in foundation of building, bridges and other structure i/c degbelling, dressing, refilling around structure with excavated earth watering and ramming lead upto one chain and lift upto 5’(S.I.No.18b, P-5).

| 909 | Cft | Rs,1663/75 | P.%0 Cft | Rs,1512/- |
| --- | --- | --- | --- | --- |

02. C.C.brick or stone ballast 1-1/2” to 2” guage Ratio (1:4:8) (S.I.No.4, P-17).

| 180 | Cft | Rs,9416/28 | P.% Cft | Rs,16949/- |
| --- | --- | --- | --- | --- |

03. Pacca brick work in f/p in cement sand mortar (1:4). (S.I.No.4, P-25).

| 269 | Cft | Rs,12501/41 | P.% Cft | Rs,33629/- |
| --- | --- | --- | --- | --- |

04. Cement Concrete Plain i/c placing compacting, finishing and curing complete (i/c screening and washing of stone aggregate without shuttering (1:2:4) (S.I.No.5h, P-18).

| 15 | Cft | Rs,14429/25 | P.% Cft | Rs,2164/- |
| --- | --- | --- | --- | --- |

05. Fabrication of mild steel r/f for cement concrete i/c cutting, bending, laying in Position making joints and fastening i/c cost of binding wire (also i/c removal of rust from bars) (S.I.No.7ii, P-20).

A)for Bed Plate.

| 5.09 | Cwt | Rs,5001/70 | P.Cwt | Rs,24535/- |
| --- | --- | --- | --- | --- |

06. R.C.C.Work i/c all labor and material except the cost of steel r/f and its labor for bending and binding which will be paid separately, This rate also i/c all kinds of  forms, moulds, lifting, shuttering, curing, rendering and finishing the exposed surface (i/c screening and washing of shingle) (a) R.C.C.Work in roof slabs beams, columns, lintels and other structural member slain in situ or pre-cast laid in Position complete in all respects Ratio (1:2:4) (S.I.No.6a, P-19).

| 111 | Cft | Rs,337/- | P.Cft | Rs,37407/- |
| --- | --- | --- | --- | --- |

07. Cement Plaster (1:4) upto 20’height ½”thick.

| 158 | Sft | Rs,2344/59 | P.% Sft | Rs,3704/-__ |
| --- | --- | --- | --- | --- |
|  |  |  | Total | Rs:165675/- |

| Note:- Subject to any changes variation in rates/ | Executive Engineer |
| --- | --- |
| Quantities in Technical Section. | Highway Division |

Khairpur

Contractor ______________

Part-B-Masonary Structure.

| Name of Work: - | Construction of Pipe Culverts on road from Drib Mehar Shah to Village Mithu Thebo mile 0/0-0/5 (1.0 K.m). |
| --- | --- |

SCHEDULE “B”

| SCQuantity | Item of work | No.L.B.D. | Rate | Unit | Amount |
| --- | --- | --- | --- | --- | --- |

01. Excavation in foundation of building, bridges and other structure i/c degbelling, dressing, refilling around structure with excavated earth watering and ramming lead upto one chain and lift upto 5’(S.I.No.18b, P-5).

| 252 | Cft | Rs,3176/25 | P.%0 Cft | Rs,800/- |
| --- | --- | --- | --- | --- |

02. C.C.brick or stone ballast 1-1/2” to 2” guage Ratio (1:4:8) (S.I.No.4, P-17).

| 48 | Cft | Rs,9416/28 | P.% Cft | Rs,4520/- |
| --- | --- | --- | --- | --- |

03. Pacca brick work in f/p in cement sand mortar (1:4). (S.I.No.4, P-25).

| 64 | Cft | Rs,12501/41 | P.% Cft | Rs,8001/- |
| --- | --- | --- | --- | --- |

04. Erection and removal of centering for R.c.c.or Plain C.C. works of portal wood (S.I.No.18b, P-21).

| 128 | Sft | Rs,3127/41 | P.% Sft | Rs,4003/- |
| --- | --- | --- | --- | --- |

05. Cement Concrete Plain i/c placing compacting, finishing and curing complete (i/c screening and washing of stone aggregate without shuttering (1:2:4) (S.I.No.5h, P-18).

| 67 | Cft | Rs,14429/25 | P.% Cft | Rs,9667/- |
| --- | --- | --- | --- | --- |

06. Cement Plaster (1:4) upto 20’height ½”thick.

| 94 | Sft | Rs,2344/59 | P.% Sft | Rs,2204/- |
| --- | --- | --- | --- | --- |

07. Providing R.c.c.Pipes of A.S.T.M.C.76-62 TC/76-70 Class II wall bed fixing in trench including cutting fitting and joints with rubbers including testing with water to specified(1.5’dia)

| 30 | Rft | Rs,618/- | P.Rft | Rs,18540/- |
| --- | --- | --- | --- | --- |
|  |  |  | Total | Rs:47735/- |

| Note:- Subject to any changes variation in rates/ | Executive Engineer |
| --- | --- |
| Quantities in Technical Section. | Highway Division |

Khairpur

Contractor ______________

Part-B-(  )-Construction/fixing of Paving blocks/Paver on New road.

| Name of Work :- |  |
| --- | --- |

SCHEDULE “B”

| Quantity. | Item of Work. | Rate. | Unit. | Amount. |
| --- | --- | --- | --- | --- |

01. Providing Sand Cushion including supplying and spreading Pit/Canal Sand of approved quality from approved Source of supply to site of work including watering and rolling etc, complete Rate includes all cost of materials T&P labour and carriage of 3-0 Miles (Average).

| 1030 | Cft | @ | Rs: | 1479.41 | P. % Cft | Rs: | 15238.00 |
| --- | --- | --- | --- | --- | --- | --- | --- |

02. Cement Concrete brick or Stone ballast 1-1/2” to 2” guage Ratio (1:4:8) (S.I.No.4, P-17).

| 515 | Cft | @ | Rs: | 9416.28 | P. % Cft | Rs: | 48494.00 |
| --- | --- | --- | --- | --- | --- | --- | --- |

03. Crushed fire bricks grated as sand (S.I.No.51 P,No.04)

| 170 | Cft | @ | Rs: | 2500.00 | P.% Cft | Rs: | 4250.00 |
| --- | --- | --- | --- | --- | --- | --- | --- |

04. Supplying and filling Sand under floor and plugging in wells. (S.I.No.29, P-31)

| 170 | Cft | @ | Rs: | 151.25 | P.% Cft | Rs: | 257.00 |
| --- | --- | --- | --- | --- | --- | --- | --- |

05. Erection and removal of centering for R.c.c.or Plain C.C. works of portal wood (S.I.No.18b, P-21)

| 150 | Sft | @ | Rs: | 3127.41 | P. % Sft | Rs: | 4691.00 |
| --- | --- | --- | --- | --- | --- | --- | --- |

06. Cement Concrete Plain i/c Placing compacting, finishing and curing complete (i/c screening and washing of stone aggregate without shuttering (1:3:6) (S.I.No.5h, P-18).

| 155 | Cft | @ | Rs: | 14429.25 | P. % Cft | Rs: | 22365.00 |
| --- | --- | --- | --- | --- | --- | --- | --- |

07. Providing & fixing cement paving blocks flooring having size of 197x97x           60 (mm) of city/quddra/cobble shape with natural colours, having strength b/w 5000 psi to 8500 psi i/c filling the joints with hill sand and laying in specified manner/pattern and design etc complete.

| 1030 | Sft | @ | Rs: | 199.77 | P. Sft | Rs: | 205763.00 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Total | Rs: | 301058.00 |

| Note:- Subject to any changes variation in rates/ | Executive Engineer |
| --- | --- |
| Quantities in Technical Section. | Highway Division |

Khairpur

Contractor ______________

| Name of Work:- | Construction of New Side Drain on road from from Drib Mehar Shah to Village Mithu Thebo mile 0/0-0/5 (1.0 K.m). |
| --- | --- |

SCHEDULE “B”

| QUANTITY | ITEM OF WORK | RATE | UNIT | AMOUNT |
| --- | --- | --- | --- | --- |

01. Excavation in foundation of building, bridges and other structure i/c degbelling, dressing, refilling around structure with excavated earth watering and ramming lead up to one chain and lift up to 5’(S.I.No.18b, P-5).

| 700 | Cft | Rs,1663/75 | P. %0 Cft | Rs,1165/- |
| --- | --- | --- | --- | --- |

02. Cement Concrete brick or Stone ballast 1-1/2” to 2” gauge Ratio (1:4:8) (S.I.No.4, P-17).

| 115 | Cft | Rs,9416/28 | P. % Cft | Rs,10829/- |
| --- | --- | --- | --- | --- |

03. Pacca brick work in foundation and Plinth in Cement Sand motor Ratio (1:4).(S.I.No.4,P-25).

| 328 | Cft | Rs,12501/41 | P. % Cft | Rs,41005/- |
| --- | --- | --- | --- | --- |

04. Cement Concrete Plain i/c Placing compacting, finishing and curing complete (i/c screening and washing of stone aggregate without shuttering (1:3:6) (S.I.No.5h, P-18).

| 37 | Cft | Rs,14429/25 | P. % Cft | Rs,5339/- |
| --- | --- | --- | --- | --- |

05. Fabrication of mild Steel reinforcement for Cement Concrete i/c cutting, bending, laying in Position making joints and fastening i/c cost of binding wire (also i/c removal of rust from bars) (S.I.No.7ii, P-20).

| 2.73 | Cwt | Rs,4820/20 | P. Cwt | Rs,13159/- |
| --- | --- | --- | --- | --- |

06. R.C.C.Work i/c all labor and material except the cost of steel r/f and its labor for bending and binding which will be paid separately, This rate also i/c all kinds of  forms, moulds, lifting, shuttering, curing, rendering and finishing the exposed surface (i/c screening and washing of shingle) (a) R.C.C.Work in roof slabs beams, columns, lintels and other structural member slain in situ or pre-cast laid in Position complete in all respects Ratio (1:2:4) (S.I.No.6a, P-19).

| 68 | Cft | Rs,337/- | P.Cft | Rs,22916/- |
| --- | --- | --- | --- | --- |

07. Cement Plaster (1:4) Up to 20’ height ½”thick.

| 488 | Sft | Rs,2344/59 | P. % Sft | Rs,11442/- |
| --- | --- | --- | --- | --- |

|  |  |  | Total | Rs:105855/- |
| --- | --- | --- | --- | --- |

| Note:- Subject to any changes variation in rates/ | Executive Engineer |
| --- | --- |
| Quantities in Technical Section. | Highway Division |

Khairpur

Contractor ______________

Part-   Construction of New Drain Cross.

| Name of Work:- | Construction of New Side Drain on road from from Drib Mehar Shah to Village Mithu Thebo mile 0/0-0/5 (1.0 K.m). |
| --- | --- |

SCHEDULE “B”

| QUANTITY | ITEM OF WORK | RATE | UNIT | AMOUNT |
| --- | --- | --- | --- | --- |

01. Excavation in foundation of building, bridges and other structure i/c degbelling, dressing, refilling around structure with excavated earth watering and ramming lead upto one chain and lift upto 5’(S.I.No.18b, P-5).

| 97 | Cft | Rs,3176/25 | P. %0 Cft | Rs,308/- |
| --- | --- | --- | --- | --- |

02. Cement Concrete brick or stone ballast 1 ½”-2” guage Ratio (1:4:8) Hillsand to be used).

| 21 | Cft | Rs,9416/28 | P.% Cft | Rs,1977/- |
| --- | --- | --- | --- | --- |

03. Pacca brick work in foundation and plinth in cement mortar Ratio (1:4)

| 45 | Cft | Rs,12501/41 | P.% Cft | Rs,5626/- |
| --- | --- | --- | --- | --- |

04. Cement Concrete Plain i/c Placing compacting, finishing and curing complete (i/c screening and washing of stone aggregate without shuttering (1:3:6) (S.I.No.5h, P-18)

| 05 | Cft | Rs,14429/25 | P.% Cft | Rs,721/- |
| --- | --- | --- | --- | --- |

05. Fabrication of mild Steel reinforcement for Cement Concrete i/c cutting, bending, laying in Position making joints and fastening i/c cost of binding wire (also i/c removal of rust from bars) (S.I.No.7ii, P-20).

| 0.863 | Cwt | Rs,5001/70 | P. Cwt | Rs,4160/- |
| --- | --- | --- | --- | --- |

06. R.C.C.Work i/c all labor and material except the cost of steel r/f and its labor for bending and binding which will be paid separately, This rate also i/c all kinds of  forms, moulds, lifting, shuttering, curing, rendering and finishing the exposed surface (i/c screening and washing of shingle) (a) R.C.C.Work in roof slabs beams, columns, lintels and other structural member slain in situ or pre-cast laid in Position complete in all respects Ratio (1:2:4) (S.I.No.6a, P-19).

| 33 | Cft | Rs,337/- | P. Cft | Rs,11121/- |
| --- | --- | --- | --- | --- |

07. Cement Plaster (1:3) upto 20’height 1/2:”thick.

| 40 | Sft | Rs,2344/59 | P.% Sft | Rs,938/- |
| --- | --- | --- | --- | --- |
|  |  |  | Total | Rs,26744/- |

| Note:- Subject to any changes variation in rates/ | Executive Engineer |
| --- | --- |
| Quantities in Technical Section. | Highway Division |

Khairpur

Contractor ______________