1) Improvement.

| Name of Work:- | Construction/Improvement of Internal roads of Police Line S.S.P. Office Khairpur mile 0/0-0/5 (1.0 K.m). |
| --- | --- |

SCHEDULE “B”

| Quantity. | Item of Work | Rate | Unit | Amount. |
| --- | --- | --- | --- | --- |

01. Excavation in existing berm for widening the road, including preparation of Sub grade after watering, rolling with power Roller, dressing the excavated stuff etc complete.

| 10197 | Cft | Rs,526/27 | P.%0 Cft | Rs,53664/- |
| --- | --- | --- | --- | --- |

02. Making diagonal groves of ½”x1-1/2” at 2 FT: centre to centre in road surface.

| 20394 | Cft | Rs,146/61 | P.% Cft | Rs:29900/- |
| --- | --- | --- | --- | --- |

03. Preparing Sub-base by supplying and spreading stone metal 1-1/2”-2”guage of approved quality from approved quarry in required thickness to proper camber and grade including hand packing filling voids with 10 Cft screening & non plastic quarry fines of approved source, watering & compacting to achieve 98-100%density as per modified AASHO Specification (Rate includes all cost of materials T&P and carriage up to site of work.

| 7648 | Cft | Rs,6259.02 | P.% Cft | Rs,478689.00 |
| --- | --- | --- | --- | --- |

04. Laying brick on end edging including supplying 9”x4-1/2”x3” first class bricks, excavation for having edging with small size parallel to the road (Rate includes lead up to site of work.

| 3399 | Rft | Rs,2740.37 | P. % Rft | Rs,93145.00 |
| --- | --- | --- | --- | --- |

05. Preparing base course i/c supplying and spreading stone metal of approved quality properly graded to maximum size of 1-1/2”in required thickness to proper camber and grade including supplying and spreading 15 Cft screenings and non plastic quarry fines, filling depressions with stone metal after rolling including watering and compacting the same so as to achieve 100%density as per modified AASHO Specifications. (Rate includes providing and using templates, camber plates, screens, forms as directed (Rate includes all cost of materials T&P and carriage up to site of work.

| 15296 | Cft | Rs,7220.14 | P. % Cft | Rs,1104392.00 |
| --- | --- | --- | --- | --- |

06. Providing surface dressing Ist coat on new or existing surface with 30 lbs of bitumen and 4.0 Cft of Crush bajri of required size including cleaning the road surface rolling etc, complete (Rate includes all cost of materials T&P and carriage up to site of work.

| 30591 | Sft | Rs,1604.95 | P. % Sft | Rs,490970/- |
| --- | --- | --- | --- | --- |

| Quantity. | Item of Work | Rate | Unit | Amount. |
| --- | --- | --- | --- | --- |

07. Providing 1½”thick (Consolidated) premixed carpet in proper camber and grade including supplying 15 Cft bajri 5 Cft hill sand (of approved quality and grade) 93 lbs bitumen of 80/100 penetration including mixing in mechanical mixer in required proportion, including heating materials and cleaning the road surface (Hill sand 3 Cft for mixing and 2.0 Cft for dusting (Rate includes all cost of materials T&P and carriage upto site of work.

| 30591 | Sft | Rs,6305.21 | P. % Sft | Rs,1928826.00 |
| --- | --- | --- | --- | --- |

08. Earth work embankment from barrow Pits including laying in 6”layers, clod breaking ramming dressing complete, lead upto 100’ft:lift upto 5ft: ( in ordinary soil.

| 26000 | Sft | Rs,7233.57 | P. %0 Cft | Rs,188072.00 |
| --- | --- | --- | --- | --- |
|  |  |  | Total | Rs:4367658.00 |

| Note:- Subject to any changes variation in rates/ | Executive Engineer |
| --- | --- |
| Quantities in Technical Section. | Highway Division |

Khairpur

Contractor ______________

2) New Construction.

| Name of Work:- | Construction/Improvement of Internal roads of Police Line S.S.P. Office Khairpur mile 0/0-0/5 (1.0 K.m). |
| --- | --- |

SCHEDULE “B”

| Quantity. | Item of Work | Rate | Unit | Amount. |
| --- | --- | --- | --- | --- |

01. Providing Sand Cushion including supplying and spreading Pit/Canal Sand of approved quality from approved Source of supply to site of work including watering and rolling etc, complete Rate includes all cost of materials T&P labour and carriage of 3-0 Miles (Average)

| 30591 | Cft | Rs,1479.41 | P.% Cft | Rs,452566.00 |
| --- | --- | --- | --- | --- |

02. Preparing Sub-base by supplying and spreading stone metal 1-1/2”-2”guage of approved quality from approved quarry in required thickness to proper camber and grade including hand packing filling voids with 10 Cft screening & non plastic quarry fines of approved source, watering & compacting to achieve 98-100%density as per modified AASHO Specification (Rate includes all cost of materials T&P and carriage up to site of work.

| 15296 | Cft | Rs,6259.02 | P.% Cft | Rs,955063.00 |
| --- | --- | --- | --- | --- |

03. Laying brick on end edging including supplying 9”x4-1/2”x3” first class bricks, excavation for having edging with small size parallel to the road (Rate includes lead up to site of work.

| 3399 | Rft | Rs,2740.37 | P. % Rft | Rs,93145.00 |
| --- | --- | --- | --- | --- |

04. Preparing base course i/c supplying and spreading stone metal of approved quality properly graded to maximum size of 1-1/2”in required thickness to proper camber and grade including supplying and spreading 15 Cft screenings and non plastic quarry fines, filling depressions with stone metal after rolling including watering and compacting the same so as to achieve 100%density as per modified AASHO Specifications. (Rate includes providing and using templates, camber plates, screens, forms as directed (Rate includes all cost of materials T&P and carriage up to site of work.

| 7648 | Cft | Rs,7220.14 | P. % Cft | Rs,552196.00 |
| --- | --- | --- | --- | --- |

05. Providing surface dressing Ist coat on new or existing surface with 30 lbs of bitumen and 4.0 Cft of Crush bajri of required size including cleaning the road surface rolling etc, complete (Rate includes all cost of materials T&P and carriage up to site of work.

| 30591 | Sft | Rs,1604.95 | P. % Sft | Rs,490970/- |
| --- | --- | --- | --- | --- |

06. Providing 1½”thick (Consolidated) premixed carpet in proper camber and grade including supplying 15 Cft bajri 5 Cft hill sand (of approved quality and grade) 93 lbs bitumen of 80/100 penetration including mixing in mechanical mixer in required proportion, including heating materials and cleaning the road surface (Hill sand 3 Cft for mixing and 2.0 Cft for dusting (Rate includes all cost of materials T&P and carriage upto site of work.

| 30591 | Sft | Rs,6305.21 | P. % Sft | Rs,1928826.00 |
| --- | --- | --- | --- | --- |

| Quantity. | Item of Work | Rate | Unit | Amount. |
| --- | --- | --- | --- | --- |

08. Earth work embankment from barrow Pits including laying in 6”layers, clod breaking ramming dressing complete, lead upto 100’ft:lift upto 5ft: ( in ordinary soil.

| 34667 | Sft | Rs,7233.57 | P. %0 Cft | Rs,250766.0 |
| --- | --- | --- | --- | --- |
|  |  |  | Total | Rs:4723532.00 |

| Note:- Subject to any changes variation in rates/ | Executive Engineer |
| --- | --- |
| Quantities in Technical Section. | Highway Division |

Khairpur

Contractor ______________

Part-B-(  )-Construction/fixing of Paving blocks/Paver on New road.

| Name of Work :- | Construction/Improvement of Internal roads of Police Line S.S.P. Office Khairpur mile 0/0-0/5 (1.0 K.m). |
| --- | --- |

MEASUREMENT CUM ABSTRACT.

| Quantity. | Item of Work. | Rate. | Unit. | Amount. |
| --- | --- | --- | --- | --- |

01. Providing Sand Cushion including supplying and spreading Pit/Canal Sand of approved quality from approved Source of supply to site of work including watering and rolling etc, complete Rate includes all cost of materials T&P labour and carriage of 3-0 Miles (Average).

| 1030 | Cft | @ | Rs: | 1479.41 | P. % Cft | Rs: | 15238.00 |
| --- | --- | --- | --- | --- | --- | --- | --- |

02. Cement Concrete brick or Stone ballast 1-1/2” to 2” guage Ratio (1:4:8) (S.I.No.4, P-17).

| 515 | Cft | @ | Rs: | 9416.28 | P. % Cft | Rs: | 48494.00 |
| --- | --- | --- | --- | --- | --- | --- | --- |

03. Crushed fire bricks grated as sand (S.I.No.51 P,No.04)

| 170 | Cft | @ | Rs: | 2500.00 | P.% Cft | Rs: | 4250.00 |
| --- | --- | --- | --- | --- | --- | --- | --- |

04. Supplying and filling Sand under floor and plugging in wells. (S.I.No.29, P-31)

| 170 | Cft | @ | Rs: | 151.25 | P.% Cft | Rs: | 257.00 |
| --- | --- | --- | --- | --- | --- | --- | --- |

05. Erection and removal of centering for R.c.c.or Plain C.C. works of portal wood (S.I.No.18b, P-21)

| 150 | Sft | @ | Rs: | 3127.41 | P. % Sft | Rs: | 4691.00 |
| --- | --- | --- | --- | --- | --- | --- | --- |

06. Cement Concrete Plain i/c Placing compacting, finishing and curing complete (i/c screening and washing of stone aggregate without shuttering (1:3:6) (S.I.No.5h, P-18).

| 155 | Cft | @ | Rs: | 14429.25 | P. % Cft | Rs: | 22365.00 |
| --- | --- | --- | --- | --- | --- | --- | --- |

07. Providing & fixing cement paving blocks flooring having size of 197x97x           60 (mm) of city/quddra/cobble shape with natural colours, having strength b/w 5000 psi to 8500 psi i/c filling the joints with hill sand and laying in specified manner/pattern and design etc complete.

| 1030 | Sft | @ | Rs: | 199.77 | P. Sft | Rs: | 205763.00 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Total | Rs: | 301058.00 |

| Note:- Subject to any changes variation in rates/ | Executive Engineer |
| --- | --- |
| Quantities in Technical Section. | Highway Division |

Khairpur

Contractor ______________

Part-(C)-Scheme Board.

| Name of Work :- | Supplying and fixing Scheme board |
| --- | --- |

SCHEDULE “B”

| Quantity | Item of Work. | Rate. | Unit. | Amount. |
| --- | --- | --- | --- | --- |

01. Providing & Fixing Informatory Sign Rectangular shape 4”x3”comprising 2Nos. 3”dia G.I.pipe Post i/c painting Writing as directed each.

| 01 | No. | Rs:16956.00 | Each | Rs: | 16956.00 |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Total | Rs: | 16956.00 |

| Note:- Subject to any changes variation in rates/ | Executive Engineer |
| --- | --- |
| Quantities in Technical Section. | Highway Division |

Khairpur

Contractor ______________