PART’A’I, M/STRUCTURE

| ## NAME OF SCHEME:- | Construction of 10 Rft: Bridge over Mehar Shah near village Dupwaro Taluka Kotdiji |
| --- | --- |

(Schedule “B”)

| QTY. | ITEM OF WORK. | UNIT | RATE | AMOUNT |
| --- | --- | --- | --- | --- |

1.   Excavation in foundation of building bridges and other structure i/c dagblling dressing refilling around structure with excavated earth watering and ramming lead up to  5.0 ft. ordinary soil.

4371   Cft.        @  Rs. 3176/25 	              P. %o  Cft.     		Rs. 13883/-

2. Cement concrete brick or stone ballast 1 ½”-2” gauge  Ratio 1:4:8.

688  		Cft.  @  Rs.9416/28            P. % Cft.      		 Rs.64784/-

3.	Pacca bricks work in foundation and plinth in cement Sand mortar Ratio 1:4.

2053  	 @  Rs.12501/410  	          P. %  Cft.       	          Rs.256654/-

4.	 Cement concrete plain i/c placing compacting finishing and curing complete i/c screening and washing of stone aggregate with out shuttering Ratio 1:2:4.

82   Cft.                      @  Rs.14429/25            P. %  Cft.               Rs. 11668/-

5.	Fabrication of mile steel reinforcement for cement concrete i/c cutting beading laying in position making joint and fasting i/c cost of steel and binding wire also i/c removal of rust from bar.

35.10  Cwt.  @  Rs.4820/20  	                       P. Cwt.                 Rs.169189/-

6.	R.C.C. work i/c all lab our and materials except the cost of steel reinforcement and its lab our for bending and binding which will be paid separately this rate also i/c all kinds of forms moulds lifting shuttering currying rending and finishing the exposed surface (i/c screening and washing of shingle) R.C.C work in roof slab beams columns rafts lintels and other structural laid in site or recast in position complete in all respect. Ratio 1:2:4: 90 Lbs. Cement 2 Cft. sand 4 Cft. Shingle 1/8”-1/4” gauge.

741  Cft.                	   @ Rs.337/-   	         P.  Cft.             	Rs. 249717

7.	Cement plaster (1:3) upto 20’ height ½”thick.

423   Sft. 	   @  Rs.2344/59 	         P. %  Sft.      		         Rs.9918/-

8.      /-Earth work for embankment from barrow pit i/cs: laying in 6”layers

clod breaking remaining dressing complete lead upto 100’ & lift upto 5.0’

(In ordinary Soil

11220 Cft:    @Rs:6278/37                 P.%0Cft:                         Rs:70443/-

Total       Rs.846256/-

| NOTE: Subject to any Changes/ Variation in Rates/ Quantity as per T.Sanction. |  |
| --- | --- |
| # CONTRACTOR | EXECUTIVE ENGINEER HIGHWAY DIVISION # KHAIRPUR |