| Name of Work. | Construction 8’’Span bridge on Ujjan Shaikh near at Village Ali Khan Ujjan U.C.Tando Masti Taluka Khairpur. |
| --- | --- |

SCHEDULE “B”

| Quantity | Item of Work | Rate | Unit | Amount |
| --- | --- | --- | --- | --- |

01. Excavation in foundation of building, bridges and other structure i/c degbelling, dressing, refilling around structure with excavated earth watering and ramming lead upto one chain and lift upto 5’(S.I.No.18b, P-5).

| 3256 | Cft | Rs,3176/25 | P. %0 Cft | Rs,10342/- |
| --- | --- | --- | --- | --- |

02. Cement Concrete brick or Stone ballast 1-1/2” to 2” guage Ratio (1:4:8) (S.I.No.4, P-17).

| 763 | Cft | Rs,9416/28 | P. % Cft | Rs,71846/- |
| --- | --- | --- | --- | --- |

03. Pacca brick work in foundation and Plinth in Cement Sand mortor Ratio (1:4).

| 1280 | Cft | Rs,12501/41 | P. % Cft | Rs,160018/- |
| --- | --- | --- | --- | --- |

04. Cement Concrete Plain i/c Placing compacting, finishing and curing complete (i/c screening and washing of stone aggregate without shuttering (1:3:6) (S.I.No.5h, P-18).

| 72 | Cft | Rs,14429/25 | P. % Cft | Rs,10389/- |
| --- | --- | --- | --- | --- |

05. Fabrication of mild Steel reinforcement for Cement Concrete i/c cutting, bending, laying in Position making joints and fastening i/c cost of binding wire (also i/c removal of rust from bars) (S.I.No.7ii, P-20).

| 25.46 | Cwt | Rs,4820/20 | P.Cwt | Rs,122722/- |
| --- | --- | --- | --- | --- |

06. R.C.C.Work i/c all labor and material except the cost of steel r/f and its labor for bending and binding which will be paid separately, This rate also i/c all kinds of  forms, moulds, lifting, shuttering, curing, rendering and finishing the exposed surface (i/c screening and washing of shingle) (a) R.C.C.Work in roof slabs beams, columns, lintels and other structural member slain in situ or pre-cast laid in Position complete in all respects Ratio (1:2:4) (S.I.No.6a, P-19).

| 580 | Cft | Rs,337/- | P.Cft | Rs,195460/- |
| --- | --- | --- | --- | --- |

07. Cement Plaster (1:3) upto 20’height ½”thick. (S.I.No.13b, P-58).

| 359 | Sft | Rs,2344/59 | P.% Sft | Rs,8417/- |
| --- | --- | --- | --- | --- |

| Quantity | Item of Work | Rate | Unit | Amount |
| --- | --- | --- | --- | --- |

08. Earth work embankment from barrow Pits including laying in 6”layers, clod breaking ramming dressing complete, lead upto 100’ft:lift upto 5ft: ( in ordinary soil).(S.I.No.2,P-01)

With extra lead.

|  | 133300 | Cft | Rs,2208/37 | P. %0 Cft |  | Rs,294375.00 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Total |  | Rs,873509.00 |  |
| Note:- Subject to any changes variation in rates/ |  |  |  |  | Executive Engineer |  |  |
| Quantities in Technical Section. |  |  |  |  | Highway Division |  |  |

Khairpur

Contractor ______________