RECONDITIONING / CONST OF ROAD FROM INDUS HIGHWAYS TO ABDUL SATTAR BHAYO TORI FARM TO QIYAS BHAYO ROAD MILE 1/7-3/7

DRAFT BIDDING DOCUMENT FOR WORKS

BILLS OF QUANTITIES

- Description and rate of Items bases on composite schedule of Rates

Part “A” Recond: (ROAD WORK)

| Item No. | Qty’s | Description of item to be executed at site |  | Rate | Unit | Amount in Rupees |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 |  | 4 | 5 | 6 |
| 01/- | 42827Cft: | Preparing base course supplying and spreading stone metal of approved quality of 6” in 2 layers from approved quarry properly grade to maximum size of 1½” in required thickness of proper camber & grade i/c S/S 15 Cft: screening & non plastic quarry fines filling & depressions with stone metal after initial rolling including watering & compacting the same so as to achieve 100% density as per modified AASHO specifications (This i/cs providing and using templates camber plates screen from as directed).Rate includes all costs of materials T&P and carriage upto site of work. |  | 9500/62 | P.% Cft: | 4068630/= |
| 02/- | 13596Rft: | Laying brick on end edging including supply of 9”x 4 ½”x3” 1st: class burnt bricks excavation for laying edging with small side parallel to the road. Rate includes all costs of materials T&P and carriage upto site of work. |  | 2751/19 | P.% Rft: | 374052/= |
| 03/- | 81576 Sft: | Providing 1st: coat of surface dressing on or new existing surface with 30 lbs: bitumen and 4.0 cft: cursh bajri of required size including cleaning the road surface rolling etc: complete.Rate includes all costs of materials T&P & carriage. |  | 1668/81 | P.% Sft: | 1361351/= |
| 04/- | 81576 Sft: | Providing 1” thick consolidated premixed carpet in proper camber and grade i/c supply of 10 Cft: bajri 4.0 Cft: hill sand of approve quality and guage and bitumen 67 lbs: of 80/100 penetration i/c mixing the mechanical mixture in required proportion i/c heating the material and cleaning the road surface. Rate i/c all cost of material T&P and carriage upto site of work. |  | 4552/11 | P.% Sft: | 3713430/= |
| 05/- | 506595Cft: | Earth Work embankment from barrow pits i/c laying in 6” layers, clod breaking remaining dressing complete, lead Upto 100’ft and lift upto 5’ft (In ordinary Soil) |  | 2208/37 | P.%0 Cft: | 1118749/= |
|  |  |  | TOTAL |  |  | 10636422/= |

_____________% Above / Below on the rates of CSR.

Amount to be added /deducted on the

Total (A) 									Rs:

Total Amount in words  ____________________________________________________

__________________________________________________________________________

Executive Engineer,

Contractor						                           Highways Division

Kashmore @ Kandhkot

*Imran*

RECONDITIONING / CONST OF ROAD FROM INDUS HIGHWAYS TO ABDUL SATTAR BHAYO TORI FARM TO QIYAS BHAYO ROAD MILE 1/7-3/7

DRAFT BIDDING DOCUMENT FOR WORKS

BILLS OF QUANTITIES

()	Description and rate of Items bases on composite schedule of Rates

Part “B” Const (ROAD WORK)

| Item No. | Qty’s | Description of item to be executed at site | Rate | Unit | Amount in Rupees |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 |
| 01/- | 344884 Cft: | Earth work for road embankment by bulldozers i/c plugging and mixing clod breaking dressing and compacting with optimum moisture content lead upto 100’ and lift upto 5.0’ in all types of soil except rock compacting upto 85% density as per modified AASHO specifications. | 8775/37 | P.0% Cft: | 3026485/= |
| 02/- | 48946 Cft: | Earth work for road embankment by bulldozers i/c plugging and mixing clod breaking dressing and compacting with optimum moisture content lead upto 100’ and lift upto 5.0’ in all types of soil except rock compacting upto 95-100% density as per modified AASHO specifications. | 2529/77 | P.% Cft: | 123822/= |
| 03/- | 24473Cft: | Preparing sub base course by supplying & spreading stone metal 1 ½”-2” gauge of approved quarry in required 3” thickness in 1 layer each 3.0 each layers with 20 cft: pit canal sand and having plasticity index of not more than 6% watering & compacting to achieve 98/100% density as per modified AASHO specifications. Rate includes all costs of materials T&P and car riage upto site of work. | 8464/45 | P.% Cft: | 2071504/= |
| 04/- | 8158Rft: | Laying brick on end edging including supply of 9”x 4 ½”x3” 1st: class burnt bricks excavation for laying edging with small side parallel to the road. Rate includes all costs of materials T&P and carriage upto site of work. | 2751/19 | P.% Rft: | 142862/= |

Contd: P/2

P-2

| Item No. | Qty’s | Description of item to be executed at site | Rate | Unit | Amount in Rupees |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 |
| 05/- | 12236Cft: | Preparing base course supplying and spreading stone metal of approved quality of 6” in 2 layers from approved quarry properly grade to maximum size of 1½” in required thickness of proper camber & grade i/c S/S 15 Cft: screening & non plastic quarry fines filling & depressions with stone metal after initial rolling including watering & compacting the same so as to achieve 100% density as per modified AASHO specifications (This i/cs providing and using templates camber plates screen from as directed).Rate includes all costs of materials T&P and carriage upto site of work. | 9500/62 | P.% Cft: | 1162495/= |
| 08/- | 48946 Sft: | Providing 1st: coat of surface dressing on or new existing surface with 30 lbs: bitumen and 4.0 cft: cursh bajri of required size including cleaning the road surface rolling etc: complete.Rate includes all costs of materials T&P & carriage. | 1668/81 | P.% Sft: | 816815/= |
| 09/- | 48946 Sft: | Providing 1” thick consolidated premixed carpet in proper camber and grade i/c supply of 10 Cft: bajri 4.0 Cft: hill sand of approve quality and guage and bitumen 67 lbs: of 80/100 penetration i/c mixing the mechanical mixture in required proportion i/c heating the material and cleaning the road surface. Rate i/c all cost of material T&P and carriage upto site of work. | 4552/11 | P.% Sft: | 2228075/= |
|  |  |  | TOTAL |  | 9572087/= |

_____________% Above / Below on the rates of CSR.

Amount to be added /deducted on the

Total (B) 									Rs:

Total Amount in words  ____________________________________________________

__________________________________________________________________________

Executive Engineer,

Contractor						                           Highways Division

Kashmore @ Kandhkot

*Imran*

RECONDITIONING / CONST OF ROAD FROM INDUS HIGHWAYS TO ABDUL SATTAR BHAYO TORI FARM TO QIYAS BHAYO ROAD MILE 1/7-3/7

DRAFT BIDDING DOCUMENT FOR WORKS

BILLS OF QUANTITIES

- Description and rate of Items bases on composite schedule of Rates.

Part  “C” 275 RFT(RETAINING WALL).

| Item No. | Qty’s | Description of item to be executed at site | Rate | Unit | Amount in Rupees |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 |
| 01/- | 5500 Cft: | Excavation in foundation of building bridge other structure i/cs dag belling dressing refilling around the structure with excavated earth watering and remaining lead upto one chain and left upto 5.0’ ordinary soil. | 3176/25 | P.%0 Cft: | 17469/= |
| 02/- | 1375Cft: | Cement concrete with bricks are stone plaster 1½” to 2” guage ratio 1:2:4 | 9416/28 | P.% Cft: | 129474/= |
| 03/- | 9213 Cft: | Pucca Brick work in foundation and plinth i/c cement sand mortor | 12501/41 | P.% Cft: | 1151692/= |
| 04/- | 78 Cft: | Cement concrete plain i/c placing compacting finishing curing complete i/c screening washing of stone aggregate with out suhttering ratio 1:2:4: | 14429/25 | P.% Cft: | 11210/= |
| 05/- | 138Sft: | Erection and removal of centering for R.C.C or plain cement work (2nd: class) for partal wood vertical (S.I.No:18)(b) P-20). | 7000/00 | P.% Sft: | 9625/= |
| 06/- | 1925Sft: | Cement Pointing Struck jointing Wall ratio 1:3 | 1213/58 | P.% Sft: | 23361/= |
| 07/- | 15400Cft: | Filling watering and remaining earth under floor with new earth excavated from side lead upto 100’ft and lift upto 5’ft. | 3630/00 | P.0% Cft: | 55902/= |
|  |  |  | TOTAL |  | 1398734/= |

Amount Total

__________________________% Above / Below on the rates of CSR. 	Total  =

Amount to be added /deducted on the

basis of premium quoted Total

Total in works in figure

Executive Engineer,

Contractor						                           Highways Division

Kashmore @ Kandhkot

*Imran*

RECONDITIONING / CONST OF ROAD FROM INDUS HIGHWAYS TO ABDUL SATTAR BHAYO TORI FARM TO QIYAS BHAYO ROAD MILE 1/7-3/7

DRAFT BIDDING DOCUMENT FOR WORKS

BILLS OF QUANTITIES

- Description and rate of Items bases on composite schedule of Rates.

PART-D  (3.0 FT: SPAN CULVERTS).

| Item No. | Qty’s | Description of item to be executed at site | Rate | Unit | Amount in Rupees |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 |
| 01/- | 1106 Cft: | Excavation in foundation of building bridge other structure i/cs dag belling dressing refilling around the structure with excavated earth watering and remaining lead upto one chain and left upto 5.0’ ordinary soil. | 3176/25 | P.%0 Cft: | 3512/= |
| 02/- | 237 Cft: | Cement concrete with bricks are stone plaster 1½” to 2” guage ratio 1:2:4 | 9416/28 | P.% Cft: | 22317/= |
| 03/- | 502 Cft: | Pucca Brick work in foundation and plinth i/c cement sand mortor | 12501/41 | P.% Cft: | 62757/= |
| 04/- | 55 Cft: | Cement concrete plain i/c placing compacting finishing curing complete i/c screening washing of stone aggregate with out suhttering ratio 1:2:4: | 14429/25 | P.% Cft: | 7936/= |
| 05/- | 58 Sft: | Erection and removal of centering for R.C.C or plain cement work (2nd: class) for partal wood vertical | 7000/00 | P.% Sft: | 4060/= |
| 06/- | A. Tor Steel 4.16 Cwt B. M/ Steel 0.907 Cwt | Fabrication of m/s reinforcement cement concrete including cutting binding laying in position making joints & fastening of i/c binding wire also removal rust from bars. | 5001/70 4820/29 | P.Cwt: P.Cwt | 20807/= 4372/= |
| 07/- | 114 Rft: | Reinforced cement concrete work including all labour and material except the cost of steel reinforcement and its labour for bending and binding which will be paid separately. This rate also includes all kind of forms moulds lifting shuttering curing rend. | 337/00 | P.Rft: | 38418/= |
| 08/- | 330 Sft: | Cement Plaster (1:3) upto 20’ height 1½” thick. | 2283/93 | P.% Sft: | 7537/= |
|  |  |  | TOTAL |  | 171716/= |

Amount Total (d)

__________________________% Above / Below on the rates of CSR. 	Total (d) =

Amount to be added /deducted on the

basis of premium quoted Total (d)

Total (d) =  in works in figure

Executive Engineer,

Contractor						                           Highways Division

Kashmore @ Kandhkot

*Imran*

CONST OF ROAD FROM KANDHKOT MALGUZAR ROAD (LOHI PULL) ROAD TO VILLAGE MUREED KHAN KHOSO ROAD MILE 0/0-1/4 I/C RETAINING WALL

DRAFT BIDDING DOCUMENT FOR WORKS

BILLS OF QUANTITIES

(A)		Description and rate of Items bases on composite schedule of Rates

Part “A”  (ROAD WORK)

| Item No. | Qty’s | Description of item to be executed at site | Rate | Unit | Amount in Rupees |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 |
| 01/- | With Out Lead 951926 Cft: With Lead 156103 Cft | Earth work for road embankment by bulldozers i/c plugging and mixing clod breaking dressing and compacting with optimum moisture content lead upto 100’ and lift upto 5.0’ in all types of soil except rock compacting upto 85% density as per modified AASHO specifications. | 3656/23 8775/86 | P.0% Cft: P.0%Cft | 3480460/= 1369938/= |
| 02/- | 97891 Cft: | Earth work for road embankment by bulldozers i/c plugging and mixing clod breaking dressing and compacting with optimum moisture content lead upto 100’ and lift upto 5.0’ in all types of soil except rock compacting upto 95-100% density as per modified AASHO specifications. | 2529/77 | P.% Cft: | 247642/= |
| 03/- | 48946Cft: | Providing sand cushion i/c supplying & spreading pit/canal sand of approved quality from approved source of supply where ever available site of work i/c carriage site of work (R.A.A). | 1479/41 | P.% Cft: | 724112/= |
| 04/- | 48946Cft: | Preparing sub base course by supplying & spreading stone metal 1 ½”-2” gauge of approved quarry in required 3” thickness in 1 layer each 3.0 each layers with 20 cft: pit canal sand and having plasticity index of not more than 6% watering & compacting to achieve 98/100% density as per modified AASHO specifications. Rate includes all costs of materials T&P and car riage upto site of work. | 8930/05 | P.% Cft: | 4370902/= |
| 05/- | 16315Rft: | Laying brick on end edging including supply of 9”x 4 ½”x3” 1st: class burnt bricks excavation for laying edging with small side parallel to the road. Rate includes all costs of materials T&P and carriage upto site of work. | 2761/22 | P.% Rft: | 450493/= |

Contd: P/2

P-2

| Item No. | Qty’s | Description of item to be executed at site | Rate | Unit | Amount in Rupees |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 |
| 06/- | 24473Cft: | Preparing base course supplying and spreading stone metal of approved quality of 6” in 2 layers from approved quarry properly grade to maximum size of 1½” in required thickness of proper camber & grade i/c S/S 15 Cft: screening & non plastic quarry fines filling & depressions with stone metal after initial rolling including watering & compacting the same so as to achieve 100% density as per modified AASHO specifications (This i/cs providing and using templates camber plates screen from as directed).Rate includes all costs of materials T&P and carriage upto site of work. | 9982/50 | P.% Cft: | 2443017/= |
| 08/- | 97891 Sft: | Providing 1st: coat of surface dressing on or new existing surface with 30 lbs: bitumen and 4.0 cft: cursh bajri of required size including cleaning the road surface rolling etc: complete.Rate includes all costs of materials T&P & carriage. | 1683/14 | P.% Sft: | 1647642/= |
| 09/- | 97891 Sft: | Providing 1” thick consolidated premixed carpet in proper camber and grade i/c supply of 10 Cft: bajri 4.0 Cft: hill sand of approve quality and guage and bitumen 67 lbs: of 80/100 penetration i/c mixing the mechanical mixture in required proportion i/c heating the material and cleaning the road surface. Rate i/c all cost of material T&P and carriage upto site of work. | 4588/88 | P.% Sft: | 4492100/= |
|  |  |  | TOTAL |  | 19226306/= |

_____________% Above / Below on the rates of CSR.

Amount to be added /deducted on the

Total (A) 									Rs:

Total Amount in words  ____________________________________________________

__________________________________________________________________________

Executive Engineer,

Contractor						                           Highways Division

Kashmore @ Kandhkot

*Imran*

CONST OF ROAD FROM KANDHKOT MALGUZAR ROAD (LOHI PULL) ROAD TO VILLAGE MUREED KHAN KHOSO ROAD MILE 0/0-1/4 I/C RETAINING WALL

DRAFT BIDDING DOCUMENT FOR WORKS

BILLS OF QUANTITIES

(A) 	Description and rate of Items bases on composite schedule of Rates.

PART-B  (3.0 FT: SPAN CULVERTS).

| Item No. | Qty’s | Description of item to be executed at site | Rate | Unit | Amount in Rupees |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 |
| 01/- | 1206 Cft: | Excavation in foundation of building bridge other structure i/cs dag belling dressing refilling around the structure with excavated earth watering and remaining lead upto one chain and left upto 5.0’ ordinary soil. | 3176/25 | P.%0 Cft: | 3831/= |
| 02/- | 237 Cft: | Cement concrete with bricks are stone plaster 1½” to 2” guage ratio 1:2:4 | 9416/28 | P.% Cft: | 22317/= |
| 03/- | 502 Cft: | Pucca Brick work in foundation and plinth i/c cement sand mortor | 12501/41 | P.% Cft: | 62757/= |
| 04/- | 55 Cft: | Cement concrete plain i/c placing compacting finishing curing complete i/c screening washing of stone aggregate with out suhttering ratio 1:2:4: | 14429/25 | P.% Cft: | 7936/= |
| 05/- | 58 Sft: | Erection and removal of centering for R.C.C or plain cement work (2nd: class) for partal wood vertical | 7000/00 | P.% Sft: | 4060/= |
| 06/- | A. Tor Steel 4.16 Cwt B. M/ Steel 0.907 Cwt | Fabrication of m/s reinforcement cement concrete including cutting binding laying in position making joints & fastening of i/c binding wire also removal rust from bars. | 5001/70 4820/29 | P.Cwt: P.Cwt | 20807/= 4372/= |
| 07/- | 114 Rft: | Reinforced cement concrete work including all labour and material except the cost of steel reinforcement and its labour for bending and binding which will be paid separately. This rate also includes all kind of forms moulds lifting shuttering curing rend. | 337/00 | P.Rft: | 38418/= |
| 08/- | 330 Sft: | Cement Plaster (1:3) upto 20’ height 1½” thick. | 2283/93 | P.% Sft: | 7537/= |
|  |  |  | TOTAL |  | 171716/= |

Amount Total (b)

__________________________% Above / Below on the rates of CSR. 	Total (b) =

Amount to be added /deducted on the

basis of premium quoted Total (b)

Total (b) =  in works in figure

Executive Engineer,

Contractor						                           Highways Division

Kashmore @ Kandhkot

*Imran*

CONST OF ROAD FROM KANDHKOT MALGUZAR ROAD (LOHI PULL) ROAD TO VILLAGE MUREED KHAN KHOSO ROAD MILE 0/0-1/4 I/C RETAINING WALL

DRAFT BIDDING DOCUMENT FOR WORKS

BILLS OF QUANTITIES

(A) 	Description and rate of Items bases on composite schedule of Rates.

Part  “C” 213 RFT(RETAINING WALL).

| Item No. | Qty’s | Description of item to be executed at site | Rate | Unit | Amount in Rupees |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 |
| 01/- | 3408 Cft: | Excavation in foundation of building bridge other structure i/cs dag belling dressing refilling around the structure with excavated earth watering and remaining lead upto one chain and left upto 5.0’ ordinary soil. | 3176/25 | P.%0 Cft: | 10825/= |
| 02/- | 852Cft: | Cement concrete with bricks are stone plaster 1½” to 2” guage ratio 1:2:4 | 9416/28 | P.% Cft: | 80226/= |
| 03/- | 3595 Cft: | Pucca Brick work in foundation and plinth i/c cement sand mortor | 12501/41 | P.% Cft: | 449425/= |
| 04/- | 53 Cft: | Cement concrete plain i/c placing compacting finishing curing complete i/c screening washing of stone aggregate with out suhttering ratio 1:2:4: | 14429/25 | P.% Cft: | 7647/= |
| 05/- | 107Sft: | Erection and removal of centering for R.C.C or plain cement work (2nd: class) for partal wood vertical (S.I.No:18)(b) P-20). | 7000/00 | P.% Sft: | 7490/= |
| 06/- | 1491Sft: | Cement Pointing Struck jointing Wall ratio 1:3 | 1213/58 | P.% Sft: | 18094/= |
| 07/- | 11928Cft: | Filling watering and remaining earth under floor with new earth excavated from side lead upto 100’ft and lift upto 5’ft. | 3630/00 | P.0% Cft: | 43299/= |
|  |  |  | TOTAL |  | 617006/= |

Amount Total

__________________________% Above / Below on the rates of CSR. 	Total  =

Amount to be added /deducted on the

basis of premium quoted Total

Total in works in figure

Executive Engineer,

Contractor						                           Highways Division

Kashmore @ Kandhkot

*Imran*

CONST OF ROAD FROM KANDHKOT MALGUZAR ROAD (LOHI PULL) ROAD TO VILLAGE MUREED KHAN KHOSO ROAD MILE 0/0-1/4 I/C RETAINING WALL

DRAFT BIDDING DOCUMENT FOR WORKS

BILLS OF QUANTITIES

(A) 	Description and rate of Items bases on composite schedule of Rates.

PART-D  (MISC:).

| Item No. | Qty’s | Description of item to be executed at site | Rate | Unit | Amount in Rupees |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 |
| 01/- | 01 Nos; | Proving and Fixing informatory sign rectangular shape 4’ x 3” comprising 02 Nos; 3” dia G.I Pipe Post i/c painting writing as directed each. | 16965/00 | P.Each | 16,965/= |
|  |  |  | TOTAL |  | 16,965/= |

Amount Total (D)

__________________________% Above / Below on the rates of CSR. 	Total (D) =

Amount to be added /deducted on the

basis of premium quoted Total (D)

Total (d) =  in works in figure

Executive Engineer,

Contractor						                           Highways Division

Kashmore @ Kandhkot

*Imran*

CONSTRUCTION OF LINK ROAD FROM HAFIZULLAH KHOSO TO VILLAGE ASADULLAH KHAN KHOSO ROAD MILE 0/1+220-0/4

DRAFT BIDDING DOCUMENT FOR WORKS

BILLS OF QUANTITIES

(A) 	Description and rate of Items bases on composite schedule of Rates

Part “A” (ROAD WORK)

| Item No. | Qty’s | Description of item to be executed at site | Rate | Unit | Amount in Rupees |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 |
| 01/- | 210151 Cft: | Earth work for road embankment by bulldozers i/c plugging and mixing clod breaking dressing and compacting with optimum moisture content lead upto 100’ and lift upto 5.0’ in all types of soil except rock compacting upto 85% density as per modified AASHO specifications. | 3656/23 | P.0% Cft: | 768360/= |
| 02/- | 21753 Cft: | Earth work for road embankment by bulldozers i/c plugging and mixing clod breaking dressing and compacting with optimum moisture content lead upto 100’ and lift upto 5.0’ in all types of soil except rock compacting upto 95-100% density as per modified AASHO specifications. | 2529/77 | P.% Cft: | 55030/= |
| 03/- | 10876Cft: | Providing sand cushion i/c supplying & spreading pit/canal sand of approved quality from approved source of supply where ever available site of work i/c carriage site of work (R.A.A). | 1479/41 | P.% Cft: | 160912/= |
| 04/- | 10876Cft: | Providing Sub Base course by supplying and spreading stone metal 1 ½” gauge of approved quality from approved quarry in required thickness to proper camber and grade i/c hand packing mixing with 10’ screening & non plastic quarry of approved quality & gauge from approved source i/c watering and compacting to achieve 98-100% density as per modified AASHO specification ( rate i/cs all cost of material T&P and carriage upto site of wok.) | 8697/23 | P.% Cft: | 945910/= |
| 05/- | 3625Rft: | Laying brick on end edging including supply of 9”x 4 ½”x3” 1st: class burnt bricks excavation for laying edging with small side parallel to the road. Rate includes all costs of materials T&P and carriage upto site of work. | 2834/15 | P.% Rft: | 102737/= |

Contd: P/2

P-2

| Item No. | Qty’s | Description of item to be executed at site | Rate | Unit | Amount in Rupees |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 |
| 06/- | 5438Cft: | Preparing base course supplying and spreading stone metal of approved quality of 6” in 2 layers from approved quarry properly grade to maximum size of 1½” in required thickness of proper camber & grade i/c S/S 15 Cft: screening & non plastic quarry fines filling & depressions with stone metal after initial rolling including watering & compacting the same so as to achieve 100% density as per modified AASHO specifications (This i/cs providing and using templates camber plates screen from as directed).Rate includes all costs of materials T&P and carriage upto site of work. | 9741/54 | P.% Cft: | 529744/= |
| 07/- | 21753 Sft: | Providing 1st: coat of surface dressing on or new existing surface with 30 lbs: bitumen and 4.0 cft: cursh bajri of required size including cleaning the road surface rolling etc: complete.Rate includes all costs of materials T&P & carriage. | 1674/52 | P.% Sft: | 364258/= |
| 08/- | 21753 Sft: | Providing 1” thick consolidated premixed carpet in proper camber and grade i/c supply of 10 Cft: bajri 4.0 Cft: hill sand of approve quality and guage and bitumen 67 lbs: of 80/100 penetration i/c mixing the mechanical mixture in required proportion i/c heating the material and cleaning the road surface. Rate i/c all cost of material T&P and carriage upto site of work. | 4580/24 | P.% Sft: | 996339/= |
|  |  |  | TOTAL |  | 3923290/= |

_____________% Above / Below on the rates of CSR.

Amount to be added /deducted on the

Total (A) 									Rs:

Total Amount in words  ____________________________________________________

__________________________________________________________________________

Executive Engineer,

Contractor						                           Highways Division

Kashmore @ Kandhkot

*Imran*

CONSTRUCTION OF LINK ROAD FROM HAFIZULLAH KHOSO TO VILLAGE ASADULLAH KHAN KHOSO ROAD MILE 0/1+220-0/4

DRAFT BIDDING DOCUMENT FOR WORKS

BILLS OF QUANTITIES

(A) 	Description and rate of Items bases on composite schedule of Rates.

PART-B  (3.0 FT: SPAN CULVERTS).

| Item No. | Qty’s | Description of item to be executed at site | Rate | Unit | Amount in Rupees |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 |
| 01/- | 1106 Cft: | Excavation in foundation of building bridge other structure i/cs dag belling dressing refilling around the structure with excavated earth watering and remaining lead upto one chain and left upto 5.0’ ordinary soil. | 3176/25 | P.%0 Cft: | 3513/= |
| 02/- | 237 Cft: | Cement concrete with bricks are stone plaster 1½” to 2” guage ratio 1:2:4 | 9416/28 | P.% Cft: | 22317/= |
| 03/- | 502 Cft: | Pucca Brick work in foundation and plinth i/c cement sand mortor | 12501/41 | P.% Cft: | 62757/= |
| 04/- | 55 Cft: | Cement concrete plain i/c placing compacting finishing curing complete i/c screening washing of stone aggregate with out suhttering ratio 1:2:4: | 14429/25 | P.% Cft: | 7936/= |
| 05/- | 58 Sft: | Erection and removal of centering for R.C.C or plain cement work (2nd: class) for partal wood vertical | 7000/00 | P.% Sft: | 4060/= |
| 06/- | A. Tor Steel 4.16 Cwt B. M/ Steel 0.907 Cwt | Fabrication of m/s reinforcement cement concrete including cutting binding laying in position making joints & fastening of i/c binding wire also removal rust from bars. | 5001/70 4820/29 | P.Cwt: P.Cwt | 20807/= 4372/= |
| 07/- | 114 Rft: | Reinforced cement concrete work including all labour and material except the cost of steel reinforcement and its labour for bending and binding which will be paid separately. This rate also includes all kind of forms moulds lifting shuttering curing rend. | 337/00 | P.Rft: | 38418/= |
| 08/- | 330 Sft: | Cement Plaster (1:3) upto 20’ height 1½” thick. | 2283/93 | P.% Sft: | 7537/= |
|  |  |  | TOTAL |  | 171717/= |

Amount Total (b)

__________________________% Above / Below on the rates of CSR. 	Total (b) =

Amount to be added /deducted on the

basis of premium quoted Total (b)

Total (b) =  in works in figure

Executive Engineer,

Contractor						                           Highways Division

Kashmore @ Kandhkot

*Imran*

REHABILITATION / RESTORATION OF ROAD FROM KAJLI LARO TO KOT DOTHI AITBAR BEROHI ROAD MILE 0/0-1/3 ( IN PORTION)

DRAFT BIDDING DOCUMENT FOR WORKS

BILLS OF QUANTITIES

- Description and rate of Items bases on composite schedule of Rates

Part “A” (ROAD WORK)

| Item No. | Qty’s | Description of item to be executed at site |  | Rate | Unit | Amount in Rupees |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 |  | 4 | 5 | 6 |
| 01/- | 2243Cft: | Preparing base course supplying and spreading stone metal of approved quality of 6” in 2 layers from approved quarry properly grade to maximum size of 1½” in required thickness of proper camber & grade i/c S/S 15 Cft: screening & non plastic quarry fines filling & depressions with stone metal after initial rolling including watering & compacting the same so as to achieve 100% density as per modified AASHO specifications (This i/cs providing and using templates camber plates screen from as directed).Rate includes all costs of materials T&P and carriage upto site of work. |  | 10464/39 | P.% Cft: | 234716/= |
| 02/- | 35893 Sft: | Providing 1st: coat of surface dressing on or new existing surface with 30 lbs: bitumen and 4.0 cft: cursh bajri of required size including cleaning the road surface rolling etc: complete.Rate includes all costs of materials T&P & carriage. |  | 1697/48 | P.% Sft: | 609276/= |
| 03/- | 89734 Sft: | Providing 1” thick consolidated premixed carpet in proper camber and grade i/c supply of 10 Cft: bajri 4.0 Cft: hill sand of approve quality and guage and bitumen 67 lbs: of 80/100 penetration i/c mixing the mechanical mixture in required proportion i/c heating the material and cleaning the road surface. Rate i/c all cost of material T&P and carriage upto site of work. |  | 4638/69 | P.% Sft: | 4162482/= |
| 04/- | 546548Cft: | Earth Work embankment from barrow pits i/c laying in 6” layers, clod breaking remaining dressing complete, lead Upto 100’ft and lift upto 5’ft (In ordinary Soil) |  | 7327/99 | P.%0 Cft: | 4005098/= |
|  |  |  | TOTAL |  |  | 9011572/= |

_____________% Above / Below on the rates of CSR.

Amount to be added /deducted on the

Total (A) 									Rs:

Total Amount in words  ____________________________________________________

__________________________________________________________________________

Executive Engineer,

Contractor						                           Highways Division

Kashmore @ Kandhkot

*Imran*

REHABILITIION / RESTORATION OF SAIFAL  TANGWANI ROAD MILE 0/0-1/2

DRAFT BIDDING DOCUMENT FOR WORKS

BILLS OF QUANTITIES

(A) 	Description and rate of Items bases on composite schedule of Rates

Part “A” (ROAD WORK)

| Item No. | Qty’s | Description of item to be executed at site |  | Rate | Unit | Amount in Rupees |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 |  | 4 | 5 | 6 |
| 01/- | 54384Cft: | Preparing Sub base course supplying and spreading with graded pit or bed run gravel having limit not greater then 25 and plastically index not greater than 6 in proper camber and grad including watering rolling and compacting in layers thickness of each compected in layer shoulder material. T&P and carriage upto site of work. |  | 7086/54 | P.% Cft: | 3853943/= |
| 02/- | 8871Cft: | Preparing base course supplying and spreading stone metal of approved quality of 6” in 2 layers from approved quarry properly grade to maximum size of 1½” in required thickness of proper camber & grade i/c S/S 15 Cft: screening & non plastic quarry fines filling & depressions with stone metal after initial rolling including watering & compacting the same so as to achieve 100% density as per modified AASHO specifications (This i/cs providing and using templates camber plates screen from as directed).Rate includes all costs of materials T&P and carriage upto site of work. |  | 9548/80 | P.% Cft: | 847074/= |
| 03/- | 1360Rft: | Laying brick on end edging including supply of 9”x 4 ½”x3” 1st: class burnt bricks excavation for laying edging with small side parallel to the road. Rate includes all costs of materials T&P and carriage upto site of work. |  | 2792/47 | P.% Rft: | 37977/= |
| 04/- | 28755Sft: | Providing 1st: coat of surface dressing on or new existing surface with 30 lbs: bitumen and 4.0 cft: cursh bajri of required size including cleaning the road surface rolling etc: complete.Rate includes all costs of materials T&P & carriage. |  | 1670/24 | P.% Sft: | 396765/= |
| 05/- | 122364ft: | Providing 1” thick consolidated premixed carpet in proper camber and grade i/c supply of 10 Cft: bajri 4.0 Cft: hill sand of approve quality and guage and bitumen 67 lbs: of 80/100 penetration i/c mixing the mechanical mixture in required proportion i/c heating the material and cleaning the road surface. Rate i/c all cost of material T&P and carriage upto site of work. |  | 4560/84 | P.% Sft: | 5580826/= |
| 06/- | 418000Cft: | Earth Work embankment from barrow pits i/c laying in 6” layers, clod breaking remaining dressing complete, lead Upto 100’ft and lift upto 5’ft (In ordinary Soil) |  | 7327/99 | P.%0 Cft: | 3063099/= |
|  |  |  | TOTAL |  |  | 13779684/= |

_____________% Above / Below on the rates of CSR.

Amount to be added /deducted on the

Total (A) 									Rs:

Total Amount in words  ____________________________________________________

__________________________________________________________________________

Executive Engineer,

Contractor						                           Highways Division

Kashmore @ Kandhkot

*Imran*

RECONDITIONING OF KANDHKOT SAIFAL ROAD TO VILLAGE MIR ASHRAF ALI KHAN BIJRANI ROAD MILE 2/2-2/6

DRAFT BIDDING DOCUMENT FOR WORKS

BILLS OF QUANTITIES

(A) 	Description and rate of Items bases on composite schedule of Rates

Part “A” (ROAD WORK)

| Item No. | Qty’s | Description of item to be executed at site |  | Rate | Unit | Amount in Rupees |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 |  | 4 | 5 | 6 |
| 01/- | 16724Cft: | Preparing base course supplying and spreading stone metal of approved quality of 6” in 2 layers from approved quarry properly grade to maximum size of 1½” in required thickness of proper camber & grade i/c S/S 15 Cft: screening & non plastic quarry fines filling & depressions with stone metal after initial rolling including watering & compacting the same so as to achieve 100% density as per modified AASHO specifications (This i/cs providing and using templates camber plates screen from as directed).Rate includes all costs of materials T&P and carriage upto site of work. |  | 9789/71 | P.% Cft: | 1637231/= |
| 02/- | 5438Rft: | Laying brick on end edging including supply of 9”x 4 ½”x3” 1st: class burnt bricks excavation for laying edging with small side parallel to the road. Rate includes all costs of materials T&P and carriage upto site of work. |  | 2782/05 | P.% Rft: | 151287/= |
| 03/- | 32630Sft: | Providing 1st: coat of surface dressing on or new existing surface with 30 lbs: bitumen and 4.0 cft: cursh bajri of required size including cleaning the road surface rolling etc: complete.Rate includes all costs of materials T&P & carriage. |  | 1677/40 | P.% Sft: | 547335/= |
| 04/- | 32630Sft: | Providing 1” thick consolidated premixed carpet in proper camber and grade i/c supply of 10 Cft: bajri 4.0 Cft: hill sand of approve quality and guage and bitumen 67 lbs: of 80/100 penetration i/c mixing the mechanical mixture in required proportion i/c heating the material and cleaning the road surface. Rate i/c all cost of material T&P and carriage upto site of work. |  | 4575/99 | P.% Sft: | 1493145/= |
| 05/- | 186184Cft: | Earth Work embankment from barrow pits i/c laying in 6” layers, clod breaking remaining dressing complete, lead Upto 100’ft and lift upto 5’ft (In ordinary Soil) |  | 7327/99 | P.%0 Cft: | 1364354/= |
|  |  |  | TOTAL |  |  | 5193352/= |

_____________% Above / Below on the rates of CSR.

Amount to be added /deducted on the

Total (A) 									Rs:

Total Amount in words  ____________________________________________________

__________________________________________________________________________

Executive Engineer,

Contractor						                           Highways Division

Kashmore @ Kandhkot

*Imran*

RECONDITIONING OF METTLED ROAD IN VILLAGE KHAWAND BUX BAHLKANI ROAD MILE 0/0+250’-0/7

DRAFT BIDDING DOCUMENT FOR WORKS

BILLS OF QUANTITIES

(A) 	Description and rate of Items bases on composite schedule of Rates

Part “A” (ROAD WORK)

| Item No. | Qty’s | Description of item to be executed at site |  | Rate | Unit | Amount in Rupees |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 |  | 4 | 5 | 6 |
| 01/- | 15509Cft: | Preparing base course supplying and spreading stone metal of approved quality of 6” in 2 layers from approved quarry properly grade to maximum size of 1½” in required thickness of proper camber & grade i/c S/S 15 Cft: screening & non plastic quarry fines filling & depressions with stone metal after initial rolling including watering & compacting the same so as to achieve 100% density as per modified AASHO specifications (This i/cs providing and using templates camber plates screen from as directed).Rate includes all costs of materials T&P and carriage upto site of work. |  | 9307/86 | P.% Cft: | 1443556/= |
| 02/- | 4923Rft: | Laying brick on end edging including supply of 9”x 4 ½”x3” 1st: class burnt bricks excavation for laying edging with small side parallel to the road. Rate includes all costs of materials T&P and carriage upto site of work. |  | 2834/16 | P.% Rft: | 139526/= |
| 03/- | 29540Sft: | Providing 1st: coat of surface dressing on or new existing surface with 30 lbs: bitumen and 4.0 cft: cursh bajri of required size including cleaning the road surface rolling etc: complete.Rate includes all costs of materials T&P & carriage. |  | 1663/08 | P.% Sft: | 491281/= |
| 04/- | 29540Sft: | Providing 1” thick consolidated premixed carpet in proper camber and grade i/c supply of 10 Cft: bajri 4.0 Cft: hill sand of approve quality and guage and bitumen 67 lbs: of 80/100 penetration i/c mixing the mechanical mixture in required proportion i/c heating the material and cleaning the road surface. Rate i/c all cost of material T&P and carriage upto site of work. |  | 4548/33 | P.% Sft: | 1343576/= |
| 05/- | 145569Cft: | Earth Work embankment from barrow pits i/c laying in 6” layers, clod breaking remaining dressing complete, lead Upto 100’ft and lift upto 5’ft (In ordinary Soil) |  | 3838/66 | P.%0 Cft: | 518657/= |
|  |  |  | TOTAL |  |  | 3936596/= |

_____________% Above / Below on the rates of CSR.

Amount to be added /deducted on the

Total (A) 									Rs:

Total Amount in words  ____________________________________________________

__________________________________________________________________________

Executive Engineer,

Contractor						                           Highways Division

Kashmore @ Kandhkot

*Imran*

RECONDITIONING OF METTLED ROAD IN VILLAGE KHAWAND BUX BAHLKANI ROAD MILE 0/0+250’-0/7

DRAFT BIDDING DOCUMENT FOR WORKS

BILLS OF QUANTITIES

(A) 	Description and rate of Items bases on composite schedule of Rates.

Part “B”(104’ RFT: R/WALL).

| Item No. | Qty’s | Description of item to be executed at site | Rate | Unit | Amount in Rupees |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 |
| 01/- | 2080 Cft: | Excavation in foundation of building bridge other structure i/cs dag belling dressing refilling around the structure with excavated earth watering and remaining lead upto one chain and left upto 5.0’ ordinary soil. | 3176/25 | P.%0 Cft: | 6607/= |
| 02/- | 520 Cft: | Cement concrete with bricks are stone plaster 1½” to 2” guage ratio 1:2:4 | 9416/28 | P.% Cft: | 48965/= |
| 03/- | 3484 Cft: | Pucca Brick work in foundation and plinth i/c cement sand mortor | 12501/41 | P.% Cft: | 435549/= |
| 04/- | 29 Cft: | Cement concrete plain i/c placing compacting finishing curing complete i/c screening washing of stone aggregate with out suhttering ratio 1:2:4: | 14429/25 | P.% Cft: | 4184/= |
| 05/- | 52 Sft: | Erection and removal of centering for R.C.C or plain cement work (2nd: class) for partal wood vertical (S.I.No:18)(b) P-20). | 7000/00 | P.% Sft: | 3640/= |
| 06/- | 728 Sft: | Cement Pointing Struck jointing Wall ratio 1:3 | 1213/58 | P.% Sft: | 8835/= |
| 07/- | 5824 Cft: | Filling watering and remaining earth under floor with new earth excavated from side lead upto 100’ft and lift upto 5’ft. | 3630/00 | P.0% Cft: | 21141/= |
|  |  |  | TOTAL |  | 528921/= |

Amount Total (b)

__________________________% Above / Below on the rates of CSR. 	Total (b) =

Amount to be added /deducted on the

basis of premium quoted Total (b)

Total (b) =  in works in figure

Executive Engineer,

Contractor						                           Highways Division

Kashmore @ Kandhkot

*Imran*

CONSTRUCTION OF LINK ROAD FROM GHOUSPUR TANGWANI ROAD TO VILLAGE ILLAHI BUX KHAN BAJKANI ROAD MILE 0/4+88’-0/6+68’

DRAFT BIDDING DOCUMENT FOR WORKS

BILLS OF QUANTITIES

(A) 	Description and rate of Items bases on composite schedule of Rates

Part “A” (ROAD WORK)

| Item No. | Qty’s | Description of item to be executed at site | Rate | Unit | Amount in Rupees |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 |
| 01/- | 164260 Cft: | Earth work for road embankment by bulldozers i/c plugging and mixing clod breaking dressing and compacting with optimum moisture content lead upto 100’ and lift upto 5.0’ in all types of soil except rock compacting upto 85% density as per modified AASHO specifications. | 8775/85 | P.0% Cft: | 1441521/= |
| 02/- | 16315 Cft: | Earth work for road embankment by bulldozers i/c plugging and mixing clod breaking dressing and compacting with optimum moisture content lead upto 100’ and lift upto 5.0’ in all types of soil except rock compacting upto 95-100% density as per modified AASHO specifications. | 2529/77 | P.% Cft: | 41273/= |
| 03/- | 8158Cft: | Providing sand cushion i/c supplying & spreading pit/canal sand of approved quality from approved source of supply where ever available site of work i/c carriage site of work (R.A.A). | 1479/41 | P.% Cft: | 120690/= |
| 04/- | 8158Cft: | Providing Sub Base course by supplying and spreading stone metal 1 ½” gauge of approved quality from approved quarry in required thickness to proper camber and grade i/c hand packing mixing with 10’ screening & non plastic quarry of approved quality & gauge from approved source i/c watering and compacting to achieve 98-100% density as per modified AASHO specification ( rate i/cs all cost of material T&P and carriage upto site of wok.) | 8371/33 | P.% Cft: | 682933/= |
| 05/- | 2719Rft: | Laying brick on end edging including supply of 9”x 4 ½”x3” 1st: class burnt bricks excavation for laying edging with small side parallel to the road. Rate includes all costs of materials T&P and carriage upto site of work. | 2857/35 | P.% Rft: | 77691/= |

Contd: P/2

P-2

| Item No. | Qty’s | Description of item to be executed at site | Rate | Unit | Amount in Rupees |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 |
| 06/- | 4078Cft: | Preparing base course supplying and spreading stone metal of approved quality of 6” in 2 layers from approved quarry properly grade to maximum size of 1½” in required thickness of proper camber & grade i/c S/S 15 Cft: screening & non plastic quarry fines filling & depressions with stone metal after initial rolling including watering & compacting the same so as to achieve 100% density as per modified AASHO specifications (This i/cs providing and using templates camber plates screen from as directed).Rate includes all costs of materials T&P and carriage upto site of work. | 9404/24 | P.% Cft: | 383505/= |
| 07/- | 16315 Sft: | Providing 1st: coat of surface dressing on or new existing surface with 30 lbs: bitumen and 4.0 cft: cursh bajri of required size including cleaning the road surface rolling etc: complete.Rate includes all costs of materials T&P & carriage. | 1665/95 | P.% Sft: | 271799/= |
| 08/- | 16315 Sft: | Providing 1” thick consolidated premixed carpet in proper camber and grade i/c supply of 10 Cft: bajri 4.0 Cft: hill sand of approve quality and guage and bitumen 67 lbs: of 80/100 penetration i/c mixing the mechanical mixture in required proportion i/c heating the material and cleaning the road surface. Rate i/c all cost of material T&P and carriage upto site of work. | 4558/04 | P.% Sft: | 743644/= |
|  |  |  | TOTAL |  | 3763056/= |

_____________% Above / Below on the rates of CSR.

Amount to be added /deducted on the

Total (A) 									Rs:

Total Amount in words  ____________________________________________________

__________________________________________________________________________

Executive Engineer,

Contractor						                           Highways Division

Kashmore @ Kandhkot

*Imran*

CONSTRUCTION OF LINK ROAD FROM GHOUSPUR TANGWANI ROAD TO VILLAGE ILLAHI BUX KHAN BAJKANI ROAD MILE 0/4+88’-0/6+68’

DRAFT BIDDING DOCUMENT FOR WORKS

BILLS OF QUANTITIES

(A) 	Description and rate of Items bases on composite schedule of Rates.

PART-B  (3.0 FT: SPAN CULVERTS).

| Item No. | Qty’s | Description of item to be executed at site | Rate | Unit | Amount in Rupees |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 |
| 01/- | 1106 Cft: | Excavation in foundation of building bridge other structure i/cs dag belling dressing refilling around the structure with excavated earth watering and remaining lead upto one chain and left upto 5.0’ ordinary soil. | 3176/25 | P.%0 Cft: | 3513/= |
| 02/- | 237 Cft: | Cement concrete with bricks are stone plaster 1½” to 2” guage ratio 1:2:4 | 9416/28 | P.% Cft: | 22317/= |
| 03/- | 502 Cft: | Pucca Brick work in foundation and plinth i/c cement sand mortor | 12501/41 | P.% Cft: | 62757/= |
| 04/- | 55 Cft: | Cement concrete plain i/c placing compacting finishing curing complete i/c screening washing of stone aggregate with out suhttering ratio 1:2:4: | 14429/25 | P.% Cft: | 7936/= |
| 05/- | 58 Sft: | Erection and removal of centering for R.C.C or plain cement work (2nd: class) for partal wood vertical | 7000/00 | P.% Sft: | 4060/= |
| 06/- | A. Tor Steel 4.16 Cwt B. M/ Steel 0.907 Cwt | Fabrication of m/s reinforcement cement concrete including cutting binding laying in position making joints & fastening of i/c binding wire also removal rust from bars. | 5001/70 4820/29 | P.Cwt: P.Cwt | 20807/= 4372/= |
| 07/- | 114 Rft: | Reinforced cement concrete work including all labour and material except the cost of steel reinforcement and its labour for bending and binding which will be paid separately. This rate also includes all kind of forms moulds lifting shuttering curing rend. | 337/00 | P.Rft: | 38418/= |
| 08/- | 330 Sft: | Cement Plaster (1:3) upto 20’ height 1½” thick. | 2283/93 | P.% Sft: | 7537/= |
|  |  |  | TOTAL |  | 171717/= |

Amount Total (b)

__________________________% Above / Below on the rates of CSR. 	Total (b) =

Amount to be added /deducted on the

basis of premium quoted Total (b)

Total (b) =  in works in figure

Executive Engineer,

Contractor						                           Highways Division

Kashmore @ Kandhkot

*Imran*

CONST OF RETAINING WALL IN VILLAGE SAEED AHMED KHAN BAJKANI U/C GULWALI TALUKA TANGWANI

DRAFT BIDDING DOCUMENT FOR WORKS

BILLS OF QUANTITIES

(A) 	Description and rate of Items bases on composite schedule of Rates.

Part “A”(100’ RFT: R/WALL).

| Item No. | Qty’s | Description of item to be executed at site | Rate | Unit | Amount in Rupees |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 |
| 01/- | 2000 Cft: | Excavation in foundation of building bridge other structure i/cs dag belling dressing refilling around the structure with excavated earth watering and remaining lead upto one chain and left upto 5.0’ ordinary soil. | 3176/25 | P.%0 Cft: | 6353/= |
| 02/- | 500 Cft: | Cement concrete with bricks are stone plaster 1½” to 2” guage ratio 1:2:4 | 9416/28 | P.% Cft: | 47081/= |
| 03/- | 3349 Cft: | Pucca Brick work in foundation and plinth i/c cement sand mortor | 12501/41 | P.% Cft: | 418672/= |
| 04/- | 28 Cft: | Cement concrete plain i/c placing compacting finishing curing complete i/c screening washing of stone aggregate with out suhttering ratio 1:2:4: | 14429/25 | P.% Cft: | 4040/= |
| 05/- | 50 Sft: | Erection and removal of centering for R.C.C or plain cement work (2nd: class) for partal wood vertical (S.I.No:18)(b) P-20). | 7000/00 | P.% Sft: | 3500/= |
| 06/- | 700 Sft: | Cement Pointing Struck jointing Wall ratio 1:3 | 1213/58 | P.% Sft: | 8495/= |
| 07/- | 5600 Cft: | Filling watering and remaining earth under floor with new earth excavated from side lead upto 100’ft and lift upto 5’ft. | 3630/00 | P.0% Cft: | 20328/= |
|  |  |  | TOTAL |  | 508469/= |

Amount Total (a)

__________________________% Above / Below on the rates of CSR. 	Total (a) =

Amount to be added /deducted on the

basis of premium quoted Total (a)

Total (a) =  in works in figure

Executive Engineer,

Contractor						                           Highways Division

Kashmore @ Kandhkot

*Imran*

REHABILITATION / RECONDITIONING OF ROAD FROM NHA MAGHAR PHATTAK TO PATT FEEDER ROAD MILE 0/5-330’-2/1+495’

DRAFT BIDDING DOCUMENT FOR WORKS

BILLS OF QUANTITIES

(A) 	Description and rate of Items bases on composite schedule of Rates

Part “A” (ROAD WORK)

| Item No. | Qty’s | Description of item to be executed at site |  | Rate | Unit | Amount in Rupees |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 |  | 4 | 5 | 6 |
| 01/- | 52463Cft: | Preparing base course supplying and spreading stone metal of approved quality of 6” in 2 layers from approved quarry properly grade to maximum size of 1½” in required thickness of proper camber & grade i/c S/S 15 Cft: screening & non plastic quarry fines filling & depressions with stone metal after initial rolling including watering & compacting the same so as to achieve 100% density as per modified AASHO specifications (This i/cs providing and using templates camber plates screen from as directed).Rate includes all costs of materials T&P and carriage upto site of work. |  | 10368/01 | P.% Cft: | 5439369/= |
| 02/- | 16655Rft: | Laying brick on end edging including supply of 9”x 4 ½”x3” 1st: class burnt bricks excavation for laying edging with small side parallel to the road. Rate includes all costs of materials T&P and carriage upto site of work. |  | 2834/16 | P.% Rft: | 472029/= |
| 03/- | 99930Sft: | Providing 1st: coat of surface dressing on or new existing surface with 30 lbs: bitumen and 4.0 cft: cursh bajri of required size including cleaning the road surface rolling etc: complete.Rate includes all costs of materials T&P & carriage. |  | 1694/60 | P.% Sft: | 1693413/= |
| 04/- | 99930Sft: | Providing 1” thick consolidated premixed carpet in proper camber and grade i/c supply of 10 Cft: bajri 4.0 Cft: hill sand of approve quality and guage and bitumen 67 lbs: of 80/100 penetration i/c mixing the mechanical mixture in required proportion i/c heating the material and cleaning the road surface. Rate i/c all cost of material T&P and carriage upto site of work. |  | 4588/63 | P.% Sft: | 4585418/= |
| 05/- | 608000Cft: | Earth Work embankment from barrow pits i/c laying in 6” layers, clod breaking remaining dressing complete, lead Upto 100’ft and lift upto 5’ft (In ordinary Soil) |  | 2207/38 | P.%0 Cft: | 1342087/= |
|  |  |  | TOTAL |  |  | 13532316/= |

_____________% Above / Below on the rates of CSR.

Amount to be added /deducted on the

Total (A) 									Rs:

Total Amount in words  ____________________________________________________

__________________________________________________________________________

Executive Engineer,

Contractor						                           Highways Division

Kashmore @ Kandhkot

*Imran*

REHABILITATION / RECONDITIONING OF ROAD FROM NHA MAGHAR PHATTAK TO PATT FEEDER ROAD MILE 0/5-330’-2/1+495’

DRAFT BIDDING DOCUMENT FOR WORKS

BILLS OF QUANTITIES

(A) 	Description and rate of Items bases on composite schedule of Rates.

PART-B  (3.0 FT: SPAN CULVERTS).

| Item No. | Qty’s | Description of item to be executed at site | Rate | Unit | Amount in Rupees |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 |
| 01/- | 180 Cft: | Dismentaling of brick work in lime or cement mortor. | 1285/63 | P.% Cft: | 2314/= |
| 02/- | 502 Cft: | Pucca Brick work in foundation and plinth i/c cement sand mortor | 12501/41 | P.% Cft: | 62757/= |
| 03/- | 325 Cft: | Cement concrete plain i/c placing compacting finishing curing complete i/c screening washing of stone aggregate with out suhttering ratio 1:2:4: | 14429/25 | P.% Cft: | 4617/= |
| 04/- | 58 Sft: | Erection and removal of centering for R.C.C or plain cement work (2nd: class) for partal wood vertical | 7000/00 | P.% Sft: | 4060/= |
| 05/- | A. Tor Steel 4.16 Cwt B. M/ Steel 1.607 Cwt | Fabrication of m/s reinforcement cement concrete including cutting binding laying in position making joints & fastening of i/c binding wire also removal rust from bars. | 5001/70 4820/29 | P.Cwt: P.Cwt | 20807/= 4372/= |
| 06/- | 114 Rft: | Reinforced cement concrete work including all labour and material except the cost of steel reinforcement and its labour for bending and binding which will be paid separately. This rate also includes all kind of forms moulds lifting shuttering curing rend. | 337/00 | P.Rft: | 38418/= |
| 07/- | 330 Sft: | Cement Plaster (1:3) upto 20’ height 1½” thick. | 2283/93 | P.% Sft: | 7537/= |
|  |  |  | TOTAL |  | 144882/= |

Amount Total (b)

__________________________% Above / Below on the rates of CSR. 	Total (b) =

Amount to be added /deducted on the

basis of premium quoted Total (b)

Total (b) =  in works in figure

Executive Engineer,

Contractor						                           Highways Division

Kashmore @ Kandhkot

*Imran*

CONSTRUCTION OF  ROAD FROM SOKAR PULL ROAD TO VILLAGE HAMID KHAN KHOSO ROAD MILE 0/0-0/2+330’

DRAFT BIDDING DOCUMENT FOR WORKS

BILLS OF QUANTITIES

(A) 	Description and rate of Items bases on composite schedule of Rates

Part “A” (ROAD WORK)

| Item No. | Qty’s | Description of item to be executed at site | Rate | Unit | Amount in Rupees |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 |
| 01/- | 191000 Cft: | Earth work for road embankment by bulldozers i/c plugging and mixing clod breaking dressing and compacting with optimum moisture content lead upto 100’ and lift upto 5.0’ in all types of soil except rock compacting upto 85% density as per modified AASHO specifications. | 3656/23 | P.0% Cft: | 701265/= |
| 02/- | 27192 Cft: | Earth work for road embankment by bulldozers i/c plugging and mixing clod breaking dressing and compacting with optimum moisture content lead upto 100’ and lift upto 5.0’ in all types of soil except rock compacting upto 95-100% density as per modified AASHO specifications. | 2529/77 | P.% Cft: | 68789/= |
| 03/- | 13596Cft: | Providing sand cushion i/c supplying & spreading pit/canal sand of approved quality from approved source of supply where ever available site of work i/c carriage site of work (R.A.A). | 1479/41 | P.% Cft: | 201141/= |
| 04/- | 5099Cft: | Preparing sub base course by supplying & spreading stone metal 1 ½”-2” gauge of approved quarry in required 3” thickness in 1 layer each 3.0 each layers with 20 cft: pit canal sand and having plasticity index of not more than 6% watering & compacting to achieve 98/100% density as per modified AASHO specifications. Rate includes all costs of materials T&P and carriage upto site of work. | 8488/50 | P.% Cft: | 432828/= |
| 05/- | 5099Cft: | Providing Sub Base course by supplying and spreading stone metal 1 ½” gauge of approved quality from approved quarry in required thickness to proper camber and grade i/c hand packing mixing with 10’ screening & non plastic quarry of approved quality & gauge from approved source i/c watering and compacting to achieve 98-100% density as per modified AASHO specification ( rate i/cs all cost of material T&P and carriage upto site of wok.) | 8650/68 | P.% Cft: | 441098/= |
| 06/- | 3399Rft: | Laying brick on end edging including supply of 9”x 4 ½”x3” 1st: class burnt bricks excavation for laying edging with small side parallel to the road. Rate includes all costs of materials T&P and carriage upto site of work. | 2834/15 | P.% Rft: | 96332/= |

Contd: P/2

P-2

| Item No. | Qty’s | Description of item to be executed at site | Rate | Unit | Amount in Rupees |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 |
| 07/- | 5099Cft: | Preparing base course supplying and spreading stone metal of approved quality of 6” in 2 layers from approved quarry properly grade to maximum size of 1½” in required thickness of proper camber & grade i/c S/S 15 Cft: screening & non plastic quarry fines filling & depressions with stone metal after initial rolling including watering & compacting the same so as to achieve 100% density as per modified AASHO specifications (This i/cs providing and using templates camber plates screen from as directed).Rate includes all costs of materials T&P and carriage upto site of work. | 9693/36 | P.% Cft: | 494264/= |
| 08/- | 20394 Sft: | Providing 1st: coat of surface dressing on or new existing surface with 30 lbs: bitumen and 4.0 cft: cursh bajri of required size including cleaning the road surface rolling etc: complete.Rate includes all costs of materials T&P & carriage. | 1673/35 | P.% Sft: | 341263/= |
| 09/- | 20394 Sft: | Providing 1” thick consolidated premixed carpet in proper camber and grade i/c supply of 10 Cft: bajri 4.0 Cft: hill sand of approve quality and guage and bitumen 67 lbs: of 80/100 penetration i/c mixing the mechanical mixture in required proportion i/c heating the material and cleaning the road surface. Rate i/c all cost of material T&P and carriage upto site of work. | 4534/64 | P.% Sft: | 924794/= |
|  |  |  | TOTAL |  | 3701774/= |

_____________% Above / Below on the rates of CSR.

Amount to be added /deducted on the

Total (A) 									Rs:

Total Amount in words  ____________________________________________________

__________________________________________________________________________

Executive Engineer,

Contractor						                           Highways Division

Kashmore @ Kandhkot

*Imran*

CONSTRUCTION OF  ROAD FROM SOKAR PULL ROAD TO VILLAGE HAMID KHAN KHOSO ROAD MILE 0/0-0/2+330’

DRAFT BIDDING DOCUMENT FOR WORKS

BILLS OF QUANTITIES

(A) 	Description and rate of Items bases on composite schedule of Rates.

PART-B  (3.0 FT: SPAN CULVERTS).

| Item No. | Qty’s | Description of item to be executed at site | Rate | Unit | Amount in Rupees |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 |
| 01/- | 1106 Cft: | Excavation in foundation of building bridge other structure i/cs dag belling dressing refilling around the structure with excavated earth watering and remaining lead upto one chain and left upto 5.0’ ordinary soil. | 3176/25 | P.%0 Cft: | 3513/= |
| 02/- | 237 Cft: | Cement concrete with bricks are stone plaster 1½” to 2” guage ratio 1:2:4 | 9416/28 | P.% Cft: | 22316/= |
| 03/- | 502 Cft: | Pucca Brick work in foundation and plinth i/c cement sand mortor | 12501/41 | P.% Cft: | 62757/= |
| 04/- | 55 Cft: | Cement concrete plain i/c placing compacting finishing curing complete i/c screening washing of stone aggregate with out suhttering ratio 1:2:4: | 14429/25 | P.% Cft: | 7936/= |
| 05/- | 58 Sft: | Erection and removal of centering for R.C.C or plain cement work (2nd: class) for partal wood vertical | 7000/00 | P.% Sft: | 4060/= |
| 06/- | A. Tor Steel 4.16 Cwt B. M/ Steel 0.907 Cwt | Fabrication of m/s reinforcement cement concrete including cutting binding laying in position making joints & fastening of i/c binding wire also removal rust from bars. | 5001/70 4820/29 | P.Cwt: P.Cwt | 20807/= 4372/= |
| 07/- | 114 Rft: | Reinforced cement concrete work including all labour and material except the cost of steel reinforcement and its labour for bending and binding which will be paid separately. This rate also includes all kind of forms moulds lifting shuttering curing rend. | 337/00 | P.Rft: | 38418/= |
| 08/- | 330 Sft: | Cement Plaster (1:3) upto 20’ height 1½” thick. | 2283/93 | P.% Sft: | 7537/= |
|  |  |  | TOTAL |  | 171716/= |

Amount Total (b)

__________________________% Above / Below on the rates of CSR. 	Total (b) =

Amount to be added /deducted on the

basis of premium quoted Total (b)

Total (b) =  in works in figure

Executive Engineer,

Contractor						                           Highways Division

Kashmore @ Kandhkot

*Imran*

CONSTRUCTION OF  ROAD FROM SOKAR PULL ROAD TO VILLAGE HAMID KHAN KHOSO ROAD MILE 0/0-0/2+330’

DRAFT BIDDING DOCUMENT FOR WORKS

BILLS OF QUANTITIES

(A) 	Description and rate of Items bases on composite schedule of Rates.

PART-C  (MISC:).

| Item No. | Qty’s | Description of item to be executed at site | Rate | Unit | Amount in Rupees |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 |
| 01/- | 01 Nos; | Proving and Fixing informatory sign rectangular shape 4’ x 3” comprising 02 Nos; 3” dia G.I Pipe Post i/c painting writing as directed each. | 16965/00 | P.Each | 16,965/= |
|  |  |  | TOTAL |  | 16,965/= |

Amount Total (C)

__________________________% Above / Below on the rates of CSR. 	Total (C) =

Amount to be added /deducted on the

basis of premium quoted Total (C)

Total (c) =  in works in figure

Executive Engineer,

Contractor						                           Highways Division

Kashmore @ Kandhkot

*Imran*

RECONDITIONING OF ROAD FROM VILLAGE ABDUL GHAFFAR CHACHAR ROAD VIA RAIS NAWAB CHACHAR TALUKA KASHMORE

DRAFT BIDDING DOCUMENT FOR WORKS

BILLS OF QUANTITIES

(A) 	Description and rate of Items bases on composite schedule of Rates

Part “A” (ROAD WORK)

| Item No. | Qty’s | Description of item to be executed at site |  | Rate | Unit | Amount in Rupees |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 |  | 4 | 5 | 6 |
| 01/- | 2843Cft: | Preparing base course supplying and spreading stone metal of approved quality of 6” in 2 layers from approved quarry properly grade to maximum size of 1½” in required thickness of proper camber & grade i/c S/S 15 Cft: screening & non plastic quarry fines filling & depressions with stone metal after initial rolling including watering & compacting the same so as to achieve 100% density as per modified AASHO specifications (This i/cs providing and using templates camber plates screen from as directed).Rate includes all costs of materials T&P and carriage upto site of work. |  | 10416/20 | P.% Cft: | 296132/= |
| 02/- | 1648Rft: | Laying brick on end edging including supply of 9”x 4 ½”x3” 1st: class burnt bricks excavation for laying edging with small side parallel to the road. Rate includes all costs of materials T&P and carriage upto site of work. |  | 2836/11 | P.% Rft: | 46739/= |
| 03/- | 9888Sft: | Providing 1st: coat of surface dressing on or new existing surface with 30 lbs: bitumen and 4.0 cft: cursh bajri of required size including cleaning the road surface rolling etc: complete.Rate includes all costs of materials T&P & carriage. |  | 1696/04 | P.% Sft: | 167704/= |
| 04/- | 9888Sft: | Providing 1” thick consolidated premixed carpet in proper camber and grade i/c supply of 10 Cft: bajri 4.0 Cft: hill sand of approve quality and guage and bitumen 67 lbs: of 80/100 penetration i/c mixing the mechanical mixture in required proportion i/c heating the material and cleaning the road surface. Rate i/c all cost of material T&P and carriage upto site of work. |  | 4597/36 | P.% Sft: | 456586/= |
| 05/- | -- | Earth Work embankment from barrow pits i/c laying in 6” layers, clod breaking remaining dressing complete, lead Upto 100’ft and lift upto 5’ft (In ordinary Soil) |  | -- | -- | -- |
|  |  |  | TOTAL |  |  | 967161/= |

_____________% Above / Below on the rates of CSR.

Amount to be added /deducted on the

Total (A) 									Rs:

Total Amount in words  ____________________________________________________

__________________________________________________________________________

Executive Engineer,

Contractor						                           Highways Division

Kashmore @ Kandhkot

*Imran*

CONSTRUCTION OF LINK ROAD IN VILLAGE MEHBOOB ALI KHAN BANGWAR U/C R.B.CHACHAR TALUKA KASHMORE MILE 0/0-0/0+400’

DRAFT BIDDING DOCUMENT FOR WORKS

BILLS OF QUANTITIES

- Description and rate of Items bases on composite schedule of Rates

Part “A” (ROAD WORK)

| Item No. | Qty’s | Description of item to be executed at site | Rate | Unit | Amount in Rupees |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 |
| 01/- | 63000 Cft: | Earth work for road embankment by bulldozers i/c plugging and mixing clod breaking dressing and compacting with optimum moisture content lead upto 100’ and lift upto 5.0’ in all types of soil except rock compacting upto 85% density as per modified AASHO specifications. | 3656/23 | P.0% Cft: | 230342/= |
| 02/- | 4944 Cft: | Earth work for road embankment by bulldozers i/c plugging and mixing clod breaking dressing and compacting with optimum moisture content lead upto 100’ and lift upto 5.0’ in all types of soil except rock compacting upto 95-100% density as per modified AASHO specifications. | 2529/77 | P.% Cft: | 12507/= |
| 03/- | 2472Cft: | Providing sand cushion i/c supplying & spreading pit/canal sand of approved quality from approved source of supply where ever available site of work i/c carriage site of work (R.A.A). | 1479/41 | P.% Cft: | 36571/= |
| 04/- | 2472Cft: | Providing Sub Base course by supplying and spreading stone metal 1 ½” gauge of approved quality from approved quarry in required thickness to proper camber and grade i/c hand packing mixing with 10’ screening & non plastic quarry of approved quality & gauge from approved source i/c watering and compacting to achieve 98-100% density as per modified AASHO specification ( rate i/cs all cost of material T&P and carriage upto site of wok.) | 9488/78 | P.% Cft: | 234563/= |
| 05/- | 824Rft: | Laying brick on end edging including supply of 9”x 4 ½”x3” 1st: class burnt bricks excavation for laying edging with small side parallel to the road. Rate includes all costs of materials T&P and carriage upto site of work. | 2867/96 | P.% Rft: | 23632/= |

Contd: P/2

P-2

| Item No. | Qty’s | Description of item to be executed at site | Rate | Unit | Amount in Rupees |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 |
| 06/- | 1236Cft: | Preparing base course supplying and spreading stone metal of approved quality of 6” in 2 layers from approved quarry properly grade to maximum size of 1½” in required thickness of proper camber & grade i/c S/S 15 Cft: screening & non plastic quarry fines filling & depressions with stone metal after initial rolling including watering & compacting the same so as to achieve 100% density as per modified AASHO specifications (This i/cs providing and using templates camber plates screen from as directed).Rate includes all costs of materials T&P and carriage upto site of work. | 10380/76 | P.% Cft: | 128306/= |
| 07/- | 4944 Sft: | Providing 1st: coat of surface dressing on or new existing surface with 30 lbs: bitumen and 4.0 cft: cursh bajri of required size including cleaning the road surface rolling etc: complete.Rate includes all costs of materials T&P & carriage. | 1700/33 | P.% Sft: | 84064/= |
| 08/- | 4944 Sft: | Providing 1” thick consolidated premixed carpet in proper camber and grade i/c supply of 10 Cft: bajri 4.0 Cft: hill sand of approve quality and guage and bitumen 67 lbs: of 80/100 penetration i/c mixing the mechanical mixture in required proportion i/c heating the material and cleaning the road surface. Rate i/c all cost of material T&P and carriage upto site of work. | 4612/22 | P.% Sft: | 228028/= |
|  |  |  | TOTAL |  | 978013/= |

_____________% Above / Below on the rates of CSR.

Amount to be added /deducted on the

Total (A) 									Rs:

Total Amount in words  ____________________________________________________

__________________________________________________________________________

Executive Engineer,

Contractor						                           Highways Division

Kashmore @ Kandhkot

*Imran*

CONSTRUCTION OF LINK ROAD AT VILLAGE SANAULLAH DASTI U/C-02 ROAD MILE 0/0-0/0+630’ TALUKA KASHMORE

DRAFT BIDDING DOCUMENT FOR WORKS

BILLS OF QUANTITIES

- Description and rate of Items bases on composite schedule of Rates

Part “A” (ROAD WORK)

| Item No. | Qty’s | Description of item to be executed at site | Rate | Unit | Amount in Rupees |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 |
| 01/- | 76000 Cft: | Earth work for road embankment by bulldozers i/c plugging and mixing clod breaking dressing and compacting with optimum moisture content lead upto 100’ and lift upto 5.0’ in all types of soil except rock compacting upto 85% density as per modified AASHO specifications. | 3656/23 | P.0% Cft: | 277873/= |
| 02/- | 7787 Cft: | Earth work for road embankment by bulldozers i/c plugging and mixing clod breaking dressing and compacting with optimum moisture content lead upto 100’ and lift upto 5.0’ in all types of soil except rock compacting upto 95-100% density as per modified AASHO specifications. | 2529/77 | P.% Cft: | 19699/= |
| 03/- | 3893Cft: | Providing sand cushion i/c supplying & spreading pit/canal sand of approved quality from approved source of supply where ever available site of work i/c carriage site of work (R.A.A). | 1479/41 | P.% Cft: | 57593/= |
| 04/- | 3893Cft: | Providing Sub Base course by supplying and spreading stone metal 1 ½” gauge of approved quality from approved quarry in required thickness to proper camber and grade i/c hand packing mixing with 10’ screening & non plastic quarry of approved quality & gauge from approved source i/c watering and compacting to achieve 98-100% density as per modified AASHO specification ( rate i/cs all cost of material T&P and carriage upto site of wok.) | 9907/83 | P.% Cft: | 385711/= |
| 05/- | 1298Rft: | Laying brick on end edging including supply of 9”x 4 ½”x3” 1st: class burnt bricks excavation for laying edging with small side parallel to the road. Rate includes all costs of materials T&P and carriage upto site of work. | 2959/19 | P.% Rft: | 38410/= |

Contd: P/2

P-2

| Item No. | Qty’s | Description of item to be executed at site | Rate | Unit | Amount in Rupees |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 |
| 06/- | 1947Cft: | Preparing base course supplying and spreading stone metal of approved quality of 6” in 2 layers from approved quarry properly grade to maximum size of 1½” in required thickness of proper camber & grade i/c S/S 15 Cft: screening & non plastic quarry fines filling & depressions with stone metal after initial rolling including watering & compacting the same so as to achieve 100% density as per modified AASHO specifications (This i/cs providing and using templates camber plates screen from as directed).Rate includes all costs of materials T&P and carriage upto site of work. | 10994/47 | P.% Cft: | 214062/= |
| 07/- | 7787 Sft: | Providing 1st: coat of surface dressing on or new existing surface with 30 lbs: bitumen and 4.0 cft: cursh bajri of required size including cleaning the road surface rolling etc: complete.Rate includes all costs of materials T&P & carriage. | 1713/23 | P.% Sft: | 133409/= |
| 08/- | 7787 Sft: | Providing 1” thick consolidated premixed carpet in proper camber and grade i/c supply of 10 Cft: bajri 4.0 Cft: hill sand of approve quality and guage and bitumen 67 lbs: of 80/100 penetration i/c mixing the mechanical mixture in required proportion i/c heating the material and cleaning the road surface. Rate i/c all cost of material T&P and carriage upto site of work. | 4615/19 | P.% Sft: | 359384/= |
|  |  |  | TOTAL |  | 1481868/= |

_____________% Above / Below on the rates of CSR.

Amount to be added /deducted on the

Total (A) 									Rs:

Total Amount in words  ____________________________________________________

__________________________________________________________________________

Executive Engineer,

Contractor						                           Highways Division

Kashmore @ Kandhkot

*Imran*

CONSTRUCTION OF LINK ROAD FROM RASOOL BUX CHACHAR TO VILLAGE GHULAM AKBER CHACHAR TALUKA KASHMORE

DRAFT BIDDING DOCUMENT FOR WORKS

BILLS OF QUANTITIES

- Description and rate of Items bases on composite schedule of Rates

Part “A” (ROAD WORK)

| Item No. | Qty’s | Description of item to be executed at site | Rate | Unit | Amount in Rupees |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 |
| 01/- | 76000 Cft: | Earth work for road embankment by bulldozers i/c plugging and mixing clod breaking dressing and compacting with optimum moisture content lead upto 100’ and lift upto 5.0’ in all types of soil except rock compacting upto 85% density as per modified AASHO specifications. | 3656/23 | P.0% Cft: | 277873/= |
| 02/- | 6798 Cft: | Earth work for road embankment by bulldozers i/c plugging and mixing clod breaking dressing and compacting with optimum moisture content lead upto 100’ and lift upto 5.0’ in all types of soil except rock compacting upto 95-100% density as per modified AASHO specifications. | 2529/77 | P.% Cft: | 171974/= |
| 03/- | 3399Cft: | Providing sand cushion i/c supplying & spreading pit/canal sand of approved quality from approved source of supply where ever available site of work i/c carriage site of work (R.A.A). | 1479/41 | P.% Cft: | 50285/= |
| 04/- | 3399Cft: | Providing Sub Base course by supplying and spreading stone metal 1 ½” gauge of approved quality from approved quarry in required thickness to proper camber and grade i/c hand packing mixing with 10’ screening & non plastic quarry of approved quality & gauge from approved source i/c watering and compacting to achieve 98-100% density as per modified AASHO specification ( rate i/cs all cost of material T&P and carriage upto site of wok.) | 9255/98 | P.% Cft: | 314611/= |
| 05/- | 1133Rft: | Laying brick on end edging including supply of 9”x 4 ½”x3” 1st: class burnt bricks excavation for laying edging with small side parallel to the road. Rate includes all costs of materials T&P and carriage upto site of work. | 2814/80 | P.% Rft: | 31891/= |

Contd: P/2

P-2

| Item No. | Qty’s | Description of item to be executed at site | Rate | Unit | Amount in Rupees |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 |
| 06/- | 1700Cft: | Preparing base course supplying and spreading stone metal of approved quality of 6” in 2 layers from approved quarry properly grade to maximum size of 1½” in required thickness of proper camber & grade i/c S/S 15 Cft: screening & non plastic quarry fines filling & depressions with stone metal after initial rolling including watering & compacting the same so as to achieve 100% density as per modified AASHO specifications (This i/cs providing and using templates camber plates screen from as directed).Rate includes all costs of materials T&P and carriage upto site of work. | 10319/83 | P.% Cft: | 175437/= |
| 07/- | 6798 Sft: | Providing 1st: coat of surface dressing on or new existing surface with 30 lbs: bitumen and 4.0 cft: cursh bajri of required size including cleaning the road surface rolling etc: complete.Rate includes all costs of materials T&P & carriage. | 1693/04 | P.% Sft: | 115093/= |
| 08/- | 6798 Sft: | Providing 1” thick consolidated premixed carpet in proper camber and grade i/c supply of 10 Cft: bajri 4.0 Cft: hill sand of approve quality and guage and bitumen 67 lbs: of 80/100 penetration i/c mixing the mechanical mixture in required proportion i/c heating the material and cleaning the road surface. Rate i/c all cost of material T&P and carriage upto site of work. | 4589/97 | P.% Sft: | 312026/= |
|  |  |  | TOTAL |  | 1449190/= |

_____________% Above / Below on the rates of CSR.

Amount to be added /deducted on the

Total (A) 									Rs:

Total Amount in words  ____________________________________________________

__________________________________________________________________________

Executive Engineer,

Contractor						                           Highways Division

Kashmore @ Kandhkot

*Imran*

CONSTRUCTION OF LINK ROAD FROM HIDAYATULLAH BANGWAR TO VILLAGE QURBAN BANGWAR U/C ZOREGARAH  TALUKA KASHMORE

DRAFT BIDDING DOCUMENT FOR WORKS

BILLS OF QUANTITIES

- Description and rate of Items bases on composite schedule of Rates

Part “A” (ROAD WORK)

| Item No. | Qty’s | Description of item to be executed at site | Rate | Unit | Amount in Rupees |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 |
| 01/- | 73697 Cft: | Earth work for road embankment by bulldozers i/c plugging and mixing clod breaking dressing and compacting with optimum moisture content lead upto 100’ and lift upto 5.0’ in all types of soil except rock compacting upto 85% density as per modified AASHO specifications. | 3656/23 | P.0% Cft: | 269453/= |
| 02/- | 7910 Cft: | Earth work for road embankment by bulldozers i/c plugging and mixing clod breaking dressing and compacting with optimum moisture content lead upto 100’ and lift upto 5.0’ in all types of soil except rock compacting upto 95-100% density as per modified AASHO specifications. | 2529/77 | P.% Cft: | 20011/= |
| 03/- | 3955Cft: | Providing sand cushion i/c supplying & spreading pit/canal sand of approved quality from approved source of supply where ever available site of work i/c carriage site of work (R.A.A). | 1479/41 | P.% Cft: | 58510/= |
| 04/- | 3955Cft: | Providing Sub Base course by supplying and spreading stone metal 1 ½” gauge of approved quality from approved quarry in required thickness to proper camber and grade i/c hand packing mixing with 10’ screening & non plastic quarry of approved quality & gauge from approved source i/c watering and compacting to achieve 98-100% density as per modified AASHO specification ( rate i/cs all cost of material T&P and carriage upto site of wok.) | 9209/42 | P.% Cft: | 364232/= |
| 05/- | 1318Rft: | Laying brick on end edging including supply of 9”x 4 ½”x3” 1st: class burnt bricks excavation for laying edging with small side parallel to the road. Rate includes all costs of materials T&P and carriage upto site of work. | 2743/37 | P.% Rft: | 36157/= |

Contd: P/2

P-2

| Item No. | Qty’s | Description of item to be executed at site | Rate | Unit | Amount in Rupees |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 |
| 06/- | 1978Cft: | Preparing base course supplying and spreading stone metal of approved quality of 6” in 2 layers from approved quarry properly grade to maximum size of 1½” in required thickness of proper camber & grade i/c S/S 15 Cft: screening & non plastic quarry fines filling & depressions with stone metal after initial rolling including watering & compacting the same so as to achieve 100% density as per modified AASHO specifications (This i/cs providing and using templates camber plates screen from as directed).Rate includes all costs of materials T&P and carriage upto site of work. | 10271/64 | P.% Cft: | 203173/= |
| 07/- | 7910 Sft: | Providing 1st: coat of surface dressing on or new existing surface with 30 lbs: bitumen and 4.0 cft: cursh bajri of required size including cleaning the road surface rolling etc: complete.Rate includes all costs of materials T&P & carriage. | 1691/75 | P.% Sft: | 133817/= |
| 08/- | 7910 Sft: | Providing 1” thick consolidated premixed carpet in proper camber and grade i/c supply of 10 Cft: bajri 4.0 Cft: hill sand of approve quality and guage and bitumen 67 lbs: of 80/100 penetration i/c mixing the mechanical mixture in required proportion i/c heating the material and cleaning the road surface. Rate i/c all cost of material T&P and carriage upto site of work. | 4586/75 | P.% Sft: | 362811/= |
|  |  |  | TOTAL |  | 1448164/= |

_____________% Above / Below on the rates of CSR.

Amount to be added /deducted on the

Total (A) 									Rs:

Total Amount in words  ____________________________________________________

__________________________________________________________________________

Executive Engineer,

Contractor						                           Highways Division

Kashmore @ Kandhkot

*Imran*

CONSTRUCTION OF LINK ROAD FROM GUBLO ROAD UPTO VILLAGE BAHAWAL DIN BANGWAR TALUKA KASHMORE MILE 0/0-0/2  TALUKA KASHMORE

DRAFT BIDDING DOCUMENT FOR WORKS

BILLS OF QUANTITIES

- Description and rate of Items bases on composite schedule of Rates

Part “A” (ROAD WORK)

| Item No. | Qty’s | Description of item to be executed at site | Rate | Unit | Amount in Rupees |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 |
| 01/- | 148000Cft: | Earth work for road embankment by bulldozers i/c plugging and mixing clod breaking dressing and compacting with optimum moisture content lead upto 100’ and lift upto 5.0’ in all types of soil except rock compacting upto 85% density as per modified AASHO specifications. | 3656/23 | P.0% Cft: | 541122/= |
| 02/- | 16315Cft: | Earth work for road embankment by bulldozers i/c plugging and mixing clod breaking dressing and compacting with optimum moisture content lead upto 100’ and lift upto 5.0’ in all types of soil except rock compacting upto 95-100% density as per modified AASHO specifications. | 2529/77 | P.% Cft: | 41273/= |
| 03/- | 8158Cft: | Providing sand cushion i/c supplying & spreading pit/canal sand of approved quality from approved source of supply where ever available site of work i/c carriage site of work (R.A.A). | 1479/41 | P.% Cft: | 120690/= |
| 04/- | 8158Cft: | Providing Sub Base course by supplying and spreading stone metal 1 ½” gauge of approved quality from approved quarry in required thickness to proper camber and grade i/c hand packing mixing with 10’ screening & non plastic quarry of approved quality & gauge from approved source i/c watering and compacting to achieve 98-100% density as per modified AASHO specification ( rate i/cs all cost of material T&P and carriage upto site of wok.) | 9069/74 | P.% Cft: | 739909/= |
| 05/- | 2719Rft: | Laying brick on end edging including supply of 9”x 4 ½”x3” 1st: class burnt bricks excavation for laying edging with small side parallel to the road. Rate includes all costs of materials T&P and carriage upto site of work. | 2771/73 | P.% Rft: | 75351/= |

Contd: P/2

P-2

| Item No. | Qty’s | Description of item to be executed at site | Rate | Unit | Amount in Rupees |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 |
| 06/- | 4079Cft: | Preparing base course supplying and spreading stone metal of approved quality of 6” in 2 layers from approved quarry properly grade to maximum size of 1½” in required thickness of proper camber & grade i/c S/S 15 Cft: screening & non plastic quarry fines filling & depressions with stone metal after initial rolling including watering & compacting the same so as to achieve 100% density as per modified AASHO specifications (This i/cs providing and using templates camber plates screen from as directed).Rate includes all costs of materials T&P and carriage upto site of work. | 10127/07 | P.% Cft: | 413083/= |
| 07/- | 16315Sft: | Providing 1st: coat of surface dressing on or new existing surface with 30 lbs: bitumen and 4.0 cft: cursh bajri of required size including cleaning the road surface rolling etc: complete.Rate includes all costs of materials T&P & carriage. | 1687/43 | P.% Sft: | 275304/= |
| 08/- | 16315Sft: | Providing 1” thick consolidated premixed carpet in proper camber and grade i/c supply of 10 Cft: bajri 4.0 Cft: hill sand of approve quality and guage and bitumen 67 lbs: of 80/100 penetration i/c mixing the mechanical mixture in required proportion i/c heating the material and cleaning the road surface. Rate i/c all cost of material T&P and carriage upto site of work. | 4577/36 | P.% Sft: | 746796/= |
|  |  |  | TOTAL |  | 2953529/= |

_____________% Above / Below on the rates of CSR.

Amount to be added /deducted on the

Total (A) 									Rs:

Total Amount in words  ____________________________________________________

__________________________________________________________________________

Executive Engineer,

Contractor						                           Highways Division

Kashmore @ Kandhkot

*Imran*

CONSTRUCTION OF LINK ROAD FROM KANDHKOT MALGUZAR TO VILLAGE ARBELO KHAN JAKHRANI ROAD MILE 0/0-0/1+330’

DRAFT BIDDING DOCUMENT FOR WORKS

BILLS OF QUANTITIES

- Description and rate of Items bases on composite schedule of Rates

Part “A” (ROAD WORK)

| Item No. | Qty’s | Description of item to be executed at site | Rate | Unit | Amount in Rupees |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 |
| 01/- | 114000Cft: | Earth work for road embankment by bulldozers i/c plugging and mixing clod breaking dressing and compacting with optimum moisture content lead upto 100’ and lift upto 5.0’ in all types of soil except rock compacting upto 85% density as per modified AASHO specifications. | 3656/23 | P.0% Cft: | 416810/= |
| 02/- | 12236Cft: | Earth work for road embankment by bulldozers i/c plugging and mixing clod breaking dressing and compacting with optimum moisture content lead upto 100’ and lift upto 5.0’ in all types of soil except rock compacting upto 95-100% density as per modified AASHO specifications. | 2529/77 | P.% Cft: | 30955/= |
| 03/- | 6118Cft: | Providing sand cushion i/c supplying & spreading pit/canal sand of approved quality from approved source of supply where ever available site of work i/c carriage site of work (R.A.A). | 1479/41 | P.% Cft: | 90510/= |
| 04/- | 6118Cft: | Providing Sub Base course by supplying and spreading stone metal 1 ½” gauge of approved quality from approved quarry in required thickness to proper camber and grade i/c hand packing mixing with 10’ screening & non plastic quarry of approved quality & gauge from approved source i/c watering and compacting to achieve 98-100% density as per modified AASHO specification ( rate i/cs all cost of material T&P and carriage upto site of wok.) | 9162/76 | P.% Cft: | 560878/= |
| 05/- | 2039Rft: | Laying brick on end edging including supply of 9”x 4 ½”x3” 1st: class burnt bricks excavation for laying edging with small side parallel to the road. Rate includes all costs of materials T&P and carriage upto site of work. | 2814/88 | P.% Rft: | 57395/= |

Contd: P/2

P-2

| Item No. | Qty’s | Description of item to be executed at site | Rate | Unit | Amount in Rupees |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 |
| 06/- | 3059Cft: | Preparing base course supplying and spreading stone metal of approved quality of 6” in 2 layers from approved quarry properly grade to maximum size of 1½” in required thickness of proper camber & grade i/c S/S 15 Cft: screening & non plastic quarry fines filling & depressions with stone metal after initial rolling including watering & compacting the same so as to achieve 100% density as per modified AASHO specifications (This i/cs providing and using templates camber plates screen from as directed).Rate includes all costs of materials T&P and carriage upto site of work. | 10223/41 | P.% Cft: | 312734/= |
| 07/- | 12236Sft: | Providing 1st: coat of surface dressing on or new existing surface with 30 lbs: bitumen and 4.0 cft: cursh bajri of required size including cleaning the road surface rolling etc: complete.Rate includes all costs of materials T&P & carriage. | 1690/31 | P.% Sft: | 206826/= |
| 08/- | 12236Sft: | Providing 1” thick consolidated premixed carpet in proper camber and grade i/c supply of 10 Cft: bajri 4.0 Cft: hill sand of approve quality and guage and bitumen 67 lbs: of 80/100 penetration i/c mixing the mechanical mixture in required proportion i/c heating the material and cleaning the road surface. Rate i/c all cost of material T&P and carriage upto site of work. | 4612/63 | P.% Sft: | 564401/= |
|  |  |  | TOTAL |  | 2240209/= |

_____________% Above / Below on the rates of CSR.

Amount to be added /deducted on the

Total (A) 									Rs:

Total Amount in words  ____________________________________________________

__________________________________________________________________________

Executive Engineer,

Contractor						                           Highways Division

Kashmore @ Kandhkot

*Imran*

CONSTRUCTION OF LINK ROAD FROM KANDHKOT MALGUZAR TO VILLAGE ARBELO KHAN JAKHRANI ROAD MILE 0/0-0/1+330’

DRAFT BIDDING DOCUMENT FOR WORKS

BILLS OF QUANTITIES

(A) 	Description and rate of Items bases on composite schedule of Rates.

PART-B  (3.0 FT: SPAN CULVERTS).

| Item No. | Qty’s | Description of item to be executed at site | Rate | Unit | Amount in Rupees |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 |
| 01/- | 1106 Cft: | Excavation in foundation of building bridge other structure i/cs dag belling dressing refilling around the structure with excavated earth watering and remaining lead upto one chain and left upto 5.0’ ordinary soil. | 3176/25 | P.%0 Cft: | 3513/= |
| 02/- | 238 Cft: | Cement concrete with bricks are stone plaster 1½” to 2” guage ratio 1:2:4 | 9416/28 | P.% Cft: | 22411/= |
| 03/- | 502 Cft: | Pucca Brick work in foundation and plinth i/c cement sand mortor | 12501/41 | P.% Cft: | 62755/= |
| 04/- | 55 Cft: | Cement concrete plain i/c placing compacting finishing curing complete i/c screening washing of stone aggregate with out suhttering ratio 1:2:4: | 14429/25 | P.% Cft: | 7936/= |
| 05/- | 58 Sft: | Erection and removal of centering for R.C.C or plain cement work (2nd: class) for partal wood vertical | 7000/00 | P.% Sft: | 8120/= |
| 06/- | A. Tor Steel 4.16 Cwt B. M/ Steel 0.907 Cwt | Fabrication of m/s reinforcement cement concrete including cutting binding laying in position making joints & fastening of i/c binding wire also removal rust from bars. | 5001/70 4820/29 | P.Cwt: P.Cwt | 20807/= 4371/= |
| 07/- | 114 Rft: | Reinforced cement concrete work including all labour and material except the cost of steel reinforcement and its labour for bending and binding which will be paid separately. This rate also includes all kind of forms moulds lifting shuttering curing rend. | 337/00 | P.Rft: | 38418/= |
| 08/- | 330 Sft: | Cement Plaster (1:3) upto 20’ height 1½” thick. | 2283/93 | P.% Sft: | 7537/= |
|  |  |  | TOTAL |  | 175862/= |

Amount Total (b)

__________________________% Above / Below on the rates of CSR. 	Total (b) =

Amount to be added /deducted on the

basis of premium quoted Total (b)

Total (b) =  in works in figure

Executive Engineer,

Contractor						                           Highways Division

Kashmore @ Kandhkot

*Imran*

CONSTRUCTION OF LINK ROAD FROM KANDHKOT MALGUZAR TO VILLAGE ARBELO KHAN JAKHRANI ROAD MILE 0/0-0/1+330’

DRAFT BIDDING DOCUMENT FOR WORKS

BILLS OF QUANTITIES

(A) 	Description and rate of Items bases on composite schedule of Rates.

PART-C  (MISC:).

| Item No. | Qty’s | Description of item to be executed at site | Rate | Unit | Amount in Rupees |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 |
| 01/- | 01 Nos; | Proving and Fixing informatory sign rectangular shape 4’ x 3” comprising 02 Nos; 3” dia G.I Pipe Post i/c painting writing as directed each. | 10800/00 | P.Each | 10,800/= |
|  |  |  | TOTAL |  | 10,800/= |

Amount Total (C)

__________________________% Above / Below on the rates of CSR. 	Total (C) =

Amount to be added /deducted on the

basis of premium quoted Total (C)

Total (c) =  in works in figure

Executive Engineer,

Contractor						                           Highways Division

Kashmore @ Kandhkot

*Imran*

CONSTRUCTION OF INTERNAL ROAD IN VILLAGE KAMIL CHACHAR TALUKA KASHMORE  ROAD MILE 0/0-0/1+330’

DRAFT BIDDING DOCUMENT FOR WORKS

BILLS OF QUANTITIES

(A)		Description and rate of Items bases on composite schedule of Rates

Part “A” (ROAD WORK)

| Item No. | Qty’s | Description of item to be executed at site | Rate | Unit | Amount in Rupees |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 |
| 01/- | -- | Earth work for road embankment by bulldozers i/c plugging and mixing clod breaking dressing and compacting with optimum moisture content lead upto 100’ and lift upto 5.0’ in all types of soil except rock compacting upto 85% density as per modified AASHO specifications. | -- | -- | -- |
| 02/- | -- | Earth work for road embankment by bulldozers i/c plugging and mixing clod breaking dressing and compacting with optimum moisture content lead upto 100’ and lift upto 5.0’ in all types of soil except rock compacting upto 95-100% density as per modified AASHO specifications. | -- | -- | -- |
| 03/- | 12273Cft: | Providing sand cushion i/c supplying & spreading pit/canal sand of approved quality from approved source of supply where ever available site of work i/c carriage site of work (R.A.A). | 1479/41 | P.% Cft: | 181568/= |
| 04/- | 6118Cft: | Providing Sub Base course by supplying and spreading stone metal 1 ½” gauge of approved quality from approved quarry in required thickness to proper camber and grade i/c hand packing mixing with 10’ screening & non plastic quarry of approved quality & gauge from approved source i/c watering and compacting to achieve 98-100% density as per modified AASHO specification ( rate i/cs all cost of material T&P and carriage upto site of wok.) | 9814/71 | P.% Cft: | 602329/= |
| 05/- | 2046Rft: | Laying brick on end edging including supply of 9”x 4 ½”x3” 1st: class burnt bricks excavation for laying edging with small side parallel to the road. Rate includes all costs of materials T&P and carriage upto site of work. | 3251/16 | P.% Rft: | 66519/= |

Contd: P/2

P-2

| Item No. | Qty’s | Description of item to be executed at site | Rate | Unit | Amount in Rupees |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 |
| 06/- | 3068Cft: | Preparing base course supplying and spreading stone metal of approved quality of 6” in 2 layers from approved quarry properly grade to maximum size of 1½” in required thickness of proper camber & grade i/c S/S 15 Cft: screening & non plastic quarry fines filling & depressions with stone metal after initial rolling including watering & compacting the same so as to achieve 100% density as per modified AASHO specifications (This i/cs providing and using templates camber plates screen from as directed).Rate includes all costs of materials T&P and carriage upto site of work. | 10898/09 | P.% Cft: | 334353/= |
| 07/- | 12273Sft: | Providing 1st: coat of surface dressing on or new existing surface with 30 lbs: bitumen and 4.0 cft: cursh bajri of required size including cleaning the road surface rolling etc: complete.Rate includes all costs of materials T&P & carriage. | 1710/37 | P.% Sft: | 209914/= |
| 08/- | 12273Sft: | Providing 1” thick consolidated premixed carpet in proper camber and grade i/c supply of 10 Cft: bajri 4.0 Cft: hill sand of approve quality and guage and bitumen 67 lbs: of 80/100 penetration i/c mixing the mechanical mixture in required proportion i/c heating the material and cleaning the road surface. Rate i/c all cost of material T&P and carriage upto site of work. | 4610/72 | P.% Sft: | 565874/= |
|  |  |  | TOTAL |  | 1960557/= |

_____________% Above / Below on the rates of CSR.

Amount to be added /deducted on the

Total (A) 									Rs:

Total Amount in words  ____________________________________________________

__________________________________________________________________________

Executive Engineer,

Contractor						                           Highways Division

Kashmore @ Kandhkot

*Imran*

CONSTRUCTION OF INTERNAL ROAD IN VILLAGE DIN MUHAMMAD NAICH TALUKA KASHMORE

DRAFT BIDDING DOCUMENT FOR WORKS

BILLS OF QUANTITIES

(A)		Description and rate of Items bases on composite schedule of Rates

Part “A” (ROAD WORK)

| Item No. | Qty’s | Description of item to be executed at site | Rate | Unit | Amount in Rupees |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 |
| 01/- | -- | Earth work for road embankment by bulldozers i/c plugging and mixing clod breaking dressing and compacting with optimum moisture content lead upto 100’ and lift upto 5.0’ in all types of soil except rock compacting upto 85% density as per modified AASHO specifications. | -- | -- | -- |
| 02/- | -- | Earth work for road embankment by bulldozers i/c plugging and mixing clod breaking dressing and compacting with optimum moisture content lead upto 100’ and lift upto 5.0’ in all types of soil except rock compacting upto 95-100% density as per modified AASHO specifications. | -- | -- | -- |
| 03/- | 6489Cft: | Providing sand cushion i/c supplying & spreading pit/canal sand of approved quality from approved source of supply where ever available site of work i/c carriage site of work (R.A.A). | 1479/41 | P.% Cft: | 95999/= |
| 04/- | 6489Cft: | Providing Sub Base course by supplying and spreading stone metal 1 ½” gauge of approved quality from approved quarry in required thickness to proper camber and grade i/c hand packing mixing with 10’ screening & non plastic quarry of approved quality & gauge from approved source i/c watering and compacting to achieve 98-100% density as per modified AASHO specification ( rate i/cs all cost of material T&P and carriage upto site of wok.) | 9768/15 | P.% Cft: | 633855/= |
| 05/- | 2163Rft: | Laying brick on end edging including supply of 9”x 4 ½”x3” 1st: class burnt bricks excavation for laying edging with small side parallel to the road. Rate includes all costs of materials T&P and carriage upto site of work. | 2931/66 | P.% Rft: | 63411/= |

Contd: P/2

P-2

| Item No. | Qty’s | Description of item to be executed at site | Rate | Unit | Amount in Rupees |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 |
| 06/- | 3245Cft: | Preparing base course supplying and spreading stone metal of approved quality of 6” in 2 layers from approved quarry properly grade to maximum size of 1½” in required thickness of proper camber & grade i/c S/S 15 Cft: screening & non plastic quarry fines filling & depressions with stone metal after initial rolling including watering & compacting the same so as to achieve 100% density as per modified AASHO specifications (This i/cs providing and using templates camber plates screen from as directed).Rate includes all costs of materials T&P and carriage upto site of work. | 10849/90 | P.% Cft: | 352079/= |
| 07/- | 12978Sft: | Providing 1st: coat of surface dressing on or new existing surface with 30 lbs: bitumen and 4.0 cft: cursh bajri of required size including cleaning the road surface rolling etc: complete.Rate includes all costs of materials T&P & carriage. | 1708/94 | P.% Sft: | 221786/= |
| 08/- | 12978Sft: | Providing 1” thick consolidated premixed carpet in proper camber and grade i/c supply of 10 Cft: bajri 4.0 Cft: hill sand of approve quality and guage and bitumen 67 lbs: of 80/100 penetration i/c mixing the mechanical mixture in required proportion i/c heating the material and cleaning the road surface. Rate i/c all cost of material T&P and carriage upto site of work. | 4605/85 | P.% Sft: | 597746/= |
|  |  |  | TOTAL |  | 1964876/= |

_____________% Above / Below on the rates of CSR.

Amount to be added /deducted on the

Total (A) 									Rs:

Total Amount in words  ____________________________________________________

__________________________________________________________________________

Executive Engineer,

Contractor						                           Highways Division

Kashmore @ Kandhkot

*Imran*

REHABILITATION OF ROAD   IN    VILLAGE    ALI   MUHAMMAD   KHAN   KHOSO    ROAD MILE 1/6-2/4

DRAFT BIDDING DOCUMENT FOR WORKS

BILLS OF QUANTITIES

(A) 	Description and rate of Items bases on composite schedule of Rates

Part “A” (ROAD WORK)

| Item No. | Qty’s | Description of item to be executed at site |  | Rate | Unit | Amount in Rupees |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 |  | 4 | 5 | 6 |
| 01/- | 612Cft: | Preparing base course supplying and spreading stone metal of approved quality of 6” in 2 layers from approved quarry properly grade to maximum size of 1½” in required thickness of proper camber & grade i/c S/S 15 Cft: screening & non plastic quarry fines filling & depressions with stone metal after initial rolling including watering & compacting the same so as to achieve 100% density as per modified AASHO specifications (This i/cs providing and using templates camber plates screen from as directed).Rate includes all costs of materials T&P and carriage upto site of work. |  | 9982/50 | P.% Cft: | 61093/= |
| 02/- | -- | Laying brick on end edging including supply of 9”x 4 ½”x3” 1st: class burnt bricks excavation for laying edging with small side parallel to the road. Rate includes all costs of materials T&P and carriage upto site of work. |  | -- | -- | -- |
| 03/- | 4895Sft: | Providing 1st: coat of surface dressing on or new existing surface with 30 lbs: bitumen and 4.0 cft: cursh bajri of required size including cleaning the road surface rolling etc: complete.Rate includes all costs of materials T&P & carriage. |  | 1684/44 | P.% Sft: | 82453/= |
| 04/- | 24473Sft: | Providing 1” thick consolidated premixed carpet in proper camber and grade i/c supply of 10 Cft: bajri 4.0 Cft: hill sand of approve quality and guage and bitumen 67 lbs: of 80/100 penetration i/c mixing the mechanical mixture in required proportion i/c heating the material and cleaning the road surface. Rate i/c all cost of material T&P and carriage upto site of work. |  | 4598/64 | P.% Sft: | 1125425/= |
| 05/- | 88000Cft: | Earth Work embankment from barrow pits i/c laying in 6” layers, clod breaking remaining dressing complete, lead Upto 100’ft and lift upto 5’ft (In ordinary Soil) |  | 7327/99 | P.%0 Cft: | 644863/= |
|  |  |  | TOTAL |  |  | 1913834/= |

_____________% Above / Below on the rates of CSR.

Amount to be added /deducted on the

Total (A) 									Rs:

Total Amount in words  ____________________________________________________

__________________________________________________________________________

Executive Engineer,

Contractor						                           Highways Division

Kashmore @ Kandhkot

*Imran*

REHABILITATION OF ROAD   IN    VILLAGE    ALI   MUHAMMAD   KHAN   KHOSO    ROAD MILE 1/6-2/4

DRAFT BIDDING DOCUMENT FOR WORKS

BILLS OF QUANTITIES

(A) 	Description and rate of Items bases on composite schedule of Rates.

PART-B  (MISC:).

| Item No. | Qty’s | Description of item to be executed at site | Rate | Unit | Amount in Rupees |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 |
| 01/- | 01 Nos; | Proving and Fixing informatory sign rectangular shape 4’ x 3” comprising 02 Nos; 3” dia G.I Pipe Post i/c painting writing as directed each. | 10800/00 | P.Each | 10,800/= |
|  |  |  | TOTAL |  | 10,800/= |

Amount Total (B)

__________________________% Above / Below on the rates of CSR. 	Total (B) =

Amount to be added /deducted on the

basis of premium quoted Total (B)

Total (b) =  in works in figure

Executive Engineer,

Contractor						                           Highways Division

Kashmore @ Kandhkot

*Imran*

RECONDITIONING OF ROAD FROM MUHAMMAD SADDIQUE CHHAJJAN TO DILMURAD CHACHAR ROAD MILE 0/0-2/0 IN PORTION TALUKA KASHMORE

DRAFT BIDDING DOCUMENT FOR WORKS

BILLS OF QUANTITIES

(A) 	Description and rate of Items bases on composite schedule of Rates

Part “A” (ROAD WORK)

| Item No. | Qty’s | Description of item to be executed at site |  | Rate | Unit | Amount in Rupees |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 |  | 4 | 5 | 6 |
| 01/- | 2843Cft: | Preparing base course supplying and spreading stone metal of approved quality of 6” in 2 layers from approved quarry properly grade to maximum size of 1½” in required thickness of proper camber & grade i/c S/S 15 Cft: screening & non plastic quarry fines filling & depressions with stone metal after initial rolling including watering & compacting the same so as to achieve 100% density as per modified AASHO specifications (This i/cs providing and using templates camber plates screen from as directed).Rate includes all costs of materials T&P and carriage upto site of work. |  | 10608/95 | P.% Cft: | 301612/= |
| 02/- | 1648 Rft | Laying brick on end edging including supply of 9”x 4 ½”x3” 1st: class burnt bricks excavation for laying edging with small side parallel to the road. Rate includes all costs of materials T&P and carriage upto site of work. |  | 3188/43 | P% Rft | 52545/= |
| 03/- | 9888Sft: | Providing 1st: coat of surface dressing on or new existing surface with 30 lbs: bitumen and 4.0 cft: cursh bajri of required size including cleaning the road surface rolling etc: complete.Rate includes all costs of materials T&P & carriage. |  | 1701/77 | P.% Sft: | 168271/= |
| 04/- | 9888Sft: | Providing 1” thick consolidated premixed carpet in proper camber and grade i/c supply of 10 Cft: bajri 4.0 Cft: hill sand of approve quality and guage and bitumen 67 lbs: of 80/100 penetration i/c mixing the mechanical mixture in required proportion i/c heating the material and cleaning the road surface. Rate i/c all cost of material T&P and carriage upto site of work. |  | 4616/76 | P.% Sft: | 456505/= |
|  |  |  | TOTAL |  |  | 978933/= |

_____________% Above / Below on the rates of CSR.

Amount to be added /deducted on the

Total (A) 									Rs:

Total Amount in words  ____________________________________________________

__________________________________________________________________________

Executive Engineer,

Contractor						                           Highways Division

Kashmore @ Kandhkot

*Imran*

CONSTRUCTION OF ROAD FROM HOUSE OF SHAHBAN   MALIK    TO BERJ LAL TALUKA KANDHKOT

DRAFT BIDDING DOCUMENT FOR WORKS

BILLS OF QUANTITIES

(A)		Description and rate of Items bases on composite schedule of Rates

Part “A” (ROAD WORK)

| Item No. | Qty’s | Description of item to be executed at site | Rate | Unit | Amount in Rupees |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 |
| 01/- | -- | Earth work for road embankment by bulldozers i/c plugging and mixing clod breaking dressing and compacting with optimum moisture content lead upto 100’ and lift upto 5.0’ in all types of soil except rock compacting upto 85% density as per modified AASHO specifications. | -- | -- | -- |
| 02/- | -- | Earth work for road embankment by bulldozers i/c plugging and mixing clod breaking dressing and compacting with optimum moisture content lead upto 100’ and lift upto 5.0’ in all types of soil except rock compacting upto 95-100% density as per modified AASHO specifications. | -- | -- | -- |
| 03/- | 3708Cft: | Providing sand cushion i/c supplying & spreading pit/canal sand of approved quality from approved source of supply where ever available site of work i/c carriage site of work (R.A.A). | 1479/41 | P.% Cft: | 54857/= |
| 04/- | 3708Cft: | Providing Sub Base course by supplying and spreading stone metal 1 ½” gauge of approved quality from approved quarry in required thickness to proper camber and grade i/c hand packing mixing with 10’ screening & non plastic quarry of approved quality & gauge from approved source i/c watering and compacting to achieve 98-100% density as per modified AASHO specification ( rate i/cs all cost of material T&P and carriage upto site of wok.) | 8091/96 | P.% Cft: | 300050/= |

Contd: P/2

P-2

| Item No. | Qty’s | Description of item to be executed at site | Rate | Unit | Amount in Rupees |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 |
| 06/- | 1854Cft: | Preparing base course supplying and spreading stone metal of approved quality of 6” in 2 layers from approved quarry properly grade to maximum size of 1½” in required thickness of proper camber & grade i/c S/S 15 Cft: screening & non plastic quarry fines filling & depressions with stone metal after initial rolling including watering & compacting the same so as to achieve 100% density as per modified AASHO specifications (This i/cs providing and using templates camber plates screen from as directed).Rate includes all costs of materials T&P and carriage upto site of work. | 9115/11 | P.% Cft: | 168994/= |
| 07/- | 7416Sft: | Providing 1st: coat of surface dressing on or new existing surface with 30 lbs: bitumen and 4.0 cft: cursh bajri of required size including cleaning the road surface rolling etc: complete.Rate includes all costs of materials T&P & carriage. | 1657/34 | P.% Sft: | 122908/= |
| 08/- | 7416Sft: | Providing 1” thick consolidated premixed carpet in proper camber and grade i/c supply of 10 Cft: bajri 4.0 Cft: hill sand of approve quality and guage and bitumen 67 lbs: of 80/100 penetration i/c mixing the mechanical mixture in required proportion i/c heating the material and cleaning the road surface. Rate i/c all cost of material T&P and carriage upto site of work. | 4502/85 | P.% Sft: | 333931/= |
|  |  |  | TOTAL |  | 980740/= |

_____________% Above / Below on the rates of CSR.

Amount to be added /deducted on the

Total (A) 									Rs:

Total Amount in words  ____________________________________________________

__________________________________________________________________________

Executive Engineer,

Contractor						                           Highways Division

Kashmore @ Kandhkot

*Imran*

CONST OF RETAINING WALL OF GHOUSPUR TOWN ROAD TO MAJID KHAN SUNDRANI ROAD

DRAFT BIDDING DOCUMENT FOR WORKS

BILLS OF QUANTITIES

(A) 	Description and rate of Items bases on composite schedule of Rates.

Part “A”(360’ RFT: R/WALL).

| Item No. | Qty’s | Description of item to be executed at site | Rate | Unit | Amount in Rupees |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 |
| 01/- | 7200 Cft: | Excavation in foundation of building bridge other structure i/cs dag belling dressing refilling around the structure with excavated earth watering and remaining lead upto one chain and left upto 5.0’ ordinary soil. | 3176/25 | P.%0 Cft: | 22869/= |
| 02/- | 1800Cft: | Cement concrete with bricks are stone plaster 1½” to 2” guage ratio 1:2:4 | 11288/75 | P.% Cft: | 20320/= |
| 03/- | 12060Cft: | Pucca Brick work in foundation and plinth i/c cement sand mortor | 12501/41 | P.% Cft: | 1507670/= |
| 04/- | 102Cft: | Cement concrete plain i/c placing compacting finishing curing complete i/c screening washing of stone aggregate with out suhttering ratio 1:2:4: | 14429/25 | P.% Cft: | 14717/= |
| 05/- | 180Sft: | Erection and removal of centering for R.C.C or plain cement work (2nd: class) for partal wood vertical (S.I.No:18)(b) P-20). | 7000/00 | P.% Sft: | 12600/= |
| 06/- | 2520Sft: | Cement Pointing Struck jointing Wall ratio 1:3 | 1213/58 | P.% Sft: | 30582/= |
| 07/- | 20160Cft: | Filling watering and remaining earth under floor with new earth excavated from side lead upto 100’ft and lift upto 5’ft. | 2208/37 | P.0% Cft: | 44520/= |
|  |  |  | TOTAL |  | 1653278/= |

Amount Total (a)

__________________________% Above / Below on the rates of CSR. 	Total (a) =

Amount to be added /deducted on the

basis of premium quoted Total (a)

Total (a) =  in works in figure

Executive Engineer,

Contractor						                           Highways Division

Kashmore @ Kandhkot

*Imran*

CONST OF RETAINING WALL OF NAJAMDIN CHACHAR TO BYE PASS KANDHKOT

DRAFT BIDDING DOCUMENT FOR WORKS

BILLS OF QUANTITIES

(A) 	Description and rate of Items bases on composite schedule of Rates.

Part “A”(525’ RFT: R/WALL).

| Item No. | Qty’s | Description of item to be executed at site | Rate | Unit | Amount in Rupees |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 |
| 01/- | 10500 Cft: | Excavation in foundation of building bridge other structure i/cs dag belling dressing refilling around the structure with excavated earth watering and remaining lead upto one chain and left upto 5.0’ ordinary soil. | 3176/25 | P.%0 Cft: | 33351/= |
| 02/- | 2625Cft: | Cement concrete with bricks are stone plaster 1½” to 2” guage ratio 1:2:4 | 9416/28 | P.% Cft: | 247177/= |
| 03/- | 17588Cft: | Pucca Brick work in foundation and plinth i/c cement sand mortor | 12501/41 | P.% Cft: | 2198747/= |
| 04/- | 148Cft: | Cement concrete plain i/c placing compacting finishing curing complete i/c screening washing of stone aggregate with out suhttering ratio 1:2:4: | 14429/25 | P.% Cft: | 21355/= |
| 05/- | 263Sft: | Erection and removal of centering for R.C.C or plain cement work (2nd: class) for partal wood vertical (S.I.No:18)(b) P-20). | 7000/00 | P.% Sft: | 18410/= |
| 06/- | 3675Sft: | Cement Pointing Struck jointing Wall ratio 1:3 | 1213/58 | P.% Sft: | 44599/= |
| 07/- | 19490Cft: | Filling watering and remaining earth under floor with new earth excavated from side lead upto 100’ft and lift upto 5’ft. | 9424/10 | P.0% Cft: | 183675/= |
|  |  |  | TOTAL |  | 2747314/= |

Amount Total (a)

__________________________% Above / Below on the rates of CSR. 	Total (a) =

Amount to be added /deducted on the

basis of premium quoted Total (a)

Total (a) =  in works in figure

Executive Engineer,

Contractor						                           Highways Division

Kashmore @ Kandhkot

*Imran*

CONSTRUCTION OF BRIDGE OF ATTA MUHAMMAD KHAN MINOR (WATER COURSES) AT VILLAGE ANWAR BAG JAKHRANI, DARYA KHAN JAKHRANI LIAQUAT ALI U/C TOJ TALUKA KASHMORE ’

DRAFT BIDDING DOCUMENT FOR WORKS

BILLS OF QUANTITIES

(A) 	Description and rate of Items bases on composite schedule of Rates.

PART-A  (3.0 FT: SPAN CULVERTS).

| Item No. | Qty’s | Description of item to be executed at site | Rate | Unit | Amount in Rupees |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 |
| 01/- | 1106 Cft: | Excavation in foundation of building bridge other structure i/cs dag belling dressing refilling around the structure with excavated earth watering and remaining lead upto one chain and left upto 5.0’ ordinary soil. | 3176/25 | P.%0 Cft: | 3513/= |
| 02/- | 194 Cft: | Cement concrete with bricks are stone plaster 1½” to 2” guage ratio 1:2:4 | 9416/28 | P.% Cft: | 18268/= |
| 03/- | 502 Cft: | Pucca Brick work in foundation and plinth i/c cement sand mortor | 12501/41 | P.% Cft: | 62755/= |
| 04/- | 55 Cft: | Cement concrete plain i/c placing compacting finishing curing complete i/c screening washing of stone aggregate with out suhttering ratio 1:2:4: | 14429/25 | P.% Cft: | 7936/= |
| 05/- | 58 Sft: | Erection and removal of centering for R.C.C or plain cement work (2nd: class) for partal wood vertical | 7000/00 | P.% Sft: | 4060/= |
| 06/- | A. Tor Steel 4.16 Cwt B. M/ Steel 0.907 Cwt | Fabrication of m/s reinforcement cement concrete including cutting binding laying in position making joints & fastening of i/c binding wire also removal rust from bars. | 5001/70 4820/29 | P.Cwt: P.Cwt | 20807/= 4371/= |
| 07/- | 114 Rft: | Reinforced cement concrete work including all labour and material except the cost of steel reinforcement and its labour for bending and binding which will be paid separately. This rate also includes all kind of forms moulds lifting shuttering curing rend. | 337/00 | P.Rft: | 38418/= |
| 08/- | 330 Sft: | Cement Plaster (1:3) upto 20’ height 1½” thick. | 2283/93 | P.% Sft: | 7537/= |
|  |  |  | TOTAL |  | 171808/= |

Amount Total (a)

__________________________% Above / Below on the rates of CSR. 	Total (a) =

Amount to be added /deducted on the

basis of premium quoted Total (a)

Total (a) =  in works in figure

Executive Engineer,

Contractor						                           Highways Division

Kashmore @ Kandhkot

*Imran*

CONSTRUCTION OF CULVERTS IN VARIOUS POINTS OF U/C MALGUZAR TALUKA KANDHKOT

DRAFT BIDDING DOCUMENT FOR WORKS

BILLS OF QUANTITIES

(A) 	Description and rate of Items bases on composite schedule of Rates.

PART-A  (3.0 FT: SPAN CULVERTS).

| Item No. | Qty’s |  | Description of item to be executed at site | Rate | Unit | Amount in Rupees |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 |  | 3 | 4 | 5 | 6 |
| 01/- | 1106 Cft: |  | Excavation in foundation of building bridge other structure i/cs dag belling dressing refilling around the structure with excavated earth watering and remaining lead upto one chain and left upto 5.0’ ordinary soil. | 3176/25 | P.%0 Cft: | 3513/= |
| 02/- | 238 Cft: |  | Cement concrete with bricks are stone plaster 1½” to 2” guage ratio 1:2:4 | 9416/28 | P.% Cft: | 22411/= |
| 03/- | 502 Cft: |  | Pucca Brick work in foundation and plinth i/c cement sand mortor | 12501/41 | P.% Cft: | 62755/= |
| 04/- | 55 Cft: |  | Cement concrete plain i/c placing compacting finishing curing complete i/c screening washing of stone aggregate with out suhttering ratio 1:2:4: | 14429/25 | P.% Cft: | 7936/= |
| 05/- | 58 Sft: |  | Erection and removal of centering for R.C.C or plain cement work (2nd: class) for partal wood vertical | 7000/00 | P.% Sft: | 4060/= |
| 06/- | A. Tor Steel 4.16 Cwt B. M/ Steel 0.907 Cwt |  | Fabrication of m/s reinforcement cement concrete including cutting binding laying in position making joints & fastening of i/c binding wire also removal rust from bars. | 5001/70 820/29 | P.Cwt: P.Cwt | 20807/= 4371/= |
| 07/- | 114 Rft: |  | Reinforced cement concrete work including all labour and material except the cost of steel reinforcement and its labour for bending and binding which will be paid separately. This rate also includes all kind of forms moulds lifting shuttering curing rend. | 337/00 | P.Rft: | 38418/= |
| 08/- | 330 Sft: |  | Cement Plaster (1:3) upto 20’ height 1½” thick. | 2283/93 | P.% Sft: | 7537/= |
| 09/- |  | 8100Cft: | Earth work on road embankment by bulldozers i/c plugging mixing clod breaking dressing and compcting with optimum moisture content lead upto 100’ and lift upto 5’ in all types of soil | 2208/37 | P.%0 Cft: | 17888/= |
|  |  |  |  | TOTAL |  | 193756/= |

Amount Total (a)

__________________________% Above / Below on the rates of CSR. 	Total (a) =

Amount to be added /deducted on the

basis of premium quoted Total (a)

Total (a) =  in works in figure

Executive Engineer,

Contractor						                           Highways Division

Kashmore @ Kandhkot

*Imran*

CONSTRUCTION OF ROAD GHOUSPUR VAN STOP (NHA) VIA MOAR MUHAMAMD MARKET, VIA MANDER VAI POLICE SATION TO NBP GHOUSPUR MILE 0/0-0/7

DRAFT BIDDING DOCUMENT FOR WORKS

BILLS OF QUANTITIES

(A)		Description and rate of Items bases on composite schedule of Rates

Part “A” (ROAD WORK)

| Item No. | Qty’s | Description of item to be executed at site | Rate | Unit | Amount in Rupees |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 |
| 01/- | -- | Earth work for road embankment by bulldozers i/c plugging and mixing clod breaking dressing and compacting with optimum moisture content lead upto 100’ and lift upto 5.0’ in all types of soil except rock compacting upto 85% density as per modified AASHO specifications. | -- | -- | -- |
| 02/- | -- | Earth work for road embankment by bulldozers i/c plugging and mixing clod breaking dressing and compacting with optimum moisture content lead upto 100’ and lift upto 5.0’ in all types of soil except rock compacting upto 95-100% density as per modified AASHO specifications. | -- | -- | -- |
| 03/- | 11556Cft: | Providing sand cushion i/c supplying & spreading pit/canal sand of approved quality from approved source of supply where ever available site of work i/c carriage site of work (R.A.A). | 1479/41 | P.% Cft: | 170960/= |
| 04/- | 37647Cft: | Providing Sub Base course by supplying and spreading stone metal 1 ½” gauge of approved quality from approved quarry in required thickness to proper camber and grade i/c hand packing mixing with 10’ screening & non plastic quarry of approved quality & gauge from approved source i/c watering and compacting to achieve 98-100% density as per modified AASHO specification ( rate i/cs all cost of material T&P and carriage upto site of wok.) | 8091/96 | P.% Cft: | 3046380/= |

Contd: P/2

P-2

| Item No. | Qty’s | Description of item to be executed at site | Rate | Unit | Amount in Rupees |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 |
| 06/- | 18823Cft: | Preparing base course supplying and spreading stone metal of approved quality of 6” in 2 layers from approved quarry properly grade to maximum size of 1½” in required thickness of proper camber & grade i/c S/S 15 Cft: screening & non plastic quarry fines filling & depressions with stone metal after initial rolling including watering & compacting the same so as to achieve 100% density as per modified AASHO specifications (This i/cs providing and using templates camber plates screen from as directed).Rate includes all costs of materials T&P and carriage upto site of work. | 9115/11 | P.% Cft: | 1715737/= |
| 07/- | 75293Sft: | Providing 1st: coat of surface dressing on or new existing surface with 30 lbs: bitumen and 4.0 cft: cursh bajri of required size including cleaning the road surface rolling etc: complete.Rate includes all costs of materials T&P & carriage. | 1657/34 | P.% Sft: | 1247861/= |
| 08/- | 75293Sft: | Providing 1” thick consolidated premixed carpet in proper camber and grade i/c supply of 10 Cft: bajri 4.0 Cft: hill sand of approve quality and guage and bitumen 67 lbs: of 80/100 penetration i/c mixing the mechanical mixture in required proportion i/c heating the material and cleaning the road surface. Rate i/c all cost of material T&P and carriage upto site of work. | 4586/21 | P.% Sft: | 3453095/= |
|  |  |  | TOTAL |  | 9634033/= |

_____________% Above / Below on the rates of CSR.

Amount to be added /deducted on the

Total (A) 									Rs:

Total Amount in words  ____________________________________________________

__________________________________________________________________________

Executive Engineer,

Contractor						                           Highways Division

Kashmore @ Kandhkot

*Imran*

CONSTRUCTION OF INTERNAL ROADS OF GHOUSPUR TOWN TALUKA KANDHKOT

DRAFT BIDDING DOCUMENT FOR WORKS

BILLS OF QUANTITIES

(A)		Description and rate of Items bases on composite schedule of Rates

Part “A” (ROAD WORK)

| Item No. | Qty’s | Description of item to be executed at site | Rate | Unit | Amount in Rupees |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 |
| 01/- | -- | Earth work for road embankment by bulldozers i/c plugging and mixing clod breaking dressing and compacting with optimum moisture content lead upto 100’ and lift upto 5.0’ in all types of soil except rock compacting upto 85% density as per modified AASHO specifications. | -- | -- | -- |
| 02/- | -- | Earth work for road embankment by bulldozers i/c plugging and mixing clod breaking dressing and compacting with optimum moisture content lead upto 100’ and lift upto 5.0’ in all types of soil except rock compacting upto 95-100% density as per modified AASHO specifications. | -- | -- | -- |
| 03/- | 28512Cft: | Providing sand cushion i/c supplying & spreading pit/canal sand of approved quality from approved source of supply where ever available site of work i/c carriage site of work (R.A.A). | 1479/41 | P.% Cft: | 421809/= |
| 04/- | 45423Cft: | Providing Sub Base course by supplying and spreading stone metal 1 ½” gauge of approved quality from approved quarry in required thickness to proper camber and grade i/c hand packing mixing with 10’ screening & non plastic quarry of approved quality & gauge from approved source i/c watering and compacting to achieve 98-100% density as per modified AASHO specification ( rate i/cs all cost of material T&P and carriage upto site of wok.) | 8091/96 | P.% Cft: | 3675611/= |

Contd: P/2

P-2

| Item No. | Qty’s | Description of item to be executed at site | Rate | Unit | Amount in Rupees |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 |
| 06/- | 22712Cft: | Preparing base course supplying and spreading stone metal of approved quality of 6” in 2 layers from approved quarry properly grade to maximum size of 1½” in required thickness of proper camber & grade i/c S/S 15 Cft: screening & non plastic quarry fines filling & depressions with stone metal after initial rolling including watering & compacting the same so as to achieve 100% density as per modified AASHO specifications (This i/cs providing and using templates camber plates screen from as directed).Rate includes all costs of materials T&P and carriage upto site of work. | 9115/11 | P.% Cft: | 2070178/= |
| 07/- | 90846Sft: | Providing 1st: coat of surface dressing on or new existing surface with 30 lbs: bitumen and 4.0 cft: cursh bajri of required size including cleaning the road surface rolling etc: complete.Rate includes all costs of materials T&P & carriage. | 1657/34 | P.% Sft: | 1505627/= |
| 08/- | 90846Sft: | Providing 1” thick consolidated premixed carpet in proper camber and grade i/c supply of 10 Cft: bajri 4.0 Cft: hill sand of approve quality and guage and bitumen 67 lbs: of 80/100 penetration i/c mixing the mechanical mixture in required proportion i/c heating the material and cleaning the road surface. Rate i/c all cost of material T&P and carriage upto site of work. | 4528/91 | P.% Sft: | 4114334/= |
|  |  |  | TOTAL |  | 11787559/= |

_____________% Above / Below on the rates of CSR.

Amount to be added /deducted on the

Total (A) 									Rs:

Total Amount in words  ____________________________________________________

__________________________________________________________________________

Executive Engineer,

Contractor						                           Highways Division

Kashmore @ Kandhkot

*Imran*

CONSTRUCTION OF LINK ROAD FROM BUXAPUR ROAD TO VILLAGE LIAQUAT KHAN DOMKI TALUKA KASHMORE MILE 0/0-0/4+330’

DRAFT BIDDING DOCUMENT FOR WORKS

BILLS OF QUANTITIES

- Description and rate of Items bases on composite schedule of Rates

Part “A” (ROAD WORK)

| Item No. | Qty’s | Description of item to be executed at site | Rate | Unit | Amount in Rupees |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 |
| 01/- | 334000Cft: | Earth work for road embankment by bulldozers i/c plugging and mixing clod breaking dressing and compacting with optimum moisture content lead upto 100’ and lift upto 5.0’ in all types of soil except rock compacting upto 85% density as per modified AASHO specifications. | 3656/23 | P.0% Cft: | 1222643/= |
| 02/- | 36709Cft: | Earth work for road embankment by bulldozers i/c plugging and mixing clod breaking dressing and compacting with optimum moisture content lead upto 100’ and lift upto 5.0’ in all types of soil except rock compacting upto 95-100% density as per modified AASHO specifications. | 2529/77 | P.% Cft: | 92865/= |
| 03/- | 18355Cft: | Providing sand cushion i/c supplying & spreading pit/canal sand of approved quality from approved source of supply where ever available site of work i/c carriage site of work (R.A.A). | 1479/41 | P.% Cft: | 271540/= |
| 04/- | 18355Cft: | Providing Sub Base course by supplying and spreading stone metal 1 ½” gauge of approved quality from approved quarry in required thickness to proper camber and grade i/c hand packing mixing with 10’ screening & non plastic quarry of approved quality & gauge from approved source i/c watering and compacting to achieve 98-100% density as per modified AASHO specification ( rate i/cs all cost of material T&P and carriage upto site of wok.) | 9675/02 | P.% Cft: | 1775850/= |
| 05/- | 6118Rft: | Laying brick on end edging including supply of 9”x 4 ½”x3” 1st: class burnt bricks excavation for laying edging with small side parallel to the road. Rate includes all costs of materials T&P and carriage upto site of work. | 2907/09 | P.% Rft: | 177856/= |

Contd: P/2

P-2

| Item No. | Qty’s | Description of item to be executed at site | Rate | Unit | Amount in Rupees |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 |
| 06/- | 9177Cft: | Preparing base course supplying and spreading stone metal of approved quality of 6” in 2 layers from approved quarry properly grade to maximum size of 1½” in required thickness of proper camber & grade i/c S/S 15 Cft: screening & non plastic quarry fines filling & depressions with stone metal after initial rolling including watering & compacting the same so as to achieve 100% density as per modified AASHO specifications (This i/cs providing and using templates camber plates screen from as directed).Rate includes all costs of materials T&P and carriage upto site of work. | 10753/52 | P.% Cft: | 986850/= |
| 07/- | 36709Sft: | Providing 1st: coat of surface dressing on or new existing surface with 30 lbs: bitumen and 4.0 cft: cursh bajri of required size including cleaning the road surface rolling etc: complete.Rate includes all costs of materials T&P & carriage. | 1706/06 | P.% Sft: | 626278/= |
| 08/- | 36709Sft: | Providing 1” thick consolidated premixed carpet in proper camber and grade i/c supply of 10 Cft: bajri 4.0 Cft: hill sand of approve quality and guage and bitumen 67 lbs: of 80/100 penetration i/c mixing the mechanical mixture in required proportion i/c heating the material and cleaning the road surface. Rate i/c all cost of material T&P and carriage upto site of work. | 4631/31 | P.% Sft: | 1700108/= |
|  |  |  | TOTAL |  | 6853990/= |

_____________% Above / Below on the rates of CSR.

Amount to be added /deducted on the

Total (A) 									Rs:

Total Amount in words  ____________________________________________________

__________________________________________________________________________

Executive Engineer,

Contractor						                           Highways Division

Kashmore @ Kandhkot

*Imran*

CONSTRUCTION OF LINK ROAD FROM QABIL MAZARI UPTO VILLAGE QAZI MAZARI TALUKA KASHMORE MILE 0/0-0/2

DRAFT BIDDING DOCUMENT FOR WORKS

BILLS OF QUANTITIES

- Description and rate of Items bases on composite schedule of Rates

Part “A” (ROAD WORK)

| Item No. | Qty’s | Description of item to be executed at site | Rate | Unit | Amount in Rupees |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 |
| 01/- | 152000Cft: | Earth work for road embankment by bulldozers i/c plugging and mixing clod breaking dressing and compacting with optimum moisture content lead upto 100’ and lift upto 5.0’ in all types of soil except rock compacting upto 85% density as per modified AASHO specifications. | 3656/23 | P.0% Cft: | 555747/= |
| 02/- | 16315Cft: | Earth work for road embankment by bulldozers i/c plugging and mixing clod breaking dressing and compacting with optimum moisture content lead upto 100’ and lift upto 5.0’ in all types of soil except rock compacting upto 95-100% density as per modified AASHO specifications. | 2529/77 | P.% Cft: | 41273/= |
| 03/- | 8158Cft: | Providing sand cushion i/c supplying & spreading pit/canal sand of approved quality from approved source of supply where ever available site of work i/c carriage site of work (R.A.A). | 1479/41 | P.% Cft: | 120690/= |
| 04/- | 8158Cft: | Providing Sub Base course by supplying and spreading stone metal 1 ½” gauge of approved quality from approved quarry in required thickness to proper camber and grade i/c hand packing mixing with 10’ screening & non plastic quarry of approved quality & gauge from approved source i/c watering and compacting to achieve 98-100% density as per modified AASHO specification ( rate i/cs all cost of material T&P and carriage upto site of wok.) | 9861/27 | P.% Cft: | 804482/= |
| 05/- | 2719Rft: | Laying brick on end edging including supply of 9”x 4 ½”x3” 1st: class burnt bricks excavation for laying edging with small side parallel to the road. Rate includes all costs of materials T&P and carriage upto site of work. | 3261/37 | P.% Rft: | 88677/= |

Contd: P/2

P-2

| Item No. | Qty’s | Description of item to be executed at site | Rate | Unit | Amount in Rupees |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 |
| 06/- | 4079Cft: | Preparing base course supplying and spreading stone metal of approved quality of 6” in 2 layers from approved quarry properly grade to maximum size of 1½” in required thickness of proper camber & grade i/c S/S 15 Cft: screening & non plastic quarry fines filling & depressions with stone metal after initial rolling including watering & compacting the same so as to achieve 100% density as per modified AASHO specifications (This i/cs providing and using templates camber plates screen from as directed).Rate includes all costs of materials T&P and carriage upto site of work. | 10946/28 | P.% Cft: | 446499/= |
| 07/- | 16315Sft: | Providing 1st: coat of surface dressing on or new existing surface with 30 lbs: bitumen and 4.0 cft: cursh bajri of required size including cleaning the road surface rolling etc: complete.Rate includes all costs of materials T&P & carriage. | 1711/80 | P.% Sft: | 279280/= |
| 08/- | 16315Sft: | Providing 1” thick consolidated premixed carpet in proper camber and grade i/c supply of 10 Cft: bajri 4.0 Cft: hill sand of approve quality and guage and bitumen 67 lbs: of 80/100 penetration i/c mixing the mechanical mixture in required proportion i/c heating the material and cleaning the road surface. Rate i/c all cost of material T&P and carriage upto site of work. | 4623/36 | P.% Sft: | 754301/= |
|  |  |  | TOTAL |  | 3090949/= |

_____________% Above / Below on the rates of CSR.

Amount to be added /deducted on the

Total (A) 									Rs:

Total Amount in words  ____________________________________________________

__________________________________________________________________________

Executive Engineer,

Contractor						                           Highways Division

Kashmore @ Kandhkot

*Imran*

CONSTRUCTION OF  ZAHIR PIR ROAD MILE 0/0-0/3

DRAFT BIDDING DOCUMENT FOR WORKS

BILLS OF QUANTITIES

- Description and rate of Items bases on composite schedule of Rates

Part “A” (ROAD WORK)

| Item No. | Qty’s | Description of item to be executed at site | Rate | Unit | Amount in Rupees |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 |
| 01/- | 180000Cft: | Earth work for road embankment by bulldozers i/c plugging and mixing clod breaking dressing and compacting with optimum moisture content lead upto 100’ and lift upto 5.0’ in all types of soil except rock compacting upto 85% density as per modified AASHO specifications. | 3656/23 | P.0% Cft: | 658121/= |
| 02/- | 32630Cft: | Earth work for road embankment by bulldozers i/c plugging and mixing clod breaking dressing and compacting with optimum moisture content lead upto 100’ and lift upto 5.0’ in all types of soil except rock compacting upto 95-100% density as per modified AASHO specifications. | 2529/77 | P.% Cft: | 82547/= |
| 03/- | 16315Cft: | Providing sand cushion i/c supplying & spreading pit/canal sand of approved quality from approved source of supply where ever available site of work i/c carriage site of work (R.A.A). | 1479/41 | P.% Cft: | 241368/= |
| 04/- | 16315Cft: | Providing Sub Base course by supplying and spreading stone metal 1 ½” gauge of approved quality from approved quarry in required thickness to proper camber and grade i/c hand packing mixing with 10’ screening & non plastic quarry of approved quality & gauge from approved source i/c watering and compacting to achieve 98-100% density as per modified AASHO specification ( rate i/cs all cost of material T&P and carriage upto site of wok.) | 10140/64 | P.% Cft: | 1654445/= |
| 05/- | 5484Rft: | Laying brick on end edging including supply of 9”x 4 ½”x3” 1st: class burnt bricks excavation for laying edging with small side parallel to the road. Rate includes all costs of materials T&P and carriage upto site of work. | 3011/29 | P.% Rft: | 165139/= |

Contd: P/2

P-2

| Item No. | Qty’s | Description of item to be executed at site | Rate | Unit | Amount in Rupees |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 |
| 06/- | 7920Cft: | Preparing base course supplying and spreading stone metal of approved quality of 6” in 2 layers from approved quarry properly grade to maximum size of 1½” in required thickness of proper camber & grade i/c S/S 15 Cft: screening & non plastic quarry fines filling & depressions with stone metal after initial rolling including watering & compacting the same so as to achieve 100% density as per modified AASHO specifications (This i/cs providing and using templates camber plates screen from as directed).Rate includes all costs of materials T&P and carriage upto site of work. | 11235/41 | P.% Cft: | 889844/= |
| 07/- | 32630Sft: | Providing 1st: coat of surface dressing on or new existing surface with 30 lbs: bitumen and 4.0 cft: cursh bajri of required size including cleaning the road surface rolling etc: complete.Rate includes all costs of materials T&P & carriage. | 1720/40 | P.% Sft: | 561373/= |
| 08/- | 32630Sft: | Providing 1” thick consolidated premixed carpet in proper camber and grade i/c supply of 10 Cft: bajri 4.0 Cft: hill sand of approve quality and guage and bitumen 67 lbs: of 80/100 penetration i/c mixing the mechanical mixture in required proportion i/c heating the material and cleaning the road surface. Rate i/c all cost of material T&P and carriage upto site of work. | 4649/43 | P.% Sft: | 1517109/= |
|  |  |  | TOTAL |  | 5549946/= |

_____________% Above / Below on the rates of CSR.

Amount to be added /deducted on the

Total (A) 									Rs:

Total Amount in words  ____________________________________________________

__________________________________________________________________________

Executive Engineer,

Contractor						                           Highways Division

Kashmore @ Kandhkot

*Imran*

W/R OF DUAL CARRIAGE WAY ROAD FROM GURDWARA CHOWK ROAD TO RAJA PETROL PUMP ROAD MILE 1/0-1/6 (SIDE DRAIN)

DRAFT BIDDING DOCUMENT FOR WORKS

BILLS OF QUANTITIES

(A) 	Description and rate of Items bases on composite schedule of Rates.

PART-A  (SIDE DRAIN).

| Item No. | Qty’s | Description of item to be executed at site | Rate | Unit | Amount in Rupees |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 |
| 01/- | 56700Cft: | Excavation in foundation of building bridge other structure i/cs dag belling dressing refilling around the structure with excavated earth watering and remaining lead upto one chain and left upto 5.0’ ordinary soil. | 3176/25 | P.%0 Cft: | 180093/= |
| 02/- | 9450Cft: | Cement concrete with bricks are stone plaster 1½” to 2” guage ratio 1:2:4 | 9416/28 | P.% Cft: | 889838/= |
| 03/- | 28350Cft: | Pucca Brick work in foundation and plinth i/c cement sand mortor | 12501/41 | P.% Cft: | 3544150/= |
| 04/- | 12474 Rft: | Reinforced cement concrete work including all labour and material except the cost of steel reinforcement and its labour for bending and binding which will be paid separately. This rate also includes all kind of forms moulds lifting shuttering curing rend. | 337/00 | P.Rft: | 4203738/= |
| 05/- | 556.88 Cwt | Fabrication of m/s reinforcement cement concrete including cutting binding laying in position making joints & fastening of i/c binding wire also removal rust from bars. | 4820/20 | P.Cwt: | 2684273/= |
| 06/- | 75600 Sft: | Cement Plaster (1:6) upto 20’ height 1½” thick. | 2283/93 | P.% Sft: | 1726651/= |
| 07/- | 75600 Sft: | Cement Plaster (1:4) upto 20’ height 1½” thick. | 3015/76 | P.% Sft: | 2279915/= |
| 08/- | 6300 Rft | Const Standard type countee block B-type 1:2;4 i/c cost of molding one had of neru shuttering erection removal etc complete | 174/00 | P.Rft | 1096200/= |
|  |  |  | TOTAL |  | 16604858/= |

Amount Total (a)

__________________________% Above / Below on the rates of CSR. 	Total (a) =

Amount to be added /deducted on the

basis of premium quoted Total (a)

Total (a) =  in works in figure

Executive Engineer,

Contractor						                           Highways Division

Kashmore @ Kandhkot

*Imran*

RECONDITIONING OF ROAD FROM UC-I OFFICE SHIKARPUR NAKKA TO KANDHKOT SHAKH AWAN MUHALLA ROAD MILE 0/5-0/6 I/C APPROACHES

DRAFT BIDDING DOCUMENT FOR WORKS

BILLS OF QUANTITIES

(A) 	Description and rate of Items bases on composite schedule of Rates

Part “A” (ROAD WORK)

| Item No. | Qty’s | Description of item to be executed at site | Rate | Unit | Amount in Rupees |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 |
| 01/- | -- | Earth work for road embankment by bulldozers i/c plugging and mixing clod breaking dressing and compacting with optimum moisture content lead upto 100’ and lift upto 5.0’ in all types of soil except rock compacting upto 85% density as per modified AASHO specifications. | -- | -- | -- |
| 02/- | -- | Earth work for road embankment by bulldozers i/c plugging and mixing clod breaking dressing and compacting with optimum moisture content lead upto 100’ and lift upto 5.0’ in all types of soil except rock compacting upto 95-100% density as per modified AASHO specifications. | -- | -- | -- |
| 03/- | 23731Cft: | Providing sand cushion i/c supplying & spreading pit/canal sand of approved quality from approved source of supply where ever available site of work i/c carriage site of work (R.A.A). | 1479/41 | P.% Cft: | 35108/= |
| 04/- | 15574Cft: | Preparing sub base course by supplying & spreading stone metal 1 ½”-2” gauge of approved quarry in required 3” thickness in 1 layer each 3.0 each layers with 20 cft: pit canal sand and having plasticity index of not more than 6% watering & compacting to achieve 98/100% density as per modified AASHO specifications. Rate includes all costs of materials T&P and carriage upto site of work. | 8168/76 | P.% Cft: | 1272203/= |
| 05/- | 7787Cft: | Providing Sub Base course by supplying and spreading stone metal 1 ½” gauge of approved quality from approved quarry in required thickness to proper camber and grade i/c hand packing mixing with 10’ screening & non plastic quarry of approved quality & gauge from approved source i/c watering and compacting to achieve 98-100% density as per modified AASHO specification ( rate i/cs all cost of material T&P and carriage upto site of wok.) | 8557/52 | P.% Cft: | 666374/= |
| 06/- | 3008Rft: | Laying brick on end edging including supply of 9”x 4 ½”x3” 1st: class burnt bricks excavation for laying edging with small side parallel to the road. Rate includes all costs of materials T&P and carriage upto site of work. | 2643/74 | P.% Rft: | 79524/= |

Contd: P/2

P-2

| Item No. | Qty’s | Description of item to be executed at site | Rate | Unit | Amount in Rupees |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 |
| 07/- | 7787Cft: | Preparing base course supplying and spreading stone metal of approved quality of 6” in 2 layers from approved quarry properly grade to maximum size of 1½” in required thickness of proper camber & grade i/c S/S 15 Cft: screening & non plastic quarry fines filling & depressions with stone metal after initial rolling including watering & compacting the same so as to achieve 100% density as per modified AASHO specifications (This i/cs providing and using templates camber plates screen from as directed).Rate includes all costs of materials T&P and carriage upto site of work. | 9596/99 | P.% Cft: | 747318/= |
| 08/- | 31147 Sft: | Providing 2 coat of surface dressing on or new existing surface with 55 lbs: bitumen and 6.75cft: cursh bajri of required size including cleaning the road surface rolling etc: complete.Rate includes all costs of materials T&P & carriage. | 2993/65 | P.% Sft: | 932432/= |
| 09/- | 31147 Sft: | Laying to proper line and grade plant mixes Ashplat concrete paver finished Hydraulic / Electronic control) prepared to specified formula according to job mix formula according to job mix formula approved by Engineer incharge i/c rolling and finished to proper line garde level and camber etc compete. Rate i/cs tack coat with bitumen 80/100 and all cost of material roller. T&P and carriage of material from source of supply tom plant and plant to site of work (S.I No: 26(b) P-11) | 9224/34 | P.% Sft: | 2873105/= |
|  |  |  | TOTAL |  | 6606064/= |

_____________% Above / Below on the rates of CSR.

Amount to be added /deducted on the

Total (A) 									Rs:

Total Amount in words  ____________________________________________________

__________________________________________________________________________

Executive Engineer,

Contractor						                           Highways Division

Kashmore @ Kandhkot

*Imran*

RECONDITIONING OF ROAD FROM UC-I OFFICE SHIKARPUR NAKKA TO KANDHKOT SHAKH AWAN MUHALLA ROAD MILE 0/5-0/6 I/C APPROACHES

DRAFT BIDDING DOCUMENT FOR WORKS

BILLS OF QUANTITIES

(A) 	Description and rate of Items bases on composite schedule of Rates.

PART-B  (SIDE DRAIN).

| Item No. | Qty’s | Description of item to be executed at site | Rate | Unit | Amount in Rupees |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 |
| 01/- | 900Cft: | Excavation in foundation of building bridge other structure i/cs dag belling dressing refilling around the structure with excavated earth watering and remaining lead upto one chain and left upto 5.0’ ordinary soil. | 3176/25 | P.%0 Cft: | 28591/= |
| 02/- | 150Cft: | Cement concrete with bricks are stone plaster 1½” to 2” guage ratio 1:2:4 | 9416/28 | P.% Cft: | 14124/= |
| 03/- | 450Cft: | Pucca Brick work in foundation and plinth i/c cement sand mortor | 12501/41 | P.% Cft: | 56256/= |
| 04/- | 198 Rft: | Reinforced cement concrete work including all labour and material except the cost of steel reinforcement and its labour for bending and binding which will be paid separately. This rate also includes all kind of forms moulds lifting shuttering curing rend. | 337/00 | P.Rft: | 66726/= |
| 05/- | 8.84 Cwt | Fabrication of m/s reinforcement cement concrete including cutting binding laying in position making joints & fastening of i/c binding wire also removal rust from bars. | 4820/20 | P.Cwt: | 42611/= |
| 06/- | 1200 Sft: | Cement Plaster (1:6) upto 20’ height 1½” thick. | 2283/93 | P.% Sft: | 27407/= |
| 07/- | 1200 Sft: | Cement Plaster (1:4) upto 20’ height 1½” thick. | 3015/76 | P.% Sft: | 36189/= |
| 08/- | 100 Rft | Const Standard type countee block B-type 1:2;4 i/c cost of molding one had of neru shuttering erection removal etc complete | 174/00 | P.Rft | 17400/= |
|  |  |  | TOTAL |  | 263572/= |

Amount Total (b)

__________________________% Above / Below on the rates of CSR. 	Total (b) =

Amount to be added /deducted on the

basis of premium quoted Total (b)

Total (b) =  in works in figure

Executive Engineer,

Contractor						                           Highways Division

Kashmore @ Kandhkot

*Imran*

CONSTRUCTION OF ROAD FROM ABDUL GHAFOOR CHACHAR VIA AMIR BUX SOLANGI UPTO DILMURAD CHACHAR TALUKA KASHMORE

DRAFT BIDDING DOCUMENT FOR WORKS

BILLS OF QUANTITIES

- Description and rate of Items bases on composite schedule of Rates

Part “A” (ROAD WORK)

| Item No. | Qty’s | Description of item to be executed at site | Rate | Unit | Amount in Rupees |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 |
| 01/- | 453000Cft: | Earth work for road embankment by bulldozers i/c plugging and mixing clod breaking dressing and compacting with optimum moisture content lead upto 100’ and lift upto 5.0’ in all types of soil except rock compacting upto 85% density as per modified AASHO specifications. | 3656/23 | P.0% Cft: | 1656272/= |
| 02/- | 48946Cft: | Earth work for road embankment by bulldozers i/c plugging and mixing clod breaking dressing and compacting with optimum moisture content lead upto 100’ and lift upto 5.0’ in all types of soil except rock compacting upto 95-100% density as per modified AASHO specifications. | 2529/77 | P.% Cft: | 123821/= |
| 03/- | 24473Cft: | Providing sand cushion i/c supplying & spreading pit/canal sand of approved quality from approved source of supply where ever available site of work i/c carriage site of work (R.A.A). | 1479/41 | P.% Cft: | 362056/= |
| 04/- | 24473Cft: | Providing Sub Base course by supplying and spreading stone metal 1 ½” gauge of approved quality from approved quarry in required thickness to proper camber and grade i/c hand packing mixing with 10’ screening & non plastic quarry of approved quality & gauge from approved source i/c watering and compacting to achieve 98-100% density as per modified AASHO specification ( rate i/cs all cost of material T&P and carriage upto site of wok.) | 9302/54 | P.% Cft: | 2276610/= |
| 05/- | 8158Rft: | Laying brick on end edging including supply of 9”x 4 ½”x3” 1st: class burnt bricks excavation for laying edging with small side parallel to the road. Rate includes all costs of materials T&P and carriage upto site of work. | 2834/16 | P.% Rft: | 231211/= |

Contd: P/2

P-2

| Item No. | Qty’s | Description of item to be executed at site | Rate | Unit | Amount in Rupees |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 |
| 06/- | 12236Cft: | Preparing base course supplying and spreading stone metal of approved quality of 6” in 2 layers from approved quarry properly grade to maximum size of 1½” in required thickness of proper camber & grade i/c S/S 15 Cft: screening & non plastic quarry fines filling & depressions with stone metal after initial rolling including watering & compacting the same so as to achieve 100% density as per modified AASHO specifications (This i/cs providing and using templates camber plates screen from as directed).Rate includes all costs of materials T&P and carriage upto site of work. | 10368/02 | P.% Cft: | 1268630/= |
| 07/- | 48946Sft: | Providing 1st: coat of surface dressing on or new existing surface with 30 lbs: bitumen and 4.0 cft: cursh bajri of required size including cleaning the road surface rolling etc: complete.Rate includes all costs of materials T&P & carriage. | 1694/60 | P.% Sft: | 829438/= |
| 08/- | 48946Sft: | Providing 1” thick consolidated premixed carpet in proper camber and grade i/c supply of 10 Cft: bajri 4.0 Cft: hill sand of approve quality and guage and bitumen 67 lbs: of 80/100 penetration i/c mixing the mechanical mixture in required proportion i/c heating the material and cleaning the road surface. Rate i/c all cost of material T&P and carriage upto site of work. | 4593/81 | P.% Sft: | 2248486/= |
|  |  |  | TOTAL |  | 8996524/= |

_____________% Above / Below on the rates of CSR.

Amount to be added /deducted on the

Total (A) 									Rs:

Total Amount in words  ____________________________________________________

__________________________________________________________________________

Executive Engineer,

Contractor						                           Highways Division

Kashmore @ Kandhkot

*Imran*

CONSTRUCTION OF ROAD FROM ABDUL GHAFOOR CHACHAR VIA AMIR BUX SOLANGI UPTO DILMURAD CHACHAR TALUKA KASHMORE

DRAFT BIDDING DOCUMENT FOR WORKS

BILLS OF QUANTITIES

(A) 	Description and rate of Items bases on composite schedule of Rates.

PART-B  (3.0 FT: SPAN CULVERTS).

| Item No. | Qty’s | Description of item to be executed at site | Rate | Unit | Amount in Rupees |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 |
| 01/- | 1106 Cft: | Excavation in foundation of building bridge other structure i/cs dag belling dressing refilling around the structure with excavated earth watering and remaining lead upto one chain and left upto 5.0’ ordinary soil. | 3176/25 | P.%0 Cft: | 3513/= |
| 02/- | 237 Cft: | Cement concrete with bricks are stone plaster 1½” to 2” guage ratio 1:2:4 | 9416/28 | P.% Cft: | 22518/= |
| 03/- | 502 Cft: | Pucca Brick work in foundation and plinth i/c cement sand mortor | 12501/41 | P.% Cft: | 62757/= |
| 04/- | 55 Cft: | Cement concrete plain i/c placing compacting finishing curing complete i/c screening washing of stone aggregate with out suhttering ratio 1:2:4: | 14429/25 | P.% Cft: | 7936/= |
| 05/- | 58 Sft: | Erection and removal of centering for R.C.C or plain cement work (2nd: class) for partal wood vertical | 7000/00 | P.% Sft: | 4060/= |
| 06/- | A. Tor Steel 4.16 Cwt B. M/ Steel 0.907 Cwt | Fabrication of m/s reinforcement cement concrete including cutting binding laying in position making joints & fastening of i/c binding wire also removal rust from bars. | 5001/70 4820/29 | P.Cwt: P.Cwt | 20807/= 4372/= |
| 07/- | 114 Rft: | Reinforced cement concrete work including all labour and material except the cost of steel reinforcement and its labour for bending and binding which will be paid separately. This rate also includes all kind of forms moulds lifting shuttering curing rend. | 337/00 | P.Rft: | 38418/= |
| 08/- | 330 Sft: | Cement Plaster (1:3) upto 20’ height 1½” thick. | 2283/93 | P.% Sft: | 7537/= |
|  |  |  | TOTAL |  | 171517/= |

Amount Total (b)

__________________________% Above / Below on the rates of CSR. 	Total (b) =

Amount to be added /deducted on the

basis of premium quoted Total (b)

Total (b) =  in works in figure

Executive Engineer,

Contractor						                           Highways Division

Kashmore @ Kandhkot

*Imran*

CONSTRUCTION OF ROAD FROM BADANI ROAD UPTO VILLAGE ALI MURAD DOMKI ROAD MILE 0/0-0/4+330’ TALUKA KASHMORE

DRAFT BIDDING DOCUMENT FOR WORKS

BILLS OF QUANTITIES

- Description and rate of Items bases on composite schedule of Rates

Part “A” (ROAD WORK)

| Item No. | Qty’s | Description of item to be executed at site | Rate | Unit | Amount in Rupees |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 |
| 01/- | 266000Cft: | Earth work for road embankment by bulldozers i/c plugging and mixing clod breaking dressing and compacting with optimum moisture content lead upto 100’ and lift upto 5.0’ in all types of soil except rock compacting upto 85% density as per modified AASHO specifications. | 3656/23 | P.0% Cft: | 972557/= |
| 02/- | 28552Cft: | Earth work for road embankment by bulldozers i/c plugging and mixing clod breaking dressing and compacting with optimum moisture content lead upto 100’ and lift upto 5.0’ in all types of soil except rock compacting upto 95-100% density as per modified AASHO specifications. | 2529/77 | P.% Cft: | 72229/= |
| 03/- | 14276Cft: | Providing sand cushion i/c supplying & spreading pit/canal sand of approved quality from approved source of supply where ever available site of work i/c carriage site of work (R.A.A). | 1479/41 | P.% Cft: | 211198/= |
| 04/- | 14276Cft: | Providing Sub Base course by supplying and spreading stone metal 1 ½” gauge of approved quality from approved quarry in required thickness to proper camber and grade i/c hand packing mixing with 10’ screening & non plastic quarry of approved quality & gauge from approved source i/c watering and compacting to achieve 98-100% density as per modified AASHO specification ( rate i/cs all cost of material T&P and carriage upto site of wok.) | 9209/42 | P.% Cft: | 1314736/= |
| 05/- | 4759Rft: | Laying brick on end edging including supply of 9”x 4 ½”x3” 1st: class burnt bricks excavation for laying edging with small side parallel to the road. Rate includes all costs of materials T&P and carriage upto site of work. | 2813/31 | P.% Rft: | 133885/= |

Contd: P/2

P-2

| Item No. | Qty’s | Description of item to be executed at site | Rate | Unit | Amount in Rupees |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 |
| 06/- | 7138Cft: | Preparing base course supplying and spreading stone metal of approved quality of 6” in 2 layers from approved quarry properly grade to maximum size of 1½” in required thickness of proper camber & grade i/c S/S 15 Cft: screening & non plastic quarry fines filling & depressions with stone metal after initial rolling including watering & compacting the same so as to achieve 100% density as per modified AASHO specifications (This i/cs providing and using templates camber plates screen from as directed).Rate includes all costs of materials T&P and carriage upto site of work. | 10271/63 | P.% Cft: | 733188/= |
| 07/- | 28552Sft: | Providing 1st: coat of surface dressing on or new existing surface with 30 lbs: bitumen and 4.0 cft: cursh bajri of required size including cleaning the road surface rolling etc: complete.Rate includes all costs of materials T&P & carriage. | 1691/74 | P.% Sft: | 483025/= |
| 08/- | 28552Sft: | Providing 1” thick consolidated premixed carpet in proper camber and grade i/c supply of 10 Cft: bajri 4.0 Cft: hill sand of approve quality and guage and bitumen 67 lbs: of 80/100 penetration i/c mixing the mechanical mixture in required proportion i/c heating the material and cleaning the road surface. Rate i/c all cost of material T&P and carriage upto site of work. | 4584/10 | P.% Sft: | 1308852/= |
|  |  |  | TOTAL |  | 5229670/= |

_____________% Above / Below on the rates of CSR.

Amount to be added /deducted on the

Total (A) 									Rs:

Total Amount in words  ____________________________________________________

__________________________________________________________________________

Executive Engineer,

Contractor						                           Highways Division

Kashmore @ Kandhkot

*Imran*

CONSTRUCTION OF ROAD FROM BADANI ROAD UPTO VILLAGE ALI MURAD DOMKI ROAD MILE 0/0-0/4+330’ TALUKA KASHMORE

DRAFT BIDDING DOCUMENT FOR WORKS

BILLS OF QUANTITIES

(A) 	Description and rate of Items bases on composite schedule of Rates.

PART-B  (3.0 FT: SPAN CULVERTS).

| Item No. | Qty’s | Description of item to be executed at site | Rate | Unit | Amount in Rupees |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 |
| 01/- | 1106 Cft: | Excavation in foundation of building bridge other structure i/cs dag belling dressing refilling around the structure with excavated earth watering and remaining lead upto one chain and left upto 5.0’ ordinary soil. | 3176/25 | P.%0 Cft: | 3513/= |
| 02/- | 237 Cft: | Cement concrete with bricks are stone plaster 1½” to 2” guage ratio 1:2:4 | 9416/28 | P.% Cft: | 22517/= |
| 03/- | 502 Cft: | Pucca Brick work in foundation and plinth i/c cement sand mortor | 12501/41 | P.% Cft: | 62757/= |
| 04/- | 55 Cft: | Cement concrete plain i/c placing compacting finishing curing complete i/c screening washing of stone aggregate with out suhttering ratio 1:2:4: | 14429/25 | P.% Cft: | 7936/= |
| 05/- | 58 Sft: | Erection and removal of centering for R.C.C or plain cement work (2nd: class) for partal wood vertical | 7000/00 | P.% Sft: | 4060/= |
| 06/- | A. Tor Steel 4.16 Cwt B. M/ Steel 0.907 Cwt | Fabrication of m/s reinforcement cement concrete including cutting binding laying in position making joints & fastening of i/c binding wire also removal rust from bars. | 5001/70 4820/29 | P.Cwt: P.Cwt | 20807/= 4372/= |
| 07/- | 114 Rft: | Reinforced cement concrete work including all labour and material except the cost of steel reinforcement and its labour for bending and binding which will be paid separately. This rate also includes all kind of forms moulds lifting shuttering curing rend. | 337/00 | P.Rft: | 38418/= |
| 08/- | 330 Sft: | Cement Plaster (1:3) upto 20’ height 1½” thick. | 2283/93 | P.% Sft: | 7537/= |
|  |  |  | TOTAL |  | 171751/= |

Amount Total (b)

__________________________% Above / Below on the rates of CSR. 	Total (b) =

Amount to be added /deducted on the

basis of premium quoted Total (b)

Total (b) =  in works in figure

Executive Engineer,

Contractor						                           Highways Division

Kashmore @ Kandhkot

*Imran*

CONSTRUCTION OF ROAD FROM SUI LINE PURANI ADIO WAH VIA WADHAYO MAZARI TALUKA KASHMORE

DRAFT BIDDING DOCUMENT FOR WORKS

BILLS OF QUANTITIES

- Description and rate of Items bases on composite schedule of Rates

Part “A” (ROAD WORK)

| Item No. | Qty’s | Description of item to be executed at site | Rate | Unit | Amount in Rupees |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 |
| 01/- | 350000Cft: | Earth work for road embankment by bulldozers i/c plugging and mixing clod breaking dressing and compacting with optimum moisture content lead upto 100’ and lift upto 5.0’ in all types of soil except rock compacting upto 85% density as per modified AASHO specifications. | 3656/23 | P.0% Cft: | 1279681/= |
| 02/- | 40788Cft: | Earth work for road embankment by bulldozers i/c plugging and mixing clod breaking dressing and compacting with optimum moisture content lead upto 100’ and lift upto 5.0’ in all types of soil except rock compacting upto 95-100% density as per modified AASHO specifications. | 2529/77 | P.% Cft: | 103184/= |
| 03/- | 20394Cft: | Providing sand cushion i/c supplying & spreading pit/canal sand of approved quality from approved source of supply where ever available site of work i/c carriage site of work (R.A.A). | 1479/41 | P.% Cft: | 301711/= |
| 04/- | 20394Cft: | Providing Sub Base course by supplying and spreading stone metal 1 ½” gauge of approved quality from approved quarry in required thickness to proper camber and grade i/c hand packing mixing with 10’ screening & non plastic quarry of approved quality & gauge from approved source i/c watering and compacting to achieve 98-100% density as per modified AASHO specification ( rate i/cs all cost of material T&P and carriage upto site of wok.) | 10000/95 | P.% Cft: | 2039593/= |
| 05/- | 6798Rft: | Laying brick on end edging including supply of 9”x 4 ½”x3” 1st: class burnt bricks excavation for laying edging with small side parallel to the road. Rate includes all costs of materials T&P and carriage upto site of work. | 2990/46 | P.% Rft: | 203291/= |

Contd: P/2

P-2

| Item No. | Qty’s | Description of item to be executed at site | Rate | Unit | Amount in Rupees |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 |
| 06/- | 10197Cft: | Preparing base course supplying and spreading stone metal of approved quality of 6” in 2 layers from approved quarry properly grade to maximum size of 1½” in required thickness of proper camber & grade i/c S/S 15 Cft: screening & non plastic quarry fines filling & depressions with stone metal after initial rolling including watering & compacting the same so as to achieve 100% density as per modified AASHO specifications (This i/cs providing and using templates camber plates screen from as directed).Rate includes all costs of materials T&P and carriage upto site of work. | 11090/85 | P.% Cft: | 1130934/= |
| 07/- | 40788Sft: | Providing 1st: coat of surface dressing on or new existing surface with 30 lbs: bitumen and 4.0 cft: cursh bajri of required size including cleaning the road surface rolling etc: complete.Rate includes all costs of materials T&P & carriage. | 1716/11 | P.% Sft: | 699967/= |
| 08/- | 40788Sft: | Providing 1” thick consolidated premixed carpet in proper camber and grade i/c supply of 10 Cft: bajri 4.0 Cft: hill sand of approve quality and guage and bitumen 67 lbs: of 80/100 penetration i/c mixing the mechanical mixture in required proportion i/c heating the material and cleaning the road surface. Rate i/c all cost of material T&P and carriage upto site of work. | 4627/49 | P.% Sft: | 1887461/= |
|  |  |  | TOTAL |  | 7645822/= |

_____________% Above / Below on the rates of CSR.

Amount to be added /deducted on the

Total (A) 									Rs:

Total Amount in words  ____________________________________________________

__________________________________________________________________________

Executive Engineer,

Contractor						                           Highways Division

Kashmore @ Kandhkot

*Imran*

CONSTRUCTION OF ROAD FROM SUI LINE PURANI ADIO WAH VIA WADHAYO MAZARI TALUKA KASHMORE

DRAFT BIDDING DOCUMENT FOR WORKS

BILLS OF QUANTITIES

(A) 	Description and rate of Items bases on composite schedule of Rates.

PART-B  (3.0 FT: SPAN CULVERTS).

| Item No. | Qty’s | Description of item to be executed at site | Rate | Unit | Amount in Rupees |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 |
| 01/- | 1106 Cft: | Excavation in foundation of building bridge other structure i/cs dag belling dressing refilling around the structure with excavated earth watering and remaining lead upto one chain and left upto 5.0’ ordinary soil. | 3176/25 | P.%0 Cft: | 3512/= |
| 02/- | 237 Cft: | Cement concrete with bricks are stone plaster 1½” to 2” guage ratio 1:2:4 | 9416/28 | P.% Cft: | 22518/= |
| 03/- | 502 Cft: | Pucca Brick work in foundation and plinth i/c cement sand mortor | 12501/41 | P.% Cft: | 62757/= |
| 04/- | 55 Cft: | Cement concrete plain i/c placing compacting finishing curing complete i/c screening washing of stone aggregate with out suhttering ratio 1:2:4: | 14429/25 | P.% Cft: | 7936/= |
| 05/- | 58 Sft: | Erection and removal of centering for R.C.C or plain cement work (2nd: class) for partal wood vertical | 7000/00 | P.% Sft: | 4060/= |
| 06/- | A. Tor Steel 4.16 Cwt B. M/ Steel 0.907 Cwt | Fabrication of m/s reinforcement cement concrete including cutting binding laying in position making joints & fastening of i/c binding wire also removal rust from bars. | 5001/70 4820/29 | P.Cwt: P.Cwt | 20807/= 4372/= |
| 07/- | 114 Rft: | Reinforced cement concrete work including all labour and material except the cost of steel reinforcement and its labour for bending and binding which will be paid separately. This rate also includes all kind of forms moulds lifting shuttering curing rend. | 337/00 | P.Rft: | 38418/= |
| 08/- | 330 Sft: | Cement Plaster (1:3) upto 20’ height 1½” thick. | 2283/93 | P.% Sft: | 7537/= |
|  |  |  | TOTAL |  | 171517/= |

Amount Total (b)

__________________________% Above / Below on the rates of CSR. 	Total (b) =

Amount to be added /deducted on the

basis of premium quoted Total (b)

Total (b) =  in works in figure

Executive Engineer,

Contractor						                           Highways Division

Kashmore @ Kandhkot

*Imran*

CONSTRUCTION OF ROAD FROM MUHAMMAD SALLAH KHAN GOLO UPTO PPL ROAD MILE 0/0-1/1  TALUKA KASHMORE

DRAFT BIDDING DOCUMENT FOR WORKS

BILLS OF QUANTITIES

- Description and rate of Items bases on composite schedule of Rates

Part “A” (ROAD WORK)

| Item No. | Qty’s | Description of item to be executed at site | Rate | Unit | Amount in Rupees |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 |
| 01/- | 680000Cft: | Earth work for road embankment by bulldozers i/c plugging and mixing clod breaking dressing and compacting with optimum moisture content lead upto 100’ and lift upto 5.0’ in all types of soil except rock compacting upto 85% density as per modified AASHO specifications. | 3656/23 | P.0% Cft: | 2486236/= |
| 02/- | 73418Cft: | Earth work for road embankment by bulldozers i/c plugging and mixing clod breaking dressing and compacting with optimum moisture content lead upto 100’ and lift upto 5.0’ in all types of soil except rock compacting upto 95-100% density as per modified AASHO specifications. | 2529/77 | P.% Cft: | 185732/= |
| 03/- | 36709Cft: | Providing sand cushion i/c supplying & spreading pit/canal sand of approved quality from approved source of supply where ever available site of work i/c carriage site of work (R.A.A). | 1479/41 | P.% Cft: | 543077/= |
| 04/- | 36709Cft: | Providing Sub Base course by supplying and spreading stone metal 1 ½” gauge of approved quality from approved quarry in required thickness to proper camber and grade i/c hand packing mixing with 10’ screening & non plastic quarry of approved quality & gauge from approved source i/c watering and compacting to achieve 98-100% density as per modified AASHO specification ( rate i/cs all cost of material T&P and carriage upto site of wok.) | 8883/50 | P.% Cft: | 3261044/= |
| 05/- | 12236Rft: | Laying brick on end edging including supply of 9”x 4 ½”x3” 1st: class burnt bricks excavation for laying edging with small side parallel to the road. Rate includes all costs of materials T&P and carriage upto site of work. | 2740/37 | P.% Rft: | 335312/= |

Contd: P/2

P-2

| Item No. | Qty’s | Description of item to be executed at site | Rate | Unit | Amount in Rupees |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 |
| 06/- | 18335Cft: | Preparing base course supplying and spreading stone metal of approved quality of 6” in 2 layers from approved quarry properly grade to maximum size of 1½” in required thickness of proper camber & grade i/c S/S 15 Cft: screening & non plastic quarry fines filling & depressions with stone metal after initial rolling including watering & compacting the same so as to achieve 100% density as per modified AASHO specifications (This i/cs providing and using templates camber plates screen from as directed).Rate includes all costs of materials T&P and carriage upto site of work. | 9934/32 | P.% Cft: | 1823444/= |
| 07/- | 73418Sft: | Providing 1st: coat of surface dressing on or new existing surface with 30 lbs: bitumen and 4.0 cft: cursh bajri of required size including cleaning the road surface rolling etc: complete.Rate includes all costs of materials T&P & carriage. | 1683/13 | P.% Sft: | 1235720/= |
| 08/- | 73418Sft: | Providing 1” thick consolidated premixed carpet in proper camber and grade i/c supply of 10 Cft: bajri 4.0 Cft: hill sand of approve quality and guage and bitumen 67 lbs: of 80/100 penetration i/c mixing the mechanical mixture in required proportion i/c heating the material and cleaning the road surface. Rate i/c all cost of material T&P and carriage upto site of work. | 4573/23 | P.% Sft: | 3357574/= |
|  |  |  | TOTAL |  | 13228139/= |

_____________% Above / Below on the rates of CSR.

Amount to be added /deducted on the

Total (A) 									Rs:

Total Amount in words  ____________________________________________________

__________________________________________________________________________

Executive Engineer,

Contractor						                           Highways Division

Kashmore @ Kandhkot

*Imran*

CONSTRUCTION OF ROAD FROM MUHAMMAD SALLAH KHAN GOLO UPTO PPL ROAD MILE 0/0-1/1  TALUKA KASHMORE

DRAFT BIDDING DOCUMENT FOR WORKS

BILLS OF QUANTITIES

(A) 	Description and rate of Items bases on composite schedule of Rates.

PART-B  (3.0 FT: SPAN CULVERTS).

| Item No. | Qty’s | Description of item to be executed at site | Rate | Unit | Amount in Rupees |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 |
| 01/- | 1106 Cft: | Excavation in foundation of building bridge other structure i/cs dag belling dressing refilling around the structure with excavated earth watering and remaining lead upto one chain and left upto 5.0’ ordinary soil. | 3176/25 | P.%0 Cft: | 3512/= |
| 02/- | 237 Cft: | Cement concrete with bricks are stone plaster 1½” to 2” guage ratio 1:2:4 | 9416/28 | P.% Cft: | 22518/= |
| 03/- | 502 Cft: | Pucca Brick work in foundation and plinth i/c cement sand mortor | 12501/41 | P.% Cft: | 62757/= |
| 04/- | 55 Cft: | Cement concrete plain i/c placing compacting finishing curing complete i/c screening washing of stone aggregate with out suhttering ratio 1:2:4: | 14429/25 | P.% Cft: | 7936/= |
| 05/- | 58 Sft: | Erection and removal of centering for R.C.C or plain cement work (2nd: class) for partal wood vertical | 7000/00 | P.% Sft: | 4060/= |
| 06/- | A. Tor Steel 4.16 Cwt B. M/ Steel 0.907 Cwt | Fabrication of m/s reinforcement cement concrete including cutting binding laying in position making joints & fastening of i/c binding wire also removal rust from bars. | 5001/70 4820/29 | P.Cwt: P.Cwt | 20807/= 4372/= |
| 07/- | 114 Rft: | Reinforced cement concrete work including all labour and material except the cost of steel reinforcement and its labour for bending and binding which will be paid separately. This rate also includes all kind of forms moulds lifting shuttering curing rend. | 337/00 | P.Rft: | 38418/= |
| 08/- | 330 Sft: | Cement Plaster (1:3) upto 20’ height 1½” thick. | 2283/93 | P.% Sft: | 7537/= |
|  |  |  | TOTAL |  | 171517/= |

Amount Total (b)

__________________________% Above / Below on the rates of CSR. 	Total (b) =

Amount to be added /deducted on the

basis of premium quoted Total (b)

Total (b) =  in works in figure

Executive Engineer,

Contractor						                           Highways Division

Kashmore @ Kandhkot

*Imran*

CONSTRUCTION OF ROAD FROM SUI LINE PURANI TO VILLAGE KHUDA BUX KHOSO TALUKA KASHMORE MILE 0/0-0/3

DRAFT BIDDING DOCUMENT FOR WORKS

BILLS OF QUANTITIES

- Description and rate of Items bases on composite schedule of Rates

Part “A” (ROAD WORK)

| Item No. | Qty’s | Description of item to be executed at site | Rate | Unit | Amount in Rupees |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 |
| 01/- | 150000Cft: | Earth work for road embankment by bulldozers i/c plugging and mixing clod breaking dressing and compacting with optimum moisture content lead upto 100’ and lift upto 5.0’ in all types of soil except rock compacting upto 85% density as per modified AASHO specifications. | 3656/23 | P.0% Cft: | 548435/= |
| 02/- | 28551Cft: | Earth work for road embankment by bulldozers i/c plugging and mixing clod breaking dressing and compacting with optimum moisture content lead upto 100’ and lift upto 5.0’ in all types of soil except rock compacting upto 95-100% density as per modified AASHO specifications. | 2529/77 | P.% Cft: | 72113/= |
| 03/- | 14276Cft: | Providing sand cushion i/c supplying & spreading pit/canal sand of approved quality from approved source of supply where ever available site of work i/c carriage site of work (R.A.A). | 1479/41 | P.% Cft: | 211201/= |
| 04/- | 14276Cft: | Providing Sub Base course by supplying and spreading stone metal 1 ½” gauge of approved quality from approved quarry in required thickness to proper camber and grade i/c hand packing mixing with 10’ screening & non plastic quarry of approved quality & gauge from approved source i/c watering and compacting to achieve 98-100% density as per modified AASHO specification ( rate i/cs all cost of material T&P and carriage upto site of wok.) | 10094/07 | P.% Cft: | 1441029/= |
| 05/- | 4759Rft: | Laying brick on end edging including supply of 9”x 4 ½”x3” 1st: class burnt bricks excavation for laying edging with small side parallel to the road. Rate includes all costs of materials T&P and carriage upto site of work. | 3323/89 | P.% Rft: | 158183/= |

Contd: P/2

P-2

| Item No. | Qty’s | Description of item to be executed at site | Rate | Unit | Amount in Rupees |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 |
| 06/- | 7138Cft: | Preparing base course supplying and spreading stone metal of approved quality of 6” in 2 layers from approved quarry properly grade to maximum size of 1½” in required thickness of proper camber & grade i/c S/S 15 Cft: screening & non plastic quarry fines filling & depressions with stone metal after initial rolling including watering & compacting the same so as to achieve 100% density as per modified AASHO specifications (This i/cs providing and using templates camber plates screen from as directed).Rate includes all costs of materials T&P and carriage upto site of work. | 11187/22 | P.% Cft: | 798544/= |
| 07/- | 28552Sft: | Providing 1st: coat of surface dressing on or new existing surface with 30 lbs: bitumen and 4.0 cft: cursh bajri of required size including cleaning the road surface rolling etc: complete.Rate includes all costs of materials T&P & carriage. | 1718/96 | P.% Sft: | 490797/= |
| 08/- | 28552Sft: | Providing 1” thick consolidated premixed carpet in proper camber and grade i/c supply of 10 Cft: bajri 4.0 Cft: hill sand of approve quality and guage and bitumen 67 lbs: of 80/100 penetration i/c mixing the mechanical mixture in required proportion i/c heating the material and cleaning the road surface. Rate i/c all cost of material T&P and carriage upto site of work. | 4660/64 | P.% Sft: | 1330706/= |
|  |  |  | TOTAL |  | 4970008/= |

_____________% Above / Below on the rates of CSR.

Amount to be added /deducted on the

Total (A) 									Rs:

Total Amount in words  ____________________________________________________

__________________________________________________________________________

Executive Engineer,

Contractor						                           Highways Division

Kashmore @ Kandhkot

*Imran*

CONSTRUCTION OF ROAD FROM SUI LINE PURANI TO VILLAGE KHUDA BUX KHOSO TALUKA KASHMORE MILE 0/0-0/3

DRAFT BIDDING DOCUMENT FOR WORKS

BILLS OF QUANTITIES

(A) 	Description and rate of Items bases on composite schedule of Rates.

PART-B  (3.0 FT: SPAN CULVERTS).

| Item No. | Qty’s | Description of item to be executed at site | Rate | Unit | Amount in Rupees |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 |
| 01/- | 1106 Cft: | Excavation in foundation of building bridge other structure i/cs dag belling dressing refilling around the structure with excavated earth watering and remaining lead upto one chain and left upto 5.0’ ordinary soil. | 3176/25 | P.%0 Cft: | 3512/= |
| 02/- | 237 Cft: | Cement concrete with bricks are stone plaster 1½” to 2” guage ratio 1:2:4 | 9416/28 | P.% Cft: | 22518/= |
| 03/- | 502 Cft: | Pucca Brick work in foundation and plinth i/c cement sand mortor | 12501/41 | P.% Cft: | 62757/= |
| 04/- | 55 Cft: | Cement concrete plain i/c placing compacting finishing curing complete i/c screening washing of stone aggregate with out suhttering ratio 1:2:4: | 14429/25 | P.% Cft: | 7936/= |
| 05/- | 58 Sft: | Erection and removal of centering for R.C.C or plain cement work (2nd: class) for partal wood vertical | 7000/00 | P.% Sft: | 4060/= |
| 06/- | A. Tor Steel 4.16 Cwt B. M/ Steel 0.907 Cwt | Fabrication of m/s reinforcement cement concrete including cutting binding laying in position making joints & fastening of i/c binding wire also removal rust from bars. | 5001/70 4820/29 | P.Cwt: P.Cwt | 20807/= 4372/= |
| 07/- | 114 Rft: | Reinforced cement concrete work including all labour and material except the cost of steel reinforcement and its labour for bending and binding which will be paid separately. This rate also includes all kind of forms moulds lifting shuttering curing rend. | 337/00 | P.Rft: | 38418/= |
| 08/- | 330 Sft: | Cement Plaster (1:3) upto 20’ height 1½” thick. | 2283/93 | P.% Sft: | 7537/= |
|  |  |  | TOTAL |  | 171751/= |

Amount Total (b)

__________________________% Above / Below on the rates of CSR. 	Total (b) =

Amount to be added /deducted on the

basis of premium quoted Total (b)

Total (b) =  in works in figure

Executive Engineer,

Contractor						                           Highways Division

Kashmore @ Kandhkot

*Imran*

CONSTRUCTION OF ROAD FROM GULSHER JAKHRANI ROAD UPTO VILLAGE AHMED NAWAZ JAKHRANI ROAD MILE 0/0-0/6  TALUKA KASHMORE

DRAFT BIDDING DOCUMENT FOR WORKS

BILLS OF QUANTITIES

- Description and rate of Items bases on composite schedule of Rates

Part “A” (ROAD WORK)

| Item No. | Qty’s | Description of item to be executed at site | Rate | Unit | Amount in Rupees |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 |
| 01/- | 470000Cft: | Earth work for road embankment by bulldozers i/c plugging and mixing clod breaking dressing and compacting with optimum moisture content lead upto 100’ and lift upto 5.0’ in all types of soil except rock compacting upto 85% density as per modified AASHO specifications. | 3656/23 | P.0% Cft: | 1718428/= |
| 02/- | 49946Cft: | Earth work for road embankment by bulldozers i/c plugging and mixing clod breaking dressing and compacting with optimum moisture content lead upto 100’ and lift upto 5.0’ in all types of soil except rock compacting upto 95-100% density as per modified AASHO specifications. | 2529/77 | P.% Cft: | 123822/= |
| 03/- | 24473Cft: | Providing sand cushion i/c supplying & spreading pit/canal sand of approved quality from approved source of supply where ever available site of work i/c carriage site of work (R.A.A). | 1479/41 | P.% Cft: | 362056/= |
| 04/- | 24473Cft: | Providing Sub Base course by supplying and spreading stone metal 1 ½” gauge of approved quality from approved quarry in required thickness to proper camber and grade i/c hand packing mixing with 10’ screening & non plastic quarry of approved quality & gauge from approved source i/c watering and compacting to achieve 98-100% density as per modified AASHO specification ( rate i/cs all cost of material T&P and carriage upto site of wok.) | 9116/30 | P.% Cft: | 2231032/= |
| 05/- | 8158Rft: | Laying brick on end edging including supply of 9”x 4 ½”x3” 1st: class burnt bricks excavation for laying edging with small side parallel to the road. Rate includes all costs of materials T&P and carriage upto site of work. | 2792/47 | P.% Rft: | 227810/= |

Contd: P/2

P-2

| Item No. | Qty’s | Description of item to be executed at site | Rate | Unit | Amount in Rupees |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 |
| 06/- | 12236Cft: | Preparing base course supplying and spreading stone metal of approved quality of 6” in 2 layers from approved quarry properly grade to maximum size of 1½” in required thickness of proper camber & grade i/c S/S 15 Cft: screening & non plastic quarry fines filling & depressions with stone metal after initial rolling including watering & compacting the same so as to achieve 100% density as per modified AASHO specifications (This i/cs providing and using templates camber plates screen from as directed).Rate includes all costs of materials T&P and carriage upto site of work. | 10175/26 | P.% Cft: | 1245045/= |
| 07/- | 48946Sft: | Providing 1st: coat of surface dressing on or new existing surface with 30 lbs: bitumen and 4.0 cft: cursh bajri of required size including cleaning the road surface rolling etc: complete.Rate includes all costs of materials T&P & carriage. | 1688/86 | P.% Sft: | 826629/= |
| 08/- | 48946Sft: | Providing 1” thick consolidated premixed carpet in proper camber and grade i/c supply of 10 Cft: bajri 4.0 Cft: hill sand of approve quality and guage and bitumen 67 lbs: of 80/100 penetration i/c mixing the mechanical mixture in required proportion i/c heating the material and cleaning the road surface. Rate i/c all cost of material T&P and carriage upto site of work. | 4592/63 | P.% Sft: | 2247909/= |
|  |  |  | TOTAL |  | 8982731/= |

_____________% Above / Below on the rates of CSR.

Amount to be added /deducted on the

Total (A) 									Rs:

Total Amount in words  ____________________________________________________

__________________________________________________________________________

Executive Engineer,

Contractor						                           Highways Division

Kashmore @ Kandhkot

*Imran*

CONSTRUCTION OF ROAD FROM GULSHER JAKHRANI ROAD UPTO VILLAGE AHMED NAWAZ JAKHRANI ROAD MILE 0/0-0/6  TALUKA KASHMORE

DRAFT BIDDING DOCUMENT FOR WORKS

BILLS OF QUANTITIES

(A) 	Description and rate of Items bases on composite schedule of Rates.

PART-B  (3.0 FT: SPAN CULVERTS).

| Item No. | Qty’s | Description of item to be executed at site | Rate | Unit | Amount in Rupees |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 |
| 01/- | 1106 Cft: | Excavation in foundation of building bridge other structure i/cs dag belling dressing refilling around the structure with excavated earth watering and remaining lead upto one chain and left upto 5.0’ ordinary soil. | 3176/25 | P.%0 Cft: | 3512/= |
| 02/- | 237 Cft: | Cement concrete with bricks are stone plaster 1½” to 2” guage ratio 1:2:4 | 9416/28 | P.% Cft: | 22518/= |
| 03/- | 502 Cft: | Pucca Brick work in foundation and plinth i/c cement sand mortor | 12501/41 | P.% Cft: | 62757/= |
| 04/- | 55 Cft: | Cement concrete plain i/c placing compacting finishing curing complete i/c screening washing of stone aggregate with out suhttering ratio 1:2:4: | 14429/25 | P.% Cft: | 7936/= |
| 05/- | 58 Sft: | Erection and removal of centering for R.C.C or plain cement work (2nd: class) for partal wood vertical | 7000/00 | P.% Sft: | 4060/= |
| 06/- | A. Tor Steel 4.16 Cwt B. M/ Steel 0.907 Cwt | Fabrication of m/s reinforcement cement concrete including cutting binding laying in position making joints & fastening of i/c binding wire also removal rust from bars. | 5001/70 4820/29 | P.Cwt: P.Cwt | 20807/= 4372/= |
| 07/- | 114 Rft: | Reinforced cement concrete work including all labour and material except the cost of steel reinforcement and its labour for bending and binding which will be paid separately. This rate also includes all kind of forms moulds lifting shuttering curing rend. | 337/00 | P.Rft: | 38418/= |
| 08/- | 330 Sft: | Cement Plaster (1:3) upto 20’ height 1½” thick. | 2283/93 | P.% Sft: | 7537/= |
|  |  |  | TOTAL |  | 171517/= |

Amount Total (b)

__________________________% Above / Below on the rates of CSR. 	Total (b) =

Amount to be added /deducted on the

basis of premium quoted Total (b)

Total (b) =  in works in figure

Executive Engineer,

Contractor						                           Highways Division

Kashmore @ Kandhkot

CONSTRUCTION OF BRIDGE OVER KANDHKOT MINOR RAJ WAH NEAR VILLAGE MUHAMMAD NAWAZ BANGWAR TALUKA KASHMORE

DRAFT BIDDING DOCUMENT FOR WORKS

BILLS OF QUANTITIES

(A) 	Description and rate of Items bases on composite schedule of Rates.

PART-A  (BRIDGE WORK).

| Item No. | Qty’s |  | Description of item to be executed at site | Rate | Unit | Amount in Rupees |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 |  | 3 | 4 | 5 | 6 |
| 01/- | 5412 Cft: |  | Excavation in foundation of building bridge other structure i/cs dag belling dressing refilling around the structure with excavated earth watering and remaining lead upto one chain and left upto 5.0’ ordinary soil. | 3176/25 | P.%0 Cft: | 17190/= |
| 02/- | 1304Cft: |  | Cement concrete with bricks are stone plaster 1½” to 2” guage ratio 1:2:4 | 9416/28 | P.% Cft: | 122788/= |
| 03/- | 3123Cft: |  | Pucca Brick work in foundation and plinth i/c cement sand mortor | 12501/41 | P.% Cft: | 390419/= |
| 04/- | 256Cft: |  | Cement concrete plain i/c placing compacting finishing curing complete i/c screening washing of stone aggregate with out suhttering ratio 1:2:4: | 14429/25 | P.% Cft: | 36939/= |
| 05/- | 92 Sft: |  | Erection and removal of centering for R.C.C or plain cement work (2nd: class) for partal wood vertical | 7000/00 | P.% Sft: | 6440/= |
| 06/- | 70.62 Cwt |  | Fabrication of m/s reinforcement cement concrete including cutting binding laying in position making joints & fastening of i/c binding wire also removal rust from bars. | 4820/20 | P.Cwt | 340402/= |
| 07/- | 1340Rft: |  | Reinforced cement concrete work including all labour and material except the cost of steel reinforcement and its labour for bending and binding which will be paid separately. This rate also includes all kind of forms moulds lifting shuttering curing rend. | 337/00 | P.Rft: | 451580/= |
| 08/- | 2020Sft: |  | Cement Plaster (1:3) upto 20’ height 1½” thick. | 2283/93 | P.% Sft: | 46135/= |
| 09/- |  | 54810Cft: | Earth work on road embankment by bulldozers i/c plugging mixing clod breaking dressing and compcting with optimum moisture content lead upto 100’ and lift upto 5’ in all types of soil | 7327/99 | P.%0 Cft: | 401647/= |
|  |  |  |  | TOTAL |  | 1813539/= |

Amount Total (a)

__________________________% Above / Below on the rates of CSR. 	Total (a) =

Amount to be added /deducted on the

basis of premium quoted Total (a)

Total (a) =  in works in figure

Executive Engineer,

Contractor						                           Highways Division

Kashmore @ Kandhkot

*Imran*

CONSTRUCTION OF BRIDGE NEAR VILLAGE SARDAR MUHAMMADI TALUKA KASHMORE

DRAFT BIDDING DOCUMENT FOR WORKS

BILLS OF QUANTITIES

(A) 	Description and rate of Items bases on composite schedule of Rates.

PART-A  (BRIDGE WORK).

| Item No. | Qty’s |  | Description of item to be executed at site | Rate | Unit | Amount in Rupees |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 |  | 3 | 4 | 5 | 6 |
| 01/- | 5412 Cft: |  | Excavation in foundation of building bridge other structure i/cs dag belling dressing refilling around the structure with excavated earth watering and remaining lead upto one chain and left upto 5.0’ ordinary soil. | 3176/25 | P.%0 Cft: | 17190/= |
| 02/- | 1304Cft: |  | Cement concrete with bricks are stone plaster 1½” to 2” guage ratio 1:2:4 | 9416/28 | P.% Cft: | 122788/= |
| 03/- | 3123Cft: |  | Pucca Brick work in foundation and plinth i/c cement sand mortor | 12501/41 | P.% Cft: | 390419/= |
| 04/- | 256Cft: |  | Cement concrete plain i/c placing compacting finishing curing complete i/c screening washing of stone aggregate with out suhttering ratio 1:2:4: | 14429/25 | P.% Cft: | 36939/= |
| 05/- | 92 Sft: |  | Erection and removal of centering for R.C.C or plain cement work (2nd: class) for partal wood vertical | 7000/00 | P.% Sft: | 6440/= |
| 06/- | 70.62 Cwt |  | Fabrication of m/s reinforcement cement concrete including cutting binding laying in position making joints & fastening of i/c binding wire also removal rust from bars. | 4820/20 | P.Cwt | 340402/= |
| 07/- | 1340Rft: |  | Reinforced cement concrete work including all labour and material except the cost of steel reinforcement and its labour for bending and binding which will be paid separately. This rate also includes all kind of forms moulds lifting shuttering curing rend. | 337/00 | P.Rft: | 451580/= |
| 08/- | 2020Sft: |  | Cement Plaster (1:3) upto 20’ height 1½” thick. | 2283/93 | P.% Sft: | 46135/= |
| 09/- |  | 54810Cft: | Earth work on road embankment by bulldozers i/c plugging mixing clod breaking dressing and compcting with optimum moisture content lead upto 100’ and lift upto 5’ in all types of soil | 7327/99 | P.%0 Cft: | 401647/= |
|  |  |  |  | TOTAL |  | 1813539/= |

Amount Total (a)

__________________________% Above / Below on the rates of CSR. 	Total (a) =

Amount to be added /deducted on the

basis of premium quoted Total (a)

Total (a) =  in works in figure

Executive Engineer,

Contractor						                           Highways Division

Kashmore @ Kandhkot

*Imran*

CONSTRUCTION OF BRIDGE OVER KANDHKOT SHAKH NEAR VILLAGE HAIBAT GOLO TALUKA KASHMORE

DRAFT BIDDING DOCUMENT FOR WORKS

BILLS OF QUANTITIES

(A) 	Description and rate of Items bases on composite schedule of Rates.

PART-A  (BRIDGE WORK).

| Item No. | Qty’s |  | Description of item to be executed at site | Rate | Unit | Amount in Rupees |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 |  | 3 | 4 | 5 | 6 |
| 01/- | 5412 Cft: |  | Excavation in foundation of building bridge other structure i/cs dag belling dressing refilling around the structure with excavated earth watering and remaining lead upto one chain and left upto 5.0’ ordinary soil. | 3176/25 | P.%0 Cft: | 17190/= |
| 02/- | 1304Cft: |  | Cement concrete with bricks are stone plaster 1½” to 2” guage ratio 1:2:4 | 9416/28 | P.% Cft: | 122788/= |
| 03/- | 3123Cft: |  | Pucca Brick work in foundation and plinth i/c cement sand mortor | 12501/41 | P.% Cft: | 390419/= |
| 04/- | 256Cft: |  | Cement concrete plain i/c placing compacting finishing curing complete i/c screening washing of stone aggregate with out suhttering ratio 1:2:4: | 14429/25 | P.% Cft: | 36939/= |
| 05/- | 92 Sft: |  | Erection and removal of centering for R.C.C or plain cement work (2nd: class) for partal wood vertical | 7000/00 | P.% Sft: | 6440/= |
| 06/- | 70.62 Cwt |  | Fabrication of m/s reinforcement cement concrete including cutting binding laying in position making joints & fastening of i/c binding wire also removal rust from bars. | 4820/20 | P.Cwt | 340402/= |
| 07/- | 1340Rft: |  | Reinforced cement concrete work including all labour and material except the cost of steel reinforcement and its labour for bending and binding which will be paid separately. This rate also includes all kind of forms moulds lifting shuttering curing rend. | 337/00 | P.Rft: | 451580/= |
| 08/- | 2020Sft: |  | Cement Plaster (1:3) upto 20’ height 1½” thick. | 2283/93 | P.% Sft: | 46135/= |
| 09/- |  | 54810Cft: | Earth work on road embankment by bulldozers i/c plugging mixing clod breaking dressing and compcting with optimum moisture content lead upto 100’ and lift upto 5’ in all types of soil | 7327/99 | P.%0 Cft: | 401647/= |
|  |  |  |  | TOTAL |  | 1813539/= |

Amount Total (a)

__________________________% Above / Below on the rates of CSR. 	Total (a) =

Amount to be added /deducted on the

basis of premium quoted Total (a)

Total (a) =  in works in figure

Executive Engineer,

Contractor						                           Highways Division

Kashmore @ Kandhkot

*Imran*

CONSTRUCTION OF BRIDGE 40’RFT OVER SHEERAN MINOR NEAR VILLAGE ABDUL SATTAR BHAYO TALUKA KANDHKOT

DRAFT BIDDING DOCUMENT FOR WORKS

BILLS OF QUANTITIES

(A) 	Description and rate of Items bases on composite schedule of Rates.

PART-A  (BRIDGE WORK).

| Item No. | Qty’s |  | Description of item to be executed at site | Rate | Unit | Amount in Rupees |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 |  | 3 | 4 | 5 | 6 |
| 01/- | 5412 Cft: |  | Excavation in foundation of building bridge other structure i/cs dag belling dressing refilling around the structure with excavated earth watering and remaining lead upto one chain and left upto 5.0’ ordinary soil. | 3176/25 | P.%0 Cft: | 17190/= |
| 02/- | 1304Cft: |  | Cement concrete with bricks are stone plaster 1½” to 2” guage ratio 1:2:4 | 9416/28 | P.% Cft: | 122788/= |
| 03/- | 3123Cft: |  | Pucca Brick work in foundation and plinth i/c cement sand mortor | 12501/41 | P.% Cft: | 390419/= |
| 04/- | 256Cft: |  | Cement concrete plain i/c placing compacting finishing curing complete i/c screening washing of stone aggregate with out suhttering ratio 1:2:4: | 14429/25 | P.% Cft: | 36939/= |
| 05/- | 92 Sft: |  | Erection and removal of centering for R.C.C or plain cement work (2nd: class) for partal wood vertical | 7000/00 | P.% Sft: | 6440/= |
| 06/- | 70.62 Cwt |  | Fabrication of m/s reinforcement cement concrete including cutting binding laying in position making joints & fastening of i/c binding wire also removal rust from bars. | 4820/20 | P.Cwt | 340402/= |
| 07/- | 1340Rft: |  | Reinforced cement concrete work including all labour and material except the cost of steel reinforcement and its labour for bending and binding which will be paid separately. This rate also includes all kind of forms moulds lifting shuttering curing rend. | 337/00 | P.Rft: | 451580/= |
| 08/- | 2020Sft: |  | Cement Plaster (1:3) upto 20’ height 1½” thick. | 2283/93 | P.% Sft: | 46135/= |
| 09/- |  | 54810Cft: | Earth work on road embankment by bulldozers i/c plugging mixing clod breaking dressing and compcting with optimum moisture content lead upto 100’ and lift upto 5’ in all types of soil | 7327/99 | P.%0 Cft: | 401647/= |
|  |  |  |  | TOTAL |  | 1813539/= |

Amount Total (a)

__________________________% Above / Below on the rates of CSR. 	Total (a) =

Amount to be added /deducted on the

basis of premium quoted Total (a)

Total (a) =  in works in figure

Executive Engineer,

Contractor						                           Highways Division

Kashmore @ Kandhkot

*Imran*

CONSTRUCTION OF BRIDGE  OVER CHANDOOL WAH AT VILLAGE ANWAR ALI GOLATO ALLAH BUX JAKHRANI, HAJI SAIFAL JAKHRANI U/C SODHI TALUKA KASHMORE

DRAFT BIDDING DOCUMENT FOR WORKS

BILLS OF QUANTITIES

(A) 	Description and rate of Items bases on composite schedule of Rates.

PART-A  (BRIDGE WORK).

| Item No. | Qty’s |  | Description of item to be executed at site | Rate | Unit | Amount in Rupees |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 |  | 3 | 4 | 5 | 6 |
| 01/- | 4728 Cft: |  | Excavation in foundation of building bridge other structure i/cs dag belling dressing refilling around the structure with excavated earth watering and remaining lead upto one chain and left upto 5.0’ ordinary soil. | 3176/25 | P.%0 Cft: | 15017/= |
| 02/- | 1136Cft: |  | Cement concrete with bricks are stone plaster 1½” to 2” guage ratio 1:2:4 | 9416/28 | P.% Cft: | 106969/= |
| 03/- | 3580Cft: |  | Pucca Brick work in foundation and plinth i/c cement sand mortor | 12501/41 | P.% Cft: | 447550/= |
| 04/- | 230Cft: |  | Cement concrete plain i/c placing compacting finishing curing complete i/c screening washing of stone aggregate with out suhttering ratio 1:2:4: | 14429/25 | P.% Cft: | 33187/= |
| 05/- | 84 Sft: |  | Erection and removal of centering for R.C.C or plain cement work (2nd: class) for partal wood vertical | 7000/00 | P.% Sft: | 5880/= |
| 06/- | 52.37 Cwt |  | Fabrication of m/s reinforcement cement concrete including cutting binding laying in position making joints & fastening of i/c binding wire also removal rust from bars. | 4820/20 | P.Cwt | 252433/= |
| 07/- | 960Rft: |  | Reinforced cement concrete work including all labour and material except the cost of steel reinforcement and its labour for bending and binding which will be paid separately. This rate also includes all kind of forms moulds lifting shuttering curing rend. | 337/00 | P.Rft: | 323520/= |
| 08/- | 2096Sft: |  | Cement Plaster (1:3) upto 20’ height 1½” thick. | 2283/93 | P.% Sft: | 47871/= |
| 09/- |  | 20400Cft: | Earth work on road embankment by bulldozers i/c plugging mixing clod breaking dressing and compcting with optimum moisture content lead upto 100’ and lift upto 5’ in all types of soil | 2208/23 | P.%0 Cft: | 45047/= |
|  |  |  |  | TOTAL |  | 1277474/= |

Amount Total (a)

__________________________% Above / Below on the rates of CSR. 	Total (a) =

Amount to be added /deducted on the

basis of premium quoted Total (a)

Total (a) =  in works in figure

Executive Engineer,

Contractor						                           Highways Division

Kashmore @ Kandhkot

*Imran*

CONSTRUCTION OF BRIDGE  OVER BUGHTI RAJ WAH NEAR VILLAGE RASOOL BUX GOLATO U/C MALGUZAR

DRAFT BIDDING DOCUMENT FOR WORKS

BILLS OF QUANTITIES

(A) 	Description and rate of Items bases on composite schedule of Rates.

PART-A  (BRIDGE WORK).

| Item No. | Qty’s |  | Description of item to be executed at site | Rate | Unit | Amount in Rupees |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 |  | 3 | 4 | 5 | 6 |
| 01/- | 4728 Cft: |  | Excavation in foundation of building bridge other structure i/cs dag belling dressing refilling around the structure with excavated earth watering and remaining lead upto one chain and left upto 5.0’ ordinary soil. | 3176/25 | P.%0 Cft: | 15017/= |
| 02/- | 1136Cft: |  | Cement concrete with bricks are stone plaster 1½” to 2” guage ratio 1:2:4 | 9416/28 | P.% Cft: | 106969/= |
| 03/- | 3580Cft: |  | Pucca Brick work in foundation and plinth i/c cement sand mortor | 12501/41 | P.% Cft: | 447550/= |
| 04/- | 230Cft: |  | Cement concrete plain i/c placing compacting finishing curing complete i/c screening washing of stone aggregate with out suhttering ratio 1:2:4: | 14429/25 | P.% Cft: | 33187/= |
| 05/- | 84 Sft: |  | Erection and removal of centering for R.C.C or plain cement work (2nd: class) for partal wood vertical | 7000/00 | P.% Sft: | 5880/= |
| 06/- | 52.37 Cwt |  | Fabrication of m/s reinforcement cement concrete including cutting binding laying in position making joints & fastening of i/c binding wire also removal rust from bars. | 4820/20 | P.Cwt | 252433/= |
| 07/- | 960Rft: |  | Reinforced cement concrete work including all labour and material except the cost of steel reinforcement and its labour for bending and binding which will be paid separately. This rate also includes all kind of forms moulds lifting shuttering curing rend. | 337/00 | P.Rft: | 323520/= |
| 08/- | 2096Sft: |  | Cement Plaster (1:3) upto 20’ height 1½” thick. | 2283/93 | P.% Sft: | 47871/= |
| 09/- |  | 20400Cft: | Earth work on road embankment by bulldozers i/c plugging mixing clod breaking dressing and compcting with optimum moisture content lead upto 100’ and lift upto 5’ in all types of soil | 2208/23 | P.%0 Cft: | 45047/= |
|  |  |  |  | TOTAL |  | 1277474/= |

Amount Total (a)

__________________________% Above / Below on the rates of CSR. 	Total (a) =

Amount to be added /deducted on the

basis of premium quoted Total (a)

Total (a) =  in works in figure

Executive Engineer,

Contractor						                           Highways Division

Kashmore @ Kandhkot

*Imran*

CONSTRUCTION OF BRIDGE  OVER KANDHKOT SHAKH NEAR VILLAGE GAMAN KHAN SABZOI TALUKA KANDHKTO

DRAFT BIDDING DOCUMENT FOR WORKS

BILLS OF QUANTITIES

(A) 	Description and rate of Items bases on composite schedule of Rates.

PART-A  (BRIDGE WORK).

| Item No. | Qty’s |  | Description of item to be executed at site | Rate | Unit | Amount in Rupees |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 |  | 3 | 4 | 5 | 6 |
| 01/- | 4728 Cft: |  | Excavation in foundation of building bridge other structure i/cs dag belling dressing refilling around the structure with excavated earth watering and remaining lead upto one chain and left upto 5.0’ ordinary soil. | 3176/25 | P.%0 Cft: | 15017/= |
| 02/- | 1136Cft: |  | Cement concrete with bricks are stone plaster 1½” to 2” guage ratio 1:2:4 | 9416/28 | P.% Cft: | 106969/= |
| 03/- | 3580Cft: |  | Pucca Brick work in foundation and plinth i/c cement sand mortor | 12501/41 | P.% Cft: | 447550/= |
| 04/- | 230Cft: |  | Cement concrete plain i/c placing compacting finishing curing complete i/c screening washing of stone aggregate with out suhttering ratio 1:2:4: | 14429/25 | P.% Cft: | 33187/= |
| 05/- | 84 Sft: |  | Erection and removal of centering for R.C.C or plain cement work (2nd: class) for partal wood vertical | 7000/00 | P.% Sft: | 5880/= |
| 06/- | 52.37 Cwt |  | Fabrication of m/s reinforcement cement concrete including cutting binding laying in position making joints & fastening of i/c binding wire also removal rust from bars. | 4820/20 | P.Cwt | 252433/= |
| 07/- | 960Rft: |  | Reinforced cement concrete work including all labour and material except the cost of steel reinforcement and its labour for bending and binding which will be paid separately. This rate also includes all kind of forms moulds lifting shuttering curing rend. | 337/00 | P.Rft: | 323520/= |
| 08/- | 2096Sft: |  | Cement Plaster (1:3) upto 20’ height 1½” thick. | 2283/93 | P.% Sft: | 47871/= |
| 09/- |  | 27200Cft: | Earth work on road embankment by bulldozers i/c plugging mixing clod breaking dressing and compcting with optimum moisture content lead upto 100’ and lift upto 5’ in all types of soil | 2208/23 | P.%0 Cft: | 60064/= |
|  |  |  |  | TOTAL |  | 1301997/= |

Amount Total (a)

__________________________% Above / Below on the rates of CSR. 	Total (a) =

Amount to be added /deducted on the

basis of premium quoted Total (a)

Total (a) =  in works in figure

Executive Engineer,

Contractor						                           Highways Division

Kashmore @ Kandhkot

*Imran*

CONSTRUCTION OF BRIDGE BAHADURPUR MINOR AT VILLAGE DEEDAR ALI JAKHRANI TALUKA KASHMORE

DRAFT BIDDING DOCUMENT FOR WORKS

BILLS OF QUANTITIES

(A) 	Description and rate of Items bases on composite schedule of Rates.

PART-A  (BRIDGE WORK).

| Item No. | Qty’s |  | Description of item to be executed at site | Rate | Unit | Amount in Rupees |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 |  | 3 | 4 | 5 | 6 |
| 01/- | 4728 Cft: |  | Excavation in foundation of building bridge other structure i/cs dag belling dressing refilling around the structure with excavated earth watering and remaining lead upto one chain and left upto 5.0’ ordinary soil. | 3176/25 | P.%0 Cft: | 15017/= |
| 02/- | 1136Cft: |  | Cement concrete with bricks are stone plaster 1½” to 2” guage ratio 1:2:4 | 9416/28 | P.% Cft: | 106969/= |
| 03/- | 3580Cft: |  | Pucca Brick work in foundation and plinth i/c cement sand mortor | 12501/41 | P.% Cft: | 447550/= |
| 04/- | 230Cft: |  | Cement concrete plain i/c placing compacting finishing curing complete i/c screening washing of stone aggregate with out suhttering ratio 1:2:4: | 14429/25 | P.% Cft: | 33187/= |
| 05/- | 84 Sft: |  | Erection and removal of centering for R.C.C or plain cement work (2nd: class) for partal wood vertical | 7000/00 | P.% Sft: | 5880/= |
| 06/- | 52.37 Cwt |  | Fabrication of m/s reinforcement cement concrete including cutting binding laying in position making joints & fastening of i/c binding wire also removal rust from bars. | 4820/20 | P.Cwt | 252433/= |
| 07/- | 960Rft: |  | Reinforced cement concrete work including all labour and material except the cost of steel reinforcement and its labour for bending and binding which will be paid separately. This rate also includes all kind of forms moulds lifting shuttering curing rend. | 337/00 | P.Rft: | 323520/= |
| 08/- | 2096Sft: |  | Cement Plaster (1:3) upto 20’ height 1½” thick. | 2283/93 | P.% Sft: | 47871/= |
| 09/- |  | -- | Earth work on road embankment by bulldozers i/c plugging mixing clod breaking dressing and compcting with optimum moisture content lead upto 100’ and lift upto 5’ in all types of soil | -- | -- | -- |
|  |  |  |  | TOTAL |  | 1221427/= |

Amount Total (a)

__________________________% Above / Below on the rates of CSR. 	Total (a) =

Amount to be added /deducted on the

basis of premium quoted Total (a)

Total (a) =  in works in figure

Executive Engineer,

Contractor						                           Highways Division

Kashmore @ Kandhkot

*Imran*

CONSTRUCTION OF BRIDGE  NEAR VILLAGE DR IQRAR KHAN JAKHRANI TALUKA KASHMORE

DRAFT BIDDING DOCUMENT FOR WORKS

BILLS OF QUANTITIES

(A) 	Description and rate of Items bases on composite schedule of Rates.

PART-A  (BRIDGE WORK).

| Item No. | Qty’s |  | Description of item to be executed at site | Rate | Unit | Amount in Rupees |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 |  | 3 | 4 | 5 | 6 |
| 01/- | 4728 Cft: |  | Excavation in foundation of building bridge other structure i/cs dag belling dressing refilling around the structure with excavated earth watering and remaining lead upto one chain and left upto 5.0’ ordinary soil. | 3176/25 | P.%0 Cft: | 15017/= |
| 02/- | 1136Cft: |  | Cement concrete with bricks are stone plaster 1½” to 2” guage ratio 1:2:4 | 9416/28 | P.% Cft: | 106969/= |
| 03/- | 3580Cft: |  | Pucca Brick work in foundation and plinth i/c cement sand mortor | 12501/41 | P.% Cft: | 447550/= |
| 04/- | 230Cft: |  | Cement concrete plain i/c placing compacting finishing curing complete i/c screening washing of stone aggregate with out suhttering ratio 1:2:4: | 14429/25 | P.% Cft: | 33187/= |
| 05/- | 84 Sft: |  | Erection and removal of centering for R.C.C or plain cement work (2nd: class) for partal wood vertical | 7000/00 | P.% Sft: | 5880/= |
| 06/- | 52.37 Cwt |  | Fabrication of m/s reinforcement cement concrete including cutting binding laying in position making joints & fastening of i/c binding wire also removal rust from bars. | 4820/20 | P.Cwt | 252433/= |
| 07/- | 960Rft: |  | Reinforced cement concrete work including all labour and material except the cost of steel reinforcement and its labour for bending and binding which will be paid separately. This rate also includes all kind of forms moulds lifting shuttering curing rend. | 337/00 | P.Rft: | 323520/= |
| 08/- | 2096Sft: |  | Cement Plaster (1:3) upto 20’ height 1½” thick. | 2283/93 | P.% Sft: | 47871/= |
| 09/- |  | 27200Cft: | Earth work on road embankment by bulldozers i/c plugging mixing clod breaking dressing and compcting with optimum moisture content lead upto 100’ and lift upto 5’ in all types of soil | 2208/23 | P.%0 Cft: | 60064/= |
|  |  |  |  | TOTAL |  | 1294249/= |

Amount Total (a)

__________________________% Above / Below on the rates of CSR. 	Total (a) =

Amount to be added /deducted on the

basis of premium quoted Total (a)

Total (a) =  in works in figure

Executive Engineer,

Contractor						                           Highways Division

Kashmore @ Kandhkot

*Imran*

RECONDITIONING OF ROAD FROM SAIFAL ROAD TO VILLAGE HAJI SALEEM KHAN BAJARANI ROAD MILE 0/0-0/4

DRAFT BIDDING DOCUMENT FOR WORKS

BILLS OF QUANTITIES

(A) 	Description and rate of Items bases on composite schedule of Rates

Part “A” (ROAD WORK)

| Item No. | Qty’s | Description of item to be executed at site |  | Rate | Unit | Amount in Rupees |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 |  | 4 | 5 | 6 |
| 01/- | 17131Cft: | Preparing base course supplying and spreading stone metal of approved quality of 6” in 2 layers from approved quarry properly grade to maximum size of 1½” in required thickness of proper camber & grade i/c S/S 15 Cft: screening & non plastic quarry fines filling & depressions with stone metal after initial rolling including watering & compacting the same so as to achieve 100% density as per modified AASHO specifications (This i/cs providing and using templates camber plates screen from as directed).Rate includes all costs of materials T&P and carriage upto site of work. |  | 9934/22 | P.% Cft: | 1701844/= |
| 02/- | 5438 Rft | Laying brick on end edging including supply of 9”x 4 ½”x3” 1st: class burnt bricks excavation for laying edging with small side parallel to the road. Rate includes all costs of materials T&P and carriage upto site of work. |  | 2771/44 | P%Rft | 150710/= |
| 03/- | 32630Sft: | Providing 1st: coat of surface dressing on or new existing surface with 30 lbs: bitumen and 4.0 cft: cursh bajri of required size including cleaning the road surface rolling etc: complete.Rate includes all costs of materials T&P & carriage. |  | 1680/79 | P.% Sft: | 548441/= |
| 04/- | 32630Sft: | Providing 1” thick consolidated premixed carpet in proper camber and grade i/c supply of 10 Cft: bajri 4.0 Cft: hill sand of approve quality and guage and bitumen 67 lbs: of 80/100 penetration i/c mixing the mechanical mixture in required proportion i/c heating the material and cleaning the road surface. Rate i/c all cost of material T&P and carriage upto site of work. |  | 4581/97 | P.% Sft: | 1495096/= |
| 05/- | 131500Cft: | Earth Work embankment from barrow pits i/c laying in 6” layers, clod breaking remaining dressing complete, lead Upto 100’ft and lift upto 5’ft (In ordinary Soil) |  | 7327/99 | P.%0 Cft: | 963631/= |
|  |  |  | TOTAL |  |  | 4859722/= |

_____________% Above / Below on the rates of CSR.

Amount to be added /deducted on the

Total (A) 									Rs:

Total Amount in words  ____________________________________________________

__________________________________________________________________________

Executive Engineer,

Contractor						                           Highways Division

Kashmore @ Kandhkot

*Imran*

W/R OF ROAD FROM REHMATABAD TO VILLAGE GHULAM SARWAR KHAN SUHRYANI ROAD MILE 0/0-0/2

DRAFT BIDDING DOCUMENT FOR WORKS

BILLS OF QUANTITIES

(A) 	Description and rate of Items bases on composite schedule of Rates

Part “A” (ROAD WORK)

| Item No. | Qty’s | Description of item to be executed at site |  | Rate | Unit | Amount in Rupees |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 |  | 4 | 5 | 6 |
| 01/- | 68524Cft: | Preparing base course supplying and spreading stone metal of approved quality of 6” in 2 layers from approved quarry properly grade to maximum size of 1½” in required thickness of proper camber & grade i/c S/S 15 Cft: screening & non plastic quarry fines filling & depressions with stone metal after initial rolling including watering & compacting the same so as to achieve 100% density as per modified AASHO specifications (This i/cs providing and using templates camber plates screen from as directed).Rate includes all costs of materials T&P and carriage upto site of work. |  | 9645/18 | P.% Cft: | 6609248/= |
| 02/- | 21754Rft | Laying brick on end edging including supply of 9”x 4 ½”x3” 1st: class burnt bricks excavation for laying edging with small side parallel to the road. Rate includes all costs of materials T&P and carriage upto site of work. |  | 2690/84 | P%Rft | 585365/= |
| 03/- | 130522Sft: | Providing 1st: coat of surface dressing on or new existing surface with 30 lbs: bitumen and 4.0 cft: cursh bajri of required size including cleaning the road surface rolling etc: complete.Rate includes all costs of materials T&P & carriage. |  | 1673/11 | P.% Sft: | 2183770/= |
| 04/- | 130522Sft: | Providing 1” thick consolidated premixed carpet in proper camber and grade i/c supply of 10 Cft: bajri 4.0 Cft: hill sand of approve quality and guage and bitumen 67 lbs: of 80/100 penetration i/c mixing the mechanical mixture in required proportion i/c heating the material and cleaning the road surface. Rate i/c all cost of material T&P and carriage upto site of work. |  | 4554/92 | P.% Sft: | 5945172/= |
| 05/- | 536000Cft: | Earth Work embankment from barrow pits i/c laying in 6” layers, clod breaking remaining dressing complete, lead Upto 100’ft and lift upto 5’ft (In ordinary Soil) |  | 7327/99 | P.%0 Cft: | 3927803/= |
|  |  |  | TOTAL |  |  | 19251358/= |

_____________% Above / Below on the rates of CSR.

Amount to be added /deducted on the

Total (A) 									Rs:

Total Amount in words  ____________________________________________________

__________________________________________________________________________

Executive Engineer,

Contractor						                           Highways Division

Kashmore @ Kandhkot

*Imran*

W/R OF ROAD FROM REHMATABAD TO VILLAGE GHULAM SARWAR KHAN SUHRYANI ROAD MILE 0/0-0/2

DRAFT BIDDING DOCUMENT FOR WORKS

BILLS OF QUANTITIES

(A) 	Description and rate of Items bases on composite schedule of Rates.

PART-B  (3.0 FT: SPAN CULVERTS).

| Item No. | Qty’s | Description of item to be executed at site | Rate | Unit | Amount in Rupees |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 |
| 01/- | 1106 Cft: | Excavation in foundation of building bridge other structure i/cs dag belling dressing refilling around the structure with excavated earth watering and remaining lead upto one chain and left upto 5.0’ ordinary soil. | 3176/25 | P.%0 Cft: | 3512/= |
| 02/- | 237 Cft: | Cement concrete with bricks are stone plaster 1½” to 2” guage ratio 1:2:4 | 9416/28 | P.% Cft: | 22518/= |
| 03/- | 502 Cft: | Pucca Brick work in foundation and plinth i/c cement sand mortor | 12501/41 | P.% Cft: | 62757/= |
| 04/- | 55 Cft: | Cement concrete plain i/c placing compacting finishing curing complete i/c screening washing of stone aggregate with out suhttering ratio 1:2:4: | 14429/25 | P.% Cft: | 7936/= |
| 05/- | 58 Sft: | Erection and removal of centering for R.C.C or plain cement work (2nd: class) for partal wood vertical | 7000/00 | P.% Sft: | 4060/= |
| 06/- | A. Tor Steel 4.16 Cwt B. M/ Steel 0.907 Cwt | Fabrication of m/s reinforcement cement concrete including cutting binding laying in position making joints & fastening of i/c binding wire also removal rust from bars. | 5001/70 4820/29 | P.Cwt: P.Cwt | 20807/= 4372/= |
| 07/- | 114 Rft: | Reinforced cement concrete work including all labour and material except the cost of steel reinforcement and its labour for bending and binding which will be paid separately. This rate also includes all kind of forms moulds lifting shuttering curing rend. | 337/00 | P.Rft: | 38418/= |
| 08/- | 330 Sft: | Cement Plaster (1:3) upto 20’ height 1½” thick. | 2283/93 | P.% Sft: | 7537/= |
|  |  |  | TOTAL |  | 171717/= |

Amount Total (b)

__________________________% Above / Below on the rates of CSR. 	Total (b) =

Amount to be added /deducted on the

basis of premium quoted Total (b)

Total (b) =  in works in figure

Executive Engineer,

Contractor						                           Highways Division

Kashmore @ Kandhkot

CONSTRUCTION OF ROAD FROM MURAD WAH UPTO COLONY-I INSPECTION BANGLOW AND LINK ROAD VILLAGE MEHAULLAH AND ASHRAF KHOSO MILE 0/0-1/5+330’

DRAFT BIDDING DOCUMENT FOR WORKS

BILLS OF QUANTITIES

- Description and rate of Items bases on composite schedule of Rates

Part “A” (ROAD WORK)

| Item No. | Qty’s | Description of item to be executed at site | Rate | Unit | Amount in Rupees |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 |
| 01/- | 684000Cft: | Earth work for road embankment by bulldozers i/c plugging and mixing clod breaking dressing and compacting with optimum moisture content lead upto 100’ and lift upto 5.0’ in all types of soil except rock compacting upto 85% density as per modified AASHO specifications. | 3656/23 | P.0% Cft: | 2500861/= |
| 02/- | 110128Cft: | Earth work for road embankment by bulldozers i/c plugging and mixing clod breaking dressing and compacting with optimum moisture content lead upto 100’ and lift upto 5.0’ in all types of soil except rock compacting upto 95-100% density as per modified AASHO specifications. | 2529/77 | P.% Cft: | 278599/= |
| 03/- | 55064Cft: | Providing sand cushion i/c supplying & spreading pit/canal sand of approved quality from approved source of supply where ever available site of work i/c carriage site of work (R.A.A). | 1479/41 | P.% Cft: | 814622/= |
| 04/- | 55064Cft: | Providing Sub Base course by supplying and spreading stone metal 1 ½” gauge of approved quality from approved quarry in required thickness to proper camber and grade i/c hand packing mixing with 10’ screening & non plastic quarry of approved quality & gauge from approved source i/c watering and compacting to achieve 98-100% density as per modified AASHO specification ( rate i/cs all cost of material T&P and carriage upto site of wok.) | 9954/39 | P.% Cft: | 5481285/= |
| 05/- | 18355Rft: | Laying brick on end edging including supply of 9”x 4 ½”x3” 1st: class burnt bricks excavation for laying edging with small side parallel to the road. Rate includes all costs of materials T&P and carriage upto site of work. | 2980/03 | P.% Rft: | 546984/= |

Contd: P/2

P-2

| Item No. | Qty’s | Description of item to be executed at site | Rate | Unit | Amount in Rupees |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 |
| 06/- | 27532Cft: | Preparing base course supplying and spreading stone metal of approved quality of 6” in 2 layers from approved quarry properly grade to maximum size of 1½” in required thickness of proper camber & grade i/c S/S 15 Cft: screening & non plastic quarry fines filling & depressions with stone metal after initial rolling including watering & compacting the same so as to achieve 100% density as per modified AASHO specifications (This i/cs providing and using templates camber plates screen from as directed).Rate includes all costs of materials T&P and carriage upto site of work. | 11042/66 | P.% Cft: | 3040265/= |
| 07/- | 110128Sft: | Providing 1st: coat of surface dressing on or new existing surface with 30 lbs: bitumen and 4.0 cft: cursh bajri of required size including cleaning the road surface rolling etc: complete.Rate includes all costs of materials T&P & carriage. | 1714/67 | P.% Sft: | 1888332/= |
| 08/- | 110128Sft: | Providing 1” thick consolidated premixed carpet in proper camber and grade i/c supply of 10 Cft: bajri 4.0 Cft: hill sand of approve quality and guage and bitumen 67 lbs: of 80/100 penetration i/c mixing the mechanical mixture in required proportion i/c heating the material and cleaning the road surface. Rate i/c all cost of material T&P and carriage upto site of work. | 4620/04 | P.% Sft: | 5087958/= |
|  |  |  | TOTAL |  | 19638906/= |

_____________% Above / Below on the rates of CSR.

Amount to be added /deducted on the

Total (A) 									Rs:

Total Amount in words  ____________________________________________________

__________________________________________________________________________

Executive Engineer,

Contractor						                           Highways Division

Kashmore @ Kandhkot

*Imran*

CONSTRUCTION OF ROAD FROM MURAD WAH UPTO COLONY-I INSPECTION BANGLOW AND LINK ROAD VILLAGE MEHAULLAH AND ASHRAF KHOSO MILE 0/0-1/5+330’

DRAFT BIDDING DOCUMENT FOR WORKS

BILLS OF QUANTITIES

(A) 	Description and rate of Items bases on composite schedule of Rates.

PART-B  (3.0 FT: SPAN CULVERTS).

| Item No. | Qty’s | Description of item to be executed at site | Rate | Unit | Amount in Rupees |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 |
| 01/- | 1106 Cft: | Excavation in foundation of building bridge other structure i/cs dag belling dressing refilling around the structure with excavated earth watering and remaining lead upto one chain and left upto 5.0’ ordinary soil. | 3176/25 | P.%0 Cft: | 3512/= |
| 02/- | 237 Cft: | Cement concrete with bricks are stone plaster 1½” to 2” guage ratio 1:2:4 | 9416/28 | P.% Cft: | 22518/= |
| 03/- | 502 Cft: | Pucca Brick work in foundation and plinth i/c cement sand mortor | 12501/41 | P.% Cft: | 62757/= |
| 04/- | 55 Cft: | Cement concrete plain i/c placing compacting finishing curing complete i/c screening washing of stone aggregate with out suhttering ratio 1:2:4: | 14429/25 | P.% Cft: | 7936/= |
| 05/- | 58 Sft: | Erection and removal of centering for R.C.C or plain cement work (2nd: class) for partal wood vertical | 7000/00 | P.% Sft: | 4060/= |
| 06/- | A. Tor Steel 4.16 Cwt B. M/ Steel 0.907 Cwt | Fabrication of m/s reinforcement cement concrete including cutting binding laying in position making joints & fastening of i/c binding wire also removal rust from bars. | 5001/70 4820/29 | P.Cwt: P.Cwt | 20807/= 4372/= |
| 07/- | 114 Rft: | Reinforced cement concrete work including all labour and material except the cost of steel reinforcement and its labour for bending and binding which will be paid separately. This rate also includes all kind of forms moulds lifting shuttering curing rend. | 337/00 | P.Rft: | 38418/= |
| 08/- | 330 Sft: | Cement Plaster (1:3) upto 20’ height 1½” thick. | 2283/93 | P.% Sft: | 7537/= |
|  |  |  | TOTAL |  | 171717/= |

Amount Total (b)

__________________________% Above / Below on the rates of CSR. 	Total (b) =

Amount to be added /deducted on the

basis of premium quoted Total (b)

Total (b) =  in works in figure

Executive Engineer,

Contractor						                           Highways Division

Kashmore @ Kandhkot

CONSTRUCTION OF ROAD FROM MURAD WAH UPTO COLONY-I INSPECTION BANGLOW AND LINK ROAD VILLAGE MEHAULLAH AND ASHRAF KHOSO MILE 0/0-1/5+330’

DRAFT BIDDING DOCUMENT FOR WORKS

BILLS OF QUANTITIES

(A) 	Description and rate of Items bases on composite schedule of Rates.

PART-C  (MISC:).

| Item No. | Qty’s | Description of item to be executed at site | Rate | Unit | Amount in Rupees |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 |
| 01/- | 01 Nos; | Proving and Fixing informatory sign rectangular shape 4’ x 3” comprising 02 Nos; 3” dia G.I Pipe Post i/c painting writing as directed each. | 16956/00 | P.Each | 16,956/= |
|  |  |  | TOTAL |  | 16,956/= |

Amount Total (C)

__________________________% Above / Below on the rates of CSR. 	Total (C) =

Amount to be added /deducted on the

basis of premium quoted Total (C)

Total (c) =  in works in figure

Executive Engineer,

Contractor						                           Highways Division

Kashmore @ Kandhkot

*Imran*

CONSTRUCTION OF ROAD FROM PAT FEEDER VIA SHAKAL KHAN MALIK UPTO GOLATO ASGHAR KHOSO AND ABDUL GHAFOOR ROAD  MILE 0/0-1/3

DRAFT BIDDING DOCUMENT FOR WORKS

BILLS OF QUANTITIES

- Description and rate of Items bases on composite schedule of Rates

Part “A” (ROAD WORK)

| Item No. | Qty’s | Description of item to be executed at site | Rate | Unit | Amount in Rupees |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 |
| 01/- | 1064000Cft: | Earth work for road embankment by bulldozers i/c plugging and mixing clod breaking dressing and compacting with optimum moisture content lead upto 100’ and lift upto 5.0’ in all types of soil except rock compacting upto 85% density as per modified AASHO specifications. | 3656/23 | P.0% Cft: | 3890229/= |
| 02/- | 89734Cft: | Earth work for road embankment by bulldozers i/c plugging and mixing clod breaking dressing and compacting with optimum moisture content lead upto 100’ and lift upto 5.0’ in all types of soil except rock compacting upto 95-100% density as per modified AASHO specifications. | 2529/77 | P.% Cft: | 227006/= |
| 03/- | 44867Cft: | Providing sand cushion i/c supplying & spreading pit/canal sand of approved quality from approved source of supply where ever available site of work i/c carriage site of work (R.A.A). | 1479/41 | P.% Cft: | 663767/= |
| 04/- | 44867Cft: | Providing Sub Base course by supplying and spreading stone metal 1 ½” gauge of approved quality from approved quarry in required thickness to proper camber and grade i/c hand packing mixing with 10’ screening & non plastic quarry of approved quality & gauge from approved source i/c watering and compacting to achieve 98-100% density as per modified AASHO specification ( rate i/cs all cost of material T&P and carriage upto site of wok.) | 9954/39 | P.% Cft: | 4466236/= |
| 05/- | 14956Rft: | Laying brick on end edging including supply of 9”x 4 ½”x3” 1st: class burnt bricks excavation for laying edging with small side parallel to the road. Rate includes all costs of materials T&P and carriage upto site of work. | 2979/23 | P.% Rft: | 445573/= |

Contd: P/2

P-2

| Item No. | Qty’s | Description of item to be executed at site | Rate | Unit | Amount in Rupees |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 |
| 06/- | 22433Cft: | Preparing base course supplying and spreading stone metal of approved quality of 6” in 2 layers from approved quarry properly grade to maximum size of 1½” in required thickness of proper camber & grade i/c S/S 15 Cft: screening & non plastic quarry fines filling & depressions with stone metal after initial rolling including watering & compacting the same so as to achieve 100% density as per modified AASHO specifications (This i/cs providing and using templates camber plates screen from as directed).Rate includes all costs of materials T&P and carriage upto site of work. | 11042/66 | P.% Cft: | 2477200/= |
| 07/- | 89734Sft: | Providing 1st: coat of surface dressing on or new existing surface with 30 lbs: bitumen and 4.0 cft: cursh bajri of required size including cleaning the road surface rolling etc: complete.Rate includes all costs of materials T&P & carriage. | 1714/67 | P.% Sft: | 1538642/= |
| 08/- | 89734Sft: | Providing 1” thick consolidated premixed carpet in proper camber and grade i/c supply of 10 Cft: bajri 4.0 Cft: hill sand of approve quality and guage and bitumen 67 lbs: of 80/100 penetration i/c mixing the mechanical mixture in required proportion i/c heating the material and cleaning the road surface. Rate i/c all cost of material T&P and carriage upto site of work. | 4633/06 | P.% Sft: | 4157430/= |
|  |  |  | TOTAL |  | 17866083/= |

_____________% Above / Below on the rates of CSR.

Amount to be added /deducted on the

Total (A) 									Rs:

Total Amount in words  ____________________________________________________

__________________________________________________________________________

Executive Engineer,

Contractor						                           Highways Division

Kashmore @ Kandhkot

*Imran*

CONSTRUCTION OF ROAD FROM PAT FEEDER VIA SHAKAL KHAN MALIK UPTO GOLATO ASGHAR KHOSO AND ABDUL GHAFOOR ROAD  MILE 0/0-1/3

DRAFT BIDDING DOCUMENT FOR WORKS

BILLS OF QUANTITIES

(A) 	Description and rate of Items bases on composite schedule of Rates.

PART-B  (3.0 FT: SPAN CULVERTS).

| Item No. | Qty’s | Description of item to be executed at site | Rate | Unit | Amount in Rupees |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 |
| 01/- | 1106 Cft: | Excavation in foundation of building bridge other structure i/cs dag belling dressing refilling around the structure with excavated earth watering and remaining lead upto one chain and left upto 5.0’ ordinary soil. | 3176/25 | P.%0 Cft: | 3512/= |
| 02/- | 237 Cft: | Cement concrete with bricks are stone plaster 1½” to 2” guage ratio 1:2:4 | 9416/28 | P.% Cft: | 22518/= |
| 03/- | 502 Cft: | Pucca Brick work in foundation and plinth i/c cement sand mortor | 12501/41 | P.% Cft: | 62757/= |
| 04/- | 55 Cft: | Cement concrete plain i/c placing compacting finishing curing complete i/c screening washing of stone aggregate with out suhttering ratio 1:2:4: | 14429/25 | P.% Cft: | 7936/= |
| 05/- | 58 Sft: | Erection and removal of centering for R.C.C or plain cement work (2nd: class) for partal wood vertical | 7000/00 | P.% Sft: | 4060/= |
| 06/- | A. Tor Steel 4.16 Cwt B. M/ Steel 0.907 Cwt | Fabrication of m/s reinforcement cement concrete including cutting binding laying in position making joints & fastening of i/c binding wire also removal rust from bars. | 5001/70 4820/29 | P.Cwt: P.Cwt | 20807/= 4372/= |
| 07/- | 114 Rft: | Reinforced cement concrete work including all labour and material except the cost of steel reinforcement and its labour for bending and binding which will be paid separately. This rate also includes all kind of forms moulds lifting shuttering curing rend. | 337/00 | P.Rft: | 38418/= |
| 08/- | 330 Sft: | Cement Plaster (1:3) upto 20’ height 1½” thick. | 2283/93 | P.% Sft: | 7537/= |
|  |  |  | TOTAL |  | 171717/= |

Amount Total (b)

__________________________% Above / Below on the rates of CSR. 	Total (b) =

Amount to be added /deducted on the

basis of premium quoted Total (b)

Total (b) =  in works in figure

Executive Engineer,

Contractor						                           Highways Division

Kashmore @ Kandhkot

CONSTRUCTION OF ROAD FROM PAT FEEDER VIA SHAKAL KHAN MALIK UPTO GOLATO ASGHAR KHOSO AND ABDUL GHAFOOR ROAD  MILE 0/0-1/3

DRAFT BIDDING DOCUMENT FOR WORKS

BILLS OF QUANTITIES

(A) 	Description and rate of Items bases on composite schedule of Rates.

PART-C  (MISC:).

| Item No. | Qty’s | Description of item to be executed at site | Rate | Unit | Amount in Rupees |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 |
| 01/- | 01 Nos; | Proving and Fixing informatory sign rectangular shape 4’ x 3” comprising 02 Nos; 3” dia G.I Pipe Post i/c painting writing as directed each. | 16956/00 | P.Each | 16,956/= |
|  |  |  | TOTAL |  | 16,956/= |

Amount Total (C)

__________________________% Above / Below on the rates of CSR. 	Total (C) =

Amount to be added /deducted on the

basis of premium quoted Total (C)

Total (c) =  in works in figure

Executive Engineer,

Contractor						                           Highways Division

Kashmore @ Kandhkot

*Imran*

CONSTRUCTION OF 20’ RFT BRIDGE OVER NORR PUR MINOR AT VILLAGE AALAM CHACHAR U/C R.B. CHACHAR TALUKA KASHMORE

DRAFT BIDDING DOCUMENT FOR WORKS

BILLS OF QUANTITIES

(A) 	Description and rate of Items bases on composite schedule of Rates.

PART-A  (BRIDGE WORK).

| Item No. | Qty’s |  | Description of item to be executed at site | Rate | Unit | Amount in Rupees |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 |  | 3 | 4 | 5 | 6 |
| 01/- | 2868 Cft: |  | Excavation in foundation of building bridge other structure i/cs dag belling dressing refilling around the structure with excavated earth watering and remaining lead upto one chain and left upto 5.0’ ordinary soil. | 3176/25 | P.%0 Cft: | 9109/= |
| 02/- | 936Cft: |  | Cement concrete with bricks are stone plaster 1½” to 2” guage ratio 1:2:4 | 9416/28 | P.% Cft: | 88136/= |
| 03/- | 2172Cft: |  | Pucca Brick work in foundation and plinth i/c cement sand mortor | 12501/41 | P.% Cft: | 271530/= |
| 04/- | 96Cft: |  | Cement concrete plain i/c placing compacting finishing curing complete i/c screening washing of stone aggregate with out suhttering ratio 1:2:4: | 14429/25 | P.% Cft: | 13852/= |
| 05/- | 57 Sft: |  | Erection and removal of centering for R.C.C or plain cement work (2nd: class) for partal wood vertical | 7000/00 | P.% Sft: | 3990/= |
| 06/- | 21.85 Cwt |  | Fabrication of m/s reinforcement cement concrete including cutting binding laying in position making joints & fastening of i/c binding wire also removal rust from bars. | 4820/20 | P.Cwt | 10532/= |
| 07/- | 400Rft: |  | Reinforced cement concrete work including all labour and material except the cost of steel reinforcement and its labour for bending and binding which will be paid separately. This rate also includes all kind of forms moulds lifting shuttering curing rend. | 337/00 | P.Rft: | 134800/= |
| 08/- | 1008Sft: |  | Cement Plaster (1:3) upto 20’ height 1½” thick. | 2283/93 | P.% Sft: | 23022/= |
| 09/- |  | -- | Earth work on road embankment by bulldozers i/c plugging mixing clod breaking dressing and compcting with optimum moisture content lead upto 100’ and lift upto 5’ in all types of soil | -- | -- | -- |
|  |  |  |  | TOTAL |  | 649760/= |

Amount Total (a)

__________________________% Above / Below on the rates of CSR. 	Total (a) =

Amount to be added /deducted on the

basis of premium quoted Total (a)

Total (a) =  in works in figure

Executive Engineer,

Contractor						                           Highways Division

Kashmore @ Kandhkot

*Imran*

CONSTRUCTION OF  BRIDGE KASHMORE MINOR AT VILLAGE NOOR DIN KHOSO TALUKA KASHMORE

DRAFT BIDDING DOCUMENT FOR WORKS

BILLS OF QUANTITIES

(A) 	Description and rate of Items bases on composite schedule of Rates.

PART-A  (BRIDGE WORK).

| Item No. | Qty’s |  | Description of item to be executed at site | Rate | Unit | Amount in Rupees |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 |  | 3 | 4 | 5 | 6 |
| 01/- | 2868 Cft: |  | Excavation in foundation of building bridge other structure i/cs dag belling dressing refilling around the structure with excavated earth watering and remaining lead upto one chain and left upto 5.0’ ordinary soil. | 3176/25 | P.%0 Cft: | 9109/= |
| 02/- | 936Cft: |  | Cement concrete with bricks are stone plaster 1½” to 2” guage ratio 1:2:4 | 9416/28 | P.% Cft: | 88136/= |
| 03/- | 2172Cft: |  | Pucca Brick work in foundation and plinth i/c cement sand mortor | 12501/41 | P.% Cft: | 271530/= |
| 04/- | 96Cft: |  | Cement concrete plain i/c placing compacting finishing curing complete i/c screening washing of stone aggregate with out suhttering ratio 1:2:4: | 14429/25 | P.% Cft: | 13852/= |
| 05/- | 57 Sft: |  | Erection and removal of centering for R.C.C or plain cement work (2nd: class) for partal wood vertical | 7000/00 | P.% Sft: | 3990/= |
| 06/- | 21.85 Cwt |  | Fabrication of m/s reinforcement cement concrete including cutting binding laying in position making joints & fastening of i/c binding wire also removal rust from bars. | 4820/20 | P.Cwt | 10532/= |
| 07/- | 400Rft: |  | Reinforced cement concrete work including all labour and material except the cost of steel reinforcement and its labour for bending and binding which will be paid separately. This rate also includes all kind of forms moulds lifting shuttering curing rend. | 337/00 | P.Rft: | 134800/= |
| 08/- | 1008Sft: |  | Cement Plaster (1:3) upto 20’ height 1½” thick. | 2283/93 | P.% Sft: | 23022/= |
| 09/- |  | -- | Earth work on road embankment by bulldozers i/c plugging mixing clod breaking dressing and compcting with optimum moisture content lead upto 100’ and lift upto 5’ in all types of soil | -- | -- | -- |
|  |  |  |  | TOTAL |  | 649760/= |

Amount Total (a)

__________________________% Above / Below on the rates of CSR. 	Total (a) =

Amount to be added /deducted on the

basis of premium quoted Total (a)

Total (a) =  in works in figure

Executive Engineer,

Contractor						                           Highways Division

Kashmore @ Kandhkot

*Imran*

REPAIR / RECONDITIOING OF ROAD FROM SODHO CHANNA IN VILLAGE DARI ROAD MILE 0/0-0/4+220’

DRAFT BIDDING DOCUMENT FOR WORKS

BILLS OF QUANTITIES

(A) 	Description and rate of Items bases on composite schedule of Rates

Part “A” (ROAD WORK)

| Item No. | Qty’s | Description of item to be executed at site |  | Rate | Unit | Amount in Rupees |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 |  | 4 | 5 | 6 |
| 01/- | 14990Cft: | Preparing base course supplying and spreading stone metal of approved quality of 6” in 2 layers from approved quarry properly grade to maximum size of 1½” in required thickness of proper camber & grade i/c S/S 15 Cft: screening & non plastic quarry fines filling & depressions with stone metal after initial rolling including watering & compacting the same so as to achieve 100% density as per modified AASHO specifications (This i/cs providing and using templates camber plates screen from as directed).Rate includes all costs of materials T&P and carriage upto site of work. |  | 9307/87 | P.% Cft: | 1395250/= |
| 02/- | 47594Rft | Laying brick on end edging including supply of 9”x 4 ½”x3” 1st: class burnt bricks excavation for laying edging with small side parallel to the road. Rate includes all costs of materials T&P and carriage upto site of work. |  | 2761/22 | P%Rft | 131406/= |
| 03/- | 28552Sft: | Providing 1st: coat of surface dressing on or new existing surface with 30 lbs: bitumen and 4.0 cft: cursh bajri of required size including cleaning the road surface rolling etc: complete.Rate includes all costs of materials T&P & carriage. |  | 1663/07 | P.% Sft: | 474840/= |
| 04/- | 28552Sft: | Providing 1” thick consolidated premixed carpet in proper camber and grade i/c supply of 10 Cft: bajri 4.0 Cft: hill sand of approve quality and guage and bitumen 67 lbs: of 80/100 penetration i/c mixing the mechanical mixture in required proportion i/c heating the material and cleaning the road surface. Rate i/c all cost of material T&P and carriage upto site of work. |  | 4541/79 | P.% Sft: | 1296772/= |
| 05/- | 215000Cft: | Earth Work embankment from barrow pits i/c laying in 6” layers, clod breaking remaining dressing complete, lead Upto 100’ft and lift upto 5’ft (In ordinary Soil) |  | 7327/99 | P.%0 Cft: | 1575518/= |
|  |  |  | TOTAL |  |  | 4873786/= |

_____________% Above / Below on the rates of CSR.

Amount to be added /deducted on the

Total (A) 									Rs:

Total Amount in words  ____________________________________________________

__________________________________________________________________________

Executive Engineer,

Contractor						                           Highways Division

Kashmore @ Kandhkot

*Imran*

CONSTRUCTION OF ROAD AND BRIDGE FROM OULA BUX GOLATO ROAD TO ABDUL HAQUE KHAN GOLATO U/C TOJ MILE 0/0-0/1+340’

DRAFT BIDDING DOCUMENT FOR WORKS

BILLS OF QUANTITIES

- Description and rate of Items bases on composite schedule of Rates

Part “A” (ROAD WORK)

| Item No. | Qty’s | Description of item to be executed at site | Rate | Unit | Amount in Rupees |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 |
| 01/- | 50000Cft: | Earth work for road embankment by bulldozers i/c plugging and mixing clod breaking dressing and compacting with optimum moisture content lead upto 100’ and lift upto 5.0’ in all types of soil except rock compacting upto 85% density as per modified AASHO specifications. | 3656/23 | P.0% Cft: | 182812/= |
| 02/- | 12360Cft: | Earth work for road embankment by bulldozers i/c plugging and mixing clod breaking dressing and compacting with optimum moisture content lead upto 100’ and lift upto 5.0’ in all types of soil except rock compacting upto 95-100% density as per modified AASHO specifications. | 2529/77 | P.% Cft: | 31268/= |
| 03/- | 6180Cft: | Providing sand cushion i/c supplying & spreading pit/canal sand of approved quality from approved source of supply where ever available site of work i/c carriage site of work (R.A.A). | 1479/41 | P.% Cft: | 91428/= |
| 04/- | 6180Cft: | Providing Sub Base course by supplying and spreading stone metal 1 ½” gauge of approved quality from approved quarry in required thickness to proper camber and grade i/c hand packing mixing with 10’ screening & non plastic quarry of approved quality & gauge from approved source i/c watering and compacting to achieve 98-100% density as per modified AASHO specification ( rate i/cs all cost of material T&P and carriage upto site of wok.) | 9349/10 | P.% Cft: | 577774/= |
| 05/- | 2060Rft: | Laying brick on end edging including supply of 9”x 4 ½”x3” 1st: class burnt bricks excavation for laying edging with small side parallel to the road. Rate includes all costs of materials T&P and carriage upto site of work. | 2834/15 | P.% Rft: | 58383/= |

Contd: P/2

P-2

| Item No. | Qty’s | Description of item to be executed at site | Rate | Unit | Amount in Rupees |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 |
| 06/- | 3090Cft: | Preparing base course supplying and spreading stone metal of approved quality of 6” in 2 layers from approved quarry properly grade to maximum size of 1½” in required thickness of proper camber & grade i/c S/S 15 Cft: screening & non plastic quarry fines filling & depressions with stone metal after initial rolling including watering & compacting the same so as to achieve 100% density as per modified AASHO specifications (This i/cs providing and using templates camber plates screen from as directed).Rate includes all costs of materials T&P and carriage upto site of work. | 10416/20 | P.% Cft: | 321861/= |
| 07/- | 12360Sft: | Providing 1st: coat of surface dressing on or new existing surface with 30 lbs: bitumen and 4.0 cft: cursh bajri of required size including cleaning the road surface rolling etc: complete.Rate includes all costs of materials T&P & carriage. | 1696/04 | P.% Sft: | 209631/= |
| 08/- | 12360Sft: | Providing 1” thick consolidated premixed carpet in proper camber and grade i/c supply of 10 Cft: bajri 4.0 Cft: hill sand of approve quality and guage and bitumen 67 lbs: of 80/100 penetration i/c mixing the mechanical mixture in required proportion i/c heating the material and cleaning the road surface. Rate i/c all cost of material T&P and carriage upto site of work. | 4614/29 | P.% Sft: | 570320/= |
|  |  |  | TOTAL |  | 2043477/= |

_____________% Above / Below on the rates of CSR.

Amount to be added /deducted on the

Total (A) 									Rs:

Total Amount in words  ____________________________________________________

__________________________________________________________________________

Executive Engineer,

Contractor						                           Highways Division

Kashmore @ Kandhkot

*Imran*

CONSTRUCTION OF ROAD AND BRIDGE FROM OULA BUX GOLATO ROAD TO ABDUL HAQUE KHAN GOLATO U/C TOJ MILE 0/0-0/1+340’

DRAFT BIDDING DOCUMENT FOR WORKS

BILLS OF QUANTITIES

(A) 	Description and rate of Items bases on composite schedule of Rates.

PART-B  (BRIDGE WORK).

| Item No. | Qty’s |  | Description of item to be executed at site | Rate | Unit | Amount in Rupees |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 |  | 3 | 4 | 5 | 6 |
| 01/- | 2868 Cft: |  | Excavation in foundation of building bridge other structure i/cs dag belling dressing refilling around the structure with excavated earth watering and remaining lead upto one chain and left upto 5.0’ ordinary soil. | 3176/25 | P.%0 Cft: | 9109/= |
| 02/- | 936Cft: |  | Cement concrete with bricks are stone plaster 1½” to 2” guage ratio 1:2:4 | 9416/28 | P.% Cft: | 88136/= |
| 03/- | 2284Cft: |  | Pucca Brick work in foundation and plinth i/c cement sand mortor | 12501/41 | P.% Cft: | 285532/= |
| 04/- | 96Cft: |  | Cement concrete plain i/c placing compacting finishing curing complete i/c screening washing of stone aggregate with out suhttering ratio 1:2:4: | 14429/25 | P.% Cft: | 13852/= |
| 05/- | 57 Sft: |  | Erection and removal of centering for R.C.C or plain cement work (2nd: class) for partal wood vertical | 7000/00 | P.% Sft: | 3990/= |
| 06/- | 29 Cwt |  | Fabrication of m/s reinforcement cement concrete including cutting binding laying in position making joints & fastening of i/c binding wire also removal rust from bars. | 5001/70 | P.Cwt | 145069/= |
| 07/- | 400Rft: |  | Reinforced cement concrete work including all labour and material except the cost of steel reinforcement and its labour for bending and binding which will be paid separately. This rate also includes all kind of forms moulds lifting shuttering curing rend. | 337/00 | P.Rft: | 134800/= |
| 08/- | 832Sft: |  | Cement Plaster (1:3) upto 20’ height 1½” thick. | 2283/93 | P.% Sft: | 19002/= |
| 09/- |  | 27200Cft: | Earth work on road embankment by bulldozers i/c plugging mixing clod breaking dressing and compcting with optimum moisture content lead upto 100’ and lift upto 5’ in all types of soil | 3600/00 | P.%0 Cft: | 19002/= |
|  |  |  |  | TOTAL |  | 797410/= |

Amount Total (b)

__________________________% Above / Below on the rates of CSR. 	Total (b) =

Amount to be added /deducted on the

basis of premium quoted Total (b)

Total (b) =  in works in figure

Executive Engineer,

Contractor						                           Highways Division

Kashmore @ Kandhkot

*Imran*

CONSTRUCTION OF ROAD FROM GHULAM HUSSAIN MAZARI ROAD TO SUI ROAD TALUKA KASHMORE

DRAFT BIDDING DOCUMENT FOR WORKS

BILLS OF QUANTITIES

- Description and rate of Items bases on composite schedule of Rates

Part “A” (ROAD WORK)

| Item No. | Qty’s | Description of item to be executed at site | Rate | Unit | Amount in Rupees |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 |
| 01/- | 346000Cft: | Earth work for road embankment by bulldozers i/c plugging and mixing clod breaking dressing and compacting with optimum moisture content lead upto 100’ and lift upto 5.0’ in all types of soil except rock compacting upto 85% density as per modified AASHO specifications. | 3656/23 | P.0% Cft: | 1265056/= |
| 02/- | 40788Cft: | Earth work for road embankment by bulldozers i/c plugging and mixing clod breaking dressing and compacting with optimum moisture content lead upto 100’ and lift upto 5.0’ in all types of soil except rock compacting upto 95-100% density as per modified AASHO specifications. | 2529/77 | P.% Cft: | 103184/= |
| 03/- | 20394Cft: | Providing sand cushion i/c supplying & spreading pit/canal sand of approved quality from approved source of supply where ever available site of work i/c carriage site of work (R.A.A). | 1479/41 | P.% Cft: | 301711/= |
| 04/- | 20394Cft: | Providing Sub Base course by supplying and spreading stone metal 1 ½” gauge of approved quality from approved quarry in required thickness to proper camber and grade i/c hand packing mixing with 10’ screening & non plastic quarry of approved quality & gauge from approved source i/c watering and compacting to achieve 98-100% density as per modified AASHO specification ( rate i/cs all cost of material T&P and carriage upto site of wok.) | 10140/64 | P.% Cft: | 2068082/= |
| 05/- | 6798Rft: | Laying brick on end edging including supply of 9”x 4 ½”x3” 1st: class burnt bricks excavation for laying edging with small side parallel to the road. Rate includes all costs of materials T&P and carriage upto site of work. | 3021/71 | P.% Rft: | 205416/= |

Contd: P/2

P-2

| Item No. | Qty’s | Description of item to be executed at site | Rate | Unit | Amount in Rupees |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 |
| 06/- | 10197Cft: | Preparing base course supplying and spreading stone metal of approved quality of 6” in 2 layers from approved quarry properly grade to maximum size of 1½” in required thickness of proper camber & grade i/c S/S 15 Cft: screening & non plastic quarry fines filling & depressions with stone metal after initial rolling including watering & compacting the same so as to achieve 100% density as per modified AASHO specifications (This i/cs providing and using templates camber plates screen from as directed).Rate includes all costs of materials T&P and carriage upto site of work. | 11235/41 | P.% Cft: | 1145675/= |
| 07/- | 40788Sft: | Providing 1st: coat of surface dressing on or new existing surface with 30 lbs: bitumen and 4.0 cft: cursh bajri of required size including cleaning the road surface rolling etc: complete.Rate includes all costs of materials T&P & carriage. | 1720/41 | P.% Sft: | 701720/= |
| 08/- | 40788Sft: | Providing 1” thick consolidated premixed carpet in proper camber and grade i/c supply of 10 Cft: bajri 4.0 Cft: hill sand of approve quality and guage and bitumen 67 lbs: of 80/100 penetration i/c mixing the mechanical mixture in required proportion i/c heating the material and cleaning the road surface. Rate i/c all cost of material T&P and carriage upto site of work. | 4642/06 | P.% Sft: | 1893403/= |
|  |  |  | TOTAL |  | 7684247/= |

_____________% Above / Below on the rates of CSR.

Amount to be added /deducted on the

Total (A) 									Rs:

Total Amount in words  ____________________________________________________

__________________________________________________________________________

Executive Engineer,

Contractor						                           Highways Division

Kashmore @ Kandhkot

*Imran*

CONSTRUCTION OF ROAD FROM GHULAM HUSSAIN MAZARI ROAD TO SUI ROAD TALUKA KASHMORE

DRAFT BIDDING DOCUMENT FOR WORKS

BILLS OF QUANTITIES

(A) 	Description and rate of Items bases on composite schedule of Rates.

PART-B  (3.0 FT: SPAN CULVERTS).

| Item No. | Qty’s | Description of item to be executed at site | Rate | Unit | Amount in Rupees |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 |
| 01/- | 1106 Cft: | Excavation in foundation of building bridge other structure i/cs dag belling dressing refilling around the structure with excavated earth watering and remaining lead upto one chain and left upto 5.0’ ordinary soil. | 3176/25 | P.%0 Cft: | 3512/= |
| 02/- | 237 Cft: | Cement concrete with bricks are stone plaster 1½” to 2” guage ratio 1:2:4 | 9416/28 | P.% Cft: | 22518/= |
| 03/- | 502 Cft: | Pucca Brick work in foundation and plinth i/c cement sand mortor | 12501/41 | P.% Cft: | 62757/= |
| 04/- | 55 Cft: | Cement concrete plain i/c placing compacting finishing curing complete i/c screening washing of stone aggregate with out suhttering ratio 1:2:4: | 14429/25 | P.% Cft: | 7936/= |
| 05/- | 58 Sft: | Erection and removal of centering for R.C.C or plain cement work (2nd: class) for partal wood vertical | 7000/00 | P.% Sft: | 4060/= |
| 06/- | A. Tor Steel 4.16 Cwt B. M/ Steel 0.907 Cwt | Fabrication of m/s reinforcement cement concrete including cutting binding laying in position making joints & fastening of i/c binding wire also removal rust from bars. | 5001/70 4820/29 | P.Cwt: P.Cwt | 20807/= 4372/= |
| 07/- | 114 Rft: | Reinforced cement concrete work including all labour and material except the cost of steel reinforcement and its labour for bending and binding which will be paid separately. This rate also includes all kind of forms moulds lifting shuttering curing rend. | 337/00 | P.Rft: | 38418/= |
| 08/- | 330 Sft: | Cement Plaster (1:3) upto 20’ height 1½” thick. | 2283/93 | P.% Sft: | 7537/= |
|  |  |  | TOTAL |  | 171717/= |

Amount Total (b)

__________________________% Above / Below on the rates of CSR. 	Total (b) =

Amount to be added /deducted on the

basis of premium quoted Total (b)

Total (b) =  in works in figure

Executive Engineer,

Contractor						                           Highways Division

Kashmore @ Kandhkot

CONSTRUCTION OF ROAD FROM GHULAM HUSSAIN MAZARI ROAD TO SUI ROAD TALUKA KASHMORE

DRAFT BIDDING DOCUMENT FOR WORKS

BILLS OF QUANTITIES

(A) 	Description and rate of Items bases on composite schedule of Rates.

PART-C  (MISC:).

| Item No. | Qty’s | Description of item to be executed at site | Rate | Unit | Amount in Rupees |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 |
| 01/- | 01 Nos; | Proving and Fixing informatory sign rectangular shape 4’ x 3” comprising 02 Nos; 3” dia G.I Pipe Post i/c painting writing as directed each. | 16956/00 | P.Each | 16,956/= |
|  |  |  | TOTAL |  | 16,956/= |

Amount Total (C)

__________________________% Above / Below on the rates of CSR. 	Total (C) =

Amount to be added /deducted on the

basis of premium quoted Total (C)

Total (c) =  in works in figure

Executive Engineer,

Contractor						                           Highways Division

Kashmore @ Kandhkot

*Imran*

CONSTRUCTION OF ROAD FROM PAT FEEDER UPTO BORDER ROAD UPTO IMAM DIN MALIK ROAD MILE 0/0-0/6+99’ TALUKA KASHMORE

DRAFT BIDDING DOCUMENT FOR WORKS

BILLS OF QUANTITIES

- Description and rate of Items bases on composite schedule of Rates

Part “A” (ROAD WORK)

| Item No. | Qty’s | Description of item to be executed at site | Rate | Unit | Amount in Rupees |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 |
| 01/- | 467000Cft: | Earth work for road embankment by bulldozers i/c plugging and mixing clod breaking dressing and compacting with optimum moisture content lead upto 100’ and lift upto 5.0’ in all types of soil except rock compacting upto 85% density as per modified AASHO specifications. | 3656/23 | P.0% Cft: | 1708886/= |
| 02/- | 50169Cft: | Earth work for road embankment by bulldozers i/c plugging and mixing clod breaking dressing and compacting with optimum moisture content lead upto 100’ and lift upto 5.0’ in all types of soil except rock compacting upto 95-100% density as per modified AASHO specifications. | 2529/77 | P.% Cft: | 126916/= |
| 03/- | 25085Cft: | Providing sand cushion i/c supplying & spreading pit/canal sand of approved quality from approved source of supply where ever available site of work i/c carriage site of work (R.A.A). | 1479/41 | P.% Cft: | 371110/= |
| 04/- | 25085Cft: | Providing Sub Base course by supplying and spreading stone metal 1 ½” gauge of approved quality from approved quarry in required thickness to proper camber and grade i/c hand packing mixing with 10’ screening & non plastic quarry of approved quality & gauge from approved source i/c watering and compacting to achieve 98-100% density as per modified AASHO specification ( rate i/cs all cost of material T&P and carriage upto site of wok.) | 9954/39 | P.% Cft: | 2497059/= |
| 05/- | 8362Rft: | Laying brick on end edging including supply of 9”x 4 ½”x3” 1st: class burnt bricks excavation for laying edging with small side parallel to the road. Rate includes all costs of materials T&P and carriage upto site of work. | 2969/61 | P.% Rft: | 248318/= |

Contd: P/2

P-2

| Item No. | Qty’s | Description of item to be executed at site | Rate | Unit | Amount in Rupees |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 |
| 06/- | 12542Cft: | Preparing base course supplying and spreading stone metal of approved quality of 6” in 2 layers from approved quarry properly grade to maximum size of 1½” in required thickness of proper camber & grade i/c S/S 15 Cft: screening & non plastic quarry fines filling & depressions with stone metal after initial rolling including watering & compacting the same so as to achieve 100% density as per modified AASHO specifications (This i/cs providing and using templates camber plates screen from as directed).Rate includes all costs of materials T&P and carriage upto site of work. | 11042/66 | P.% Cft: | 1384970/= |
| 07/- | 50169Sft: | Providing 1st: coat of surface dressing on or new existing surface with 30 lbs: bitumen and 4.0 cft: cursh bajri of required size including cleaning the road surface rolling etc: complete.Rate includes all costs of materials T&P & carriage. | 1714/67 | P.% Sft: | 860233/= |
| 08/- | 50169Sft: | Providing 1” thick consolidated premixed carpet in proper camber and grade i/c supply of 10 Cft: bajri 4.0 Cft: hill sand of approve quality and guage and bitumen 67 lbs: of 80/100 penetration i/c mixing the mechanical mixture in required proportion i/c heating the material and cleaning the road surface. Rate i/c all cost of material T&P and carriage upto site of work. | 4633/06 | P.% Sft: | 2324631/= |
|  |  |  | TOTAL |  | 9522123/= |

_____________% Above / Below on the rates of CSR.

Amount to be added /deducted on the

Total (A) 									Rs:

Total Amount in words  ____________________________________________________

__________________________________________________________________________

Executive Engineer,

Contractor						                           Highways Division

Kashmore @ Kandhkot

*Imran*

CONSTRUCTION OF ROAD FROM PAT FEEDER UPTO BORDER ROAD UPTO IMAM DIN MALIK ROAD MILE 0/0-0/6+99’ TALUKA KASHMORE

DRAFT BIDDING DOCUMENT FOR WORKS

BILLS OF QUANTITIES

(A) 	Description and rate of Items bases on composite schedule of Rates.

PART-B  (3.0 FT: SPAN CULVERTS).

| Item No. | Qty’s | Description of item to be executed at site | Rate | Unit | Amount in Rupees |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 |
| 01/- | 1106 Cft: | Excavation in foundation of building bridge other structure i/cs dag belling dressing refilling around the structure with excavated earth watering and remaining lead upto one chain and left upto 5.0’ ordinary soil. | 3176/25 | P.%0 Cft: | 3512/= |
| 02/- | 237 Cft: | Cement concrete with bricks are stone plaster 1½” to 2” guage ratio 1:2:4 | 9416/28 | P.% Cft: | 22518/= |
| 03/- | 502 Cft: | Pucca Brick work in foundation and plinth i/c cement sand mortor | 12501/41 | P.% Cft: | 62757/= |
| 04/- | 55 Cft: | Cement concrete plain i/c placing compacting finishing curing complete i/c screening washing of stone aggregate with out suhttering ratio 1:2:4: | 14429/25 | P.% Cft: | 7936/= |
| 05/- | 58 Sft: | Erection and removal of centering for R.C.C or plain cement work (2nd: class) for partal wood vertical | 7000/00 | P.% Sft: | 4060/= |
| 06/- | A. Tor Steel 4.16 Cwt B. M/ Steel 0.907 Cwt | Fabrication of m/s reinforcement cement concrete including cutting binding laying in position making joints & fastening of i/c binding wire also removal rust from bars. | 5001/70 4820/29 | P.Cwt: P.Cwt | 20807/= 4372/= |
| 07/- | 114 Rft: | Reinforced cement concrete work including all labour and material except the cost of steel reinforcement and its labour for bending and binding which will be paid separately. This rate also includes all kind of forms moulds lifting shuttering curing rend. | 337/00 | P.Rft: | 38418/= |
| 08/- | 330 Sft: | Cement Plaster (1:3) upto 20’ height 1½” thick. | 2283/93 | P.% Sft: | 7537/= |
|  |  |  | TOTAL |  | 171717/= |

Amount Total (b)

__________________________% Above / Below on the rates of CSR. 	Total (b) =

Amount to be added /deducted on the

basis of premium quoted Total (b)

Total (b) =  in works in figure

Executive Engineer,

Contractor						                           Highways Division

Kashmore @ Kandhkot

CONSTRUCTION OF 20’RFT BRIDGE GHOUSPUR MINOR IN VILLAGE MOULANA BAGE OGHAE TALUKA KANDHKOT

DRAFT BIDDING DOCUMENT FOR WORKS

BILLS OF QUANTITIES

(A) 	Description and rate of Items bases on composite schedule of Rates.

PART-A  (BRIDGE WORK).

| Item No. | Qty’s |  | Description of item to be executed at site | Rate | Unit | Amount in Rupees |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 |  | 3 | 4 | 5 | 6 |
| 01/- | 3720 Cft: |  | Excavation in foundation of building bridge other structure i/cs dag belling dressing refilling around the structure with excavated earth watering and remaining lead upto one chain and left upto 5.0’ ordinary soil. | 3176/25 | P.%0 Cft: | 11815/= |
| 02/- | 824Cft: |  | Cement concrete with bricks are stone plaster 1½” to 2” guage ratio 1:2:4 | 9416/28 | P.% Cft: | 77590/= |
| 03/- | 2634Cft: |  | Pucca Brick work in foundation and plinth i/c cement sand mortor | 12501/41 | P.% Cft: | 329287/= |
| 04/- | 145Cft: |  | Cement concrete plain i/c placing compacting finishing curing complete i/c screening washing of stone aggregate with out suhttering ratio 1:2:4: | 14429/25 | P.% Cft: | 20922/= |
| 05/- | 63 Sft: |  | Erection and removal of centering for R.C.C or plain cement work (2nd: class) for partal wood vertical | 7000/00 | P.% Sft: | 4410/= |
| 06/- | 40.62 Cwt |  | Fabrication of m/s reinforcement cement concrete including cutting binding laying in position making joints & fastening of i/c binding wire also removal rust from bars. | 4820/20 | P.Cwt | 195796/= |
| 07/- | 468Rft: |  | Reinforced cement concrete work including all labour and material except the cost of steel reinforcement and its labour for bending and binding which will be paid separately. This rate also includes all kind of forms moulds lifting shuttering curing rend. | 337/00 | P.Rft: | 157716/= |
| 08/- | 1624Sft: |  | Cement Plaster (1:3) upto 20’ height 1½” thick. | 2283/93 | P.% Sft: | 37091/= |
| 09/- |  | -- | Earth work on road embankment by bulldozers i/c plugging mixing clod breaking dressing and compcting with optimum moisture content lead upto 100’ and lift upto 5’ in all types of soil | -- | -- | -- |
|  |  |  |  | TOTAL |  | 834628/= |

Amount Total (a)

__________________________% Above / Below on the rates of CSR. 	Total (a) =

Amount to be added /deducted on the

basis of premium quoted Total (a)

Total (a) =  in works in figure

Executive Engineer,

Contractor						                           Highways Division

Kashmore @ Kandhkot .

*Imran*

W/R FROM KANDHKOT BYE PASS TO ZANGIPUR MUHALLA VIA FOUJ ALI KHAN DAHANI HOUSE (KANDHKOT TOWN) MILE 0/0-0/5

DRAFT BIDDING DOCUMENT FOR WORKS

BILLS OF QUANTITIES

- Description and rate of Items bases on composite schedule of Rates

Part “A” (ROAD WORK)

| Item No. | Qty’s | Description of item to be executed at site | Rate | Unit | Amount in Rupees |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 |
| 01/- | 81576Cft: | Earth work excavation dressed upto shingle through kessi phwram or shawrvel remaining the disbars from site of work fro clearance of street road and disposal away from site (S.I No; 1(a) P-1 | 6928/48 | P.0% Cft: | 565198/= |
| 02/- | 20394Cft:-- | Providing sand cushion i/c supplying & spreading pit/canal sand of approved quality from approved source of supply where ever available site of work i/c carriage site of work (R.A.A). | 1479/41 | P.% Cft: | 301710/= |
| 03/- | 20394Cft: | Providing Sub Base course by supplying and spreading stone metal 1 ½” gauge of approved quality from approved quarry in required thickness to proper camber and grade i/c hand packing mixing with 20 Cft Pit sand, screening & non plastic quarry of approved quality & gauge from approved source i/c watering and compacting to achieve 98-100% density as per modified AASHO specification ( rate i/cs all cost of material T&P and carriage upto site of wok.) | 8210/60 | P.% Cft: | 1674470/= |
| 04/- | 20394Cft: | Providing Sub Base course by supplying and spreading stone metal 1 ½” gauge of approved quality from approved quarry in required thickness to proper camber and grade i/c hand packing mixing with 10’ screening & non plastic quarry of approved quality & gauge from approved source i/c watering and compacting to achieve 98-100% density as per modified AASHO specification ( rate i/cs all cost of material T&P and carriage upto site of wok.) | 8604/13 | P.% Cft: | 1754726/= |
| 05/- | 6798Rft: | Laying brick on end edging including supply of 9”x 4 ½”x3” 1st: class burnt bricks excavation for laying edging with small side parallel to the road. Rate includes all costs of materials T&P and carriage upto site of work. | 2668/34 | P.% Rft: | 181394/= |

Contd: P/2

P-2

| Item No. | Qty’s | Description of item to be executed at site | Rate | Unit | Amount in Rupees |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 |
| 06/- | 42318Cft: | Preparing base course supplying and spreading stone metal of approved quality of 6” in 2 layers from approved quarry properly grade to maximum size of 1½” in required thickness of proper camber & grade i/c S/S 15 Cft: screening & non plastic quarry fines filling & depressions with stone metal after initial rolling including watering & compacting the same so as to achieve 100% density as per modified AASHO specifications (This i/cs providing and using templates camber plates screen from as directed).Rate includes all costs of materials T&P and carriage upto site of work. | 9645/18 | P.% Cft: | 4081647/= |
| 07/- | 81576Sft: | Providing 2 coat of surface dressing on or new existing surface with 55 lbs: bitumen and 6.75cft: cursh bajri of required size including cleaning the road surface rolling etc: complete.Rate includes all costs of materials T&P & carriage. | 2996/59 | P.% Sft: | 2444498/= |
| 08/- | 81576Sft: | Laying to proper line and grade plant mixes Ashplat concrete paver finished Hydraulic / Electronic control) prepared to specified formula according to job mix formula according to job mix formula approved by Engineer incharge i/c rolling and finished to proper line garde level and camber etc compete. Rate i/cs tack coat with bitumen 80/100 and all cost of material roller. T&P and carriage of material from source of supply tom plant and plant to site of work (S.I No: 26(b) P-11) | 9224/53 | P.% Sft: | 7525003/= |
| 09/- | 205000 Cft | Earth work on road embankment by bulldozers i/c plugging mixing clod breaking dressing and compcting with optimum moisture content lead upto 100’ and lift upto 5’ in all types of soil | 7327/99 | P0% Cft | 1502238/= |
|  |  |  | TOTAL |  | 20030884/= |

_____________% Above / Below on the rates of CSR.

Amount to be added /deducted on the

Total (A) 									Rs:

Total Amount in words  ____________________________________________________

__________________________________________________________________________

Executive Engineer,

Contractor						                           Highways Division

Kashmore @ Kandhkot

*Imran*

W/R FROM KANDHKOT BYE PASS TO ZANGIPUR MUHALLA VIA FOUJ ALI KHAN DAHANI HOUSE (KANDHKOT TOWN) MILE 0/0-0/5

DRAFT BIDDING DOCUMENT FOR WORKS

BILLS OF QUANTITIES

(A) 	Description and rate of Items bases on composite schedule of Rates.

PART-B  (MISC:).

| Item No. | Qty’s | Description of item to be executed at site | Rate | Unit | Amount in Rupees |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 |
| 01/- | 01 Nos; | Proving and Fixing informatory sign rectangular shape 4’ x 3” comprising 02 Nos; 3” dia G.I Pipe Post i/c painting writing as directed each. | 16956/00 | P.Each | 16,956/= |
| 02/- | 13200 Rft | Colour Line | 41/24 | P.Rft | 544368/= |
| 03/- | 2000 Rft | Cate Eyes | 596/23 | P.Rft | 1192460/= |
|  |  |  | TOTAL |  | 1753984/= |

Amount Total (B)

__________________________% Above / Below on the rates of CSR. 	Total (B) =

Amount to be added /deducted on the

basis of premium quoted Total (B)

Total (b) =  in works in figure

Executive Engineer,

Contractor						                           Highways Division

Kashmore @ Kandhkot

*Imran*

W/R FROM SHAFI MUHAMMAD / KANDHKOT MALGUZAR ROAD TO CONNECT BAHOO POLICE CHECK POST ALONG WITH KANDHKOT SHAKH MILE 0/0-0/4

DRAFT BIDDING DOCUMENT FOR WORKS

BILLS OF QUANTITIES

- Description and rate of Items bases on composite schedule of Rates

Part “A” (ROAD WORK)

| Item No. | Qty’s | Description of item to be executed at site | Rate | Unit | Amount in Rupees |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 |
| 01/- | 32630Cft: | Earth work excavation dressed upto shingle through kessi phwram or shawrvel remaining the disbars from site of work fro clearance of street road and disposal away from site (S.I No; 1(a) P-1 | 6928/48 | P.0% Cft: | 226076/= |
| 02/- | 8158Cft: | Providing sand cushion i/c supplying & spreading pit/canal sand of approved quality from approved source of supply where ever available site of work i/c carriage site of work (R.A.A). | 1479/41 | P.% Cft: | 120690/= |
| 03/- | 8158Cft: | Providing Sub Base course by supplying and spreading stone metal 1 ½” gauge of approved quality from approved quarry in required thickness to proper camber and grade i/c hand packing mixing with 20 Cft Pit sand, screening & non plastic quarry of approved quality & gauge from approved source i/c watering and compacting to achieve 98-100% density as per modified AASHO specification ( rate i/cs all cost of material T&P and carriage upto site of wok.) | 8210/60 | P.% Cft: | 669821/= |
| 04/- | 8158Cft: | Providing Sub Base course by supplying and spreading stone metal 1 ½” gauge of approved quality from approved quarry in required thickness to proper camber and grade i/c hand packing mixing with 10’ screening & non plastic quarry of approved quality & gauge from approved source i/c watering and compacting to achieve 98-100% density as per modified AASHO specification ( rate i/cs all cost of material T&P and carriage upto site of wok.) | 8604/13 | P.% Cft: | 701925/= |
| 05/- | 5438Rft: | Laying brick on end edging including supply of 9”x 4 ½”x3” 1st: class burnt bricks excavation for laying edging with small side parallel to the road. Rate includes all costs of materials T&P and carriage upto site of work. | 2668/34 | P.% Rft: | 145104/= |

Contd: P/2

P-2

| Item No. | Qty’s | Description of item to be executed at site | Rate | Unit | Amount in Rupees |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 |
| 06/- | 34465Cft: | Preparing base course supplying and spreading stone metal of approved quality of 6” in 2 layers from approved quarry properly grade to maximum size of 1½” in required thickness of proper camber & grade i/c S/S 15 Cft: screening & non plastic quarry fines filling & depressions with stone metal after initial rolling including watering & compacting the same so as to achieve 100% density as per modified AASHO specifications (This i/cs providing and using templates camber plates screen from as directed).Rate includes all costs of materials T&P and carriage upto site of work. | 9645/18 | P.% Cft: | 3324211/= |
| 07/- | 65260Sft: | Providing 2 coat of surface dressing on or new existing surface with 55 lbs: bitumen and 6.75cft: cursh bajri of required size including cleaning the road surface rolling etc: complete.Rate includes all costs of materials T&P & carriage. | 2996/59 | P.% Sft: | 1955418/= |
| 08/- | 65260Sft: | Laying to proper line and grade plant mixes Ashplat concrete paver finished Hydraulic / Electronic control) prepared to specified formula according to job mix formula according to job mix formula approved by Engineer incharge i/c rolling and finished to proper line garde level and camber etc compete. Rate i/cs tack coat with bitumen 80/100 and all cost of material roller. T&P and carriage of material from source of supply tom plant and plant to site of work (S.I No: 26(b) P-11) | 9224/47 | P.% Sft: | 6019889/= |
| 09/- | 251000 Cft | Earth work on road embankment by bulldozers i/c plugging mixing clod breaking dressing and compcting with optimum moisture content lead upto 100’ and lift upto 5’ in all types of soil | 7327/99 | P0% Cft | 1839325/= |
|  |  |  | TOTAL |  | 15002459/= |

_____________% Above / Below on the rates of CSR.

Amount to be added /deducted on the

Total (A) 									Rs:

Total Amount in words  ____________________________________________________

__________________________________________________________________________

Executive Engineer,

Contractor						                           Highways Division

Kashmore @ Kandhkot

*Imran*

W/R FROM SHAFI MUHAMMAD / KANDHKOT MALGUZAR ROAD TO CONNECT BAHOO POLICE CHECK POST ALONG WITH KANDHKOT SHAKH MILE 0/0-0/4

DRAFT BIDDING DOCUMENT FOR WORKS

BILLS OF QUANTITIES

(A) 	Description and rate of Items bases on composite schedule of Rates.

PART-B  (MISC:).

| Item No. | Qty’s | Description of item to be executed at site | Rate | Unit | Amount in Rupees |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 |
| 01/- | 01 Nos; | Proving and Fixing informatory sign rectangular shape 4’ x 3” comprising 02 Nos; 3” dia G.I Pipe Post i/c painting writing as directed each. | 16956/00 | P.Each | 16,956/= |
|  |  |  | TOTAL |  | 1753984/= |

Amount Total (B)

__________________________% Above / Below on the rates of CSR. 	Total (B) =

Amount to be added /deducted on the

basis of premium quoted Total (B)

Total (b) =  in works in figure

Executive Engineer,

Contractor						                           Highways Division

Kashmore @ Kandhkot

*Imran*

W/R FROM SHAFI FROM KANDHKOT SAIFAL ROAD TO CONNECT AWAN MUHALLA ROAD MILE 0/0-0/4

DRAFT BIDDING DOCUMENT FOR WORKS

BILLS OF QUANTITIES

- Description and rate of Items bases on composite schedule of Rates

Part “A” (ROAD WORK)

| Item No. | Qty’s | Description of item to be executed at site | Rate | Unit | Amount in Rupees |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 |
| 01/- | 32630Cft: | Earth work excavation dressed upto shingle through kessi phwram or shawrvel remaining the disbars from site of work fro clearance of street road and disposal away from site (S.I No; 1(a) P-1 | 6928/48 | P.0% Cft: | 226076/= |
| 02/- | 8158Cft: | Providing sand cushion i/c supplying & spreading pit/canal sand of approved quality from approved source of supply where ever available site of work i/c carriage site of work (R.A.A). | 1479/41 | P.% Cft: | 120690/= |
| 03/- | 8158Cft: | Providing Sub Base course by supplying and spreading stone metal 1 ½” gauge of approved quality from approved quarry in required thickness to proper camber and grade i/c hand packing mixing with 20 Cft Pit sand, screening & non plastic quarry of approved quality & gauge from approved source i/c watering and compacting to achieve 98-100% density as per modified AASHO specification ( rate i/cs all cost of material T&P and carriage upto site of wok.) | 8210/60 | P.% Cft: | 669821/= |
| 04/- | 8158Cft: | Providing Sub Base course by supplying and spreading stone metal 1 ½” gauge of approved quality from approved quarry in required thickness to proper camber and grade i/c hand packing mixing with 10’ screening & non plastic quarry of approved quality & gauge from approved source i/c watering and compacting to achieve 98-100% density as per modified AASHO specification ( rate i/cs all cost of material T&P and carriage upto site of wok.) | 8604/13 | P.% Cft: | 701925/= |
| 05/- | 5438Rft: | Laying brick on end edging including supply of 9”x 4 ½”x3” 1st: class burnt bricks excavation for laying edging with small side parallel to the road. Rate includes all costs of materials T&P and carriage upto site of work. | 2668/34 | P.% Rft: | 145104/= |

Contd: P/2

P-2

| Item No. | Qty’s | Description of item to be executed at site | Rate | Unit | Amount in Rupees |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 |
| 06/- | 34465Cft: | Preparing base course supplying and spreading stone metal of approved quality of 6” in 2 layers from approved quarry properly grade to maximum size of 1½” in required thickness of proper camber & grade i/c S/S 15 Cft: screening & non plastic quarry fines filling & depressions with stone metal after initial rolling including watering & compacting the same so as to achieve 100% density as per modified AASHO specifications (This i/cs providing and using templates camber plates screen from as directed).Rate includes all costs of materials T&P and carriage upto site of work. | 9645/18 | P.% Cft: | 3324211/= |
| 07/- | 74160Sft: | Providing 2 coat of surface dressing on or new existing surface with 55 lbs: bitumen and 6.75cft: cursh bajri of required size including cleaning the road surface rolling etc: complete.Rate includes all costs of materials T&P & carriage. | 2996/35 | P.% Sft: | 2222093/= |
| 08/- | 65260Sft: | Laying to proper line and grade plant mixes Ashplat concrete paver finished Hydraulic / Electronic control) prepared to specified formula according to job mix formula according to job mix formula approved by Engineer incharge i/c rolling and finished to proper line garde level and camber etc compete. Rate i/cs tack coat with bitumen 80/100 and all cost of material roller. T&P and carriage of material from source of supply tom plant and plant to site of work (S.I No: 26(b) P-11) | 9224/47 | P.% Sft: | 6019889/= |
| 09/- | 247600 Cft | Earth work on road embankment by bulldozers i/c plugging mixing clod breaking dressing and compcting with optimum moisture content lead upto 100’ and lift upto 5’ in all types of soil | 7327/99 | P0% Cft | 1814410/= |
|  |  |  | TOTAL |  | 15244219/= |

_____________% Above / Below on the rates of CSR.

Amount to be added /deducted on the

Total (A) 									Rs:

Total Amount in words  ____________________________________________________

__________________________________________________________________________

Executive Engineer,

Contractor						                           Highways Division

Kashmore @ Kandhkot

*Imran*

W/R FROM SHAFI FROM KANDHKOT SAIFAL ROAD TO CONNECT AWAN MUHALLA ROAD MILE 0/0-0/4

DRAFT BIDDING DOCUMENT FOR WORKS

BILLS OF QUANTITIES

(A) 	Description and rate of Items bases on composite schedule of Rates.

PART-B  (MISC:).

| Item No. | Qty’s | Description of item to be executed at site | Rate | Unit | Amount in Rupees |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 |
| 01/- | 01 Nos; | Proving and Fixing informatory sign rectangular shape 4’ x 3” comprising 02 Nos; 3” dia G.I Pipe Post i/c painting writing as directed each. | 16956/00 | P.Each | 16,956/= |
|  |  |  | TOTAL |  | 1753984/= |

Amount Total (B)

__________________________% Above / Below on the rates of CSR. 	Total (B) =

Amount to be added /deducted on the

basis of premium quoted Total (B)

Total (b) =  in works in figure

Executive Engineer,

Contractor						                           Highways Division

Kashmore @ Kandhkot

*Imran*

W/R FROM KANDHKOT BYE PASS ROAD VIA JHOLEY LAL PETROL PUMP TO RAILWAY CROSSING ROAD VIA CLOCK TOWER KANDHKOT ROAD MILE 0/0-1/4+330’

DRAFT BIDDING DOCUMENT FOR WORKS

BILLS OF QUANTITIES

- Description and rate of Items bases on composite schedule of Rates

Part “A” (ROAD WORK)

| Item No. | Qty’s | Description of item to be executed at site | Rate | Unit | Amount in Rupees |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 |
| 01/- | 83737Cft: | Dismentling of brick work in lin or cement motor | 1285/63 | P.% Cft: | 1076547/= |
| 02/- | 33000Cft: | Dismentling of cement concrete plain 1:4:8 | 1663/75 | P.% Cft: | 549038/= |
| 03/- | 11896Cft: | Earth work excavation dressed upto shingle through kessi phwram or shawrvel remaining the disbars from site of work fro clearance of street road and disposal away from site (S.I No; 1(a) P-1 | 6928/48 | P.0% Cft: | 82421/= |
| 04/- | 33990Cft: | Providing sand cushion i/c supplying & spreading pit/canal sand of approved quality from approved source of supply where ever available site of work i/c carriage site of work (R.A.A). | 1479/41 | P.% Cft: | 502851/= |
| 05/- | 33990Cft: | Providing Sub Base course by supplying and spreading stone metal 1 ½” gauge of approved quality from approved quarry in required thickness to proper camber and grade i/c hand packing mixing with 20 Cft Pit sand, screening & non plastic quarry of approved quality & gauge from approved source i/c watering and compacting to achieve 98-100% density as per modified AASHO specification ( rate i/cs all cost of material T&P and carriage upto site of wok.) | 8286/01 | P.% Cft: | 2816416/= |

Contd: P/2

P-2

| Item No. | Qty’s | Description of item to be executed at site | Rate | Unit | Amount in Rupees |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 |
| 06/- | 33990Cft: | Providing Sub Base course by supplying and spreading stone metal 1 ½” gauge of approved quality from approved quarry in required thickness to proper camber and grade i/c hand packing mixing with 10’ screening & non plastic quarry of approved quality & gauge from approved source i/c watering and compacting to achieve 98-100% density as per modified AASHO specification ( rate i/cs all cost of material T&P and carriage upto site of wok.) | 8557/57 | P.% Cft: | 2908718/= |
| 07/- | 16995Cft: | Preparing base course supplying and spreading stone metal of approved quality of 6” in 2 layers from approved quarry properly grade to maximum size of 1½” in required thickness of proper camber & grade i/c S/S 15 Cft: screening & non plastic quarry fines filling & depressions with stone metal after initial rolling including watering & compacting the same so as to achieve 100% density as per modified AASHO specifications (This i/cs providing and using templates camber plates screen from as directed).Rate includes all costs of materials T&P and carriage upto site of work. | 9596/99 | P.% Cft: | 1631008/= |
| 08/- | 67980Sft: | Providing 2 coat of surface dressing on or new existing surface with 55 lbs: bitumen and 6.75cft: cursh bajri of required size including cleaning the road surface rolling etc: complete.Rate includes all costs of materials T&P & carriage. | 2994/13 | P.% Sft: | 2035410/= |
| 09/- | 67980Sft: | Laying to proper line and grade plant mixes Ashplat concrete paver finished Hydraulic / Electronic control) prepared to specified formula according to job mix formula according to job mix formula approved by Engineer incharge i/c rolling and finished to proper line garde level and camber etc compete. Rate i/cs tack coat with bitumen 80/100 and all cost of material roller. T&P and carriage of material from source of supply tom plant and plant to site of work (S.I No: 26(b) P-11) | 9224/46 | P.% Sft: | 6270788/= |
|  |  |  | TOTAL |  | 17873196/= |

_____________% Above / Below on the rates of CSR.

Amount to be added /deducted on the

Total (A) 									Rs:

Total Amount in words  ____________________________________________________

__________________________________________________________________________

Executive Engineer,

Contractor						                           Highways Division

Kashmore @ Kandhkot

*Imran*

W/R FROM KANDHKOT BYE PASS ROAD VIA JHOLEY LAL PETROL PUMP TO RAILWAY CROSSING ROAD VIA CLOCK TOWER KANDHKOT ROAD MILE 0/0-1/4+330’

DRAFT BIDDING DOCUMENT FOR WORKS

BILLS OF QUANTITIES

(A) 	Description and rate of Items bases on composite schedule of Rates.

PART-B  (MISC:).

| Item No. | Qty’s | Description of item to be executed at site | Rate | Unit | Amount in Rupees |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 |
| 01/- | 01 Nos; | Proving and Fixing informatory sign rectangular shape 4’ x 3” comprising 02 Nos; 3” dia G.I Pipe Post i/c painting writing as directed each. | 16956/00 | P.Each | 16,956/= |
| 02/- | 16500 Rft | Colour Line | 41/24 | P.Rft | 680460/= |
|  |  |  | TOTAL |  | 697416/= |

Amount Total (B)

__________________________% Above / Below on the rates of CSR. 	Total (B) =

Amount to be added /deducted on the

basis of premium quoted Total (B)

Total (b) =  in works in figure

Executive Engineer,

Contractor						                           Highways Division

Kashmore @ Kandhkot

*Imran*

W/R FROM KANDHKOT BYE PASS ROAD CLOCK TOWER CHOWK TO SINDH BANK AND WIDENING OF DUAL CARRIAGE WAY @ MITHA CHOWK ROAD MILE 0/0-0/3

DRAFT BIDDING DOCUMENT FOR WORKS

BILLS OF QUANTITIES

- Description and rate of Items bases on composite schedule of Rates

Part “A” (ROAD WORK)

| Item No. | Qty’s | Description of item to be executed at site | Rate | Unit | Amount in Rupees |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 |
| 01/- | 12656Cft: | Dismentling of brick work in lin or cement motor | 1285/63 | P.% Cft: | 162709/= |
| 02/- | 5280Cft: | Dismentling of cement concrete plain 1:4:8 | 1663/75 | P.% Cft: | 87846/= |
| 03/- | 95040Cft: | Earth work excavation dressed upto shingle through kessi phwram or shawrvel remaining the disbars from site of work fro clearance of street road and disposal away from site (S.I No; 1(a) P-1 | 6928/48 | P.0% Cft: | 658483/= |
| 04/- | 23760Cft: | Providing sand cushion i/c supplying & spreading pit/canal sand of approved quality from approved source of supply where ever available site of work i/c carriage site of work (R.A.A). | 1479/41 | P.% Cft: | 351507/= |
| 05/- | 23760Cft: | Providing Sub Base course by supplying and spreading stone metal 1 ½” gauge of approved quality from approved quarry in required thickness to proper camber and grade i/c hand packing mixing with 20 Cft Pit sand, screening & non plastic quarry of approved quality & gauge from approved source i/c watering and compacting to achieve 98-100% density as per modified AASHO specification ( rate i/cs all cost of material T&P and carriage upto site of wok.) | 8286/01 | P.% Cft: | 1968756/= |

Contd: P/2

P-2

| Item No. | Qty’s | Description of item to be executed at site | Rate | Unit | Amount in Rupees |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 |
| 06/- | 23760Cft: | Providing Sub Base course by supplying and spreading stone metal 1 ½” gauge of approved quality from approved quarry in required thickness to proper camber and grade i/c hand packing mixing with 10’ screening & non plastic quarry of approved quality & gauge from approved source i/c watering and compacting to achieve 98-100% density as per modified AASHO specification ( rate i/cs all cost of material T&P and carriage upto site of wok.) | 8557/57 | P.% Cft: | 2033278/= |
| 07/- | 23760Cft: | Preparing base course supplying and spreading stone metal of approved quality of 6” in 2 layers from approved quarry properly grade to maximum size of 1½” in required thickness of proper camber & grade i/c S/S 15 Cft: screening & non plastic quarry fines filling & depressions with stone metal after initial rolling including watering & compacting the same so as to achieve 100% density as per modified AASHO specifications (This i/cs providing and using templates camber plates screen from as directed).Rate includes all costs of materials T&P and carriage upto site of work. | 9596/99 | P.% Cft: | 2280245/= |
| 08/- | 71280Sft: | Providing 2 coat of surface dressing on or new existing surface with 55 lbs: bitumen and 6.75cft: cursh bajri of required size including cleaning the road surface rolling etc: complete.Rate includes all costs of materials T&P & carriage. | 2993/91 | P.% Sft: | 21340591/= |
| 09/- | 71280Sft: | Laying to proper line and grade plant mixes Ashplat concrete paver finished Hydraulic / Electronic control) prepared to specified formula according to job mix formula according to job mix formula approved by Engineer incharge i/c rolling and finished to proper line garde level and camber etc compete. Rate i/cs tack coat with bitumen 80/100 and all cost of material roller. T&P and carriage of material from source of supply tom plant and plant to site of work (S.I No: 26(b) P-11) | 9224/40 | P.% Sft: | 6575152/= |
| 10/- | 3960Rft: | Const standered type concrete block B-type 1:2:4 i/c cost of molding one hand of neru shuttering erection removal etc complete | 174/00 | P.Rft | 689040/= |
|  |  |  | TOTAL |  | 16941075/= |

_____________% Above / Below on the rates of CSR.

Amount to be added /deducted on the

Total (A) 									Rs:

Total Amount in words  ____________________________________________________

__________________________________________________________________________

Executive Engineer,

Contractor						                           Highways Division

Kashmore @ Kandhkot

*Imran*

W/R FROM KANDHKOT BYE PASS ROAD CLOCK TOWER CHOWK TO SINDH BANK AND WIDENING OF DUAL CARRIAGE WAY @ MITHA CHOWK ROAD MILE 0/0-0/3

DRAFT BIDDING DOCUMENT FOR WORKS

BILLS OF QUANTITIES

(A) 	Description and rate of Items bases on composite schedule of Rates.

PART-B  (MISC:).

| Item No. | Qty’s | Description of item to be executed at site | Rate | Unit | Amount in Rupees |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 |
| 01/- | 01 Nos; | Proving and Fixing informatory sign rectangular shape 4’ x 3” comprising 02 Nos; 3” dia G.I Pipe Post i/c painting writing as directed each. | 16956/00 | P.Each | 16,956/= |
| 02/- | 21120 Rft | Colour Line | 41/24 | P.Rft | 870989/= |
| 03/- | 2600 Rft | Cate Eyes | 596/23 | P.Rft | 1550198/= |
|  |  |  | TOTAL |  | 2438143/= |

Amount Total (B)

__________________________% Above / Below on the rates of CSR. 	Total (B) =

Amount to be added /deducted on the

basis of premium quoted Total (B)

Total (b) =  in works in figure

Executive Engineer,

Contractor						                           Highways Division

Kashmore @ Kandhkot

*Imran*

WIDENING OF ROAD FROM CLOCK TOWER TO SHAMS PUMP ROAD MILE 0/0-0/2

DRAFT BIDDING DOCUMENT FOR WORKS

BILLS OF QUANTITIES

- Description and rate of Items bases on composite schedule of Rates

Part “A” (ROAD WORK)

| Item No. | Qty’s | Description of item to be executed at site | Rate | Unit | Amount in Rupees |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 |
| 01/- | 12903Cft: | Dismentling of brick work in lin or cement motor | 1285/63 | P.% Cft: | 165885/= |
| 02/- | 10560Cft: | Dismentling of cement concrete plain 1:4:8 | 1663/75 | P.% Cft: | 175692/= |
| 03/- | 89760Cft: | Earth work excavation dressed upto shingle through kessi phwram or shawrvel remaining the disbars from site of work fro clearance of street road and disposal away from site (S.I No; 1(a) P-1 | 6928/48 | P.0% Cft: | 621900/= |
| 04/- | 22440Cft: | Providing sand cushion i/c supplying & spreading pit/canal sand of approved quality from approved source of supply where ever available site of work i/c carriage site of work (R.A.A). | 1479/41 | P.% Cft: | 331980/= |
| 05/- | 22440Cft: | Providing Sub Base course by supplying and spreading stone metal 1 ½” gauge of approved quality from approved quarry in required thickness to proper camber and grade i/c hand packing mixing with 20 Cft Pit sand, screening & non plastic quarry of approved quality & gauge from approved source i/c watering and compacting to achieve 98-100% density as per modified AASHO specification ( rate i/cs all cost of material T&P and carriage upto site of wok.) | 8210/59 | P.% Cft: | 1842456/= |
| 06/- | 22440Cft: | Providing Sub Base course by supplying and spreading stone metal 1 ½” gauge of approved quality from approved quarry in required thickness to proper camber and grade i/c hand packing mixing with 10’ screening & non plastic quarry of approved quality & gauge from approved source i/c watering and compacting to achieve 98-100% density as per modified AASHO specification ( rate i/cs all cost of material T&P and carriage upto site of wok.) | 8604/11 | P.% Cft: | 1930762/= |

Contd: P/2

P-2

| Item No. | Qty’s | Description of item to be executed at site | Rate | Unit | Amount in Rupees |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 |
| 07/- | 22440Cft: | Preparing base course supplying and spreading stone metal of approved quality of 6” in 2 layers from approved quarry properly grade to maximum size of 1½” in required thickness of proper camber & grade i/c S/S 15 Cft: screening & non plastic quarry fines filling & depressions with stone metal after initial rolling including watering & compacting the same so as to achieve 100% density as per modified AASHO specifications (This i/cs providing and using templates camber plates screen from as directed).Rate includes all costs of materials T&P and carriage upto site of work. | 9645/17 | P.% Cft: | 2164376/= |
| 08/- | 44880Sft: | Providing 2 coat of surface dressing on or new existing surface with 55 lbs: bitumen and 6.75cft: cursh bajri of required size including cleaning the road surface rolling etc: complete.Rate includes all costs of materials T&P & carriage. | 2996/35 | P.% Sft: | 1344762/= |
| 09/- | 44880Sft: | Laying to proper line and grade plant mixes Ashplat concrete paver finished Hydraulic / Electronic control) prepared to specified formula according to job mix formula according to job mix formula approved by Engineer incharge i/c rolling and finished to proper line garde level and camber etc compete. Rate i/cs tack coat with bitumen 80/100 and all cost of material roller. T&P and carriage of material from source of supply tom plant and plant to site of work (S.I No: 26(b) P-11) | 9224/47 | P.% Sft: | 4139942/= |
| 10/- | 1320Sft: | Preparing and laying fiar face reinforcement cement New Jercy barrier having 300 psi compressive cylindrical strength concrete (ready mixed concrete) as per drwing i/c cost of as per 3.5 Cft i/c cost of excavation | 4016/72 | P.% Sft: | 5302070/= |
| 11/- | 1320Rft: | Const standered type concrete block B-type 1:2:4 i/c cost of molding one hand of neru shuttering erection removal etc complete | 174/00 | P.Rft | 229680/= |
|  |  |  | TOTAL |  | 18249505/= |

_____________% Above / Below on the rates of CSR.

Amount to be added /deducted on the

Total (A) 									Rs:

Total Amount in words  ____________________________________________________

__________________________________________________________________________

Executive Engineer,

Contractor						                           Highways Division

Kashmore @ Kandhkot

*Imran*

WIDENING OF ROAD FROM CLOCK TOWER TO SHAMS PUMP ROAD MILE 0/0-0/2

DRAFT BIDDING DOCUMENT FOR WORKS

BILLS OF QUANTITIES

(A) 	Description and rate of Items bases on composite schedule of Rates.

PART-B  (MISC:).

| Item No. | Qty’s | Description of item to be executed at site | Rate | Unit | Amount in Rupees |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 |
| 01/- | 01 Nos; | Proving and Fixing informatory sign rectangular shape 4’ x 3” comprising 02 Nos; 3” dia G.I Pipe Post i/c painting writing as directed each. | 16956/00 | P.Each | 16,956/= |
| 02/- | 3960 Rft | Colour Line | 41/24 | P.Rft | 163310/= |
| 03/- | 1250 Rft | Cate Eyes | 596/23 | P.Rft | 745288/= |
|  |  |  | TOTAL |  | 925554/= |

Amount Total (B)

__________________________% Above / Below on the rates of CSR. 	Total (B) =

Amount to be added /deducted on the

basis of premium quoted Total (B)

Total (b) =  in works in figure

Executive Engineer,

Contractor						                           Highways Division

Kashmore @ Kandhkot

*Imran*

RECONDITIONING OF ROAD FROM KARAMPUR MAGSI ROAD TO SHABIRABAD BHOWA ROAD MILE 0/0-2/0

DRAFT BIDDING DOCUMENT FOR WORKS

BILLS OF QUANTITIES

(A) 	Description and rate of Items bases on composite schedule of Rates

Part “A” (ROAD WORK)

| Item No. | Qty’s | Description of item to be executed at site |  | Rate | Unit | Amount in Rupees |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 |  | 4 | 5 | 6 |
| 01/- | 68523Cft: | Preparing base course supplying and spreading stone metal of approved quality of 6” in 2 layers from approved quarry properly grade to maximum size of 1½” in required thickness of proper camber & grade i/c S/S 15 Cft: screening & non plastic quarry fines filling & depressions with stone metal after initial rolling including watering & compacting the same so as to achieve 100% density as per modified AASHO specifications (This i/cs providing and using templates camber plates screen from as directed).Rate includes all costs of materials T&P and carriage upto site of work. |  | 9115/11 | P.% Cft: | 6245946/= |
| 02/- | 21753Rft | Laying brick on end edging including supply of 9”x 4 ½”x3” 1st: class burnt bricks excavation for laying edging with small side parallel to the road. Rate includes all costs of materials T&P and carriage upto site of work. |  | 2813/31 | P%Rft | 611979/= |
| 03/- | 130522Sft: | Providing 1st: coat of surface dressing on or new existing surface with 30 lbs: bitumen and 4.0 cft: cursh bajri of required size including cleaning the road surface rolling etc: complete.Rate includes all costs of materials T&P & carriage. |  | 1657/34 | P.% Sft: | 2163193/= |
| 04/- | 130522Sft: | Providing 1” thick consolidated premixed carpet in proper camber and grade i/c supply of 10 Cft: bajri 4.0 Cft: hill sand of approve quality and guage and bitumen 67 lbs: of 80/100 penetration i/c mixing the mechanical mixture in required proportion i/c heating the material and cleaning the road surface. Rate i/c all cost of material T&P and carriage upto site of work. |  | 4553/66 | P.% Sft: | 5943528/= |
| 05/- | 208000Cft: | Earth Work embankment from barrow pits i/c laying in 6” layers, clod breaking remaining dressing complete, lead Upto 100’ft and lift upto 5’ft (In ordinary Soil) |  | 7327/99 | P.%0 Cft: | 15424222/= |
|  |  |  | TOTAL |  |  | 16488868/= |

_____________% Above / Below on the rates of CSR.

Amount to be added /deducted on the

Total (A) 									Rs:

Total Amount in words  ____________________________________________________

__________________________________________________________________________

Executive Engineer,

Contractor						                           Highways Division

Kashmore @ Kandhkot

*Imran*

RECONDITIONING OF ROAD FROM KARAMPUR MAGSI ROAD TO SHABIRABAD BHOWA ROAD MILE 0/0-2/0

DRAFT BIDDING DOCUMENT FOR WORKS

BILLS OF QUANTITIES

(A) 	Description and rate of Items bases on composite schedule of Rates

Part “B” (ROAD WORK CUTS)

| Item No. | Qty’s | Description of item to be executed at site |  | Rate | Unit | Amount in Rupees |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 |  | 4 | 5 | 6 |
| 01/- | 374500Cft: | Earth Work embankment from barrow pits i/c laying in 6” layers, clod breaking remaining dressing complete, lead Upto 100’ft and lift upto 5’ft (In ordinary Soil) |  | 8775/75 | P.%0 Cft: | 3286518/= |
|  |  |  | TOTAL |  |  | 3286518/= |

_____________% Above / Below on the rates of CSR.

Amount to be added /deducted on the

Total (B) 									Rs:

Total Amount in words  ____________________________________________________

__________________________________________________________________________

Executive Engineer,

Contractor						                           Highways Division

Kashmore @ Kandhkot

*Imran*

CONSTRUCTION OF BRIDGE OVEWR TOUJ SHAKH NEAR VILLAGE MUHAMMAD KHAN GOLO TALUKA KASHMORE

DRAFT BIDDING DOCUMENT FOR WORKS

BILLS OF QUANTITIES

(A) 	Description and rate of Items bases on composite schedule of Rates.

PART-A  (BRIDGE WORK).

| Item No. | Qty’s |  | Description of item to be executed at site | Rate | Unit | Amount in Rupees |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 |  | 3 | 4 | 5 | 6 |
| 01/- | 9072 Cft: |  | Excavation in foundation of building bridge other structure i/cs dag belling dressing refilling around the structure with excavated earth watering and remaining lead upto one chain and left upto 5.0’ ordinary soil. | 3176/25 | P.%0 Cft: | 28815/= |
| 02/- | 2476Cft: |  | Cement concrete with bricks are stone plaster 1½” to 2” guage ratio 1:2:4 | 9416/28 | P.% Cft: | 233543/= |
| 03/- | 6558Cft: |  | Pucca Brick work in foundation and plinth i/c cement sand mortor | 12501/41 | P.% Cft: | 819842/= |
| 04/- | 665Cft: |  | Cement concrete plain i/c placing compacting finishing curing complete i/c screening washing of stone aggregate with out suhttering ratio 1:2:4: | 14429/25 | P.% Cft: | 95955/= |
| 05/- | 149Sft: |  | Erection and removal of centering for R.C.C or plain cement work (2nd: class) for partal wood vertical | 7000/00 | P.% Sft: | 10430/= |
| 06/- | 74 Cwt |  | Fabrication of m/s reinforcement cement concrete including cutting binding laying in position making joints & fastening of i/c binding wire also removal rust from bars. | 5001/70 | P.Cwt | 370224/= |
| 07/- | 2875Rft: |  | Reinforced cement concrete work including all labour and material except the cost of steel reinforcement and its labour for bending and binding which will be paid separately. This rate also includes all kind of forms moulds lifting shuttering curing rend. | 337/00 | P.Rft: | 968875/= |
| 08/- | 3800Sft: |  | Cement Plaster (1:3) upto 20’ height 1½” thick. | 2283/93 | P.% Sft: | 86789= |
|  |  |  |  | TOTAL |  | 2625471/= |

Amount Total (b)

__________________________% Above / Below on the rates of CSR. 	Total (a) =

Amount to be added /deducted on the

basis of premium quoted Total (a)

Total (a) =  in works in figure

Executive Engineer,

Contractor						                           Highways Division

Kashmore @ Kandhkot

*Imran*