Schedule-A to Bid

SCHEDULE OF PRICES

| NAME OF WORK | CONSTRUCTION OF CC PAVER IN CITY GARHI CHAKAR. |
| --- | --- |

|  | CC PAVER |  |  |  |
| --- | --- | --- | --- | --- |
| 01 | Preparing Sub-Base by supplying and spreading well graded pit or bed run gravel having a liquit limit not greater than 25 and plasticity index not greater than 6 in proper camber and grade i/c watering rolling and compacting in layers. Thickness of each compacted layer not exceeding 6” compacted upto 98-100% density as per modified ASSHO density (Rate i/c all cost of material T&P carriage upto site of work) SCI No: 11 (d) P No: 04. |  |  |  |
|  | 24480.00CFT | RS: 3341/71 | P.%0CFT | RS: 818051/00 |
| 02 | Supplying Spreading crush powder from Approved quality to site of work i/c lading un loading dressing leveling as per design etc complete (Rate Approved) |  |  |  |
|  | 8323.20CFT | RS: 4500/00 | P%CFT | RS: 374544/00 |
| 03 | Cement concrete plain i/c placing compacting finishing and complete i/c screening and washing of stone aggregate without shuttering ratio 1:2:4 (CSI No: 5 P No: 15) |  |  |  |
|  | 2019.60CFT | RS: 14429/25 | P.%CFT | RS: 291413/00 |
| 04 | Providing fixing Cement Paving Blocks flooring having size of 197x97x60(M.M) of city Quddra / Cobble shape with natural colours having strength between 5000 PSI to 8500 PSI i/c filling the joints with hill sand in specified manner / pattern & design / etc complete (CSI No: 71 P No: 49) |  |  |  |
|  | 48960.00SFT | RS: 199/77 | P.SFT | RS: 9780739/00 |
|  |  |  |  | RS: 11264747/00 |

| CONTRACTOR | EXECUTIVE ENGINEER PUBLIC HEALTH ENGG: DIVISION GHOTKI @ MIRPUR MATHELO |
| --- | --- |

CONDITION

| 01 | The work shall have to be executed according to PWD/PHE specification |
| --- | --- |
| 02 | No premium on non schedule item shall be allowed to the contractor |
| 03 | Any error omission in the description of item of work and the rate shall be governed with the relevant schedule of rate and estimate |
| 04 | The contractor shall have to bring the material to be used from the quarries mentioned in the estimate |

| CONTRACTOR | EXECUTIVE ENGINEER PUBLIC HEALTH ENGG: DIVISION GHOTKI @ MIRPUR MATHELO |
| --- | --- |