BILL OF QUANTITY

(B) DESCRIPTION AND RATE OF ITEMS BASED ON COMPOSITE

SHEDULE  OF RATES

NAME OF WORK:-	REHABILITATION OF RURAL DRAINAGE SCHEME MISRI JAKHRIJO TALUKA & DISTRICT MATIARI.:::::::::

S# Items Qty: Rate Unit Amount.

GROUP’A’  REPAIR OF SURFACE DRAINS i/c  SLUDGE SILT CLEARANCE

1	Dismantling brick work in lime or cement motor

(GSINO:13 P-10)

2025.00Cft	@Rs: 1285/63		P % Cft.	          Rs:      26,034 /-

2	Earth work excavation in ashes sand soft soil or silt clearance

Undressed Lead up to 50 ft (GSI No: 2 P/1)

5735.0Cft	@Rs: 1663/75		P %0Cft.	          Rs:        9,542/-

- Re-handling of earth work (b) upto a lead of 50 ft

( GSI NO: 9 P/2)

7760.00Cft	@Rs:1058/75		P%0 Cft		Rs:      8,216/-

4	Extra for every 50 ft additional lead of part therefore

(a) for earth Work (Soft ordinary Soil) hard

and very hard (GSI NO: 8(a) P/2).

7760.00Cft	@ Rs: 4938/22		P%0 Cft		Rs:        38,321/-

4	Excavation in foundation of building bridges and other structure i/c

dag belling dressing refilling around the structure with excavated

earth watering ramming lift upto 5ft and lead upto one chain (In

ordinary soil) etc(G.S.I.NO:18 P/4)

3010.00Cft	@ Rs: 3176/25		P%0 Cft		Rs:        9,561/-

5	Cement concrete plain i/c placing compacting finishing and curing complete

i/c screening and washing of stone aggregate without shuttering ratio(1: 4:8)

(G.S.I.NO:5P-17)

602.50Cft	@Rs: 11288/75		P %Cft			Rs:        68,015/-

6	Pacca brick work in foundation and plinth in cement

sand motor (1.6) (GSI NO: 4P/21)

4350.00Cft	@Rs: 11948/36		P% Cft			Rs:       5,19,754/-

7	Constt: of standard open drains cunette  block of c.c 1:2:4

In situ to the design profile i/c cost of mould as per drawing

i/c applying floating cost of cement 1/32” thick to the exposed

face finished smooth curing etc complete as per detailed

drawing .(PHSI No:d(ii)P/58).

T-A 		500.00Rft	@Rs: 94/-		P: Rft			Rs:         47,000/-

T-B		400.00Rft	@Rs: 174/-		P : Rft			Rs:          69,600/-

8	Cement plaster 1:4 up to 12’ fit height 1/2” thick ( GSI No: 11/b P-52).

9988.00Sft	@Rs: 2283/93		P% Sft			Rs:       2,28,119/-

- R.C.C work. In roof slab beams columns raffs lintel

and other structural Member laind in   situ (Ratio. 1: 2: 4)

90(b) count rept sand 4 cft shingle 1/8’ to ¼’ gauge

(GSI No: 6(a) P-12)

208.00 Cft	@ Rs: 337/-		P .Cft 			Rs:       70,096/-

10	 Fabrication of mild steel reinforcement for c.c i/c cutting

bending laying in Position making joints and fastening

i/c cost binding wire also removal of Rust from bars etc

(GSI No: 8 (a) P/17 (1) using for bars.

7.42 Cwt..	@ Rs: 5001/70		P.Cwt 			Rs:      37,113/-

G. Total             Rs:   11,31,371/-

| 1. | GROUP’B’ ( I ) COLLECTING TANK SIZE(12 DIA) Excavation of well dry upto 20’ below ground level and disposal of soil within one chain(GSINO:1(a)P-86) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| i) | 0’-5’Depth |  |  |  |  |  |  |
|  | 883.12 Cft | @Rs:2247/48 |  |  | P%0Cft |  | Rs: 1,985/= |
| 2. | Wet sinking of well for depth below spring level by means of divers i/c all charges for shoring loading and removing excavated material within one chian(GSINO:4(a)p-87) |  |  |  |  |  |  |
| i) | 0’-5’Depth |  |  |  |  |  |  |
|  | 825.23 Cft | @Rs: 11763/80 |  |  | P%0Cft |  | Rs: 9,708/= |
| ii) | 5’-10’Depth |  |  |  |  |  |  |
|  | 825.23 Cft | @Rs: 24200/- |  |  | P%0Cft |  | Rs: 19,971/= |
| iii) | 10’-15’Depth |  |  |  |  |  |  |
|  | 247.56 Cft | @Rs: 37230/77 |  |  | P%0Cft |  | Rs: 9,217/= |
| 3. | Bailing or pumping out sub soil water during excavation concreting cost in situ concrete or masonary work in foundation etc (PHSINO:18,P-75) |  |  |  |  |  |  |
|  | 5941.66 Cft | @Rs:543/- |  |  | P%Cft |  | Rs: 32,263/= |
| 4. | Cement concrete brick or stone ballast 1 ½” to 2” guage ratio (1:4:8)(GSINO:4 P-15) |  |  |  |  |  |  |
|  | 231.18 Cft | @Rs:9416/28 |  |  | P%Cft |  | Rs: 21,769/= |
| 5. | Cement concrete plain i/c placing compacting finishing and curing complete i/c screening and washing of stone aggregate without shuttering (G.S.I.NO: 5 P/16) |  |  |  |  |  |  |
| i) | Ratio(1:3:6) |  |  |  |  |  |  |
|  | 113.00 Cft | @Rs:12595/- |  |  | P%Cft |  | Rs: 14,233/= |
| ii) | Ratio(1:2:4) |  |  |  |  |  |  |
|  | 113.00 Cft | @Rs:14429/25 |  |  | P%Cft |  | Rs: 16,305/= |
| 6. | R.C.C. work i/c all labour and material except the cost of steel reinforcement and its labour for bending and binding which will be paid separately. This rates also i/c all kinds forms moulds lifting shuttering ;curing rendering and finishing the exposed surface i/c screening and washing of shingle (a) RCC work in roof slab beams columns rafts lintels and other structural member laid in situ or precast laid in position in all respect ratio(1:2:4)et (G.S.I.NO:5 P/17 |  |  |  |  |  |  |
|  | 750.00 Cft | @Rs:337/- |  |  | P.Cft |  | Rs: 2,52,750/= |
| 7. | Fabrication of mild steel reinforcement for cement concrete i/c cutting bending laying in position making joints and fastening i/c cost of binding wire also i/c removal of rust from bars etc (G.S.I.NO:8 P/17) |  |  |  |  |  |  |
|  | 28.51 Cwt | @Rs:5001/70 |  |  | P.Cwt |  | Rs: 1,42,598/= |
| 8. | Providing expansion joint in cement work of 9” wide corrugated PVC water stop with bulbs i/c soldering cost of material and lab our etc complete(PHSINO:2,P-56) |  |  |  |  |  |  |
|  | 125.46 Rft | @Rs:86/81 |  |  | P.Rft |  | Rs: 10,790/= |
| 9. | Cement plaster(1:4) upto 12’ ft height 1/2” thick (GSINO:11/bP-57 |  |  |  |  |  |  |
|  | 602.88 Sft | @Rs:2283/93 |  |  | P% Sft |  | Rs: 13,769/= |
| 10 | Supply dampo((GSINO:11(b)P-57) |  |  |  |  |  |  |
|  | 15.40 Lbs | @Rs: 56/18 |  |  | P. Lbs |  | Rs: 875/= |
| 11 | P/F in position in cc foundation (1:3:6)M.S.ladder of M.S. angle iron 2’x2”x1/4” 1” apart with M.S. bar ¾” dia spaced 1’ apart i/c painting three coat with anticorrosive steel paint etc complete. (PHSINO:3,P-55) |  |  |  |  |  |  |
|  | 17.0Rft | @Rs:799/- |  |  | P.Rft |  | Rs; 13,583/= |
|  |  |  |  |  | Total |  | Rs: 5,59,816/= |
| 1. | GROUP’B’ (II ) INTERCONNECTION Excavation for pipe lines in trenches and pits in Soft Soil i/c trimming and dressing sides to true alignment and shape leveling of beds of trenches to correct level and grade cutting joint holes and disposal of surplus earth within one chain as directed by Engineer In charge providing fence guards light flags and wherever required lift upto 5ft and Lead upto one chain (P.H.S.I.NO:1 P/46) |  |  |  |  |  |  |
|  | 192.00Cft | @Rs:3600/- |  | P%0Cft |  |  | Rs: 691/= |
| 2. | Providing layingRCC pipes of A.STM C-76-62 class ii wall B and faxing In trench i/c cutting fitting and jointing witrh rubber ring i/c testing With water to specified pressure.(P H I No:B 1 P-17). |  |  |  |  |  |  |
|  | 12” dia. |  |  |  |  |  |  |
|  | 16.0 Rft | @Rs:412/- |  | P.Rft |  |  | Rs: 6,592/= |
| 3. | Refilling excavated the stuff in trenches 6” thick layers i/c watering ramming to full compaction etc complete (GSINO:24 P-77) |  |  |  |  |  |  |
|  | 172.80 Cft | @Rs:2760/- |  | P%0Cft |  |  | Rs: 477/= Rs: 7,760/= |
| 1. | GROUP ‘B’ (III ) SCREENING CHAMBER.SIZE(5’x8’)2Nos Excavation for tank and reservoirs in soft soil i/c trimming and dressing side to true alignment design section/profiles and shape leveling of bed of trenches to correct level and i/c laying of earth in 6”layers for construction of banks and dressing and disposal of surplus excavated earth within one chain as directed by engineer in charge providing fence guards light flags and temporary crossing for non vehicular traffic wherever required lift upt 5ft(1.52m) and lead upto one chain(30.5m). (PHSINO:1,P-67) |  |  |  |  |  |  |
| i) | 0’-5’Depth |  |  |  |  |  |  |
|  | 440.0 Cft | @Rs:3000/- |  | P%0Cft |  |  | Rs: 1,320/= |
| 2. | Excavation for tank and reservoirs in wet soil i/c trimming and dressing side to true alignment design section/profiles and shape leveling of bed of trenches to correct level and i/c laying of earth in 6”layers for construction of banks and dressing and disposal of surplus excavated earth within one chain as directed by engineer in charge providing fence guards light flags and temporary crossing for non vehicular traffic wherever required lift upt 5ft(1.52m) and lead upto one chain(30.5m). (PHSINO:14P-50) |  |  |  |  |  |  |
| i) | 0’-5’Depth |  |  |  |  |  |  |
|  | 352.00 Cft | @Rs:4800/- |  | P%0Cft |  |  | Rs: 1,690/= |
| 3. | Cement concrete brick or stone ballast 1 ½” to 2” guage ratio (1:4:8)(GSINO:4 P-15) |  |  |  |  |  |  |
|  | 44.00 Cft | @Rs:9416/28 |  | P%Cft |  |  | Rs: 4,143/= |
| 4. | Cement concrete plain i/c placing compacting finishing and curing complete i/c screening and washing of stone aggregate without shuttering ratio(1:2:4) (G.S.I.NO: 5 P/17) |  |  |  |  |  |  |
|  | 10.00 Cft | @Rs:14429/25 |  | P%Cft |  |  | Rs: 1,443/= |
| 5. | R.C.C. work i/c all labour and material except the cost of steel reinforcement and its labour for bending and binding which will be paid separately. This rates also i/c all kinds forms moulds lifting shuttering ;curing rendering and finishing the exposed surface i/c screening and washing of shingle (a) RCC work in roof slab beams columns rafts lintels and other structural member laid in situ or precast laid in position in all respect ratio(1:2:4)et (G.S.I.NO:5 P/17 |  |  |  |  |  |  |
|  | 242.27 Cft | @Rs:337/- |  | P.Cft |  |  | Rs: 81,645/= |
| 6. | Fabrication of mild steel reinforcement for cement concrete i/c cutting bending laying in position making joints and fastening i/c cost of binding wire also i/c removal of rust from bars etc (G.S.I.NO:8 P/17) |  |  |  |  |  |  |
|  | Using tor bar |  |  |  |  |  |  |
|  | 9.73 Cwt | @Rs:5001/70 |  | P.Cwt |  |  | Rs: 48,667/= |
| 7. | Pucca brick work in foundation and plinth in cement sand mortor (1:6) etc (G.S.I.NO:4 P/21) |  |  |  |  |  |  |
|  | 20.34 Cft | @Rs:11948/36 |  | P%Cft |  |  | Rs: 2,430/= |
| 8. | Cement plaster(1:4)upto 12’ ft height 1/2” thick (GSINO:11/b P-57) |  |  |  |  |  |  |
|  | 187.56 Sft | @Rs:2283/93 |  | P%Sft |  |  | Rs: 4,284/= |
| 9. | Providing expansion joint in cement work of 9” wide corrugated PVC water stop with bulbs i/c soldering cost of material and lab our etc complete(PHSINO:2,P-56) |  |  |  |  |  |  |
|  | 30.00Rft | @Rs:86/- |  | P. Rft |  |  | Rs: 2,580/= |
| 10 | P/F in position in cc foundation (1:3:6 )M.S.ladder of M.S.angle iron 2’x2”x1/4” 1” apart with M.S.bar ¾” dia spaced 1’ apart i/c painting three coat with anticorrosive steel paint etc complete .(PHSINO:3,P-55) |  |  |  |  |  |  |
|  | 10.0Rft | @Rs:799/- |  | P.Rft |  |  | Rs: 7,990/= |
| 11 | Fabrication of heavy steel work with angles tees flat iron round iron and sheet iron for making trusses girders tanks etc i/c cutting diverting handling assembling and fixing but excluding erection in position(GSIN:2,P-91). |  |  |  |  |  |  |
|  | 1.67 Cwt | @Rs: 4928/49 |  | P.Cwt |  |  | Rs: 8,231/= |
| 12 | Erection and fitting in position of iron trusses and testing of water Tank etc (G S I No: 3 P-91) |  |  |  |  |  |  |
|  | 1.67 Cwt | @Rs:271/04 |  | P.Cwt |  |  | Rs: 453/= |
| 13 | Providing and fixing C.I.main hole cover with frame i/c cost of material etc(PHSINO:1,P-13) |  |  |  |  |  |  |
|  | 1.75Cwt | @Rs: 6985/- |  | P.Cwt |  |  | Rs; 12,224/= |
|  |  |  |  | Total |  |  | Rs: 1,75,568/= |
| 1. | GROUP ‘C’ PUMP HOUISE Excavation in foundation of building bridges and other structure i/c dag belling dressing refilling around the structure with excavated earth watering ramming lift upto 5ft and lead upto one chain (In ordinary soil) etc (G.S.I.NO:18 P-4) |  |  |  |  |  |  |
|  | 235.00Cft |  | @Rs:3176/25 |  |  | P%0 Cft | Rs: 746/= |
| 2. | Cement concrete brick or stone ballast 1 ½” to 2” guage ratio (GSINO:4 P-15 ) |  |  |  |  |  |  |
| i) | Ratio(1:4:8) |  |  |  |  |  |  |
|  | 47.00 Cft |  | @Rs:9416/28 |  |  | P% Cft | Rs: 4,426/= |
| ii) | Ratio(1:5:10) |  |  |  |  |  |  |
|  | 57.00 Cft |  | @Rs:8694/95 |  |  | P% Cft | Rs: 4,980/= |
| 3. | Cement concrete plain i/c placing compacting finishing and curing complete i/c screening and washing of stone aggregate without shuttering ratio(1:3:6) (G.S.I.NO: 5(f) P/16 ) |  |  |  |  |  |  |
|  | 188.00 Cft |  | @Rs:12595/- |  |  | P% Cft | Rs: 23,679/= |
| 4. | Erection and removal of centering for R.C.C.plain c.c.works pertal wood (2nd class) vertical (G.S.I.NO:18 P/20) |  |  |  |  |  |  |
|  | 376.0Csft |  | @Rs: 3127/41 |  |  | P% Sft | Rs: 11,759/= |
| 5. | Pucca brick work in ground floor in cement sand mortor (1:6)(GSINO:4 P-21) |  |  |  |  |  |  |
|  | 298.51 Cft |  | @Rs:12674/36 |  |  | P% Cft | Rs: 37,834/= |
| 6. | R.C.C. work i/c all labour and material except the cost of steel reinforcement and its labour for bending and binding which will be paid separately. This rates also i/c all kinds forms moulds lifting shuttering ;curing rendering and finishing the exposed surface i/c screening and washing of shingle (a) RCC work in roof slab beams columns rafts lintels and other structural member laid in situ or precast laid in position in all respect ratio(1:2:4)et (G.S.I.NO:5 P/19 |  |  |  |  |  |  |
|  | 94.13 Cft |  | @Rs:337/- |  |  | P.Cft | Rs: 31,722/= |
| 7 | Fabrication of mild steel reinforcement for cement concrete i/c cutting bending laying in position making joints and fastening i/c cost of binding wire also i/c removal of rust from bars etc (G.S.I.NO:8 P/17) |  |  |  |  |  |  |
|  | 3.78Cwt |  | @Rs:5001/70 |  |  | P.Cwt | Rs: 18,906/= |
| 8. | Filling watering and ramming earth under floor with new earth (excavated from outside) lead one chain lift upto 5ft(GSINO:82,P-5) |  |  |  |  |  |  |
|  | 229.12 Cft |  | @Rs:3630/- |  |  | P%0Cft | Rs: 832/= |
| 9. | Making & fixing steel grated doors with 1/16” thick sheeting including angle iron frame 2”x2” 3/8”x ¾” square bars 4” center to center with locking arrangement (GSI.No.24 P/92) |  |  |  |  |  |  |
|  | 22.45Sft |  | @Rs:726/72 |  |  | P.Sft | Rs: 16,533/= |
| 10 | P/F iron steel grill of ¼”x1/4”side flat iron of approved design i/c painting 3 coats weight not less than 3.7 lbs of finished grill(GSINO:26 P92) |  |  |  |  |  |  |
|  | 30.00Sft |  | @Rs:180/50 |  |  | P.Sft | Rs: 5415/= |
| 11 | Fabrication of heavy steel work with angles tees flat iron round iron and sheet iron for making trusses girders tanks etc i/c cutting diverting handling assembling and fixing but excluding erection in position(GSIN:2,P-91). |  |  |  |  |  |  |
|  | 2.54Cwt |  | @Rs:4928/49 |  |  | P.Cwt | Rs: 12,518/= |
| 12 | Cement plaster(1:4)upto 12’ ft height 3/8” thick (GSINO:11/a P-52) |  |  |  |  |  |  |
|  | 940.25 Sft |  | @Rs:2197/52 |  |  | P%Sft | Rs: 20,662/= |
| 13 | Cement plaster(1:6)upto 12’ ft height 1/2” thick (GSINO:13(b) P-52) |  |  |  |  |  |  |
|  | 940.25 Sft |  | @Rs:2206/60 |  |  | P%Sft | Rs: 20,748/= |
| 14 | Providing and laying 1” thick topping cement concrete(1;2:4)i/c surface finishing and dividing into panels(2”Thick) (PHSINO:16,P-42) |  |  |  |  |  |  |
|  | 120.0 Sft |  | @Rs:3275/50 |  |  | P%Sft | Rs: 3,931/= |
| 15 | White wash One coat(GSINO:24P-54) |  |  |  |  |  |  |
|  | 940.25Sft |  | @Rs: 416/63 |  |  | P.%Sft | Rs: 3,917/= |
|  | Colore washing (GSINo: 26 c P-56 ) |  |  |  |  |  |  |
|  | 940.25 Sft |  | @Rs: 859/90 |  |  | P%Sft | Rs: 8,085/= |
| 16 | Painting New Surface.Preparing (Door and windows) Surface and painting of any type (i/c angle two coats (GSINO:5 ( c) P-70) |  |  |  |  |  |  |
|  | 105.50 Sft |  | @Rs: 2116/41 |  |  | P.%Sft | Rs: 2,233/= |
| 17 | Erection and fitting in position of iron trusses and testing of water tank etc (GSINO:3 P-91) |  |  |  |  |  |  |
|  | 2.54 Cwt |  | @Rs:271/04 |  |  | P.Cwt | Rs: 688/= |
| 19 | Wight wash three coats (G S I No: 26 c P-54) |  |  |  |  |  |  |
|  | 120.00 Sft |  | @Rs: 829/95 |  |  | P%Sft | Rs: 996/= |
|  |  |  |  |  |  | Total | Rs: 2,30,610/= |

GROUP ‘D’ PUMPMING MACHINERY NEW /REPAIR

| 1. | Supplying and installing in position i/c transportation to site of work at Rural Drainage Scheme Misri Jakhrijo electric pumping set consisting of A.C.electric motor of 5.50 BHP 3 phase 50 cycles 400/420 volts 960 RPM type ILA 133-4 AA60 coupled with KSB pump type (KWPZ-65-315)size 3”x2 ½ mounted on common steel frame or base plate capable of discharging 70 gallons per minute against a head of 40 ft i/c installation of pumping set on c.c foundation ratio(1:2 4 )and ratio(1:4:8)with ballast with m.s nuts and bolts of 1/2”dia to be embedded in c.c foundation i/c providing and fixing on prepared board A.C.50 cycles 3 phase 12/25 amperes trip rating push button type star delta starter complete with no volts and over load release and required i/c providing and fixing on prepared board iron (clad)triple pole and neutral switch fuse with rewire able fuse link carried 500 volts grade 30 amperes and providing and fixing volts meter 0.500 volts and amperes meter 0-50 amperes installed on metal led board i/c providing and fixing metal led board to accommodate main or sub main control switch recessed in the wall size 3’x2 ½’ i/c providing phase indication bulbs with holder and providing internal electric wiring for main with pvc insulated wire 660 volts grade in 25 mm( ½ )G.I.plate buried in the ground at depth of ( 12 )for base plate and testing of the pumping set against the required head for 72 hours etc complete R.A attached |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 1 Set | @Rs: 724065/= | P.Set |  | Rs: 7,24,065/= |
| 2. | Providing and installing in position i/c transportation to site of work at Rural Drainage Scheme Misri Jakhrajo Distt: Matiari 8 BHP Diesel pumping set consisting (Fixed))(Golden pump) with 2200 RPM directed coupled with non clogging pump of KSB(KWPZ 65x315 size 3”x2 ½ with 70 FT GPM discharge against the head of 40 ft mounted on common steel frame or base i/c making cement concrete foundation ratio(1:2:4)and CC (1:4:8) with m.s. nuts and bolts of 1.04 dia to be embedded in cc foundation i/c testing or pumping set against the required head of 72 hours etc etc complete (R.A.Approved) |  |  |  |  |
|  | 1 Set @ Rs: 524470/= P.Set |  |  |  | Rs: 5,24,470/- |
| 3. | Providing A.C electric motor 5.50 (Type 1LA -130-4AA60) three phase 50 (siemes made) cyeles 400/420 volts of 1450 RPM (M.R) (only Supply) |  |  |  |  |
|  | 1 No | @Rs: 50694/= |  | Each | Rs: 50694/= |
| 4 | Providing Diesel Engine 8 HP (Type chaina made) 2200RPM(M.R) self start with 12 volt battery |  |  |  |  |
|  | 1 No | @Rs: 23400/- |  | Each | @Rs: 23,400/= |
| 5 | Repair non clogging sullage pump size 2 ½ x3 etc complete (R.A Approved) |  |  |  |  |
|  | 2 No | @Rs: 10000/- |  | Each | Rs: 20,000/= |
| 6 | Providing and faxing motor control unit(MCU) type 17A K.S.B (rate approved) 5.50 BHP |  |  |  |  |
|  | 1 No | @Rs: 45000/- |  | Each | Rs: 45,000/= |
|  | Wiring pvc installed with 4-7-036 (4mm) from core copper conductor 300/500 (ESINO: 32P-5) |  |  |  |  |
|  | 300.00Rft | @Rs: 108/84 |  | P.Rft | Rs: 32,652/= |
| 7 | Manufacturing supplying and fixing block steel ms pipe madeout of ms sheet of 3/86 thick(R.Approved) Based on schdule |  |  |  |  |
| i) | 3”Dia |  |  |  |  |
|  | 96.00 Rft | @Rs:385/89 |  | P.Rft | Rs 37,045/= |
|  | 4”Dia |  |  |  |  |
|  | 38.00.Rft | @Rs: 506/60 |  | P.Rft | Rs: 19,251/= |
| 9. | Manufacturing supplying M.s.flanges of different dia made of m.s.sheet of holes and welding of flanges with m.s pipe etc complete(R,A) |  |  |  |  |
|  | 3”Dia |  |  |  |  |
|  | 16 Nos | @RS: 500/89 |  | Each | Rs: 8,014/= |
|  | 4”Dia |  |  |  |  |
|  | 20 Nos | @RS: 596/75 |  | Each | Rs: 11,935/= |
| 10. | Supplying C.I.sluice valve heavy pattern(test pressure 21 Kg sq cm or 300 lbs sq inch)(SMI No2- P-10) |  |  |  |  |
|  | 3”Dia 2 Nos | @RS: 4290/- |  | Each | Rs: 8,580/- |
| 11. | Supplying C.I.Reflex valve heavy pattern(test pressure 21 Kg sq cm or 300 lbs sq inch)(SMI NO: 6-P-11) |  |  |  |  |
|  | 3”Dia |  |  |  |  |
|  | 2 Nos | @Rs: 1543/75 |  | Each | Rs 3,088/= |
| 12. | Supplying C.I.tapper flat bottomed or central tapered flanged ends with holes i/c turning and facing turning flanges for all size. (S M I) |  |  |  |  |
|  | 3”x2-1/2” |  |  |  |  |
|  | 0.51 Cwt | @RS: 6096/- |  | P.Cwt | Rs: 3,109/= |
| 13. | Supplying C.I.bend with flanged ends with holes i/c turning and facing of flanges for all size.(S M I No: 8 P-8-14) |  |  |  |  |
|  | 3”Dia 90o(F) 4”Dia90o(f) |  |  |  |  |
|  | 1.64 Cwt | @Rs: 6096/- |  | P.Cwt | Rs: 9,997/- |
| 14. | C.I.Tee flanged ends with holes i/c turning and facing of flanges. (S M I No:12 P No |  |  |  |  |
|  | 0.45 Cwt | @Rs: 6096/- |  | P.Cwt | Rs: 2,743/- |
| 15 | Supplying CI foot valve heavy pattern test (SmINO:4 P-12) |  |  |  |  |
|  | 3”Dia |  |  |  |  |
|  | 4 Nos | @Rs: 731/25 |  | Each | Rs: 2,925/= |
| 16 | Supplying CI short piece(SMINO:9 P-15) |  |  |  |  |
|  | 4”Dia |  |  |  |  |
|  | 1 No | @Rs: 487/50 |  | Each | Rs: 488/= |
| 17 | Jointing C.I./M.S.flanges pipes and specials flanged and inside trench i/c supplying rubber packing of the required thickness nuts bolts with washers etc complete.(PHSINO:1P-40) |  |  |  |  |
| i) | 3”Dia |  |  | 16.0 Joints |  |
|  | 16.0 Joints | @Rs: 499/- |  | P.Joint | Rs: 7,984/= |
| ii) | 4”Dia |  |  |  |  |
|  | 16.0 Joint | @Rs: 513/- |  | P.Joint | Rs: 8,208/= |
| 18 | Repair of Transformer Tacking out 25 K V transformer & repair of 3 Nos coils i/c changing transformer & repair of oil from disposal work for necessary repair @ Hyd:and referring at same place (R.A approved C.E PHED vide his No: 513 dt: 26/2/2011 or W/Supply scheme Nabi Bux Mahajor |  |  |  |  |
|  | 1No | @Rs: 66800/- |  | Each | Rs: 66,800/- |
|  |  |  |  | .Total | Rs: 16,10,448/= |

GROUP ‘E’ RISING MAIN 4” DIA

| 1. | Excavation for pipe lines in trenches and pits in Soft Soil i/c trimming and dressing sides to true alignment and shape leveling of beds of trenches to correct level and grade cutting joint holes and disposal of surplus earth within one chain as directed by Engineer Incharge providing fence guards light flags and wherever required lift upto 5ft and Lead upto one chain (P.H.S.I.NO:1 P/60) |  |  |  |
| --- | --- | --- | --- | --- |
|  | 4”Dia |  |  |  |
|  | 6000.00 Cft | @Rs:3600/- | P%0Cft | Rs: 21,600/= |
| 2 | Providing laying and jointing F/C pressure pipes class B (Equivalent make)Fixing in trench i/c cutting fitting and jointing with with one rubber ring i/c testing with water to a head 61 meter or 200ft (R.A approved) |  |  |  |
|  | 4”Dia |  |  |  |
|  | 1000.00Rft | @Rs: 297/53 | P.Rft | Rs: 2,97,530/= |
| 3. | Supply C.I special of A.C pressure pipe B CLASS( Stander weight of AC L SMNO 10P/3) |  |  |  |
| i) | C.I Bend 90”C (PHSINo:2 p-13) |  |  |  |
|  | 1 Nos | @Rs: 893/75 | Each | Rs: 894/= |
| ii) | C.I Bend 45”C (PHSINo:1p-13) |  |  |  |
|  | 1 Nos | @Rs: 731/25 | Each | Rs: 731/= |
| 4 | Supply A.C Colour B class (PHSINO:4P-4) |  |  |  |
|  | 4”Dia |  |  |  |
|  | 1 Nos | @Rs: 157/- | Each | Rs: 157/- |
| 5. | One pairs of rubber for A.C pressure pipe B class (conet joint PHSINO:7 P-4) |  |  |  |
|  | 4”Dia |  | 1 Nos |  |
|  | 1 Nos | @Rs: 95/- | Each | Rs: 95/- |
| 6. | Making joint to C.I specials fitting (i/c laying of specials and cost of solvent cement)of required diameter and testing the joints along with PVC pipe line B” ,C”, and D” class to the specified pressure and making good to all leaky joints etc complete.(PHSINo(n)1 P-80) |  |  |  |
|  | 4”Dia |  |  |  |
|  | 1 No | @Rs:62/- | Each | Rs: 62/= |
| 7. | Refilling excavated the stuff intrenches 6” thick layers i/c watering ramming to full compaction etc complete (PHSINO:24P-77) |  |  |  |
|  | 5400.00 Cft | @Rs:2760/- | P%0Cft Total | Rs: 14,904/= Rs: 3,35,973/= |

GROUP ‘F’  CLOUR  WASING OF COMPOUND WALL,PUMP HOUSE

JANGAL CLEARANCE  OF OXIDATION POND.

(I ) JANGAL CLEARANCE OF OXIDATION POND.

| 1. | Jangal clearance and removing with in 100 ft (G.S.I.NO:4- P 95) |  |  |  |
| --- | --- | --- | --- | --- |
|  | 6528.00Sft | @Rs: 75/63 | P%0Sft Total | Rs: 494/- Rs: 494/- |
| (II )COLOUR WASH OF COMPOUND WALL. - Painting old surfaces ( c) painting doors and windows Any type i/c three coats (G S I No: 4 c P- 68) 144.00Sft @Rs: 1662/21 P%Sft Rs: 2,394/= 2 Distempering ( b) two costs (GSINo:24P-54) Compound wall 2800.0Sft @Rs: 1043/90 P%Sft Rs: 29,229/= Total Rs: 31,623/= |  |  |  |  |

(III )REPAIR & COLOUR WASHING OF PUMP HOUSE .

| 1. | Removing Cement plaster or lime plaster (G S I No:53P-13) |  |  |  |
| --- | --- | --- | --- | --- |
|  | 386.25Sft | @Rs: 121/- | P% Sft | Rs: 467/- |
| 2 | Cement plaster 1:4 upto12’ height (b) ½ thick (G S I No:11(b)P-52) |  |  |  |
|  | 386.25 Sft | @Rs: 2283/93 | P% Sft | Rs: 8,822/- |

| 3. | White Washing of 2 coots (G S I No: 26(b) P-54) |  |  |  |
| --- | --- | --- | --- | --- |
|  | 202.50 Sft | @Rs:425/84 | P% Sft | Rs: 862/- |
| 4 | Distempering 3 coots (GSINo: 24(b)P54 |  |  |  |
|  | 873.75 Sft | @Rs: 1079/65 | P% Sft | Rs: 9433/- |
| 5. | Painting old surfaces ( c) painting doors and windows any type i/c three coats (G S I No: 4 c P- 68) |  |  |  |
|  | 85.50Sft | @Rs: 1662/21 | P% Sft Total | Rs: 1,421/- Rs: 21,005/= |

Amount Total (a) + (b) + (c) + (d) + (e) & (f)  	=	Rs: 41,04,668/-

________ % above / below on the rate of CSR,

Amount to be added / deducted on the basis of

premium, Total (b) Rs. ________________

Total (A) = a + b in works & Figures_______________________________________________

__________________________________________

CONDITIONS.

1	Any error or omissions in the description of items, rates, unit quantity &

Calculation will be governed by the relevant schedule of rates.

2.	The contractor must remove the surplus earth, material, Garbage from

Site of work on his own cost.

3.	No premium will be allowed on non-schedule items & market rates.

4.	No separate cartage will be paid to the contractor.

5.	The work will be completed within specified period.

6.	The contractor is bound the work will be carried out as per agreement

and standard PWD specification .

(MUHAMMAD  AKBAR BALOCH)

CONTRACTOR					EXECUTIVE ENGINEER

PUBLIC HEALTH ENGG: DIVISION

MATIARI.