NAME OF WORK:- 	M/R TO SECRETARY BUNGALOW NO.02 TOWN HOUSE GOR-III KARACHI (DIFFERENT WORK).

SCHEDULE B

| S.NO. | DESCRAPTION OF ITEMS | QUTY | RATE | UNIT | AMOUNT |
| --- | --- | --- | --- | --- | --- |
|  | PART – A (CIVIL WORKS) |  |  |  |  |
|  | S/F in position aluminum channel framing for slidding windows & vent of alcop make in 5mm thick tinted glass glazing (Belgium) etc complete. (Dulex model Bonze) (S.I.85a/108). | 19.96 Sft | 1449/69 | Sft | 28936 |
|  | P/F in position doors & windows and ventilators of first class deodar wood frame and 1-3/4” thick commercial ply venner shutter of first class deodar skeleton (hollow) and commercial ply wood (3-Ply) on both sides (S.I.9/58(-) 25-b/61) (1227/36(-)370/83. | 49.51 Sft | 856/53 | Sft | 42406 |
|  | P/F approved quality mortice lock. | 6 Nos. | 1786/13 | Each | 10717 |
|  | Scraping ordinary distempering oil bound distemper or paint on walls (S.I.54-b/13). | 1411.38 | 226/88 | % Sft | 3202 |
|  | Distempering two Coats (S.I 24-b/54). | 161.29 | 1043/90 | % Sft | 1684 |
|  | Preparing the surface & Painting with matt finish paint on masonry walls (2-coats) (S.I 31/A+B/P.73). | 1411.38 | 1772/38 | % Sft | 25015 |
|  | Preparing the surface & Painting with Weather Coat paint on masonry walls (2-coats) (S.I 31/A+B/P.73). | 1938.46 | 1498/48 | % Sft | 29047 |
|  | Preparing the surface & Painting of doors & Windows any type i/c edges 2nd & each subsequent coat (S.I 4-C-(i+ii)/68). | 397.25 | 1160/06 | % Sft | 4608 |
|  | Total: |  |  |  | 145615 |
|  | Less % below |  |  |  |  |
|  | Total Part A |  |  |  |  |
|  | PART – B (W/S & S/F WORKS) |  |  |  |  |
|  | S/F Fiber Glass tank of approved design and wall thickness set as specified i/c cost of nuts bolts and fixing in plate form of cement concrete 1.3.6 and make in connection in let & out let for pipe connection 1.2.4 (1000 gallons) (S.I. 13-C/21) | 1 No | 58962.94 | Each | 58963 |
|  | P/F CP Long bib cock ½ dia etc complete (S.I. 13(a)/19). | 4 Nos. | 1109/46 | Each | 4438 |
|  | P/F Muslim shower with double bib cock with crystal head etc complete (S.I. 19(a)/19). | 4 Nos. | 3432 | Each | 13728 |
|  | P/F Steel sink stainless local make complete with cost of wrought iron brackets & 6 inches built in wall 1-1/2” GP rubber brass west with 1-1/2” PVC waste pipe & making requisite number of holes in wall and plinth & floor for pipe connection & making good in CC 1.2.4 a steel sink stainless sixed 33” x 18” local make standard pattern (S.I.6/19-C) | 1-No | 5712/30 | Each | 5712 |
|  | P/F Sink Mixture with crystal head etc comlete | 1-No | 2745/60 | Each | 2745 |
|  | Total: |  |  |  | 85586 |
|  | Less % above |  |  |  |  |
|  | Total Part B |  |  |  |  |
|  | PART – C (ELECTRIC WORKS) |  |  |  |  |
|  | Wiring for light or fan point with 3/029 PVC insulated wire in 20mm (3/4”) PVC conduit recessed in the wall or column as required. (SI.124/P-15) | 10 Nos. | 1130/- | Each | 11300 |
|  | Wiring for Plug point with 3/029 PVC insulated wire in 20mm (3/4”) PVC conduit recessed in the wall col etc as required. | 3- Nos. | 985/- | Each | 2955 |
|  | P/F one way SP 5 amps switch flush type (S.I.219/33). | 15-Nos. | 58/- | Each | 870 |
|  | P/F two pin SP 5 amps plug & socket flush type (S.I.225/33). | 3-Nos. | 83/- | Each | 249 |
|  | P/L main or Sub manm PVC insulted with size 2-7/029 copper conductor in a dia PVC conduit on surface. | 35.70 | 171/- | Each | 6105 |
|  | Total Part C |  |  |  | 21479 |
|  | French polishing of doors & windows / floor work etc i/c preparing surface & painting by using Indian Lakhadan sprit pingment i/c lacquer tarpene oil etc complete as directed by the Engineer incharge. | 852 Sft |  | Sft |  |
|  | P/F Italian Spanish tilesor equelent quality as approved design sixe upto (24”x12”) laid over 1.2 cement mortor ¾” thick i/c filling the joints with white cement or tile grout in desired shape i/c finishing and cutting tile to proper profile in floor or dado i/c all cost of material labour as directed by the Engineer incharge | 529 Sft |  | Sft |  |
|  | P/F Plasitc tank 500 Gallons heavy guze i/c clutch nuts valve & necessary fitting etc complete. | 2 Nos. |  | Each |  |
|  | P/F UPVC Pipe 1” dia i/c cutting jointing with AGM salution with necessary fitting etc complete. | 137.17 Rft |  | P. Rft |  |
| B | --do—1/2 dia | 120 Rft |  | P. Rft |  |
|  | P/F CP double bib cock heavy pattern etc complete. | 3 Nos. |  | Each |  |
|  | P/F Sink Mixture heavy pattern as directed by Engineer incharge etc complete. | 1 No. |  | Each |  |
|  | P/F Handle Valve 1” dia etc complete | 4 Nos. |  | Each |  |
|  | P/F Exhaust Fan Plastic Body equelent quality i/c necessary electric connection as per approved quality etc complete. | 1 No. |  | Each |  |
|  | P/F Exhaust Fan Plastic Body equelent quality i/c necessary electric connection as per approved quality etc complete. | 1 No. |  | Each |  |
|  | P/F Wall Bracket Fan 18” dia make Pak or equelent quality necessary election etc competes. | 1 No. |  | Each |  |
|  | P/F Ceiling Fan 56” dia Sweep dia make Pak or equelent quality necessary election etc complete. | 4 No. |  | Each |  |
|  | P/F 4” dia UPVC Pipe of AGM Pak Arab or equilent quality make sch 80 a Water supply for roof and water and joints with AGM solution and making hole in walls plinth hand floor for pipe connection etc complete as directed by the Engineer incharge | 20.00 Rft |  | Rft |  |
| B | --do—2” dia | 6.50 Rft |  | Rft |  |
|  | P/F ½” dia CPVC Pipe of AGM Pak Arab or equilent quality make sch 80 a Water supply for roof and water and joints with AGM solution and making hole in walls plinth and floor for pipe connection etc complete as directed by the Engineer incharge | 15.00 Rft |  | Rft |  |
|  | P/F Motor Pump Piston Pump 2-Hp section 2x2 i/c all necessary fitting etc complete. | 1 No. |  | Each |  |
|  | P/F Motor Pump Piston Pump 1-Hp section 1x1 i/c all necessary fitting etc complete. | 1 No. |  | Each |  |
|  | Removing & debries dismantled material from site to out side KMC limit etc. | 4 Nos. |  | Each |  |
|  | Dismantling of tiles floor & dado etc complete. | 266.56 |  | Sft |  |
|  | P/F Basin Mixture heavy pattern CP coated lever type master make or equelent quality as directed by the Engineer incharge | 3 Nos. |  | Each |  |
|  | P/F Energy Saver 25 Watt i/c holder as per approved quality i/c necessary electric connection etc complete | 30 Nos. |  | Each |  |
|  | P/F Power plug 15 amps imported quality etc complete. | 6 Nos. |  | Each |  |
|  | P/F Commode seat cover imported quality etc complete. | 3 Nos. |  | Each |  |
|  | TOTAL |  |  |  |  |
|  | Grand Total: |  |  |  |  |
|  | SAY RS. |  |  |  |  |

# TERMS AND CONDTION

- Any typographical errors in the schedule B are subject to the correction with Reference to the Schedule of rate General 2012 enforced from 12.07.2012 as approved by the S R C, Sindh Karachi.

- Arbitration clause stands deleted from the agreement.

- No premium shall be paid on a Non schedule items.

- 100% WELL Graded crushed bajri shall be used in items of work without any

extra payment.

- No Any items of exceed then the Quantity Provided in the Schedule B without

Prior of the Executive Engineer.

6.  All sample of items should be got approved by the Executive Engineer.

CONTRACTOR       SUB ENGINEER                                    ASSISTANT ENGINEER

PROVINCIAL BUILDINGS SUB DIVISION-IX

KARACHI.