# general Abstract of cost

| S.No | Item Of Work | Sch: Amount | Non Sch: Amount | Total |
| --- | --- | --- | --- | --- |
| Part – I | Silt Clearance | Rs.71126/- | --- | Rs.71126/- |
| Part – II | Surface Drains | Rs.644551/- | --- | Rs.644551/- |
| Part – IIi | Catch Pit x 6 | Rs.101988/- | --- | Rs.101988/- |
| Part – IV | C .C Toping | Rs.372831/- | --- | Rs.372831/- |
| Part– V | Diesel Engine 16 BHP | . --- . | Rs.203000/- | Rs.203000/- |
|  | Total | Rs.1190496/- | Rs.203000/- | Rs.1393496/- |

CertificateS

- Any error or omission in the description of items of works rates original schedule will be correct. Accordingly in the instruction of the Executive Engineer, Public Health Engineering Division-I Khairpur.

- No premium will be allowed on non-schedule items.

- The work to be carried out as per PWD/PHE specifications approval rates working estimates.

CONTRACTOR.				            	          EXECUTIVE ENGINEER

PUBLIC HEALTH ENGINEERING

DIVISION-I KHAIRPUR

SCHEDULE-B

NAME OF WORK:- 	DRAINAGE SCHEME & DISPOSAL WORK AT VILLAGE WARIS

GHAMBHIR U/C PIR MANGIO TALUKA & DISTRICT KHAIRPUR

.

Sub-Head – I 		SILT CLEARANCE

1.	Earth work excavation in ashes sand soft soil or silt clearance (CSI No. 2 P- 1)

Qty: 11350.00 cft		@Rs.1134/38		P%0 cft		Rs.	12875/-

2.	Cleaning of Collecting Tank / Sump below grand level i/c dewatering of sullage water and disposal of garbage solid waste etc out side the tank etc complete (PHSI No. 1 P- No. 33)

Qty: 2794.60 cft		@Rs.11900/-		P%0 cft		Rs.	33256/-

3.	Extra for every 50 ft. additional lead of part there of   (CSI No. 8 (a) P.No.3) Total lead (2600-100)=30 lead @ Rs: 100/78 per lead 3023/40/=%o cft 1500/50=30 lead

Qty: 4790.0 cft		@Rs:3023/40		 P%o cft		Rs  	14482/-

4.	Manufacturing & supplying of 21” RCC man hole cover cast in 1:2:4 concrete Ratio 3 inch deep at center reinforced with 1/2” dia for steel bars at 4” c/c welded to 3/16” thick 2” wide M.S Plate two hook 3/8” dia tor bar including compacting i/c curing, and transportation with in 10 miles.

Qty.10Nos			@Rs:1051/25		Each 		Rs:	10513/-

Total:	Rs.	71126/-

Part - II			Surface drains

1.	Excavation in foundation of building bridges and other structure i/c dag belling refilling around the structure with excavated earth watering ramming lead up to one chain and lift up to 5 ft. (PH Sch. No.18 P.NO.4)

Qty. 4370.00	 cft		 @Rs.3176/25			P% 0 cft	Rs.	13880/-

2.	Cement concrete plain i/c placing compacting finishing curing complete i/c screening and washing stone aggregate w/o shuttering (CSI No.5 (i) P.NO.17)

(a) Ratio 1:4:8

Qty. 848.75 	cft		@Rs.11288/75		P% cft			Rs:	95813/-

(b) Ratio 1:2:4

Qty.31.94	cft		@Rs.14429/25		P% cft			Rs:	4609/-

3.	Pacca Brick work in foundation and plinth in cement sand Ratio (1;6)  (CSI No.7 P.NO.23)

Qty: 2353.12	cft		@Rs.11948/36		P% cft.			Rs.      281159/-

3.	Construction of standard of open drains cunnette block of cement concrete 1:2:4 in situ to design profile i/c cost of mould as per drawing i/c applying floating coat of cement 1/32” thick to the exposed face finished smooth curing etc complete as per detail drawing   (PHSI No.D P.NO.44)

Type – I

Qty: 1000.00	 rft		@Rs. 94/-		P rft			Rs.    	  94000/-

Type – II

Qty: 325.00	 rft		@Rs. 174/-		P rft			Rs.    	  56550/-

4.	Cement plaster 1:4 20’ height ½” thick (CSI No.11 P.NO.57)

Qty. 3637.50	sft		@Rs. 2283/93		P% sft			Rs.	83078/-

5.	RCC work in roof slab, beams columns rafts, lintels, and other structure members laid in situ of pre cost laid in position complete in all respect 1:2:4  (CSI No.7 P.NO.17)

Qty. 30.00	 cft		@Rs.337/-		P cft			Rs.	10110/-

6	Fabrication of mild steel reinforcement for cement concrete i/c cutting bending laying in position making K=joints and fastering i/c cost of binding wire also i/c removal of rust from bars (CSI No.7(ii) P.NO.19).

Qty: 1.07	 cwt		 @Rs.5001/70		P cwt			Rs.	  5352/-

Total		Rs.	644551/-

PART - III		Catch Pits

01.	Excavation in foundation building bridges and other structure i/c dag  bailing 	dressing refilling around the structure with excavated earth watering and ramming 	lead up 	to once chain and lift up to 5’ ft. (in ordinary soil) (CSI No.18 P.No.4)

Qty: 80.00 Cft 		@Rs. 3176/25		P%0 Cft 	Rs. 	254/=

02.	Cement concrete plain i/c placing compacting finishing curing complete i/c screening 	and 	washing stone aggregate w/o shuttering  (CSI No.5(I) P.No.15)

“A”	Ratio: 1:4:8

Qty: 10.00 Cft 		@Rs. 11288/75		P% Cft 	Rs. 	1129/=

“B”	Ratio: 1:2:4

Qty: 1.50. Cft 		@Rs. 14429/25		P% Cft 	Rs. 	216/=

03.	Erection and removal of centering for RCC and Plain CC work of partial wood,

(CSI No.19 (b) P.No.17)

Qty.2.50 cft.			@Rs. 3127/41			P% sft.		Rs. 	78/-

04.	Pacca brick work in foundation plinth in cement sand morotor  building i/c sticking of joints up ot 50 ft. height (in cement sand mortar) Ratio 1:6 (CSI No.4 (c) P.20).

Qty: 34.12 Cft 		@Rs. 11948/36		P% Cft 	Rs. 	4077/-

05.	Cement Plaster (1:4) up to 12 ft: Height ½ “thick.(CSI No 1.P.No. 58)

Qty: 44.75 Sft 		@Rs. 2283/93		P% Sft 	Rs. 	1022/-

06.	Providing RCC Pipe & collars of  class “A” and fixing in  trenches to required depth and Fixing to in position cutting and jointing with maxphalt composition and cement mortar1:1 and testing with rater pressure etc a head executed stuff. (WSI No2 P-24)

9” dia

Qty: 16.0 Rft  		@Rs.142/-			P-Rft	 	Rs. 	2272/=

07.	Fabrication of heavy steel work with angle tees flate iron round steel iron trues girder 	tanks etc i/c cutting riveting drilling bending assembling and fixing erecting in 	position excluding erection in position. (CSI NO. 2, P. NO. 91).

Qty 0.429 Cwt   		@Rs.4928/49			P-Cwt	 	Rs. 	2112/=

08.	Small iron work such as guessed in best plate knees bends strops sings etc i/c cutting drilling riveting handling assembling and fixing excluded erection and in  position (covers of catch pits). (CSI NO. 1, P. NO. 96)

Qty: 0.589	cwt			@Rs: 6420/61		P. cwt		Rs:	3784/-

9.	RCC work in roof slab, beams columns rafts, lintels, and other structure members laid in situ of pre cost laid in position complete in all respect 1:2:4  (CSI No.7 P.NO.17)

Qty. 3.87	 cft			@Rs.337/-		P cft		Rs.	1304/-

6	Fabrication of mild steel reinforcement for cement concrete i/c cutting bending laying in position making K=joints and fastering i/c cost of binding wire also i/c removal of rust from bars (CSI No.7(ii) P.NO.19).

Qty: 0.15	 cwt			 @Rs.5001/70		P cwt		Rs.	  750/-

Total:	Rs. 	16998/-

Part-iV			C.C Topping

1.	Barrow pit excavation undressed lead up to 100 rft in ordinary soil (CSI no 3 Page 1)

Qty. 5000.00  cft		@Rs. 2117/50		P%o cft		Rs. 	10588/-

2.	Earth work compaction Laying earth in 6” thick layer, leveling and dressing etc complete. (CSI.NO.13 (b) P/03)

Qty: 5000.00 cft		@Rs: 263/-		P% cft			Rs: 	1315/-

3.	Cement concrete brick or stone ballast 1 ½” to 2” gauge Ratio 1:4:8 (CSI No. 9 P. 16).

Qty: 1650.00 	cft		@Rs.9416/28		P% cft			Rs. 	155368/-

4.	Cement concrete plain 1:2:4 i/c placing compacting finishing and curing complete i/c screening and washing of stone aggregate without shuttering (CSI No.5 P.No.18)

Ratio 1: 2 : 4

Qty. 1250.00 cft		@Rs.14429/25	P% cft			Rs.	180365/-

5.	Extra for every 50 ft: additional lead or part therefore earth work soft or ordinary hard very hard.(CSI No: 8 P.No:3)

Qty: 5000.00 cft		@Rs:5039/-		P.%o cft		Rs:	25195/-

Total		Rs:    372831/-

PART V			DIESEL ENGINE 16 BHP

1	Providing and installing Diesel oil Engine 16 BHP Golden made Model-1100B 2200RPM coupled with golden non clogging Horizontal semi open impeller pump type GSWG-103 size 4”x3” capable discharge 70 GPM against the head of 47 ft i/c frame pully for engine and pump with belt and installing of pump set i/c cement concrete foundation with MS nuts Bolts i/c testing the pumping machinery sets against required head for 48 hours etc complete.

Qty: 1 Set			@Rs. 203000/-		P. set		Rs.	203000/-

Total:	Rs. 	203000/-