# general Abstract of cost

| S.No | name of Work | Sch; Amount | Non Sch: Amount | Total |
| --- | --- | --- | --- | --- |
| Part – I | SURFACE DRAINS | Rs.308428/- | --- | Rs.308428/- |
| Part – II | RCC Sewer | Rs.44692/- | --- | Rs.44692/- |
| Part – III | P.e Rising main | Rs.1318520/- | Rs.4725/- | Rs.1323245/- |
| Part-- iv | PUMPING MACHINERY | --- | Rs.959000/- | Rs.959000/- |
| Part – v | INTER CONNECTION | Rs.111014/- | Rs.31050 | Rs.142064/- |
| Part VI RENOVATION OF CIVIL STRUCTURE |  |  |  |  |
|  | SURFACE DRAINS | RS.1641671/- | --- | RS.1641671/- |
|  | Collecting Tank | Rs.31891/- | --- | Rs.31891/- |
|  | SCREENING CHAMBER | Rs.30508/- | --- | Rs.30508/- |
|  | PUMP HOUSE | Rs.72816/- | --- | Rs.72816/- |
|  | Compound wall | Rs.378896/- | --- | Rs.378896/- |
|  | Oxidation Pond | Rs.32954/- | --- | Rs.32954/- |
| Part – VII | CC Block | Rs.306085/- | --- | Rs.306085/- |
|  | Total: | Rs.4277475/- | Rs.994775/- | Rs.5272250/- |

CertificateS

- Any error or omission in the description of items of works rates original schedule will be correct. Accordingly in the instruction of the Executive Engineer, Public Health Engineering Division-I Khairpur.

- No premium will be allowed on non-schedule items.

- The work to be carried out as per PWD/PHE specifications approval rates working estimates.

CONTRACTOR.				                    EXECUTIVE ENGINEER

PUBLIC HEALTH ENGINEERING

DIVISION-I KHAIRPUR

SCHEDULE-B

NAME OF WORK:-	 REHABILITATION OF DRAINAGE SCHEME WADA MACHYOON U/C WADA MACHYOON TALUKA KHAIRPUR

PART –  I			Surface Drains

1.   	Excavation in foundation of building bridges and other structure i/c dag belling refilling around the

structure with excavated earth watering ramming lead up to one chain and lift up to 5 ft. (PH Sch.

No.18 P.NO.4)

Qty.1694.50 cft.		@Rs.3176/25			P%0 cft.		Rs. 	5382/-

2.   	Cement concrete plain i/c placing compacting finishing curing complete i/c screening and washing

stone aggregate w/o shuttering  (CSI No.5(i) P.NO.17)

- Ratio 1:4:8

Qty. 372.00 cft.		@Rs. 11288/75		P% cft.			Rs. 	41994/-

3.   	Pacca Brick work in foundation and plinth in cement sand Ratio (1;6)  (CSI No.7 P.NO.23)

Qty. 975.00 cft.		@Rs. 11948/36		P% cft.			Rs. 	116497/-

4.   	Construction of standard of open drains cunette block of cement concrete 1:2:4 in situ to design

profile i/c cost of mould as per drawing i/c applying floating coat of cement 1/32” thick to the

exposed face finished smooth curing etc complete as per detail drawing  (PHSI No.D P.NO.44)

Type-I			.

Qty.300.00 rft.			@Rs. 94/-			P rft.			Rs. 	28200/-

Type-II			.

Qty. 100 rft.			@Rs. 174/-			P rft.			Rs. 	17400/-

Type-III

Qty. 100 rft.			@Rs. 309/-			P rft.			Rs. 	30900/-

5.   	Cement plaster 1:4 12’ height ½” thick (CSI No.11 P.NO.57)

Qty. 1450.00 Sft.		@Rs. 2283/93			P% Sft.		Rs. 	33117/-

6.	RCC work in roof slab, beams columns rafts, lintels, and other structure members laid in situ of pre 	cost laid in position complete in all respect 1:2:4  (CSI No.7 P.NO.17)

Qty. 48.40 cft.			@Rs 337/-			P cft.			Rs. 	16311/-

7. 	Fabrication of mild steel reinforcement for cement concrete i/c cutting bending laying in

position making K=joints and fastering i/c cost of binding wire also i/c removal of rust

from bars (CSI No.7 (ii) P.NO.19).

Qty.1.72 cwt			@Rs. 5001/70			P cwt.			Rs. 	8603/-

8	Providing RCC Pipe with collars of and digging the trenches to required depth and 	Fixing to in position cutting and jointing with maxphalt composition and cement 	mortar 	1:1 and testing with rater pressure etc a head executed stuff. (PHSI No 1 P-	15)

a) 6” Dia Pipe

Qty: 40.0 Rft  		@Rs.250/60			P-Rft		 	Rs. 	10024/-

Total	Rs:	308428/-

### PART - II 			RCC Sewer 15” Dia

1.	Dismantling and removing road metaling (CSI. No.51 P.NO.13)

Qty.168.00 cft.		@Rs. 605/-			P% cft.		Rs. 	1016/-

2.	Excavation for pipe lines in trenches and pits in soft soil i/c trimming and dressing sides to true alignment and shape leveling of beds of trenches to correct level and grade cutting joints holes and disposal work of surplus earth within one chain as directed by Engineer in charge providing fence guards, lights, flanges and temporary crossing for non vehicular traffic wherever required lift up to 5’ ft. (1.50m) and lead up to one chain (30.50m) (PHSI No.01 P.No.60).

Qty..336.0 cft.			@Rs. 3600/-			P%0 cft.		Rs. 	1210/-

3.	Excavation for pipe lines in trenches and pits in wet soil i/c trimming and dressing sides to true alignment and shape leveling of beds of trenches to correct level and grade cutting joints holes and disposal work of surplus earth within one chain as directed by Engineer in charge providing fence guards, lights, flanges and temporary crossing for non vehicular traffic wherever required lift up to 5’ ft. (1.50m) and lead up to one chain (30.50m) (PHSI No.14 P.No.66).

Qty.336.00 cft	.		@Rs. 5400/-			P%0 cft.			Rs. 	1814/-

4.	Excavation for pipe lines in trenches and pits in slushy daldaly soil i/c trimming and dressing sides to true alignment and shape leveling of beds of trenches to correct level and grade cutting joints holes and disposal work of surplus earth within one chain as directed by Engineer in charge providing fence guards, lights, flanges and temporary crossing for non vehicular traffic wherever required lift up to 5’ ft. (1.50m) and lead up to one chain (30.50m) (PHSI No.14 P.No.67).

Qty.168.00 cft.			@Rs. 10800/-			P%0 cft.		Rs. 	1814/-

5.	Bailing or pumping out sub soil water during excavation concreting cost is stiu concrete or masonry work im foundation etc (CSI NO – 18 Page 52)

Qty: 168.00	cft		@Rs: 543/=			P% cft			Rs:	912/-

6.	Providing and fixing driving open timbering to trenches for depth up to 10 ft (SMI No. 73 P- No. 5)

QTY.96 CFT.			@RS.8340/-			P% CFT.		RS. 	8006/-

7.	Added For Additional lift of every 3 ft or part thereof (PHSI No. 16 P- No. 24)

Qty.168.00 cft.			@Rs. 550/-			P%0 cft.		Rs. 	92/-

8.	stone crushed graded crated 1” ring (SMI No. 73 P- No. 5)

Qty: 84.00 cft		@Rs.3100/-			P% cft			Rs.	2604/-

9.	Providing RCC Pipe (ASTM) C-76-62 T.C 76/70 class-II wall and fixing in trenches  	cutting and jointing with rubber rings i/c testing with water to a specified pressure. 	(PHSI No 1 P-17).

##### Qty.48.00 rft.			@Rs.515/-			P rft.			Rs. 	24720/-

10.	Refilling the excavated stuff in trenches in cement in 6” thick layers i/c watering 	trimming 	on full compaction etc complete (PHSI No.24 P.No.77).

100

##### Qty.907.20 cft.		@Rs.2760/-			P%0 cft.		Rs. 	2504/-

Total:	Rs.	44692/-

Part -III		P.E RISING MAIN 8” DIA

1.	Excavation for pipe line in trenches and pits soft soil i/c trimming and dressing sides to true alignment and shape leveling of beds of trenches to correct level and grade cutting joints holes and disposal of surplus earth within one chain as directed by Engineer inharge providing fence guards, lights flages and temporary crossing for non vehicular traffic where ever required lift up to 5 ft and lead upto one chain. (PH SCHI No 1 Page 60)

Qty: 12250.00 cft		@Rs:3600/-		P%o c ft		Rs:	44100/-

2.	Providing Laying & fixing in trenches i/c fitting , jointing and testing in all respect the high density polyethylene P.E pipes (HDPE-100) for water supply conforming ISO 4427/DIN 8074/8075 B.S 3580& PSI 3051 (PHSI No.1 (n) P-25)

8” Dia PN-8 (225mm)

Qty: 1400.00	rft		@Rs:860/-		P. rft			Rs:	1204000/-

3.	High density Polyethylene fitting (P.E -100) per each equivalent  (SMI No.IX P-No18)

PN-08

(i) Elbow 90o 8” Dia (225 mm)

Qty: 04	Nos		@Rs:6874/-		Each			Rs:	27496/-

(ii) Reducer 8”x6” (200mmx160mm)

Qty: 01	No		@Rs:5295/-		Each			Rs:	5295/-

(iii) But Fusion joints (SMI No.1 P-20)

Qty: 04	Joints		@Rs:1800/-		Each			Rs:	7200/-

4.	Refilling the excavated stuff in trenches in cement in 6” thick layers i/c watering trimming on full compaction etc complete (PHSI No.24 P.No.77).

Qty: 11025.00 cft		@Rs: 2760/-		P. %o cft		Rs:	30429/-

Total.	Rs.	1318520/-

Non Schedule item

1.	 Flange	Adopter 8” Dia(MR)

Qty: 01	No		@Rs:4725/-		Each			Rs:	4725/-

Total.	Rs.	4725/-

PART – IV			PUMPING MACHINERY

1	Supplying installing in position transportation at site for drainage scheme Wada Machyoon     Taluka & District Khairpur Electric set 1450 RMP (KSB) ILA  - 133 – 6 AA 60 3 – made by 15 BHP 3-Phase 50 Cycles coupled KSB Non – Clogging pump size 5” x 4 1/2” mounted on common steel frame capable of discharging 600 gallons per minute against a head of 40 ft i/c installation of pumping set cement concrete foundation ratio 1:4:8 and ratio 1:2:4 with ballast with  MS nuts & bolts ½” dia to be embedded in C C foundation i/c providing and fixing star delta starter complete with no volts and over load release & required i/c providing and fixing on prepared board iron clad triple pole and neutral switch fuse with rewire able type fuse link carried 500 volts grade i/c providing fixing metal board accommodate main or sub main control switch recessed in the wall size 3” x 2 ½” i/c providing phase indication bulbs with holder and providing natural electric wiring for main switch 3 – 7/0.85 (7.036) PVC insulated wise 660 volts grade in 25 mm 1” PVC conduct on surface as required i/c providing and fixing earthing set 0.6 mx 6.3 mm 2 x 2 ½” C.I Plate buried in the ground at depth of 3.7 mm (12”) or less as per PAK PWD specification and testing of the pumping set against set required head of 72 hours etc complete. (R.A Attached)

Qty: 1.00	 Set		@Rs.750000/-			P. set		Rs.	750000/-

Diesel Oil Engine

2.	Supplying installing in position i/c transportation to site of work at Diesel work Diesel Engine pumping set 16 BHP with 1450 RPM coupled with Golden non clogging pump size 3” x4”” mounted on steel frame or base plate capable of discharge 260 GPM against a head of 40’ ft i/c making cement concrete foundation (Ratio 1:2:4) and C.C (1:4:8) with ballast with MS nuts and bolts of 5/8” Dia to be embedded in CC foundation i/c testing of pumping set against the required head of 72 hours etc complete.

Qty: 1.00	 Set		@Rs.209000/-			P. set		Rs.	209000/-

part – III 		NTER CONNECTION

1.	Providing Laying & fixing in trenches i/c cutting fitting and jointing out testing in all respect the high testing polyethylene P.E pipe (PHSI No-1 P- No 25)

6” Dia P.E Pipe PN-10

For 2 sets

Qty: 34.00	rft		@Rs:530/-			P. rft		Rs:	18020/-

(ii)	38” dia 90mm

Qty: 16.00	rft		@Rs:178/-			P. rft		Rs:	2848/-

2.	Manufacturing & installing with welding MS pip 6” Dia made out of MS sheet 3/8” thick cutting welding drilling etc complete

6” Dia

Qty: 8.00 rft.			@Rs: 749/34			P-rft		Rs:	5995/-

3” Dia

Qty: 8.00 rfts.			@Rs:385/89			P-rft		Rs:	3087/-

3.	Providing & installing Mild steel flange made out M.S Steet 3/8 thick directing hole etc complete. Based on schedule item .

i)	6” Dia

Qty: 14.00 Nos.		@Rs: 804/77			Each		Rs:	11267/-

ii)	3” Dia

Qty: 4.00 Nos.			@Rs: 500/89			Each		Rs:	2004/-

4.	Supplying CI sluice valve heavy pattern test pressure 21.0 kg sq cm 300 lbs sq inch.

(MSI No.2 P.9)

6” Dia

Qty. 2.00 Nos			@Rs.9360/-			Each		Rs.	18720/-

5.	Supplying reflex valve heavy pattern test pressure 21.0 kg or CM or 300 lbs sq inch. (SMI NO 6 P No

a)	6” Dia

Qty. 2.00 Nos			@Rs. 4062/50			Each		Rs. 	8125/-

b)	3” Dia

Qty. 1.00 Nos			@Rs. 1543/75			Each		Rs. 	1544/-

6.	CI Bend with flanged end with holes i/c trenches and facing of flanged for all size (SMI No.8 P.12).

a)	6” Bend 90 Degree

Qty. 0.63 cwt			@Rs. 6096/-			P. cwt		Rs. 	3840/-

7.	CI Tappers flat bottomed

Qty. 0.35 cwt			@Rs. 6096/-			P. cwt		Rs. 	2134/-

8	CI Tee flanged end with holes i/c turning & facing of flanges.

Tee 6”x6”x6”

Qty: 0.312	cwt		@Rs:6096/-			P. cwt		Rs:	1902/-

9.	Jointing CI/MS flanged pipe to special flanged and inside threaded i/c supplying rubber packing the required thickness nuts and bolts with wisher and other required for jointing and testing the joint to the specified pressure etc complete (PHSI No.1 P.35).

i)3” dia

Qty: 4.00	Joints		@Rs: 499/-			P. Joint	Rs:	1996/-

ii)6” dia

Qty:14.00	Joints		@Rs: 938/-			P. Joint	Rs:	13132/-

10	But fusion joint(SMI No11 P-20)

i)6” dia

Qty: 14.00	Joints		@Rs: 1000/-			P. Joint	Rs:	14000/-

ii)3” dia

Qty: 4.00	Joints		@Rs: 600/-			P. Joint	Rs:	2400/-

Total: -		Rs.	111014/-

NON SCHEDULE ITEM

1.	Flange Adaptor(NSI)

i)	6”

Qty. 14.00 Nos		@Rs. 2025/-			Each		Rs. 	28350/-

ii)	3”

Qty. 4.00 Nos			@Rs. 675/-			Each		Rs. 	2700/-

Total: -		Rs.	31050/

PART-V  	RENOVATION OF CIVIL STRUCTURE

PART-I 			SURFACE DRAINS

1.	Earth wok excavation in undressed lead up single through of kassi Phariah or shavel in ashes sand sol or silt clearance   (CSI No2 P-I)

Qty. 11340 rft			@Rs. 1134/38		P.%0cft		Rs. 	15008/-

2.	Dismantling Brick Work line or cement mortar (CSI  No.13 P-10)

Qty: 337.50 cft		@Rs.1285/63		P% Cft			Rs.	4339/-

3.	Dismantling CC Reinforced separating (CSI NO 20 P-10).

Qty: 24.20 cft			@Rs.5445/-		P% Cft			Rs.	1318/-

4.	Pacca Brick work in foundation and plinth in cement sand Ratio (1;6)  (CSI No.7 P.NO.23)

Qt.7800.00 cft.		@Rs.11948/36	P% cft.			Rs. 	931972/-

5.  	 Cement plaster 1:4 20’ height ½” thick (CSI No.11 P.NO.57)

Qt.21200 Sft.		@Rs. 2283/93			P% Sft.		Rs. 	484193/-

6.   	Construction of standard of open drains cunette block of cement concrete 1:2:4 in situ to design

profile i/c cost of mould as per drawing i/c applying floating coat of cement 1/32” thick to the

exposed face finished smooth curing etc complete as per detail drawing  (PHSI No.D P.NO.44)

Type-I

Qt. 1500.00 rft.		Rs. 94/-		P rft.			Rs. 	141000/-

7.	RCC work in roof slab, beams columns rafts, lintels, and other structure members laid in situ of pre 	cost laid in position complete in all respect 1:2:4  (CSI No.7 P.NO.17)

Qt.79.20 cft.			Rs. 337/-		P cft.			Rs. 	26690/-

08 	Fabrication of mild steel reinforcement for cement concrete i/c cutting bending laying in

position making K=joints and fastering i/c cost of binding wire also i/c removal of rust

from bars (CSI No.7 (ii) P.NO.19).

Qt.2.82 cwt			@Rs. 5001/70		P cwt.			Rs. 	14105/-

09. 	Cement concrete plain i/c placing compacting finishing curing complete i/c screening and washing

stone aggregate w/o shuttering  (CSI No.5(i) P.NO.17)

Ratio 1:2:4

Qty. 159.72 cft.		@Rs. 14429/25	P% cft.			Rs. 	23046/-

Total;            Rs. 	1641671/-

PART-II 			RCC COLLECTING TANK 15”

1	Pacca brick work in other than building  i/c striking of joints upto 20 ft height in cement  sand mortar 1:6 (CSI No – 7 (e) Page -22)

Qt.100.48 cft.			@Rs.12346/65		P% cft.			Rs. 	12406/-

2.  	Cement plaster 1:4 20’ height ½” thick (CSI No.11 P.NO.57)

Qt.251.20 Sft.			@Rs. 2283/93			P% Sft.		Rs. 	5737/-

3	Cleaning of Collecting Tank / Sump below ( PHSI No1 P-59)

Qt.1059.75 cft.		@Rs.11900/-			P%0 cft.		Rs. 	12611/-

04	Extra for Pacca brick in staining of wells of other circular masonry   (csi No.10 P.No.23)

Qt: 100.48 cft.			@Rs. 1131/63			P% cft.			Rs. 	1137/-

Total;            	Rs. 	31891/-

PART-III 			SCREENING CHAMBER”

1	Pacca brick work in other than building  i/c striking of joints upto 20 ft height in cement  sand mortar 1:6 (CSI No – 7 (e) Page -22)

Qt.43.50 cft.			@Rs.12346/65		P% cft.			Rs. 	5371/-

2.  	Cement plaster 1:4 20’ height ½” thick (CSI No.11 P.NO.57)

Qt.116.00 Sft.			@Rs. 2283/93			P% Sft.		Rs. 	2649

3	Cleaning of Collecting Tank / Sump below ( PHSI No1 P-59)

Qt.160.00 cft.			@Rs.11900/-			P%0 cft.		Rs. 	1904/-

4.	Fabrication of heavy steel work with angles tees flat iron and sheet iron making trusses girders tanks etc i/c cutting revering handing assembling and fixing but eluding eriction in position (CSI No.2 (a) P.NO.90)

Qty: 3.99	cwt		@Rs: 4928/49			P. cwt			Rs:	19665/-

5.	Erection and fitting in position of iron trusses staging of water tank etc complete (CSI no - 3 Page 96)

Qty: 3.39	cwt		@Rs: 271/04			P. cwt			Rs:	  919/-

Total;            	Rs. 	30508/-

Part – v 	-(A)	pump house

01	Providing and laying 1” thick topping cement concrete (1:2:4) i/c surface fixing and deriding in to panels. (CSI No 16 P No 42)

Qty: 208.0	sft		@Rs.4411/82		P% sft			Rs:	  9177/-

02	Removing cement or line pilaster . (CSI No 53 P No 13)

Qty: 896.0	sft		@Rs.121/-		P% sft			Rs:	  1084/-

03	Applying Floating Coat of cement 1/32” thick  . (CSI No 14 P No 52)

Qt: 896.0 sft	.		@Rs. 660/-			P%0 sft.		Rs. 	5914/

04.	Cement Plaster (1:4) up to 20 ft: Height ½ “thick. (CSI No 1.P.No. 58)

Qty: 896.0	sft		@Rs.2179/52			P% sft			Rs:	20464/-

5.	Fabrication of heavy steel work with angles tees flat iron and sheet iron making trusses girders tanks etc i/c cutting revering handing assembling and fixing but eluding eriction in position (CSI No.2 (a) P.NO.90)

Qty: 2.37	cwt		@Rs: 4928/49			P. cwt			Rs:	11681/-

6.	White washing (one coats) CSI No – 26 Page 60)

Qty: 1688	sft		@Rs: 416/63			P% sft			Rs:	7033/-

7.	Color Washing Two Coats (CSI No – 25 Page 60)

Qty: 1688.0 	sft		@Rs: 859/90			P% sft			Rs:	14515/-

8	Painting old surface Preparing surface painting doors & windows any type i/c edge (CSI No – 5 page 77)

Qty: 152.00 	sft		@Rs: 1160/06			P% sft			Rs:	1763/-

9	Preparing surface painting doors & windows any type i/c edge (CSI No – 5 page 77)

Qty: 56.00	sft		@Rs.2116/41			P% sft			Rs:	1185/-

Total;            	Rs. 	72816/-

part V		Compound Wall

1.	 Pacca Brick work in foundation and plinth in cement sand Ratio (1;6)  (CSI No.7 P.NO.23)

Qt.224.0 cft.			@Rs.11948/36		P% cft.			Rs. 	26764/-

2.	DPC with cement sand & shingle concrete 1:2:4 i/c coated of asphaltic mixture (CSI No.27 P.23).

3” thick

Qty. 112.00 sft		@Rs.4982/18			P% sft			Rs.	5580/-

3.	Pacca brick work other than building i/c striking of joints up ot 50 ft. height (in cement sand mortar) Ratio 1:4  (CSI No.7 P.25).

Qty. 1002.00 cft		@Rs.12346/65		P% cft			Rs.	123713/-

4.  	Cement plaster (1:4) upto 12’height ½ ”thick   (CSI No.11 (b) P.NO.58).

Qty.3885.50 	 sft		@Rs.2283/93			P% sft			Rs.	88742/-

5.	White washing (one coats) CSI No – 26 Page 60)

Qty: 10374.08	sft		@Rs:416/63			P% sft			Rs:	43221/-

6.	Colour Washing Two Coats (CSI No – 25 Page 60)

Qty: 10374.00 sft		@Rs: 859/90			P% sft			Rs:	89206/-

7	Painting old surface Preparing surface painting doors & windows any type i/c edge (CSI No – 5 page 77)

Qty: 144.0 	sft		@Rs: 1160/06			P% sft			Rs:	1670/-

Total;            	Rs. 	378896/-

PART –VI		 OXIDATION POND

1.	Earth work excavation in ashes sand soft soil or silt clearance undressed lead up to 50 ft  (CSI No.2 P-1)

Qty: 16215.0cft		@Rs. 1663/75 		P% cft			Rs.	26977/-

02.	Earth work compaction (soft ordinary hard soil Laying earth in 6” thick layer, leveling and dressing complete. (CSI.NO.13 (b) P/03)

Qty. 11368.00 cft		@Rs: 354/-			P%0 cf		Rs. 	4024/-

03	Extra for every 50 ft. additional or part there of for earth work soft ordinary hard and very hard. (csi No.8 P.No.3)

Qt: 4847.0 cft.			@Rs. 403/12			P%0 cft.		Rs. 	1953/-

Total:	Rs. 	32954/-

PART - VII	C.C bLOCK

01.	Barrow pit excavation undressed led up to 100 rft in ordinary soil (CSI no.3 p.1)..

Qt.4000.00 cft.		@Rs.2117/50			P%0 cft.		Rs. 	8470/-

02.	Extra for every 50 ft: additional lead or part therefore earth work soft or ordinary hard very hard.(CSI No: 8 P.No:3) Total land 2600 (-) 100 =  50 leads  @Rs. 100/78 Per Lead Rs. 5039/=

Qty: 4000 cft		@Rs: 5039/-		P%0 cf			Rs. 	20156/-

03.	Earth work compaction (soft ordinary hard soil Laying earth in 6” thick layer, leveling and dressing complete. (CSI.NO.13 (b) P/03)

Qty: 4000.00	 Cft 		@Rs:-263/-			P%0 Cft	 	Rs. 	1052/-/-

04.	Cement concrete brick or stone ballast 1 ½” to 2”  guage Ratio 1:4:8 (CSI No.9 P.16).

RATIO 1:4:8

Qt.1320.00 cft.		@Rs. 9416/28			P% cft.			Rs. 	124295/-

05.	Cement concrete plain i/c placing compacting finishing curing complete i/c screening and washing stone aggregate w/o shuttering  (CSI No.5(i) P.NO.17)

Ratio 1:2:4

Qt. 1000.0 cft.			@Rs. 14429/25		P% cft.			Rs.	 144293/-

06	Erection and removal of centering for RCC and Plain CC work of partial wood,

(CSI No.19 (b) P.No.18)

Qty. 250.00sft.		@Rs. 3127/41			P% sft.			Rs. 	7819/-

Total:-		Rs:	306085/-