# abstract of cost

| S.No | name of Work | Sch; Amount | Non Sch: Amount | Total |
| --- | --- | --- | --- | --- |
| Part – I | Tubwell 2 Nos | Rs.311542/- | --- | Rs.311542/- |
| Part-- II | PUMP HOUSE 8’x8’ 2Nos | Rs.328222/- | --- | Rs.328222/- |
| Part – III | pumping machinNery I/C inter Connection 2sets | Rs.56246/- | Rs.1154800/- | Rs.1211046/- |
| Part – III | RISING MAIN 4” DIA | Rs.4352154/- | Rs.107010/- | Rs.4459164/- |
| Part-- iv | P.E DISTRIBUTION SYSTEM | Rs.2937631/- | Rs.3780/- | Rs.2941411/- |
| Part-- iv | STAFF QUARTER | Rs.274395/- | --- | Rs.274395/- |
| Part-- iv | LSR 2Nos | Rs.356598/- | --- | Rs.356598/- |
|  | Total: | Rs.8516788/- | Rs.129590/- | Rs.9776378/- |

CertificateS

- Any error or omission in the description of items of works rates original schedule will be correct. Accordingly in the instruction of the Executive Engineer, Public Health Engineering Division-I Khairpur.

- No premium will be allowed on non-schedule items.

- The work to be carried out as per PWD/PHE specifications approval rates working estimates.

CONTRACTOR.				                EXECUTIVE ENGINEER

PUBLIC HEALTH ENGINEERING

DIVISION-I KHAIRPUR

SCHEDULE-B

Name of work :-	WATER SUPPLY SCHEME FOR VILLAGE GHULAM MUHAMMAD JAGEERANI & AALAM GOPANG   U/C BUGRO TALUKA KHAIRPUR

PART – I		TUBE WELL

- Boring for tube well in all water bearing soil from ground level up ro 100” ft 30.50

meter depth i/c sinking and will drawing of crossing pipe 12 dia boring (PHSI No.1

P-36)

12” Dia

Qty: 95.00 Rft. 		@Rs.743/-		P-Rft		Rs.	70585/-

2.	Supplying and installing PVC strainer “B”  class of approved design quality and make necessary sockets etc complete for 8” Dia PVC  strainer (PHSI.NO:1 P-43)

8” Dia

Qty: 69.00 Rft. 		@Rs.554/05		P-Rft		Rs.	38229/-

3.	Supplying and installing PVC Blind Pipe “B”  class of approved design quality and make necessary sockets etc complete for 8” Dia PVC  strainer (PHSI.NO:1 P-37)

8” dia

Qty: 26.00 Rft. 		@Rs516/80		P-Rft		Rs.	13437/-

4.	Surrounding with grated Bajri size 3/8 40 1/8 Ca to 3 mm in between bare and bind pipe for the  following dia meter of strainer (PHSI No:13 P-35)

12” dia

Qty: 95.00 Rft. 		@Rs193/-		P-Rft		Rs.	9975/-

5.	Supplying MS bail plug (PHSI No: 8 P-43)

Qty: 1.00 No.	 		@Rs.2607/-		Each		Rs.	2607/-

6.	Supplying and fixing well 8” PVC Plug(SMI No P-5)

Qty: 1.00 No			@Rs.1000/-		Each		Rs.	1000/-

7.	Development of Tub-Well (PHSI No80 P-3)

Qty: 95 rft			@Rs.200/-		P-Rft		Rs.	19000/-

8	Taking and preserving 21 lbs (Kg) samples of Estrte from bore hole (PHSI No.6 P.43)

Qty: 4.00 Sample		@Rs.101/-		P-Sample	Rs.	404/-

9	Taking Samples of water from bore hole and placing into two separate bottles

(PHSI No: 7 P-43)

Qty: 4.00 Sample 		@Rs. 131/-		P.Sample	Rs.	534/-

Total:	Rs.	92771/-

Part –II		pump house 8 x 8

1.	Excavation in foundation building bridges and other structure i/c dag  bailing dressing refilling around the structure with excavated earth watering and ramming lead up to once chain and lift up to 5’ ft. (in ordinary soil) (CSI No.18 P.No.4)

Qty: 140.00	cft		@Rs: 3176/25		P% cft			Rs:	445/-

2.	Cement concrete brick or stone ballast ½ to 2 gauges Ratio 1:4:8 (CSI No.6 P.No.16).

Ratio: 1:4:8

Qty: 54.19	cft		@Rs: 9416/28		P% cft			Rs:	5105/-

3. 	Pacca brick work in foundation plinth in cement sand mortar 1:6 (CSI No – 4 Page  23 )

Qty: 248.93	cft		@Rs: 11948/36	P% cft			Rs:	29743/-

4.	Damp proof course of cement sand and shingle concrete 1:2:4 i/c two coats of asphalted mixture (CSI No.28 P.No.18)

Qty: 39.20	cft		@Rs: 4982/18		P% cft			Rs:	1953/-

5.	Rough Cost srtucco cement plaster ¾” thick in proportional 1:1 ½ :1 ½ cement hill sand bajri in pattern (CSI No 32 P-55)

Qty: 158.08	cft		@Rs: 2306/10		P% cft			Rs:	3645/-

6	Pacca brick work in ground floor i/c striking of jointing cement sand mortar 1:6 (CSI No: 5

P-24)

Qty: 218.63	cft		@Rs: 12674/36		P% cft		Rs:	27709/-

7.	Fabrication of heavy steel work with angles tees flat iron and sheet iron making trusses girders tanks etc i/c cutting revering handing assembling and fixing but eluding eriction in position (CSI No.2 (a) P.NO.90)

Qty: 4.41	cwt		@Rs: 4928/49		P. cwt			Rs:	21735/-

8.	Errection & fitting  in position iron trusses fixing in position (C,S,I No.6 P-No.91)

Qty: 4.41	cwt		@Rs: 271/04		P. cwt			Rs:	1195/-

9.	Supplying fixing in position iron steel gril using sold square bars of size ¾” x ¼” etc complete (CSI NO 20 – Page 93)

Qty: 24.00	sft		@Rs: 180/50		P. sft			Rs:	4332/-

10.	RCC work in roof slab, beams columns rafts, lintels, and other structure members laid in situ of pre cost laid in position complete in all respect (PHSI No.6 P.NO.17)

Qty: 4.88	cft		@Rs: 337/-		P. cft			Rs:	1645/-

11.	Fabrication of mild steel reinforcement for cement concrete i/c cutting bending laying in position making joints and festering i/c cost of binding wire also i/c removal of rust from bars (CSI No.7 (a) P.NO.19

Qty: 0.19 	cwt		@Rs: 5001/70		P. cwt			Rs:	950/-

12	Providing and laying 3” thick topping cement concrete finishing diving to panels (SCI  No   P No  )

Qty: 64	sft		@Rs: 3275/50		P% sft			Rs:	  2096/-

13	Providing & Fixing MS Girder Cross (SMI No160 P-6)

Qty: 2.04 	cwt		@Rs: 3850/		P. cwt			Rs:	7854/-

14	Providing & Fixing MS Tee Iron (SMI No140 P-6)

Qty: 1.30 	cwt		@Rs: 3575/		P. cwt			Rs:	4678/-

15	2nd class tills roofing consisting of 4” earth & 1: thick mud plaster with Gobri leping over ½” thick cement plaster 1:6 with 34 lbs of hot bitumen coating sand blinded providing over one layer of 12”x6”x2” thick sand washed layer of 1:6 cement pointing under side of tiles complete i/c curing etc.

Qty: 90.25	sft		@Rs: 7607/25		P%sft			Rs:	  6866/-

16	Khaprail of CC 12”x6”x1” of approved designed/shape lead flat in 1:2 grey cement mortor over of bind of ¾” thick cement motor 1:2(CSI No-3 P-35)

Qty: 90.25	sft		@Rs: 8977/90		P% sft			Rs:	  8103/-

17.	Cement Plaster (1:4) up to 12’ ft: Height3/8.thick (CSI No 11.P.No. 58)

Qty: 598.00	sft		@Rs: 2206/60		P% sft			Rs:	13151/-

18.	Aster Plaster (1:6) up to 12 ft: Height 3/8”thick. (CSI No 11.P.No. 59)

Qty: 596.00	sft		@Rs: 2197/52		P% sft			Rs:	13097/-

19.	White washing (two Coats) (CSI No – 26 Page 53)

Qty: 596	sft		@Rs: 416/63		P% sft			Rs:	2483/-

18.	Color Washing Two Coats (CSI No – 25 (a) Page 53)

Qty: 596.0 	sft		@Rs: 859/90		P% sft			Rs:	5125/-

19.	Preparing surface painting doors & windows any type i/c edge (CSI No – 5 page 76)

Qty: 104.00	sft		@Rs: 2116/41		P% sft			Rs:	2201/-

Total	Rs:	164111/-

PART – II

(A)PUMPING MACHINERY & INTER CONNECTION 2sets

1	Providing laying and fixing tranches i/c fitting jointing and testing etc complete in all respect the high density polyethylene PF pipe (HDPE-100) for W/S confirming ISO conforming ISO 4427/DIN 8074/8075 B.S 3580 & PSI 3051 (GSI -1 P-25)

PN-10 (3” dia 90 mm)

Qty: 20.00	rft		@Rs: 178/-			P. Rft		Rs:	3560/-

2	CI Bend with flanged end with Socket at other and for all size (PHSI (M) 1 No4 P-12) 3” dia

Qty. 1 cwt			@Rs. 6096/-			P. cwt		Rs. 	23777/-

3.	CI sluice valve heavy pattern test pressure 21.0 kg sq cm 300 lbs sq inch.

(MSI No.2 P.9)

4” Dia

Qty. 1.00 Nos			@Rs.4290/-			Each		Rs.	4290/-

4	CI reflex valve heavy pattern test pressure 21.0 kg or CM or 300 lbs sq inch. (SMI NO 4 P No  12   )

3” dia

Qty. 1.00 Nos			@Rs. 1543/75			Each		Rs. 	1544/-

5.	Providing installing MS Flanged made out MS plate having a thickness and total weight as mentioned against each item. It includes the cost of making holes facing welding nuts bolts rubber packing whit load fitter cartage etc complete (PHSI 1-8P-   (Part –C)

Qty: 8.00 Nos.			@Rs: 945/-			Each		Rs:	7560/-

6.	But Fusion joint (PHS (M) P-20)

3” Dia

Qty. 8.00 Nos			@Rs.600/-			Each		Rs.	4800/-

7	Jointing CI/MS flanged pipe to special flanged and inside threaded i/c supplying rubber packing the required thickness nuts and bolts with wisher and other required for jointing and testing the joint to the specified pressure etc complete (PHSI No.1 P.35).

3” dia

Qty: 8.00	Joints		@Rs: 499/-			P. Joint	Rs:	3992/-

Total: -		Rs.	28123/-

2x28123	Rs.	56246/-

NON SCHEDULE ITEM

PUMPING MACHINERY

1	supplying installing in position i/c transportation  at site of work for rural water supply scheme for facility for Guhlam Muhammad Jagirani  Khairpur   golden saer submersible pump NR-151C/15 type 6(21/2) with  15 BHP 3 phase   electric motor original sear – Italy for discharge 120 GPM at 180 ft head with local made accessories for 180 ft setting as under electric drop cable 20mm 2-3 core into 70ft water proof joint GI pipe 2-1/2 11L M grade into 70ft GI bend 2-1/2”x 1GI delivery pipe 2-1/2” 11l-M grade x 5ft  MS Foundation plate with T-Bolts MS holding clamp 2-1/2”x1 socket 2-1/2”x70 MS bore cover plate into 1 wire rope with clamp x70 feet i/c providing and fixing automatic MCU (Motor control unit consist oil stator (MEECO Pakistan) MCCB breaker 10-A to 50-A (Shihin Taiwan) cutout 60-A china Amp: meter 60-A (SELCO china) volt meter 500-V (SELCO China) phase indication light (China) include Transformer oil (2litter ) Panel Board 18”x26” and internal wiring etc complete and providing fixing earthing arrangements and testing of the pumping set against 72 hours (R.A)

Qty: 1.00	 Sets		@Rs.569000/-			P. set		Rs.	569000/-

2	flange adopter” dia (N.S.I)

3” Dia

Qty. 8.00 Nos			@Rs.675/-			Each		Rs.	5400/-

Total: -		Rs.	574400/-

2x574400	Rs.	574400/-

Part –IV  		P.E PIPE Line

RISING MAIN 6” DIA

1.	Dismantling and removing road mettaling (CSI No. 5 P-No.14)

Qty:2000.00 Cft		@Rs.605`		P% Cft			Rs.	12100/-

2.	Excavation for pipe line in trenches and pits soft soil i/c trimming and dressing sides to true alignment and shape leveling of beds of trenches to correct level and grade cutting joints holes and disposal of surplus earth within one chain as directed by Engineer inharge providing fence guards, lights flages and temporary crossing for non vehicular traffic where ever required lift up to 5 ft and lead upto one chain. (PH SCHI No 1 Page 60)

Qty: 61000.00 cft		@Rs:3600/-		P%o c ft		Rs:	219600/-

3.	Providing Laying & fixing in trenches i/c cutting fitting and jointing out testing in all respect the high testing polyethylene P.E pipe (PHSI No-1 P- No 25)

(160 mm)

4” Dia

Qty: 9000.00	rft		@Rs:440/-		P. rft			Rs:	3960000/-

4.	P.E Bend 90o degree 6” Dia (SMI NO 1 P No 13 )

PN-8

Qty: 2.00	Nos		@Rs:4349/-		Each			Rs:	8698/-

5	CI sluice valve heavy pattern test pressure 21.0 kg sq cm 300 lbs sq inch.

(MSI No.2 P.9)

6” Dia

Qty. 1.00 Nos			@Rs.9360/-		Each			Rs.	9360/-

6.	P.E Bend 45o degree 6” Dia (110 mm)

Qty: 2.00	Nos		@Rs:3480		Each			Rs:	6560/-

7	FH Tee 6” Dia (SMI No. 10 P- No 16)

Qty: 1.00	Nos		@Rs: 1375/-		Each			Rs:	1375/-

8.	Equal Tee160x16x160

Qty: 1.00	No		@Rs:5219/-		Each			Rs:	5219/-

9	Butt fusion joint (SMI No.H P.20).

6’ Dia

Qty: 11.00	Joints		@Rs: 1000/-		Each			Rs:	11000/-

10.	Air valve double aiting 3” x 1” (Local) (SMI No.8 P-No.12)

Qty: 1.00	Nos		@Rs: 4000/-		Each			Rs:	4000/-

11.	Providing chamber 2 ½ x 2 ½ (750x750 mm) inside dimension 4 ½ (1372 mm) deep as per approved design for sluice valve 3” to 12” dia with 18” (457 mm) dia inside cast iron cover and frame cwt=  (cwt 3 qe) fixed in RCC 1:2:4 (102 mm) thick brick masonry walls set in 1:6 cement mortar 6” (1152 mm) thick cement plaster 1:3 to all inside wall surfaces and to top i/c providing and fixing MS foot rest at every one foot beyond 2 ½ ft depth curing excavation back filling and disposal of surplus earth etc complete (PHSI No.1  P.  49).

Qty. 2.00 Nos			@RS: 18820/20		Each		Rs:	 37640/-

12.	Providing installing MS Flanged made out MS Sheet having a thickness and total weight as mentioned against each item. It includes the cost of making holes facing welding nuts bolts rubber packing whit load fitter cartage etc complete (PHSI 1-8P-   (Part –C)

Qty: 2.00 Nos.			@Rs: 804/77			Each		Rs:	1610/-

13.	Refilling the excavated stuff in trenches in cement in 6” thick layers i/c watering trimming on full compaction etc complete (PHSI No.24 P.No.77).

Qty: 56700.00 cft		@Rs: 2760/-		P. %o cft		Rs:	156492/-

toalRs.	4352154/-

Non Schedule item

1	PE  Reducer 6” x 4” dia (NSI   )

Qty: 1	No			@Rs:3270/-		Each			Rs:	3270/-

2	Tee un equal 6’x6’x4’ (NSI)

Qty: 1.00	No		@Rs:9100/-		Each			Rs:	91000/-

3	Tee un equal 6’x6’x3’ (NSI)

Qty: 1.00	No		@Rs:8690/-		Each			Rs:	8690/-

4	Flange Adopter (NSI)

Qty: 2.00 Nos.			@Rs: 2025/-			Each		Rs:	4050/-

toal Rs.	107010/-

PART – V	P.E  DISTRIBUTION SYSTEM 4”& 3” DIA

1	Excavation for pipe lines in trenches and pits in soft soil i/c trimming and dressing sides to true alignment and shape leveling of beds of trenches to correct level and grade cutting joints holes and disposal work of surplus earth within one chain as directed by Engineer in charge providing fence guards, lights, flanges and temporary crossing for non vehicular traffic wherever required lift up to 5’ ft. (1.50m) and lead up to one chain (30.50m) (PHSI No.1 P.No.60).

Qty: 70500.00 Cft		@Rs.3600/-		P% Cft		          Rs.   253800/-

3	Providing Laying P.E Pressure pipes of class B Equivalent & Fxing in trenches i/c cutting fitting and testing etc complete (PHSI No-1 P- No 25)

4” Dia (110mm)

Qty: 9000.00	rft		@Rs:224/-		P. rft			Rs:	2016000/-

3” Dia (90mm)

Qty: 2750.00	rft		@Rs:148/-		P. rft			Rs:	407000/-

4.	MS Flange made out of MS Sheet

3”Dia

Qty: 2.00 Nos			@Rs.500/89		Each			Rs.	1002/-

4”Dia

Qty: 2.00 Nos			@Rs.596/75		Each			Rs.	1194/-

5.	PE Reducer 4” x 3” Dia (NSI)

Qty: 1.00 No			@Rs.1810/-		Each			Rs.	1810/-

6.	Tee Equal (4” x 4” x 4”) Dia (SMI No. 6 P-No. 14)

Qty:1.00	No		@Rs:2483/-		Each			Rs:	2483/-

7	Tee Equal (3” x 3” x 3”) Dia (SMI No. 6 P-No. 14)

Qty:2.00	Nos		@Rs:1676/-		Each			Rs:	3352/-

8.	PE Bend 90 degree 4” Dia (SMI NO  P No  13)

110 mm PN-10

Qty: 2.00	Nos		@Rs: 2969/-		Each			Rs:	4138/-

9.	P.E Bend 90 degree 3” Dia (SMI NO  P No  13)

90 mm

Qty: 2.00	Nos		@Rs: 1397/-		Each			Rs:	2794/-

10.	Bend 45 degree 4” Dia (SMI NO  P No  13)

110mm)

Qty: 2.00	Nos		@Rs: 1655/-		Each			Rs:	3310/-

3”dia 90mm

Qty: 2.00	Nos		@Rs: 1118/-		Each			Rs:	2236/-

13.	Sluice valve heavy pattern (Test pressure 21.0 kg/sq c. 300 lbs/sginth) imported (SMI No.2 P.11).

4” Dia

Qty: 1.00	No		@Rs: 5460/-		Each			Rs:	5460/-

3” Dia

Qty: 1.00	No		@Rs: 4290/-		Each			Rs:	4290/-

14	Providing chamber 2 ½ x 2 ½ (750x750 mm) inside dimension 4 ½ (1372 mm) deep as per approved design for sluice valve 3” to 12” dia with 18” (457 mm) dia inside cast iron cover and frame cwt=  (cwt 3 qe) fixed in RCC 1:2:4 (102 mm) thick brick masonry walls set in 1:6 cement mortar 6” (1152 mm) thick cement plaster 1:3 to all inside wall surfaces and to top i/c providing and fixing MS foot rest at every one foot beyond 2 ½ ft depth curing excavation back filling and disposal of surplus earth etc complete (PHSI No.1  P.  49).

Qty. 2.00 Nos			@RS: 18820/20		Each		Rs:	37640/-

15.	Butt fujion joints for special for PE pipe (SMI No. 11 P.No- 20)

4” Dia

Qty: 10.00	Nos		@Rs: 1000/-		Each			Rs:	10000

3” Dia

Qty: 10.00	Nos		@Rs: 600/-		Each			Rs:	6000/-

15.	Refilling the excavated stuff in trenches in cement in 6” thick layers i/c watering trimming on full compaction etc complete (PHSI No.24 P.No.77).

Qty: 63450.00 cft		@Rs: 2760/-		P. %o cft		Rs:	175122/-

Total:	Rs.	2937631/-NON SCHEDULE ITEM

11	Flange Adopter (NSI)

3”dia

Qty: 2.00	Nos		@Rs: 675/-		Each			Rs:	1350/-

4”dia

Qty: 2.00	Nos		@Rs: 1215/-		Each			Rs:	2430/-

Total:	Rs.	3780/-

Part – Vi 		STAFF QUARTER

1.	Excavation in foundation building bridges and other structure i/c dag  bailing dressing refilling around the structure with excavated earth watering and ramming lead up to once chain and lift up to 5’ ft. (in ordinary soil) (CSI No.18 P.No.4)

Qty: 290.62	cft		@Rs: 3176/25		P%0 cft		Rs:	923/-

2.	Cement concrete brick or stone ballast ½ to 2 gauges Ratio 1:4:8 (CSI No.6 P.No.16).

Ratio: 1:4:8

Qty: 143.30	cft		@Rs: 9416/28		P% cft			Rs:	13494/-

3. 	Pacca brick work in foundation plinth in cement sand mortar 1:6 (CSI No – 5 Page  24)

Qty: 455.30	cft		@Rs: 11948/36	P% cft			Rs:	54401/-

4	Damp proof course of cement sand and shingle concrete 1:2:4 i/c two coats of asphalted mixture (CSI No.27 P.No.23)

Qty: 81.10	cft		@Rs: 4982/18		P% cft			Rs:	4041/-

5.	Pacca brick work in ground floor i/c stricking of jointing cement sand mortar 1:6 (CSI No: 5

P-25)

Qty: 365.30	cft		@Rs: 12674/36		P% cft		Rs:	46616/-

9.	RCC work in roof slab, beams columns rafts, lintels, and other structure members laid in situ of pre cost laid in position complete in all respect (PHSI No.6 P.NO.17)

Qty: 137.87	cft		@Rs: 337/-		P. cft			Rs:	46468/-

10.	Fabrication of mild steel reinforcement for cement concrete i/c cutting bending laying in position making joints and festering i/c cost of binding wire also i/c removal of rust from bars (CSI No.7 (a) P.NO.19

Qty: 5.54 	cwt		@Rs: 5001/70		P. cwt			Rs:	27709/-

11.	Fabrication of mild steel reinforcement for cement concrete i/c cutting bending laying in position making joints and festering i/c cost of binding wire also i/c removal of rust from bars (CSI No.7 (a) P.NO.19

Qty: 4.41 cft			@Rs:4928/49		P%0 cft		Rs:	21735/-

12.	Errection & fitting  in position iron trusses fixing in position (C,S,I No.6 P-No.91)

Qty: 4.41	cwt		@Rs: 271/04		P. cwt			Rs:	1195/-

13	Supplying fixing in position iron steel gril using sold square bars of size ¾” x ¼” etc complete (CSI NO 26 – Page 97)

Qty: 24.00	sft		@Rs:180/50		P. sft			Rs:	4332/-

14.	Filling water and ramming earth in floor with surplus earth from foundation and lead upto one chain and lift upto 5’ ft (CSI No 7 page 21)

Qty: 531.84 cft		@Rs.1512/50		P%0 cft		Rs:	804/-

15.	Cement Plaster (1:6) up to 20 ft: Height ½” thick (CSI No 11.P.No. 58)

Qty: 633.24	sft		@Rs.2206/60		P% sft			Rs:	12826/-

16.	Cement Plaster (1:4) up to 12 ft: Height 3/8 thick (CSI No 11.P.No. 58)

Qty: 581.24	sft		@Rs.2197/52		P% sft			Rs:	12773/-

17.	Cement painting struck joint on walls 1:3 (CSI No – 19 Page 59) (Ratio 1.3)

Qty: 377.50	sft		@Rs.1213/58		P% sft			Rs:	4241/-

18.	Providing and laying 1” thick topping cement concrete (1:2:4) i/c surface fixing and deriding in to panels. (CSI No 16 P No 47)

3” thick

Qty: 177.08	sft		@Rs.4411/82		P% sft			Rs:	7812/-

19.	Preparing surface painting doors & windows any type i/c edge (CSI No – 5 page 76)

Qty: 104.00	sft		@Rs.2116/41		P% sft			Rs:	2201/-

20.	White washing (Three coats) CSI No – 26 Page 55)

Qty: 219.00	sft		@Rs.829/95		P% sft			Rs:	1818/-

21.	White washing (one Coat) (CSI No – 26 Page 55)

Qty: 581.24 sft		@Rs.416/63		P% sft			Rs:	2422/-

22.	Colour Washing Two Coats (CSI No – 25 Page 55)

Qty: 958.74 	sft		@Rs.859/90		P% sft			Rs:	8244/-

Total	Rs:	274395/-

PART - VI			 L.S.R

1.	Excavation in foundation building bridges and other structure i/c dag  bailing dressing refilling around the structure with excavated earth watering and ramming lead up to once chain and lift up to 	5’ ft. (in ordinary soil) (CSI No.18 P.No.5)

Qty: 140.00 Cft		@Rs. 3176/25		P%0 Cft		Rs. 	445/-

2	Cement concrete brick or stone ballast ½ to 2 gauge Ratio 1:4:8 (CSI No.6 P.No.16).

Qty: 35.00 Cft			@Rs. 9416/28		P%0 Cft		Rs. 	3296/-

3	Pacca Brick work in foundation and plinth in cement send mortar 1:4 (CSI No.4 P.No.20)

Qty: 137.79 Cft		@Rs. 11948/36		P% Cft		Rs. 	16464/-

4.	RCC work in roof slab, beams columns rafts, lintels, and other structure members laid in situ of pre cost laid in position complete in all respect (PHSI No.6 P.NO.16)

Qty: 176.21 Cft		@Rs. 337/-			P% Cft		Rs. 	59383/-

5.	Fabrication of mild steel reinforcement for cement concrete i/c cutting bending laying in position making joints and festering i/c cost of binding wire also i/c removal of rust from bars (CSI No.7 (a) P.NO.16)

Qty: 7.86 Cwt			@Rs. 5001/70			P- Cwt		Rs. 	19313/-

6.	Cement Plaster (1:6) up to 20 ft: Height ½ “thick. (CSI No 1.P.No. 58)

Qty: 490.00 Sft		@Rs. 2206/60			P% Sft		Rs. 	10812/-

7.	Cement Plaster (1:4) up to 12 ft: Height ½ “thick. (CSI No 1.P.No. 58)

Qty: 490.00 Sft		@Rs. 2197/52			P% Sft		Rs. 	10768/-

9	Providing and fixing C-I man hole cover frames of material etc complete (PHSI No.1 P-23

Qty: 1.0   Cwt			@Rs. 6985/			P-Cwt		Rs. 	6985/-

10	Supply Puddio/Dampo (M.S.L 4 P-No 16)

Qty: 55  Kg 			@Rs.3850/-			P-Kg.		Rs. 	8470/-

11.	Pointing G.I Pipe & special etc i/c fixing cutting and fitting complete with i/c the cost of cutting trenches 200 ft and handling (SMI No. 1 P- No. 12)

½ “Dia Pipe

Qty: 6.00  Rft			@Rs: 47/72			P-Rft		Rs: 	286/-

12.	Special G I Socket (SMI No 2 P- 07)

Qty: 3.00 Nos			@Rs. 18/-			Each		Rs.	54/-

13.	Special and fixing in position C P Bip cock (SSMI No 9 P-No.13)

Qty: 3.00 Nos			@Rs. 125/-			Each		Rs.	375/-

14.	Manufacturing & installing with welding MS pipe 3” Dia made out of MS sheet 3/16” thick cutting welding drilling etc complete (Based On Sch. Rate)

Qty: 25.00  Rft		@Rs: 385/89			P-Rft		Rs: 	9647/-

15.	Manufacturing & installing with welding MS flange 3” Dia made out of MS sheet 3/8” thick cutting welding drilling etc complete (Based On Sch. Rate)

Qty: 6.00  Nos			@Rs: 500/89			Each		Rs: 	3005/-

16.	CI Bend with flange (CSII No.   P.   ).

Qty: 0.26 Cwt			@Rs. 6096/-			P-Cwt		Rs. 	1585/-

17.	P/F CI ventilator (weight not less than 80 lbs each) in tank roof of approved design with water tight joint i/c painted etc complete.(SCI No. 4 page 43)

Qty: 1.00 No			@Rs: 4870/-			Each		Rs: 	4870/-

18.	Supplying and Filling sand under floor and plugging in walls (CSI No. 29 P-No.26)

Qty: 87.320  Cft		@Rs.1141/25			P% Cft		Rs.	997/-

19	Supplying reflex valve heavy pattern test pressure 21.0 kg or CM or 300 lbs sq inch. (SMI NO 4 P No  12   )

3” dia

Qty. 1.00 Nos			@Rs. 1543/75			Each		Rs. 	1544/-

Total:	Rs. 	178299/-