# general Abstract of cost

| S.No | Item Of Work | Sch: Amount | Non Sch: Amount | Total |
| --- | --- | --- | --- | --- |
| Part – I | New External Drain | Rs.1709540/- | --- | Rs.1709540/- |
| Part – II | Pump Machinery | --- | Rs.1096000 | Rs.1096000/- |
| Part – IIi | Inter Connection | Rs.160187/- | Rs.32400/- | Rs.192587/- |
| Part – IV | P.E Rising Main | Rs.504991/- | --- | Rs.504991/- |
| Part – IV | MS Chamber | Rs.37096/- | --- | Rs.37096/- |
| Part – IV | Repair of Pump House | Rs.60946/- | --- | Rs.60946/- |
| Part– V | New Transformer & Rewinding of transformer | Rs.14447/- | Rs.88400/- | Rs.102847/- |
|  | Total | Rs.2487207/- | Rs.1216800/- | Rs.3704007/- |

CertificateS

- Any error or omission in the description of items of works rates original schedule will be correct. Accordingly in the instruction of the Executive Engineer, Public Health Engineering Division-I Khairpur.

- No premium will be allowed on non-schedule items.

- The work to be carried out as per PWD/PHE specifications approval rates working estimates.

CONTRACTOR.				            	          EXECUTIVE ENGINEER

PUBLIC HEALTH ENGINEERING

DIVISION-I KHAIRPUR

SCHEDULE-B

NAME OF WORK:-	DRAINAGE / DISPOSAL WORK SCHEME THERHI TALUKA & DISTRICT KHAIRPUR

Part-I			Surface drains

1.	Excavation in foundation of building bridges and other structure i/c dag belling refilling around the structure with excavated earth watering ramming lead up to one chain and lift up to 5 ft

(SCI- No .18 P.NO.4) in Ordinary soil

Qty. 12000  cft		@Rs.3176/25		P%o cft		Rs.	38115/-

2.	Cement concrete plain i/c placing compacting finishing curing complete i/c screening and washing stone aggregate w/o shuttering (CSI No.5 (i) P.NO.17)

(a) Ratio 1:4:8

Qty. 1687.50  cft.		@Rs.11288/75 	P% cft.			   Rs.	190498/-

3	Pacca Brick work in foundation and plinth in cement sand Ratio (1;6)  (CSI No.7 P.NO.23)

Qty. 6750.0 cft		@Rs.11948/36	P% cft.			Rs.      806514/-

4.	Construction of standard of open drains cunnette block of cement concrete 1:2:4 in situ to design profile i/c cost of mould as per drawing i/c applying floating coat of cement 1/32” thick to the exposed face finished smooth curing etc complete as per detail drawing   (PHSI No.D P.NO.44)

For type – II

Qty. 2250.00 cft.		@Rs.174/00		P rft.			Rs.	391500/-

5.	Cement plaster 1:4 20’ height ½” thick (CSI No.11 P.NO.57)

Qty.9000.0 Sft			@Rs.2283/93		P% sft			Rs.	205554/-

6       RCC work in roof slab, beams columns rafts, lintels, and other structure members laid in situ of

pre cost laid in position complete in all respect 1:2:4  (CSI No.7 P.NO.17)

Qty.150.00 cft			@Rs.337/-		P cft			Rs.	50550/-

7	Fabrication of mild steel reinforcement for cement concrete i/c cutting bending laying in position making K=joints and fastering i/c cost of binding wire also i/c removal of rust from bars (CSI No.7(ii) P.NO.19).using for bars

Qty. 5.36 cwt			@Rs.5001/70		P cwt			Rs.	  26809/-

Total		Rs:	1709540/-

NON SCHEDULE ITEM

PART II			PUMPING MACHINERY 10 BHP

1	supplying installing in position i/c transportation  at site of work electric motor set 7.50 BHP (Siemens made)  4 pole 1500RPM  3 phase 400 VAC 50 Cycles coupled KSB non clogging sullage pump (type KWPZ 65-315) size 3” x2 ½ i/c steel frame and coupling with sleeve etc complete capable of discharging 290 Gallons per minute against a head of 45 ft i/c installation of pumping set in cement concrete foundation ratio 1:2:4 with MS nuts & Bolts to be embedded i/c providing and fixing automatic motor control unit (MCU) consist oil stator (MECO Pakistan) MCCB Breaker 10-A to 50-A (Shihlin Taiwan) cutout 60-A (China) ampere meter (60-A) (SELCO China) volt meter(500-V) (SELCO China) phase indication light (China) i/c transformer oil (2 Letter) (Panel Board 18’x26’x9’ & internal wiring etc complete) and providing internal electric wiring main or sub-main PVC insulated with size 3-7/0.044 (6mm2)i/c providing and fixing earthing set 0.6m in to 6.3mm 2x2x1/2 CI plate buried in the ground at depth of 3.7mm (12”)or less as per PAK PWD specification and testing of the pumping set required head of 72 hours etc complete (R.A)

Qty: 1	 Sets			@Rs. 686000/-		P. set		Rs.	686000/-

2.	Providing and installing Diesel oil Engine 16 BHP Golden made Model-1100B 2200RPM coupled with golden non clogging Horizontal semi open impeller pump type GSWG-104 size 4”x5” capable discharge 70 GPM against the head of 47 ft i/c frame pully for engine and pump with belt and installing of pump set i/c cement concrete foundation with MS nuts Bolts i/c testing the pumping machinery sets against required head for 48 hours etc complete.

Qty: 2Sets			@Rs. 205000/-		P. set		Rs.	410000/-

Total	Rs:	1096000/-

NTER CONNECTION

1.	P.E Pipe (HDPE -100) PN-10(PHSI P-26)

Qty: 20.00	rft		@Rs: 178/-			P. Rft		Rs:	3560/-

6” dia

Qty: 38.00	rft		@Rs: 530/-			P. Rft		Rs:	20140/-

2.	Manufacturing & installing Mild steel flange 4” &3” dia made out M.S Steet 3/16 thick directing hole etc complete.(based on Sch. Rate)

a)	3” Dia

Qty: 08.00 Nos.			@Rs: 500/89		Each		Rs:	4007/-

b)	6” Dia

Qty: 16.00 Nos.			@Rs: 804/77		Each		Rs:	12876

3.	M/S Pipe 3” Dia

a)	1” Dia

Qty: 12.00 rft.			@Rs: 385/89		P-Rft		Rs:	11989/-

6” Dia

Qty: 16.00	rft		@Rs: 749/34		P. Rft		Rs:	11989/-

4.	Supplying CI sluice valve heavy pattern test pressure 21.0 kg sq cm 300 lbs sq inch.

(MSI No.2 P.9)

3” Dia

Qty. 2.00 Nos.			@Rs.1543/75			Each		Rs.	3088/-

6” Dia 				.

Qty. 2.00 Nos.			@Rs.4062/50			Each		Rs.	8125/-

5.	Supplying reflex valve heavy pattern test pressure 21.0 kg or CM or 300 lbs sq inch. (SMI NO 6 P No     )

a)	6” Dia  for Suction

Qty. 4.00 Nos			@Rs. 9360			Each		Rs. 	37440/-

6.	CI Bend with flanged end with holes i/c trenches and facing of flanged for all size (SMI No.8 P.12).

a)	6” Bend 90 Degree

Qty. 1.25cwt			@Rs. 6096/-			P. cwt		Rs. 	7620/-

7.	CI tapers flat bottomed central flanged end with tapered holes i/c turning and facing of flanged of size (CMI No – 9 Page 11)

A) Reducer 6x5

B) 6x2 1/2

Qty: 0.54	cwt		@Rs:6096/-			P. cwt		Rs:	3292/-

8.	foot valve heavy pattern with one type gate (imported) (PHSI No3P.No10)

3” Dia

Qty. 2.00 Nos.			@Rs.731/25			Each		Rs.	1463/-

6” Dia

Qty. 2.00 Nos.			@Rs.1381/85			Each		Rs.	2763/-

9.	Supplying CI tees flanged ends with end with tapered holes i/c turning and facing of flanged of size (CMI No – 9 Page 11)

CC Tee

Qty: 0.451	cwt		@Rs:6096/-			P. cwt		Rs:	2749/-

10	But fusion joints

3” Dia

Qty. 12.00 Nos.		@Rs.600/-			Each		Rs.	7200/-

6” Dia

Qty. 12.00 Nos.		@Rs.1000/-			Each		Rs.	12000/-

11	Jointing CI/MS flanged pipe to special flanged and inside threaded i/c supplying rubber packing the required thickness nuts and bolts with wisher and other required for jointing and testing the joint to the specified pressure etc complete (PHSI No.1 P.35).

3” dia

Qty: 12.00	Joints		@Rs: 499/-			P. Joint	Rs:	5988/-

6” dia

Qty: 12.00	Joints		@Rs: 938/-			P. Joint	Rs:	11256/-

Total: -	Rs.	160187/-

NON SCHEDULE ITEM

1	Flange Adopter

3” Dia

Qty. 12.00 Nos.		@Rs.675/-			Each		Rs.	8100/-

4” Dia 		.

Qty. 12.00 Nos.		@Rs.2025/-			Each		Rs.	24300/-

Total: -	Rs.	32400/--

Part –IV  		P.E PIPE Line

RISING MAIN 6” DIA

1.	Dismantling and removing road mettaling (CSI No. 5 P-No.14)

Qty:225.00 Cft		@Rs.605/-		P% Cft			Rs.	1361/-

2.	Excavation for pipe line in trenches and pits soft soil i/c trimming and dressing sides to true alignment and shape leveling of beds of trenches to correct level and grade cutting joints holes and disposal of surplus earth within one chain as directed by Engineer inharge providing fence guards, lights flages and temporary crossing for non vehicular traffic where ever required lift up to 5 ft and lead upto one chain. (PH SCHI No 1 Page 60)

Qty: 7650.00 cft		@Rs:3600/-		P%o c ft		Rs:	27540/-

3.	Providing Laying & fixing in trenches i/c cutting fitting and jointing out testing in all respect the high testing polyethylene P.E pipe (PHSI No-1 P- No 25)

(160 mm)

6” Dia

Qty: 1000.00	rft		@Rs:440/-		P. rft			Rs:	440000/-

4.	PE Elbow (160mm) 90o degree 6” Dia (SMI NO B-6 P No 13 )

Qty: 2.00	Nos		@Rs:3480/-		Each			Rs:	6960/-

5.	PE Elbow 160 mm 45o degree 6” Dia (110 mm)

Qty: 2.00	Nos		@Rs:2784/-		Each			Rs:	5568/-

6.	Butt fusion joint (SMI No.H P.20).

6’ Dia

Qty: 4.00	Joints		@Rs: 1000/-		Each			Rs:	4000/-

7.	Refilling the excavated stuff in trenches in cement in 6” thick layers i/c watering trimming on full compaction etc complete (PHSI No.24 P.No.77).

Qty: 7087.50 cft		@Rs: 2760/-		P. %o cft		Rs:	19562/-

toalRs.	504991/-

part – V 		M.S CHAMBER

1.	Fabrication of heavy steel work with angles tees flat iron and sheet iron making trusses girders tanks etc i/c cutting riveting handing assembling and fixing but eluding eriction in position (CSI No.2 (a) P.NO.90)

Qty: 5.78	cwt		@Rs: 4928/49		P. cwt			Rs:	28487/-

2.	Erection and fitting in position of iron trusses staging of water tank etc complete (CSI no - 3 Page 96)

Qty: 5.78	cwt		@Rs: 271/04		P. cwt			Rs:	  1567/-

3.	Cement plaster (1:4) ½ thick upto 12 height in cement sand mortor (CSI  11 (b)  No-  P-52  )

Qty: 127.12 cft		@Rs:2283/93		P%o c ft		Rs:	2903/-

4	Pacca Brick work in foundation and plinth in cement sand Ratio (1;6)  (CSI No.7 P.NO.23)

Qty. 34.64 cft			@Rs.11948/36	P% cft.			Rs.      4139/-

Total		Rs.	37096/-

part – VI 		REPAIR OF PUMP HOUSE

1.	Cement plaster (1:4) up to 12’ height  ½ thick (CSI No.11(b) P.No.53)

Qty: 1164	sft		@Rs: 2283/93		P%sft 			Rs:	26585/-

2	Providing and laying 1” thick topping cement concrete (1:2:4) i/c surface fixing and deriding in to panels. (CSI No

Qty: 360	sft		@Rs: 4411/82		P% sft			Rs:	  15883/-

3	White washing (one Coat) (CSI No – 26 Page 60)

Qty: 1164	sft		@Rs: 416/63		P% sft			Rs:	4850-

4	Color Washing Two Coats (CSI No – 25 Page 60)

Qty: 1164. 	sft		@Rs: 859/90		P% sft			Rs:	10009/-

5.	Preparing surface painting doors & windows any type i/c edge (CSI No – 5 page 76)

Painting old surface

Qty: 312.0	sft		@Rs: 1160/06		P% sft			Rs:	3619/-

Total	Rs:	60946/-

PART – VII	NEW TRANSFORMER & REWINDING OF EXISTING TRANSFORMER

1..	Rewinding of transformer …(NSI)

H.T Coil

Qty: 3. 	Nos		@Rs: 15000/-		each			Rs:	45000/-

L.T Coil

Qty: 3. 	Nos		@Rs: 8000/-		each			Rs:	24000/-

2.	Transformer oil  (NSI)

Qty50 litter			@Rs.180/- 		P-Litter		Rs.	14400/-

3.	Labor charge (NSI)

Qty 1 Job			@Rs.5000/-		P-Job			Rs.	5000/-

Total	Rs	88400/-

4.	PVC 3 Core Cable 16mm (ES-I No1 P-15)

Qty: 60. 	rft		@Rs: 240/78	/-	P-rft			Rs:	14447/-

Total	Rs:	102847/-