# abstract of cost

| S.No | name of Work | Sch; Amount | Non Sch: Amount | Total |
| --- | --- | --- | --- | --- |
| Part – I | Tubwell | Rs.128169/- | Rs.350/- | Rs.128519/- |
| Part – II | Pump house | Rs.172294/- | --- | Rs.172294/- |
| Part – III | pumping machinery | --- | Rs.503000/- | Rs.503000/- |
| Part-- iv | Inter connection | Rs.21859/- | Rs.4050/- | Rs.25909/- |
| Part – V | P.e Rising main | Rs.2995475/- | --- | Rs.2995475/- |
| Part-- VI | P.E DISTRIBUTION SYSTEM | Rs.1767765/- | Rs.6500/- | Rs.1774265/- |
| Part – VII | Staff quorter | Rs.274395/- | --- | Rs.274395/- |
| Part – VIII | LSR | Rs.178299/- | --- | Rs.178299/- |
|  | Total: | Rs.5538256/- | Rs.513900/- | Rs.6052156/- |

CertificateS

- Any error or omission in the description of items of works rates original schedule will be correct. Accordingly in the instruction of the Executive Engineer, Public Health Engineering Division-I Khairpur.

- No premium will be allowed on non-schedule items.

- The work to be carried out as per PWD/PHE specifications approval rates working estimates.

CONTRACTOR.				              EXECUTIVE ENGINEER

PUBLIC HEALTH ENGINEERING

DIVISION-I KHAIRPUR

SCHEDULE-B

NAME OF WORK:-		WATER SUPPLY SCHEME VILLAGE HIDAYATULLAH

KATOHAR AND OTHERS U/C SHADI SHAHEED TALUKA & DISTRICT KHAIRPUR

PART – I		TUBE WELL

- Boring for tube well in all water bearing soil from ground level up ro 100” ft 30.50

meter depth i/c sinking and will drawing of crossing pipe 12 dia boring (PHSI No.1

P-36)

12” Dia

Qty: 75.00 Rft. 		@Rs.743/-		P-Rft		Rs.	55725/-

2.	Supplying and installing PVC strainer “B”  class of approved design quality and make necessary sockets etc complete for 8” Dia PVC  strainer (PHSI.NO:1 P-43)

8” Dia

Qty: 50.00Rft. 		@Rs.554/05		P-Rft		Rs.	27703/-

3.	Supplying and installing PVC Blind Pipe “B”  class of approved design quality and make necessary sockets etc complete for 8” Dia PVC  strainer (PHSI.NO:1 P-37)

8” dia

Qty: 26.00 Rft. 		@Rs516/80		P-Rft		Rs.	13436/-

4.	Surrounding with grated Bajri size 3/8 40 1/8 Ca to 3 mm in between bare and bind pipe for the  following dia meter of strainer (PHSI No:13 P-35)

12” dia

Qty: 75.00 Rft. 		@Rs.105/-		P-Rft		Rs.	7875/-/-

5.	Supplying MS bail plug (PHSI No: 8 P-43)

Qty: 1.00 No.	 		@Rs.2607/-		Each		Rs.	2607/-

6.	Supplying and fixing well 8” dia CI Dead ends (PHSI No 13 p_No.15)

Qty: 1.00 No			@Rs.1360/-		Each		Rs.	1360/-

7.	Development of Tub Well (PHSI No.   P-    )

Qty: 75.00 hour		@Rs.200/-		P-hour 		Rs.	15000/-

8.	Taking and preserving 21 lbs (Kg) samples of Estrte from bore hole (PHSI No.6 P.43)

Qty: 4.00 Sample		@Rs.101/-		P-Sample	Rs.	404/-

9	Taking Samples of water from bore hole and placing into two separate bottles

(PHSI No: 7 P-43)

Qty: 4.00 Sample 		@Rs. 131/-		P.Sample	Rs.	534/-

10       Providing strong substation traly build wooden blocked box with compartment for

preserving 2 lbs (1 KG) sample stator (PHSI No5 P-43

Qty: 1.00 No	 		@Rs. 3530/-		Each		Rs.	3538/-

Total:-	Rs.	128169/-

NON SCHEDULE ITEM

1	supplying  & fixing well 8” suspension clap (R.A)

Qty: 1.00 No			@Rs.350/-		Each		Rs.	350/-

Total:-	Rs.	350/-

Part – iI 		pump house 8 x 8)

1.	Excavation in foundation building bridges and other structure i/c dag  bailing dressing refilling around the structure with excavated earth watering and ramming lead up to once chain and lift up to 5’ ft. (in ordinary soil) (CSI No.18 P.No.4)

Qty: 147.50	cft		@Rs: 3176/25		P% cft			Rs:	468/-

2.	Cement concrete brick or stone ballast ½ to 2 gauges Ratio 1:4:8 (CSI No.6 P.No.16).

Ratio: 1:4:8

Qty: 59.61	cft		@Rs: 9416/28		P% cft			Rs:	5571/-

3. 	Pacca brick work in foundation plinth in cement sand mortar 1:6 (CSI No – 4 Page  23 )

Qty: 265.95	cft		@Rs: 11948/36	P% cft			Rs:	31777/-

4.	Damp proof course of cement sand and shingle concrete 1:2:4 i/c two coats of asphalted mixture (CSI No.28 P.No.18) 3” thick

Qty: 39.75	cft		@Rs: 4982/18		P% cft			Rs:	1980/-

8.	Pacca brick work in ground floor i/c stricken of jointing cement sand mortar 1:6 (CSI No: 5

P-24)

Qty: 218.63	cft		@Rs: 12674/36		P% cft		Rs:	27710/-

9.	RCC work in roof slab, beams columns rafts, lintels, and other structure members laid in situ of pre cost laid in position complete in all respect (PHSI No.6 P.NO.17)

Qty: 70.50	cft		@Rs: 337/-		P. cft			Rs:	23759/-

10.	Fabrication of mild steel reinforcement for cement concrete i/c cutting bending laying in position making joints and festering i/c cost of binding wire also i/c removal of rust from bars (CSI No.7 (a) P.NO.19

Qty: 2.831 	cwt		@Rs: 5001/70		P. cwt			Rs:	14160/-

11.	Filling water and ramming earth in floor with surplus earth from foundation and lead upto one chain and lift upto 5’ ft (CSI No 21 page 5)

Qty: 218.31 cft		@Rs: 1512/30		P%0 cft		Rs:	330/-

12.	Providing and laying 1” thick topping cement concrete (1:2:4) i/c surface fixing and deriding in to panels. (CSI No 16 P No 42)

Qty: 58.22	sft		@Rs: 4411/82		P% sft			Rs:	2569/-

13.	Fabrication of heavy steel work with angles tees flat iron and sheet iron making trusses girders tanks etc i/c cutting reveting handing assembliing and fixing but eluding eriction in position (CSI No.2 (a) P.NO.90)

Qty: 4.41	cwt		@Rs: 4928/49		P. cwt			Rs:	21735/-

14.	Errection & fitting  in position iron trusses fixing in position (C,S,I No.6 P-No.91)

Qty: 4.41	cwt		@Rs: 271/04		P. cwt			Rs:	1195/-

15.	Supplying fixing in position iron steel gril using sold square bars of size ¾” x ¼” etc complete (CSI NO 20 – Page 93)

Qty: 24.00	sft		@Rs: 180/50		P. sft			Rs:	4332/-

17.	Aster Plaster (1:6) up to 12 ft: Height 1/2“thick. (CSI No 11.P.No. 59)

Qty: 425.92	sft		@Rs: 2206/60		P% sft			Rs:	9398/-

18.	Cement Plaster (1:4) up to 12 ft: Height 3/8. (CSI No 11.P.No. 58)

Qty: 456.58	sft		@Rs: 2197/52		P% sft			Rs:	10033/-

19.	Cement painting struck joint on walls 1:3 (CSI No – 19 Page 58) (Ratio 1.3)

Qty: 328	sft		@Rs: 1213/58		P% sft			Rs:	3981/-

20.	White washing (one Coat) (CSI No – 26 Page 60)

Qty: 784.58	sft		@Rs: 416/63		P% sft			Rs:	3269/-

21.	White washing (Three coats) CSI No – 26 Page 59)

Qty: 130.00	sft		@Rs: 829/95		P% sft			Rs:	1079/-

22.	Colour Washing Two Coats (CSI No – 25 Page 60)

Qty: 784.58 	sft		@Rs: 859/90		P% sft			Rs:	6747/-

23.	Preparing surface painting doors & windows any type i/c edge (CSI No – 5 page 76)

Qty: 104.00	sft		@Rs: 2116/41		P% sft			Rs:	2201/-

Total	Rs:	172294/-

NON SCHEDULE ITEM

PART – III			PUMPING MACHINERY

1	supplying installing in position i/c transportation  at site of work for rural water supply scheme for village Hidayatullah change golden saer submersible pump NR-95/E-24 type 4(2) 7.50 BHP 3 phase   electric motor original sear – Italy for discharge 50 GPM at 130 ft head with local made accessories for 35 ft setting as under electric drop cable 10mm 2-3 core into 60ft water proof joint GI pipe 2-1/2 11L M grade into 5ft MS Foundation plate with T-Bolts MS holding clamp 2-2x1 socket 2-2x35 MS bore cover plate into 1 wire rope with clamp x80 feet i/c providing and fixing automatic MCU (Motor control unit consist oil stator (MEECO Pakistan) MCCB breaker 10-A to 50-A (Shihin Taiwan) cutout 60-A china Amp: meter 60-A (SELCO china) volt meter 500-V (SELCO China) phase indication light (China) include Transformer oil (2litter ) Panel Board 18”x296” and internal wiring ETC complete and providing fixing earthing arrangements and testing of the pumping set against 72 hours (R.A)

Qty: 1.00	 Sets		@Rs.503000/-			P. set		Rs.	503000/-

part – IV 		NTER CONNECTION

1.	Providing laying and fixing tranches i/c fitting jointing and testing etc complete in all respect the high density polyethylene PF pipe (HDPE-100) for W/S confirming ISO conforming ISO 4427/DIN 8074/8075 B.S 3580 & PSI 3051 (GSI -1 P-25)

PN-10 (3” dia 90 mm)

Qty: 20.00	rft		@Rs: 178/-			P. Rft		Rs:	3560/-

2.	providing  installing MS Flanged made out of MS Sheet 3/8” thick i/c cutting drilling holes etc complete

Qty: 6.00 Nos.			@Rs: 500/89			Each		Rs:	3005/-

3	Supplying reflex valve heavy pattern test pressure 21.0 kg or CM or 300 lbs sq inch. (SMI NO 4 P No  12   )

3” dia

Qty. 1.00 Nos			@Rs. 1543/75			Each		Rs. 	1544/-

4.	CI Bend with flanged end with Socket at other and for all size (PHSI (M) 1 No4 P-12) 3” dia

Qty. 0.26 cwt			@Rs. 6096/-			P. cwt		Rs. 	1585

5.	Supplying CI sluice valve heavy pattern test pressure 21.0 kg sq cm 300 lbs sq inch.

(MSI No.2 P.9)

4” Dia

Qty. 1.00 Nos			@Rs.5460/-			Each		Rs.	5460/-

7.	But Fusion joint

3” Dia

Qty. 7.00 Nos			@Rs.600/-			Each		Rs.	4200/-

8.	Jointing CI/MS flanged pipe to special flanged and inside threaded i/c supplying rubber packing the required thickness nuts and bolts with wisher and other required for jointing and testing the joint to the specified pressure etc complete (PHSI No.1 P.35).

3” dia

Qty: 5.00	Joints		@Rs: 499/-			P. Joint	Rs:	2495/-

Total: -		Rs.	21859/-

NON SCHEDULE ITEM

1.	flange adopter” dia (N.S.I)

3” Dia

Qty. 6.00 Nos			@Rs.675/-			Each		Rs.	4050/-

Total: -		Rs.	4050/-

Part –V		P.E PIPE Rising main 4” DIa

1.	Dismantling and removing road mettaling (CSI No. 5 P-No.14)

Qty:2250.00 Cft		@Rs.605/-		P% Cft			Rs.	13613/-

2.	Excavation for pipe line in trenches and pits soft soil i/c trimming and dressing sides to true alignment and shape leveling of beds of trenches to correct level and grade cutting joints holes and disposal of surplus earth within one chain as directed by Engineer inharge providing fence guards, lights flages and temporary crossing for non vehicular traffic where ever required lift up to 5 ft and lead upto one chain. (PH SCHI No 1 Page 60)

Qty: 45000.00 cft		@Rs:3600/-		P%o c ft		Rs:	162000/-

3.	Providing Laying & fixing in trenches i/c cutting fitting and jointing out testing in all respect the high testing polyethylene P.E pipe (PHSI No-1 P- No 25)

(110 mm)

6” Dia

Qty: 6000.00	rft		@Rs:6000/-		P. rft			Rs:	2640000/-

4	P.E Tee un equal 6x6x4 dia (SMI No.06 P.4)

Qty: 2.00	Nos		@Rs:3003/-		Each			Rs:	6006/-

5.	P.E Elbow  90 degree (SMI NO 1 P No 13 )

6” Dia

Qty: 2.00	Nos		@Rs:3480/-		Each			Rs:	6960/-

6.	Bend 45 degree (110 mm)

6” Dia

Qty: 2.00	Nos		@Rs:2784/-		Each			Rs:	5568/-

7.	P.E Tee 3” Dia (SMI No.10- P No.16)

3” Dia

Qty: 1.00	Nos		@Rs:1375/-		Each			Rs:	1375/-

8)	P.E Equal Tee160x16x160

Qty: 1.00	No		@Rs:4175		Each			Rs:	4175/-

8.	Butt fusion joint (SMI No.H P.20).

6’ Dia

Qty: 10.00	Joints		@Rs: 1000/-		Each			Rs:	10000/-

09.	Air valve double acting 3” x 1” (Local) (SMI No.8 P-No.12)

Qty: 1.00	Nos		@Rs: 4000/-		Each			Rs:	4000/-

10.	Providing chamber 2 ½ x 2 ½ (750x750 mm) inside dimension 4 ½ (1372 mm) deep as per approved design for sluice valve 3” to 12” dia with 18” (457 mm) dia inside cast iron cover and frame cwt=  (cwt 3 qe) fixed in RCC 1:2:4 (102 mm) thick brick masonry walls set in 1:6 cement mortar 6” (1152 mm) thick cement plaster 1:3 to all inside wall surfaces and to top i/c providing and fixing MS foot rest at every one foot beyond 2 ½ ft depth curing excavation back filling and disposal of surplus earth etc complete (PHSI No.1  P.  49).

Qty. 1.00 Nos			@RS: 18820/20		Each		Rs:	 18820/-

11	Refilling the excavated stuff in trenches in cement in 6” thick layers i/c watering trimming on full compaction etc complete (PHSI No.24 P.No.77).

Qty: 44550.00 cft		@Rs: 2760/-		P. %o cft		Rs:	122958/-

Total Rs.	2995475/-

PART – V	I  P.E  DISTRIBUTION SYSTEM 4”& 3” DIA

1.	Dismantling and removing road mettaling (CSI No. 5 P-No.14)

Qty: 1000.00 Cft		@Rs.605/-		P% Cft			Rs.	6050/-

2	Excavation for pipe lines in trenches and pits in soft soil i/c trimming and dressing sides to true alignment and shape leveling of beds of trenches to correct level and grade cutting joints holes and disposal work of surplus earth within one chain as directed by Engineer in charge providing fence guards, lights, flanges and temporary crossing for non vehicular traffic wherever required lift up to 5’ ft. (1.50m) and lead up to one chain (30.50m) (PHSI No.1 P.No.60).

Qty: 44300.00 Cft		@Rs.3600/00		P%0 Cft	      Rs.   159480/-

3	Providing Laying P.E Pressure pipes of class B Equivalent & Fxing in trenches i/c cutting fitting and testing etc complete (PHSI No-1 P- No 25)

3” Dia

Qty: 3050.00	rft		@Rs:148/-		P. rft			Rs:	451400/-

4” Dia

Qty: 4500.00	rft		@Rs:224/-		P. rft			Rs:	1008000/-

4.	Making joints CI MS specials filling i/c laying of the required along with the asbestos cement pressure pipe line to the specified pressure and making good to all leaky joints etc complete (PHSI No.1 P.80).

3” Dia

Qty: 02.00	Joint		@Rs.62/-		P. Joint		Rs:	124/-

5	Tee Equal (90mm ) (SMI No. 6 P-No. 14)

3” Dia

Qty:2.00	Nos		@Rs:1341/-		Each			Rs:	2682/-

4” Dia

Qty:2.00	Nos		@Rs:1986/-		Each			Rs:	3972/-

-

6	Elbow 90 degree (SMI NO  P No  13)

3” Dia

Qty: 2.00	Nos		@Rs: 1118		Each			Rs:	2236/-

4” Dia

Qty: 2.00	Nos		@Rs: 1655/-		Each			Rs:	3310/-

7	Bend 45 degree 3” Dia (SMI NO  P No  13)

3”dia 90mm

Qty: 2.00	Nos		@Rs: 894/-		Each			Rs:	1788/-

4”dia 90mm

Qty: 2.00	Nos		@Rs: 1324/-		Each			Rs:	2648/-

8.	Short Piece (SMI No. 9 P- No 15)

3” Dia

Qty: 2.00	Nos		@Rs: 375/-		Each			Rs:	750/-

9.	Butt fujion joints for special for PE pipe (SMI No. 11 P.No- 20)

3” Dia

Qty: 8.00	Nos		@Rs: 600/-		Each			Rs:	4800/-

4” Dia

Qty: 8.00	Nos		@Rs: 1000/-		Each			Rs:	8000/-

10.	Refilling the excavated stuff in trenches in cement in 6” thick layers i/c watering trimming on full compaction etc complete (PHSI No.24 P.No.77).

Qty: 40770.0 cft		@Rs: 2760/-		P. %o cft		Rs:	112525/-

Total:	Rs.	1767765/-

NON SCHEDULE ITEM

1.	P.E Reducer 6” x 3” Dia (NSI)

Qty: 2.00 Nos			@Rs.3250/-		Each			Rs.	6500/-

Total:	Rs.	6500/-

Part – ViI 		STAFF QUARTER

1.	Excavation in foundation building bridges and other structure i/c dag  bailing dressing refilling around the structure with excavated earth watering and ramming lead up to once chain and lift up to 5’ ft. (in ordinary soil) (CSI No.18 P.No.4)

Qty: 290.62	cft		@Rs: 3176/25		P% cft			Rs:	923/-

2.	Cement concrete brick or stone ballast ½ to 2 gauges Ratio 1:4:8 (CSI No.6 P.No.16).

Ratio: 1:4:8

Qty: 143.30	cft		@Rs: 9416/28		P% cft			Rs:	13494/-

3. 	Pacca brick work in foundation plinth in cement sand mortar 1:6 (CSI No – 5 Page  24)

Qty: 455.30	cft		@Rs: 11948/36	P% cft			Rs:	54401/-

4.	Damp proof course of cement sand and shingle concrete 1:2:4 i/c two coats of asphalted mixture (CSI No.27 P.No.23)

Qty: 81.10	cft		@Rs: 4982/18		P% cft			Rs:	4041/-

5.	Pacca brick work in ground floor i/c stricking of jointing cement sand mortar 1:6 (CSI No: 5

P-25)

Qty: 365.30	cft		@Rs: 12674/36		P% cft		Rs:	46299/-

6.	RCC work in roof slab, beams columns rafts, lintels, and other structure members laid in situ of pre cost laid in position complete in all respect (PHSI No.6 P.NO.17)

Qty: 137.89	cft		@Rs: 337/-		P. cft			Rs:	46469/-

10.	Fabrication of mild steel reinforcement for cement concrete i/c cutting bending laying in position making joints and festering i/c cost of binding wire also i/c removal of rust from bars (CSI No.7 (a) P.NO.19

Qty: 5.54 	cwt		@Rs: 5001/70		P. cwt			Rs:	27709/-

11.	Fabrication of heavy steel work with angles tees flat iron and sheet iron making trusses girders tanks etc i/c cutting revering handing assembling and fixing but eluding eriction in position (CSI No.2 (a) P.NO.90)

Qty:4.41	cwt		@Rs: 4928/49		P. cwt			Rs:	21735/-

12.	Errection & fitting  in position iron trusses fixing in position (C,S,I No.6 P-No.91)

Qty: 4.41	cwt		@Rs: 271/04		P. cwt			Rs:	1195/-

13.	Supplying fixing in position iron steel gril using sold square bars of size ¾” x ¼” etc complete (CSI NO 26 – Page 97)

Qty: 24.00	sft		@Rs:180/50		P. sft			Rs:	4332/-

14.	Filling water and ramming earth in floor with surplus earth from foundation and lead upto one chain and lift upto 5’ ft (CSI No 7 page 21)

Qty: 531.84 cft		@Rs.1512/50		P%0 cft		Rs:	804/-

15.	Cement Plaster (1:6) up to 12 ft: Height 1/2” thick (CSI No 11.P.No. 58)

Aster in side walls

Qty: 581.24	sft		@Rs.2206/60		P% sft			Rs:	12826/-

16.	Cement Plaster (1:4) up to 12 ft: Height 3/8 thick (CSI No 11.P.No. 58)

Qty: 581.24	sft		@Rs.2197/52		P% sft			Rs:	12773/-

17.	Cement painting struck joint on walls 1:3 (CSI No – 19 Page 59) (Ratio 1.3)

Qty: 377.50	sft		@Rs.1213/58		P% sft			Rs:	4581/-

18	Providing and laying 1” thick topping cement concrete (1:2:4) i/c surface fixing and deriding in to panels. (CSI No 16 P No 47)

Qty: 177.08	sft		@Rs.4411/82		P% sft			Rs:	  7812/-

19	Preparing surface painting doors & windows any type i/c edge (CSI No – 5 page 76)

Qty: 104.00	sft		@Rs.2116/41		P% sft			Rs:	2201/-

20.	White washing (Three coats) CSI No – 26 Page 55)

Qty: 219.00	sft		@Rs.829/95		P% sft			Rs:	1818/-

21.	White washing (one Coat) (CSI No – 26 Page 55)

Qty: 581.24 sft		@Rs.416/63		P% sft			Rs:	2422/-

22.	Color Washing Two Coats (CSI No – 25 Page 55)

Qty: 958.74 	sft		@Rs.859/90		P% sft			Rs:	8244/-

Total	Rs:	274395/-

PART - VIII			 L.S.R

1.	Excavation in foundation building bridges and other structure i/c dag  bailing dressing refilling around the structure with excavated earth watering and ramming lead up to once chain and lift up to 	5’ ft. (in ordinary soil) (CSI No.18 P.No.5)

Qty: 140.00 Cft		@Rs. 3176/25		P%0 Cft		Rs. 	445/-

2	Cement concrete brick or stone ballast ½ to 2 gauge Ratio 1:4:8 (CSI No.6 P.No.16).

Qty: 35.00 Cft			@Rs. 9416/28		P%0 Cft		Rs. 	3296/-

3	Pacca Brick work in foundation and plinth in cement send mortar 1:4 (CSI No.4 P.No.20)

Qty: 137.79 Cft		@Rs. 11948/36		P% Cft		Rs. 	16464/-

4.	RCC work in roof slab, beams columns rafts, lintels, and other structure members laid in situ of pre cost laid in position complete in all respect (PHSI No.6 P.NO.16)

Qty: 176.21Cft		@Rs. 337/-			P% Cft		Rs. 	59383/-

5.	Fabrication of mild steel reinforcement for cement concrete i/c cutting bending laying in position making joints and festering i/c cost of binding wire also i/c removal of rust from bars (CSI No.7 (a) P.NO.16)

Qty: 7.86 Cwt			@Rs. 5001/70			P- Cwt		Rs. 	39313/-

6.	Cement Plaster (1:6) up to 12 ft: Height 1/8 “thick. (CSI No 1.P.No. 58)

Qty: 490.00 Sft		@Rs. 2206/60			P% Sft		Rs. 	10812/-

7.	Cement Plaster (1:4) up to 12 ft: Height 3/8 “thick. (CSI No 1.P.No. 58)

Qty: 490.00 Sft		@Rs. 2197/52			P% Sft		Rs. 	10768/-

8	Providing and fixing C-I man hole cover frames of material etc complete (PHSI No.1 P-23

Qty: 1.0   Cwt			@Rs. 6985/			P-Cwt		Rs. 	6985/-

9	Supply Puddlo/Dampo (M.S.L 4 P-No 16)

Qty: 55  kg 			@Rs.3850/-			P-Kg		Rs. 	8470/-

10.	Pointing G.I Pipe & special etc i/c fixing cutting and fitting complete with i/c the cost of cutting trenches 200 ft and handling (SMI No. 1 P- No. 12)

Qty: 6.00  Rft			@Rs: 47/72			P-Rft		Rs: 	286/-

11.	Special G I Socket (SMI No 2 P- 07)

Qty: 3.00 Nos			@Rs. 18/-			Each		Rs.	54/-

13.	Special and fixing in position C P Bip cock (SSMI No 9 P-No.13)

Qty: 3.00 Nos			@Rs. 125/-			Each		Rs.	375/-

14.	Manufacturing & installing with welding MS flange 3” Dia made out of MS sheet 3/16” thick cutting welding drilling etc complete (Based on schedule rate)

Qty: 25.00  Rft		@Rs: 385/89			P-Rft		Rs: 	9647/-

15.	Manufacturing & installing with welding MS flange 3” Dia made out of MS sheet 3/8” thick cutting welding drilling etc complete

Qty: 6.00  Nos			@Rs: 500/89			Each		Rs: 	3005/-

16.	CI Bend with flange (CSII No.   P.   ).

3” Dia

Qty: 0.26 Cwt			@Rs. 6096/-			P-Cwt		Rs. 	1585/-

17.	P/F CI ventilator (weight not less than 80 lbs each) in tank roof of approved design with water tight joint i/c painted etc complete.(SCI No. 4 page 43)

Qty: 1.00 No			@Rs: 4870/-			Each		Rs: 	4870/-

18	Supplying and Filling sand under floor and plugging in walls (CSI No. 29 P-No.26)

Qty: 87.32  Cft		@Rs.1141/25			P% Cft		Rs.	997/-

19	Reflex valve heavy pattern (SMI No.1 P.11).

3” Dia

Qty: 1.00	No		@Rs: 1543/75			Each		Rs:	1544/-

Total:	Rs. 	178299/-