SCHEDULE-B

NAME OF WORK:-		 REHABILITATION OF DRAINAGE SCHEME

PIR MANGIO U/C PIR MANGIO  TALUKA & DISTRICT KHAIRPUR

PART I			PUMPING MACHINERY 10 BHP

1	supplying installing in position i/c transportation  at site of work electric Pumping  set 7.50 BHP (Siemens made)  4 pole 1500RPM  3 phase 400 VAC 50 Cycles coupled KSB non clogging sullage pump (type KWPZ 65-315) size 3” x2 ½ i/c steel frame and coupling with sleeve etc complete capable of discharging 160 Gallons per minute against a head of 45 ft i/c installation of pumping set in cement concrete foundation ratio 1:2:4 with MS nuts & Bolts to be embedded i/c providing and fixing automatic motor control unit (MCU) consist oil stator (MECO Pakistan) MCCB Breaker 10-A to 50-A (Shihlin Taiwan) cutout 60-A (China) ampere meter (60-A) (SELCO China) volt meter(500-V) (SELCO China) phase indication light (China) i/c transformer oil (2 Letter) (Panel Board 18’x26’x9’ & internal wiring etc complete) and providing internal electric wiring main or sub-main PVC insulated with size 3-7/0.044 (6mm2)i/c providing and fixing earthing set 0.6m in to 6.3mm 2x2x1/2 CI plate buried in the ground at depth of 3.7mm (12”)or less as per PAK PWD specification and testing of the pumping set required head of 72 hours etc complete (R.A)

Qty: 1	 Sets			@Rs. 703000/-		P. set		Rs.	703000/-

2.	Supplying and installing Diesel Oil Enginei/c transportation  to site drainage scheme Pir Mangio 16 BHP 220 RPM (KSB company) standard accession Diesel Engine oil with Golden Pump    non Clogging pump type 65 – 315, size 3x 2 ½” capable of going Diesel 70 Gallons per minute a head of 45 mounted from are based plate installing of pump set cement concrete foundation (Ratio 1:2:4) with CC stone 55/8 to be embedded  in cc foundation plinth of the pumping set for 72 hours under actual condition etc complete.

Qty: 1 Set			@Rs: 205000/-		P. Set		Rs:	205000/-

Total	Rs:	908000/-INTER CONNECTION

1.	P.E Pipe (HDPE -100) PN-10

3” dia

Qty: 20.00	rft		@Rs: 178/-			P. Rft		Rs:	3560/-

4” dia

Qty: 27.00	rft		@Rs: 530/-			P. Rft		Rs:	14310/-

3.	M/S Pipe 3” Dia

a)	1” Dia

Qty: 6.00 Nos.				@Rs: 385/89		Each		Rs:	2315/-

a)	6” Dia

Qty: 8.00 Nos.			@Rs: 749/34			Each		Rs:	5995/-

4.	Supplying CI Reflex valve heavy pattern test pressure 21.0 kg sq cm 300 lbs sq inch.

(MSI No.2 P.9)

3” Dia

Qty. 1.00 Nos.			@Rs.1543/75			Each		Rs.	1544/-

6” Dia

Qty. 3.00 Nos.			@Rs.4062/-			Each		Rs.	12188/-

5.	Supplying sluice  valve heavy pattern test pressure 21.0 kg or CM or 300 sq inch. (SMI NO 6 P No     )

a)	6” Dia

Qty. 2.00 Nos			@Rs. 9360/-			Each		Rs. 	18720/-

6.	CI Bend with flanged end with holes i/c trenches and facing of flanged for all size (SMI No.8 P.12).

a)	6” Bend 90 Degree

Qty. 0.80cwt			@Rs. 6096/-			P. cwt		Rs. 	4877/-

7.	CI tapers flat bottomed central flanged end with tapered holes i/c turning and facing of flanged of size (CMI No – 9 Page 11)

Qty: 042	cwt		@Rs:6096/-			P. cwt		Rs:	2560-

8.	foot valve heavy pattern with one type gate (imported) (PHSI No3P.No10)

3” Dia

Qty. 1.00 Nos.			@Rs.731/25			Each		Rs.	731/-

6” Dia

Qty. 1.00 Nos.			@Rs.1381/25			Each		Rs.	1381/-

9.	Supplying CI tees flanged ends with end with tapered holes i/c turning and facing of flanged of size (CMI No – 9 Page 11)

CC Tee 6”

Qty: 0.22	cwt		@Rs:6096/-			P. cwt		Rs:	1341/-

10	But fusion joints

3” Dia

Qty. 06.00 Nos.		@Rs.600/-			Each		Rs.	3600/-

6” Dia

Qty. 12.00 Nos.		@Rs.1000/-			Each		Rs.	12000/-

11	Jointing CI/MS flanged pipe to special flanged and inside threaded i/c supplying rubber packing the required thickness nuts and bolts with wisher and other required for jointing and testing the joint to the specified pressure etc complete (PHSI No.1 P.35).

3” dia

Qty: 06.00	Joints		@Rs: 499/-			P. Joint	Rs:	2994/-

6” dia

Qty: 12.00	Joints		@Rs: 938/-			P. Joint	Rs:	11256/-

Total: -	Rs.	99372/-

NON SCHEDULE ITEM

1	Manufacturing & installing Mild steel flange 4” &3” dia made out M.S Steet 3/16 thick directing hole etc complete. (RA)

a)	3” Dia

Qty: 4.00 Nos.			@Rs: 500/89			Each		Rs:	2004/-

b)	6” Dia

Qty: 12.00 Nos.		@Rs: 804/77		`	Each		Rs:	9657/-

2	Flange Adopter

3” Dia

Qty. 6.00 Nos.			@Rs.675/-			Each		Rs.	4050/-

4” Dia

Qty. 12.00 Nos.		@Rs.2025/-			Each		Rs.	24300/-

Total: -	Rs.	40011/-

Part – II	REPAIR Of STAFF QUARTER

1.	Dismantling CC reinforcement separating from concrete cleaning straitening the same (CSI No.20 P-No11)

Qt.107.72 cft.			@Rs.5445/-			P% cft.		Rs.	5865/-

2.	Pacca brick work in ground floor i/c stricking of jointing cement sand mortar 1:6 (CSI No: 5

P-25)

=	91.87

Qty: 89.62	cft		@Rs:12674/36	P% cft			Rs:	11359/-

3.	Supplying Girder cross 1st Quality (SMI No.140PNo-6)

Qty: 4.07 	cwt		@Rs: 3850/-		P. cwt			Rs:	15670/-

4.	Supplying angle iron T-Iron strips grating CI mould item & rools  (SMI No.140PNo-6)

Qty: 3.79 	cwt		@Rs: 3575/-		P. cwt			Rs:	13549/-

5.	2nd class tiles roofing consist on 4” earth and 1” mud plaster with gobri leaping over 1/2” cement plaster 1:6 with 34 lbs of hot bitumen coating sand blinded provide over one layer of 12”x6”x2” tiles laid in 1:6 cement plaster i/c 1:2 (CSI No.1P No-32).

Qty: 256.50 	Sft		@Rs: 7607/25		P. %Sft		Rs:	19513/-

6.	Cement Plaster (1:6) up to 12 ft: Height ½ “thick. (CSI No 1.P.No. 58)

Qty: 728.00	sft		@Rs: 2206/60		P% sft			Rs:	16064/-

7	Cement Plaster (1:4) up to 12 ft: Height 3/8”thick. (CSI No 1.P.No. 58)

Qty: 1260.00	sft		@Rs: 2197/52		P% sft			Rs:	27689/-

8	Fabrication of heavy steel work with angles tees flat iron and sheet iron making trusses girders tanks etc i/c cutting revering handing assembling and fixing but eluding eriction in position (CSI No.2 (a) P.NO.90)

Qty: 4.41	cwt		@Rs: 4928/49		P. cwt			Rs:	21735/-

9.	Errection & fitting  in position iron trusses fixing in position (C,S,I No.6 P-No.91)

Qty: 4.41	cwt		@Rs: 271/04		P. cwt			Rs:	1195/-

09	Providing and laying 1” thick topping cement concrete (1:2:4) i/c surface fixing and deriding in to panels. (CSI No 16 P No 47)

Qty: 192.00	sft		@Rs.4411/82		P% sft			Rs:	  8471/-

Pointing New surface

09.	Preparing surface painting doors & windows any type i/c edge (CSI No – 5 page 77)

Qty: 104.0	sft		@Rs: 1489/68		P% sft			Rs:	1549/-

Total Rs: 	142659/-

part – III 		M.S SCREEN FOR CHAMBER

1.	Small iron work as guest in best plate knees bends stops etc i/c cutting drilling riveting handling assembling and fixing excluded erection and in position covers  (CSI No 1.P.No. 58)

Qty: 3.57	Cwt		@Rs: 6420/61		P-cwt			Rs:	22922/-

2.	Erection and fitting in position of iron trusses staging of water tank etc complete (CSI no - 3 Page 96)

Qty: 3.57	cwt		@Rs: 271/04		P. cwt			Rs:	  968/-

Total		Rs.	23890/-

Part-IV 	C.I Main Holes (Covers)

1.	Manufacturing & Supplying of 24” Main Hole (Schch No-1 P-31)

Qt.20 Cwt		@Rs.1051/-			P-Cwt			Rs. 	21025/-

Total:	Rs.	21025/-

# abstract of cost

| S.No | name of Work | Sch; Amount | Non Sch: Amount | Total |
| --- | --- | --- | --- | --- |
| Part – I | pumping machinery | -- | Rs.908000/- | Rs.908000/- |
| Part – I | INTER CONNECTION | Rs. 99372/- | Rs.40011/- | Rs.139383/- |
| Part –III | REPAIR Staff quorter | Rs.142659/- | --- | Rs.142659/- |
| Part – IV | MS Screen for chamber | Rs.23890/- | --- | Rs.23890/- |
| Part—V | RCC MAIN HOLE | Rs.21025/- | --- | Rs.21025/- |
|  | Total: | Rs.286946/- | Rs.948011/- | Rs.1234957/- |

CertificateS

- Any error or omission in the description of items of works rates original schedule will be correct. Accordingly in the instruction of the Executive Engineer, Public Health Engineering Division-I Khairpur.

- No premium will be allowed on non-schedule items.

- The work to be carried out as per PWD/PHE specifications approval rates working estimates.

CONTRACTOR.				              EXECUTIVE ENGINEER

PUBLIC HEALTH ENGINEERING

DIVISION-I KHAIRPUR