BILL OF QUANTITIES

NAME OF WORK:-	Providing Installing 10 BHP Electric Submersible pumping set at Repair and 			Maintenance of Water Supply Scheme  Thana Ahmed Khan  (Taluka Thana 			Bula Khan)  (District Jamshoro)

(A) 	Description and rate of item based on Composite schedule of Rates

| Item No: | Quantity | Description of Item to be executed at site | Rate | Unit | Amount |
| --- | --- | --- | --- | --- | --- |
| 01 | 02 | 03 | 04 | 05 | 06 |
| 01 | 1 set | Providing and installing in position i/c transportation site of work at Thana Ahmed Khan Grund Fos HMA Submersible pump type HSP BA- 25+10 HP Motor complete capable to discharge 40 G.P.M against a head of 350’ft 10 HP HMA Motor (3x400Vx2 Pole ) i/c G.I. Riser pipe of 2½ dia x 30 Nos: of 10’ft each with flanged ends. The flanges are notched to accommodate the cable i/c top set of 2½” dia containing bore hole cover plate with 90 bend slice valve jacket nuts and bolts and 2 sets of installation claps (G.I.) i/c nipple of 2½” dia (G.I.) i/c 10 HP Motor control unit consist of metallic box circuit breaker, ammeter, am-meter selector switch volt meter, volt meter selectror switch electronic over current relay, magnetic contractor under / Over voltage relay (EVR) dry running protection relay indication lamps & On/Off switch Electrodes and cable 1 x 1.50 mm2 x 100 m drop cable 3 x 4 mm2 x 100 meter cable clams 30 Nos: cable termination Kit (TK) x 0.1 No: and making necessary arrangement for testing of the complete pumping set against the reqired head for 72 hours etc: complete. | 1046000/- | P/set | 10,46,000/- |
| 02 |  | Providing G.I. Pipe of Heavy quality (R.A.A.Based on schedule) |  |  |  |
|  | 30.0 Rft | 3” dia | 304/02 | P/Rft | 9,121/- |
| 03 |  | Manufacturing supplying M.S. flange of different dia made out Of M.S. Steel of 3/8” thick i/c cutting drilling holes and welding Of flange with M.S pipe etc: (R.A.A. Based on schedule) |  |  |  |
|  | 6 Nos: | 3” dia | 500/89 | P/Rft | 3,005/- |
| 04 | 50.00 Kg: | C.I. Tapper flate bottomed or central tapered flanged ends with holes Including turning and facing turning and facing of flanges for all size (P.H.S.I.M.I.NO;9, P/13) | 120/- | P/Kg: | 6,000/- |
| 05 | 1 No: | Reflex valve heavy pattern test pressure 300 lbs/sq; inch (P.H.S.(M)I-6, P/12) | 1543/75 | Each | 1,544/- |

(Cont: on page No:02)

(Page No:02)

| Item No: | Quantity | Description of Item to be executed at site | Rate | Unit | Amount |
| --- | --- | --- | --- | --- | --- |
| 01 | 02 | 03 | 04 | 05 | 06 |
| 06 |  | Jointing C.I/M.S flanged pipes and specials flanged and inside a trenches i/c supplying rubber packing of the required thickness nuts bolts with washers etc and other tools required for jointing and testing the joints to the specified pressure etc: complete.(P.H.S.I.NO;1, P/40) |  |  |  |
|  | 9 Joints | 3” dia | 499/- | P/Joint | 4,999/- |
|  |  | Cost for 3 sets 3 x 1071804 = Rs:32,15,412/- |  | Total | 10,71,804/- |

Amount Total (a) + (b) Rs:___________________

________% above / below on the rates of CSR, Amount to be added / deducted on the basis

Total (b) of premium quoted ______________

Total (A) = a + b in works & Figures______________________________________________

Contractor 						 	EXECUTIVE ENGINEER

PUBLIC HEALTH ENGG:DIVISION

JAMSHORO

BILL OF QUANTITIES

NAME OF WORK:-	Providing Installing 10 BHP Pumping Machinery of Tube Well for Repair and 			Maintenance of Water Supply Scheme  Thana Ahmed Khan  (Taluka Thana 			Bula Khan)  (District Jamshoro)

(A) 	Description and rate of item based on Composite schedule of Rates

| Item No: | Quantity | Description of Item to be executed at site | Rate | Unit | Amount |
| --- | --- | --- | --- | --- | --- |
| 01 | 02 | 03 | 04 | 05 | 06 |
| 01 | 1 set | Providing and installing in position i/c transportation site of work at Thana Ahmed Khan Grund Fos HMA Submersible pump type HSP BA- 25+10 HP Motor complete capable to discharge 40 G.P.M against a head of 350’ft 10 HP HMA Motor (3x400Vx2 Pole ) i/c G.I. Riser pipe of 2½ dia x 30 Nos: of 10’ft each with flanged ends. The flanges are notched to accommodate the cable i/c top set of 2½” dia containing bore hole cover plate with 90 bend slice valve jacket nuts and bolts and 2 sets of installation claps (G.I.) i/c nipple of 2½” dia (G.I.) i/c 10 HP Motor control unit consist of metallic box circuit breaker, ammeter, am-meter selector switch volt meter, volt meter selectror switch electronic over current relay, magnetic contractor under / Over voltage relay (EVR) dry running protection relay indication lamps & On/Off switch Electrodes and cable 1 x 1.50 mm2 x 100 m drop cable 3 x 4 mm2 x 100 meter cable clams 30 Nos: cable termination Kit (TK) x 0.1 No: and making necessary arrangement for testing of the complete pumping set against the reqired head for 72 hours etc: complete. |  | P/set |  |
| 02 |  | Providing G.I. Pipe of Heavy quality (R.A.A.Based on schedule) |  |  |  |
|  | 30.0 Rft | 3” dia |  | P/Rft |  |
| 03 |  | Manufacturing supplying M.S. flange of different dia made out Of M.S. Steel of 3/8” thick i/c cutting drilling holes and welding Of flange with M.S pipe etc: (R.A.A. Based on schedule) |  |  |  |
|  | 6 Nos: | 3” dia |  | P/Rft |  |
| 04 | 18.61 Kg: | C.I. Tapper flate bottomed or central tapered flanged ends with holes Including turning and facing turning and facing of flanges for all size (P.H.S.I.M.I.NO;9, P/13) |  | P/Kg: |  |
| 05 | 1 No: | Reflex valve heavy pattern test pressure 300 lbs/sq; inch (P.H.S.(M)I-6, P/12) |  | Each |  |

(Cont: on page No:02)

(Page No:02)

| Item No: | Quantity | Description of Item to be executed at site | Rate | Unit | Amount |
| --- | --- | --- | --- | --- | --- |
| 01 | 02 | 03 | 04 | 05 | 06 |
| 06 |  | Jointing C.I/M.S flanged pipes and specials flanged and inside a trenches i/c supplying rubber packing of the required thickness nuts bolts with washers etc and other tools required for jointing and testing the joints to the specified pressure etc: complete.(P.H.S.I.NO;1, P/40) |  |  |  |
|  | 9 Joints | 3” dia |  | P/Joint |  |
|  |  |  |  | Total |  |

Amount Total (a) + (b) Rs:___________________

________% above / below on the rates of CSR, Amount to be added / deducted on the basis

Total (b) of premium quoted ______________

Total (A) = a + b in works & Figures______________________________________________

Contractor 						 	EXECUTIVE ENGINEER

PUBLIC HEALTH ENGG:DIVISION

JAMSHORO