BILL OF QUANTITIES

NAME OF WORK:-	Providing Laying Jointing and testing PVC Pipe for Rising Main Water 				Supply Scheme Khanote (Phase-I ) (Taluka Manjhand) District Jamshoro.

(A) 	Description and rate of item based on Composite schedule of Rates

| Item No: | Quantity | Description of Item to be executed at site | Rate | Unit | Amount |
| --- | --- | --- | --- | --- | --- |
| 01 | 02 | 03 | 04 | 05 | 06 |
| 01 | 100851.75 Cft | Excavation for pipe lines into trenches and pits in Soft soils i/c trimming and dressing sides to true alignment and shape leveling of beds of trenches to correct level and grade cutting joint holes and disposal of surplus earth within a one chain as directed by Engineer incharge providing fence guards lights flags and temporary crossing for non vehicular traffic where ever required lift upto 5’ft(1.52m) and lead upto one chain (30.5mm) (P.H.S.I.NO; 1, P/60) | 3600/- | P%0cft | 363,866/- |
| 02 | 39375.00 Cft | Excavation for pipe lines into trenches and pits in all kind of soil i/c trimming and dressing sides to true alignment and shape leveling of beds of trenches to correct level and grade cutting joint holes and disposal of surplus earth within a one chain as directed by Engineer incharge providing fence guards lights flags and temporary crossing for non vehicular traffic where ever required lift upto 5’ft(1.52m) and lead upto one chain (30.5mm) (P.H.S.I.NO; 3, P/61) | 4650/- | P%0cft | 183,894/- |
| 03 |  | Providing PVC pipe of class ‘B’ (equivalent make)fixing in trenches i/c cutting and jointing with “Z” joints with one rubber rings i/c testing with water to head of 61 meter or 200’ft. (P.H.S.I.NO;1,P/21) |  |  |  |
|  | 19941.0 Rft | 6” dia | 262/- | P/Rft | 52,24,542/- |
| 04 |  | Supplying of G.I. Pipe approved quality to be tested with pressure 200’ft (P.H.S.I.NO;1, P/ ) |  |  |  |
|  | 150.0 Rft | 6” dia | 695/04 | P/Rft | 104,256/- |
| 05 |  | Manufacturing & fixing M.S Flange made of M.S. Sheet ½” thickness making holes, facing welding, nuts bolts, rubber packing (Rate approved (Based on sch: ) |  |  |  |
|  | 16 Nos: | 6” dia | 804/77 | Each | 12,876/- |
| 06 |  | Laying C.I/M.S pipe (S&S) flanged and specials and fixing in Trenches i/c cutting fitting and testing with water to specified Pressure (P.H.S.I.M.NO;B, P/54) |  |  |  |
|  | 150.0 Rft | 6” dia | 10/68 | P/Rft | 1,602/- |
| 07 | 26.72 Kg: | C.I. bends with flanged ends with holes including tuning and facing of flanges for all sizes (P.H.S.M. I.NO.8 P-98) | 120/- | P/Kg: | 3,206/- |
| 08 | 39.90 Kg: | C.I TEE Flange ends with holes i/c curing and facing of Flanged for all size (sch of (M) 11/P/11) | 120/- | P/kg: | 4,788/- |
| 09 |  | PVC ‘Z’ Joints fitting (P.H.S(M)I-16, P/28) |  |  |  |
|  | 6 Nos: | PVC Bend 6” dia 22½,45, 90 | 1913/- | Each | 11,478/- |

(Cont: on page No:02)

(Page No:02)

| Item No: | Quantity | Description of Item to be executed at site | Rate | Unit | Amount |
| --- | --- | --- | --- | --- | --- |
| 01 | 02 | 03 | 04 | 05 | 06 |
| 10 |  | PVC ‘Z’ Joints fitting (P.H.S(M)I-16, P/28) | 568/75 | Each | 1,138/- |
|  | 2 Nos: | TEE EQUAL 6”x6”x6” | 1800/- | Each | 3,600/- |
| 11 |  | C.I.Sluice valve heavy (Test pressure 21.0 Kg/sq:cm or 300 lbs/sq: inch (Imported)(M.S.I.NO;2(d)P/97) |  |  |  |
|  | 2 Nos | 6” dia | 9360/- | Each | 18,720/- |
| 12 |  | C.I. Tail Piece for PVC Pipe (P.H.S(M)I-1,P/97) |  |  |  |
|  | 4 Nos: | 6” dia | 618/75 | Each | 2,475/- |
| 13 |  | C.I Jibault joint (P.H.S.M.NO;14, P/103) |  |  |  |
|  | 4 Nos: | 6” dia | 1062/50 | Each | 4,250/- |
| 14 |  | Air valve double acting (P.H.S.I.NO:8, P/98) |  |  |  |
|  | 7 Nos: | 6” dia | 4000/- | Each | 28,000/- |
| 15 |  | Fire hydrant tee (P.H.S.I.NO;8, P/98) |  |  |  |
|  | 7 Nos: | 6” dia | 1375/- | Each | 9,625/- |
| 16 | 9 Nos: | Providing chamber 3”x2” (915x615mm) inside dimension 4½”(1372 mm) deep as per approved design for sluice valve 3” to 12” dia with 18” (457mm) dia inside cast iron cover and frame (wt= 1 cwt 3 qr) fixed in RCC 1:2:4 (102 mm) thick with 5 lbs steel per cft)9” (299 mm) thick brick masonary wall set in 1:6 cement mortar 6” (1152mm ) thick cement concrete 1:3:6 in foundation 1” thick cement concrete 1:2:4 flooring ½” (12.5mm) thick cement plaster 1:3 to all inside wall surface and to top i/c providing and fixing M.S. foot beyond 2½” depth curring excavation back filling and disposal of surplus earth etc; complete (P.H.S.I.NO:1, P/49) | 18820/- | Each | 131,740/- |
| 17 |  | Making joints to PVC. specials fitting (i/c laying specials and cost of sovent cement of required diameter and testing the joint along with the asbestos cement pressure pipe line to the specified pressure and making good to all leaky joints etc: complete.(P.H.S.I.NO:1, P/56) |  |  |  |
|  | 15 Nos: | 6” dia | 70/- | Each | 1,050/- |
| 18 |  | Jointing C.I/M.S flanged pipes and specials flanged and inside a trenches i/c supplying rubber packing of the required thickness nuts bolts with washers etc and other tools required for jointing and testing the joints to the specified pressure etc: complete.(P.H.S.I.NO;1, P/35) |  |  |  |
|  | 16 Nos: | 6” dia | 938/- | P/Joint | 15,008/- |
| 19 | 126204.0 Cft | Refilling the excavated stuff in trenches 6” thick layers i/c watering ramming to full compaction etc: complete (P.H.S.I.NO;24, P/53) | 2760/- | P%0cft | 348,323/- |
|  |  |  |  | Total | 64,73,299/- |

(Cont: on page No:03)

(Page no:03)

Amount Total (a) + (b) Rs:___________________

________% above / below on the rates of CSR, Amount to be added / deducted on the basis

Total (b) of premium quoted ______________

Total (A) = a + b in works & Figures______________________________________________

Contractor 						 	EXECUTIVE ENGINEER

PUBLIC HEALTH ENGG:DIVISION

JAMSHORO