BILL OF QUANTITIES

NAME OF WORK:-	Rehabilitation & Maintenance Of Drainage Scheme  Thana Ahmed Khan  				(Taluka Thana Bula Khan)  (District Jamshoro).

(A) 	Description and rate of item based on Composite schedule of Rates

| Item No: | Quantity | Description of Item to be executed at site | Rate | Unit | Amount |
| --- | --- | --- | --- | --- | --- |
| 01 | 02 | 03 | 04 | 05 | 06 |
|  |  | GROUP ‘A’ CONSTRUCTION OPEN SURFACE DRAINS TYPE ‘A’ TYPE ‘B’ & TYPE ‘C’ |  |  |  |
| 01 | 3380.0 Cft | Dismantling cement concrete plain (1:4:8)(G.S.INO;19( a)P/10) | 1663/75 | P%cft | 55,234/- |
| 02 | 1690.0 Cft | Dismantling Cement concrete plain (1:2:4) (G.S.I.NO;19( c)P/11) | 3327/50 | P%cft | 56,235/- |
| 03 | 1744.50 Cft | Dismantling and removing road (G.S.I.NO; 51,P/13) | 605/- | P%cft | 10,554/- |
| 04 | 40890.0 Cft | Excavation in foundation of building bridges and other structures including dabbling dressing refilling around structures with excavated earth watering and ramming lead upto 5’ft (in hard soil ) (G.S.I.NO;18,. P/4) | 3554/38 | P%0cft | 145,339/- |
| 05 | 7688.0 Cft | Cement concrete plain i/c placing compacting finishing and curring complete(i/c screening and washing of stone aggregate without shuttering (Ratio 1:4:8) (G.S.I.NO;5Ip/18) | 11288/75 | P%cft | 867,879/- |
| 05 |  | Construction of standard open drains cunette block of cement concrete 1:2:4 in situe to the design profile i/c cost of mould as per drawing i/c applying floating coat of cement 1/32” thick to the exposed face finished smooth curring etc: complete as per detailed drawing (P.H.S.I.NO;1, P/58) |  |  |  |
|  | 4500.0 Rft | Type ‘A’ | 94/- | P/Rft | 423,000/- |
|  | 2000.0 Rft | Type ‘B’ | 174/- | P/Rft | 348,000/- |
|  | 3000.0 Rft | Type ‘C’ | 309/- | P/Rft | 927,000/- |
| 06 | 20250.0 Cft | Pacca brick work in foundation and plinth in cement sand mortar (Ratio 1:6) (G.S.INO;4 ( c) P/20) | 11948/36 | P%cft | 24,19,541/- |
| 07 | 30600.00 Sft | Cement plaster (1:4) upto 12’ft height thick ½” thick (G.S.I.NO;11( b) P/51) | 2283/93 | P%sft | 698,883/- |
| 08 | 1820.65 Cft | Reinforced cement concrete work including all labour and material except the cost of steel reinforcement and its labour for bending and binding, which will be paid separately. This rate also includes all kinds of forms moulds, lifting, shuttering currying rendering and finishing the exposed surface)\ (including screening and washing of shingle)(a) R.C.C. Work in roof slab, beams, columns, rafts, lintels and other structural members laid in suite or precuts laid in position complete in all respect (i) Ratio 1:2:4) 90 lbs cement 2 cft sand 4 cft shingle 1/8” to ¼” gauge.(G.S.I.NO;6(a) (i)P/19) | 337/- | P/Cft | 613,559/- |

(Cont: on page No:02)

(Page No:02)

| Item No: | Quantity | Description of Item to be executed at site | Rate | Unit | Amount |
| --- | --- | --- | --- | --- | --- |
| 01 | 02 | 03 | 04 | 05 | 06 |
| 09 | 65.02 Cwt | Fabrication of mild steel reinforcement for cement concrete including cutting bending laying in position making joints and fastenings including cost of binding wire (also includes removal of rust from bars (G.S.I.NO;8(ii)(a)P/17) | 5001/70 | P/Cwt | 325,211/- |
|  |  |  |  | Total | 76,94,462/- |
|  |  | GROUP ‘B’ C.C. BLOCK |  |  |  |
| 01 | 5610.0 Cft | Barrow pit excavation undressed lead upto 100’ft(in ordinary soil)(G.S.I.NO;3(a)P/1) | 2117/50 | P%0cft | 11,879/- |
| 02 | 46500.0 Cft | Laying earth in 6” layers leveling dressing and watering for compaction Etc: complete (G.S.I.NO;13( b) P/3) | 354/- | P%0cft | 16,461/- |
| 03 | 20460.0 Cft | Cement concrete brick or stone ballast 1½” to 2” guage(Ratio 1:4:8) (G.S.I.NO;4(b)P/17) | 9416/28 | P%cft | 19,26,570/- |
| 04 | 15500.0 Cft | Cement concrete plain i/c placing compacting finishing and curring complete(i/c screening and washing of stone aggregate without shuttering(Ratio 1:2:4)(G.S.I.NO;5(a)p/18) | 14429/25 | P%cft | 22,36,534/- |
| 05 | 2969.50 Sft | Errection and removal of centering for R.C.C or plain cement concrete works of deodar wood)2nd class)(for partal wood)(Vertical)(G.S.I.NO;19, P/21) | 3127/41 | P%sft | 92,848/- |
| 06 | 5610.0 Cft | Extra for every 50’ ft additional or part there off (a) for earth work (soft ordinary hard and very hard)(G.S.I.NO:P, P/3) | 5039/- | P%0cft | 28,269/- |
|  |  |  |  | Total | 43,12,581/- |
|  |  | GROUP ‘C’ CLEANING OF SEWER LINE I/C SILT CLEARANCE OF OLD SURFACE DRAINS |  |  |  |
| 01 | 6171.32 Cft | Cleaning of under ground sewer line by Mechinical or manual metled (P.H.S.I.NO;2, P/59) | 56/30 | P/Cft | 347,445/- |
| 02 | 6171.32 Cft | Rehandling of earth Lead upto single throw of kassi phawrah or a shawel (G.S.I.NO; 9, P/2) | 756/25 | P%0cft | 4,667/- |
| 03 | 6171.32 Cft | Earth work for slush or daldal i/c dewaring (G.S.I.NO;16, P/1) | 2420/- | P%0cft | 14,935/- |
| 01 | 5980.0 Cft | Silt clearance of surface drains Earth work excavating undressed lead upto a single thro of kassi phawrah or shavel (G.S.I.NO;190 , P/.1) | 1134/38 | P%0cft | 6,784/- |
| 02 | 5980.0 Cft | Earth work for slush or daldal i/c dewatering (G.S.INO;1, P/61) | 2420/- | P%0cft | 14,472/- |
| 03 | 5980.0 Cft | Rehandling of earth Lead upto single throw of kassi phawrah or a shawel (G.S.I.NO; 9, P/2) | 756/25 | P%0cft | 4,522/- |
|  |  |  |  | Total | 392,825/- |

(Cont: on page No:03)

(Page No:03)

A   B  S   T   R   A   C   T

| Group Name | Schedule Item | Non Schedule | Total |
| --- | --- | --- | --- |
| Group ‘A’ Constt: Open surface Type ‘A’ , ‘B’ & ‘C’ | 74,94,462/- | ----- | 74,94,462/- |
| Group ‘B’ C.C. Block | 43,12,581/- | ------ | 43,12,581/- |
| Group ‘C’ Cleaning of Sewer line i/c silt clearance of surface | 392,825/- | ------ | 392,825/- |
| Total | 1,21,99,868/- | ------ | 1,21,99,868/- |

Amount Total (a) + (b) Rs:___________________

________% above / below on the rates of CSR, Amount to be added / deducted on the basis

Total (b) of premium quoted ______________

Total (A) = a + b in works & Figures______________________________________________

Contractor 						 	EXECUTIVE ENGINEER

PUBLIC HEALTH ENGG:DIVISION

JAMSHORO