BILL OF QUANTITIES

NAME OF WORK:- Water Supply Scheme Manzoorabad Taluka  Manjhand District 					Jamshoro.

(A) 	Description and rate of item based on Composite schedule of Rates

| Item No: | Quantity | Description of Item to be executed at site | Rate | Unit | Amount |
| --- | --- | --- | --- | --- | --- |
| 01 | 02 | 03 | 04 | 05 | 06 |
|  |  | GROUP ‘A’ STORAGE TANKS 1 NO: SIZE (100’x150’) I/C SUMP WELL |  |  |  |
| 01 |  | Excavation for tank and reservoir in Hard soil i/c trimming sides to true alignment /design sections / profiles and shape leveling of beds of trenches to correct level and grade i/c laying of earth in 6” layer for construction of banks and dressing and disposal of surplus earth . Excavated earth within a one chain as directed by engineer incharge i/c providing fence guards. Lights. Flags and temporary crossings for Non vehicular traffic where ever required lift upto 5’ ft 91.52M) and lead upto one chain (30.5M) (P.H.S.I.No 3P/68) |  |  |  |
|  | 50648.20 Cft | 0’ to 5’ depth | 3300/- | P%0cft | 167,139/- |
| 02 |  | Excavation for tank and reservoir in soft rock by hammering and chiseling where blasting is not practicableor is prohibited i/c trimming and dressing sides to true alignment /design sections / profiles and shape leveling of beds of trenches to correct level and grade i/c laying of earth in 6” layer for construction of banks and dressing and disposal of surplus earth . Excavated earth within a one chain as directed by engineer incharge i/c providing fence guards. Lights.Flags and temporary crossings for non vehicular traffic where ever required lift upto 5’ ft 91.52M) and lead upto one chain (30.5M)(P.H.S.I.No 8P/70) |  |  |  |
|  | 35097.97 Cft | 5’ to 10’ depth | 10170/ | P%0cft | 356,946/- |
| 03 | 68596.93 Cft | Extra for 50’ft additional lead or part there of for earth work (soft Ordinary or hard very hard) (P.H.S.I.No 8(a)P/2) | 1813/50 | P%0cft | 124,401/- |
| 04 | 6354.20 Cft | Cement concrete brick or stone ballast 1½” to 2” guage Ratio 1:4:8 (G.S.I.NO:4(b)P/17) | 9416/28 | P%cft | 598,329/- |
| 05 | 9157.50 Cft | Pacca brick work in foundation and plinth Cement Sand mortar (ratio 1:6) (G.S.I.No 4(e)P/20) | 11948/36 | P%cft | 10,94,171/- |

(Cont: on page No:02)

(Page No:02)

| Item No: | Quantity | Description of Item to be executed at site | Rate | Unit | Amount |
| --- | --- | --- | --- | --- | --- |
| 01 | 02 | 03 | 04 | 05 | 06 |
| 06 | 3946.81 Cft | Cement Concrete plain i/c placing compacting Finishing the curing complete i/c screening and washing stone ggregate with out shuttering (Ratio 1;2;4) (G.S.I.No 5(f)P/15) | 14429/25 | P%cft | 569,495/- |
| 07 | 253.0 Sft | Errection and removal of centering for R.C.C or Plain Cement Concrete works of Deodar wood(2nd class )(Vertical)(G.S.I.No 19(b)(ii)P/17) | 3127/41 | P%sft | 7,912/- |
| 08 | 4230.93 sft | Cement Plaster (1;4)up to 12’ft height 3/4” thick In cement sand mortar (G.S.I.No 11(c)P/51) | 3015/76 | P%sft | 127,595/- |
| 09 | 1536.0 Sft | Cement Pointing struck joints on wall (1;3)up to 20’ft height(G.S.I.No 19(b)P/52) | 1213/58 | P%sft | 18,641/- |
| 10 | 111.07 Cft | Reinforced cement concrete work including all labour and material except the cost of steel reinforcement and its labour for bending and binding, which will be paid separately. This rate also includes all kinds of forms moulds, lifting, shuttering currying rendering and finishing the exposed surface)\ (including screening and washing of shingle)(a) R.C.C. Work in roof slab, beams, columns, rafts, lintels and other structural members laid in suite or precuts laid in position complete in all respect (i) Ratio 1:2:4) 90 lbs cement 2 cft sand 4 cft shingle 1/8” to ¼” gauge.(G.S.I.NO;6(a) (i)P/19) | 337/- | P/Cft | 37,431/- |
| 11 | 4.46 Cwt | Fabrication of mild steel reinforcement for cement concrete including cutting bending laying in position making joints and fastenings including cost of binding wire (also includes removal of rust from bars (G.S.I.NO;8(ii)(a)P/17) | 5001/70 | P/Cwt | 22,308/- |
|  |  | INTER CONNECTION |  |  |  |
| 12 | 168.0 cft | Excavation for pipe lines into trenches and pits in hard soils i/c trimming and dressing sides to true alignment and shape leveling of beds of trenches to correct level and grade cutting joint holes and disposal of surplus earth within a one chain as directed by Engineer incharge providing fence guards lights flags and temporary crossing for non vehicular traffic where ever required lift upto 5’ft(1.52m) and lead upto one chain(30.5mm) (P.H.S.I.NO; 1, P/61) | 3900/- | P%0cft | 6,552/- |
| 13 |  | Providing laying R.C.C pipes of A.S.T.M C-76-62 P.V.C Pipe class “B” equvalant make fixing in T/C 76-70 of class ii wall B and fixing in trench i/c cutting fitting and jointing with rubber ring i/c testing with water specified pressure (P.H.S.I.NO;B, (i) P/17) |  |  |  |
|  | 12.00 Rft | 12” dia | 412/- | P/Rft | 4,944/- |

(Cont: on page No:03)

(Page No:03)

| Item No: | Quantity | Description of Item to be executed at site | Rate | Unit | Amount |
| --- | --- | --- | --- | --- | --- |
| 01 | 02 | 03 | 04 | 05 | 06 |
| 14 | 151.20 Cft | Refilling the excavated stuff in trenches 6” thick layers i/c watering ramming to full compaction etc: complete (P.H.S.I.NO;24, P/53) | 2760/- | P%0cft | 417 |
|  |  |  |  | Total | 31,36,284/- |
|  |  | GROUP ‘B’ CONSTT: OF PUMP HOUSE SIZE (10’ x 12’) |  |  |  |
| 01 | 239.0 Cft | Excavation in foundation of building bridges and other structures including dabbling dressing refilling around structures with excavated earth watering and ramming lead upto 5’ft (in hard soil) (G.S.I.NO;18,. P/4) | 3554/38 | P%0cft | 849/- |
| 02 | 90.56 Cft | Cement concrete brick or stone ballast 1½” to 2” gauge Ratio 1:4:8) (G.S.I.NO;4(b)P/16) | 9416/28 | P%cft | 8,527/- |
| 03 | 166.25 Cft | Cement concrete plain including placing compacting finishing and curring complete including screening and washing of stone aggregate without shuttering (Ratio 1:2:4) (G.S.I.NO;5(c)P/05) | 12595/- | P%cft | 20,939/- |
| 04 | 329.0 Sft | Errection and removal of centering for R.C.C or plain cement concrete works of deodar wood)2nd class)(for partal wood) (Vertical)(G.S.I.NO;19,(b) P/17) | 3127/41 | P%sft | 10,289/- |
| 05 | 218.50 Cft | Filling watering and ramming earth under floor with new earth (Excavated from out side)Lead upto one chain and lift upto 5’ft(G.S.I.NO;22, P/44) | 1512/- | P%0cft | 330/- |
| 06 | 272.85 Cft | Pacca brick work ground floor i/c sticking of joint Upto joint upto 20’ height in (1:6) cement (G.S.I.5, P/20) | 12674/36 | P%cft | 34,506/- |
| 07 | 124.67 Cft | Reinforced cement concrete work including all labour and material except the cost of steel reinforcement and its labour for bending and binding, which will be paid separately. This rate also includes all kinds of forms moulds, lifting, shuttering currying rendering and finishing the exposed surface)\ (including screening and washing of shingle)(a) R.C.C. Work in roof slab, beams, columns, rafts, lintels and other structural members laid in suite or precuts laid in position complete in all respect (i) Ratio 1:2:4) 90 lbs cement 2 cft sand 4 cft shingle 1/8” to ¼” gauge.(G.S.I.NO;6(a) (i)P/19) | 337/- | P/Cft | 42,014/- |
| 08 | 5.00 Cwt | Fabrication of mild steel reinforcement for cement concrete including cutting bending laying in position making joints and fastenings including cost of binding wire (also includes removal of rust from bars (G.S.I.NO;8(ii)P/24) | 5001/70 | P/Cwt | 25,009/- |

(Cont: on page No:04)

(Page No:04)

| Item No: | Quantity | Description of Item to be executed at site | Rate | Unit | Amount |
| --- | --- | --- | --- | --- | --- |
| 01 | 02 | 03 | 04 | 05 | 06 |
| 09 | 832.50 sft | Cement plaster (1:6) upto ½” thick upto 20’ft height. (G.S.I.NO; P/ ) | 2206/60 | P%sft | 18,370/- |
|  | 832.50 Sft | (Ratio 1:4) 3/8” thick | 2197/52 | P%sft | 18,294/- |
| 10 | 120.00 Sft | Providing & laying 2” thick topping cement concrete (1:2:4) i/c surface finishing and dividing into panels(G.S.I.NO;16( c ) P/43) | 3275/29 | P%sft | 3,930/- |
| 11 | 832.50 Sft | White washing (1 coat) (G.S.I.NO;26, P/55) | 416/63 | P%sft | 3,468/- |
| 12 | 120.0 Sft | White washing (3 coats) (G.S.I.NO;26, P/53) | 829/95 | P%Sft | 996/- |
| 13 | 18.00 Sft | Making and fixing steel grated door with 1/16” thick sheeting i/c angle iron frame2”x2”x3/8” and ¾” square bars 4” center to center with locking arrangement (G.S.I.NO;28, P/97) | 726/72 | P/Sft | 13,081/- |
| 14 | 2.20 Cwt | Fabrication of heavy steel work with angles tees, flat iron , round iron and sheet iron for making trusses girdes tands etc including cutting tanks etc.i/c cutting drilling riverting handling assembling and fixing but excluding erection in position (G.S.I.No.2,P-91) | 4928/49 | P/Cwt | 10,843/- |
| 15 | 24.00 Sft | Supplying and fixing in position iron / steel grill of ¾” x ¼” size flate iron of approved design i/c painting 3 coats etc: complete (weight not to be less than 3.7 lbs/sq: feet of finished (G.S.I.NO:26 P/92) | 2116/41 | P/sft | 4,332/- |
| 16 | 2.20 Cwt | Erection and fitting in position of iron trusses staging of water tanks etc. (G.S.I.No.3,P-90). | 271/04 | P/Cwt | 596/- |
| 17 | 84.00 Sft | PAINTING NEW SURFACE. Preparing surface and painting of doors and windows in any type (3 coats) (G.S.I.NO;5P/69) | 2116/41 | P%sft | 1,778/- |
|  |  |  |  | Total | 225,310/- |

(Cont: on page No;05)

(Page No:05)

| Item No: | Quantity | Description of Item to be executed at site | Rate | Unit | Amount |
| --- | --- | --- | --- | --- | --- |
| 01 | 02 | 03 | 04 | 05 | 06 |
|  |  | GROUP ‘C’ P/I & TESTING PUMPING MACHINARY (40 BHP) |  |  |  |
| 01 | 2 sets | Supplying and installing i/c transportation to site of work for Water supply Scheme at Manzoorabad electric pumping set consisting of A.C. electric motor of 40 BHP 3 Phase 50 cycles 60/200 volts 2900 RPM (ILG 4 206 20AA 60) (coupled with Centrifugal K.S.B Pump type ETA noram 80-26 Size 4” x 3” mounted on common steel frame or base plate (KSB made) capable of discharging 370 gallons per minute against a head of 150ft i/c installation of pumping set on C.C. foundation Ratio 1:2:4 and Ratio 1:4:8 with ballast with M.S. nuts and bolts of 5/8” dia to be embedded in C.C. foundation i/c providing and fixing MCU (KSB made) all asseceries consisting ASD (Automatic star delta starter) (circuit breaker MCCB over load relay, timer relay push button indicators and internal wiring ) providing internal electric wiring from MCU to electric motor 3 – 19 1.53 (19/0.064)PVC insulated wire 660 volts grade in 40 mm (1½”) PVC conduit on surface as required i/c providing and fixing earthling set with size (2x2x1/4”)G.I. Plate buried in the ground at the ground at depth 7mm (12’ft) for base plate and testing of the pumping set against the required head for 72 hours etc: complete. | 569560/- | P/Set | 11,39,120/- |
| 02 |  | Manufacturing & installing M.S pipe 4” & 6” dia made out ¼” sheet 3/16” Thick cutting folding curvature and shape welding etc complete (R.A.A Based on Sch: rates) |  |  |  |
|  | 20.0 Rft | 4” dia for suction | 506/60 | P/Rft | 10,132/- |
|  | 28.0 Rft | 4” dia for deliver | 506/60 | P/rft | 14,185/- |
| 03 |  | Manufacturing & installing M.S flanged 4” & 6” dia made out M.S Sheet 3/8” Thick cutting drilling holes etc complete(R.A.A Base on Sch: rates) |  |  |  |
|  | 16 Nos: | 4” dia | 596/75 | Each | 9,548/- |
| 04 |  | C.I.Sluice valve heavy pattern (Test pressure 21.0kg/sq:cm or 300lb/sq:inch)(Imported). (P.H.S.I.(M) NO:2, P/9) |  |  |  |
|  | 1 No: | 3” dia | 4290/- | Each | 4,290/- |
| 05 |  | Reflex valve heavy pattern test pressure 300 lbs/sq: inch in pointed (P.H.S(M) -6, P/10) |  |  |  |
|  | 1 No; | 3” dia | 1543/76 | Each | 1,544/- |

(Cont: on page No:06)

(Page No:06)

| Item No: | Quantity | Description of Item to be executed at site | Rate | Unit | Amount |
| --- | --- | --- | --- | --- | --- |
| 01 | 02 | 03 | 04 | 05 | 06 |
| 06 | 12.41 Kg: | C.I. Tee flanged ends with holes including turning & facing of flange for All size (PHSMINo:12,P-12) | 120/- | P/Kg: | 1,489/- |
| 07 | 1 No: | Foot valve heavy pattern with cone type gate (P.H.S.(M)I-4, P/10) | 731/25 | Each | 731/- |
| 08 | 13.36 Kg: | C.I.Tapper flat bottomed or central tapered flange end/tees bend with holesi/c turning and facing of flange for all size (P.H.S.(M)I-1, P/11) | 120/- | P/Kg: | 1,603/- |
| 09 | 47.75 Kg: | C.I.Bend/tees with flanges and with holes i/c turning and facing of flanged for all size (P.H.S.(M)I-8, P/11) | 120/- | P/ Kg: | 5,730/- |
| 10 |  | Jointing C.I/M.S flanged pipes and specials flanged and inside a trenches i/c supplying rubber packing of the required thickness nuts bolts with washers etc and other tools required for jointing and testing the joints to the specified pressure etc: complete.(P.H.S.I.NO;1, P/35) |  |  |  |
|  | 11 Joints | 4” dia | 513/- | P/joint | 4,543/- |
|  |  |  |  | Total | 11,92,915/- |
|  |  | GROUP ‘D’ P/L/J & TESTING PVC RISNG MAIN 6” DIA |  |  |  |
| 01 | 14409.0 Cft | Excavation for pipe lines into trenches and pits in Soft soil i/c trimming and dressing sides to true alignment and Shape leveling of beds of trenches to correct level and grade cutting joint holes and disposal of surplus earth within a one chain as directed by Engineer incharge providing fence guards lights flags and temporary crossing for non vehicular traffic where ever required lift upto 5’ft(1.52m) and lead upto one chain (30.5mm) (P.H.S.I.NO; 4, P/60) | 3600/- | P%0cft | 51,872/- |
| 02 | 14409.0 Cft | Excavation for pipe lines into trenches and pits in wet soil i/c trimming and dressing sides to true alignment and shape leveling of beds of trenches to Correct level and grade cutting joint holes and disposal of surplus earth within a one chain as directed by Engineer incharge providing fence guards lights flags and temporary crossing for non vehicular traffic where ever required lift upto 5’ft(1.52m) and lead upto one chain (30.5mm) (P.H.S.I.NO; 6, P/62) | 5400/- | P%0cft | 77,809/- |

(Cont: on page No:07)

(Page No:07)

| Item No: | Quantity | Description of Item to be executed at site | Rate | Unit | Amount |
| --- | --- | --- | --- | --- | --- |
| 01 | 02 | 03 | 04 | 05 | 06 |
| 03 | 21613.50 Cft | Excavation for pipe lines into trenches and pits in slushy daldali soil i/c trimming and dressing sides to true alignment and Shape leveling of beds of trenches to correct level and grade cutting joint holes and disposal of surplus earth within a one chain as directed by Engineer incharge providing fence guards lights flags and temporary crossing for non vehicular traffic where ever required lift upto 5’ft(1.52m) and lead upto one chain (30.5mm) (P.H.S.I.NO; 6, P/62) | 10800/- | P%0cft | 233,426/- |
| 04 |  | Providing PVC pipe of class ‘B’ (equivalent make)fixing in trenches i/c cutting and jointing with “Z” joints with one rubber rings i/c testing with water to head of 61 meter or 200’ft.(P.H.S.I.NO;1,P/22) |  |  |  |
|  | 6404.0 Rft | 6” dia | 262/- | P/Rft | 16,77,848/- |
| 05 |  | P.V.C. ‘Z’ Joints fitting (P.H.S.(M)I-16, P/28) |  |  |  |
|  | 2 Nos: | PVC BEND 6” dia 900 | 1913/- | Each | 3,826/- |
| 06 |  | P.V.C Socket (P.H.S(M)I-16,P/113) |  |  |  |
|  | 12 Nos: | 6” dia | 1575/- | Each | 18,900/- |
| 07 |  | C.I. Gibault joint(P.H.S.I(M)-14, P/16) |  |  |  |
|  | 5 Nos: | 6” dia | 96/58 | Each | 483/- |
| 08 |  | Making joints specials fitting (i/c laying of specials and cost of solvent cement of required diameter and testing the joints alongwith PVC Pipe line ‘B’ ‘C’ and ‘D’ class of the specified pressure and making Good to all leaky joints etc: complete(P.H.S.I.NO:1, P/35) |  |  |  |
| 09 | 3 Nos: | Air valve double acting (P.H.S.M.I.NO;8, P/98) | 4000/- | Each | 12,000/- |
| 10 | 3 Nos: | Fire hydrant Tee (P.H.S.I.NO;8, P/9) | 1375/- | Each | 4,125/- |
| 11 | 3 Nos: | Providing chamber 3”x2” (915x615mm) inside dimension 4½”(1372 mm) deep as per approved design for sluice valve 3” to 12” dia with 18” (457mm) dia inside cast iron cover and frame (wt= 1 cwt 3 qr) fixed in RCC 1:2:4 (102 mm) thick with 5 lbs steel per cft)9” (299 mm) thick brick masonary wall set in 1:6 cement mortar 6” (1152mm ) thick cement concrete 1:3:6 in foundation 1” thick cement concrete 1:2:4 flooring ½” (12.5mm) thick cement plaster 1:3 to all inside wall surface and to top i/c providing and fixing M.S. foot beyond 2½” depth curring excavation back filling and disposal of surplus earth etc; complete (P.H.S.I.NO:1, P/49) | 18820/- | Each | 56,460/- |

(Cont: on page No:08)

(Page No:08)

| Item No: | Quantity | Description of Item to be executed at site | Rate | Unit | Amount |
| --- | --- | --- | --- | --- | --- |
| 01 | 02 | 03 | 04 | 05 | 06 |
| 12 | 45388.35 Cft | Refilling the excavated stuff in trenches 6” thick layers i/c watering ramming to full compaction etc: complete (P.H.S.I.NO;24, P/53) | 2760/- | P%0cft | 12,272/- |
|  |  |  |  | Total | 22,64,401/- |
|  |  | GROUP ‘E’ P/LJ & TESTING PVC TRUNK MAIN 6” DIA |  |  |  |
| 01 | 4893.7 Cft | Excavation for pipe lines into trenches and pits in all kind of soil i/c trimming and dressing sides to true alignment and Shape leveling of beds of trenches to correct level and grade cutting joint holes and disposal of surplus earth within a one chain as directed by Engineer incharge providing fence guards lights flags and temporary crossing for non vehicular traffic where ever required lift upto 5’ft(1.52m) and lead upto one chain (30.5mm) (P.H.S.I.NO; 4, P/60) | 4650/- | P%0cft | 22,756/- |
| 02 | 6525.00 Cft | Excavation for pipe lines into trenches and pits in soft soil i/c trimming and dressing sides to true alignment and shape leveling of beds of trenches to correct level and grade cutting joint holes and disposal of surplus earth within a one chain as directed by Engineer incharge providing fence guards lights flags and temporary crossing for non vehicular traffic where ever required lift upto 5’ft(1.52m) and lead upto one chain (30.5mm) (P.H.S.I.NO; 6, P/62) | 10770/- | P%0cft | 70,234/- |
| 03 |  | Providing PVC pipe of class ‘B’ (equivalent make)fixing in trenches i/c cutting and jointing with “Z” joints with one rubber rings i/c testing with water to head of 61 meter or 200’ft.(P.H.S.I.NO;1,P/22) |  |  |  |
|  | 1450.0 Rft | 6” dia | 262/- | P/Rft | 379,900/- |
| 04 |  | P.V.C. ‘Z’ Joints fitting (P.H.S.(M)I-16, P/28 |  |  |  |
|  | 2 Nos: | PVC BEND 6” dia 900 | 1913/- | P/Rft | 3,826/- |
| 05 |  | P.V.C Socket (P.H.S(M)I-16,P/113) |  |  |  |
|  | 10 Nos: | 6” dia | 1575/- | Each | 15,750/- |
| 06 |  | C.I. Gibault joint(P.H.S.I(M)-14, P/16) |  |  |  |
|  | 5 Nos: | 6” dia | 96/58 | Each | 483/- |
| 07 |  | Making joints specials fitting (i/c laying of specials and cost of solvent cement of required diameter and testing the joints alongwith PVCPipe line ‘B’ ‘C’ and ‘D’ class of the specified pressure and making Good to all leaky joints etc: complete(P.H.S.I.NO:1, P/35) |  |  |  |
|  | 34 Nos: | 6” dia | 70/- | Each | 2,380/- |

(Cont: on page No:09)

(Page No;09)

| Item No: | Quantity | Description of Item to be executed at site | Rate | Unit | Amount |
| --- | --- | --- | --- | --- | --- |
| 01 | 02 | 03 | 04 | 05 | 06 |
| 08 | 1 No: | Air Valve Double acting (P.H.S.M.NO:8, P/98) | 4000/- | Each | 4,000/- |
| 09 | 1 No: | Fir hydrant Tee (P.H.S.I.NO;8 P/98) | 1375/- | Each | 1,375/ |
| 10 | 3 Nos: | Providing chamber 3”x2” (915x615mm) inside dimension 4½”(1372 mm) deep as per approved design for sluice valve 3” to 12” dia with 18” (457mm) dia inside cast iron cover and frame (wt= 1 cwt 3 qr) fixed in RCC 1:2:4 (102 mm) thick with 5 lbs steel per cft)9” (299 mm) thick brick masonary wall set in 1:6 cement mortar 6” (1152mm ) thick cement concrete 1:3:6 in foundation 1” thick cement concrete 1:2:4 flooring ½” (12.5mm) thick cement plaster 1:3 to all inside wall surface and to top i/c providing and fixing M.S. foot beyond 2½” depth curring excavation back filling and disposal of surplus earth etc; complete (P.H.S.I.NO:1, P/49) | 18820/- | Each | 56,460/- |
| 11 |  | Supplying of G.I. Pipe approved quality to be tested with pressure 200 ‘ft (P.H.S.I.NO;1, P/ ) |  |  |  |
|  | 150.0 Rft | 6” dia | 695/04 | P/Rft | 104,256/- |
| 12 |  | Fir hydrant Tee (P.H.S.I.NO;8 P/98) |  |  |  |
| 13 |  | Providing & fixing M.S.flange made of M.S. plate, having a thickness and total weight as mentioned against each item as mentioned against each item It includes the cost of making holes, facing, welding, nuts bolts, rubber packing, white lead fitter, catage etc complete. (P.H.S.of M.S.Speials, item No:8,Page No.8) |  |  |  |
|  | 30 Nos: | 6” dia | 804/77 | Each | 24,143/- |
| 14 |  | Laying C.I/M.S pipe (S&S) flanged and specials and fixing in Trenches i/c cutting fitting and testing with water to specified Pressure (P.H.S.I.M.NO;B, P/54) |  |  |  |
|  | 150.00 Rft | 6” dia | 11/64 | P/Rft | 1,746/- |
| 15 | 200 Kg: | C.I. bends with flanged ends with holes including tuning and facing of flanges for all sizes (P.H.S.M. I.NO.8 P-98) | 120/- | P/kg: | 24,000/- |
| 16 | 130 Kg: | C.I TEE Flange ends with holes i/c curing and facing of Flanged for all size (sch of (M) 11/P/11) | 120/- | P/Kg: | 15,600/- |
| 17 |  | C.I.Sluice valve heavy (Test pressure 21.0 Kg/sq:cm or 300 lbs/sq: inch (Imported)(M.S.I.NO;2(d)P/97) |  |  |  |
|  | 2 Nos: | 6” dia | 9360/- | Each | 18,720/- |
| 18 |  | C.I.Tail Piece for P.V.C. Pipe (P.H.S.(M)I-1, P/101) |  |  |  |
|  | 4 Nos: | 6” dia | 618/75 | Each | 2,475/- |

(Cont: on page No:10)

(Page No:10)

| Item No: | Quantity | Description of Item to be executed at site | Rate | Unit | Amount |
| --- | --- | --- | --- | --- | --- |
| 01 | 02 | 03 | 04 | 05 | 06 |
| 19 | 10276.65 Cft | Refilling the excavated stuff in trenches 6” thick layers i/c watering ramming to full compaction etc: complete (P.H.S.I.NO;24, P/53) | 2760/- | P%0cft | 7,091/- |
|  |  |  |  | Total | 755,235/- |
|  |  | GROUP ‘F’ P/L/J & TESTING G.I.PIPE 3” DIA |  |  |  |
| 01 |  | Supplying G.I. Pipe of approved quality to be tested with pressure Head 200 ft (Medium) (S.S.O.M.I.NO;1/h, I (iii) P/125) |  |  |  |
|  | 1000.0 Rft | 3” dia | 304/02 | P/Rft | 304,020/- |
| 02 |  | Painting 2 coats of bitumen iron paint and warping basin cloth soaked in maxphaltic composition to pipes for external works. (W.S.& S I.No.2 ,Page No.15) Dismantling of Cement concrete (1:2:4) (G.S.I.No;19 , P/10) |  |  |  |
|  | 1000.00 Rft | 3” dia | 13/- | P/Rft | 13,000/- |
| 03 | 500.0 Cft | Dismantling of Cement concrete (1:4:8)(G.S.I.NO;19 , P/10) | 3327/50 | P%cft | 16,638/- |
| 04 | 740.0 Cft | Dismantling of cement concrete (1:4:8) (G.S.I.NO;19, P/10) | 1663/15 | P%cft | 12,307/- |
| 05 | 300.0 Cft | Excavation for pipe lines into trenches and pits in soft soil i/c trimming and dressing sides to true alignment and shape leveling of beds of trenches to correct level and grade cutting joint holes and disposal of surplus earth within a one chain as directed by Engineer incharge providing fence guards lights flags and temporary crossing for non vehicular traffic where ever required lift upto 5’ft(1.52m) and lead upto one chain (30.5mm) (P.H.S.I.NO; 6, P/62) | 3600/- | P%0cft | 1,080/- |
| 06 |  | Providing & fixing M.S.flange made of M.S. plate, having a thickness and total weight as mentioned against each item as mentioned against each item It includes the cost of making holes, facing, welding, nuts bolts, rubber packing, white lead fitter, catage etc complete. (P.H.S.of M.S.Speials, item No:8,Page No.8) |  |  |  |
|  | 200 Nos: | 3” dia | 945/- | Each | 189,000/- |
| 07 |  | Laying C.I/M.S pipe (S&S) and specials and fixing in Trenches i/c cutting fitting and testing with water to specified Pressure (P.H.S.I.NO;B, P/78) |  |  |  |
|  | 1000.0 Rft | 3” dia | 5/82 | P/Rft | 5,820/- |

(Cont: on page No:11)

(Page No:11)

| Item No: | Quantity | Description of Item to be executed at site | Rate | Unit | Amount |
| --- | --- | --- | --- | --- | --- |
| 01 | 02 | 03 | 04 | 05 | 06 |
| 08 | 133.60 Kg: | Supplying C.I. bend with flanged ends with holes i/c turning and facing of flanged (S.O.M.I.NO;8,P/11) | 120/- | P/Kg: | 16,032/- |
| 09 | 54.42 Kg: | C.I TEE Flange ends with holes i/c curing and facing of Flanged for all size (sch of (M) 11/P/11) | 120/- | P/Kg: | 6,530/- |
| 10 |  | C.I.Sluice valve heavy pattern (Test pressure 21.0kg/sq:cm or 300lb/sq:inch) (Imported).(P.H.S.I.(M) NO:2, P/9) |  |  |  |
|  | 5 Nos: | 3” dia | 4290/- | Each | 21,450/- |
| 11 | 10 Nos: | Supplying C.I. Short piece (SMI.NO P/ 107) | 387/- | Each | 3,870/- |
| 12 |  | Jointing C.I/M.S flanged pipes and specials flanged and inside a trenches i/c supplying rubber packing of the required thickness nuts bolts with washers etc and other tools required for jointing and testing the joints to the specified pressure etc: complete.(P.H.S.I.NO;1, P/40) |  |  |  |
|  | 100 Joints | 3” dia | 499/- | P/joint | 49,900/- |
| 13 |  | Making joints specials fitting (i/c laying of specials and cost of solvent cement of required diameter and testing the joints alongwith PVC Pipe line ‘B’, ‘C’ and ‘D’ class of the specified pressure and making Good to all leaky joints etc; complete(PHSI.NO;1, P/35) |  |  |  |
|  | 10 Nos: | 3” dia | 96/- | Each | 960/- |
| 14 | 270.0 cft | Refilling the excavated stuff in trenches 6” thick layers i/c watering ramming full compaction etc: complete (P.H.S.I.NO;24, P/53) | 2760/- | P%0cft | 745/- |
|  |  |  |  | Total | 641,352/- |

A   B  S   T   R   A   C   T

| Group Name | Schedule Item | Non Schedule | Total |
| --- | --- | --- | --- |
| Group ‘A’ Storage Tank 1 No; Size (100’x150) | 31,36,284/- | ----- | 31,36,284/- |
| Group ‘B’ Constt: of Pump House 10’x12’ | 225,310/- | ----- | 225,310/- |
| Group ‘C’ P/ I Pumping Machinery 40 BHP | 53,795/- | 11,39,120/- | 11,92,915/- |
| Group ‘D’ P/L/J & Testing PVC Rising Main 6” dia | 22,64,401/- | ----- | 22,64,401/- |
| Group ‘E’ P/L/J Testing PVC Trunk main 6” dia | 225,310/- | ----- | 225,310/- |
| Group ‘F’ P/L/J G.I. Pipe 3’ dia | 6,41,352/- | ----- | 6,41,352/- |
| Total | 65,46,452/- | 11,39,120/- | 76,85,572/- |

(Cont: on page No:12)

(Page No:12)

Amount Total (a) + (b) Rs:___________________

________% above / below on the rates of CSR, Amount to be added / deducted on the basis

Total (b) of premium quoted ______________

Total (A) = a + b in works & Figures______________________________________________

Contractor 						 	EXECUTIVE ENGINEER

PUBLIC HEALTH ENGG:DIVISION

JAMSHORO