BILL OF QUANTITIES

NAME OF WORK:- Water Supply Scheme Khosa village near Power House Colony Taluka 				Manjhand District Jamshoro.

(A) 	Description and rate of item based on Composite schedule of Rates

| Item No: | Quantity | Description of Item to be executed at site | Rate | Unit | Amount |
| --- | --- | --- | --- | --- | --- |
| 01 | 02 | 03 | 04 | 05 | 06 |
|  |  | GROUP ‘A’ CONSTT: OF STORAGE TANKS 1 No. SIZE (120x60) |  |  |  |
| 01 |  | Excavation for tanks and reservoirs in soft soil i/c trimming and dressing sides to true alignment/design correct/profile and shape, leveling of beds of tanks to correct level and grade i/c laying of earth 6” layers for construction of tanks and dressing and disposal of surplus excavated earth within one chain as directed by Engineer incharge i/c providing fence guards lights flags wherever required lift upto 5’feet and lead upto one chain (P.H.S.NO;1, P/67) |  |  |  |
|  | 8559.0 Cft | 0’ to 5’ depth | 3000/- | P%0cft | 25,677/- |
| 02 |  | Excavation for tanks and reservoirs in hard soil i/c trimming and dressing sides to true alignment/design correct/profile and shape,leveling of beds of tanks to correct level and grade i/c laying of earth 6” layers for construction of tanks and dressing and disposal of surplus excavated earth within one chain as directed by Engineer incharge i/c providing fence guards lights flags wherever required lift upto 5’feet and lead upto one chain (P.H.S.NO;5, P/69) |  |  |  |
|  | 25727.96 Cft | 5’ to 8’ depth | 3300/- | P%0cft | 84,902/- |
| 03 | 27267.0 Cft | Extra for 50’ft additional lead or part there of for earth work (soft Ordinary or hard very hard) (P.H.S.I.No 8(a)P/2) | 1813/50 | P%0cft | 49,452/- |
| 04 | 3598.90 Cft | Cement concrete brick or stone ballast 1½” to 2” guage Ratio 1:4:8 (G.S.I.NO:4(b)P/17) | 9416/28 | P%cft | 338,883/- |
| 05 | 6855.58 Cft | Pacca brick work in foundation and plinth Cement Sand mortar (ratio 1:6) (G.S.I.No 4(e)P/20) | 11948/36 | P%cft | 819,129/- |
| 06 | 2071.17 Cft | Cement Concrete plain i/c placing compacting Finishing the curing complete i/c screening and washing stone ggregate with out shuttering (Ratio 1;2;4) (G.S.I.No 5(f)P/15) | 14429/25 | P%cft | 298,941/- |
| 07 | 241.56 Sft | Errection and removal of centering for R.C.C or Plain Cement Concrete works of Deodar wood(2nd class )(Vertical)(G.S.I.No 19(b)(ii)P/17) | 3127/41 | P%sft | 7,555/- |

(Cont: on page No:02)

(Page No:02)

| Item No: | Quantity | Description of Item to be executed at site | Rate | Unit | Amount |
| --- | --- | --- | --- | --- | --- |
| 01 | 02 | 03 | 04 | 05 | 06 |
| 08 | 229.22 Cft | Reinforced cement concrete work including all labour and material except the cost of steel reinforcement and its labour for bending and binding, which will be paid separately. This rate also includes all kinds of forms moulds, lifting, shuttering currying rendering and finishing the exposed surface)\ (including screening and washing of shingle)(a) R.C.C. Work in roof slab, beams, columns, rafts, lintels and other structural members laid in suite or precuts laid in position complete in all respect (i) Ratio 1:2:4) 90 lbs cement 2 cft sand 4 cft shingle 1/8” to ¼” gauge.(G.S.I.NO;6(a) (i)P/19) | 337/- | P/Cft | 77,247/- |
| 09 | 9.20 Cwt | Fabrication of mild steel reinforcement for cement concrete including cutting bending laying in position making joints and fastenings including cost of binding wire (also includes removal of rust from bars (G.S.I.NO;8(ii)(a)P/17) | 5001/70 | P/Cwt | 46,016/- |
| 10 | 2944.92 Cft | Cement Plaster (1;4)up to 12’ft height 3/4” thick In cement sand mortar (G.S.I.No 11(c)P/51) | 2283/93 | P%sft | 67,260/- |
| 11 | 75.21 Cft | Supplying Dampo (For mixing with cement plaster for water proofing purposes (G.S.I.No:46, P/46) | 56/82 | P/Kg: | 4,274/- |
|  |  | INTER CONNECTION |  |  |  |
| 12 | 210.0 Cft | Excavation for pipe lines into trenches and pits in hard soils i/c trimming and dressing sides to true alignment and shape leveling of beds of trenches to correct level and grade cutting joint holes and disposal of surplus earth within a one chain as directed by Engineer incharge providing fence guards lights flags and temporary crossing for non vehicular traffic where ever required lift upto 5’ft(1.52m) and lead upto one chain(30.5mm) (P.H.S.I.NO; 1, P/61) | 3900/- | P%0cft | 819/- |
| 13 |  | Providing laying R.C.C pipes of A.S.T.M C-76-62 P.V.C Pipe class “B” equvalant make fixing in T/C 76-70 of class ii wall B and fixing in trench i/c cutting fitting and jointing with rubber ring i/c testing with water specified pressure (P.H.S.I.NO;B, (i) P/17) |  |  |  |
|  | 15.0 Rft | 12” dia | 412/- | P/Rft | 6,180/- |
|  | 189.0 Cft | Refilling the excavated stuff in trenches 6” thick layers i/c watering ramming to full compaction etc: complete (P.H.S.I.NO;24, P/53) | 2760/- | P%0cft | 522/- |
|  |  |  |  | Total | 18,26,857/- |

(Cont: on page No:03)

(Page No:03)

| Item No: | Quantity | Description of Item to be executed at site | Rate | Unit | Amount |
| --- | --- | --- | --- | --- | --- |
| 01 | 02 | 03 | 04 | 05 | 06 |
|  |  | GROUP ‘B’ CONSTT: OF PUMP HOUSE SIZE (10’ x 12’) |  |  |  |
| 01 | 239.00 cft | Excavation in foundation of building bridges and other structures including dabbling dressing refilling around structures with excavated earth watering and ramming lead upto 5’ft (in hard soil) (G.S.I.NO;18,. P/4) | 3554/38 | P%0cft | 849/- |
| 02 | 90.56 cft | Cement concrete brick or stone ballast 1½” to 2” gauge Ratio 1:4:8) (G.S.I.NO;4(b)P/16) | 9416/28 | P%cft | 8,527/- |
| 03 | 166.25 cft | Cement concrete plain including placing compacting finishing and curring complete including screening and washing of stone aggregate without shuttering (Ratio 1:2:4) (G.S.I.NO;5(c)P/05) | 12595/- | P%cft | 20,939/- |
| 04 | 329.00 sft | Errection and removal of centering for R.C.C or plain cement concrete works of deodar wood)2nd class)(for partal wood) (Vertical)(G.S.I.NO;19,(b) P/17) | 3127/41 | P%sft | 10,289/- |
| 05 | 218.50 cft | Filling watering and ramming earth under floor with new earth (Excavated from out side)Lead upto one chain and lift upto 5’ft(G.S.I.NO;22, P/44) | 1512/- | P%0cft | 330/- |
| 06 | 272.25 cft | Pacca brick work other than building i/c sticking of joint Upto joint upto 20’ height in (1:6) cement (G.S.I.5, P/20) | 12674/36 | P%cft | 34,506/- |
| 07 | 124.67 cft | Reinforced cement concrete work including all labour and material except the cost of steel reinforcement and its labour for bending and binding, which will be paid separately. This rate also includes all kinds of forms moulds, lifting, shuttering currying rendering and finishing the exposed surface)\ (including screening and washing of shingle)(a) R.C.C. Work in roof slab, beams, columns, rafts, lintels and other structural members laid in suite or precuts laid in position complete in all respect (i) Ratio 1:2:4) 90 lbs cement 2 cft sand 4 cft shingle 1/8” to ¼” gauge.(G.S.I.NO;6(a) (i)P/19) | 337/- | P/Cft | 42,014/- |
| 08 | 5.00 cwt | Fabrication of mild steel reinforcement for cement concrete including cutting bending laying in position making joints and fastenings including cost of binding wire (also includes removal of rust from bars (G.S.I.NO;8(ii)P/24) | 5001/70 | P/Cwt | 25,009/- |
| 09 | 832.50 sft | Cement plaster (1:6) upto ½” thick upto 20’ft height. (G.S.I.NO; P/ ) | 2206/60 | P%sft | 18,370/- |
|  | 832.50 Sft | (14) 3/8” thick | 2197/52 | P%sft | 18,294/- |
| 10 | 120.0 Sft | Providing & laying 2” thick topping cement concrete (1:2:4) i/c surface finishing and dividing into panels(G.S.I.NO;16( c ) P/43) | 3275/29 | P%sft | 3,930/- |
| 11 | 832.50 Sft | White washing (1 coat) (G.S.I.NO;26, P/55) | 416/63 | P%sft | 3,468/- |
| 12 | 120.0Sft | White washing(3coats)GSI.NO;26,P/53) | 829/95 | P%Sft | 996/- |

(Cont: on page No:04)

(Page No:04)

| Item No: | Quantity | Description of Item to be executed at site | Rate | Unit | Amount |
| --- | --- | --- | --- | --- | --- |
| 01 | 02 | 03 | 04 | 05 | 06 |
| 13 | 832.50 Sft | Colour washing (2 coats) | 859/90 | P%sft | 7,159/- |
| 13 | 18.00 Sft | Making and fixing steel grated door with 1/16” thick sheeting i/c angle iron frame2”x2”x3/8” and ¾” square bars 4” center to center with locking arrangement (G.S.I.NO;28, P/97) | 726/72 | P/Sft | 13,081/- |
| 14 | 2.20 Cwt | Fabrication of heavy steel work with angles tees, flat iron , round iron and sheet iron for making trusses girdes tands etc including cutting tanks etc.i/c cutting drilling riverting handling assembling and fixing but excluding erection in position (G.S.I.No.2,P-91) | 4928/72 | P/cwt | 10,843/- |
| 15 | 24.00 sft | Supplying and fixing in position iron / steel grill of ¾” x ¼” size flate iron of approved design i/c painting 3 coats etc: complete (weight not to be less than 3.7 lbs/sq: feet of finished (G.S.I.NO:26 P/92) | 2116/41 | P/sft | 4,332/- |
| 16 | 2.20 Cwt | Erection and fitting in position of iron trusses staging of water tanks etc. (G.S.I.No.3,P-90). | 271/04 | P/Cwt | 596/- |
| 17 | 84.0 Sft | PAINTING NEW SURFACE. Preparing surface and painting of doors and windows in any type (3 coats) (G.S.I.NO;5P/69) | 2116/41 | P%sft | 1,778/- |
|  |  |  |  | Total | 225,310/- |
|  |  | GROUP ‘C’ P/L/J & TESTING PVC RISNG MAIN 4” DIA |  |  |  |
| 01 | 10000.0 Cft | Excavation for pipe lines into trenches and pits in all kind of soil i/c trimming and dressing sides to true alignment and Shape leveling of beds of trenches to correct level and grade cutting joint holes and disposal of surplus earth within a one chain as directed by Engineer incharge providing fence guards lights flags and temporary crossing for non vehicular traffic where ever required lift upto 5’ft(1.52m) and lead upto one chain (30.5mm) (P.H.S.I.NO; 4, P/60) | 4650/- | P%0cft | 46,500/- |
| 02 | 20000.0 Cft | Excavation for pipe lines into trenches and pits in soft rock soil i/c trimming and dressing sides to true alignment and Shape leveling of beds of trenches to correct level and grade cutting joint holes and disposal of surplus earth within a one chain as directed by Engineer incharge providing fence guards lights flags and temporary crossing for non vehicular traffic where ever required lift upto 5’ft(1.52m) and lead upto one chain (30.5mm) (P.H.S.I.NO; 6, P/62) | 10770/- | P%0cft | 215,400/- |

(Cont: on page No;05)

(Page No:05)

| Item No: | Quantity | Description of Item to be executed at site | Rate | Unit | Amount |
| --- | --- | --- | --- | --- | --- |
| 01 | 02 | 03 | 04 | 05 | 06 |
| 03 |  | Providing PVC pipe of class ‘B’ (equivalent make)fixing in trenches i/c cutting and jointing with “Z” joints with one rubber rings i/c testing with water to head of 61 meter or 200’ft.(P.H.S.I.NO;1,P/22) |  |  |  |
|  | 5000.0 Rft | 4” dia | 137/- | P/Rft | 685,000/- |
| 04 |  | P.V.C. ‘Z’ Joints fitting (P.H.S.(M)I-16, P/28) |  |  |  |
|  | 2 Nos: | PVC BEND 4” dia 900 | 956/- | Each | 1,912/- |
|  | 6 Nos: | Tee 4” x 4” | 1181/- | Each | 7,086/- |
| 05 |  | P.V.C Socket (P.H.S(M)I-16,P/113) |  |  |  |
|  | 10 Nos: | 4” dia | 1125/- | Each | 11,250/- |
| 06 |  | C.I. Gibault joint(P.H.S.I(M)-14, P/16) |  |  |  |
|  | 5 Nos: | 4” dia | 731/25 | Each | 3,656/- |
| 07 |  | Making joints specials fitting (i/c laying of specials and cost of solvent cement of required diameter and testing the joints alongwith PVC Pipe line ‘B’ ‘C’ and ‘D’ class of the specified pressure and making Good to all leaky joints etc: complete(P.H.S.I.NO:1, P/35) |  |  |  |
|  | 14 Nos: | 4” dia | 70/- | Each | 980/- |
| 08 |  | Supplying G.I. Pipe of approved quality to be tested with pressure Head 200 ft\ Medium) (S.S.O.M.I.NO;1/h, I (iii) P/125) |  |  |  |
|  | 200.0 Rft | 4” dia | 431/93 | P/Rft | 86,386/- |
| 09 |  | Providing & fixing M.S.flange made of M.S. plate, having a thickness and total weight as mentioned against each item as mentioned against each item It includes the cost of making holes, facing, welding, nuts bolts, rubber packing, white lead fitter, catage etc complete. (P.H.S.of M.S.Speials, No:8,P No.8) |  |  |  |
|  | 20 Nos: | 4” dia | 596/75 | Each | 11,935/- |
| 10 |  | Laying C.I/M.S pipe (S&S) and specials and fixing in Trenches i/c cutting fitting and testing with water to specified Pressure (P.H.S.I.NO;B, P/78) |  |  |  |
|  | 200.0 Rft | 4” dia | 7/76 | P/Rft | 1,552/- |
| 11 | 53.44 Kg: | Supplying C.I. bend with flanged ends with holes i/c turning and facing of flanged (S.O.M.I.NO;8,P/11) | 120/- | P/Kg: | 6,413/- |
| 12 | 2 Nos: | Air Valve Double acting (P.H.S.M.NO:8, P/98) | 4000/- | Each | 8,000/- |
| 13 | 2 Nos: | Fir hydrant Tee (P.H.S.I.NO;8 P/98) | 1112/50 | Each | 2,225/- |

(Cont: on page No:05)

(Page No:05)

| Item No: | Quantity | Description of Item to be executed at site | Rate | Unit | Amount |
| --- | --- | --- | --- | --- | --- |
| 01 | 02 | 03 | 04 | 05 | 06 |
| 14 | 2 Nos: | Providing chamber 3”x2” (915x615mm) inside dimension 4½”(1372 mm) deep as per approved design for sluice valve 3” to 12” dia with 18” (457mm) dia inside cast iron cover and frame (wt= 1 cwt 3 qr) fixed in RCC 1:2:4 (102 mm) thick with 5 lbs steel per cft)9” (299 mm) thick brick masonary wall set in 1:6 cement mortar 6” (1152mm ) thick cement concrete 1:3:6 in foundation 1” thick cement concrete 1:2:4 flooring ½” (12.5mm) thick cement plaster 1:3 to all inside wall surface and to top i/c providing and fixing M.S. foot beyond 2½” depth curring excavation back filling and disposal of surplus earth etc; complete (P.H.S.I.NO:1, P/49) | 18820/- | Each | 37,640/- |
| 15 | 27000.0 Cft | Refilling the excavated stuff in trenches 6” thick layers i/c watering ramming to full compaction etc: complete (P.H.S.I.NO;24, P/53) | 2760/- | P%0cft | 74,520/- |
|  |  |  |  | Total | 12,00,455/- |
|  |  | GROUP ‘D’ P/LJ & TESTING PVC TRUNK MAIN 4” DIA |  |  |  |
| 01 | 2000.0 Cft | Excavation for pipe lines into trenches and pits in all kind of soil i/c trimming and dressing sides to true alignment and Shape leveling of beds of trenches to correct level and grade cutting joint holes and disposal of surplus earth within a one chain as directed by Engineer incharge providing fence guards lights flags and temporary crossing for non vehiculartraffic where ever required lift upto 5’ft(1.52m) and lead upto one chain (30.5mm) (P.H.S.I.NO; 4, P/60) | 4650/- | P%0cft | 9,302/- |
| 02 | 4000.0 Cft | Excavation for pipe lines into trenches and pits in soft rock soil i/c trimming and dressing sides to true alignment and Shape leveling of beds of trenches to correct level and grade cutting joint holes and disposal of surplus earth within a one chain as directed by Engineer incharge providing fence guards lights flags and temporary crossing for non vehicular Traffic where ever required lift upto 5’ft(1.52m) and lead upto one chain (30.5mm) (P.H.S.I.NO; 6, P/62) | 10770/- | P%0cft | 43,080/- |
| 03 |  | Providing PVC pipe of class ‘B’ (equivalent make)fixing in trenches i/c cutting and jointing with “Z” joints with one rubber rings i/c testing with water to head of 61 meter or 200’ft.(P.H.S.I.NO;1,P/22) |  |  |  |
|  | 1000.0Rft | 4” dia | 137/- | P/Rft | 137,000/- |

(Cont: on page No:06)

(Page No;06)

| Item No: | Quantity | Description of Item to be executed at site | Rate | Unit | Amount |
| --- | --- | --- | --- | --- | --- |
| 01 | 02 | 03 | 04 | 05 | 06 |
| 04 |  | P.V.C. ‘Z’ Joints fitting (P.H.S.(M)I-16, P/28 |  |  |  |
|  | 2 Nos: | PVC BEND 4” dia 900 | 956/- | Each | 1,912/- |
|  | 2 Nos: | Tee 4” x 4” | 1181/- | Each | 2,362/- |
| 05 |  | P.V.C Socket (P.H.S(M)I-16,P/113) |  |  |  |
|  | 10 Nos: | 4” dia | 1125/- | Each | 11,250/- |
| 06 |  | C.I. Gibault joint(P.H.S.I(M)-14, P/16) |  |  |  |
|  | 5 Nos: | 4” dia | 731/25 | Each | 3,656/- |
| 07 |  | Making joints specials fitting (i/c laying of specials and cost of solvent cement of required diameter and testing the joints alongwith PVCPipe line ‘B’ ‘C’ and ‘D’ class of the specified pressure and making Good to all leaky joints etc: complete(P.H.S.I.NO:1, P/35) |  |  |  |
|  | 14 Nos: | 4” dia | 70/- | Each | 980/- |
| 08 |  | Supplying G.I. Pipe of approved quality to be tested with pressure Head 200 ft (Medium) (S.S.O.M.I.NO;1/h, I (iii) P/125) |  |  |  |
|  | 100.0 Rft | 4” dia | 431/93 | P/Rft | 43,193/- |
| 09 |  | Providing & fixing M.S.flange made of M.S. steel 3/8” 4” dia making holes facing welding, nuts bolts, rubber packing, white lead fitter, catage etc complete. (R.A. Based on schedule item) |  |  |  |
|  | 10 Nos: | 4” dia | 598/75 | Each | 5,968/- |
| 10 |  | Laying C.I/M.S pipe (S&S) and specials and fixing in Trenches i/c cutting fitting and testing with water to specified Pressure (P.H.S.I.NO;B, P/78) |  |  |  |
|  | 100.0 Rft | 4” dia | 7/76 | Each | 776/- |
| 11 | 40.08 Kg: | Supplying C.I. Bend with flanged ends with holes i/c turning and facing of flanged (S.O.M.I.NO;8, P/11) | 120/- | P/kg: | 4,810/- |
| 12 |  | C.I Sluice Valve heavy pattern test(test pressure 21.0 kg /sq.com or 300ib/sq .inch)imported (P.H.S.M.NO:2, P/9) |  |  |  |
|  | 1 No: | 4’ dia | 5460/- | Each | 5,460/- |
| 13 | 1 No; | Providing chamber 3”x2” (915x615mm) inside dimension 4½”(1372 mm) deep as per approved design for sluice valve 3” to 12” dia with 18” (457mm) dia inside cast iron cover and frame (wt= 1 cwt 3 qr) fixed in RCC 1:2:4 (102 mm) thick with 5 lbs steel per cft)9” (299 mm) thick brick masonary wall set in 1:6 cement mortar 6” (1152mm ) thick cement concrete 1:3:6 in foundation 1” thick cement concrete 1:2:4 flooring ½” (12.5mm) thick cement plaster 1:3 to all inside wall surface and to top i/c providing and fixing M.S. foot beyond 2½” depth curring excavation back filling and disposal of surplus earth etc; complete (P.H.S.I.NO:1, P/49) | 18820/- | Each | 18,820/- |

(Cont: on page No:07)

(Page No:07)

| Item No: | Quantity | Description of Item to be executed at site | Rate | Unit | Amount |
| --- | --- | --- | --- | --- | --- |
| 01 | 02 | 03 | 04 | 05 | 06 |
| 14 | 5400.0 Cft | Refilling the excavated stuff in trenches 6” thick layers i/c watering ramming to full compaction etc: complete (P.H.S.I.NO;24, P/53) | 2760/- | P%0cft | 14,904/- |
|  |  |  |  | Total | 303,471/- |
|  |  | GROUP ‘E’ P/L/J & TESTING PVC TRUNK MAIN 3” DIA |  |  |  |
| 01 | 17938.0 cft | Excavation for pipe lines into trenches and pits in all kind of soil i/c trimming and dressing sides to true alignment and Shape leveling of beds of trenches to correct level and grade cutting joint holes and disposal of surplus earth within a one chain as directed by Engineer incharge providing fence guards lights flags and temporary crossing for non vehicular traffic where ever required lift upto 5’ft(1.52m) and lead upto one chain (30.5mm) (P.H.S.I.NO; 4, P/60) | 4650/- | P%0cft | 83,412/- |
| 02 | 35880.0 Cft | Excavation for pipe lines into trenches and pits in soft rock soil i/c trimming and dressing sides to true alignment and Shape leveling of beds of trenches to correct level and grade cutting joint holes and disposal of surplus earth within a one chain as directed by Engineer incharge providing fence guards lights flags and temporary crossing for non vehicular traffic where ever required lift upto 5’ft(1.52m) and lead upto one chain (30.5mm) (P.H.S.I.NO; 6, P/62) | 10770/- | P%0cft | 386,428/- |
| 03 |  | Providing PVC pipe of class ‘B’ (equivalent make)fixing in trenches i/c cutting and jointing with “Z” joints with one rubber rings i/c testing with water to head of 61 meter or 200’ft.(P.H.S.I.NO;1,P/22) |  |  |  |
|  | 2970.0 Rft | 4” dia | 137/- | P/Rft | 406,890/- |
|  | 6000.0 Rft | 3” dia | 90/- | P/Rft | 540,000/- |
| 04 |  | P.V.C. ‘Z’ Jointing fitting (P.H.S.I.I-16, P/28) |  |  |  |
|  | 4 Nos: | i) P.V.C. Bend 3” dia | 731/- | Each | 2,924/- |
|  | 5 Nos: | 4” dia | 956/- | Each | 4,780/- |
|  |  | ii)TEE EQUAL |  |  |  |
|  | 6 Nos: | 4”x4”x3” | 1181/- | Each | 7,086/- |
|  | 5 Nos: | 3”x3”x3” | 900/- | Each | 1,800/- |
| 05 |  | C.I. Sluice valve heavy pattern (test pressure 21.0 kg:/sq: cm or 300lbs/sq: inch) (Imported) (P.H.S.I-(M)2, P/9) |  |  |  |
|  | 3 Nos: | 4” dia | 5460/- | Each | 16,380/- |
|  | 3 Nos: | 3” dia | 4290/- | Each | 12,870/- |
| 06 |  | C.I. Tail Piece (P.H.S.I-(M)2, P/9) |  |  |  |
|  | 6 Nos: | 4” dia | 487/50 | Each | 2,925/- |
|  | 6 Nos: | 3” dia | 375/- | Each | 2,250/- |

(Cont: on page No:08)

(Page No:08)

| Item No: | Quantity | Description of Item to be executed at site | Rate | Unit | Amount |
| --- | --- | --- | --- | --- | --- |
| 01 | 02 | 03 | 04 | 05 | 06 |
| 07 |  | Making joints specials fitting (i/c laying of specials and cost of solvent cement of required diameter and testing the joints alongwith PVCPipe line ‘B’ ‘C’ and ‘D’ class of the specified pressure and making Good to all leaky joints etc: complete(P.H.S.I.NO:1, P/35) |  |  |  |
|  | 10 Joints | 4” dia | 70/- | P/Joint | 700/- |
|  | 6 Joints | 3” dia | 56/- | P/Joint | 354/- |
|  | 6 Nos: | Providing chamber 3”x2” (915x615mm) inside dimension 4½”(1372 mm) deep as per approved design for sluice valve 3” to 12” dia with 18” (457mm) dia inside cast iron cover and frame (wt= 1 cwt 3 qr) fixed in RCC 1:2:4 (102 mm) thick with 5 lbs steel per cft)9” (299 mm) thick brick masonary wall set in 1:6 cement mortar 6” (1152mm ) thick cement concrete 1:3:6 in foundation 1” thick cement concrete 1:2:4 flooring ½” (12.5mm) thick cement plaster 1:3 to all inside wall surface and to top i/c providing and fixing M.S. foot beyond 2½” depth curring excavation back filling and disposal of surplus earth etc; complete (P.H.S.I.NO:1, P/49) | 18820/- | Each | 112,920/- |
|  | 48436.0 Cft | Refilling the excavated stuff in trenches 6” thick layers i/c watering ramming to full compaction etc: complete (P.H.S.I.NO;24, P/53) | 2760/- | P%0cft | 133,684/- |
|  |  |  |  | Total | 17,15,403/- |
|  |  | GROUP ‘F’ P/I PUMPING MACHINERY 10 BHP |  |  |  |
| 01 | 2 Sets | Supplying & Installing in position i/c transportation to site of work at Khosa village near power house colony pumping set consisting of A,C electric motor 10BHP 3phase 50 cycles 400/420 volts of (2900 RPM) siemens made (Type ILA 7 – 132-3AA 60 coupled with centrifugal pump (Type 68-200) size 34”x2½” mounted on common steel i/c frame or base plate capable of discharge 110 gallons per minute against a head of 130 i/c installing of Pumping set on cement concrete foundation Ratio 1:2:4 and C.C. 1:4;8 with ballast with M.S. nuts and bolts of 5/8” dia to be embedded in C.C Foundation i/c Providing and fixing on prepared board iron (clade) triple pole and switch recessed in the wall size 3”x½” i/c providing phase indication bulbs with holders and providing internal electric wiring for main with 3-7/0.85(7/0.036)PVC insulated wire 660 volts grade in 25 mm (1”)PVC conduit on surface as required i/c providing and fixing earth set with 2’x2’x1/4’ copper plate buried in ground at depth of12”deep by 72 hours etc: complete | 331230/- | P/Set | 662,460/- |

(Page No:09)

| Item No: | Quantity | Description of Item to be executed at site | Rate | Unit | Amount |
| --- | --- | --- | --- | --- | --- |
| 01 | 02 | 03 | 04 | 05 | 06 |
| 02 |  | Manufacturing fixing M.S.pipe 3” dia made out of M.S. sheet of 72 mm thick (R.A. Based on sch: ) |  |  |  |
|  | 44.00 Rft | 3” dia | 385/89 | P/Rft | 16,979/- |
|  | 22.00 rft | 4” dia | 506/60 | P/Rft | 11,145/- |
| 03 |  | Manufacturing supplying M.S. flange of different dia made out of M.S. sheet of various thickness i/c cutting drilling welding oflanges with M.S. pipe (M.R) |  |  |  |
|  | 8 Nos: | 3” dia | 500/89 | Each | 4,007/- |
|  | 10 Nos: | 4” dia | 696/75 | Each | 5,968/- |
| 04 |  | C.I.Bend/tees with flanges and with holes i/c turning and facing of flanged for all size (P.H.S.(M)I-8, P/11) |  |  |  |
|  | 0.26 Cwt | 3” dia | 6096/- | P/Cwt | 1,585/- |
|  | 0.56 Cwt | 4” dia | 6096/- | P/Cwt | 3,414/- |
| 05 | 0.24 Cwt | C.I. Tee with one end flanged and spigot at the order end for all Size (P.H.S.I.NO;M , P/11) | 6096/- | P/Cwt | 1,468/- |
| 06 | 0.41 Cwt | C.I.Tapper flat bottomed or central tapered flange end/tees bend with holes i/c turning and facing of flange for all size (P.H.S.(M)I-1, P/11) | 6096/- | P/Cwt | 2,499/- |
| 07 |  | C.I.Sluice valve heavy pattern (Test pressure 21.0kg/sq:cm or 300lb/sq: inch)(Imported) .(P.H.S.I.(M) NO:2, P/9) |  |  |  |
|  | 2 Nos: | 3” dia | 4290/- | Each | 8,580/- |
| 08 |  | Foot valve heavy pattern with cone type gate (P.H.S.(M)I-4, P/10) |  |  |  |
|  | 2 Nos: | 3” dia | 731/25 | Each | 1,463/- |
| 09 |  | Reflex valve heavy pattern test pressure 300lbs/sq: inch in pointed (P.H.S(M) -6, P/10) |  |  |  |
|  | 2 Nos: | 3” dia | 1543/75 | Each | 3,088/- |
| 10 |  | Jointing C.I/M.S flanged pipes and specials flanged and inside a trenches i/c supplying rubber packing of the required thickness nuts bolts with washers etc and other tools required for jointing and testing the joints to the specified pressure etc: complete.(P.H.S.I.NO;1, P/35) |  |  |  |
|  | 12 Joints | 3” dia | 499/- | P/joint | 5,988/- |
|  | 8 joints | 4” dia | 513/- | P/Joint | 4,104/- |
|  |  |  |  | Total | 732,743/- |

(Cont: on page NO;10)

(Page No:10)

A   B  S   T   R   A   C   T

| Group Name | Schedule Item | Non Schedule | Total |
| --- | --- | --- | --- |
| Group ‘A’ Construction of Storage tank size 120’x60’ | 18,26,857/- | ----- | 18,26,857/- |
| Group ‘B’ Construction of Pump House size 10’x12’ | 225,310/- | ----- | 225,310/- |
| Group ‘C’ P/L/J & Testing PVC Pipe Rising Main 4”dia | 12,00,455/- | ----- | 12,00,455/- |
| Group ‘D’ P/L/J & Testing PVC Pipe Trunk Main 4”dia | 303,471/- | ----- | 303,471/- |
| Group ‘E’ P/L/J & Testing PVC Pipe Trunk Main 3’ dia | 17,15,403/- | ----- | 17,15,403/- |
| Group ‘F’ P/I Pumping Machinery 10 BHP | 70,283/- | 662,460/- | 732,743/- |
| Total | 53,41,779/- | 662,460/- | 60,04,239/- |

Amount Total (a) + (b) Rs:___________________

________% above / below on the rates of CSR, Amount to be added / deducted on the basis

Total (b) of premium quoted ______________

Total (A) = a + b in works & Figures______________________________________________

Contractor 						 	EXECUTIVE ENGINEER

PUBLIC HEALTH ENGG:DIVISION

JAMSHORO