BILL OF QUANTITIES

Description and rate of items based on Composite Schedule of Rates

| Name of Works | (Provincial ADP # 158 of 2015-16) Establishment of Public Schools in Sindh @ UC Bhitshah Taluka Hala District Matiari 2008-09 Program (01 Unit) @ Internal & External Facilities (Roads) |
| --- | --- |

| Item No. | Quantities | Description of item to be executed at site | Rate | Unit | Amount in Rupees |
| --- | --- | --- | --- | --- | --- |
| 01 | 167442 | COMPACTION 85% DENSITY Earth work for road embankment by dumpers, Tractor Trolleys , including ploughing , mixing , cold breaking, dressing , compacting with optimum moisture content. Lead upto 2.00 miles and lift upto 5.00 ft in all types of soil except rock. (if earth work is done by other than departmental) | 9448/05 | P%0 Cft | 15,82,000/- |
|  | 20462 | COMPACTION 95-100% DENSITY | 11977/87 | P%0 Cft | 2,45,091/- |
| 02 | 8769 | Preparing Sub Base Course by supplying and spreading stone metal 1 ½” to 2” gauge of approved quality from approved quarry in required thickness to proper camber & grade i/c hand picking filling voids with 10Cft. screenings having plasticity index of not more than 6% of suitable quality watering and compacting to achieve 98-100% density as per modified AASHO Specification. Rate i/c all costs of materials T&P carriage upto site of work (3 chains) | 8030/32 | P.%Cft | 7,04,178/- |
| 03 | 2923 | Laying brick on end edging including supply of 9” x 4 ½” x 3” 1st class burnt bricks excavation for laying edging with small size parallel to the road. (Rate includes all cost of materials, T&P, labours and carriage upto site of work (3 chains) | 2729/95 | P.%Rft | 79,796/- |
| 04 | 8769 | Preparing base Course i/c supply and spreading stone metal of approved quality properly graded to maximum size to 1 1/2” in required thickness to proper camber & grade i/c supplying and spreading 15 cft screenings and non-plastic quarry fines, filling depressions with stone metal after initial rolling i/c watering and compaction the same so as to achieve 100% density as per modified AASHO specifications. Rate i/cs providing and using template. Camber plates screens, forms as directed (Rate includes all costs of materials T & P and carriage upto 3 chains. | 9051/31 | P%Cft | 7,93,709/- |

| Item No. | Quantities | Description of item to be executed at site | Rate | Unit | Amount in Rupees |
| --- | --- | --- | --- | --- | --- |
| 05 | 17538 | Providing surface dressing 1st coat on new or existing surface with 30 lbs bitumen and 4.0 cft of bajri of required size i/c cleaning the road surface rolling etc complete. (Rate includes all cost of materials T&P and carriage up to 3 chains) | 1611/37 | P.%Sft | 2,82,602/- |
| 06 | 17538 | Providing 1 ½ ” thick carpet (consolidated) premixed in proper camber & grade i/c supplying 15cft crushed bajri 5cft hill sand of approved quality and grade bitumen 93 lbs of 80-100 penetration i/c mixing. Mechanical mixer in required proportion i/c heating materials & cleaning the road surface. (Hill sand 3cft) for mixing and 2 Cft for dusting). Rate i/c all cost of materials T&P labour and carriage upto site of work | 6227/44 | P.%Sft | 10,92,168/- |
| TOTAL |  |  |  |  | 47,79,544/- |

____% above / below on the rates of CSR Rs.__________/-		Amount to be added/deducted on the basis

of premium quoted TOTAL

Total = in words & figures____________________________________________________________________________

__________________________________________________________________________________________________

Contractor								Executive Engineer

/									      Education Works Division

Note: Any omission / error if occurs will be corrected				          Matiari